3GPP TSG-RAN WG1 Meeting #121	Tdoc R1-2503754

St Julian’s, Malta, May 19th – May 23rd, 2025

Agenda Item:	9.15.2

Source:	Ericsson

Title:	UE features for NR MIMO Phase 5

Document for:	Discussion, Decision

# Introduction

In this contribution we provide our views on UE features for Rel-19 NR MIMO.

# Discussion

## UE initiated beam management

In the agreed UE feature list [1], there are some brackets and yellow highlighting. We will address these in turn. Our proposed updates are collected in the Appendix.

59-1-1:

  - This is the basic feature group, and it applies to event-2 and Mode A. The brackets in the FG name and consequence if not supported should be removed.

  - There is no need for any pre-requisite feature group. The underlying functionality is very basic (e.g., aperiodic beam reporting), and we typically do not include that.

  - FR1/FR2 differentiation is not applicable as the FG is reported per band.

59-1-2:

  - There is no need for any component that limits X: this RRC parameter is introduced to give the gNB enough time to prepare for the reception of the PUSCH, whereas the UE can prepare the PUSCH even before transmitting the first PUCCH. Note: there is a typo in the endorsed feature list [1]: this component should be highlighted and in brackets.

  - FR1/FR2 differentiation is not applicable as the FG is reported per band.

  - The note can be removed. This assumes that the event evaluation is independent of how the report is delivered, i.e., if the UE supports Mode B for event-2 and additionally indicates that it supports, e.g., event-1, the UE will have to support event-1 both for Mode A and Mode B.

59-1-3:

  - In RAN1#120bis, the following was agreed

Agreement

Regarding triggering event determination for Event 2, on the measurement window for initiating the UE-initiated/event-driven beam reporting procedure, support Option-4.

- Option-4: If an Event-2 instance for a new beam is obtained at the time t and the timer for the new beam is not running, UE starts the timer for the new beam, where the expiry time of the timer is equal to the NW-configured length of the time window (T_window)

  - Note: Timer is new beam specific.

- Note: T_window is the agreed time window parameter for measurement.

- Introduce separate UE capability to limit the number of timers. There is only 1 timer per new beam.

Above agreement is captured in RAN1 specifications.

Agreement

Regarding triggering event determination for Event 2, on the measurement window for initiating the UE-initiated/event-driven beam reporting procedure, support Option-4.

- Option-4: If an Event-2 instance for a new beam is obtained at the time t and the timer for the new beam is not running, UE starts the timer for the new beam, where the expiry time of the timer is equal to the NW-configured length of the time window (T_window)

  - Note: Timer is new beam specific.

- Note: T_window is the agreed time window parameter for measurement.

- Introduce separate UE capability to limit the number of timers. There is only 1 timer per new beam.

Above agreement is captured in RAN1 specifications.

- The agreement introduces a timer as a tool to determine when to trigger the reporting, and since the handling of a timer may be associated with some complexity, it was agreed to include this as a UE capability. Since the timer is an integral part of the functionality described in FG 59-1-3, it makes sense to include the maximum number of running timers as a component in FG 59-1-3. If event instances are obtained for more new beams than the maximum number of timers, it is up to the UE which new beams to evaluate.

  - There is no need for any additional pre-requisite feature group.

  - FR1/FR2 differentiation is not applicable as the FG is reported per band.

59-1-4:

  - We propose that this FG applies both to Mode A and Mode B. A UE that supports event-1, supports event-1 for the same modes as for event-2. In other words, if the UE reports support for FG 59-1-1, FG 59-1-2 and FG 59-1-4, the UE support Mode A for event-2 and event-1, and Mode B for event-2 and event-1. The reason for this is that the event evaluation should be relatively independent from the reporting procedure.

  - We propose to make FG 59-1-1 a prerequisite for this FG. This would mean that the UE would inherit several of the components from FG 59-1-1, and that the components 2, 3, and 4 can be removed.

  - FR1/FR2 differentiation is not applicable as the FG is reported per band.

59-1-5:

  - We propose to make FG 59-1-1 a prerequisite for this FG. This would mean that the UE would inherit several of the components from FG 59-1-1, and that the components 2 and 4 can be removed.

  - FR1/FR2 differentiation is not applicable as the FG is reported per band.

  - There is no need for any candidate values for component 3: the UE should support all possible values of Q (1..8)

59-1-6:

  - As the other 59-1-x feature groups, FG 59-1-6 should be reported per band.

  - FDD/TDD differentiation is n/a. FR1/FR2 differentiation is n/a. Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 is n/a.

## UE features for CSI enhancements

### FGs related to refined Type I codebooks

For the features related to refined Type I codebooks, the reporting type is yet to be agreed. In our view, the reporting type for these FGs should be ‘per band’.  Hence, we propose the following:

Support ‘per band’ reporting for the following FGs:

- FGs related to Single panel Type I codebooks (i.e., FGs 59-2-1-1, 59-2-1-1a, 59-2-1-1b, 59-2-1-1c, 59-2-1-1d, 59-2-1-1e)

- FGs related to Multi panel Type I codebooks (i.e., 59-2-1-2, 59-2-1-2a, 59-2-1-2b)

### FGs related to refined Type II codebooks

For the features related to refined Type II codebooks, the reporting type is yet to be agreed. In our view, the reporting type for these FGs should be the same as the respective parent codebooks supported in previous releases.  We have the following proposals:

For FGs 59-2-1-3, 59-2-1-3a, and 59-2-1-3b, support the same reporting type as Rel-16 eType-II codebook.

For FGs 59-2-1-4 and 59-2-1-4a, support the same reporting type as Rel-17 FeType-II codebook.

For FGs 59-2-1-5, 59-2-1-5a, and 59-2-1-5b, support the same reporting type as Rel-18 eType-II Doppler codebook.

### FG 59-2-1-6

This feature should only be applicable to aperiodic CSI-RS as for periodic CSI-RS slot offset can already be configured per CSI-RS resource.

Support only aperiodic CSI-RS in FG 59-2-1-6.

### FG 59-2-1-7

As this feature was only agreed to be applicable to Type I single panel codebooks, we propose to set the pre-requisites of this feature as one of 59-2-1-1, 1a, and 1b, or one of 59-2-1-1c, 1d, or 1e.  The reporting type for this feature should be ‘per band’.

For FG 59-2-1-7, the pre-requisites should be one of 59-2-1-1/1a/1b, or one of 59-2-1-1c/1d/1e.

For FG 59-2-1-7, the reporting type should be ‘per band’.

## UE features for 3 Tx

During RAN1#120-bis, the list of RAN1 UE features for Rel-19 NR MIMO Phase 5 [1]was further discussed including UE features for reporting 3 Tx PUSCH capability. The following sections address specific issues related to the tentatively agreed components of the feature groups for 3 Tx PUSCH capability.

### Non-codebook UE capability

The feature group to report capability for non-codebook based PUSCH for 3 Tx for single TRP (S-TRP) is shown below:

| 59. NR_MIMO_Ph5 | 59-3-1 | Non-codebook based PUSCH transmission for 3TX for single TRP | 1. Maximal number of supported layers (non-codebook transmission scheme) 2. Maximum number of SRS resource per set (SRS set use is configured as for non-codebook transmission) 3. Maximum number of simultaneous transmitted SRS resources at one symbol | Component 1 candidate values: {1, 2, 3} Component 2 candidate values: {1,2,3[, 4]} Component 3 candidate values: {1,2,3[, 4]} |
| --- | --- | --- | --- | --- |

It is decided that the maximum number of 1-port SRS resources per SRS set to support 3 Tx non-codebook PUSCH for S-TRP could take values from 1-3, while the maximum value of 4 is FFS. Meanwhile, the Rel-15 UE capability parameter for the maximum number of SRS resources in non-codebook based operation allows values from 1-4, with the maximum number of layers being 1, 2, or 4. In this case, the maximum number of SRS resources and layers align. However, it should also be observed that a Rel-15 UE can declare, for example, a maximum one layer capability while supporting up to 4 SRS resources in non-codebook based operation.

For 3 Tx operation, the maximum number of layers is 3. One possible reason for limiting the 3 Tx non-codebook capability to a maximum of 3 SRS resources is to simplify operation for UEs with 3 Tx chains that do not virtualize their antennas. Such UEs could toggle their Tx chains on or off without needing to switch between antennas or implement virtualization. However, these UEs could alternatively be designed to support at most 3 SRS resources. On the other hand, maximizing the number of SRS resources offers potential diversity benefits, especially when the maximum number of layers is lower. This is in our understanding a primary motivation for having the smaller layer capability than the number of SRS resources discussed above.

There does not seem to be a benefit to reduce the maximum number of SRS resources for non-codebook based operation below the 4 supported in Rel-15. Supporting a maximum of 3 resources would instead limit the potential for diversity gain, and be less consistent with the principle of allowing a maximum layer capability that is less than the number of SRS resources.

Support maxNumberSRS-ResourcePerSet = 4 for Rel-19 3 Tx non-codebook based operation.

Similar to S-TRP, it was tentatively decided that the maximum number of 1-port SRS resources per SRS resource set to support 3 Tx non-codebook PUSCH for multi TRP (M-TRP) could take values from 1-3, while the maximum value of 4 is FFS, as shown below.

| NR MIMO Phase 5 | 59-3-4a | M-TRP PUSCH repetition (type A) of 3-antenna-port PUSCH transmission – non-codebook based | Support of M-TRP PUSCH repetition for 3-antenna-port PUSCH transmission with type A for non-codebook based - sequential mapping for repetitions larger than 2 - cyclic mapping for 2 repetitions 2. Support of two SRS resource sets with usage set to 'non-codebook' 3. Supported number of SRS resources in one SRS resource set | Component 3 candidate values: {1,2,3[.4]} Note: Two linked PDCCH candidates are not expected to be associated with different CORESETPoolIndex values |
| --- | --- | --- | --- | --- |
| NR MIMO Phase 5 | 59-3-5a | M-TRP PUSCH repetition (type B) of 3-antenna-port PUSCH transmission – non-codebook based | Support of M-TRP PUSCH repetition for 3-antenna-port PUSCH transmission with type B for non-codebook based - sequential mapping for repetitions larger than 2 - cyclic mapping for 2 repetitions 2. Support of two SRS resource sets with usage set to 'non-codebook' 3. Supported number of SRS resources in one SRS resource set | Component 3 candidate values: {1,2,3[,4]} Note: Two linked PDCCH candidates are not expected to be associated with different CORESETPoolIndex values |

Based on the discussion for the S-TRP non-codebook case, the supported number of SRS resources in one SRS resource set should include the value of 4. This also maintains the consistency between S-TRP and M-TRP non-codebook schemes.

Based on the discussion regarding the S-TRP non-codebook case and to maintain the consistency between S-TRP and M-TRP non-codebook schemes, the supported number of SRS resources in one SRS resource set should include the value of 4 in Component 4 of FG 59-3-4a and 59-3-5a.

Support the value of 4 for the maximum number of SRS resources in one SRS resource set for Rel-19 3 Tx non-codebook based M-TRP operations.

### Codebook UE capability

The feature group to report capability for codebook based PUSCH for 3 Tx for S-TRP is shown below:

| 59. NR_MIMO_Ph5 | 59-3-2 | Codebook based PUSCH transmission for 3TX for single TRP | 1. Maximal number of PUSCH MIMO layers for codebook-based PUSCH 2. Maximum number of 4-port SRS resources per SRS resource set with usage set to 'codebook’ for codebook-based 3Tx PUSCH [3. Maximum number of simultaneous transmitted SRS resources at one symbol with usage set to 'codebook’ for codebook-based 3Tx PUSCH] 4. Codebook based PUSCH transmission with port 1003 disabled when 4 port SRS resources with port 1003 disabled are configured to the UE | Component 1 candidate values: {1, 2,3} Component 2 candidate values: {1,2} [Component 3 candidate values: {1,2,3}] |
| --- | --- | --- | --- | --- |

In Section 7.1 of TS 38.213, for full power Mode 0 operation (i.e. ul-FullPowerTransmission in PUSCH-Config is set to fullpower), there is no dependency on the maximum number of SRS ports supported by the UE in one SRS resource. On the other hand, for Rel-15 power-scaling, power is split among PUSCH ports according to the maximum number of SRS ports supported by the UE in one SRS resource, as illustrated below:

- else, if each SRS resource in the SRS-ResourceSet with usage set to 'codebook' has more than one SRS port, the UE scales the linear value by the ratio of the number of antenna ports with a non-zero PUSCH transmission power to the maximum number of SRS ports supported by the UE in one SRS resource.

In RAN1#120-bis, it was agreed that the existing power-scaling rules in TS 38.213 would remain unchanged for 3 Tx operation. However, for 3 Tx codebook based PUSCH, the UE will report a capability indicating support for a maximum of 4 SRS ports in a resource, with port 1003 disabled, as reflected in Component 4 of FG 59-3-2. This configuration is open to two conflicting interpretations:

- The maximum number of SRS ports supported by the UE in a single resource is 3 (ports {1000, 1001, 1002}, excluding port 1003), which would achieve the intended power scaling behavior for 3 Tx PUSCH.

- The maximum number of SRS ports is 4, since the UE reports 4 ports in an SRS resource. In this case, Rel-15 power-scaling would apply based on four ports, resulting in a power split across four ports even though the UE physically transmits using only three Tx chains, potentially preventing it from reaching full power.

Furthermore, the legacy specification has explicitly mentioned the maximum number of supported SRS ports supported by the UE. Specifically, for Rel-15 {1, 2, 4} Tx UEs, this capability is specified in Component 7 of FG 2-53:

| 2-53 | SRS resources | 1) Maximum number of aperiodic SRS resources (configured to UE) per BWP 2) Maximum number of aperiodic SRS resources (configured to UE) per BWP per slot 3) Maximum number of periodic SRS resources (configured to UE) per BWP 4) Maximum number of periodic SRS resources (configured to UE) per BWP per slot 5) Maximum number of semi-persistent SRS resources (configured to UE) per BWP 6) Maximum number of semi-persistent SRS resources (configured to UE) per BWP per slot 7) Maximum number of SRS port per resource | Mandatory with capability signalling Component-1: candidate value: {from 1, 2, 4, 8, 16} Component-2 candidate value: {1,2,3,4,5,6} Component-3: candidate value: {from 1, 2, 4, 8, 16} Component-4 candidate value: {1,2,3,4,5, 6} Component-5: candidate value: {from 1, 2, 4, 8, 16} } Component-6 candidate value: {1, 2,3,4,5, 6} Component-7 candidate values: {1, 2, 4} |
| --- | --- | --- | --- |

The Rel-18 8 Tx codebook-based capability is signaled differently from the Rel-15 {1, 2, 4} Tx capabilities and is captured in Component 2 of FG 40-7-1, which defines the capability for 8 Tx codebook based PUSCH:

| 40-7-1 | Basic features for Codebook-based 8Tx PUSCH | 1. Maximum number of PUSCH MIMO layers for codebook based PUSCH 2. Maximum number of 8 port SRS resources per SRS resource set with usage set to 'codebook’ for codebook-based 8Tx PUSCH 3. SRS 8 Tx ports—codebook | Component 1 candidate values: {1,2 ,3,4 ,5,6,7,8} Component 2 candidate values: {1,2} Component 3 candidate values: {noTDM, TDM and noTDM} Note: the candidate value signalled in component 3 only applies to codebook2/codebook3/codebook4 A UE that supports FG 40-7-1 must support at least one of FGs 40-7-1a/b/c/d |
| --- | --- | --- | --- |

Accordingly, we propose further specification enhancement to remove any ambiguity in the specification and prevent incorrect Rel-15 power-scaling behavior for 3 Tx codebook based PUSCH, with two alternatives.

In the first alternative, the UE behavior can be clarified in TS 38.214 to indicate that the maximum number of SRS ports supported by UE used in the scaling of PUSCH transmission power as described in clause 7.1 of TS 38.213, is calculated using 3 when [UE feature 59-3-2] is supported along with the reported maximum number of SRS ports supported by the UE in one SRS resource for Rel-15 and/or Rel-18 operation. For example, when a 4 Tx UE reports maxNumberSRS-Ports-PerResource=4 according to 2-53 and support for 59-3-2 (indicating support for 3 Tx and a 4-port SRS resource with port 1003 disabled), the number of ports supported by UE in a SRS resource for SRS transmission is 4 (to support Rel-19 3 Tx as well as Rel-15 4 Tx codebook based PUSCH), or else 3 with support for 59-3-2 (but without maxNumberSRS-Ports-PerResource=4 and so no support for Rel-15 4 Tx codebook based PUSCH). Note that there are two possible values for the maximum number of SRS ports in one SRS resource, corresponding to UE features 2-53 component 7 and 40-7-1 component 2 so we have ‘maximum number(s) of SRS ports’ in the proposed text.

The UE does not transmit SRS on antenna port 1003 if [RRC_parameter] is configured, where [RRC parameter] is only applicable to 4-port SRS resource(s) in an SRS resource set with usage set to ‘codebook’ or ‘antennaSwitching’. The maximum number of ports supported by the UE in an SRS resource, for scaling of PUSCH transmission power as described in clause 7.1 of [6, TS 38.213], is calculated using 3, if [UE feature 59-3-2] is supported, and the maximum number(s) of SRS ports supported by the UE in one SRS resource.

In the second alternative, which aligns with the agreement made in RAN1#117 to allow reporting a maximum of 3 SRS ports for 3 Tx PUSCH operations, as shown below,

Agreement (RAN1#117) For codebook-based UL transmission by a 3TX UE, subject to its capability, - A 3TX UE may report a maximum number of 3 layers - A 3TX UE may report a maximum number of SRS ports of up to 3 Note: SRS resource definition is not changed nor the number of SRS ports in the SRS resource.

This approach adds a note in FG 59-3-2 to indicate that the number of ports supported by UE for transmission in an SRS resource is 3. The ‘for transmission’ is needed since the UE must support a 4 port SRS resource. It is also aligned with the agreement below. With this note, the maximum number of SRS ports supported by the UE would be calculated according to the number of ports supported by the UE ‘for transmission’, which may or may not be clear.

Agreement (RAN1#121bis) Adopt the following changes in Clause 6.2.1 in TS38.214. ### 6.2.1 UE sounding procedure -------------------------------------------Unchanged parts are omitted------------------------------------------- [The UE does not transmit SRS on antenna port 1003 if [RRC_parameter] is configured.] where [RRC_parameter] is only applicable when SRS resource(s) are configured with 4 ports with usage as codebook or antenna switching. -------------------------------------------Unchanged parts are omitted-------------------------------------------.

| 59. NR_MIMO_Ph5 | 59-3-2 | Codebook based PUSCH transmission for 3TX for single TRP | 1. Maximal number of PUSCH MIMO layers for codebook-based PUSCH 2. Maximum number of 4-port SRS resources per SRS resource set with usage set to 'codebook’ for codebook-based 3Tx PUSCH [3. Maximum number of simultaneous transmitted SRS resources at one symbol with usage set to 'codebook’ for codebook-based 3Tx PUSCH] 4. Codebook based PUSCH transmission with port 1003 disabled when 4 port SRS resources with port 1003 disabled are configured to the UE | Component 1 candidate values: {1, 2,3} Component 2 candidate values: {1,2} [Component 3 candidate values: {1,2,3}] Note: When configured according to Component 4, the number of ports supported by UE for transmission in an SRS resource is 3. |
| --- | --- | --- | --- | --- |

Another way would be to change the note to something like the following, which would be in our view clearer, since it would not require interpreting what ‘number of ports supported by the UE for transmission’ means. However, this defines the algorithm for computing the maximum number SRS ports for the specific purpose of PUSCH power-scaling, which is a new behavior rather than a capability, and so should be in the behavioral specs.

Note: When determining the maximum number of SRS ports in an SRS resource supported by the UE for the purpose of PUSCH power scaling, support for this feature corresponds to a value of 3 SRS ports in an SRS resource.

The existing UE capability reporting 4 ports in an SRS resource with port 1003 disabled can create potential ambiguity regarding the correct scaling factor for codebook based PUSCH.

Updating TS 38.214 to clarify that the maximum number of ports supported by the UE in an SRS resource, for scaling of transmission power as described in clause 7.1 of TS 38.213, is calculated using 3, if [UE feature 59-3-2] is supported as well as the maximum number(s) of SRS ports supported by the UE in one SRS resource, would help ensure correct Rel-15 power-scaling behavior for 3Tx codebook based PUSCH.

Alternatively, adding a note under FG 59-3-2 to specify that when transmitting according to Component 4, the number of ports supported by the UE for transmission in an SRS resource is 3 may help to clarify the correct Rel-15 power-scaling behavior for 3Tx codebook-based PUSCH, but could confuse readers since ‘number of SRS ports for transmission’ is used for 3 Tx, while the ‘number of SRS ports’ is used for 2, 4, and 8 Tx.

Defining a procedure for how the maximum number of SRS ports is calculated for the purpose of PUSCH power-scaling seems to be needed to avoid the ambiguity on the number of SRS ports when port 1003 is disabled. Since behavior should not be defined in UE capability specifications, this calculation does not seem appropriate to add as a note to 38.306, but instead should be defined in a procedural specification such as 38.214.

Update TS 38.214 to calculate the maximum number of ports supported by the UE in an SRS resource, for scaling of PUSCH transmission power as described in clause 7.1 of [6, TS 38.213], using 3, if [UE feature 59-3-2] is supported, and the maximum number(s) of SRS ports supported by the UE in one SRS resource.

### 3T6R antenna switching UE capability

The RAN1 UE features list for Rel-19 NR MIMO Phase 5 includes UE capabilities for supporting 3T6R antenna switching. This feature group was updated at RAN1#120-bis, as outlined below:

| NR MIMO Phase 5 | 59-3-3 | 3T6R Antenna switching | 1. Support of 3T6R SRS Tx port switching with port 1003 disabled when 4 port SRS resources with port 1003 disabled are configured to the UE 2. Report the entry number of the first-listed band with UL in the band combination that affects this DL 3. Report the entry number of the first-listed band with UL in the band combination that switches together with this UL [4. Support of 3T6R antenna switching configuration(s) as an allowing downgrading configuration of 4T8R] | Component 2 candidate value: {1,2, … 32} Component 3 candidate value: {1,2, … 32} [FFS: New component for downgrade antenna switching configurations or a new Note: This UE feature can be signalled together with srs-AntennaSwitching8T8R-r18, srs-AntennaSwitchingBeyond4RX-r17, supportedSRS-TxPortSwitch-v1610, or supportedSRS-TxPortSwitch to indicate SRS antenna switching downgrading capability for a UE with 4Rx, 6Rx or 8Rx.] |
| --- | --- | --- | --- | --- |

For the 3T6R antenna switching capability, it remains FFS whether to introduce a new component to represent downgrading options or to signal 3T6R capability with legacy antenna switching mechanisms. Additionally, in RAN1#120-bis, a new component, i.e., Component 4, was placed under FFS. This component is intended to indicate that 3T6R antenna switching can serve as downgraded configurations of 4T8R.

Firstly, Component 4 and the accompanying note appear redundant, as both aim to define associated antenna switching capabilities reportable under the 3T6R framework. Secondly, given the current lack of consensus on how to represent associated antenna switching capabilities for 3T6R, we propose defining a explicit set of such capabilities within Component 1. This set includes {t1r1, t2r2, t1r2, t3r3, t2r4, t1r4, t2r6, t1r6, t3r6, t1r8, t2r8}, aligning with the Rel-17 antenna switching capability (i.e., FG 28-8-3). In this approach, the UE reports its supported combinations from the predefined set, and the network configures one of the reported antenna switching options using a bitmap.

| NR MIMO Phase 5 | 59-3-3 | 3T6R Antenna switching for > 4 Rx | 1. Support of 3T6R SRS Tx port switching with port 1003 disabled when 4 port SRS resources with port 1003 disabled are configured to the UE 2. Report the entry number of the first-listed band with UL in the band combination that affects this DL 3. Report the entry number of the first-listed band with UL in the band combination that switches together with this UL [4. Support of 3T6R antenna switching configuration(s) as an allowing downgrading configuration of 4T8R] | Component 1 candidate values: a combination from the set {t1r1, t2r2, t1r2, t3r3, t2r4, t1r4, t2r6, t1r6, t3r6, t1r8, t2r8} Component 2 candidate value: {1,2, … 32} Component 3 candidate value: {1,2, … 32} [FFS: New component for downgrade antenna switching configurations or a new Note: This UE feature can be signalled together with srs-AntennaSwitching8T8R-r18, srs-AntennaSwitchingBeyond4RX-r17, supportedSRS-TxPortSwitch-v1610, or supportedSRS-TxPortSwitch to indicate SRS antenna switching downgrading capability for a UE with 4Rx, 6Rx or 8Rx.] |
| --- | --- | --- | --- | --- |

- To prevent conflicting configurations when reporting antenna switching capabilities for 3T6R, an explicit set of supported antenna switching combinations, {t1r1, t2r2, t1r2, t3r3, t2r4, t1r4, t2r6, t1r6, t3r6, t1r8, t2r8}, can be defined for Component 1 of FG 59-3-3, consistent with Rel-17 antenna switching capability. The UE can report one or more supported combinations from this set, allowing the network to configure a valid option via a bitmap.

Include a set of antenna switching capabilities {t1r1, t2r2, t1r2, t3r3, t2r4, t1r4, t2r6, t1r6, t3r6} for Component 1 of FG 59-3-3 and update the name to include ‘for > 4 Rx’.

## Asymmetric mTRP

The UE capabilities for asymmetric mTRP have been further discussed in RAN1#120bis meeting. Here we provide our view on remaining issues.

### New component for 59-9-4b

For Asymmetric mTRP, two agreements on inter-cell use case (where UE may expect to receive SSB from UL TRP) need to be addressed with corresponding UE capability:

Agreement(RAN1#120)

For a UE provided with SSB-MTC-AddtionalPCI and not configured with multi-DCI based mTRP, support to reuse the DCI field ‘PRACH association indicator’ in DCI format 1_0 to indicate PL RS for PDCCH-order PRACH:

- The bit field index 0 of this field indicates the DL RS that DMRS of PDCCH order DCI is QCLed with is used as PL RS for PRACH;

- The bit field index 1 of this field is mapped to the additional PCI associated with the active TCI states and indicates the indicated SSB in this DCI is used as PL RS for PRACH:

  - In this case, the PRACH configuration associated with addition PCI is used.

- This DCI field is present when the corresponding RRC parameter is configured and multi-DCI based mTRP is not configured.

Agreement(RAN1#120bis)

- For the scenario that the UE is configured with PL offset in joint/UL TCI state(s), when the UE is configured with rel-19 2 TAs, the UE maintains one single downlink reference timing

- For the scenario that the UE is not configured with PL offset in joint/UL TCI state(s) and UE may expect to receive SSB from UL TRP(s), when the UE is configured with rel-19 2 TAs:

  - The UE maintains two downlink reference timings;

  - Baseline assumption for this feature is that Rx timing difference between two DL reference timings is no larger than one CP length, while it is subject to optional UE capability that the Rx timing difference between two DL reference timings can be assumed to be larger than CP length.

  - The reference point for PRACH transmission is indicated as follows:

    - if “PRACH association indicator” in DCI format 1_0 is 0, the reference timing is the first detected path (in time) of one of the corresponding downlink reference signal(s) of DL TCI state(s) of the reference cell associated with the first TAG.

    - if “PRACH association indicator” in DCI format 1_0 is 1, the reference timing is the first detected path (in time) of one of the corresponding downlink reference signal(s) of DL TCI state(s) of the reference cell associated with the second TAG

  - Above applies for the case UE is configured with SSB-MTC-additionalPCI

In addition, one FG 59-4-4b has been agreed for inter-cell two TA.

| 59. NR_MIMO_Ph5 | 59-4-4b | Support two TAs enhancement for inter-cell beam management operation | Support of two TAs without the restriction of multi-DCI based multi-TRP operation for inter-cell beam management [FFS: whether “Support of ‘PRACH association indicator’ in DCI format 1_0 to indicate PL RS for PDCCH-order PRACH” is component or separate FG] |
| --- | --- | --- | --- |

To reflect these agreements, we should introduce one new UE FG to indicate PL-RS indication for inter-cell, as well as one new UE component in 59-4-4b for reference timing indication in PDCCH order. Hence,

Introduce a component in 59-4-4b to indicate UE capability to apply reference timing indicated in PDCCH

| 59. NR_MIMO_Ph5 | 59-4-4b | Support two TAs enhancement for inter-cell beam management operation | Support of two TAs without the restriction of multi-DCI based multi-TRP operation for inter-cell beam management Support of ‘PRACH association indicator’ in DCI format 1_0 to apply reference timing of the reference cell associated with first or second TAG. [FFS: whether “Support of ‘PRACH association indicator’ in DCI format 1_0 to indicate PL RS for PDCCH-order PRACH” is component or separate FG] |
| --- | --- | --- | --- |

Introduce a FG, e.g., FG 59-4-10, to support indicate PL-RS for PDCCH-order PRACH for inter-cell.

| 59. NR_MIMO_Ph5 | 59-4-10 | DCI format 1_0 to indicate PL RS for PDCCH-order PRACH for inter-cell | supporting of “PRACH association indicator” in DCI format 1_0 to indicate PL RS for PDCCH-order PRACH for inter-cell |  | yes | n/a | DCI 1_0 indicating PL RS for PDCCH-order PRACH is not supported |
| --- | --- | --- | --- | --- | --- | --- | --- |

# Conclusion

In this contribution we provided our views on CSI enhancements and 3 Tx UE features for Rel-19 NR MIMO.

We made the following observations:

Observation 1	There does not seem to be a benefit to reduce the maximum number of SRS resources for non-codebook based operation below the 4 supported in Rel-15. Supporting a maximum of 3 resources would instead limit the potential for diversity gain, and be less consstent with the principle of allowing a maximum layer capability that is less than the number of SRS resources.

Observation 2	Based on the discussion regarding the S-TRP non-codebook case and to maintain the consistency between S-TRP and M-TRP non-codebook schemes, the supported number of SRS resources in one SRS resource set should include the value of 4 in Component 4 of FG 59-3-4a and 59-3-5a.

Observation 3	The existing UE capability reporting 4 ports in an SRS resource with port 1003 disabled can create potential ambiguity regarding the correct scaling factor for codebook based PUSCH.

Observation 4	Updating TS 38.214 to clarify that the maximum number of ports supported by the UE in an SRS resource, for scaling of transmission power as described in clause 7.1 of TS 38.213, is calculated using 3, if [UE feature 59-3-2] is supported as well as the maximum number(s) of SRS ports supported by the UE in one SRS resource, would help ensure correct Rel-15 power-scaling behavior for 3Tx codebook based PUSCH.

Observation 5	Alternatively, adding a note under FG 59-3-2 to specify that when transmitting according to Component 4, the number of ports supported by the UE for transmission in an SRS resource is 3 may help to clarify the correct Rel-15 power-scaling behavior for 3Tx codebook-based PUSCH, but could confuse readers since ‘number of SRS ports for transmission’ is used for 3 Tx, while the ‘number of SRS ports’ is used for 2, 4, and 8 Tx.

Observation 6	Defining a procedure for how the maximum number of SRS ports is calculated for the purpose of PUSCH power-scaling seems to be needed to avoid the ambiguity on the number of SRS ports when port 1003 is disabled. Since behavior should not be defined in UE apability specifications, this calculation does not seem appropriate to add as a note to 38.306, but instead should be defined in a procedural specification such as 38.214.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Support ‘per band’ reporting for the following FGs:

	FGs related to Single panel Type I codebooks (i.e., FGs 59-2-1-1, 59-2-1-1a, 59-2-1-1b, 59-2-1-1c, 59-2-1-1d, 59-2-1-1e)

	FGs related to Multi panel Type I codebooks (i.e., 59-2-1-2, 59-2-1-2a, 59-2-1-2b)

Proposal 2	For FGs 59-2-1-3, 59-2-1-3a, and 59-2-1-3b, support the same reporting type as Rel-16 eType-II codebook.

Proposal 3	For FGs 59-2-1-4 and 59-2-1-4a, support the same reporting type as Rel-17 FeType-II codebook.

Proposal 4	For FGs 59-2-1-5, 59-2-1-5a, and 59-2-1-5b, support the same reporting type as Rel-18 eType-II Doppler codebook.

Proposal 5	Support only aperiodic CSI-RS in FG 59-2-1-6.

Proposal 6	For FG 59-2-1-7, the pre-requisites should be one of 59-2-1-1/1a/1b, or one of 59-2-1-1c/1d/1e.

Proposal 7	For FG 59-2-1-7, the reporting type should be ‘per band’.

Proposal 8	Support maxNumberSRS-ResourcePerSet = 4 for Rel-19 3 Tx non-codebook based operation.

Proposal 9	Support the value of 4 for the maximum number of SRS resources in one SRS resource set for Rel-19 3 Tx non-codebook based M-TRP operations.

Proposal 10	Update TS 38.214 to calculate the maximum number of ports supported by the UE in an SRS resource, for scaling of PUSCH transmission power as described in clause 7.1 of [6, TS 38.213], using 3, if [UE feature 59-3-2] is supported, and the maximum number(s) of SRS ports supported by the UE in one SRS resource.

Proposal 11	Include a set of antenna switching capabilities {t1r1, t2r2, t1r2, t3r3, t2r4, t1r4, t2r6, t1r6, t3r6} for Component 1 of FG 59-3-3 and update the name to include ‘for > 4 Rx’.

Proposal 12	Introduce a component in 59-4-4b to indicate UE capability to apply reference timing indicated in PDCCH

Proposal 13	Introduce a FG, e.g., FG 59-4-10, to support indicate PL-RS for PDCCH-order PRACH for inter-cell.

# References

R1-2502977, “Updated RAN1 UE features list for Rel-19 NR after RAN1 #120bis”, RAN1#120bis, Wuhan, China, April 2025

# Appendix

| Index | Feature group | Components | Prerequisite feature groups | Consequence if the feature is not supported by the UE | Type | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 59-1-1 | UE-initiated/event-driven beam management for Event-2 and Mode A | 1. Support of UE-initiated/event-driven beam report based on one event instance 2. Support of Event-2 based measurement and report 3. Support of Mode A UE-initiated/event-driven beam report 4. Maximum number of the configured RS(s) for new beam in the RS resource set 5. Support of current beam measurement by using QCL RS in the indicated TCI state and the corresponding QCL SSB for Scheme-1 and Scheme-2, respectively 6. Support the first PUCCH and second PUSCH from the same PUCCH group |  | UEI/ED beam report is not supported for Event-2 and Mode A | Per band | n/a | n/a | n/a | Component 4 candidate values: {1, 2, …, 64} Note For Component 4 and Component 5, an SSB can be associated with the serving cell PCI or a PCI other than the serving cell PCI |
| 59-1-2 | UE-initiated/event-driven beam management Mode B | 1. Support of Mode B UE-initiated/event-driven beam report | 59-1-1 | UEI/ED beam report Mode-B is not supported | Per band | n/a | n/a | n/a |  |
| 59-1-3 | Triggering event determination via detecting ≥ M event instances for at least one new beam within a time window. | 1. Triggering UEI/ED beam report procedure via detecting ≥ M event instance(s) for at least one new beam within a time window, where M>1 2. Maximum number of running timers | 59-1-1 | Triggering event determination via detecting ≥ M event instances within a time window is not supported | Per band | n/a | n/a | n/a | Component candidate values: {1,2,..16} Note: if the number of new beams that trigger event instances is larger than the number of timers, it is up to the UE which new beams to consider. |
| 59-1-4 | UE-initiated/event-driven beam management for Event-1 based measurement and report | 1. Support of Event-1 based measurement and report that L1-RSRP of the current beam becomes worse than a configured threshold based on one event instance | 59-1-1 | UE-initiated/event-driven beam management for Event-1 based measurement is not supported | Per Band | n/a | n/a | n/a | Note: the event definition Event 1: Quality of the current beam is worse than a certain threshold |
| 59-1-5 | UE-initiated/event-driven beam management for Event-7 based measurement and report | 1. Support of Event-7 that L1-RSRP of at least one new beam becomes a threshold value better than the RS derived from the activated TCI state with the Q-th best quality based on one event instance 3. Support of the RS derived from the activated TCI state with the Q-th best quality measurement by using QCL RS in the activated TCI state with the Q-th best quality and the corresponding QCL SSB for the activated TCI state with the Q-th best quality for Scheme-1 and Scheme-2, respectively | 59-1-1 | UE-initiated/event-driven beam management for Event-7 based measurement is not supported | Per Band | n/a | n/a | n/a |  |
| 59-1-6 | First PUCCH and second PUSCH from different PUCCH groups | Support first PUCCH and second PUSCH from different PUCCH groups | FG 59-1-1 | First PUCCH and second PUSCH from different PUCCH groups is not supported | Per band | n/a | n/a | n/a |  |