3GPP TSG-RAN WG1 Meeting #121	R1-2503979

St Julian’s, Malta, May 19th – 23st, 2025

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.214 | CR | Draft | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on PTRS to PDSCH EPRE ratio |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MIMO_evo_DL_UL-Core |  |  |  |  |  | Date: |  |  | 2025-05-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In Rel18 a new PT-RS to PDSCH EPRE ratio table 4.1-2A is introduced to support enhanced DM-RS type. RAN1 #114 Agreement If enhanced-dmrs-Type_r18 is configured for PDSCH, support the following values of PT-RS EPRE to PDSCH EPRE per layer per RE: Table 4.1-2A: PT-RS EPRE to PDSCH EPRE per layer per RE () epre-Ratio; The number of PDSCH layers with DM-RS associated to the PT-RS port. 1; 2; 3; 4; 5; 6; 7; 8. 0; 0; 3; 4.77; 6; 7; 7.78; 8.45; 9. 1; 0; 0; 0; 0; 0; 0; 0; 0. 2; reserved. 3; reserved. In 38.331, epre-Ratio is an optional RRC parameter configured in IE PTRS-DownlinkConfig. The field decription of epre-Ratio states, “If the field is not provided, the UE applies value 0 (see TS 38.214 [19], clause 4.1)”. However, in clause 4.1 the new table 4.1-2A is not added in the part of description when epre-Ratio is not configured. It makes the UE behaviour undefined when dmrs-TypeEnh is configured, but epre-Ratio is not configured. |
| Summary of change: |  | Add “Table 4.1-2A” to the part of text describes the default behaviour when epre-Ratio is not configured to clarify the the UE behaviour and to align with 38.331. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | If network has implemented the CR, i.e.,according to 38.331 and doesn’t configure the epre-Ratio, however the UE has not implemented accordingly, the UE may not follow the intended behaviour. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | N | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | N | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | N | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## 4.1	Power allocation for downlink

## ---------------<Unchanged text omitted>-----------------------------

When the UE is scheduled with one or two PT-RS ports associated with the PDSCH,

-	if the UE is configured with the higher layer parameter epre-Ratio, the ratio of PT-RS EPRE to PDSCH EPRE per layer per RE for each PT-RS port () is given by Table 4.1-2 or Table 4.1-2A according to the epre-Ratio, the PT-RS scaling factor specified in clause 7.4.1.2.2 of [4, TS 38.211] is given by.

-	otherwise, the UE shall assume epre-Ratio is set to state '0' in Table 4.1-2 or Table 4.1-2A if not configured.

Table 4.1-2: PT-RS EPRE to PDSCH EPRE per layer per RE (), if dmrs-TypeEnh is not configured in DMRS-DownlinkConfig

| epre-Ratio | The number of PDSCH layers with DM-RS associated to the PT-RS port |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 1 | 2 | 3 | 4 | 5 | 6 |
| 0 | 0 | 3 | 4.77 | 6 | 7 | 7.78 |
| 1 | 0 | 0 | 0 | 0 | 0 | 0 |
| 2 | reserved |  |  |  |  |  |
| 3 | reserved |  |  |  |  |  |

Table 4.1-2A: PT-RS EPRE to PDSCH EPRE per layer per RE (), if dmrs-TypeEnh is configured in DMRS-DownlinkConfig

| epre-Ratio | The number of PDSCH layers with DM-RS associated to the PT-RS port |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| 0 | 0 | 3 | 4.77 | 6 | 7 | 7.78 | 8.45 | 9 |
| 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 2 | reserved |  |  |  |  |  |  |  |
| 3 | reserved |  |  |  |  |  |  |  |

For link recovery, as described in clause 6 of [6, TS 38.213] the ratio of the PDCCH EPRE to NZP CSI-RS EPRE is assumed as 0 dB.