3GPP TSG RAN WG1 #121		R1-2504010

St Julian’s, Malta, May 19th – 23rd, 2025

Source:	Panasonic

Title:	Discussion on SBFD random access operation

Agenda Item: 		9.3.2

Document for:	Discussion

Introduction

A work item on evolution of NR duplex operation has been approved for Rel.19 [1]. This document provides our view on SBFD random access operation.

Discussion

UE behavior with legacy-RO

In RAN1#120bis, the following agreement was reached about UE behavior when UE transmits PRACH in legacy-RO.

Agreement For CBRA, if a SBFD-aware UE transmits Msg1 in legacy-RO, the UE follows the legacy behavior during the random access procedure. - FFS: Whether specification change is necessary

Regarding FFS in the agreement, we think that clarification is necessary to distinguish between CBRA case and CFRA case. For example, if a SBFD-aware UE transmits PRACH in legacy-RO for CBRA, and if gNB schedules Msg3 in SBFD symbols configured as flexible by tdd-UL-DL-ConfigurationCommon, then SBFD-aware UE should behave as if Msg3 is scheduled in non-SBFD symbol because gNB does not know whether the UE is SBFD-aware UE or non-SBFD aware UE for CBRA case. On the other hand, for CFRA case, if gNB schedules Msg3 in SBFD symbols, and if separate Msg3 configurations are configured, SBFD-aware UE should use Msg3 configurations for SBFD symbol, which is more useful for adjustment to SBFD symbol (e.g., CLI reduction). The specification needs to be clarified so that SBFD-aware UE has different behavior between CFRA and CBRA cases.

Proposal 1: For CFRA, if a SBFD-aware UE transmits Msg1 in legacy-RO, the UE follows SBFD behavior during the random access procedure (e.g., the UE follows separate configurations for SBFD symbol).

Msg3 enhancement

Separate msg3-DeltaPreamble

In RAN1#120bis, the following agreement was reached on Msg3 PUSCH transmission power for RACH configuration Option 2.

Agreement For determination of the Msg3 PUSCH transmission power for RACH configuration Option 2: - preambleReceivedTargetPower configured for legacy-RO is used if Msg3 PUSCH is transmitted in non-SBFD symbols; - preambleReceivedTargetPower configured for additional-RO is used if Msg3 PUSCH is transmitted in SBFD symbols; - FFS whether/how to support separate msg3-DeltaPreamble for non-SBFD symbols and SBFD-symbols.

As described in FFS in the agreement, whether to support separate msg3-DeltaPreamble needs to be clarified. If separate msg3-DeltaPreamble is supported, Msg3 transmission power can be adjusted depending on symbol types or RO types. This would be useful to reduce UE-to-UE CLI. In addition, it would be natural to support separate msg3-DeltaPreamble for RACH configuration Option 2 (i.e., RACH configuration Option 2 supports separate configurations for SBFD and non-SBFD symbols).

Proposal 2: For determination of the Msg3 PUSCH transmission power for RACH configuration Option 2, support separate msg3-DeltaPreamble.

If separate msg3-DeltaPreamble is supported, how to apply msg3-DeltaPreamble needs to be clarified. In RAN1#120bis, the following alternatives were discussed [2].

- Additional msg3-DeltaPreamble can be configured optionally. - Down-select from the following two alternatives: - Alt-1: legacy msg3-DeltaPreamble is used if Msg3 PUSCH is transmitted in non-SBFD symbols; additional msg3-DeltaPreamble is used if Msg3 PUSCH is transmitted in SBFD symbols. - Alt-2: legacy msg3-DeltaPreamble is used if legacy-RO is used for latest Msg1 transmission; additional msg3-DeltaPreamble is used if additional-RO is used for latest Msg1 transmission.

For Alt-1, Msg3 transmission power is adjusted depending on symbol type of Msg3. For Alt-2, Msg3 transmission power is adjusted depending on RO type of PRACH. We think that Alt-1 is more useful because Msg3 transmission power can be adjusted depending on symbol type. Transmission power adjustment depending on RO type can be already achieved by separate preambleReceivedTargetPower.

Proposal 3: For separate msg3-DeltaPreamble, legacy msg3-DeltaPreamble is used if Msg3 PUSCH is transmitted in non-SBFD symbols; additional msg3-DeltaPreamble is used if Msg3 PUSCH is transmitted in SBFD symbols.

Power ramping

In RAN1#120bis, the following agreement was reached on Msg3 PUSCH transmission power for RACH configuration Option 2.

Agreement - For determination of Msg3 PUSCH transmission power, ∆Prampup_requested,b,f,c is provided by MAC layer and is the amount of power ramping applied to the latest Random Access Preamble transmission, regardless of the used RO types and symbol type of Msg3 PUSCH. - FFS: For RACH configuration Option 2, interpretation of ∆Prampup_requested,b,f,c when separate PREAMBLE_POWER_RAMPING_STEP configurations for different RO types when UE switches from additional-ROs to legacy-ROs. - This does not imply that separate PREAMBLE_POWER_RAMPING_STEP configurations are supported.

Regarding FFS in the agreement, whether to support separate PREAMBLE_POWER_RAMPING_STEP configurations for legacy-ROs and additional-ROs needs to be clarified for RACH configuration Option 2. We think that separate PREAMBLE_POWER_RAMPING_STEP configurations can be useful in some cases. For example, in order to reduce UE-to-UE CLI, the transmission power of the first PRACH transmission in additional-RO can be configured to be lower than that of legacy-RO. In this case, it can be useful that PREAMBLE_POWER_RAMPING_STEP for additional-RO is larger than that of legacy-RO in order to increase PRACH transmission power for additional-RO. In addition, it would be natural to support separate PREAMBLE_POWER_RAMPING_STEP configurations for RACH configuration Option 2.

Proposal 4: For RACH configuration Option 2, support separate PREAMBLE_POWER_RAMPING_STEP configurations.

If separate PREAMBLE_POWER_RAMPING_STEP configurations are supported, the interpretation of ∆Prampup_requested,b,f,c needs to be clarified. The following alternatives can be considered.

- Alt 1: ∆Prampup_requested,b,f,c is the amount of power ramping applied to the latest PRACH transmission.

- Alt 2: ∆Prampup_requested,b,f,c is the amount of power ramping calculated based on PREAMBLE_POWER_RAMPING_STEP depending on symbol type.

For Alt 1, whether Msg3 is transmitted in SBFD symbols or non-SBFD symbols is not taken into account. Alt 1 is aligned with the current specification. In the current specification, the amount of power ramping of the latest PRACH transmission is just used for Msg3 according to the following description in 38.321 [3].

38.321[3] section 5.4.1 2> if the Random Access Response reception is considered successful: 3> if the Random Access Response includes a MAC subPDU with RAPID only: 4> consider this Random Access procedure successfully completed; 4> indicate the reception of an acknowledgement for SI request to upper layers. 3> else: 4> apply the following actions for the Serving Cell where the Random Access Preamble was transmitted: 5> process the received Timing Advance Command (see clause 5.2); 5> indicate the preambleReceivedTargetPower and the amount of power ramping applied to the latest Random Access Preamble transmission to lower layers (i.e. (PREAMBLE_POWER_RAMPING_COUNTER – 1) × PREAMBLE_POWER_RAMPING_STEP); 5> if the Random Access procedure for an SCell is performed on uplink carrier where pusch-Config is not configured: 6> ignore the received UL grant. 5> else: 6> process the received UL grant value and indicate it to the lower layers.

For Alt 2, the amount of power ramping can be different based on the symbol type of Msg3. If Msg3 is scheduled in non-SBFD symbols, the amount of power ramping is calculated based on PREAMBLE_POWER_RAMPING_STEP for non-SBFD symbols. If Msg3 is scheduled in SBFD symbol, the amount of power ramping is calculated based on PREAMBLE_POWER_RAMPING_STEP for SBFD symbol. Different amounts of power ramping can be used to adjust Msg3 transmission power depending on the symbol type.

We think that Alt 1 is simple and sufficient. In order to adjust Msg3 transmission power depending on symbol type, other parameters can be used for RACH configuration Option 2 (e.g., msg3-DeltaPreamble).

Proposal 5: For RACH configuration Option 2, ∆Prampup_requested,b,f,c is the amount of power ramping applied to PRACH transmission.

Conclusion

In this contribution, we made the following proposals.

Proposal 1: For CFRA, if a SBFD-aware UE transmits Msg1 in legacy-RO, the UE follows SBFD behavior during the random access procedure (e.g., the UE follows separate configurations for SBFD symbol).

Proposal 2: For determination of the Msg3 PUSCH transmission power for RACH configuration Option 2, support separate msg3-DeltaPreamble.

Proposal 3: For separate msg3-DeltaPreamble, legacy msg3-DeltaPreamble is used if Msg3 PUSCH is transmitted in non-SBFD symbols; additional msg3-DeltaPreamble is used if Msg3 PUSCH is transmitted in SBFD symbols.

Proposal 4: For RACH configuration Option 2, support separate PREAMBLE_POWER_RAMPING_STEP configurations.

Proposal 5: For RACH configuration Option 2, ∆Prampup_requested,b,f,c is the amount of power ramping applied to PRACH transmission.

- Reference

- RP-241614, "WID revision: Evolution of NR duplex operation: Sub-band full duplex (SBFD)", Huawei

- R1-2503017, "Summary#2 on SBFD random access operation", Moderator (CMCC)

- 3GPP TS 38.321 V18.4.0 (2024-12)