3GPP TSG RAN WG1 #121			R1-2504045

St Julian’s, Malta, May 19th – 23th, 2025

Agenda item:		9.3.2

Source:	China Telecom

Title:	Discussion on SBFD random access operation

Document for:		Discussion

Introduction

In this contribution, we discuss SBFD random access operation.

Discussion

Power control

In RAN1 #120 meeting [1], the following conclusion was drawn with respect to power control parameter for RACH configuration Option 1.

Conclusion There is no RAN1 consensus on the following proposal: For RACH configuration Option 1, for support of separate power control parameters for PRACH transmission in additional-ROs and legacy-ROs, - also support separate powerRampingStep and powerRampingStepHighPriority for additional-ROs.

For RACH configuration option 2, although we don’t explicitly discuss whether to support separate power control parameters for legacy RO and additional RO, we think it is naturally supported. The reasons are listed as follows:

The definition of RACH configuration option 2 is: Use two separate RACH configurations, including one legacy RACH configuration and one additional RACH configuration, as agreed in RAN1 #116bis meeting. We can find the detailed RACH configuration in TS 38.331 as follows:

While RACH-ConfigGeneric and ra-Prioritization are configured in RACH-ConfigCommon, thus if two separate RACH configuration are utilized, there will be naturally two separate RACH-ConfigGeneric and ra-Prioritization configurations. Therefore, all the parameters in RACH-ConfigGeneric and ra-Prioritization are separately configured, including preambleReceivedTargetPower, powerRampingStep, powerRampingStepHighPriority.

We don’t think any further discussion or conclusion is needed.

Proposal 1: For RACH configuration option 2, since two separate RACH configurations are used, all the parameters in the RACH configurations are naturally separately configured. No further discussion is needed.

In RAN2 #129bis meeting, the following agreement was achieved:

RO-Type change procedure on RO type selection from legacy RO to additional RO in SBFD symbols is supported when the number of PRACH transmission attempts exceed a threshold. If fallback from legacy RO to additional RO occurs, no further fallback to legacy RO is supported.

Switching from legacy RO to additional RO is supported. Thus, we need to discuss the related power ramping mechanism.

As in RAN1 #120bis meeting, the following agreement/conclusion was achieved with respect to power ramping for switching from additional-ROs to legacy-ROs:

Agreement For RACH configuration Option 1 and Option 2, when a SBFD-aware UE switches from additional-ROs to legacy-ROs in PRACH transmission re-attempt in one RACH procedure, support Alt-1: Increase the power ramping counter. Conclusion For RACH configuration Option 2, when a SBFD-aware UE switches from additional-ROs to legacy-ROs in PRACH transmission re-attempt in one RACH procedure, there is no RAN1 consensus to support a power offset to compensate the power ramping difference.

Thus, we think the above agreement/conclusion can be extended to the case when switching from legacy-ROs to additional-ROs.

Proposal 2: Update the following Agreement/Conclusion achieved in RAN1 #120bis as follows (in red):

Agreement

For RACH configuration Option 1 and Option 2, when a SBFD-aware UE switches from additional-ROs to legacy-ROs or from legacy-ROs to additional-ROs in PRACH transmission re-attempt in one RACH procedure, support Alt-1: Increase the power ramping counter.

Conclusion

For RACH configuration Option 2, when a SBFD-aware UE switches from additional-ROs to legacy-ROs or from legacy-ROs to additional-ROs in PRACH transmission re-attempt in one RACH procedure, there is no RAN1 consensus to support a power offset to compensate the power ramping difference.

Conclusions

In this contribution, we discuss SBFD random access operation and have following proposals:

Proposal 1: For RACH configuration option 2, since two separate RACH configurations are used, all the parameters in the RACH configurations are naturally separately configured. No further discussion is needed.

Proposal 2: Update the following Agreement/Conclusion achieved in RAN1 #120bis as follows (in red):

Agreement

For RACH configuration Option 1 and Option 2, when a SBFD-aware UE switches from additional-ROs to legacy-ROs or from legacy-ROs to additional-ROs in PRACH transmission re-attempt in one RACH procedure, support Alt-1: Increase the power ramping counter.

Conclusion

For RACH configuration Option 2, when a SBFD-aware UE switches from additional-ROs to legacy-ROs or from legacy-ROs to additional-ROs in PRACH transmission re-attempt in one RACH procedure, there is no RAN1 consensus to support a power offset to compensate the power ramping difference.

References

- 3GPP RAN1 Chairman’s Notes, RAN1 meeting #120, Athens, Greece, February 17th – 21st, 2025.