3GPP TSG RAN WG1 #121			R1-2504051

St Julian’s, Malta, May 19th – 23th, 2025

Agenda Item:	9.5.2

Source:	China Telecom

Title:	Discussion on on-demand SIB1 for idle/inactive mode UEs

Document for:	Discussion and decision

- Introduction

The new Rel-19 WI of network energy saving NR was approved in RAN#102 and revised in RAN#105 [1], with the following objectives included.

- Specify support for on-demand SIB1 for UEs in idle/inactive mode [RAN1/2/3] - Specify procedures and signalling method(s) for Case 2 [RAN1/2] - Case 2: UE obtains UL WUS configuration from Cell A, UE transmits UL WUS on NES Cell, UE receives on-demand SIB1 from NES Cell - Triggering method by UL WUS using PRACH - Specify inter NG-RAN node signalling at least for the configuration of UL WUS [RAN3] - Note 1: No modification of SSB will be discussed under this objective - Note 2: - UL WUS: Uplink wake-up signal - Cell A: A cell that is periodically transmitting at least its own SIB1 - NES Cell: A cell that may transmit SIB1 transmission in response to UL WUS from a UE - Note 3: - RAN1 strives to minimize impact to legacy UE - RAN1 specification impact to support this feature should be minimized

In the revised WID, on-demand SIB1 is limited to the Case 2, where Cell A is used for providing the UL WUS configuration, UE transmits UL WUS on NES Cell, the on-demand SIB1 transmission and the following RA are performed on NES Cell, the procedure of which can be shown in Fig 1.

Fig 1 illustration of mechanism of on-demand SIB1

In this contribution, we focus on the details of on-demand SIB1 in such case, providing some observations and considerations on such mechanism.

- Considerations on on-demand SIB1

In last meeting, the following agreements were reached in last meeting[2].

Agreement The following agreement is updated as follows: The UE assumes that, in the OD-SIB1 window, PDCCH for an OD-SIB1 message is transmitted in PDCCH monitoring occasions corresponding only to at least the SSB associated with the transmitted PRACH for UL-WUS if this is indicated via UL WUS configuration by a 1-bit indication - If the 1-bit indication is included in UL WUS config and indicate as TRUE, the UE assumes that, in the OD-SIB1 window, PDCCH for an OD-SIB1 message is transmitted in PDCCH monitoring occasions corresponding only to the SSB associated with the PRACH for UL-WUS. - If the 1-bit indication is not included in UL WUS config or the 1-bit indication is FALSE, the UE assumes the same as legacy, i.e., in the OD-SIB1 window, PDCCH for an OD-SIB1 message is transmitted in at least one PDCCH monitoring occasion corresponding to each transmitted SSB. - Further study possible down selection from the following options to indicate additional information related to SSBs associated with PDCCH monitoring occasions - Alt1: The information is provided in UL-WUS config - Alt2: The information is provided in RAR in response to UL-WUS transmission - Alt3: No additional information is provided Agreement RAN1 to discuss the value range for od-sib1-duration. For example, - Option 1: Use the value-range for si-windowLength (from 5 slots to 5120 slot) as starting point. - Option 2: The duration of the time window can be a multiple of 160ms (i.e. legacy SIB1 periodicity). - Option 3: The duration of the time window can be a multiple of the on-demand SIB1 repetition periodicity (if supported) - Other options are not precluded.

For the indication of SSB associated relation with PDCCH monitoring occasions, according to 3GPP TS 38.304[3], shown as follows, the PDCCH monitoring occasion is associated the corresponding SSB according to the index, which is sufficient for UE to monitor the OD-SIB1 message transmitted in PDCCH monitoring occasions. We don’t see the need to indicate any other information related to SSBs associated with PDCCH monitoring occasions.

7 Paging 7.1 Discontinuous Reception for paging <unrelated parts omitted> When SearchSpaceId other than 0 is configured for pagingSearchSpace, the UE monitors the (i_s + 1)th PO. A PO is a set of 'S*X ' consecutive PDCCH monitoring occasions where 'S' is the number of actual transmitted SSBs determined according to ssb-PositionsInBurst in SIB1 and X is the nrofPDCCH-MonitoringOccasionPerSSB-InPO if configured or is equal to 1 otherwise. The [x*S+K]th PDCCH monitoring occasion for paging in the PO corresponds to the Kth transmitted SSB, where x=0,1,…,X-1, K=1,2,…,S. The PDCCH monitoring occasions for paging which do not overlap with UL symbols (determined according to tdd-UL-DL-ConfigurationCommon) are sequentially numbered from zero starting from the first PDCCH monitoring occasion for paging in the PF. When firstPDCCH-MonitoringOccasionOfPO is present, the starting PDCCH monitoring occasion number of (i_s + 1)th PO is the (i_s + 1)th value of the firstPDCCH-MonitoringOccasionOfPO parameter; otherwise, it is equal to i_s * S*X. If X > 1, when the UE detects a PDCCH transmission addressed to P-RNTI within its PO, the UE is not required to monitor the subsequent PDCCH monitoring occasions for this PO. NOTE 1: A PO associated with a PF may start in the PF or after the PF. NOTE 2: The PDCCH monitoring occasions for a PO can span multiple radio frames. When SearchSpaceId other than 0 is configured for paging-SearchSpace the PDCCH monitoring occasions for a PO can span multiple periods of the paging search space.

Proposal 1: Not support to provide any additional information related to SSBs associated with PDCCH monitoring occasions.

Agreement RAN1 to discuss the value range for od-sib1-duration. For example, - Option 1: Use the value-range for si-windowLength (from 5 slots to 5120 slot) as starting point. - Option 2: The duration of the time window can be a multiple of 160ms (i.e. legacy SIB1 periodicity). - Option 3: The duration of the time window can be a multiple of the on-demand SIB1 repetition periodicity (if supported) - Other options are not precluded.

For the value range for od-sib1-duration, we don’t think there is a need to restrain the value be a multiple of legacy SIB periodicity. As illustrated in the following figure, even though the periodicity of SIB1 is 160ms, with a length of OD-SIB window smaller than 160ms, UE can still be able to detect the SIB1. The exact length of OD-SIB monitoring window can be totally up to gNB’s implement on when to transmit the OD-SIB1 refer to the starting time of OD-SIB1 window.

Proposal 2: Support Option 1, i.e., use the value range for si-windowLength (from 5 slots to 5120slots) as a starting point for the value range of od-sib1-duration.

Besides, the case can happen when OD-SIB1 is transmitted during the RAR windows, while the OD-SIB1 window is still not started. In such case, UE may have the capability to monitor the OD-SIB1 since the related configuration of OD-SIB1 has already transmitted to UE. Thus, we think it should be up to UE’s implement to decide whether to monitor OD-SIB1 before the starting time of OD-SIB1 window.

Proposal 3: It should be up to UE’s implement to monitor OD-SIB1 during the RAR window before the OD-SIB1 window starts.

- Conclusion

In this paper, we discuss on-demand SIB1operation and have the following observations and proposals.

Proposal 1: Proposal 1: Not support to provide any additional information related to SSBs associated with PDCCH monitoring occasions.

Proposal 2: Support Option 1, i.e., use the value range for si-windowLength (from 5 slots to 5120slots) as a starting point for the value range of od-sib1-duration.

Proposal 3: It should be up to UE’s implement to monitor OD-SIB1 during the RAR window before the OD-SIB1 window starts.

- References

3GPP RP-242354, “Revised WID for Enhancements of network energy savings for NR”, Ericsson, RAN#105, Melbourne, Sep 9-12, 2024.

“Chair notes RAN1 #120-bis”, April, 2025.

3GPP TS 38.304, “User Equipment (UE) procedures in Idle mode and RRC Inactive state (Release 18)”