3GPP TSG RAN WG1 #121									R1-2504064

St Julian’s, Malta.  19-23 May 2025

Agenda Item 	:	9.4.4

Source 	:	Sony

Title 	:	Multiple access and timing relationships for Ambient IoT

Document for 	:	Discussion

# Introduction

In RAN1#120 Athens, the following agreements were made relating to multiplexing, multiple access and timing relationships for Ambient IoT:

Agreement Support FDMA with Y≥1 D2R transmissions for Msg.1 from multiple devices in response to a R2D transmission triggering random access. - The maximum value of Y >1 is determined in AI 9.4.2, based on feasible values of small frequency shift to be decided in AI 9.4.2. - Signalling details should be discussed and determined in AI 9.4.4. Agreement Support following two options for Msg2 transmission in response to multiple Msg1 transmissions, which are initiated by a R2D transmission triggering random access. - Option 1: A PRDCH for Msg2 transmission corresponds to a A-IoT Msg1 received from one device - Option 2: A PRDCH for Msg2 transmission corresponds to multiple A-IoT Msg1 received from different devices - FFS the maximum number of Msg1 responses that can be carried within a PRDCH for Msg2. R1-2501356 FL summary #2 on other aspects for Rel-19 Ambient IoT Moderator (vivo) Agreement Support FDMA of D2R transmissions for Msg3 from multiple devices in response to a PRDCH for Msg2 transmission corresponding to multiple Msg1 during access procedure. R1-2501357 FL summary #3 on other aspects for Rel-19 Ambient IoT Moderator (vivo) Agreement Support the following option(s) for frequency domain resource for Msg3 transmission: - Option 2: The frequency domain resource for Msg3 can be determined based on explicit indication in the PRDCH for Msg2 transmission for one or multiples devices. - FFS: support option 1 as a default if PRDCH does not provide the indication or based on a rule - Option 1: the frequency domain resource for Msg3 reuses the frequency domain resource for Msg1 for the same device Agreement The following is supported for indicating the D2R chip duration: - The D2R chip duration is indicated in R2D control information from predefined a set of D2R chip duration values Agreement The payload size (i.e. TBS-like) for PDRCH transmission with variable size is explicitly indicated in the corresponding R2D control information.

The following agreements were made in RAN1#120bis Wuhan:

R1-2502498 FL summary #1 on other aspects for Rel-19 Ambient IoT Moderator (vivo) Agreement For scheduling D2R transmission, any scheduling information related to resource allocation that needs to be signaled is indicated by higher-layer signaling via the corresponding PRDCH. R1-2502499 FL summary #2 on other aspects for Rel-19 Ambient IoT Moderator (vivo) Agreement For Msg 1 transmission determined by one R2D transmission triggering random access, support X=1 and X=2 time domain resource(s) for D2R transmission(s) for Msg1 only, where each D2R transmission for Msg1 occurs in one time domain resource of the X time domain resource(s) in Rel-19. - All devices support the above - Note: the impact of specification support (at least including signalling overhead) for X=2 to a reader supporting only X=1 should be minimized Only support X=1 time domain resource for D2R transmission for Msg3 in response to a PRDCH for Msg2 transmission. R1-2502500 FL summary #3 on other aspects for Rel-19 Ambient IoT Moderator (vivo) Agreement A device is not required to monitor a corresponding R2D transmission earlier than TD2R_min after the end of a D2R transmission from the device. Conclusion For any timing requirement that needs to be specified from device perspective by RAN1, the timing(s) do not include the impacts of SFO from the RAN1 perspective. - Whether 3GPP needs to specify some requirement on device’s SFO is up to RAN4 discussion. - Note: RAN1 will not specify TR2D_min or TR2D_max in Rel-19 - Note: SFO may still be considered when discussing time-domain resources for Msg1 when X=2 Agreement Use 1 bit to indicate the value of X (X=1 or X=2) time domain resource(s) for Msg1 transmission(s). Agreement For X=1 or X=2, from device perspective, the starting time for the first Msg1 time domain resource is Toffset1 after the end of the corresponding R2D transmission triggering random access. - FFS Toffset1 is predefined in the spec or indicated implicitly or explicitly by the R2D transmission triggering random access. - FFS detailed value(s) of Toffset1, considering at least the device processing time including FEC impact - FFS: how to define the end of the R2D transmission in 9.4.1 Working assumption For X=2, from device perspective, for determination of the starting time for the second Msg1 time domain resource: - Derived based on the starting time of the first Msg1 time domain resource plus Toffset2 - FFS Toffset2 is predefined in the spec or derived by a rule or indicated implicitly or explicitly by the R2D transmission triggering random access. Agreement At least for CBRA, for FDMA of multiple Msg1 transmissions in response to a R2D transmission triggering random access, support only the same data rate for the FDMed Msg1 transmissions. Working assumption For indicating the payload size (i.e. TBS-like) for PDRCH transmission with variable size: - 7 bits for byte-level D2R payload size indication

In this document, we consider the relationship between messages in the initial access procedure of Ambient IoT:

# Relationship between messages in the initial access procedure

TR 38.769 discussed how to monitor for Msg2 according to the text below:

To determine the time duration for monitoring Msg2 in response to multiple Msg1 transmissions initiated by a R2D transmission triggering random access, the following options have been identified: Option 1: Predefined Option 2: Indicated in the R2D transmission triggering random access Note: Option 1 and Option 2 may not be mutually exclusive. For a device to determine the reference time for the starting time for monitoring Msg2 in response to multiple Msg1 transmissions initiated by a R2D transmission triggering random access, the following options have been identified: Option 1: End of time domain resource where the device transmitted the Msg1 Option 2: End of the last of X time domain resource(s) determined for Msg1 transmission(s) Option 3: End of the R2D transmission triggering random access Option 4: End of a R2D transmission carrying Msg2 Note: The reference time is used to determine the starting time for Msg2 monitoring, it does not mean that the starting time always equals the reference time.

From TR 38.769 section 6.1.4 [2] it is also indicated that “at least slotted-ALOHA based access and FDMA” are studied. Given this, the device randomly selects resource in PDRCH for Msg1 by using slotted-ALOHA. The following agreement was made in RAN1#120 Athens:

Agreement Support following two options for Msg2 transmission in response to multiple Msg1 transmissions, which are initiated by a R2D transmission triggering random access. - Option 1: A PRDCH for Msg2 transmission corresponds to a A-IoT Msg1 received from one device - Option 2: A PRDCH for Msg2 transmission corresponds to multiple A-IoT Msg1 received from different devices - FFS the maximum number of Msg1 responses that can be carried within a PRDCH for Msg2.

We observe that the method of monitoring Msg2 for a device type 1 AIoT device has to be simple or at least work with a limited amount of related control information.

Based on above agreement, Msg2 can be transmitted in response to only one device or multiple devices. Selecting the same Msg2 monitoring window for both options may lead to extra delay for Msg2 transmissions corresponding to a Msg1 received from one device. It also may result in extra power consumption for these AIoT devices as Msg2 may become longer if Msg2 transmissions correspond to Msg1 from multiple devices. To enable a simple monitoring technique, schemes that allow for limited associated control information are necessary. In our view it is beneficial to include information such as a bit indicator in Msg0/paging message to indicate whether the Msg2 transmission is in response to one device or multiple devices.

Observation 1 – Selecting the same Msg2 monitoring window for both options may lead to extra delay or unnecessary power consumption at the AIoT Device

Proposal 1 – Include information such as a bit indicator in Msg0/paging message to indicate whether the Msg2 transmission is in response to one device or multiple devices.

We can then select different reference times, using the alternatives from SI [2], and monitoring time durations, based on Msg2 length, suitable for the operation of the two options and associate those values to the bit indicator carried by Msg0. Based on this, the device can determine the Msg2 monitoring window.

Proposal 2 –Associate different Msg2 monitoring windows, i.e., reference time and time duration, to Msg2 transmission corresponding to a Msg1 received from one device or from multiple devices.

Additionally, the D2R Msg1 can be both TDMA/FDMA multiplexed while R2D can be only TDMA multiplexed. It is, therefore, necessary to include an additional association of Msg2 transmission to FDMA multiplexed Msg1 transmission, this is particularly if Msg2 is transmitted in response to multiple Msg1 devices.  This further association to FDMA based Msg1 can either be implicitly indicated by the Msg1 or at least semi-persistently configured/associated between Msg1 and Msg2. Two examples are shown in Figure 1.  The left part shows a case where responses from multiple Msg1 from the same TDMA time domain resource (X=2 time domain resources are shown in the figure) are grouped into the same Msg2 and the right part where every Msg1 has a separate Msg2.

Figure 1 - Scheduling of Msg2 in association to Msg1

Proposal 3: Support association between the selected Msg1 time and frequency resources to its Msg2 time resources.

In the RAN1#120 Athens meeting, the following agreement was made:

Agreement Support the following option(s) for frequency domain resource for Msg3 transmission: - Option 2: The frequency domain resource for Msg3 can be determined based on explicit indication in the PRDCH for Msg2 transmission for one or multiples devices. - FFS: support option 1 as a default if PRDCH does not provide the indication or based on a rule - Option 1: the frequency domain resource for Msg3 reuses the frequency domain resource for Msg1 for the same device

In RAN1#120-bis Wuhan meeting, the following was further agreed.

Agreement For Msg 1 transmission determined by one R2D transmission triggering random access, support X=1 and X=2 time domain resource(s) for D2R transmission(s) for Msg1 only, where each D2R transmission for Msg1 occurs in one time domain resource of the X time domain resource(s) in Rel-19. - All devices support the above - Note: the impact of specification support (at least including signalling overhead) for X=2 to a reader supporting only X=1 should be minimized Only support X=1 time domain resource for D2R transmission for Msg3 in response to a PRDCH for Msg2 transmission.

With regards to the FFS we believe it is important to allocate resources with as small amount of control information as possible. Therefore, we support to make option 1 as default when X=1.

Proposal 4: support option 1 as a default if PRDCH does not provide the indication or based on a rule

- Option 1: the frequency domain resource for Msg3 reuses the frequency domain resource for Msg1 for the same device when X=1.

# Conclusion

This document has considered the relationship between messages in the initial access procedure of Ambient IoT. The following proposals are made:

Observation 1 – Selecting the same Msg2 monitoring window for both options may lead to extra delay or unnecessary power consumption at the AIoT Device

Proposal 1 – Include information such as a bit indicator in Msg0/paging message to indicate whether the Msg2 transmission is in response to one device or multiple devices

Proposal 2 –Associate different Msg2 monitoring windows, i.e., reference time and time duration, to Msg2 transmission corresponding to a Msg1 received from one device or from multiple devices.

Proposal 3: Support association between the selected Msg1 time and frequency resources to its Msg2 time resources.

Proposal 4: support option 1 as a default if PRDCH does not provide the indication or based on a rule

- Option 1: the frequency domain resource for Msg3 reuses the frequency domain resource for Msg1 for the same device when X=1.

References

[1]	RP-243326. “Solutions for Ambient IoT (Internet of Things) in NR”. RANP#106. Madrid, Spain. December 2024.

[2]	TR38.769. “Study on solutions for Ambient IoT (Internet of Things) in NR”.