3GPP TSG RAN WG1 #121	R1-2504151

St Julian’s, Malta, May 19th – 23th, 2025

Agenda item:	9.11.3

Source: 	ETRI

Title:	Discussion on NR-NTN uplink capacity/throughput enhancement

Document for:	Discussion/Decision

Introduction

This contribution presents ETRI’s views on NR-NTN uplink capacity/throughput enhancement.

Discussion

Table 1. RAN1 agreements on OCC for PUSCH [3].

Agreement RAN1 assumes that the UE is required to maintain phase continuity and power consistency for the duration of one OCC group with PUSCH. - FFS: under which conditions the above applies, e.g. under the same conditions that phase continuity applies for DMRS bundling Agreement Send LS to RAN4 on requirements for the phase continuity and power consistency: RAN1 has agreed the following: - RAN1 assumes that the UE is required to maintain phase continuity and power consistency for the duration of one OCC group with PUSCH. - FFS: under which conditions the above applies, e.g. under the same conditions that phase continuity applies for DMRS bundling RAN1 ask RAN4 whether the same phase continuity requirements as for DMRS bundling can be applied for OCC length 2 and/or OCC length 4 under the same conditions as for DMRS bundling, or if new requirements are needed. Agreement The draft LS in R1-2503070 is endorsed. Final LS is in R1-2503071. Agreement OCC length and OCC sequence for OCC with CG PUSCH Type 1 is configured by RRC higher-layers. - Up to RAN2 whether to signal this with one or two RRC parameters - Note: OCC lengths and sequences to be provided in the table of RRC parameters to be prepared by the WI rapporteur Agreement For OCC with CG-PUSCH, the RV sequence applied across OCC groups is RRC configured among the RV sequences defined for legacy CG PUSCH, i.e., [0,2,3,1], [0,3,0,3] or [0,0,0,0]. - Note: no new RRC parameter is needed for the above - FFS: if OCC group dropping is later agreed, how to count RVs may need to be discussed for that case Agreement For resolving the overlapping PUSCH repetitions with inter-slot OCC and PUCCH with/without repetitions, the legacy timeline conditions for UCI multiplexing or prioritization for dropping PUSCH [/ PUCCH] applies according with the following update: - “the first PUSCH repetition of an OCC group” is used instead of “PUSCH repetition” for the legacy timeline condition, where the legacy PUSCH repetition that overlaps with a PUCCH belongs to the OCC group Note: how to capture the above agreement is up to the spec editor. Agreement For the OCC length applied for OCC DG PUSCH and CG PUSCH Type 2, consider the following options: - Option 1: Configured by RRC higher-layer parameter. - Note: this does not preclude jointly configuring OCC sequence and OCC length with the same RRC configuration - Option 2: Max value is configured by RRC higher-layer parameter, and the applied value is implicitly determined from the configured repetition number. - Option 3: Candidates values are configured by RRC higher-layer parameter, and the applied value is indicated by scheduling DCI for DG PUSCH or by activation DCI for CG PUSCH Type 2. - Note: this does not preclude jointly configuring OCC sequence and OCC length with the same RRC configuration - Option 4: Single value is indicated by scheduling DCI for DG PUSCH and by activation DCI for CG PUSCH Type 2 (no RRC configuration of candidate values). - Option 5: Max value is configured by RRC higher-layer parameter, and the applied value is indicated by scheduling DCI for DG PUSCH and by activation DCI for CG PUSCH Type 2. Note: it is not precluded to select a different option for DG PUSCH and CG PUSCH Type 2. Agreement If PUCCH without repetitions overlaps with inter-slot OCC with any PUSCH repetitions in an OCC group with a same priority index for PUCCH/PUSCH, the legacy conditions and rules for UCI multiplexing or prioritization for dropping applies with the following updates: - If the UCI is multiplexed on the PUSCH repetition according to legacy rules and the updated timeline conditions for UCI multiplexing are satisfied, UCI is multiplexed on all PUSCH repetitions without A-CSI reports within an OCC group with inter-slot OCC overlaps with the PUCCH. (Option 3-a) - FFS: PUSCH repetition with A-CSI reports - If the PUSCH repetition is dropped according to legacy rules and the updated timeline conditions for PUSCH dropping are satisfied, UCI is transmitted on PUCCH and all PUSCH repetitions within the OCC group that overlaps with the PUCCH are dropped (Option 2) - FFS: if PUCCH is overlap with PUSCH repetition in both time and frequency domain. - UE does not expect there are multiple PUCCHs without repetitions in different PUCCH slots with a same or different UCI types other than SR overlapping with multiple PUSCH repetitions in the same OCC group. - FFS: whether the above applies only when at least one of the overlapping PUCCHs result in a UCI being multiplexed on the PUSCH Note 1: If the UCI on the PUCCH is dropped according to legacy rules and [updated] timeline conditions for UCI dropping are satisfied, there is no [additional] spec impact. (Option 1) Note 2: There can be multiple PUCCHs with same or different UCI types in the same slot (i.e. CSI report and HARQ-ACK) as in the legacy specifications Working assumption 1: The above agreement applies to different priority indexes for PUCCH/PUSCH if no additional specification impact is identified. Working assumption 2: The above agreement applies to PUCCH with repetitions if no additional specification impact is identified. Agreement For the OCC sequence applied for OCC DG PUSCH and CG PUSCH Type 2, the sequence is indicated dynamically in DCI - FFS: with a new field or reusing an existing field

It has been foreseen that commercial NTN deployments in near future need cover a scenario with different types of UEs including NTN-based aircraft [1], [2]. Therefore, even if we assume a certain level of UE grouping by network implementation to multiplex UEs with similar channel properties considering OCC demuxing performance, required details for the OCC-based multiplexing (e.g., OCC length, maximum number of multiplexed UEs, etc.) could be dynamically varying. For instance, the gNB may want to multiplex a smaller number of UEs (e.g., 1 or 2 UEs only) for a UE group comprises UEs with high FO and/or TO, while the gNB may prefer to apply medium or high multiplexing order (e.g., 4 UEs) for a UE group comprises UEs with low FO and/or TO. In this regard, we think both 1) OCC sequence book from semi-static configuration perspective and 2) OCC sequence indication/allocation from semi-static and/or dynamic signalling perspective should be supported to prevent frequent RRC reconfiguration and to reduce additional delay to apply/adapt OCC techniques.

More specifically, a new DCI field with 1 bit (for OCC length 2) or 2 bit (for OCC length 4) payload can be added within the scheduling/activation DCI format to indicated OCC sequence index. In case that both OCC length 2 and 4 are configured for PUSCH transmission scheduled/activated by a DCI format, an additional indication on the OCC type (i.e., applicable OCC length) for actual PUSCH transmission can be required. OCC sequence indication could be considered as not only OCC sequence indication (itself) but also OCC type indication. For example, if the bitwidth of OCC sequence indication is 2 bit, then OCC type indication might be considered as OCC length 4 (1 bit for OCC length 2, 0 bit for No OCC, respectively). Therefore, OCC type indication field might be omitted.

Proposal 1. For DG PUSCH and CG PUSCH type 2, support option 3 (first preference) or option 5 (second preference):

- Candidates (or Max value) of occ-Length: On top of the UE capability reports on OCC length 2 and 4, introduce an RRC signalling to configure candidates of occ-Length

- occ-Length & occ-Sequence are explicitly indicated by a new DCI field

  - Note: occ-Length could be omitted by the association with the bitwidth of occ-Sequence

Proposal 2. For DG PUSCH, For CG PUSCH type 2, and CG PUSCH type 1,

- OCC is disabled, if occ-Length configuration/indication is absent

- OCC is disabled, if occ-Length is not configured as 2 nor 4

Furthermore, RAN1 needs to finalize OCC sequence details for OCC length 4, e.g., by reusing the legacy Hadamard code. In RAN1#119, inter-slot OCC with OCC length 4 using Hadamard sequence was agreed, but the exact OCC sequence was not clarified. Thus, the exact OCC sequence needs to be clarified.

Proposal 3. Regarding the OCC sequence, support the following orthogonal sequences for OCC length 4

| n | wni |
| --- | --- |
| 0 | +1+1+1+1 |
| 1 | +1-1+1-1 |
| 2 | +1+1-1-1 |
| 3 | +1-1-1+1 |

Table 2 shows TBoMS RRC configurations by TS 38.331. As in Table 2 various combinations of “number of slots for single TBoMS (configured by numberOfSlotsTBoMS)” and “repetition (configured by numberOfRepetitions)”.

Table 2. RRC configurations for TBoMS.

PUSCH-Allocation-r16 ::= SEQUENCE { mappingType-r16 ENUMERATED {typeA, typeB} OPTIONAL, -- Cond NotFormat01-02-Or-TypeA startSymbolAndLength-r16 INTEGER (0..127) OPTIONAL, -- Cond NotFormat01-02-Or-TypeA startSymbol-r16 INTEGER (0..13) OPTIONAL, -- Cond RepTypeB length-r16 INTEGER (1..14) OPTIONAL, -- Cond RepTypeB numberOfRepetitions-r16 ENUMERATED {n1, n2, n3, n4, n7, n8, n12, n16} OPTIONAL, -- Cond Format01-02 ..., [[ numberOfRepetitionsExt-r17 ENUMERATED {n1, n2, n3, n4, n7, n8, n12, n16, n20, n24, n28, n32, spare4, spare3, spare2, spare1} OPTIONAL, -- Cond Format01-02-For-TypeA numberOfSlotsTBoMS-r17 ENUMERATED {n1, n2, n4, n8, spare4, spare3, spare2, spare1} OPTIONAL, -- Need R extendedK2-r17 INTEGER (0..128) OPTIONAL -- Cond MultiPUSCH ]] }

Given that RAN1 already agreed to support OCC for PUSCH with Type A repetition with code length 4, it seems that there is no need to preclude OCC for TBoMS transmission (2 slots are allocated for the single TBoMS and 2 repetitions are configured) as shown by Table 3. Therefore, RAN1 needs to clarify whether or not to support OCC for TBoMS transmission in NTN.

In addition, it seems that the total number of PUSCH repetitions with OCC schemes need to be discussed to finalize this agenda. In legacy NR, the total number of PUSCH repetitions is typically determined as (N∙K), where N represents numberOfSlotsTBoMS and K represents numberOfRepetitions. Given the nature of OCC, the total number of PUSCH repetitions should be associated with the OCC length somehow. For instance, if the total number of PUSCH repetitions remains the same as in legacy NR, i.e., (N∙K), regardless of the OCC length (L), a constraint will arise where the OCC length (L) must be a divisor of (N∙K) in order to maintain the orthogonality of OCC. In other words, for maintaining the orthogonality of OCC, the total number of PUSCH repetitions (N∙K) must be a multiple of (L). In another alternative, the total number of PUSCH repetitions can be determined by (N∙K∙L), so the orthogonality of OCC can be maintained always.

Table 3. Examples of OCC for TBoMS transmission (2 slots are allocated for the single TBoMS and 2 repetitions are configured).

| 0 | 1 | 2 | 3 | Slot number |
| --- | --- | --- | --- | --- |
| 0 | 0 | 2 | 2 | Used RV index per slot |
| 1 | 1 | 1 | 1 | OCC #1 |
| 1 | 1 | -1 | -1 | OCC #2 |

Proposal 4. RAN1 to clarify whether or not to support OCC for TBoMS transmission in NTN. If supported, RAN1 to consider the following cases:

- Case #1: numberOfSlotsTBoMS = 2, numberOfRepetitions = 2

| 0 | 1 | 2 | 3 | Slot number |
| --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | Used RV index per slot |
| 1 | 1 | 1 | 1 | OCC #1 |
| 1 | 1 | -1 | -1 | OCC #2 |

- Case #2: numberOfSlotsTBoMS = 2, numberOfRepetitions = 4

| 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | Slot number |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 2 | 2 | 2 | 2 | Used RV index per slot |
| 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | OCC #1 |
| 1 | 1 | -1 | -1 | 1 | 1 | -1 | -1 | OCC #2 |

- Case #3: numberOfSlotsTBoMS = 2, numberOfRepetitions = 8

| 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | Slot number |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 2 | 2 | 2 | 2 | 3 | 3 | 3 | 3 | 1 | 1 | 1 | 1 | Used RV index per slot |
| 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | OCC #1 |
| 1 | 1 | -1 | -1 | 1 | 1 | -1 | -1 | 1 | 1 | -1 | -1 | 1 | 1 | -1 | -1 | OCC #2 |

Proposal 5. RAN1 to clarify the total number of PUSCH repetitions with OCC schemes:

- Option#1: total number of PUSCH repetitions = N∙K (legacy)

  - N: numberOfSlotsTBoMS

  - K: numberOfRepetitions

  - The total number of repetitions (N∙K) is multiples of OCC length L

- Option#2: total number of PUSCH repetitions = N∙K∙L (new)

Conclusion

In this contribution, ETRI’s views on NR-NTN uplink capacity/throughput enhancement were shown and the following proposals were made:

Proposal 1. For DG PUSCH and CG PUSCH type 2, support option 3 (first preference) or option 5 (second preference):

- Candidates (or Max value) of occ-Length: On top of the UE capability reports on OCC length 2 and 4, introduce an RRC signalling to configure candidates of occ-Length

- occ-Length & occ-Sequence are explicitly indicated by a new DCI field

  - Note: occ-Length could be omitted by the association with the bitwidth of occ-Sequence

Proposal 2. For DG PUSCH, For CG PUSCH type 2, and CG PUSCH type 1,

- OCC is disabled, if occ-Length configuration/indication is absent

- OCC is disabled, if occ-Length is not configured as 2 nor 4

Proposal 3. Regarding the OCC sequence, support the following orthogonal sequences for OCC length 4

| n | wni |
| --- | --- |
| 0 | +1+1+1+1 |
| 1 | +1-1+1-1 |
| 2 | +1+1-1-1 |
| 3 | +1-1-1+1 |

Proposal 4. RAN1 to clarify whether or not to support OCC for TBoMS transmission in NTN. If supported, RAN1 to consider the following cases:

- Case #1: numberOfSlotsTBoMS = 2, numberOfRepetitions = 2

| 0 | 1 | 2 | 3 | Slot number |
| --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | Used RV index per slot |
| 1 | 1 | 1 | 1 | OCC #1 |
| 1 | 1 | -1 | -1 | OCC #2 |

- Case #2: numberOfSlotsTBoMS = 2, numberOfRepetitions = 4

| 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | Slot number |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 2 | 2 | 2 | 2 | Used RV index per slot |
| 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | OCC #1 |
| 1 | 1 | -1 | -1 | 1 | 1 | -1 | -1 | OCC #2 |

- Case #3: numberOfSlotsTBoMS = 2, numberOfRepetitions = 8

| 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | Slot number |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 2 | 2 | 2 | 2 | 3 | 3 | 3 | 3 | 1 | 1 | 1 | 1 | Used RV index per slot |
| 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | OCC #1 |
| 1 | 1 | -1 | -1 | 1 | 1 | -1 | -1 | 1 | 1 | -1 | -1 | 1 | 1 | -1 | -1 | OCC #2 |

Proposal 5. RAN1 to clarify the total number of PUSCH repetitions with OCC schemes:

- Option#1: total number of PUSCH repetitions = N∙K (legacy)

  - N: numberOfSlotsTBoMS

  - K: numberOfRepetitions

  - The total number of repetitions (N∙K) is multiples of OCC length L

- Option#2: total number of PUSCH repetitions = N∙K∙L (new)

References

- RP‑240737, LS on Seamless Air Alliance for Non-Terrestrial Network (NTN) Mobile VSAT Enhancements in Rel-19, Seamless Air Alliance NTN Working Group

- RP‑240079, Motivation to support mobile VSAT in NGSO deployment scenarios, Eutelsat Group, Fraunhofer IIS, Fraunhofer HHI, Airbus, ESA, Novamint, MediaTek, Sharp, Thales, SyncTechno Inc., Continental Automotive, TTP, Lockheed Martin, Robert Bosch GmbH

- 3GPP RAN1#120-bis Chair notes