3GPP TSG RAN WG1 Meeting #121	R1-2504152

St Julian’s, Malta, 19-23 May, 2025

Agenda item:	9.11.4 IoT NTN uplink capacity/throughput enhancement

Source: 	ETRI

Title:	Discussion on uplink capacity/throughput enhancement for IoT NTN

Document for:	Discussion/Decision

Introduction

Agreement

For the 3.75kHz SCS symbol-based OCC scheme, the granularity of spreading for data is one symbol.

Agreement

Dynamic activation / deactivation of OCC is supported by DCI.

- FFS: details of signalling by DCI

Conclusion

RAN1 assumes no specification change to support pairing UEs with different modulation orders.

Agreement

For 3.75kHz SCS OCC for NPUSCH format 1, RAN1 down selects between the following mappings between DMRS sequence samples and active TDM DMRS slots:

- Option 1: Sequential mapping of samples of the original DMRS sequence to active DMRS slots

- Option 2: Dropping of samples of the original DMRS sequence in blanked slots

Agreement

For NPUSCH Format 1 single-tone 15kHz SCS, for CDM DMRS with legacy pattern:

- DMRS symbols are spread before the OCC is applied

  - Option 1_1: according to the formula:

ru,OCCMn+m=runqm, 0≤n<X/M, m=0, …, M-1

Where: M is the OCC length, q is the assigned OCC codeword for the UE, run is the reference signal sequence defined in TS36.211 section 10.1.4.1.1 and X is the total number of slots in the NPUSCH transmission after OCC is applied

Agreement

The OCC sequence index is signalled using DCI format N0.

This contribution describes ETRI’s views on IoT NTN uplink capacity enhancement.

Agreement

The RRC parameters for IoT-NTN UL capacity enhancements are:

| RAN2 Parent IE | Parameter name in the spec | New or existing? | Description | Value range | UE-specific or Cell-specific |
| --- | --- | --- | --- | --- | --- |
| NPUSCH-ConfigDedicated-NB | npusch-OCC-Enabled | new | The parameter is used to enable OCC for NPUSCH format 1 single tone. | ENUMERATED {‘true’} | UE-specific |

Agreement

For indicating OCC sequence index and activation / deactivation, the following fields may be repurposed or constrained to be not present in the DCI:

- Modulation and coding scheme

- Repetition number

- Redundancy version

- Number of scheduled TB for Unicast

- Subcarrier indication

- Resource reservation

In this contribution, the details of uplink capacity enhancement for IoT NTN are discussed.

NPUSCH capacity enhancements

In the RAN1#120b meeting, the following agreements were made in [1]:

- 2.1  3.75kHz TDM DMRS multiplexing in the OCC scheme

For the previous meeting, the following mapping approaches between the DMRS sequence samples and active TDM DMRS slots are considered

- Option 1: Sequential mapping of samples of the original DMRS sequence to active DMRS slots

- Option 2: Dropping of samples of the original DMRS sequence in blanked slots

For Option 1, interference problem may happen when two adjacent cells operate with and without OCC, respectively. In such scenarios, the DMRS sequences located in legacy DMRS positions may differ, leading to interference between adjacent cells. However, Option 2 may provide a simpler and more robust solution. It maintains a consistent understanding of legacy DMRS positions on both the UE and gNB sides and ensures consistent inter-cell interference randomization regardless of OCC use.

Proposal 1: For the mapping issue between DMRS sequence samples and active TDM DMRS slots, we support option 2, where the original DMRS sequence samples are dropped in blanked slots.

- 2.2 The number of Repetition

- The interpretation of the repetition number transmitted via DCI format N0 was discussed in the previous meeting as follows

Proposal 4.8.1-1v2:

When [Nrep in 213 or] MrepNPUSCH [in 211] is the value from the repetition field of the DCI, M is the OCC factor and  Mrepdata is the number of repetitions of the data before OCC itself, which of the following understandings applies:

- understanding 1:  MrepNPUSCH=M×Mrepdata

- understanding 2: MrepNPUSCH =Mrepdata

The objective of the WID is capacity enhancement rather than coverage enhancement. Accordingly, the actual uplink repetition number should remain consistent with legacy configuration. In this context, understanding 1 where the number of scheduled repetitions is interpreted as a multiple of the OCC length, accounting for both the transmitted data and spreading is considered the correct interpretation. This approach ensures that the total number of transmission slots remains consistent with the legacy formula.

Proposal 2. For the number of repetition via DCI format N0, support the understanding 1

- 2.3 OCC Configuration signaling

In order to indicate both OCC activation/deactivation and OCC sequence index, legacy fields in DCI format N0 should be repurposed or indicated joint signalling using existing field. (e.g., MCS field and Subcarrier indication field).

Proposal 3. RAN1 to repurpose the signaling utilizing the existing DCI format N0 field of OCC related indication for NPUSCH format 1.

- 2.4 Modulation issue

For the orthogonality issues caused by different modulation schemes in single tone mode, the π/4-QPSK modulation scheme can be impacted. Therefore, it is desirable to consider QPSK for OCC mode like multi-tone mode instead of π/4-QPSK.

Proposal 4. RAN1 to consider QPSK modulation for OCC mode single tone mode instead of π/4-QPSK .

Conclusion

In this contribution, ETRI presented our views on uplink capacity/throughput enhancement for IoT NTN. We made the following observations and proposals:

Proposal 1: For the mapping issue between DMRS sequence samples and active TDM DMRS slots, support option 2, where the original DMRS sequence samples are dropped in blanked slots.

Proposal 2. For the number of repetition via DCI, support the understanding 1

Proposal 3. RAN1 to repurpose the signaling utilizing the existing DCI format N0 field of OCC related indication for NPUSCH format 1.

Proposal 4. RAN1 to consider QPSK modulation for OCC mode single tone mode instead of π/4-QPSK.

References

- 3GPP RAN1#120b, RAN1 Chair’s Notes

- RP-250472, Revised WID on NTN for IoT Phase 3, RAN#107

- R1-2502857, FL Summary for IoT NTN in Rel-19, RAN1#120b

- 3GPP TR 36.763, v.17.0.0, Study on NB-IoT/eMTC support for NTN, 2021-06

- 3GPP TR 38 863, v.17.0.0, Solutions for NR to support NTN: NTN related RF and co-existence aspect, 2022-06.

- 3GPP TS 36.213, “Evolved Universal Terrestrial Radio Access (E-UTRA); Physical layer procedures (Release 13),” TS 36.213 v14.4.0, 2017-09

- 3GPP TS 36.211, “Evolved Universal Terrestrial Radio Access (E-UTRA); Physical channel and modulation (Release 17),” TS 36.211 v17.4.0, 2023-09