3GPP TSG RAN WG1 #121		R1-2504219

St Julian’s, Malta, May 19th – 23rd, 2025

Source:	OPPO

Title:	   Discussion on UE feature for low-band CA via switching

Agenda Item:	9.15.12

Document for:	Discussion and Decision

# Introduction

This contribution elaborates our views on UE features for Rel-19 low-band CA via switching.

# Discussion

For FG component

Based on the WID [2] and RAN1 agreements in RAN1 #120bis, the FG components should at least cover both semi-static configuration and switching characteristics.

Proposal 1: The components of the main FG for LBCA at least include following.

- Component 1: Semi-static configuration of periodic switching pattern in bitmap for the switching between the following two cases, where FDD carrier 1 is PCell and SDL carrier 2 is SCell.

- Case 1: Tx/Rx on FDD carrier 1 and no Rx on SDL carrier 2

- Case 2: Rx on SDL carrier 2 and no Tx/Rx on FDD carrier 1

- Component 2: The time unit of the semi-statically configured switching pattern is 1ms slot, and the switching pattern periodicity is 40 1ms-slots.

For prerequisite FG

Although this low-band CA via switching is titled in “CA”, the components in basic CA FG 6-5, such as “Up to16 DL carriers”, provides a capability threshold much higher than what LBCA needs.

Proposal 2: The main LBCA FG has no prerequisite FG.

For FG granularity

Given LBCA is an aggregation of two carriers, the granularity type should be per band-combination.

Proposal 3: The granularity of LBCA FG is per BC.

For FDD/TDD differentiation

The concerned LBCA is specifically defined over a FDD carrier and a SDL carrier, the FDD/TDD differentiation does not apply.

Proposal 4: The FDD/TDD differentiation for LBCA FG is marked as N/A.

For FR1/FR2 differentiation

Given both carriers are restricted to SCS=15kHz, both carriers have to be in FR1.

Proposal 5: The FR1/FR2 differentiation for LBCA FG is marked as FR1 only.

# Conclusion

This contribution concludes with following proposals:

Proposal 1: The components of the main FG for LBCA at least include following.

- Component 1: Semi-static configuration of periodic switching pattern in bitmap for the switching between the following two cases, where FDD carrier 1 is PCell and SDL carrier 2 is SCell.

- Case 1: Tx/Rx on FDD carrier 1 and no Rx on SDL carrier 2

- Case 2: Rx on SDL carrier 2 and no Tx/Rx on FDD carrier 1

- Component 2: The time unit of the semi-statically configured switching pattern is 1ms slot, and the switching pattern periodicity is 40 1ms-slots.

Proposal 2: The main LBCA FG has no prerequisite FG.

Proposal 3: The granularity of LBCA FG is per BC.

Proposal 4: The FDD/TDD differentiation for LBCA FG is marked as N/A.

Proposal 5: The FR1/FR2 differentiation for LBCA FG is marked as FR1 only.

A preliminary UE feature table based on above discussions is given in the Appendix.

# Reference

- TS 38.822 v18.0.0,  NR User Equipment (UE) feature list (Release 18)

- RP-250247, Revised WID on Low NR band carrier aggregation via switching, AT&T

# Appendix

| Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation |  | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| X-1 | Low-band carrier aggregation of a FDD carrier and a SDL carrier via switching | 1. Semi-static configuration of periodic switching pattern in bitmap for the switching between the following two cases, where FDD carrier 1 is PCell and SDL carrier 2 is SCell. - Case 1: Tx/Rx on FDD carrier 1 and no Rx on SDL carrier 2 - Case 2: Rx on SDL carrier 2 and no Tx/Rx on FDD carrier 1 2. The time unit of the semi-statically configured switching pattern is 1ms slot, and the switching pattern periodicity is 40 1ms-slots. |  | YES | n/a | UE does not support carrier aggregation via switching on a pair of a FDD carrier and a SDL carrier. | Per BC | N/A | FR1 only |  | The feature is for SCS=15kHz only on both FDD carrier and SDL carrier. | Optional with capability signalling |