3GPP TSG RAN WG1 #121			R1-2504237

St Julian’s, Malta, May 19th – 23th, 2025

Agenda Item:	9.6.1

Source:	Panasonic

Title:	Discussion on the LP-WUS and LP-SS design

Document for:	Discussion/Decision

# Introduction

In the previous meetings, the agreements listed the appendix were achieved for LP-WUS and LP-SS design [1] [2] [3] [7][8][9].

In this contribution, we further share our views on the design of the LP-WUS and LP-SS.

# Discussion

On how the overlaid OFDM sequence carry LP-WUS information

On whether/how the overlaid OFDM sequence carry LP-WUS information, below agreements were achieved in the previous meetings.

Agreement from RAN1#119 meeting Update the existing agreement as shown below: In case of overlaid OFDM sequence carrying information, support option 2: - Option 2: One sequence is selected from multiple candidates overlaid OFDM sequences on each OOK ‘ON’ symbol, and OFDM-based LP-WUR obtain LP-WUS information at least by overlaid OFDM sequence(s). Consider the following two sub-options for potential down-selection. - Option 2-1: The overlaid OFDM sequence(s) carry part of information bits of LP-WUS. OFDM-based LP-WUR can obtain the whole information bits by OFDM sequence(s) and location of the OFDM sequence(s)/OOK ‘ON’ symbols. - Option 2-2: The overlaid OFDM sequence(s) carry all information bits of LP-WUS. OFDM-based LP-WUR can obtain the whole information bits by the overlaid OFDM sequence(s) - FFS the how the information bits are carried by the overlaid OFDM sequence(s) Note: the overlaid OFDM sequence in each OOK ‘ON’ symbol can be different according to information bits to be carried by the overlaid OFDM sequence within the LP-WUS. Agreement from RAN1#119 Regarding the maximum number of candidates overlaid sequences to carry LP-WUS information per OOK ON chip for one cell: - support maximum 4 candidates overlaid sequences for M=4 Agreement from RAN1#120 For idle mode, regarding the maximum number of candidates overlaid sequences to carry LP-WUS information per OOK ON chip for one cell: - support maximum 16 candidates overlaid sequences for M=1 - support maximum 8 candidates overlaid sequences for M=2 - For candidate overlaid sequences across all OOK ON chips of LP-WUS, the number of roots (in specification) is up to [FFS: X], FFS whether the number of roots can be different for different M value. - FFS: The number of overlaid sequences applicable for a UE is no more than 2 per OOK ON chip. Agreement from RAN1#120bis For WUS information carried by the overlaid OFDM sequence(s), down-select between Alt 1 and Alt2: - Alt 1: Raw information bits are mapped to sequence(s) - N raw information bits are divided into K segments from MSB to LSB, where K= ceil (N/log2L), L is the number of candidate overlaid OFDM sequences for one OOK ON chip. - In one OOK ON chip, a segment of information bits is mapped to one sequence sequentially, e.g., for a segment of 2 information bits, 00 is mapped to sequence #1, 01 is mapped to seq #2. - In case N/log2L is not an integer, Bit 0 as MSB is used for padding Agreement from RAN1#120bis For RRC connected mode, maximum number of candidates overlaid sequences to carry LP-WUS information per OOK ON chip for one cell, - support maximum 16 candidates overlaid sequences for M=1 - support maximum 8 candidates overlaid sequences for M=2 - support maximum 4 candidates overlaid sequences for M=4 (agreed in RAN1#119) - For candidate overlaid sequences across all OOK ON chips of LP-WUS, the number of roots (in specification) is up to [FFS: X], FFS whether the number of roots can be different for different M value. - FFS: The number of overlaid sequences applicable for a UE is no more than 2 per OOK ON chip. - FFS: Whether or not the candidate set of overlaid sequences change across OOK ON chips Working Assumption from RAN1#120bis At least for FR1, support maximum 2 roots to be used for overlaid OFDM sequences for M=1/2/4 (max 2 for each M value). - Different roots can be picked for different M values.

Regarding the FFS bullet on whether the candidate set of overlaid sequences change across OOK ON chips, we support that the candidate set of overlaid sequence may change across OOK ON chip with simple rules of sequence hopping, for better interference randomization. Different sets of L candidate overlaid OFDM sequences are defined and mapped to different OOK ON chips.

Proposal 1: The candidate set of overlaid sequences may change across OOK ON chips based on the configuration, e.g., when multiple sets of L candidate overlaid OFDM sequences are configured, each set of overlaid sequences is used sequentially in each OOK ON chip.

Regarding the maximum number of roots to be used for overlaid sequence in the working assumption and the number of roots (up to [FFS:X] in the agreement) in specification, our preference is:

- Per OOK ON chip, to support maximum 2 roots to be configured for overlaid OFDM sequences. This limits the maximum UE detection complexity.

- The maximum number of roots in the specification is up to 4 for each M value. Whether different roots are picked for different M values is up to gNB implementation. This allows gNB to handle the inter-cell and intra-cell interference with more flexibility.

Proposal 2: Per OOK ON chip, to support maximum 2 roots to be configured and used for overlaid OFDM sequences.

Proposal 3: The maximum number of roots in the specification can be up to 4 for each M value. Whether different roots are picked for different M values is up to gNB implementation.

On frequency resource of LP-WUS/LP-SS

Regarding frequency location of LP-WUS and LP-SS, below agreement was achieved in the previous meeting.

Agreement RAN1#120bis For RRC idle/inactive, support Alt 1 for both Redcap and non-redcap UEs: - Alt 2: LP-WUS/LP-SS frequency resource can be out of initial DL BWP where UE receives paging but within the same carrier. - FFS whether to define the maximum frequency gap between LP-WUS/LP-SS and SSB

Even Alt 2 is agreed, we think the maximum frequency gap between LP-WUS/LP-SS and SSB is needed. This would address the following concerns:

- The initial DL BWP range can be wide, which can be as large as the entire carrier bandwidth, i.e., 100 MHz for FR1 and 400 MHz for FR2.

- Despite LP-WUS/LP-SS is within or outside initial DL BWP, RF retuning would be challenging if the gap from SSB is too large, if LR reuses RF of MR.

- To handle the large frequency gap, either an additional RF module is introduced just for LR or large retuning delay and complexity are expected for the LR reusing the RF of MR.

Therefore, at least for OFDM-based LP-WUR, we see the merit of reusing current MR RF, if the frequency location of LP-WUS/LP-SS is sufficiently close to CD-SSB. It is noted that the main UE behaviour in IDLE/INACTIVE mode before monitoring paging is to track and measure the CD-SSB. Therefore, we think it would simplify the implementation of LP-WUR to locate LP-WUS/LP-SS in the frequency within a maximum gap from SSB such that the MR RF can be effectively reused with less issue, despite LP-WUS/LP-SS is within or out of initial DL BWP Note that MR RF filter width can be different from bands to bands. Therefore, instead of common bandwidth like initial DL BWP, some band specific discussion is required.

Observation 1: For OFDM-based LP-WUR, it is beneficial to reuse MR RF by limiting frequency location of LP-WUS/LP-SS within a maximum gap from CD-SSB.

As the band specific requirement, in TS 38.101-1, User Equipment (UE) radio transmission and reception, Clause 5.3.1, it is stated that “From a UE perspective, the UE is configured with one or more BWP / carriers, each with its own UE channel bandwidth. The UE does not need to be aware of the BS channel bandwidth or how the BS allocates bandwidth to different UEs.” The requirements of the combination of channel bandwidths, SCS and operating bands shown in Table 5.3.5-1 of TS 38.101-1. Depending on the level of LR reusing MR RF, a possible small channel bandwidth, e.g., 5 or 10 MHz can be starting point to define the frequency location of LP-WUS/LP-SS. If more flexibility from network side is desired, larger values need to be discussed. By using band specific common value among UEs who support this band, the network is not required to consider the UE specific the maximum frequency gap between LP-WUS/LP-SS and SSB. It can be common value among UEs in the band.

Proposal 4: To facilitate the implementation and deployment of the OFDM-based LP-WUR, it is proposed to define frequency location of LP-WUS/LP-SS with below restriction,

- LP-WUS/LP-SS frequency location has a maximum gap of X MHz from the CD-SSB. The candidate value of X should consider and balance the need from UE side to reuse MR RF and network side LP-WUS/LP-SS resource allocation flexibility.

- The channel bandwidth in the Table 5.3.5-1 of TS 38.101-1 can be a starting point to decide X, if RAN1 is to agree on the X value. And send LS to RAN4 to confirm the feasibility. Or

- Alternatively, to send an LS to RAN4 to request to decide value X.

Regarding the M value of LP-WUS and LP-SS, below was agreed in the previous meeting.

Agreement from RAN1#120bis meeting For the M value for LP-WUS and LP-SS, supports: - Alt2: The M values for LP-WUS and LP-SS can be configured to be same or different. M value for LP-WUS cannot be larger than that of LP-SS. - At least for M values larger than 1 (applies to both LP-WUS and LP-SS) - FFS: If M=1 is configured for either of LP-WUS or LP-SS, M=1 is configured for both

Regarding the FFS bullet, which is open for how to handle the M=1 case. Our views are:

- For M larger than 1, the EPRE of LP-WUS and LP-SS can be assumed same or almost the same without special handling, depending on how balanced between 0 and 1 chips. Due to that the time domain signal has half of the duration with OFF chips, in frequency domain the EPRE is equivalent to be halved.

- For M =1, as the OOK ON symbol takes the whole OFDM symbol, when gNB uses the same EPRE for LP-WUS/LP-SS as the other channels/signals, the equivalent EPRE of LP-WUS/LP-SS would be 3dB higher than the case of M > 1.

- When M for LP-SS is larger than 1 and M=1 for LP-WUS, the concern behind the FFS bullet would be the possible 3dB gap, if gNB does not handle specifically. If EPRE of LP-WUS and LP-SS are required to be aligned, which has clear merit for AGC training, t/f synchronization, and so on, gNB need to perform power boosting for LP-SS by implementation. Therefore, we see below options:

  - Opt.1: M must be 1 for both LP-WUS and LP-SS, if M=1 is configured.

    - EPREs of LP-WUS and LP-SS are assumed to be the same.

  - Opt.2: It is supported that LP-SS is with M>1 and LP-WUS is with M=1.

    - Opt.2.1: EPREs of LP-WUS and LP-SS are assumed to be the same. This means gNB needs to perform 3dB power boosting for LP-SS.

    - Opt.2.2: EPRE of LP-WUS is assumed 3dB larger than EPRE of LP-SS.

    - Opt.2.3: EPRE offset of LP-WUS and LP-SS is configurable

- We support Option 2.3, which is best for LP-SS synchronization performance and less limitation for network implementation.

- Option 1 is acceptable for us due to simple standardization efforts.

Proposal 5: For the M value for LP-WUS and LP-SS, it is supported that LP-SS is with M>=1 and LP-WUS with M=1. EPRE offset between LP-SS and LP-WUS is configurable.

Potential misalignment between overlaid sequence detection and OOK binary sequence detection

For OFDM-based LR, in our understanding, when channel condition is not good, the information bits UE detects from overlaid sequence can be different from the OOK symbols. Whether such misaligned detection can be identified by UE depends on the receiver implementation.

- If/when LR to perform early termination, only the overlaid sequences are detected. Early part of OOK symbols would be received and to detect OOK ON chips can be up to UE implementation.

- If/when LR to perform both overlaid and OOK detection, LR must detect the ON chips first in order to perform the overlaid sequence detection as a second step. When channel condition is not good, LR may receive and buffer more symbols of LP-WUS to perform decoding to make use of more energy of LP-WUS to improve reliability.

- OOK binary sequence is protected by RM coding or repetition, depending on the number of information bits.

- Overlaid sequence has no protection as raw information bits are directly mapped to sequence after segmentation. Depending on the future discussion, overlaid sequence may be repeated after the first raw information bits are mapped.

Therefore, if/when UE is able to identify the misalignment, to follow the OOK binary sequence due to the protection of the channel coding might be useful. As not all LR capable UE may support the identification of the misalignment between overlaid sequence detection and OOK binary sequence detection, up to UE implementation can be also one way. We are also open to other schemes to protect reliability, e.g., UE falls back to monitor PDCCH subject to conditions when such misalignment happens.

Proposal 6: If/When UE detects misalignment of the information bits between the overlaid sequence detection and OOK binary sequence detection, what is the UE behaviour should be discussed.

# Conclusion

- Based on the discussion, the following proposals are highlighted:

Proposal 1: The candidate set of overlaid sequences may change across OOK ON chips based on the configuration, e.g., when multiple sets of L candidate overlaid OFDM sequences are configured, each set of overlaid sequences is used sequentially in each OOK ON chip.

Proposal 2: Per OOK ON chip, to support maximum 2 roots to be configured and used for overlaid OFDM sequences.

Proposal 3: The maximum number of roots in the specification can be up to 4 for each M value. Whether different roots are picked for different M values is up to gNB implementation.

Observation 1: For OFDM-based LP-WUR, it is beneficial to reuse MR RF by limiting frequency location of LP-WUS/LP-SS within a maximum gap from CD-SSB.

Proposal 4: To facilitate the implementation and deployment of the OFDM-based LP-WUR, it is proposed to define frequency location of LP-WUS/LP-SS with below restriction,

- LP-WUS/LP-SS frequency location has a maximum gap of X MHz from the CD-SSB. The candidate value of X should consider and balance the need from UE side to reuse MR RF and network side LP-WUS/LP-SS resource allocation flexibility.

- The channel bandwidth in the Table 5.3.5-1 of TS 38.101-1 can be a starting point to decide X, if RAN1 is to agree on the X value. And send LS to RAN4 to confirm the feasibility. Or

- Alternatively, to send an LS to RAN4 to request to decide value X.

Proposal 5: For the M value for LP-WUS and LP-SS, it is supported that LP-SS is with M>=1 and LP-WUS with M=1. EPRE offset between LP-SS and LP-WUS is configurable.

Proposal 6: If/When UE detects misalignment of the information bits between the overlaid sequence detection and OOK binary sequence detection, what is the UE behaviour should be discussed.

# Reference

[1] 3GPP RAN1#117 meeting Chairman’s notes

[2] 3GPP RAN1#118 meeting Chairman’s notes

[3] 3GPP RAN1#118bis meeting Chairman’s notes

[4] RP-233291, Views on LP-WUS/WUR for R19, Vodafone

[5] RP-241824, Revised WID: Low-power wake-up signal and receiver for NR (LP-WUS/WUR)

[6] R1-2406785, Discussion on the LP-WUS and LP-SS design, 3GPP TSG RAN WG1 #118, Panasonic

[7] 3GPP RAN1#119 meeting Chairman’s notes

[8] 3GPP RAN1#120 meeting Chairman’s notes

[9] 3GPP RAN1#120bis meeting Chairman’s notes

# Appendix

Agreements from RAN1#117 meeting

Agreement

Support overlaid OFDM sequence based on existing NR sequence type for LP-WUS

- Down select among gold sequence, m sequence and ZC sequence

  - FFS the overlaid OFDM sequence is time or frequency domain sequence.

  - FFS how to reuse the existing sequences

  - Note: Strive to minimize the impact on OOK detection performance

- If overlaid OFDM sequence is supported for LP-SS, the same sequence type is used for both LP-SS and LP-WUS

Agreement

For the LP-SS sequence used in a cell,

- Option 1: the information necessary for determining the sequence is explicitly configured

- FFS: Additional support of determining the sequence by predefined rule without configuration

Agreement

The LP-WUS and LP-SS design assumes the residual frequency error after frequency error correction without considering impact of drift, is up to X ppm for OOK-based LP-WUR.

- FFS X which is no larger than 20ppm

- Initial frequency error assumption: up to company report

For the overlaid OFDM sequence design of LP-WUS, it is assumed that the residual frequency error for OFDM-based LP-WUR after frequency error correction without considering impact of drift is not larger than Y.

- FFS Y which is no larger than 20ppm and lower than X

- Initial frequency error assumption: up to company report

Agreement

For overlaid OFDM sequence(s) for LP-WUS in time or frequency domain, down-selection from the following:

- Option 1-1: overlaid sequence(s) are the sequence(s) of an OOK on symbol before DFT/LS processing

  - The length of overlaid sequence(s) depends on the number of REs used for LP-WUS and the value of M

- Option 1-2: overlaid sequence(s) are the sequence(s) of an OFDM symbol before DFT/LS processing

  - The length of overlaid sequence(s) depends on the number of REs used for LP-WUS

- Option 2: overlaid sequence(s) are the sequence(s) of an OFDM symbol before IFFT processing

  - The length of overlaid sequence(s) depends on the number of REs used for LP-WUS

- Option 3: overlaid sequence(s) are the sequence(s) of an OOK on symbol in time domain after IFFT processing

  - The length of overlaid sequence(s) depends on IFFT size and the value of M

FFS: same or different options are applied for OOK-1 and OOK-4 M>1.

Agreement

Update agreement in last meeting as below:

From RAN1 perspective, support X PRBs for LP-WUS and LP-SS with SCS 30kHz (blanked guard RBs are not included) for a channel bandwidth equal or larger than 5MHz

- X to be down-selected between = 11 and 12 PRBs

- FFS if other number of PRBs needed, for LP-SS and LP-WUS with a channel bandwidth equal or less than 5MHz

FFS: Whether the above is applicable to FR2

Agreement

Further down-select the number of binary LP-SS sequences for the ‘ON-OFF’ pattern:

- 3

- 4

- 8

- 16

Agreement

For the binary LP-SS sequence type for the ‘ON-OFF’ pattern in a LP-SS, further down-selection from the following:

- Gold sequence

- M sequence

- Computer searched sequence

- FFS: the length of LP-SS sequence

Agreement

The following SNR values for LP-WUR are reported by companies to achieve coverage of PUSCH for message3 for difference noise figures for calibration purposes:

- NF of LR = NF of MR+ 8dB: the reported SNR value range is [-9.05,2.94] dB

- NF of LR = NF of MR+ 5dB:  the reported SNR value range is [-6.5, 5.58] dB

- NF of LR = NF of MR+ 2dB: the reported SNR value range is [-4.04,7.95] dB

- Note 1: The above is observed based on the following assumptions used for calibration:

-	Carrier frequency: 2.6 GHz

-	The number of Tx chains: 1

-	MIL of MSG 3: use the average one in R17 coverage, i.e.,153.51 dB for non-redcap UE

-	Transmit antenna gain correction factors for WUS: up to company report

-	Noise Figure: All three values +2dB, +5dB, +8dB on top of NF of MR (7dB) are to be reported, SNR for different assumptions on NF are determined separately

Agreement

For evaluation of LP-WUS and LP-SS design to achieve coverage of PUSCH for message3 from RAN1 perspective, at least the following SNR values should be considered:

- SNR=-3dB for NF of LR = NF of MR+ 8dB

- SNR= -0.5dB for NF of LR = NF of MR+ 5dB

- SNR=2dB for NF of LR = NF of MR+ 2dB

- Note 1: The NF of MR is assumed as 7dB

Agreement

For RRC idle/inactive state, down-select among the following options for at least indicating subgroup information using LP-WUS:

- Option 1: A LP-WUS indicates a bitmap with each bit corresponding to one subgroup of N subgroups for part of, one or more PO(s), e.g., N is 8~16, 24

  - Number of information bits for a LP-WUS is at least N, single LP-WUS to wake up one or more subgroups

- Option 2: A LP-WUS indicates a codepoint value corresponding to one or more subgroup(s) from N subgroups for part of, one or more POs, e.g., N is 8~256

  - Number of information bits for a LP-WUS is at least ceil (log2(X1)), where X1 is the number of codepoints indicating one or more subgroups. X1 is reported by companies, X1 could be smaller, equal to or larger than N.

- Option 3: A LP-WUS indicates multiple codepoint values with each corresponding to one or more subgroup(s) from N subgroups for part of, one or more POs, e.g., N is 8~256

  - Number of information bits for a LP-WUS is at least K*ceil (log2(X2)), where X2 is the number of codepoints indicating one or more subgroups. X2 is reported by companies, X2 could be smaller, equal to or larger than N.

  - K is the number of multiple codepoint values in a LP-WUS where K is larger than 1

- How to satisfy FAR is reported by companies, e.g., FEC/CRC

- Note: multiple TDMed LP-WUSs can be used to support more subgroups for each option.

- Note: Y% effective paging rate per PO is reported by companies

- The followings are considered when down-select among options:

  - The number of supported UE subgroups per PO: M

  - Average network overhead to indicate the number of UE subgroups M per PO

  - False wake up rate due to subgroup-based indication, which will impact the power saving gain

  - Paging latency

Note: Coverage target shall be met under 1%BLER, 1% FAR (for false alarm from noise)

Agreements from RAN1#118 meeting

Agreement

For RRC idle/inactive state, support the following option for at least indicating subgroup information using LP-WUS:

- Option 2: A LP-WUS indicates a codepoint value corresponding to one or more subgroup(s) from N subgroups for part of, one or more POs

  - UE monitoring one or more MOs (up to X MOs) for the same beam within an LO is supported

    - Value of X is larger than 1. FFS on additional details of X.

Agreement

[H][FL1] Proposal 3.1-1: Confirm the Working Assumption that OOK-4 with M=4 for 15KHz SCS is supported for LP-WUS.

Agreement For OOK-4 with M >1, support M=2 & M=4 (working assumption) for LP-WUS. - M=4 for 15KHz SCS - M=4 for 30KHz SCS (working assumption) - FFS M=1 for OOK-4

Agreement

Regarding the LP-WUS information to trigger PDCCH monitoring of RRC connected UEs (for non-CA case), select at least one from the following

- Option 1: A bitmap with each bit corresponding to [one or more] UEs

- Option 3: A codepoint value corresponding to [one or more] UEs

- FFS details for extension of option 1 and/or 3 when UE is configured with CA

Agreement

For overlaid OFDM sequences for LP-WUS, support option 1-1 for OOK-4 M>1.

- Option 1-1: overlaid sequence(s) are the sequence(s) of an OOK on symbol before DFT/LS processing

Agreement

For overlaid OFDM sequences for LP-WUS, further down-selection between following two options for OOK-1 and OOK-4 with M=1(if supported)

- Option 1-1: Overlaid sequence(s) are the sequence(s) of an OOK on symbol before DFT/LS processing

- Option 2: Overlaid sequence(s) are the sequence(s) of an OFDM symbol before IFFT processing

- Note: Different options for OOK-1 and OOK-4 with M=1 (if supported) is not precluded – in which case, it should be deemed necessary

Agreement

The number of binary LP-SS sequences is 4.

Agreement

To determine the binary sequences for LP-SS, the evaluation assumes the following:

- Sync accuracy: timing estimation error smaller than T us for P=90 % of the time for at least SNR=-3dB and SNR=-6dB (lower priority),

  - T=2us(optional), 5us for OOK-1

  - T=1us(optional), 2us for OOK-4 with M=2

  - T=0.5us(optional), 1us for OOK-4 with M=4

  - Other values of SNR, T, M are up to company report.

  - Additionally, companies can submit results on SINR with details on how the interference was modelled.

  - Assume one-shot estimation

  - The time error is based on 20ppm maximum frequency error for the detection of the first LP-SS

  - Note: Companies can assume other values inside of 20ppm maximum as long as the values are justified

- RRM measurement accuracy: measurement accuracy within range ± XdB for Q=90% measurements based on Y LP-SS samples within a period comparable to Z=the length of I-DRX cycle that is larger or equal to 1.28s for at least SNR=-3dB and SNR=-6dB (lower priority), X, Y, and Z is up to company report, other values of SNR are up to company report.

  - As a starting point, the time error is based on 5-10ppm residual frequency error.

  - Note: 5-10ppm assumes frequency error correction is performed, e.g., RTC calibration and/or MR assistance

  - Note: Companies can assume other values outside of 5-10ppm as long as the values are justified

- The frequency error: up to company report

- Sampling rate: 3.84MHz (optional) or 7.68MHz assumed for 30kHz SCS.

- Channel: AWGN and TDL-C 300ns is assumed.

- Consider cross-correlation for 4 binary sequences under time and frequency error

- Companies are encouraged to provide details on other additional simulation assumptions, high level description of their receiver algorithm, and assessment on power consumption

Agreement

For OOK-based LP-WUR, the LP-WUS design assumes the following:

- As starting point, the time error is based on the 5-10ppm residual frequency error after frequency error correction without considering impact of drift.

- The frequency error: up to company report.

- Note: 5-10ppm assumes frequency error correction is performed, e.g., RTC calibration and/or MR assistance

- Note: Companies can assume other values outside of 5-10ppm as long as the values are justified

Agreement

As a starting point, for both time error and frequency error, the overlaid OFDM sequence design of LP-WUS assumes that the residual frequency error is 0.1-5ppm for OFDM-based LP-WUR after frequency error correction without considering impact of drift.

- Note: Companies can use any value within the above range with justification on the value (including impact on power consumption)

Agreement

Support Manchester coding for LP-WUS

- FFS other coding schemes

Agreements from RAN1#118bis meeting

Working Assumption

For overlaid OFDM sequences for LP-WUS in time or frequency domain, for OOK-1 and OOK-4 M=1

- Option 1-1(compromised): overlaid sequence(s) are the sequence(s) of an OOK on symbol before DFT/LS processing for OOK-4 with M=1.

  - ZC sequence in time domainDFT/LSIFFT to result in a frequency domain sequence that is ZC sequence or a sequence that is close to ZC sequence, no additional operation

    - It doesn’t preclude additional operation for OOK-4 with M>1.

    - DFT size is 2^n

Note1: OOK-1 is considered as specific case of OOK-4 with M=1.

Note: Concerns were raised by Ericsson and CATT that the above is not beneficial in terms of implementation complexity

Agreement

In case of overlaid OFDM sequence carrying information, support option 2:

- Option 2: One sequence is selected from multiple candidates overlaid OFDM sequences on each OOK ‘ON’ symbol, and OFDM-based LP-WUR obtain LP-WUS information at least by overlaid OFDM sequence(s). Consider the following two sub-options for potential down-selection.

- Option 2-1: The overlaid OFDM sequence(s) carry part of information bits of LP-WUS. OFDM-based LP-WUR can obtain the whole information bits by OFDM sequence(s) and location of the OFDM sequence(s)/OOK ‘ON’ symbols.

- Option 2-2: The overlaid OFDM sequence(s) carry all information bits of LP-WUS. OFDM-based LP-WUR can obtain the whole information bits by the overlaid OFDM sequence(s)

Note: the overlaid OFDM sequence in each OOK ‘ON’ symbol can be different according to information bits to be carried by the overlaid OFDM sequence within the LP-WUS.

Agreement

From RAN1 perspective, support X=11 PRBs for LP-WUS and LP-SS with SCS 15kHz (blanked guard RBs are not included) for a channel bandwidth equal or larger than 5 MHz

Agreement

To determine the binary sequences for LP-SS, for each M value, down-select the sequence length L from the corresponding candidate values:

- M=1, L= {4,6,8}

- M=2, L= {8,12,16,24}

- M=4, L= {16,24,32,56}

- FFS whether one or more than one L value (s) applied to each of M values M=1,2,4

- FFS the L values applied to other applicable SCS(s)

- FFS the L values if other M values in addition to M=1,2,4 are supported.

- FFS whether the time estimation averaged across multiple LP-SS occasions is applied

- Note 1: with sequence length L, it includes Manchester coding (if supported for LP-SS)

- Note 2: This doesn’t preclude any of three agreed sequence types

Above applies at least for both 15kHz and 30kHz

Agreement

For LP-SS periodicity, support at least 320ms

- FFS: 80ms,160ms, 640ms,1280ms, 2560ms, 5120ms, 10240ms

Agreement

For overlaid OFDM sequence at least for LP-WUS,

- For OOK-4 with M>1, support ZC sequence for overlaid sequence.

  - FFS details

Agreement

From RAN1 perspective, support X=11 PRBs for LP-WUS and LP-SS with SCS 60kHz and 120kHz for FR2.

Agreement

For the SCS used for LP-WUS and LP-SS, further discuss the following options:

- The single SCS is configured by gNB

- The single SCS is determined by pre-defined rule

Agreement

In case of overlaid OFDM sequence not carrying information:

- Option 1: Single overlaid sequence is on each OOK ‘ON’ symbol.

- Note 1: multiple overlaid OFDM sequences are specified.

- Note 2: gNB can configure different overlaid OFDM sequence(s) for different cells.

Agreement

Regarding the maximum number of candidate overlaid sequences to carry LP-WUS information per OOK ON chip for one cell, down-select from the below:

- 4

- 8

- 16

- 32

- 64 for M not larger than 2

- FFS whether the same or different set of candidate overlaid sequences are used for different cells

Agreement

To determine the binary sequences for LP-SS, consider the following for performance comparison:

- Sync accuracy T’: the achieved sync accuracy T’ for a set of 4 sequences with the same number of occupied OFDM symbols (timing estimation error smaller than ≤T’ us for P=90 % of the time for SNR=-3dB, -6dB (low priority))

  - Assume the sliding window size is at least [-25us, 25us].

- RRM measurement accuracy X: the achieved measurement accuracy within range ± XdB for Q=90% measurements based on Y LP-SS samples within a period comparable to Z=the length of I-DRX cycle that is larger or equal to 1.28s

  - Assume X= 3.5dB shall be satisfied for LP-RSRP and LP-RSRQ, based on Y=1, 4 and Z=1.28s for comparison

- Cross correlation: maximum cross-cor value/ maximum auto-cor value

  - FFS how to use cross-correlation for comparison

- Note: refer to RAN4 RRM assumption for the inter-cell interference

Agreement

Down-select among the following alternatives for LP-WUS information for OOK to meet target requirements:

- adding CRC

- channel coding other than Manchester coding, including rate matching if any

- mapped to a binary sequence/codeword, include truncation if any

- repetition, including rate matching if any

- Note: combination of options above is not precluded

- Note: combination of Manchester coding with above is assumed

FFS how LP-WUS is transmitted within a MO/MOs

Agreement

Support overlaid OFDM sequence(s) for LP-SS:

- LP-SS reuses the overlaid OFDM sequence(s) specified for LP-WUS. The design on overlaid OFDM sequence(s) specified for LP-WUS doesn’t target for sync and RRM measurement performance based on overlaid OFDM sequence for LP-SS.

- Whether to transmit LP-SS by using a specified overlaid OFDM sequence is configurable.

  - Applicable at least for OOK-1 and FFS for OOK-4

- From RAN1 perspective, it is not intended to introduce new RAN4 requirements specific to overlaid sequences

Reminder to companies who are interested in LP-SS:

Evaluation results on binary sequences for LP-SS are to be collected in RAN1 #119 meeting for comparison.

Agreements from RAN1#119 meeting

Agreement

Update the existing agreement as shown below:

In case of overlaid OFDM sequence carrying information, support option 2:

- Option 2: One sequence is selected from multiple candidates overlaid OFDM sequences on each OOK ‘ON’ symbol, and OFDM-based LP-WUR obtain LP-WUS information at least by overlaid OFDM sequence(s). Consider the following two sub-options for potential down-selection.

  - Option 2-1: The overlaid OFDM sequence(s) carry part of information bits of LP-WUS. OFDM-based LP-WUR can obtain the whole information bits by OFDM sequence(s) and location of the OFDM sequence(s)/OOK ‘ON’ symbols.

  - Option 2-2: The overlaid OFDM sequence(s) carry all information bits of LP-WUS. OFDM-based LP-WUR can obtain the whole information bits by the overlaid OFDM sequence(s)

    - FFS the how the information bits are carried by the overlaid OFDM sequence(s)

Note: the overlaid OFDM sequence in each OOK ‘ON’ symbol can be different according to information bits to be carried by the overlaid OFDM sequence within the LP-WUS.

Agreement

For the overlaid OFDM sequence in time domain, the sequence is generated based on

,

- M=1, is given by the largest prime number such that,is the overlaid OFDM sequence length.

  - The base overlaid sequence is generated by extension of

,

  - With CS(s) applied to the base overlaid OFDM sequence if any:denotes the potential cyclic shift (s)

,,

- M=2, 4, is down-selected from the following:

  - Alt1: is given by the largest prime number such that,is the overlaid OFDM sequence length.

    - The base overlaid sequenceis generated by extension of

,

    - With CS(s) applied to the base overlaid OFDM sequence if any:denotes the potential cyclic shift (s)

,

    - Note it doesn’t preclude any pulse shaping scheme if any.

  - Alt2:is given by the largest prime number such that,  is the overlaid OFDM sequence length.

    - The base overlaid sequenceis generated by inserting zeros before and/or after . The total number of zeros is.

For example,,

    - With CS(s) applied to the base overlaid OFDM sequence if any: denotes the potential cyclic shift (s)

For example,,

        - where,

    - FFS the value of G, with G>0.

- FFS value(s) of root q and/or CS  for generating the candidate sequences if applied

- FFS on whether changes are needed to handle intra-/inter-cell interference

- Note: the overlaid OFDM sequence in time domain is based on potential modification to ZC sequence as listed above.

  - Above overrides any previous RAN1 agreement / working assumption

Agreement

At least for M>1, for the overlaid OFDM sequence in time domain, down-select the sequence length LZC from the following:

- Alt1:, with

  - FFS how to map the generated overlaid OFDM sequence to the frequency domain

- Alt2:

- Note: X is the number of RBs of LP-WUS/LP-SS bandwidth (blanked guard RBs are not included)

Agreement

CS(s) and/or root(s) used for overlaid OFDM sequence in the time domain are derived from RRC signalling:

- FFS: the set of values of CS and root for configuration

- FFS: details of the RRC signaling

Agreement

For the length L of LP-SS binary sequence, limit the selection to the following:

- M=1, L= {4, 6, 8}

- M=2, L= {8, 12, 16}

- M=4, L= {16, 32}

Agreement

To compare LP-SS binary sequence performance, cross-correlation is deprioritized over sync accuracy, i.e., cross-correlation within a set of 4 sequences is further compared for different sets of sequences with similar sync accuracy.

Agreement

For the M value for LP-WUS and LP-SS, down-select one alternative from the following in RAN1#120:

- Alt1: The M values for LP-WUS and LP-SS are always same.

- Alt2: The M values for LP-WUS and LP-SS can be configured to be same or different. M value for LP-WUS cannot be larger than that of LP-SS.

- Alt3: The M values for LP-WUS and LP-SS can be configured to be same or different.

Agreement

Regarding the maximum number of candidates overlaid sequences to carry LP-WUS information per OOK ON chip for one cell:

- support maximum 4 candidates overlaid sequences for M=4

For future meetings:

Companies are encouraged to consider [H][FL4] Proposal 3.3-1 in R1-2410838

Agreement

Regarding whether to support additional sync signal to LP-SS, down-selection from the following:

- Option 1A: No additional sync signal, LP-SS periodicity =320ms

  - Additionally support at least one of [80ms,160ms]

    - FFS: whether different values can be applicable for different M values

- Option 1B: No additional sync signal, LP-SS periodicity =320ms

  - No additional support of other periodicities

- Option 2: With additional sync signal, LP-SS periodicity =320ms

  - FFS additionally support other LP-SS periodicities

  - FFS details on additional sync signal

  - FFS additional sync signal is configurable and/or conditionally present

For future meetings:

Companies are encouraged to consider the following for future discussions

Regarding the LP-WUS information to trigger PDCCH monitoring of RRC connected UEs,

- Option 1: A bitmap with each bit corresponding to [one or more] UEs

  - 1A No CRC

    - Per bit coding, e.g., one bit is mapped to one of 2 codeword/sequences

    - The maximum number of codewords targeting per bit position in the bitmap for a UE is 1

    - FFS coded bit length

    - Single MO

  - 1B No CRC

    - Bitmap-level coding, e.g., one bitmap of L bits is mapped to one of 2L codeword/sequences

    - The maximum number of codewords/sequences targeting for a UE is 2L-1

    - Single MO

  - 1C With CRC

    - FFS CRC length

    - Single MO

- Option 2: A codepoint value corresponding to [one or more] UEs

  - 2A only 1-1 mapping from codepoint to UE

    - The maximum number of codepoints targeting per MO for a UE is 1

    - FFS the maximum number of MOs to be detected by a UE

  - 2B 1-to-1, 1-to-all mapping from codepoint to UEs

    - The maximum number of codepoints targeting per MO for a UE is up to 2

    - FFS the maximum number of MOs to be detected by a UE

  - 2C 1-to-X mapping from codepoint to UEs,

    - The maximum number of codepoints targeting per MO for a UE is up to

    - FFS the maximum number of MOs to be detected by a UE

- Note: L is the number of UEs associated monitoring the same WUS

Agreement

For RRC idle/inactive state, for the case where associated CD-SSB and initial DL BWP have the same SCS at least when the associated CD-SSB and LP-WUS are on the same carrier

- The single SCS for LP-WUS/LP-SS is same as the associated CD-SSB

- FFS: Case where associated CD-SSB and initial DL BWP have different SCSs

- FFS: Which initial BWP for RedCap

Agreement

For collecting LP-SS binary sequence results

- Endorse the template for collecting LP-SS binary sequence results in R1-2410925

- Email discussion for collecting LP-SS binary sequences by 10th Jan.

- Please include your cross-checking results (e.g., a separate excel sheet) in the t-doc for #120.

Agreements from RAN1#120 meeting

Agreement

For LP-WUS information carried by OOK, select alt 1 for codepoint to meet FAR and MDR performance.

- Alt 1: Coding with rate matching if any

  - For more than 2 information bits: RM coding as in section 5.3.3.3 of 38.212 (FFS: whether scrambling in the same section is applied or not)

  - 2 information bits (if supported): coding as in section 5.3.3.2 of 38.212 (FFS: whether scrambling in the same section is applied or not)

  - 1 information bit (if supported): repetition as in section 5.3.3.1 of 38.212 (FFS: whether scrambling in the same section is applied or not)

  - FFS: Code block length

  - FFS: Further repetition before or after Manchester coding

Conclusion

For RRC idle/inactive, LP-WUS is not supported for the case where associated CD-SSB and initial DL BWP have different SCSs

Agreement

At least for M>1, for the overlaid OFDM sequence length, support

- Alt2:

- Note: X is the number of RBs of LP-WUS/LP-SS bandwidth (blanked guard RBs are not included)

Agreement

Proposal 4.3-1: Update the agreements in RAN1 #118bis as below

Agreement

Support overlaid OFDM sequence(s) for LP-SS:

- LP-SS reuses the overlaid OFDM sequence(s) specified for LP-WUS. The design on overlaid OFDM sequence(s) specified for LP-WUS doesn’t target for sync and RRM measurement performance based on overlaid OFDM sequence for LP-SS.

  - Applicable to both OOK-1 and OOK-4

- Whether to transmit LP-SS by using a specified overlaid OFDM sequence is configurable.

  - Applicable at least for OOK-1 only and FFS for OOK-4

- From RAN1 perspective, it is not intended to introduce new RAN4 requirements specific to overlaid sequences

Agreement

For RRC connected, for LP-WUS SCS:

- Alt 1: LP-WUS SCS is same as the active DL BWP

Working Assumption

Regarding the LP-WUS information to trigger PDCCH monitoring of RRC connected UEs:

- Codepoint based

  - The maximum number of codepoints checked per MO by a UE is up to 8

    - Depending on UE capability, a UE may support less than 8

Agreement

For M=2, 4, is given by the largest prime number such that,is the overlaid OFDM sequence length.

- The base overlaid sequenceis generated by extension of

,

- With CS(s) applied to the base overlaid OFDM sequence if any:denotes the potential cyclic shift (s)

,

- Note it doesn’t preclude any pulse shaping scheme if any.

Agreement

For RRC connected, LP-WUS frequency resource can be outside of active DL BWP but has to be within the same carrier as the active DL BWP

- Basic capability is LP-WUS, if present, frequency resource within active DL BWP. LP-WUS frequency resource outside of active DL BWP is subject to separate UE capability

- No RAN1 optimization specific to the case where LP-WUS frequency resource is outside of active DL BWP

Agreement

For M=1, L=4 (if supported), the set of LP-SS binary sequences is:

[0 1 0 1]

[0 1 1 0]

[1 0 0 1]

[1 0 1 0]

Agreement

For M=1, L=6 (if supported), the set of LP-SS sequence is:

[1 0 1 0 1 0]

[0 1 0 1 0 1]

[1 0 0 1 0 1]

[1 0 1 0 0 1]

Agreement

For M=1, L=8 (if supported), the set of LP-SS sequence is:

[1 0 1 0 0 1 0 1]

[1 0 1 0 1 0 0 1]

[1 0 0 1 0 1 0 1]

[0 1 0 1 0 1 0 1]

Agreement

For M=2, L=8 (if supported), the set of LP-SS sequence is:

[0 1 0 1 1 0 0 1]

[0 1 1 0 0 1 0 1]

[0 1 1 0 1 0 0 1]

[1 0 0 1 0 1 1 0]

Agreement

For M=2, L=12 (if supported), the set of LP-SS sequence is:

[1 0 0 1 1 0 0 1 1 0 0 1]

[0 1 1 0 1 0 0 1 1 0 0 1]

[0 1 1 0 0 1 1 0 1 0 0 1]

[0 1 1 0 0 1 0 1 1 0 0 1]

Agreement

For M=2, L=16 (if supported), the set of LP-SS sequence is:

[1 0 0 1 0 1 0 1 1 0 0 1 1 0 0 1]

[1 0 0 1 1 0 0 1 0 1 1 0 0 1 0 1]

[1 0 0 1 1 0 1 0 0 1 0 1 1 0 0 1]

[1 0 1 0 0 1 1 0 0 1 1 0 0 1 0 1]

Agreement

For M=4, L=16 (if supported), the set of LP-SS sequence is down-selected between:

- Set 1

[0 1 1 0 1 0 0 1 1 0 1 0 1 0 1 0]

[0 1 1 0 1 0 1 0 1 0 0 1 1 0 1 0]

[1 0 1 0 0 1 1 0 1 0 1 0 1 0 0 1]

[1 0 1 0 1 0 0 1 1 0 1 0 0 1 1 0]

- Set 2

[1 0 0 0 1 0 0 0 0 0 0 1 0 0 1 0]

[1 0 0 0 0 1 0 0 1 0 0 0 0 0 1 0]

[1 0 0 0 0 1 0 0 0 1 0 0 1 0 0 0]

[1 0 0 0 0 1 0 0 0 0 0 1 0 0 0 1]

Agreement

For M=4, L=32(if supported), the set of LP-SS sequence is down-selected between:

- Set 1:

[0 1 0 1 1 0 1 0 1 0 1 0 1 0 0 1 1 0 1 0 0 1 1 0 0 1 1 0 0 1 0 1]

[0 1 1 0 0 1 0 1 0 1 1 0 0 1 0 1 1 0 0 1 1 0 1 0 1 0 1 0 0 1 0 1]

[0 1 0 1 0 1 0 1 1 0 1 0 1 0 0 1 1 0 1 0 1 0 0 1 1 0 1 0 0 1 1 0]

[0 1 0 1 0 1 1 0 0 1 0 1 1 0 1 0 0 1 1 0 0 1 1 0 1 0 1 0 0 1 0 1]

- Set 2

[0 0 0 1 1 0 0 0 0 0 0 1 0 0 0 1 1 0 0 0 0 0 0 1 0 0 0 1 0 1 0 0]

[0 0 0 1 0 1 0 0 1 0 0 0 1 0 0 0 0 1 0 0 0 1 0 0 0 0 0 1 0 1 0 0]

[0 0 1 0 0 0 1 0 1 0 0 0 0 1 0 0 1 0 0 0 0 0 0 1 0 0 1 0 0 0 1 0]

[0 0 1 0 0 1 0 0 0 0 0 1 1 0 0 0 0 0 1 0 1 0 0 0 0 1 0 0 0 0 1 0]

Agreement

For idle mode, regarding the maximum number of candidates overlaid sequences to carry LP-WUS information per OOK ON chip for one cell:

- support maximum 16 candidates overlaid sequences for M=1

- support maximum 8 candidates overlaid sequences for M=2

- For candidate overlaid sequences across all OOK ON chips of LP-WUS, the number of roots (in specification) is up to [FFS: X], FFS whether the number of roots can be different for different M value.

  - FFS: The number of overlaid sequences applicable for a UE is no more than 2 per OOK ON chip.

Agreement

For WUS information carried by the overlaid OFDM sequence(s), consider at least the following alternatives:

- Alt 1: Raw information bits are mapped to sequence(s)

- Alt 2: Raw information bits are mapped to sequence(s) after channel coding

  - Same channel coding scheme(s) as OOK is applied.

    - FFS same or different rate matching and/or repetition factor as OOK

- Alt 3: Codepoint/Subgroup is mapped to sequence(s)

Agreements from RAN1#120bis meeting

Agreement

From specification design perspective, for both RRC idle/inactive and RRC connected mode, support up to 5 information bits and up to 32 codepoints for a LP-WUS.

Agreement

Support maximum length for a LP-WUS (actual OFDM symbols used) which is larger than one slot

- Non-consecutive OFDM symbols at least across multiple slots can be used for a LP-WUS transmission

Agreement

For RRC connected mode, maximum number of candidates overlaid sequences to carry LP-WUS information per OOK ON chip for one cell,

- support maximum 16 candidates overlaid sequences for M=1

- support maximum 8 candidates overlaid sequences for M=2

- support maximum 4 candidates overlaid sequences for M=4 (agreed in RAN1#119)

- For candidate overlaid sequences across all OOK ON chips of LP-WUS, the number of roots (in specification) is up to [FFS: X], FFS whether the number of roots can be different for different M value.

  - FFS: The number of overlaid sequences applicable for a UE is no more than 2 per OOK ON chip.

- FFS: Whether or not the candidate set of overlaid sequences change across OOK ON chips

Agreement

For M=1, the overlaid OFDM sequence length LZC=12*X, X is the number of RBs of LP-WUS/LP-SS bandwidth (blanked guard RBs are not included)

- The agreement overrides previous WA

Agreement

Regarding whether to support additional sync signal to LP-SS, support the following for LP-WUS with M=1, M=2:

- Option 1A: No additional sync signal, LP-SS periodicity =320ms

  - Additionally support LP-SS periodicity of 160ms

Working Assumption

At least for FR1, support maximum 2 roots to be used for overlaid OFDM sequences for M=1/2/4 (max 2 for each M value).

- Different roots can be picked for different M values.

Agreement

For > 2 information bits with RM coding, support one of the following alternatives for rate matching after RM coding, when code block (coded bits) length after rate matching < 32

- Alt 1: Rate matching after RM coding is applied, as section 5.4.3 in TS 38.212.

Agreement

For the length L of LP-SS binary sequence, supports

- M=1, L= {6,8}

- M=2, L= {12,16}

- M=4, L= {16,32}

Note: For each value of M, UE supports both L values – no UE capability.

Note: There is no consensus in RAN1 that SNR of -3dB can be fulfilled with the smaller value of L for each M. Definition of the proper requirements for different values of L is up to RAN4.

Agreement

For LP-SS with M=4, supports set1 (balanced).

Agreement

For the root(s) used for overlaid OFDM sequences, gNB can configure any value from 1 ~ (Bzc -1), where Bzc is given by the largest prime number such that BZC<LZC, where  LZC is the overlaid OFDM sequence length.

Agreement

For WUS information carried by the overlaid OFDM sequence(s), down-select between Alt 1 and Alt2:

- Alt 1: Raw information bits are mapped to sequence(s)

  - N raw information bits are divided into K segments from MSB to LSB, where K= ceil (N/log2L), L is the number of candidate overlaid OFDM sequences for one OOK ON chip.

  - In one OOK ON chip, a segment of information bits is mapped to one sequence sequentially, e.g., for a segment of 2 information bits, 00 is mapped to sequence #1, 01 is mapped to seq #2.

  - In case N/log2L is not an integer, Bit 0 as MSB is used for padding

Agreement

For > 2 information bits with RM coding, support candidate code block (coded bits) length after rate matching within the below range:

- Alt 1a: code block (coded bits) length: [3,4]~[32,64]

  - FFS whether all or a subset of lengths is selected.

  - FFS whether same range applies to all M values

  - FFS: above lengths do not consider repetition (if supported) after rate matching

Agreement

For RRC idle/inactive, support Alt 1 for both Redcap and non-redcap UEs:

- Alt 2: LP-WUS/LP-SS frequency resource can be out of initial DL BWP where UE receives paging but within the same carrier.

    - FFS whether to define the maximum frequency gap between LP-WUS/LP-SS and SSB

Working Assumption

Support 1 & 2 information bits for LP-WUS. No additional restriction is introduced on the supported number of subgroups/codepoints by LP-WUS.

- 2 information bits: coding is done as described in the first row in the table in section 5.3.3.2 of 38.212 (scrambling is not applied)

- 1 information bit: repetition is done as described in the first row in the table in section 5.3.3.1 of 38.212 (scrambling is not applied)

Agreement

For the M value for LP-WUS and LP-SS, supports:

- Alt2: The M values for LP-WUS and LP-SS can be configured to be same or different. M value for LP-WUS cannot be larger than that of LP-SS.

  - At least for M values larger than 1 (applies to both LP-WUS and LP-SS)

  - FFS: If M=1 is configured for either of LP-WUS or LP-SS, M=1 is configured for both