3GPP TSG RAN WG1 Meeting #121	R1-2504271

St Julian's, Malta, 19th -23rd May, 2025

Agenda Item:	9.11.2

Source:	MediaTek Inc.

Title:	Support of RedCap and eRedCap UEs with NR NTN operating in FR1-NTN bands

Document for:	Discussion and Decision

- 1 Introduction

In RAN#105 [1], a revised WID on NR NTN enhancements was endorsed for Release 19. This contribution aims to discuss support of RedCap and eRedCap UEs with NR NTN operating in FR1-NTN bands objective as shown below:

For HD-FDD RedCap Ues and eRedCap Ues, specify enhancements for mitigating issues caused by TA mismatch between actual TA used by the UE and assumed TA for the UE at the gNB[RAN1]

- Enhancements are targeted for the following HD collisions cases (cases 3 and 4):

  - Semi-statically configured DL reception collides with semi-statically configured UL transmission

  - Dynamically scheduled DL reception collides with dynamic scheduled UL transmission

- Note: enhancements for other HD collision cases are not targeted. Enhancements for HD collision cases 3 and 4 that may improve other HD collision cases are not precluded.

- Note: potential work in RAN2, if any, starts after RAN1 has decided on the solution.

Notes for this objective:

- GNSS (Global Navigation Satellite Systems) capabilities and simultaneous GNSS and NR-NTN operation is supported in RedCap/eRedCap UE.

This contribution aims to discuss support of RedCap and eRedCap UEs with NR NTN operating in FR1-NTN bands.

- 2 Discussion on HD collision rules Case 3

For Rel-19 HD-FDD (e)Redcap UE in RRC connected mode, for collision case 3,

- Handling of collision with Type-0/0A/1/2-PDCCH CSS in RRC-Connected mode is left to UE implementation whether to prioritize UL or prioritize DL with the constraint in the following note.

- For other use cases, default priority rule for collision case 3 in RRC-Connected mode is that DL is prioritized.

  - Network is allowed to indicate UL overriding DL for all the other use cases above

    - This is signaled by RRC configuration

Note: UE shall comply to the following existing procedure in 38.331:

- UEs in RRC_CONNECTED shall monitor for SI change indication in any paging occasion at least once per modification period if the UE is provided with common search space, including pagingSearchSpace, searchSpaceSIB1 and searchSpaceOtherSystemInformation, on the active BWP to monitor paging, as specified in TS 38.213 [13], clause 13.

RAN1#119 made the following agreement on Case 3.

RAN1#120 made the following agreement and conclusion on Case 3.

Agreement

When network indicates UL overriding DL in RRC-Connected mode for collision case 3, one UE-specific RRC parameter is used to indicate overriding for all the applicable other use cases except for collisions with Type-0/0A/1/2-PDCCH CSS.

Conclusion

For Rel-19 HD-FDD (e)Redcap UE with CG-SDT procedure ongoing in RRC-inactive mode, for collision case 3,

- Handling of collision with PDCCH CSS in RRC-inactive mode is left to UE implementation whether to prioritize UL or prioritize DL with the constraint in the following note.

Note: UE shall comply to the following existing procedure in 38.331:

- UEs in RRC_INACTIVE while SDT procedure is not ongoing shall monitor for SI change indication in its own paging occasion(s) that the UE monitors as specified in TS 38.304 [20].

- UEs in RRC_INACTIVE while SDT procedure is ongoing shall monitor for SI change indication in any paging occasion at least once per modification period, if the initial downlink BWP on which the SDT procedure is ongoing is associated with a CD-SSB.

- ETWS or CMAS capable UEs in RRC_INACTIVE while SDT procedure is not ongoing shall monitor for indications about PWS notification in its own paging occasion every DRX cycle.

- ETWS or CMAS capable UEs in RRC_INACTIVE while SDT procedure is ongoing shall monitor for indication about PWS notification in any paging occasion at least once every defaultPagingCycle.

RAN1 agreements on Case 3 are sufficient and further details can be discussed in the maintenance phase as needed.

# 3 Discussion on HD collision rules Case 4

RAN1#120bis agreed to revise the working assumption on collision case 4 as shown below:

Agreement

Update the working assumption in RAN1 #120 with the following updates:

For Rel-19 NTN HD-FDD (e)Redcap UE in RRC connected mode, the following handling rule for collision case 4 is supported:

- Handling of collision with PDSCH (at least for system information) scheduled by Type-0/0A-PDCCH CSS in RRC-Connected mode is left to UE implementation whether to prioritize UL or prioritize DL with the constraint in the note2.

- Handling of collision with PDSCH scheduled by Type-1/2-PDCCH CSS in RRC-Connected mode is left to UE implementation whether to prioritize UL or prioritize DL with the constraint in the note2.

- FFS: handling of PDCCH ordered PRACH transmission

- For other use cases, default priority rule for collision case 4 in RRC-Connected mode is that DL is prioritized.

  - Network is allowed to indicate UL overriding DL for all cases

    - This is signaled by one UE specific RRC parameter

  - Note1: if DL is prioritized, the DL prioritization applies only if the UL cancellation timeline can be satisfied, otherwise UL is prioritized.

Note2: UE shall comply to the following existing procedure in 38.331:

- UEs in RRC_CONNECTED shall monitor for SI change indication in any paging occasion at least once per modification period if the UE is provided with common search space, including pagingSearchSpace, searchSpaceSIB1 and searchSpaceOtherSystemInformation, on the active BWP to monitor paging, as specified in TS 38.213 [13], clause 13.

The PDCCH ordered PRACH transmission could be used in case gNB receiver determines that UE may lose UL synchronization. Following Msg1 RACH preamble, a Msg2 TAC can be used by the UE to keep synchronized for UL transmission. A typical UE behaviour would be to prioritize PDCCH ordered PRACH transmission. This can be left to UE implementation without need for specification.

Proposal 1:  Handling of PDCCH ordered PRACH transmission is up to device implementation.

- 4 Discussion on TA reporting

To mitigate the collisions of case3 and case4, the following TA reporting enhancements for Rel-19 NTN HD-FDD (e)Redcap UE can be further studied:

- Finer TA report granularity

- Smaller TA offset threshold

- Trigger a TA report when the collision is detected at the UE

- TA reporting with a new triggering mechanism from gNB

- TA drifting rate reporting

Note: The benefit, complexity, power consumption and signaling overhead of each scheme is to be further investigated.

Note: other solutions can also be studied

TA reporting is beneficial to mitigate the TA mismatch between actual TA used by the UE and assumed TA for the UE at the gNB for HD-FDD RedCap/eRedCap UE in NTN from RAN1 perspective.

RAN1#118 and RAN1#117  made the following observation on TA report

In the legacy Rel-17 specifications, Koffset is a configured parameter that needs to be larger or equal to the sum of the service link RTT and the Common TA (i.e. the RTT between the satellite and the Uplink Synchronization Reference Point (USRP) as illustrated in the Figure in the Appendix). When NR NTN UE initiates the cell access via Random Access procedure, the Koffset is initially based on max RTT in the satellite cell as configured by cell-specific Koffset in SIB19. After UE moves to connected, gNB can send UE specific configuration of Koffset based on the TA report (i.e. the max RTT for the UE is based on its own RTT over the service link and the common TA). This requires no change in the specifications for support of RedCap in NTN in Release 19. Clause 16.14.2.1 of TS 38.300 specifies details of scheduling and timing with the Koffset is copied in the appendix. The granularity of UE-specific Koffset is 1 ms.

A redcap/eRedCap Half-Duplex FDD UE which supports pre-compensation of TA for UL transmission in NR NTN can transmit the TA report via MAC-Control Element in Msg5 and in RRC connected if variation of TA is larger than offsetThresholdTA in IE TAR-Config based on its capability. The RRC parameter offsetThresholdTA-r17 is configured by RRC signalling and can have value in {ms0dot5, ms1, ms2, ms3, ms4, ms5, ms6 ,ms7, ms8, ms9, ms10, ms11, ms12, ms13, ms14, ms15}. The UE-specific TA report can be used by the gNB to configure a smaller UE-specific Koffset to optimize UL scheduling operations. A smaller value than 0.5 ms for offsetThresholdTA-r17 would require a TA report granularity smaller than 1 ms. The benefits for this should be clarified as a smaller value of offsetThresholdTA-r17 increases the signalling overhead and results in triggering more frequent TA reports.

We summarise the existing Release 17 specified TA report MAC CE and Report Trigger mechanisms:

- RA procedure (initial access, RRC Re-establishment, Handover)

- Event-trigger (variation of TA larger than an offsetThresholdTA and config/reconfig offsetThresholdTA)

- SR triggered by a TA report (optional with UE capability) when there is no available SR resource.

In case UE detects a collision, the simplest way would be for the UE to trigger a TA report if it supports this capability. This would be a small change in the specifications. However the gNB would be unable to determine that the reported TA is triggered by a collision detection or other events.

Observation 1: A smaller value than 0.5 ms for offsetThresholdTA-r17 requires a TA report granularity smaller than 1 ms, increases signalling overhead and may not improve slot-based scheduling since the gNB schedules resources on a slot basis.

Observation 2: The network may not be able to determine whether a RedCAp UE triggered a Timing Advance report if a collision is detected by the UE or some other events.

# 5 Conclusion

In this contribution, the following observations and proposals were made:

HD collision rules Case 4:

Proposal 1:  Handling of PDCCH ordered PRACH transmission is up to device implementation.

TA reporting:

Observation 1: A smaller value than 0.5 ms for offsetThresholdTA-r17 requires a TA report granularity smaller than 1 ms, increases signalling overhead and may not improve slot-based scheduling since the gNB schedules resources on a slot basis.

Observation 2: The network may not be able to determine whether a RedCAp UE triggered a Timing Advance report if a collision is detected by the UE or some other events.

# 6 References

- [1]	RP-243300, Moderator (Thales), New WID: Non-Terrestrial Networks (NTN) for NR Phase 3, RAN#106, Madrid, Spain, December 9-12, 2024.

- R1-2403051, Moderator (CATT), Summary #3 for Support of RedCap and eRedCap UEs with NR NTN operating in FR1-NTN bands, RAN1#120bis, Wuhan, China, April 7th – 11th, 2025

# 7 Appendix

TS 38.300 Section 16.14.2.1 Scheduling and Timing

DL and UL are frame aligned at the uplink time synchronization reference point (RP) with an offset given by NTA,offset (see clause 4.2 of TS 38.213 [38]).

To accommodate the propagation delay in NTNs, several timing relationships are enhanced by a Common Timing Advance (Common TA) and two offsets Koffset and kmac:

-	 Common TA is a configured timing offset that is equal to the RTT between the RP and the NTN payload.

-	Koffset is a configured scheduling offset that needs to be larger or equal to the sum of the service link RTT and the Common TA.

-	kmac is a configured offset that is approximately equal to the RTT between the RP and the gNB.

The scheduling offset Koffset is used to allow the UE sufficient processing time between a downlink reception and an uplink transmission, see TS 38.213 [38].

The offset kmac is used to delay the application of a downlink configuration indicated by a MAC CE command on PDSCH, see TS 38.213 [38], and in estimation of UE-gNB RTT, see TS 38.321 [6]. It may be provided by the network when downlink and uplink frame timing are not aligned at gNB. The kmac is also used in the random access procedure, to determine the start time of RAR window/MsgB window after a Msg1/MsgA transmission (see TS 38.213 [38]).

The Service link RTT, Feeder link RTT, RP, Common TA, kmac and TTA (see clause 16.14.2.2) are illustrated in Figure 16.14.2.1-1.

TS 38.300 Figure 16.14.2.1-1: Illustration of timing relationship (for collocated gNB and NTN