3GPP TSG RAN WG1 #121		R1-2504285

St Julian, Malta, May 19th – May 23rd, 2025

Agenda Item:	9.1.2

Source:	Indian Institute of Technology Madras (IITM)

Title:	Discussion on Specification Support of AI/ML for Positioning Accuracy Enhancement

Document for: 	Discussion

# Introduction

This document contains our discussions, observations, and proposals for enhancing the positioning accuracy of AI/ML-based systems. In this document, we specifically address the remaining issues marked as “FFS” for model input.

# Background

During RAN1#120 & RAN1#120 bis, significant progress was made in addressing the challenge of channel measurement to obtain model inputs. In the upcoming RAN1#121, several open issues will be highlighted for discussion. The following are the open issues that are still under discussion for the RAN1 #121 meeting [1].

# Signalling and measurement enhancements: Feedback on the suboptimal {Nt, Nt’, k} setting:

Agreement

For Rel-19 AI/ML based positioning, for Case 3b, “FFS: k” in RAN1#119 agreement is resolved by supporting:

- k = {0...5}

Agreement

For Rel-19 AI/ML based positioning, for Case 3b, “FFS: Nt' values” in RAN1#119 agreement is resolved by supporting:

- Nt' = {8, 16, 24}

In RAN1#120, the following agreements were made to clarify the path-based measurements and facilitate sample-based reporting.

Agreement

For AI/ML based positioning Case 3b, with the existing definition of two channel measurement types (A) or type (B):

(A) path-based measurement, i.e., measurement in the existing specifications (up to Rel-18), (B) Rel-19 enhanced measurement (see definition in RAN1#119 agreement for Case 3b).

- Descriptive info of the channel measurement is included together with the channel measurement values, either explicitly or implicitly.

- The descriptive info of the channel measurement includes at least:

  - measurement type (A) or type (B) and

  - measurement parameters.

- Regarding measurement parameters,

  - For (A) path-based measurement, the measurement parameters include: number of additional paths, k.

  - For (B) Rel-19 enhanced measurement, the measurement parameters include at least: Nt', k.

    - FFS: Nt

- Note: Type (A) and Type (B) are used for discuss purpose only.

In this analysis, we will examine whether the Nt value should be reported alongside Nt’ and k.

Observation 1: Impact of reporting Nt

When the {Nt, Nt’, k} is chosen sub optimally, it can result in an incomplete or inaccurate representation of the channel characteristics. The inclusion of Nt (window size) as a configurable parameter in Rel-19 enhanced measurement (Type B) is crucial for the following reasons:

- Nt specifies Delay Spread:

Window size (Nt) determines the level of detail in timing signal paths within the CIR. In regions with many reflections, a larger Nt is needed to capture delayed signals beyond the first detected path. In simpler environments with fewer reflections, a smaller Nt is sufficient. However, if Nt is not specified along with k & Nt’ values, crucial timing details may be missed which will cause under reporting or Overhead Inflation.

  - Under-reporting: A fixed Nt may truncate critical multipath components outside the window, degrading AI/ML model accuracy.

  - Over-reporting: Excessively large static Nt introduces redundant samples increasing reporting overhead by up to 40%.

Observation 2: Impact of reporting Nt along with K on Sampling Efficiency

The parameter k (T = 2^k × Tc) determines the granularity of the sampling rate. Impact of reporting Nt on k will :

- CIR Sampling Interval: Determined by k, affecting the granularity of CIR sampling.

- CIR Reporting Window: Determined by Nt, determining the amount of sampled CIR reported.

- Impact of Nt on k: Reporting Nt along with Nt ensures the CIR window length matches the sampling granularity to avoid aliasing and optimize reporting efficiency.

Observation 3: Linking Nt (samples above a threshold) to Nt’ avoids static thresholds. Methods include:

- Dynamic Threshold Compute: Calculate the threshold as a percentage of the average power within the Nt window.

- This reduces false positives from noise compared to fixed thresholds.

Proposals:

Based on the above observations we propose the following:

Proposal 1: Nt should be reported as one of the enhanced measurement parameters for Rel-19.

Proposal 2: Reporting Nt along with k will avoid aliasing and increase the reporting efficiency.

Proposal 3: Reporting Nt along with Nt’ will avoid and static thresholds.