3GPP TSG RAN WG1 #121	    R1-2504286

St Julian’s, Malta, May19th – 23rd, 2025

Agenda item:	9.3.1

Source: 	Lenovo

Title: 	SBFD TX/RX Measurement Procedures

Document for:	Discussion and Decision

# Discussions on remaining issues

Remaining issue 1: Collision handling in half-duplex CA

In RAN1#120 meeting, the following agreement was reached to support SBFD operation on one TDD carrier in multi-carrier scenario.

Agreement Support SBFD operation on one TDD carrier in multi-carrier scenario. - Note: If the TDD carrier for SBFD operation is an Scell, support UE dedicated signaling for cell-specific configuration of time and frequency location of SBFD subbands for the Scell. - FFS whether/how to handle half-duplex CA case

Regarding the FFS, the existing directional collision handling for half-duplex TDD CA does not consider scenario when at least one cell is configured with SBFD operations, where an UL subband is configured on DL and/or flexible symbols and a SBFD-aware UE can be scheduled with either DL reception or UL transmission on those symbols. For collision handling across cells in half-duplex TDD CA, we support treating SBFD symbols as flexible symbol and reuse the existing directional collision handling, which is a sufficient and a straightforward approach with less spec impact.

- For SBFD operation in half-duplex CA scenario, the existing directional collision handling for half-duplex TDD CA are reused by treating SBFD symbols as flexible symbols.

Remaining issue 2: Wideband PRG of PDSCH

In RAN1#118 meeting, it was concluded that “If PRG is determined as wideband, UE does not expect to be scheduled with non-contiguous PRBs in SBFD symbols”.

Conclusion If PRG is determined as wideband, UE does not expect to be scheduled with non-contiguous PRBs in SBFD symbols.

If prb-BundlingType=staticBundling and ‘wideband PRG’ is configured, the above conclusion implies that only one DL subband can be utilized for PDSCH reception.  On the other hand, if prb-BundlingType=dynamicBundling, the PRG size is determined based on a 1-bit PRB bundling size indicator in a DCI format, the number of scheduled PRBs, and DL BWP size. If PRB bundling size indicator indicates ‘0’, the PRG size is determined from ‘bundleSizeSet2’. Otherwise, the PRG size is determined from ‘bundleSizeSet1’. If ‘bundleSizeSet1’ includes ‘n2-wideband’ or ‘n4-wideband’, then the ‘wideband PRG’ is determined only if the number of scheduled PRBs exceeds half of DL BWP PRBs. With DUD SBFD configuration, even if a PDSCH spans all the PRBs in one DL subband, the ‘wideband PRG’ cannot be used, since the number of PRBs in one DL subband may not exceed half PRBs of DL BWP. To address this issue, the following proposal from the last meeting can be supported.

- Support the following “Proposed Agreement” from RAN1#120bis meeting.

Proposed Agreement: For dynamic determination of the precoding bundling for a PDSCH, when DCI bitfield (bundling size indicator) indicates ‘1’ and first set configured with two values as ‘n4-wideband’ or ‘n2-wideband’, the condition for determining wideband or narrowband depends on whether the number of scheduled PRBs is larger than half of the size of intersection between active DL BWP and DL subband in which the PDSCH is located.

Remaining issue 3: Separate configurations of p-Max

In RAN1#120bis meeting, the following proposal regarding support of separate configurations of p-Max for SBFD and non-SBFD symbols was discussed, but with no agreement. In our view, this proposal is beneficial, and we believe that it is necessary to defined two different maximum power level for SBFD and non-SBFD symbols.

Support separate configurations of p-Max for SBFD and non-SBFD symbols. - Send an LS to RAN4 to inform the agreement and ask whether RAN4 sees any issue.

- Support separate configurations of p-Max for SBFD and non-SBFD symbols.

Remaining issue 4: PDSCH with fdmschemeB

In legacy, when fdmschemeB is configured for a UE, for wideband case, the PDSCH occasion corresponding to first TCI state is located at the first half PRBs of the allocated resource, and the PDSCH occasion corresponding to second TCI state is located at the remaining allocated PRBs. When the allocated resource is overlapping with UL subband, the PRBs number for each TCI state should be decided. For example, the invalid RPBs in UL subband could be excluded firstly, then the PDSCH occasion corresponding to first TCI state is located at the first half valid PRBs of the allocated PRBs, and the PDSCH occasion corresponding to second TCI state is located at the remaining valid RRBs of the allocated PRBs. For PRG(s) with size of 2 and 4, whether the invalid PRG should be indexed to determine the PDSCH occasion corresponding to each TCI state should also be determined.

- For SBFD-aware UEs configured with fdmschemeB and two TCI states for a PDSCH transmission, if the allocated resource overlaps with SBFD UL subband boundary, the actual PRBs for each PDSCH occasion corresponding to each TCI state should be determined.

Remaining issue 5: Radio link monitoring

In Section 5 of 38.213, the radio link monitoring procedures is specified, as copied below, where UE indicates “out-of-sync” to higher layers when the radio link quality is worse than the Qout threshold for all radio link monitoring resources, and “in-sync” when the radio link quality is better than the Qin threshold for any radio link monitoring resource. Similar mechanism is used for beam failure detection.

5 Radio link monitoring [38.213] ----- In non-DRX mode operation, the physical layer in the UE assesses once per indication period the radio link quality, evaluated over the previous time period defined in [10, TS 38.133] against thresholds (Qout and Qin) configured by rlmInSyncOutOfSyncThreshold. The UE determines the indication period as the maximum between the shortest periodicity for radio link monitoring resources and 10 msec. In DRX mode operation, the physical layer in the UE assesses once per indication period the radio link quality, evaluated over the previous time period defined in [10, TS 38.133], against thresholds (Qout and Qin) provided by rlmInSyncOutOfSyncThreshold. The UE determines the indication period as the maximum between the shortest periodicity for radio link monitoring resources and the DRX period. The physical layer in the UE indicates, in frames where the radio link quality is assessed, out-of-sync to higher layers when the radio link quality is worse than the threshold Qout for all resources in the set of resources for radio link monitoring. When the radio link quality is better than the threshold Qin for any resource in the set of resources for radio link monitoring, the physical layer in the UE indicates, in frames where the radio link quality is assessed, in-sync to higher layers.

It has been supported that frequency resource allocation for a configured CSI-RS resource could across downlink subbands for SBFD-aware UEs, and the actual CSI-RS resource allocation with non-contiguous CSI-RS resource is derived by excluding frequency resources outside DL usable PRBs. For a CSI report associated with P/SP CSI-RS, the valid symbol type for CSI derivation for P/SP CSI-RS resources for the CSI report is explicitly configured. Only CSI-RS transmission occasions within the valid symbol types are used for CSI derivation.

For BFD or RLM, similar method could be used for the configured CSI-RS for measurement. Thus, two measurement results corresponding to two symbol type could be derived based on the configured CSI-RS. As for which should be used, there could be two methods. For the first method, only one measurement result should be used, thus gNB could configure the valid symbol type used for measurement and compare the measurement result to the configured Qout. The second method is to use both measurement results, thus gNB could configure separate Qout threshold for SBFD symbols and non-SBFD symbols to account for different radio link quality and interference situations between the two symbol types.

- For SBFD-aware UEs, the valid symbol type used for measurement or separate Q thresholds for SBFD symbols and non-SBFD symbols should be provided for RLM or BFD.

# Conclusions

In summary, we propose the followings for Rel-19 NR subband non-overlapping full duplex operation:

- For SBFD operation in half-duplex CA scenario, the existing directional collision handling for half-duplex TDD CA are reused by treating SBFD symbols as flexible symbols.

- Support the following “Proposed Agreement” from RAN1#120bis meeting.

Proposed Agreement: For dynamic determination of the precoding bundling for a PDSCH, when DCI bitfield (bundling size indicator) indicates ‘1’ and first set configured with two values as ‘n4-wideband’ or ‘n2-wideband’, the condition for determining wideband or narrowband depends on whether the number of scheduled PRBs is larger than half of the size of intersection between active DL BWP and DL subband in which the PDSCH is located.

- Support separate configurations of p-Max for SBFD and non-SBFD symbols.

- For SBFD-aware UEs configured with fdmschemeB and two TCI states for a PDSCH transmission, if the allocated resource overlaps with SBFD UL subband boundary, the actual PRBs for each PDSCH occasion corresponding to each TCI state should be determined.

- For SBFD-aware UEs, the valid symbol type used for measurement or separate Q thresholds (Qout and Qin) for SBFD symbols and non-SBFD symbols should be provided for RLM or BFD.