3GPP TSG-RAN WG1 Meeting #121	R1-2504292

St Julian's, Malta, 19 - 23 May, 2025

Agenda Item:	5

Source:	Huawei, HiSilicon

Title:	Discussion on LS rely on CB-msg3-EDT

Document for:	Discussion and Decision

- Introduction

In RAN2#129bis, RAN2 sent a LS to RAN1 on CB-msg3-EDT [1] with the following agreements.

Agreements: in RAN2#129

1.	RAN2 assumes that at least the following will be part of the shared resources configuration for CB-msg3 (FFS on other aspects)

-	Time domain resources for (N)PUSCH occasions: periodicity and start time (e.g., start subframe, start SFN)

-	Frequency domain resources for (N)PUSCH occasions

-	repetition number

-	(N)PDCCH resource

-	MCS

6.	As Signalling design Baseline RAN2 assumes the PUR config and the NPRACH config for shared (N)PUSCH config can be used and some of the parameters can be included in a new CB EDT config.

Agreements: In RAN2#129bis

4.	We don’t introduce support for eMTC CE mode B case (it will not be possible to signal resources to be used for this case)

5.	We specify support for NB-IoT with 15kHz with no specific enhancements, leaving to NW implementation whether to implement this or not, accepting potential performance degradation.

7.	The start of CB-msg3 EDT transmission window is aligned with the start of time domain (N)PUSCH resource.

8.	The CB-msg3 EDT transmission window length and periodicity may be different. FFS on possible signalling optimization in case the length and periodicity are the same.

9.	RAN2 assumes power ramping should be supported for CB-msg3-EDT (for both eMTC and NB-IoT) should be supported and will ask RAN1 for confirmation and in case which parameters should apply

(CB-Msg3-EDT configuration for eMTC)

10.	For eMTC, introduce a new IE (e.g. CB-Msg3-ConfigSIB-r19) for shared resources configuration of CB-Msg3 in SIB2.

11.	For eMTC, introduce MPDCCH configuration in shared resources configuration. The fields in IE PUR-MPDCCH-Config-r16 could be reused as baseline. Confirm with RAN1 on the detail parameters (e.g. whether additional narrow band is needed).

12.	We will not support TDD related parameters.

13.	For eMTC, introduce PUSCH configuration in shared resources configuration. The fields in IE PUR-PUSCH-Config-r16 could be reused as baseline. Confirm with RAN1 on the detail parameters. (e.g. whether pusch-CyclicShift-r16, pusch-NB-MaxTBS-r16 are needed, whether prb-AllocationInfo should be defined as a “set” format with intention to provide a set of shared frequency-domain resources).

14.	For eMTC, check with RAN1 if anything is needed for PDSCH configuration in shared resources configuration

15.	For eMTC, introduce PUCCH configuration in shared resources configuration. The fields in IE PUR-PUCCH-Config-r16 could be reused as baseline. Confirm with RAN1 on the detail parameters.

(CB-Msg3 configuration for NB-IoT)

16.	For NB-IoT, introduce a new IE (e.g. CB-Msg3-ConfigSIB-NB-r19) for shared resources configuration of CB-Msg3 in SIB2-NB and SIB22-NB for non-anchor carrier.

17.	For NB-IoT, introduce below physical layer parameters in shared resources configuration as below:

-	Number of resource units for NPUSCH (as in npusch-NumRUsIndex-r16)

-	Number of repetitions for NPUSCH (as in npusch-NumRepetitionsIndex-r16)

-	Set of subcarriers (similar to npusch-SubCarrierSetIndex but change it to a “set”), FFS whether subcarriers are provided as a contiguous set.

-	MCS configuration for NPUSCH (as in npusch-MCS-r16).

-	PDCCH parameters (as in NPDCCH-ConfigDedicated-NB-r13)

-	The non-anchor carrier index for monitoring Msg4. If this field is absent, anchor carrier is assumed to be used.

NOTE: confirm with RAN1 is needed

Agreements (part3):

1.	The CB-msg3-EDT configuration (e.g., number of replicas, number of time resources and number of frequency resources) is CE level specific.

Based on the above agreements, RAN2 asks RAN1 8 questions. In the paper, we discussed and provided answers to these questions.

- Discussion

Q1: For both eMTC and NB-IoT, RAN2 assumes power ramping should be supported for CB-msg3-EDT. RAN2 would like to confirm with RAN1 on whether to have power ramping on this Msg3 (re-)transmission. If RAN1 confirms, please provide the control parameters for power ramping.

For both eMTC and NB-IoT, power ramping is for RACH procedure, and not supported for PUR or legacy EDT. Support power ramping for CB-msg3-EDT will increase the complexity. Moreover, CB-msg3-EDT is configured by CE-level, when the initial CB-msg3-EDT is failed, UE can retransmit in higher CE-level or fall back to the legacy 4-step RACH procedure.

Proposed response to Q1: Power ramping for CB-msg3-EDT is not necessary.

Q2: For eMTC, RAN2 agrees to have MPDCCH configuration for shared resource configuration and decides to reuse the parameters from PUR MPDCCH configuration (in IE PUR-MPDCCH-Config-r16, as below) as baseline. RAN2 think TDD parameter is not needed and maybe narrowband configuration should be defined as a set, not one single narrow band. RAN2 would like to check with RAN1 on how to define the detail MPDCCH configuration.

PUR-MPDCCH-Config-r16 ::=		SEQUENCE {

mpdcch-FreqHopping-r16			BOOLEAN,

mpdcch-Narrowband-r16			INTEGER (1..maxAvailNarrowBands-r13),

mpdcch-PRB-PairsConfig-r16		SEQUENCE{

numberPRB-Pairs-r16				ENUMERATED {n2, n4, n6, spare1},

resourceBlockAssignment-r16		BIT STRING (SIZE(4))

},

mpdcch-NumRepetition-r16		ENUMERATED {r1, r2, r4, r8, r16, r32, r64, r128, r256},

mpdcch-StartSF-UESS-r16			CHOICE {

fdd								ENUMERATED {v1, v1dot5, v2, v2dot5, v4, v5, v8, v10},

tdd							ENUMERATED {v1, v2, v4, v5, v8, v10, v20, spare1}

},

mpdcch-Offset-PUR-SS-r16	ENUMERATED {zero, oneEighth, oneQuarter,

threeEighth, oneHalf, fiveEighth,

threeQuarter, sevenEighth}

}

MPDCCH for shared resource configuration is transmitted in the common search space, UEs with CB-msg3-EDT will use the same MPDCCH. The parameters from PUR MPDCCH configuration can be reused, and one single narrow band is enough for MPDCCH configuration, no need to define the narrowband configuration as a set.

Proposed response to Q2: One single narrow band is enough for MPDCCH configuration, no need to define the narrowband configuration as a set.

Q3: For eMTC, RAN2 agrees to have PUSCH configuration for shared resource configuration and decides to reuse the parameters from PUR PUSCH configuration (in IE PUR-PUSCH-Config-r16, as below) as baseline. RAN2 has some questions on some parameters:

- Whether pusch-CyclicShift-r16 and pusch-NB-MaxTBS-r16 are needed

- Whether prb-AllocationInfo should be defined as a “set” format with intention to provide a set of shared frequency-domain resources

Note that:

- CE mode B related parameters will not be included as RAN2 agreed to preclude CE mode B.

- The power related parameters (p0-UE-PUSCH-r16 and alpha-r16) should be updated based on the result of Q1.

RAN2 would like to check with RAN1 on how to define the detail PUSCH configuration.

PUR-PUSCH-Config-r16 ::=		SEQUENCE {

pur-GrantInfo-r16				CHOICE {

ce-ModeA						SEQUENCE {

numRUs-r16						BIT STRING (SIZE(2)),

prb-AllocationInfo-r16			BIT STRING (SIZE(10)),

mcs-r16							BIT STRING (SIZE(4)),

numRepetitions-r16				BIT STRING (SIZE(3))

},

ce-ModeB						SEQUENCE {

subPRB-Allocation-r16			BOOLEAN,

numRUs-r16						BOOLEAN,

prb-AllocationInfo-r16			BIT STRING (SIZE(8)),

mcs-r16							BIT STRING (SIZE(4)),

numRepetitions-r16				BIT STRING (SIZE(3))

}

}	OPTIONAL,	-- Need ON

pur-PUSCH-FreqHopping-r16		BOOLEAN,

p0-UE-PUSCH-r16					INTEGER (-8..7),

alpha-r16						Alpha-r12,

pusch-CyclicShift-r16			ENUMERATED {n0, n6},

pusch-NB-MaxTBS-r16				BOOLEAN,

locationCE-ModeB-r16			INTEGER (0..5)	OPTIONAL – Cond SubPRB

}

eNB can configure different pusch-CyclicShift-r16 for UEs share same TF resource in PUR to avoid the interference between UEs during channel estimation. However, the configuration of CB-Msg3-EDT is not UE specific. It means different UEs in the same frequency-time-domain resource will use same cyclic shift of DMRS. There is no need to keep pusch-CyclicShift-r16 for CB-msg3-EDT.

In PUR, pusch-NB-MaxTBS-r16 is a UE-specific configuration corresponding to UE capability, while PUSCH configuration for shared resource configuration for CB-msg3-EDT is shared among all UEs. Moreover, CB-msg3-EDT is mainly for small-sized traffic, UE can use the legacy 4-step RACH to transmit the large TBS.

Multiple shared frequency-domain resources can be configured in order to provide more access possibilities, define the prb-AllocationInfo as a “set” can be a solution for the PUSCH configuration.

Proposed response to Q3: For eMTC, no need to keep pusch-CyclicShift-r16 and pusch-NB-MaxTBS-r16 for PUSCH configuration. prb-AllocationInfo can be defined as a “set”.

Q4: For eMTC, RAN2 has discussed the PDSCH configuration for shared resource configuration. Some companies think PUR PDSCH configuration is configured according to UE capability, which is not the case for CB-Msg3/Msg4 configuration. So, RAN2 does not know whether the parameters of PUR PDSCH configuration (in pur-PDSCH-FreqHopping and pur-PDSCH-maxTBS within IE PUR-Config-r16) is needed. RAN2 would like to check with RAN1 on whether and how to define the detail PDSCH configuration.

PUR-Config-r16 ::=		SEQUENCE {

<Skip unrelated Part>

pur-PDSCH-FreqHopping-r16		BOOLEAN,

<Skip unrelated Part>

...,

[[	pur-PDSCH-maxTBS-r17		BOOLEAN						OPTIONAL	-- Need ON

]]

}

Both pur-PDSCH-FreqHopping-r16 and pur-PDSCH-maxTBS-r17 are UE-specific configuration corresponding to UE capability for PUR. Since CB-Msg4 targets multiple UEs simultaneously, the PDSCH configuration should work for all UEs. No need to keep these parameters corresponding to UE capability from PUR PDSCH configuration for PDSCH configuration for shared resource configuration.

Proposed response to Q4: For eMTC, no need to keep pur-PDSCH-FreqHopping and pur-PDSCH-maxTBS within IE PUR-Config-r16 for PDSCH configuration for shared resource configuration.

Q5: For eMTC, RAN2 agrees to have PUCCH configuration for shared resource configuration and decides to reuse the parameters from PUR PUCCH configuration (in IE PUR-PUCCH-Config-r16, as below) as baseline. RAN2 would like to check with RAN1 on how to define the detailed PUCCH configuration.

PUR-PUCCH-Config-r16 ::=			SEQUENCE {

n1PUCCH-AN-r16						INTEGER (0..2047)			OPTIONAL,	-- Need ON

pucch-NumRepetitionCE-Format1-r16	ENUMERATED {n1, n2, n4, n8}	OPTIONAL	-- Need ON

}

PUCCH in CB-msg3-EDT is used for HARQ feedback to acknowledge Msg4. However, NW cannot distinguish the different UEs when PUCCH is configured in the shared resource. RAN1 will discuss the details once RAN2 has a scheme on how the HARQ feedback is used for each response in Msg4 when there is multiplexing in Msg4.

Proposed response to Q5: More detail is required from RAN2 on how the HARQ feedback is used for each response in Msg4.

Q6: For NB-IoT, RAN2 agrees to reuse the physical layer parameters pur-PhysicalConfig-r16 in PUR-Config-NB as baseline. RAN2 assumes to have below parameters:

- Number of resource units for NPUSCH (as in npusch-NumRUsIndex-r16)

- Number of repetitions for NPUSCH (as in npusch-NumRepetitionsIndex-r16)

- Set of subcarriers (similar to npusch-SubCarrierSetIndex but change it to a “set”), FFS whether subcarriers are provided as a contiguous set.

- MCS configuration for NPUSCH (as in npusch-MCS-r16).

Note that the power parameters (p0-UE-NPUSCH-r16 and alpha-r16) should be updated according to the result of Q1.

RAN2 would like check with RAN1 on how to define the detail physical layer parameters.

pur-PhysicalConfig-r16				SEQUENCE {

carrierConfig-r16					CarrierConfigDedicated-NB-r13,

npusch-NumRUsIndex-r16				INTEGER (0..7),

npusch-NumRepetitionsIndex-r16		INTEGER (0..7),

npusch-SubCarrierSetIndex-r16		CHOICE {

khz15								INTEGER (0..18),

khz3dot75							INTEGER (0..47)

},

npusch-MCS-r16						CHOICE {

singleTone							INTEGER (0..10),

multiTone							INTEGER (0..13)

},

p0-UE-NPUSCH-r16					INTEGER (-8..7),

alpha-r16							ENUMERATED {al0, al04, al05, al06,

al07, al08, al09, al1},

npusch-CyclicShift-r16				ENUMERATED {n0, n6},

npdcch-Config-r16					NPDCCH-ConfigDedicated-NB-r13

}	OPTIONAL,	-- Need ON

For NB-IoT, npusch-SubCarrierSetIndex can be kept to indicate the set of subcarriers used for CB-msg3-EDT. Multiple subcarriers should be configured in order to provide more access possibilities, change to a “set” can be a solution for the multiple subcarriers configuration.

The power parameters (p0-UE-NPUSCH-r16 and alpha-r16) are no need to be updated since power ramping is not necessary for CB-msg3-EDT, and npusch-CyclicShift-r16 can also be removed similar to eMTC.

Proposed response to Q6: For NB-IoT, set of subcarriers should be contained in physical layer parameters and the value can be changed to a “set”. p0-UE-NPUSCH-r16 and alpha-r16 are no need to be updated, npusch-CyclicShift-r16 can be removed.

Q7: For NB-IoT, RAN2 agrees to have NPDCCH configuration for shared resource configuration and decides to reuse the parameters from PUR PDCCH configuration (in IE NPDCCH-ConfigDedicated-NB-r13, as below) as baseline. RAN2 would like to check with RAN1 on how to define the detail NPDCCH configuration.

NPDCCH-ConfigDedicated-NB-r13 ::=	SEQUENCE {

npdcch-NumRepetitions-r13			ENUMERATED {r1, r2, r4, r8, r16, r32, r64, r128,

r256, r512, r1024, r2048,

spare4, spare3, spare2, spare1},

npdcch-StartSF-USS-r13				ENUMERATED {v1dot5, v2, v4, v8, v16, v32, v48, v64},

npdcch-Offset-USS-r13				ENUMERATED {zero, oneEighth, oneFourth, threeEighth}

}

NPDCCH-ConfigDedicated-NB-v1530 ::=	SEQUENCE {

npdcch-StartSF-USS-v1530			ENUMERATED {v96, v128}

}

For NB-IoT, RAN1 support RAN2 to reuse the parameters from PUR PDCCH configuration as baseline to have NPDCCH configuration for shared resource configuration. The aggregation and repetition levels defining the NPDCCH common search spaces are listed in [2] Table 16.6-2 by substituting the value of Rmax with configured parameter npdcch-NumRepetition, the locations of starting subframe can be obtained by parameters npdcch-StartSF and npdcch-Offset.

Proposed response to Q7: For NB-IoT, RAN1 agree with RAN2 to reuse the parameters from PUR PDCCH configuration as baseline to have NPDCCH configuration.

Q8: For both eMTC and NB-IoT, any other L1 parameters are needed for CB-Msg3-EDT procedure in additional to previous discussion?

Proposed response to Q8: From RAN1 perspective, no other L1 parameter is needed for CB-Msg3-EDT procedure for eMTC and NB-IoT based on the current RAN2 agreements.

- Conclusion

Based on the discussion above, we had following proposals:

Proposed response to Q1: Power ramping for CB-msg3-EDT is not necessary.

Proposed response to Q2: One single narrow band is enough for MPDCCH configuration, no need to define the narrowband configuration as a set.

Proposed response to Q3: For eMTC, no need to keep pusch-CyclicShift-r16 and pusch-NB-MaxTBS-r16 for PUSCH configuration. prb-AllocationInfo can be defined as a “set”.

Proposed response to Q4: For eMTC, no need to keep pur-PDSCH-FreqHopping and pur-PDSCH-maxTBS within IE PUR-Config-r16 for PDSCH configuration for shared resource configuration.

Proposed response to Q5: More detail is required from RAN2 on how the HARQ feedback is used for each response in Msg4.

Proposed response to Q6: For NB-IoT, set of subcarriers should be contained in physical layer parameters and the value can be changed to a “set”. p0-UE-NPUSCH-r16 and alpha-r16 are no need to be updated, npusch-CyclicShift-r16 can be removed.

Proposed response to Q7: For NB-IoT, RAN1 agree with RAN2 to reuse the parameters from PUR PDCCH configuration as baseline to have NPDCCH configuration.

Proposed response to Q8: From RAN1 perspective, no other L1 parameter is needed for CB-Msg3-EDT procedure for eMTC and NB-IoT based on the current RAN2 agreements.

# References

[1] R2-2503175, LS on CB-msg3-EDT, April, 2025.

[2] 3GPP TS 36.213 Evolved Universal Terrestrial Radio Access (E-UTRA); Physical layer procedures .