3GPP TSG RAN WG1 #121                                                                     R1- 2504313

St Julian’s, Malta, May 19th – 23th, 2025

Agenda Item:	9.2.3

Source:	Apple Inc.

Title:	Remaining issues on R19 MIMO 3Tx transmission

Document for:	Discussion/Decision

# Introduction

In RAN#102, a new work item was endorsed in RP-234007, to specify NR MIMO Phase 5 in Rel-19, where one of the objectives is related to a 3Tx UE, later updated in RP-242394, as follows:

- Specify non-coherent UL codebook to facilitate 3-antenna-port codebook-based transmissions, enhancement(s) to enable 3T6R SRS antenna switching, as well as UE capability signaling for 3T3R antenna switching and 3-antenna-port non-codebook-based transmissions, without enhancement on UL full power transmission and without enhancement on SRS resource

Note: UL full power transmission mode 1 and 2 are not supported.

Note: Other than UE capability signaling, no other enhancement is specified for 3T3R SRS antenna

switching.

In this paper, we share our views on remaining issue related to Rel-19 MIMO 3Tx UE.

# TP for 38.214 Section 6.2.1

In RAN1#119, it was agreed that

Agreement

For a UE operating with 3TX,

- the RRC parameter for enabling a configured 4-port SRS resource with port 1003 disabled, is applied for all SRS resource set(s) for SRS usage set as codebook.

- the RRC parameter for enabling a configured 4-port SRS resource with port 1003 disabled, is applied for all SRS resource set(s) for SRS usage set as antenna switching.

- Note: The RRC parameter mentioned in the above two bullets is configured per set

A UE does not expect an SRS resource to be configured in both codebook and antenna switching resource sets with conflicting (disabled state) configuration for the port 1003.

The above agreement is captured currently in 38.214 Sec. 6.2.1 as follows

- The UE is not expected to be configured with an SRS resource which is in both SRS-ResourceSet with usage ‘codebook’ and ‘antennaSwitching’ and have different configuration for higher layer parameter [RRC parameter].

The above text sounds like the RRC configuration is applied at resource level, while based on the agreement made in RAN1#119, application of the RRC parameter is at resource set level. To resolve this ambiguity, we propose the following with modification in red.

Proposal 1: Adopt the following TP in 38.214, Sec. 6.2.1, for R19

The UE is not expected to be configured with an SRS resource which is in both SRS-ResourceSet with usage ‘codebook’ and ‘antennaSwitching’ and the two SRS-ResourceSet with usage ‘codebook’ and ‘antennaSwitching’ have different configuration for higher layer parameter [RRC parameter].

# Conclusion

In this contribution, we shared our views on remaining issues regarding R19 MIMO 3Tx UL transmission. The following proposal is made:

Proposal 1: Adopt the following TP in 38.214, Sec. 6.2.1, for R19

The UE is not expected to be configured with an SRS resource which is in both SRS-ResourceSet with usage ‘codebook’ and ‘antennaSwitching’ and the two SRS-ResourceSet with usage ‘codebook’ and ‘antennaSwitching’ have different configuration for higher layer parameter [RRC parameter].

# References

- RP-243265, “Way forward on REL-19 MIMO 3T3R – Offline Discussions”, RAN1 Chair (Samsung), Madrid, Spain, Dec. 2024