3GPP TSG RAN WG1 #121	                                         	           R1-2504331

St Julian’s, Malta, May 19th – 23rd, 2025

Agenda Item:	9.6.2

Source:	Moderator (Apple)

Title:	Summary #1 on LP-WUS operation in IDLE/INACTIVE mode

Document for:	Discussion/Decision

# Introduction

One of the objectives in Rel-19 WI on LP-WUS/WUR (RP-241824) is to support LP-WUS operation in idle/inactive mode:

- For IDLE/INACTIVE modes - Specify procedure and configuration of LP-WUS indicating paging monitoring triggered by LP-WUS, including at least configuration, sub-grouping and entry/exit condition for LP-WUS monitoring (RAN2, RAN1, RAN3, RAN4)

This contribution summarizes the discussions on LP-WUS operation in IDLE/INACTIVE modes in RAN1#121.

Section 2 captures the agreements as the outcome of RAN1#121 discussions. Section 3 captures the proposals for online sessions. Section 4 documents the detailed discussions. Section 5 provides the list of remaining issues for the maintenance discussion. Agreements from previous meetings are captured in the Appendix.

# RAN1#121 Agreements

# Proposals for Online Sessions

## Proposals for Monday Online

Proposal 8-1

The number of MOs per beam per LO is configured by gNB, with candidate value set of {1, 2, 3, 4}.

Down-select between proposal 4-1 and 4-2 (down-select between Alt 1 and Alt 2):

Proposal 4-1

For LP-SS, the LP-SS occasions are indexed sequentially in time, and the n-th LP-SS occasion is associated with the beam of the n-th transmitted SSB, n = 1, 2, …, N, where N is the number of actual transmitted SSBs determined according to ssb-PositionsInBurst in SIB1.

The RAN1#120bis agreement is updated as follows:

For LP-WUS, the N * M LP-WUS MOs in an LO are indexed sequentially in time, from 1 to N*M, where N is the number of actual transmitted SSBs determined according to ssb-PositionsInBurst in SIB1 the number of beams corresponding to LP-WUS, and M is the number of LP-WUS MOs for each beam.

- The (n*M+m+1)-th LP-WUS MO corresponds to the (n+1)-th beam, which is associated with the beam of the (n+1)-th transmitted SSB, where m=0,1,…,M-1, n=0,1,2,…,N-1. (multiple MOs first, beam second)

Proposal 4-2

It can be configured by the gNB in the LP-WUS/LP-SS configuration on whether LP-WUS/LP-SS transmission is present or not for each beam of the actual transmitted SSBs determined according to ssb-PositionsInBurst in SIB1.

- The detailed signaling design is up to RAN2

- For the LP-WUS MO and LP-SS occasion determination,

  - Alt 1: S is the number of actual transmitted SSBs determined according to ssb-PositionsInBurst in SIB1, and LP-WUS MOs and LP-SS occasions are determined assuming S beams. gNB transmits LP-WUS/LP-SS in all or a subset of the S beams according to the LP-WUS/LP-SS configuration.

  - Alt 2: N is the number of beams for LP-WUS/LP-SS, which is determined by the LP-WUS/LP-SS configuration. LP-WUS MOs and LP-SS occasions are determined assuming N beams.

Proposal 2-1r1

For PO-to-LO association and codepoint determination, assume NPOLO is the number of POs associated with a LO, NSGPO is the number of subgroups per PO for LP-WUS, and  iSG is the subgroup ID of a UE (0≤iSG<NSGPO).

- The PO index within the LO is defined as

iPO=UE_ID mod N∙Ns+is mod NPOLO,

- The reference PF for the LO of a PO is provided by (SFN for PF) – floor(iPO/NS) * T/N, which is the first PF of the PF(s) associated with the LO.

- For the codepoints,

  - If NSGPO>1,

    - Alt 1: (the last few codepoints are common codepoints)

      - The codepoint for subgroup iSG in PO iPO is iPO*NSGPO+iSG.

      - The codepoint for all the subgroups in PO iPO is NPOLP-WUS*NSGPO+iPO.

    - Alt 2: (the codepoints for each PO are consecutive)

      - The codepoint for subgroup iSG in PO iPO is iPO*(NSGPO+1)+iSG.

      - The codepoint for all the subgroups in PO iPO is (iPO+1)*NSGPO+1.

    - Alt 3: (PO index + subgroup index)

      - The PO index is indicated in the K1 MSBs and  the subgroup index is indicated in the K2 LSBs, where  K1=log2NPOLO , and K2=log2NSGPO+1.

      - The common codepoint for all the subgroups in a PO is represented by all the K2 bits being ‘1’s.

  - If NSGPO=1, the codepoint for PO iPO is iPO.

- Here UE_ID (for LP-WUS), N, NS, is and T are defined in TS 38.304

Proposal 6-1

Each LP-SS transmission for each beam is contained within one slot.

# LP-WUS Operation in Idle/Inactive Mode

## LP-WUS MO configuration

For the determination of available symbols for LP-WUS and LP-WUS MO configuration, the following was agreed:

Agreement

UE determines whether a symbol is available for LP-WUS based on:

- Alt 1: Time-domain pattern configured by the gNB

  - Alt 1A: Periodic time-domain pattern

    - E.g. (1) 1-slot periodicity, the pattern indicates the available symbols in each slot; (2) multi-slot or frame-level periodicity with a bitmap indication; (3) Search space set-like pattern; (4) multi-level time-domain patterns; (5) reuse the mechanism of rate matching pattern

  - Alt 1B: Per-MO pattern, applicable for all Mos

- Alt 2: Information from existing configurations available for idle/inactive UEs such as [SSB, CORESET/Type-0 CSS, TDD DL/UL configuration, etc].

- Alt 3: Combination of Alt 1 and Alt 2

- Alt 4: NW ensures LP-WUS configuration without collision with existing signal(s)

Agreement

For the determination of starting time locations of LP-WUS MOs and LP-WUS transmissions in a LO,

- A reference point is the start of a reference frame determined by the frame-level offset from the start of the first PF of the PF(s) associated with the LO.

- The starting time location of the first LP-WUS MO in a LO is indicated by an offset w.r.t. the reference point.

  - FFS slot-level or symbol-level offset

- The starting time locations of the subsequent LP-WUS MOs in a LO are determined based one of the following alternatives:

  - Alt 1: An offset is indicated for each of the subsequent LP-WUS MOs.

    - FFS slot-level or symbol-level offset

  - Alt 2: The start time location of a subsequent LP-WUS MO is determined implicitly at least based on the previous LP-WUS MO.

    - FFS additional configuration to control the subsequent MO locations, e.g.,

      - Alt 2A: configuration of a single gap between the end of the previous MO (or a set of previous MOs) and the start of the next MO

      - Alt 2B: configuration of candidate starting locations for MOs, similar to search space configuration

  - FFS restriction on MO locations, e.g. only on DL slots

  - FFS minimum gap is needed between two MOs to ensure LR processing time

Agreement

Terminology definition

- Nominal MO duration: this includes both the available and unavailable symbols

- Actual LP-WUS duration: the actual number of OFDM symbols used for LP-WUS transmission as assumed by the UE

Nominal MO duration and actual LP-WUS duration, if defined, are determined using one of the following alternatives:

- Alt A: Nominal MO duration is configured. (e.g. in unit of slots)

  - Actual LP-WUS duration is the number of available symbols within the MO.

  - Actual LP-WUS duration can vary from one MO to another MO.

- Alt B: Actual LP-WUS duration is configured. (e.g. in unit of OFDM symbols, or M and L values)

  - From the start of a MO, MO extends until the number of available OFDM symbols reaches the configured actual LP-WUS duration.

    - FFS: Additional termination condition such as time window

- Alt C: Both nominal MO duration and actual LP-WUS duration are configured.

  - If the number of available OFDM symbols within the nominal MO duration is less than the actual LP-WUS duration, the MO is considered as invalid (no LP-WUS monitoring in this MO) and dropped/deferred.

    - FFS UE behavior if there is no valid MO for the beam(s) that the UE monitors

  - Note: the number of available OFDM symbols within a nominal MO duration can be different for different MOs.

- Alt D: Nominal MO duration is configured. The actual LP-WUS duration is determined based on the same pattern for the available symbols for all the MOs (e.g. by using a per-MO pattern), which is the same for all the MOs.

Here is a summary of companies’ views on the alternatives in the above agreements:

Determination of available symbols for LP-WUS:

- Alt 1: [3] ZTE, [7] TCL, [11] xiaomi (?), [16] ETRI, [19] LG (can be considered in addition to Alt 2), [21] Apple

  - Alt 1A: [16] ETRI, [11] xiaomi, [19] LG, [21] Apple

    - [11] xiaomi: The mechanism of rate matching pattern is reused to determine whether a symbol is available for LP-WUS.

    - [19] LG: For Alt. 1A, re-use the structure of RateMatchPattern or invalidSymbolPattern

    - [21] Apple: a periodic slot-level pattern indicating which slots are available and a symbol-level pattern for a slot indicating which consecutive symbols are available in a slot.

  - Alt 1B (per-MO pattern): [3] ZTE

- Alt 2: [6] Samsung, [13] InterDigital, [15] Sony, [17] OPPO, [19] LG, [23] Sharp, [25] Nordic

  - [23] Sharp: UL slot and SSB resource collisions

  - [25] Nordic: TYPE#0/TYPE#2 CSS and associated CORESET(s), and transmitted SSBs indicated in SIB1 on a camping cell; UL slot

- Alt 3 (Alt 1 + Alt 2): [1] Futurewei, [2] HW/HiSi, [4] vivo (Alt 1A), [8] Nokia, [10] CMCC (Alt 1A), [13] InterDigital (can consider), [14] Ericsson (?), [18] Panasonic, [22] QC, [24] DCM (Alt 1A)

  - Alt 1

    - [1] Futurewei: a time-domain pattern (i.e., Alt 1B: Per-MO pattern, applicable for all MOs)

    - [2] HW/HiSi: The time-domain pattern is a periodic time domain pattern (i.e. Alt. 1A). The pattern can reuse the mechanism of rate matching pattern.

    - [4] vivo: For Alt 1A, support to partially reuse the mechanism of rate matching pattern (i.e., reuse bitmaps including symbolsInResourceBlock and periodicityAndPattern).

    - [8] Nokia:

      - Use default PDSCH TDRA table to indicate the available symbols in a slot(s) (in a MO) for LP-WUS. FFS whether this would be common indication for all slots, or slot/MO specific indication.

      - To support indication of TRS resource, consider re-using the resource indication approach defined for Rel-17 IDLE mode TRS. FFS whether this would be common indication for all slots, or slot/MO specific indication.

    - [14] Ericsson:

      - Bitmap1 (slot-level bitmap) indicating whether slots in a number of frames (N) are available for LP-WUS MOs. The slot-level bitmap repeats every N frames. The value of N can be 4.

      - Bitmap2 (symbol-level bitmap per MO) indicating whether symbols in a MO are available for LP-WUS. Bitmap2 is the same for every MO.

    - [18] Panasonic: A configured pattern with parameters of monitoringSlotPeriodicityAndOffset, duration and monitoringSymbolsWithinSlot, which is similar and compatible with PDCCH search space.

    - [22] QC: based on mechanism of NR rate matching pattern.

  - Alt 2

    - [2] HW/HiSi: The existing configuration includes SSB, Type-0 CSS, TDD DL/UL configuration

    - [4] vivo: For Alt 1A, support to partially reuse the mechanism of rate matching pattern (i.e., reuse bitmaps including symbolsInResourceBlock and periodicityAndPattern).

    - [8] Nokia: At least Type0-CSS and CORESET#0 configuration

    - [10] CMCC: Further consider the case when the time gap between LP-WUS occasion and DL/UL transmission is smaller than a threshold/predefined value.

    - [14] Ericsson: If a RE of any available symbol of a MO overlaps with a SSB/LP-SS RE, the UE may skip LP-WUS monitoring for that MO.

    - [18] Panasonic: Additional limitations within a time window before SSB; Additional limitations within only DL symbols/slot.

    - [22] QC: Determination of available symbols for LP-WUS transmission should be based on simple rules, e.g., if an existing NR signal is considered as higher priority than LP-WUS, LP-WUS is not transmitted in symbols configured for the existing signal irrespective of the frequency gap between the existing signal and LP-WUS.

- Alt 4: [15] Sony

Nominal MO duration and actual LP-WUS duration

- Alt A (nominal with variable actual): [1] Futurewei (allow MO dropping), [5] Spreadtrum, [13] InterDigital, [16] ETRI, [19] LG

- Alt B (actual): [2] HW/HiSi (with max nominal LO duration), [6] Samsung, [7] TCL, [9] CATT, [10] CMCC, [15] Sony, [17] OPPO, [18] Panasonic, [19] LG (with a time window for termination per MO or per LO), [21] Apple (with a max nominal MO duration), [24] DCM (with max gap between available symbols)

- Alt C (nominal + actual): [4] vivo, [11] xiaomi, [22] QC, [23] Sharp, [25] Nordic

- Alt D (nominal + per-MO pattern): [3] ZTE, [14] Ericsson

Starting time location of the first MO

- Slot-level offset: [1] Futurewei, [4] vivo, [6] Samsung, [21] Apple, [22] QC, [24] DCM, [25] Nordic

- Symbol-level offset: [2] HW/HiSi, [5] Spreadtrum, [7] TCL (both slot-level and symbol-level), [13] InterDigital, [14] Ericsson (both slot-level and symbol-level), [15] Sony, [17] OPPO

Starting time location of the subsequent MOs

- Alt 1 (explicit): [9] CATT (symbol-level), [13] InterDigital (symbol-level), [16] ETRI, [22] QC

- Alt 2 (implicit): [1] Futurewei, [2] HW/HiSi, [3] ZTE (?), [4] vivo, [5] Spreadtrum, [6] Samsung, [7] TCL, [8] Nokia (?), [10] CMCC, [11] xiaomi, [14] Ericsson (no additional configuration), [15] Sony, [18] Panasonic

  - Alt 2A (gap): [1] Futurewei, [6] Samsung, [7] TCL(?), [10] CMCC, [11] xiaomi, [18] Panasonic

  - Alt 2B: [3] ZTE (? use search space like pattern to determine the (nominal) MO location), [8] Nokia (?), [17] OPPO, [21] Apple

    - [3] ZTE: Alt D (nominal) + search space like pattern

    - [8] Nokia: Consider search space style MO starting symbol/slot pattern definition, where period (with offset) and duration (number of slots/MOs) is used to determine the pattern of candidate MO locations. Based on collision e.g. with UL slots, candidate MO locations can be determined as valid or invalid.

- [22] QC: LP-WUS detection/processing time is determined by RAN4.

  - Define UE capability for the time gap between MOs with at least two candidate values. One value corresponds to the concurrent reception and processing of LP-WUS from different MOs and the other value corresponds to the sequential reception and processing of LP-WUS from different MOs.

For the available symbols for LP-WUS, the view is very diverse. The following is proposed as the starting point for discussion.

Proposal 1-1

UE determines whether a symbol is available for LP-WUS based on combination of Alt 1 and Alt 2:

- Alt 1: Time-domain pattern configured by the gNB

  - Alt 1-1: reuse the rate matching pattern

  - Alt 1-2: a periodic slot-level availability pattern and a symbol-level availability pattern

  - Alt 1-3: a periodic slot-level availability pattern and a per-MO time domain pattern is configured

  - …

- Alt 2: The following symbols are not available for LP-WUS [if there is overlap in frequency domain]

  - [SSB]

  - [CORESET/Type-0 CSS]

  - [UL slots/symbols]

| Company | Comments |
| --- | --- |
| Qualcomm | Support Alt 1-1 and Alt 2 |
| CMCC | Support Alt 1-2 and Alt 2 |
| vivo | Support alt 1-1 and alt 2. |
| Xiaomi | Support Alt 1-1 and Alt 2 |
| Huawei, HiSilicon | Support this direction. For alt 2, also consider the LP-SS resource, which should also have higher priority than LP-WUS. But LP-SS resource may be considered in a second step after available resources for LR are determined. |
| Samsung | Support Alt2. For Alt 1, the first issue we need to discuss is whether the group-cast LP-WUS config in SIB is low priority than unicast signals/channels or not. In legacy, we have never had a signal or channel config in SIB with low priority than unicast signals/channels. If the answer is ‘yes’, we are open for ‘Alt 1’, but the solutions depend on how many/which kind of unicast signals/channels is high priority than LP-WUS, and then, we can check the feasibility of each option under Alt 1 due to the different periodicity of the unicast signals/channels. |
| CATT | In Alt1, LP-WUR will calculate the available symbols based on the time-domain patterns, more power consumption will be needed for both Alt1 and Alt2. We prefer Alt4. |
| Nokia1 | We see also that both approaches are needed. For Alt1 solution we considered using the default TDRA (16 values) to indicate the symbol level availability pattern, to limit the size of the configuration needed and accounting that system already for broadcast delivery needs to work with this. In my reading the details for the symbol and per-MO time domain pattern configurations are still to be further discussed i.e. what is the approach to provide the symbol level pattern (at slot/MO granularity). On related question, some companies noted that they would not prefer to support slots where available symbols are sparce (e.g. two SSBs). Considering slot with PDCCH and TRS, would there be any limitation in this perspective for the symbol level pattern? |
| FW | In Alt 1, our understanding is that per-MO time domain pattern configuration may be sufficient if combined with [UL slots/symbols] in Alt 2. |
| InterDigital | Fine with the direction and support Alt 1-1 for Alt 1. |
| DCM | Support Alt 1-1 or Alt 1-2 and Alt 2 |
| ZTE, Sanechips | Based on alt1, the WUR complexity due to alt2 can be reduced. Based on alt2, the gNB configuration is more flexible and the signaling overhead can be reduced. We think only one of two options is needed. We do not need to specify two alternatives. If alt 1 can converge to a specific solution, we could be flexible with alt1 and alt2 together. Otherwise, we only support alt2 or alt1 for WI completion. |
| Sharp | Support Alt2, the LR should only be required to handle limited process. |
| OPPO | Prefer Alt 2. |
| Spreadtrum | We are fine with the proposal. |

For the nominal MO duration and the actual LP-WUS duration, the view is diverse among companies. But there seems to be more support for Alt B. Defining a maximum nominal MO duration may also be able to address some concerns from the proponents of the other options. The following proposal based on Alt B is used as a starting point for discussion.

Proposal 1-2

The actual LP-WUS duration is configured (Alt B).

- From the start of a LP-WUS MO, MO extends until the number of available OFDM symbols reaches the configured actual LP-WUS duration.

- The maximum nominal MO duration is configured.

  - A MO is [deferred] if the nominal MO duration would exceed the maximum nominal MO duration.

| Company | Comments |
| --- | --- |
| Qualcomm | Alt B with maximum nominal MO duration is similar to Alt C. We prefer Alt C as the configured nominal MO durations have same duration in time, but we can accept Alt B. |
| CMCC | Support |
| vivo | We support altC, since it is more straightforward to determine a LP-WUS MO. If the maximum nominal MO duration also needs to be configured for altB, we don’t think there is difference on signalling overhead compared to alt C. Besides, one questions for the proponent of alt B, how to deal with the start time of an actual LP-WUS duration after there is an invalid MO which exceeds the maximum nominal MO duration? It should be solved otherwise alt B is incomplete. |
| Xiaomi | Not support Alt B. To reduce UE complexity, we support the case when the nominal MOs has the same length of duration. Alt C can be supported, with the assumption that the nominal MOs has the same length of duration. |
| Huawei, HiSilicon | We support Alt B. However, instead of configuring a max nominal MO duration, we suggest to configure a maximum nominal LO duration. The the above figure can be found in our tdoc, where the max nominal MO duration is 8. In this case, due to the number of available symbols in MO#3 is not enough, MO#3 is dropped, while there are still some unused available symbols in the end of the current LO (before the next LO). The unused available symbols cannot be efficiently utilized, resulting in only 3 available MOs To solve this issue, we suggest to configure a max nominal LO duration, which provide a resource pool for all the MOs. Specifically, the Y available symbols within the max nominal LO duration can be equally allocated to each beam. For example, if the actual LP-WUS duration is L, and there are N beams, then there can be floor(floor(Y/N)/L) available MOs for each beam. |
| Samsung | Support in general. In our understanding, configure maximum nominal MO duration in proposal 1-2 is different with Alt C since Alt C require to define a nominal MO duration explicitly, and the UE need to compare the number of available OFDM symbols within the nominal MO duration with the actual LP-WUS duration to check whether the MO is valid or not. But in proposal 1-2, UE do not need to check the validation of a nominal MO, which we think is an over design for Alt C, UE only need to check if the actual LP-WUS duration exceed a maximum nominal MO duration, the LP-WUS transmission should be deferred to avoid this issue. To vivo, the issues can be solved by several options, for example, the starting time of an actual LP-WUS duration can be determined by the end of the maximum nominal MO duration and/or a predefined offset. |
| CATT | Support the proposal. |
| Nokia1 | Agree that actual LP-WUS duration needs to be determined. The need for the maximum or nominal MO duration depends on the approach we choose for the MO pattern determination (Proposal 1-4), thus the need should be further considered with the noted dependency. |
| FW | Our understanding is that if a per-MO pattern is considered in Proposal 1-1, this already implies that a nominal MO is defined/configured, which, from our perspective, seems simpler to determine the starting time locations of initial and subsequent MOs in an LO. In our view, the current proposal is very similar to Alt C with the difference that an LO duration may be varying and the spacing between subsequent MOs is not consistent. From that perspective, Alt C might be more preferred than the current proposal. |
| DCM | Support. Alt B is better than Alt C in perspective of NW overhead. Alt C would not be accepted when NW resource is limited. Alt A and Alt D would not work. But one concern on Alt B is if there is a large gap between available symbols (by long time duration configuration/signals such as TDD UL configuration and Paging PDSCH), feasibility on LP-WUS detection is unclear. Thus, we propose FFS for constraint on Alt B. One example constraint on Alt B is as following as our tdoc: - Support maximum gap between available LP-WUS resource. - If consecutive unavailable symbols for LP-WUS among available symbols for LP-WUS within MO is larger than the maximum gap, LP-WUS is not transmitted before the unavailable symbols within MO |
| ZTE, Sanehips | A nominal MO and actual LP-WUS can be configured. What is maximum nominal MO duration? Does it mean there is a nominal MO duration less than the maximum one? |
| Sharp | We agree the actual LP-WUS duration should be configured, but if no other restriction, UE need to derive each LP-WUS MO position for each paging cycle separately based on collisions in the cycle, it will consume a LR too much memory resources. |
| OPPO | Support the proposal. |

Proposal 1-3

The starting time location of the first LP-WUS MO in a LO is indicated by an offset w.r.t. the reference point, where the offset is:

- Alt 1: a slot-level offset

  - The starting symbol within the slot is determined based on other signaling/rules. FFS details.

- Alt 2: a symbol-level offset

| Company | Comments |
| --- | --- |
| Qualcomm | Support Alt 1. |
| CMCC | Support Alt 1. |
| vivo | Support alt1. Slot-level offset is enough. |
| Xiaomi | Support Alt 1. |
| Huawei, HiSilicon | Support Alt 2, which is the same as PEI. |
| Samsung | Support Alt 1. |
| CATT | Support Alt2. Symbol-level offset is for indicating the exact starting location of a MO. |
| Nokia1 | Support Alt1. Assuming that the availability of symbols is determined at least in symbol level slot level offset would seem sufficient. |
| FW | OK, with preference to Alt 1. |
| InterDigital | Support Alt 2. |
| DCM | Support Alt 1. It would be useful for long time duration configurations/signals such as TDD UL configuration and paging PDSCH. |
| ZTE, Sanechips | If nominal MO is defined based on slots, then slot level offset is needed, instead of symbol level offset, since the actual LP-WUS would define the starting symbol. |
| OPPO | Support Alt2. |
| Spreadtrum | Support Alt 2, which is the same as PEI. |

Proposal 1-4

The start time location of a subsequent LP-WUS MO is determined implicitly at least based on the previous LP-WUS MO.

- Alt 1: No additional configuration

- Alt 2: A single gap, in unit of OFDM symbols, between the end of the previous MO and the start of the next MO is configured.

- Alt 3: Candidate start time locations for LP-WUS MOs are configured. FFS details.

| Company | Comments |
| --- | --- |
| Qualcomm | Alt 2 |
| CMCC | Support Alt 2. |
| vivo | Support alt1, i.e., no gap between adjacent MOs. And for the concern on the LP-WUS processing time, which is at the symbol level rather than at the millisecond level, can be covered by the nominal MO duration. |
| Xiaomi | Support Alt 2, with the assumption that the nominal MOs has the same length of duration. |
| Huawei, HiSilicon | Support this direction, and we prefer Alt 2. Considering that there can be some UEs cannot finish processing parallelly, an OFDM symbol is needed after each LP-WUS transmission. To support this, the configured gap should be no smaller than 1 OFDM symbol. |
| Samsung | Support Alt 2. |
| CATT | May I ask dose the Alt1 in Proposal 1-4 corresponds to the Alt 1: An offset is indicated for each of the subsequent LP-WUS MOs that we agreed last meeting? |
| Nokia1 | In order to facilitate avoiding e.g. UL slots or slots with SSBs we think that some flexibility is preferred. Hence prefer Alt3, while noting that Alt2 with some flexibility e.g. having ‘burst of MO’ separated by a gap could also be considered. Effectively, if LP-WUS is long, per MO gap would need to be configurable. For alt 3, periodicity and slot indication pattern could be considered. |
| FW | We prefer Alt 2 but prefer that the single gap be in units of slots rather than OFDM symbols. |
| InterDigital | Support Alt 2. |
| DCM | Support Alt 1. Collision can be avoided by non-consecutive LP-WUS transmission (available/unavailable symbols for LP-WUS). Gap between MOs is not needed for it. For LP-WUS processing time, when UE needs to detect two consecutive MOs is when LP-WUS repetition. But LP-WUS repetition is performed in MO not across MOs. Thus, Gap between MOs are also not needed for LP-WUS processing time. |
| ZTE, Sanechips | We want to clarify the starting MO and sebsequent MOs are in the same LO. Based on this, Alt1 is preferred to save the SIB1 overhead. |
| Sharp | Alt2 can be supported If nominal MO is supported, otherwise, we prefer alt3. |
| OPPO | If the start of the MO is the first available symbol for the LP-WUS transmission, after the single gap from the end of the previous MO, it may don’t have the available symbol for LP-WUS, i.e. not the start of the next MO. Thus, we prefer Alt 3. |
| Spreadtrum | We prefer Alt 2. |

Proposal 1-5

If the number of POs associated with a LO is less than Ns (the number of POs per PF), down-select from the following alternatives to determine the LP-WUS MOs for the multiple LOs with the same reference PF:

- Alt 1: additional frame-level offset(s) are configured.

- Alt 2: additional slot-level or symbol-level offset(s) are configured for the first LP-WUS MO in the PO(s).

- Alt 3: a frame-level offset is configured for the gap between any two adjacent LOs with the same reference PF.

| Company | Comments |
| --- | --- |
| Qualcomm | Alt 1 |
| CMCC | Support Alt 1 as the most straightforward way. |
| vivo | Support alt 1. For the case PO-to-LO association is smaller than Ns, a list of frame-level offsets needs to be configured, and each frame-level offset value determines the start of a frame for each LO. Different from R17 PEI design which a PEI occasion is only several ms, a LO length can be very large e.g., dozens of ms, lots of signaling overhead e.g., additional dozens of bits can be saved by configured a list of frame-level offsets compared with a list of slot-level offsets. |
| xiaomi | Support Alt1. Similar issue has been discussed in R17 PEI, and Alt1 was adopted at that time. |
| Huawei, HiSilicon | We support Alt2, which is the same way as PEI to solve the same issue. |
| Samsung | Support Alt 1. |
| CATT | Prefer Alt 1. |
| Nokia1 | Alt1 |
| FW | We prefer either Alt 2 or Alt 3. For Alt 2, we assume that it should be “in the LO(s)” instead of the “in the PO(s)”. |
| InterDigital | Support Alt 2. |
| ZTE, Sanechips | The key issue is the overhead, alt3 provides the minimum overhead, which is preferred from our perspective. For alt1, if different frame level offsets are introduced, correspondingly, the slot level offset for different LO also should be applied, since they can not share the same slot level offset. Therefore, for alt1, actually, additional frame-level offset(s) and corresponding slot slot or symbol level offsets are configured also. For alt2, at maximum, 4 additional offset would be introduced. Based on two offsets for different wake-up delay, in total, there would be at least 8 offsets configured, which is too complicated. The reason why we should avoid adopting the same mechanism as PEI is LP-WUS introduce two different offsets for wake-up delay, which actually double the offsets configuration. |
| OPPO | Support Alt2. |

This is a collection of companies’ proposals on the frame-level offset:

| [4] vivo | Proposal 7: For the case PO-to-LO association is smaller than Ns, a list of frame-level offsets needs to be configured, and each frame-level offset value determines the start of a frame for each LO. Proposal 8: A UE applies the (flooris/POtoLO association+1) -th value out of Ns/POtoLO association values of the frame-level offset list, wherein is is the associated PO index of the UE, POtoLO association is the configured number of PO per LO. |
| --- | --- |
| [8] Nokia | Observation 13: To reduce the range of values needed for the PF-LO offset, making the applied value range dependent on the SSB periodicity could be considered, similarly as for the configuration of first paging PDCCH monitoring occasion is dependent on the applied sub-carrier spacing. |
| [9] CATT | Proposal 4: A time gap i.e. Z should be specified for indicating the time gap between the start of two neighbor PFs associated with the LO, the first PF of the PFs associated with the LO can be provided by: (SFN for PF) - floor (ipoWUS/Ns)T/N*Z. where ipoWUS=floor{Y/X}* Ns +i_s, Y=UE_IDmodN⋅NS+i_s, X= (Ns*N)/( NPOWUS/Ns). SFN for PF, T, Ns, and N are determined UE_ID are defined in TS 38.304. Z and NPOWUScan be configured in SIB1. |

## PO-to-LO association and codepoint

There are different proposals on how the codepoints are determined (as shown at the end of this subsection), especially in case that multiple POs are associated with one LO. The following proposal captures 3 alternatives that the moderator considers as representative, with the understanding that there are other alternatives with small variations but no fundamental difference.

Proposal 2-1

For PO-to-LO association and codepoint determination, assume NPOLO is the number of POs associated with a LO, NSGPO is the number of subgroups per PO for LP-WUS, and  iSG is the subgroup ID of a UE (0≤iSG<NSGPO).

- The PO index within the LO is defined as

iPO=UE_ID mod N∙Ns+is mod NPOLO,

- The reference PF for the LO of a PO is provided by (SFN for PF) – floor(iPO/NS) * T/N, which is the first PF of the PF(s) associated with the LO.

- For the codepoint,

  - Alt 1: (the last few codepoints are common codepoints)

    - The codepoint for subgroup iSG in PO iPO is iPO*NSGPO+iSG.

    - The codepoint for all the subgroups in PO iPO is NPOLP-WUS*NSGPO+iPO.

  - Alt 2: (the codepoints for each PO are consecutive)

    - The codepoint for subgroup iSG in PO iPO is iPO*(NSGPO+1)+iSG.

    - The codepoint for all the subgroups in PO iPO is (iPO+1)*NSGPO+1.

  - Alt 3: (PO index + subgroup index)

    - The PO index is indicated in the K1 MSBs and  the subgroup index is indicated in the K2 LSBs, where  K1=log2NPOLO , and K2=log2NSGPO+1.

    - The common codepoint for all the subgroups in a PO is represented by all the K2 bits being ‘1’s.

- Here UE_ID (for LP-WUS), N, NS, is and T are defined in TS 38.304

| Company | Comments |
| --- | --- |
| Qualcomm | Alt 2 |
| vivo | Support alt 1 for the simplicity. Not support alt 3 since the indication of the index of PO is no needed and no benefit to be carried by LP-WUS. |
| Xiaomi | Generally OK with the proposal, and we prefer Alt 2. |
| Huawei, HiSilicon | Either alternative is OK for us, and we think all of them are equivalent to each other technically. Maybe we can list several acceptable alternatives, and leave it to editor’s decision to get a simple way to specify this. |
| Samsung | Support Alt 1. |
| CATT | For the mapping between code point and subgroup, we prefer Alt1, since it has smallest spec impact. However, the PO index within the LO is related with the number of associated PF and whether the associated PF is consecutive and non-consecutive, which does not discuss yet for LP-WUS. The first two sub-bullet in Proposal 2-1 implies that the associated PF for LO is 2 consecutive PFs which is same as Rel-17 PEI. However, the maximum number of associated PF for LO can be 4 e.g. Config#2: Ns=1, NPOLO=4, and these four associated PFs can be consecutive or non- consecutive based on gNB configuration. Thus, the equations for the PO index within the LO and the first PF of the PF(s) associated with the LO will not be same as Rel-17 PEI. For example, when the maximum number of associated PF for LO is 2, and associated PF are non- consecutive with T/N*2 time gap, then the first PF of the PFs associated with the LO should be provided by (SFN for PF) – floor(iPO/NS) * T/N*2. |
| Nokia1 | Fine with the proposal, would lean towards Alt2. A question that we need also account the special case that there is only one sub-group configured and then there would not be need for the common point. |
| FW | Prefer Alt 2 |
| InterDigital | Support Alt 1. |
| ZTE, Sanechips | Based on the following conclusion Conclusion For idle/inactive mode, how to map a UE to a subgroup ID for LP-WUS is left to RAN2 to decide.. This formula iPO=UE_ID mod N∙Ns+is mod NPOLO should be up to RAN2 For the following sentence - The reference PF for the LO of a PO is provided by (SFN for PF) – floor(iPO/NS) * T/N, which is the first PF of the PF(s) associated with the LO. We already agreed on the following, there is no need to further introduce (SFN for PF) – floor(iPO/NS) * T/N to redundantly describe that, which should be up to RAN2. Agreement For the determination of starting time locations of LP-WUS MOs and LP-WUS transmissions in a LO, - A reference point is the start of a reference frame determined by the frame-level offset from the start of the first PF of the PF(s) associated with the LO. - The starting time location of the first LP-WUS MO in a LO is indicated by an offset w.r.t. the reference point. - FFS slot-level or symbol-level offset. For the codepoint indication, alt2 can use the unified formula as following: - The codepoint for subgroup iSG in PO iPO is iPO*(NSGPO+1)+iSG, wherein iSG =NSGPO for common codepoint, otherwise iSG=subgroup ID. 2^n-1 is the number of subgroups Additionally, alt2 and alt3 is the same thing, as following shown - iPO*(NSGPO+1)+iSG - Wherein iPO is indicated via K1=log2NPOLO bits in the MSB bits, iSG is indicated via K2=log2NSGPO+1 LSB bits Therefore, we prefer Alt2 as a unified solution. |
| Sharp | Prefer Alt3 |
| OPPO | Support Alt 3. Actually, the number of bits that gNB needs to indicate for one subgroup is same for all the alternatives. When we discuss the LP-WUS design, we heard the views that if UE could early termination, UE can have more power saving gain. It is also similar for alt3. For Alt3, in some UE implementation for OOK-based receiver, UE could achieve early termination by the detection of PO index. In other hands, Alt2 and alt 3 is similar in our understanding. |
| Spreadtrum | We prefer Alt 1. |
| Moderator | Proposal 2-1r1 For PO-to-LO association and codepoint determination, assume NPOLO is the number of POs associated with a LO, NSGPO is the number of subgroups per PO for LP-WUS, and iSG is the subgroup ID of a UE (0≤iSG<NSGPO). - The PO index within the LO is defined as iPO=UE_ID mod N∙Ns+is mod NPOLO, - The reference PF for the LO of a PO is provided by (SFN for PF) – floor(iPO/NS) * T/N, which is the first PF of the PF(s) associated with the LO. - For the codepoints, - If NSGPO>1, - Alt 1: (the last few codepoints are common codepoints) - The codepoint for subgroup iSG in PO iPO is iPO*NSGPO+iSG. - The codepoint for all the subgroups in PO iPO is NPOLP-WUS*NSGPO+iPO. - Alt 2: (the codepoints for each PO are consecutive) - The codepoint for subgroup iSG in PO iPO is iPO*(NSGPO+1)+iSG. - The codepoint for all the subgroups in PO iPO is (iPO+1)*NSGPO+1. - Alt 3: (PO index + subgroup index) - The PO index is indicated in the K1 MSBs and the subgroup index is indicated in the K2 LSBs, where K1=log2NPOLO , and K2=log2NSGPO+1. - The common codepoint for all the subgroups in a PO is represented by all the K2 bits being ‘1’s. - If NSGPO=1, the codepoint for PO iPO is iPO. - Here UE_ID (for LP-WUS), N, NS, is and T are defined in TS 38.304 |

Collection of companies proposals:

| [2] Huawei/HiSi | - If gNB configures X POs mapped to one LO, and Y subgroups per PO, for the determination of codepoint value, down-select between the following options - Option 1: - For a subgroup whose subgroupID = y and relative PO index = x, the codepoint value of this subgroup is x∙Y+y. - The common codepoint value for PO index = x is X∙Y+x. - Option 2: - For a subgroup whose subgroupID = y and relative PO index = x, the codepoint value of this subgroup is x∙2log2⁡(Y+1)+y. - The common codepoint value for PO index = x is x∙2log2⁡(Y+1)+Y - Option 3: - PO index is mapped into the most significant bits of codepoints while subgroupID is mapped into the rest log2Y+1 bits and the common codepoint is mapped into the rest log2Y+1 bits with all bit value ‘1”. Note: x is 0 ~ X-1, y is 0 ~ Y-1 |
| --- | --- |
| [4] vivo | Proposal 1: - UE identifies its subgroup specific codepoint value as follows: - Let iPO denotes the PO index associated with a LP-WUS of the UE, with starting value of 0: iPO=UE_IDmodN⋅NS+i_smodNPOLP-WUS, where UE_ID, N, NS and i_s are defined in clause 7 of TS 38.304. - NPOLP-WUS denotes the configured number of PO associated with a LP-WUS. - A UE determines its subgroup specific codepoint value as “iPO*NSGPO+iSG”, where - NSGPO denotes the configured number of LP-WUS subgroup per PO and 1≤NSGPO* NPOLP-WUS≤31; - iSG is the LP-WUS subgroup ID per PO for the UE and 0≤iSG<NSGPO. - For a UE with PO index iPO, its common codepoint value is “NPOLP-WUS*NSGPO+iPO”. - When a UE detects a codepoint value carried by a LP-WUS which is equal to “NPOLP-WUS*NSGPO+iPO” or “iPO*NSGPO+iSG”, the UE monitors its PO; otherwise, the UE is not required to monitor PO. Proposal 11: If one LO is associated with POs of two or four PFs, these associated PFs are consecutive PFs calculated by the parameters PF_offset, T, Ns, and N. The first PF of the PFs associated with the LO is provided by (SFN for PF) - floor (iPO/Ns)*T/N, where SFN for PF is determined in clause 7.1 of TS 38.304, iPO is defined as the associated PO index for a LO, T, Ns, and N are determined in clause 7.1 of TS 38.304. |
| [8] Nokia | Proposal 12: Adjusting the higher layer information bits to account PO common codepoint(s) and multiple PO(s) as follows: - When subgroupNumber PO_LPWUS >1 the physical layer information bits corresponding subgroup ID n, can be defined as: CKn'=CKn+(subgroupNumber_PO_LPWUS+1)*(l_POindex-1), where CKn is the information bits corresponding to the subgroup ID n. - The information bits corresponding to the PO common codepoint as: CKc'=(subgroupNumber_PO_LPWUS+1)*(l_POindex-1). Proposal 13: Same approach used to adjust the information bits for a codepoint is used to determine the information bits for overlay sequence. |
| [9] CATT | Proposal 7: For all subgroup indexes in a PO, the codepoint corresponding to all subgroups index is {(NSGPO*NPOWUS+iPOWUS)}, where NSGPO is the number of subgroup per PO, NPOWUS is the number of PO per LO, iPOWUS is the PO index, 0≤iPOWUS<NPOWUS. |
| [12] NEC | Proposal 2: for the codepoint value(s) of common codepoint(s) for PO(s), support one of the following three options:  Option 1: the first or last N codepoint values are used for the N common codepoints for N POs respectively.  Option 2: (Nsg+1) consecutive codepoint values are used for a PO, wherein the first/last codepoint value in the (Nsg+1) codepoint values is used for the common codepoint for the PO, and the rest Nsg codepoint values are used for Nsg subgroups.  Option 3: only one codepoint value (e.g., the first or last codepoint value) is used for the common codepoint of N POs, UE further determines which PO is indicated based on in which MO the common codepoint is detected. |
| [17] OPPO | Proposal 3: Support PO indication field in LP-WUS, i.e., LP-WUS consist of 2 fields, one is used to indicate the associated PO, the other one is used to indicate the codepoint. |
| [21] Apple | Proposal 1: For a UE monitoring LP-WUS, - The PO index within the LO is defined as jPO=UE_ID mod N∙Ns+is mod NPOLO, - where NPOLO is the number of POs associated with a LO, and UE_ID (for LP-WUS), N, Ns and is are defined in 38.304. - For the determination of the reference PF, the first PF of the PF(s) associated with the LO is provided by (SFN for PF) – floor(jPO/Ns) * T/N. - If NPOLO=1, a LP-WUS includes K1=log2subgroupNumberPO_LPWUS+1 bits. If either the K1 bits are all 1’s or the value is equal to the subgroup index of the UE, the UE is considered to have received an indication in the WUS for its subgroup to monitor the associated PO. - If NPOLO>1, a LP-WUS includes K2+K1 bits, where K2=log2NPOLO , and K1=log2subgroupNumberPO_LPWUS+1. If the value of the K2 MSBs is equal to jPO, and either the K1 LSB bits are all 1’s or the value is equal to the subgroup index of the UE, the UE is considered to have received an indication in the WUS for its subgroup to monitor the associated PO. |
| [22] QC | Proposal 11: The number of codepoints configured for LP-WUS is equal to • (1 + number of UE subgroups per PO) * number of POs mapped to a LO • Codepoint value is from 0 to number of codepoints configured for LP-WUS - 1 Proposal 12: Within a LP-WUS MO, the codepoint values are sequentially allocated for the common codepoint for each PO followed by codepoints for individual UE subgroups for each PO of all POs associated with the LO. |
| [23] Sharp | Proposal 1: Codepoints should be evenly allocated across each PO associated with a given LO. Proposal 2: Each UE monitors two codepoint values. In cases where only one subgroup per PO configured, both codepoints may carry the same value. |
| [25] Nordic | Proposal-6: Consider (at least for overlaid design) one more codepoint/sub-group to be dedicated to early termination of monitoring for the UEs. |

## Wake-up delay

The following was agreed for the UE capability report on the wake-up delay:

Agreement

For UE capability report on the wake-up delay:

- Alt 1: For the 3 candidate values for the wake-up delay capability report, support {[70ms], [500ms] and [900ms]}.

  - The reported values assume SSB periodicity of 20ms, where [70ms] assumes [3] SSBs needed for synchronization, [500ms] and [900ms] assume [5] SSBs needed for synchronization.

  - FFS: translation of wake-up delay for different SSB periodicities

  - FFS: different sets of 3 candidate value for different SSB periodicities

- Send an LS to RAN4 after the down-selection to confirm the number of SSBs. Final LS in R1-2501624.

Agreement

On how to handle the UE capability report on the wake-up delay for SSB periodicities other than 20ms, consider the following alternatives for possible down-selection in RAN1#121.

- Alt 1: Do not report for SSB periodicities other than 20ms

  - Note: LP-WUS is not supported for SSB periodicities larger than 20ms

- Alt 2: For UE capability report on the wake-up delay, the UE reports one of the following 3 capabilities (the values in one of the columns):

| SSB periodicity (ms) | Wake-up delay (ms) UE capability 1 | Wake-up delay (ms) UE capability 2 | Wake-up delay (ms) UE capability 3 |
| --- | --- | --- | --- |
| [5] | [x] | [x] | [x] |
| [10] | [x] | [x] | [x] |
| 20 | [70] | [500] | [900] |
| [40] | [x] | [x] | [x] |
| [80] | [x] | [x] | [x] |
| [160] | [x] | [x] | [x] |

- Alt 3: It is up to RAN4

Companies’ views on UE capability report on the wake-up delay:

- Alt 1: [3] ZTE, [5] Spreadtrum, [9] CATT, [24] DCM, [25] Nordic

  - [24] DCM: Ask RAN4 whether to support SSB periodicities other than 20ms

- Alt 2: [1] Futurewei, [2] HW/HiSi (?), [8] Nokia, [14] Ericsson (?), [17] OPPO, [18] Panasonic, [19] LG, [20] MediaTek, [21] Apple

- Alt 3: [6] Samsung, [10] CMCC, [13] InterDigital, [16] ETRI, [17] OPPO (preferred)

- Wait for RAN4 LS reply: [11] xiaomi

Also note that the following was agreed in RAN4#114bis (R4-2504908):

Issue 1-2-6: On MR wake up delay related requirements for LP-WUR and RAN1 LS R1-2501624 Issue 1-2-6-1: On the number of SSBs Agreement: Aim to conclude the issue by Tuesday in RAN4 #115. Issue 1-2-6-2: On SSBs number for different SSB periodicities Agreement Same SSB number for different SSB periodicity Agreement When SSB periodicity is 5 ms or 10ms, a lower bound 20 ms will be used for MR wake up delay. Note: This above agreement based on the assumption that the wake up duration is defined as the ramping time + SSB number * SSB periodicity

Given that we decided not to leave the issue to RAN4 on 20ms SSB periodicity case, and RAN4 plans to provide a reply LS by Tuesday, it seems counter-productive to go with Alt 3 and give the issue to RAN4 now. Alt 1 on the other hand seems a bit restrictive. Therefore, the proposal below is to adopt the direction of Alt 2, and the values can be finalized based on RAN4 reply LS.

Proposal 3-1

For UE capability report on the wake-up delay, the UE reports one of the following 3 capabilities (the values in one of the columns):

| SSB periodicity (ms) | Wake-up delay (ms) UE capability 1 | Wake-up delay (ms) UE capability 2 | Wake-up delay (ms) UE capability 3 |
| --- | --- | --- | --- |
| 5/10/20 | [70] | [500] | [900] |
| 40 | [x] | [x] | [x] |
| 80 | [x] | [x] | [x] |
| 160 | [x] | [x] | [x] |

| Company | Comments |
| --- | --- |
| Qualcomm | Fine with the proposal. |
| CMCC | Okay |
| vivo | OK. For SSB periodicity larger than 20ms, the corresponding wake-up delay can be according scaled by ramping up time + SSB number * SSB periodicity. |
| xiaomi | General OK with the proposal. Accurate values can be determined based on RAN4 reply. |
| Huawei, HiSilicon | OK |
| Samsung | Agree with xiaomi that the accurate values can be made according to the LS reply from RAN 4. |
| Nokia1 | Fine with the proposal |
| FW | OK |
| DCM | We prefer Alt 1 (do not support larger than 20ms SSB periodicity) to reduce implementation work to support other SSB periodicities. But we can accept Alt B. Can we add “the values can be finalized based on RAN4 reply LS” to the proposal ? |
| ZTE, Sanechips | NO. Alt 1 is preferred. For SSB periodicity larger than 20ms, it is mainly used for Network energy saving. By introducing LP-WUS and LP-SS, all the NW energy saving gain would be eaten by LP-SS and LP-WUS. Therefore, it should not be supported. Additionally, for SSB periodicity 160ms, based on ramping up time + SSB number * SSB periodicity, the wake-up delay would be 400ms+5*160ms=1200ms. For the legacy NR UE supporting 160ms SSB periodicity, there is no any scaling for UE wake-up due to different SSB periodicity. It is totally not needed to define such kind of wake-up delay which is different from the legacy UE. |
| OPPO | OK |

Collection of the related companies’ proposals:

| [10] Huawei/HiSi | Proposal 16: For UE capability report on the 3 wake-up delays, from RAN1 perspective, the number of SSBs need for synchronization are confirmed to be {3,5,5}, respectively. Proposal 17: 70ms is the minimum value of the wake-up delay assuming UE’s MR wakes up from deep sleep mode in IDLE mode, and for different SSB periodicities other than 20ms: • 70ms is kept for SSB periodicity of 5ms and 10ms; • The wake-up delay is translated for SSB periodicities larger than 20ms, i.e. wake-up delay = 10ms+3*SSB_periodicity |
| --- | --- |
| [15] xiaomi | Proposal 4: For the translation of wake-up delay for different SSB periodicities, the following Table 1 can be considered, Table 1 Mapping between reported wake-up delay and actual wake-up delay Reported wake-up delay; Actual wake-up delay. [70] ms; ms. [500] ms; ms. [900]ms; ms. Note: T is SSB periodicity.. |
| [16] LG | Proposal #12: For UE capability report on the wake-up delay, consider following options to derive sets of 3 candidate value for different SSB periodicities:  Option 1: Reporting alternative wake-up delay assuming larger SSB periodicities:  Support that UEs additionally reports wake-up delays assuming alternative SSB periodicities (e.g., 40ms, 80ms or 160ms). If a non-reported SSB periodicity is configured, use the reported value corresponding to the closest configured SSB period to derive actual wake-up delay.  Option 2: Table-Based Reporting:  This option maintains Alt. 1 structure by utilizing pre-defined table indices linked to SSB periodicity ranges. UEs report an index (Low/Mid/High) only and use the index with the current SSB periodicity to determine the wake-up delay from a predefined table. Proposal #13: UEs with different wake-up delay should be assigned to different UE subgroups Proposal #14: UEs having shorter wake-up delay can be assigned to a specific subgroup(s) separately  The number of those subgroups can be separately configured  Wake-up delays below a certain threshold are considered as shorter wake-up delay |
| [23] Ericsson | Proposal 9 For UE capability report on the wake-up delay, support different sets of 3 candidate values for different SSB periodicities. |

## Mapping between LP-WUS/LP-SS and SSB beams

On how the mapping between LP-WUS/LP-SS and SSB beams is determined, there are mainly two approaches, one is implicit mapping (in order mapping), the other is explicit configuration. Two proposals were discussed in RAN1#120bis (R1-2503147), one for implicit mapping, and the other one for explicit mapping (proposed by the proponents).

Proposal 4-1 from R1-2503147 (implicit) For LP-SS, the LP-SS occasions are indexed sequentially in time, the k-th LP-SS occasion corresponds to the k-th transmitted SSB, k = 1, 2, …, N, where N is the number of actual transmitted SSBs determined according to ssb-PositionsInBurst in SIB1. Proposal 4-1A-r1 from R1-2503147 (explicit) It is supported that gNB transmits LP-WUS/LP-SS associated with SSB indexes, and it is configured by gNB in the SIB for LP-WUS configuration on whether LP-WUS/LP-SS transmission is present or not for each SSB index. - The detailed signaling design is up to RAN2 - For the LP-WUS MO and LP-SS occasion determination, the same LP-WUS/LP-SS is transmitted repeatedly in N beams, respectively, where - Alt1: S is the number of actual transmitted SSBs determined according to ssb-PositionsInBurst in SIB1, and LP-WUS MOs and LP-SS occasions are determined assuming S beams. gNB transmits LP-WUS/LP-SS in a subset of the S beams according to the LP-WUS configuration in SIB. - Alt2: N is the number of SSBs associated with LP-WUS/LP-SS, which is determined by the SIB for LP-WUS configuration. LP-WUS MOs and LP-SS occasions are determined assuming N beams.

Summary of companies’ views on mapping between LP-WUS/LP-SS and SSB beams:

- Implicit: [4] vivo, [8] Nokia (?), [14] Ericsson, [21] Apple

- Explicit: [1] Futurewei, [2] HW/HiSi, [5] Spreadtrum, [13] InterDigital

  - Alt 1: [2] HW/HiSi, [13] InterDigital

  - Alt 2: [1] Futurewei

There is very limited input to RAN1#121.

The following two proposals are formulated for the implicit and explicit approaches, and we should down-select to one of them.

Proposal 4-1

For LP-SS, the LP-SS occasions are indexed sequentially in time, and the n-th LP-SS occasion is associated with the beam of the n-th transmitted SSB, n = 1, 2, …, N, where N is the number of actual transmitted SSBs determined according to ssb-PositionsInBurst in SIB1.

The RAN1#120bis agreement is updated as follows:

For LP-WUS, the N * M LP-WUS MOs in an LO are indexed sequentially in time, from 1 to N*M, where N is the number of actual transmitted SSBs determined according to ssb-PositionsInBurst in SIB1 the number of beams corresponding to LP-WUS, and M is the number of LP-WUS MOs for each beam.

- The (n*M+m+1)-th LP-WUS MO corresponds to the (n+1)-th beam, which is associated with the beam of the (n+1)-th transmitted SSB, where m=0,1,…,M-1, n=0,1,2,…,N-1. (multiple MOs first, beam second)

Proposal 4-2

It can be configured by the gNB in the LP-WUS/LP-SS configuration on whether LP-WUS/LP-SS transmission is present or not for each beam of the actual transmitted SSBs determined according to ssb-PositionsInBurst in SIB1.

- The detailed signaling design is up to RAN2

- For the LP-WUS MO and LP-SS occasion determination,

  - Alt 1: S is the number of actual transmitted SSBs determined according to ssb-PositionsInBurst in SIB1, and LP-WUS MOs and LP-SS occasions are determined assuming S beams. gNB transmits LP-WUS/LP-SS in all or a subset of the S beams according to the LP-WUS/LP-SS configuration.

  - Alt 2: N is the number of beams for LP-WUS/LP-SS, which is determined by the LP-WUS/LP-SS configuration. LP-WUS MOs and LP-SS occasions are determined assuming N beams.

Please comment on your support of Proposal 4-1 or 4-2 above, and Alt 1 vs Alt 2 if you support Proposal 4-2. Please also comment if you have strong concern on any of the two proposals.

| Company | Comments |
| --- | --- |
| Qualcomm | Fine with the proposal. |
| CMCC | Support Alt 1 in proposal 4-2. |
| vivo | Support proposal 4-1. |
| Xiaomi | We think the main essential difference is whether to allow only partial of SSB beams have corresponding LP-WUS while other beams do not. This issue has been discussed multiple times and no consensus. We prefer Proposal 4-2-Alt 1 slightly. |
| Huawei, HiSilicon | We prefer proposal 4-2, which provide gNB more flexibility to avoid some unnecessary LP-WUS/LP-SS transmission in some beam directions. Proposal 4-1 provides no benefit for gNB implementation. Under proposal 4-2, either alternative is OK for us. Considering that we only have one meeting, Alt 1 is slightly preferred by us to reduce the effort of discussion. One thing we would like to clarify is that since we already agreed that the MOs for the same beam will be located together, even with Alt 1 the reserved resources are not so fragmented. Thus Alt 1 still provide the benefit for gNB to reuse the resources efficiently. |
| FW | Prefer Proposal 4-2 with preference to Alt 2 for LP-WUS. |
| InterDigital | We support Proposal 4-2. Prefer Alt 1. |
| ZTE, Sanechips | Support 4-1 with no any new agreement. and no need to discuss this again and again. |
| OPPO | OK with the proposal. |

Collection of related proposals:

| [1] Futurewei | Observation 1: Determining the LP-WUS MOs based on the actually transmitted SSBs rather than the indicated subset of SSBs associated with LP-WUS/LP-SS may limit the number of LOs that can be configured per PF due to large span of resources needed for an LO. Proposal 1: Support Alt2 that only LP-WUS MOs are determined assuming the N SSBs associated with LP-WUS/LP-SS which are determined by SIB for LP-WUS configuration. |
| --- | --- |
| [2] HW/HiSi | Proposal 7: It is supported that gNB transmits LP-WUS/LP-SS associated with SSB indexes, and it is configured by gNB in the SIB for LP-WUS configuration on whether LP-WUS/LP-SS transmission is present or not for each SSB index. • The detailed signaling design is up to RAN2 • For the MO location determination for LP-WUS/LP-SS, the same LP-WUS/LP-SS is transmitted repeatedly in N MOs via N beams, respectively, where N is the number of actual transmitted SSBs determined according to ssb-PositionsInBurst in SIB1, and gNB can transmits LP-WUS/LP-SS in a subset of the N MOs according to the SIB for LP-WUS configuration |
| [4] vivo | Proposal 14: The number of LP-WUS/LP-SS beams is the same as the number of SSB beams determined by ssb-PositionsInBurst in SIB1. |
| [5] Spreadtrum | Proposal 7: gNB needs to inform UE which SSB beams are used to transmit LP-WUS/LP-SS. |
| [8] Nokia | Proposal 11: Define that there is one-to-one association between LP-SS and LP-WUS MO time locations and SSB beams. This can be done as logical indexing. |
| [13] InterDigital | Proposal 1: Support indication on a subset of SSB beams for LP-WUS operation with Alt1 (S is the number of actual transmitted SSBs determined according to ssb-PositionsInBurst in SIB1, and LP-WUS MOs and LP-SS occasions are determined assuming S beams) for the MO and LP-SS occasion determination. |
| [14] Ericsson | Support one-to-one beam association for LP-SS/LP-WUS based on the SSB beam time-domain index. That is, the k-th LP-SS/LP-WUS occasion/beam corresponds to the k-th transmitted SSB/SSB beam, where k = 1, 2, …, N, and N is the number of actual transmitted SSBs. |
| [21] Apple | Proposal 9: The mapping between LP-WUS/LP-SS beams and SSB beams is determined with a 1-to-1 mapping sequentially in time in an implicit way. |

[13] InterDigital: Proposal 8: Each LP-WUS is QCLed with LP-SS in QCL Type A and QCL Type D (when applicable).

## EPRE ratio configuration for LP-WUS and LP-SS

The following was agreed in RAN1#120:

Agreement

The EPRE ratio between LP-WUS/LP-SS and SSB can be configured by the gNB.

- FFS whether there is a common configuration or separate configuration for LP-WUS and LP-SS

- Above is not applicable for the case when all the subcarriers for LP-WUS/LP-SS are transmitted with zero power (from baseband perspective)

- Above does not mandate any UE implementation for different receiver types

- FFS: whether/how to support for the case when there are different number of OOK ON symbols in different OFDM symbols (if supported)

Companies’ views on common or separate EPRE ratio configuration for LP-WUS and LP-SS:

- Common: [3] ZTE, [5] Spreadtrum, [8] Nokia, [13] InterDigital, [16] ETRI, [17] OPPO, [21] Apple, [22] QC

- Separate: [1] Futurewei, [4] vivo, [6] Samsung, [14] Ericsson (the ratio is limited to be within a certain range), [18] Panasonic, [16] LG, [20] MediaTek, [25] Nordic

There is good support for both common and separate configuration.

Given that this is somewhat related to the issue whether we support the case of LP-WUS with M = 1 and LP-SS with M>1, we will delay this discussion till the later days of the week.

## LP-SS

Companies’ proposals on LP-SS are summarized at the end of this subsection.

With the agreed periodicity of 160ms and 320ms, the main open issues for LP-SS configuration includes how to determine the time-domain resource for each LP-SS occasion. Based on the comments in RAN1#120bis, many companies prefer to agree on the signaling mechanism for LP-WUS MO configuration, and try to leverage same/similar mechanism for LP-SS.

Therefore, the detailed discussion in on hold temporarily, until some progress is made on LP-WUS MO configuration.

Proposal 6-1

Each LP-SS transmission for each beam is contained within one slot.

| Company | Comments |
| --- | --- |
| Qualcomm | Support the proposal. |
| CMCC | Okay |
| xiaomi | Previously, we have an agreement that the LP-SS transmission will be on consecutive symbols. If the length of LP-SS binary sequence is 32, then, if we want to contain LP-SS in one slot, M=1 will not be able to be applied. Currently we see no obvious reasons to exclude M=1 case. |
| Huawei, HiSilicon | Support in general. But just want to ask what’s the additional information for this proposal compared with previous agreement ‘Each LP-SS transmission for each beam always occupies consecutive OFDM symbols’? |
| Samsung | Support |
| Nokia1 | Fine with the proposal, but think that for LP-SS the start symbol(s) in a slot are needed in addition i.e. not sure that the LP-WUS style approach is needed. |
| FW | OK |
| InterDigital | Fine |
| ZTE, Sanechips | Yes. I guess compared with previous agreement, it indicates different beam occupies different slot. |
| OPPO | Support |

LP-SS configuration related proposals:

| [1] Futurewei | Proposal 3: Support a frame-level offset along with the periodicity for LP-SS timing configuration. Proposal 4: Support the reuse of SSB patterns to obtain timing for candidate LP-SS within one or two frames depending on the configured LP-SS length and the Case for SSB pattern. |
| --- | --- |
| [4] vivo | Proposal 12: For LP-SS transmission, support the following designs: - LP-SS reuses the available symbol determination method defined for LP-WUS. - Nominal LP-SS occasion needs to be configured. - Actual LP-SS transmission duration is derived by the configured LP-SS sequence length. - If the number of consecutive available OFDM symbols within the nominal LP-SS occasion is less than the actual LP-SS transmission duration, the occasion is considered as invalid (no LP-SS monitoring in this occasion) and deferred it to the next valid occasion. - Two adjacent nominal LP-SS occasions are placed back-to-back i.e., no gap between them. - gNB configures LP-SS periodicity and a slot-level offset for the first nominal LP-SS occasion, e.g., - slot160, INTEGER {0..159} - slot320, INTEGER {0..319} |
| [5] Spreadtrum | Proposal 4: The starting time location of the first LP-SS occasion is configured w.r.t. the reference point provided by as LP-SS_periodicityoffset, using a symbol-level offset Proposal 5: The starting time location of the subsequent LP-SS occasion(s) is implicitly derived based on the time location of the first LP-SS occasion (i.e., Option 1) |
| [8] Nokia | Proposal 13: For LP-SS a reference time can be configured through (sub-frame or radio frame level) offset and periodicity. Proposal 14: Support configuration of the start time of LP-SS (corresponding to each SSB) in a slot. Consider either fixed pattern or indication of start symbol in sub-set of symbols. Proposal 15: Supporting configuration of starting PRB on common resource block grid for LP-SS and LP-WUS. Proposal 16: Consider the maximum frequency separation between SSB and LP-WUS that can be supported by OFDM-based WUR. |
| [10] CATT | Proposal 4. For design on LP-SS, support to reuse the mechanism in LP-WUS MO/resource determination as much as possible. - Support Alt 3 (Alt 1A within Alt 1) for available resource determination. - Support Alt 2 (Alt 2A within Alt 2) for starting time location determination. - Support Alt B for terminology definition. |
| [20] MTK | Proposal 2: Updated LP-SS Configuration: • LP-SS_Mvalue: {1, 2, 4}. • LP-SS_SequenceLength: Configured based on M (e.g., for M=1, L from {6,8}). • LP-SS_Binary_Seq_Index: 0-3. • LP-WUS-LP-SS_StartRB: Starting PRB index. • LP-SS_Periodicity: {160ms (for M=1,2), 320ms}. • LP-SS_OverlaidSequence_Enable: Boolean. |
| [21] Apple | Proposal 5: For LP-SS, a periodicity is configured, with candidate value set of {160ms, 320ms}. Proposal 6: For LP-SS, a list of slot offset values and one or more starting symbols in a slot are configured, which are used to determine the starting symbols of the LP-SS transmissions for multiple beams. |

## RRM measurement

There is an FFS from the following agreement from RAN1#120bis:

Agreement

For OFDM-based LP-WUR, reuse the LP-SS based LP-RSRP/LP-RSRQ definition of OOK-based LP-WUR.

FFS: Whether OFDM receiver can measure LP-SS if overlaid OFDM sequence is not configured (M=1).

Proposal 7-1

OFDM-based LP-WUR can measure LP-SS if overlaid OFDM sequence is not configured (M=1).

| Company | Comments |
| --- | --- |
| Qualcomm | Support the proposal. |
| CMCC | Okay |
| vivo | OK |
| xiaomi | We are trying to understand the intention of the proposal. Does it require the OFDM-based LR should be able to measure OOK-based signal？ |
| Huawei, HiSilicon | Support. But it may be also related to some discussion under 9.6.1, thus we suggest to discuss this after the related discussion in 9.6.1. |
| Nokia1 | Fine with the proposal. |
| FW | Support |
| InterDigital | Support the proposal. |
| ZTE, Sanechips | OK |
| OPPO | In our understanding, “overlaid OFDM sequence is not configured” means LP-SS don’t reuse the specified overlaid OFDM sequence for LP-WUS. In this case, the waveform generation of LP-SS is different, how to measure the LP-SS if UE don’t know the exactly signal. |

This is a collection of companies’ proposals on the RRM measurement related issues:

| [2] HW/HiSi | Proposal 8: The LP-SS based LP-RSRP/LP-RSRQ definition can be used for OFDM-based LP-WUR also for the case when overlaid OFDM sequence is not configured (M=1) Proposal 13: RAN1 concludes that both the unit of the LP-RSRP value and the LP-RSSI value are dBm per 11 RBs. |
| --- | --- |
| [3] ZTE | Proposal 1: The OFDM WUR can measure LP-SS even if overlaid OFDM sequence is not configured.  FFS whether OFDM WUR could detect LP-WUS ON-OFF pattern when overlaid sequence does not carry information. Proposal 2: It should be up to gNB to configure whether OFDM WUR use the LP-SS for measurement. |
| [4] vivo | Proposal 13: The set of SSB for LP-SSS-RSRP/RSRQ measurement is determined by ssb-PositionsInBurst in SIB1 rather than SSB-ToMeasure. |
| [8] Nokia | Observation 1: It would appear possible for OFDM-based WUR to emulate envelope detector, thus LP-SS based metrics would also be applicable when M=1. Observation 2: There should be no RAN4 requirement impact supporting this scenario. Proposal 1: Support LP-SS based metrics also for OFDM-based WUR when M=1. Proposal 2: If both LP-SS based and SSB based thresholds are configured, OFDM-based WUR is assumed to apply the SSB-based threshold (and metric) for evaluation. Observation 3: For the OFDM-based WUR, the time domain resources for RSSI in the SS-RSRQ would need to be defined. Proposal 3: For OFDM-based WUR SS-RSRQ measurement metric, clarify that measurementSlots and endSymbol are not applicable and that RSSI is measured over the fixed set of symbols in the slots containing the SSB(s). Down select among following options for the RSSI measurement symbols: - same symbols as for the SSB, or symbols as determined for configuration endSymbol=1 (i.e. {0,1,2..,11} for SCS<480kHz) is the slot where SSB is received. Proposal 4: LP-RSRP and LP-RSRQ measurements should be associated to a particular beam. Update the LP-RSRP and LP-RSRQ metric definitions for example as follows: “LP-RSRP shall be measured only among the reference signals corresponding to LP-SS associated with the same SS/PBCH block index.” Proposal 5: For LP-RSRP and LP-RSRQ measurements, it should be possible for the network to configure a sub-set of LP-SS’s from which LP-RSRP and LP-RSRQ can be measured, similarly as done for SSBs with SSB-ToMeasure. Observation 4: It is expected that performance requirements will ensure consistent reporting from different UEs. |
| [13] InterDigital | Proposal 2: Support entry/exit conditions considering measurement of both LP-SS and SSB for OFDM based LR. |
| [14] Ericsson | Proposal 24 Support that OFDM receiver can measure LP-SS even if overlaid OFDM sequence is not configured (M=1). |
| [18] Panasonic | Proposal 7: When M=1 and overlaid OFDM sequence is not configured, OFDM-based receiver can measure LP-SS to generate LP-RSRP/LP-RSRQ reusing the definition for OOK-based receiver. |
| [22] QC | Proposal 10: At least for M=1, OFDM receiver can measure LP-SS based LP-RSRP/LP-RSRQ if overlaid OFDM sequence is not configured. |

## RRC parameters

In this section, we discuss the necessary RRC parameters and the corresponding value ranges/sets based on the agreements.

Proposal 8-1

The number of MOs per beam per LO is configured by gNB, with candidate value set of {1, 2, 3, 4}.

| Company | Comments |
| --- | --- |
| Qualcomm | Fine with the proposal. |
| CMCC | Okay |
| vivo | OK |
| xiaomi | OK with the proposal |
| Huawei, HiSilicon | OK |
| Samsung | Support |
| FW | OK |
| InterDigital | Fine |
| ZTE, Sanechips | We are fine. |
| OPPO | OK |

Proposal 8-2

The candidate value set for the frame-level offset(s) is {[8, 9, …, 180]} in unit of frames.

| Company | Comments |
| --- | --- |
| Qualcomm | Fine with the proposal. |
| CMCC | Okay |
| vivo | Fine with the lower boundary, and how to get the 180frames as the upper boundary of the frame-level offset? From our views, it depends on the maximum total number of LO for the case PO-to-LO association is less than Ns, the maximum length of a LO, whether there is any time gap flexibility needs to be reserved between LOs etc. |
| Xiaomi | The longest wake up delay is less than 1000ms, that is 100 frames. We do not think the candidate values should be as large as 180. |
| Huawei, HiSilicon | Generally OK. But where does the upper bound 180 come from? But we are considering whether we can down-select some of the values. Since the reported capability values are {70, 400, 800}, the configured offset value may only some values around 70. 400, and 800ms. Some values, like 100~300ms, may not necessary. If the candidate values can be reduced, the signaling overhead for SIB may be reduced. |
| Samsung | Fine with the lower value. The largest value can be discussed further. |
| Nokia1 | In principle fine, but do we need full range of values in between e.g. 80ms and 500ms or 400ms and 1000ms? |
| ZTE, Sanechips | We are fine. |
| OPPO | OK |

List of potential RRC parameters (as a reminder):

- EPRE ratio

- Offset for the first MO

- Additional frame-level offset or offset for the first MO in case the number of POs per LO < Ns

- Other parameters for MO configurations

  - Available symbols

  - Nominal/actual MO durations

  - …

TPs: [4] vivo (Section 5)

## [Deprioritized] eDRX

eDRX related proposals:

| [3] ZTE | Proposal 6: For eDRX, the LO periodicity could be the same as IDRX FFS how to align with PTW to save LO monitoring |
| --- | --- |
| [8] Nokia | Proposal 17: If LP-WS operation with eDRX is supported and UE is configured with eDRX and LP-WUS, the LP-WUS monitoring is determined based on paging occasions so that UE can monitor LP-WUS associated to the POs that are within the PTW. |
| [9] CATT | Proposal 8: When eDRX is configured, LP-WUS should be monitored within PTW. |

It has been agreed in RAN2 to support eDRX for LP-WUS. The details will be  discussed in RAN2, and it can be deprioritized in RAN1 for now.

## Other

Any topics that are being discussed in AI 9.6.1 will not be discussed here, including but not limited:

- Frequency resource allocation of LP-WUS/LP-SS

#### Question 8-1:

Please provide comments on any topics that you think are critical for the completion of the WI.

| Company | Comment |
| --- | --- |
| Qualcomm | In R1-2504401 in agenda 9.6.1, we proposed gNB should transmit LP-WUS from the first MO and occupies consecutive MOs if more than one LP-WUS is to be transmitted in a period, to facilitate UE early termination of LP-WUS reception. UE can stop LP-WUS monitoring if UE has detected an LP-WUS for itself, or UE does not detect any LP-WUS for any UE in the MO. According to 9.6.1 FL’s assessment, it seems better to discuss our proposal in 9.6.2. |
| vivo | We propose to discuss the potential restriction on mapping multiple POs to one LO. From our view, the POs associated with one LO should be consecutive POs from the network perspective to avoid introduce any other signalling to indicate which PO is associated with a LO. |
| ZTE, Sanechips | Since eDRX is supported in RAN2, we need to discuss some potential spec impacts if any to complete the WI on time. |

# Guidance for RAN1#122 Discussions

# References

- R1-2503229	Remaining Issues on LP-WUS Operation in IDLE/INACTIVE Modes	FUTUREWEI

- R1-2503291	Procedures and functionalities of LP-WUS in IDLE/INACTIVE mode	Huawei, HiSilicon

- R1-2503319	Discussion on LP-WUS operation in IDLE/INACTIVE mode	ZTE Corporation, Sanechips

- R1-2503370	Remaining issues on LP-WUS operation in IDLE/INACTIVE modes	vivo

- R1-2503523	Discussion on LP-WUS operation in IDLE/INACTIVE modes	Spreadtrum, UNISOC

- R1-2503574	Discussion on LP-WUS operation in IDLE/INACTIVE modes	Samsung

- R1-2503643	Discussion on LP-WUS Operation in IDLE/INACTIVE modes	TCL

- R1-2503695	LP-WUS operation in IDLE/Inactive mode	Nokia

- R1-2503801	System design and procedure of LP-WUS operation for UE in IDLE/Inactive Modes	CATT

- R1-2503839	Discussion on LP-WUS operation in IDLE/INACTIVE modes	CMCC

- R1-2503890	Discussion on LP-WUS operation in Idle/Inactive modes	Xiaomi

- R1-2503939	Discussion on LP-WUS operation in RRC IDLE/INACTIVE mode	NEC

- R1-2503965	Discussion on LP-WUS operation in IDLE/INACTIVE modes	InterDigital, Inc.

- R1-2504019	LP-WUS operation in IDLE and INACTIVE modes	Ericsson

- R1-2504067	LP-WUS operation in IDLE / INACTIVE modes	Sony

- R1-2504144	Discussion on LP-WUS operation in IDLE/INACTIVE modes	ETRI

- R1-2504188	Further consideration on LP-WUS operation in RRC_IDLE/INACTIVE modes	OPPO

- R1-2504238	Discussion on LP-WUS operation in IDLE/INACTIVE modes	Panasonic

- R1-2504251	Discussion on LP-WUS operation in IDLE/INACTIVE modes	LG Electronics

- R1-2504266	LP-WUS operation in IDLE/INACTIVE modes	MediaTek Inc.

- R1-2504329	LP-WUS operation in IDLE/INACTIVE modes	Apple

- R1-2504402	LP-WUR operation in idle and inactive modes	Qualcomm Incorporated

- R1-2504438	Discussion on LP-WUS operation in IDLE/INACTIVE modes	Sharp

- R1-2504509	Discussion on LP-WUS operation in IDLE/INACTIVE modes	NTT DOCOMO, INC.

- R1-2504574	On LP-WUS operation in IDLE/Inactive	Nordic Semiconductor ASA

# Appendix A: Agreements from previous meetings

## RAN1#116 agreements

Agreement

Multi-beam operations are supported for LP-WUS and LP-SS for idle mode

Agreement

LP-WUS occasions (LOs) are defined for LP-WUS monitoring.

- Each LO has one or more LP-WUS monitoring occasions (MOs), where UE can monitors for LP-WUS transmission in each of the LP-WUS MOs.

  - Different LP-WUS MOs may correspond to different beams in multi-beam operation

  - It is not precluded that FFS whether or not each LO is defined as a time window that covers the corresponding LP-WUS MOs

  - FFS details

- It is at least supported that a UE monitors LOs with a configured periodicity.

- Each UE has a periodicity for LO monitoring, and it is at least supported that a UE monitors one LO per period.

  - FFS: A UE does not expect its LP-WUS monitoring occasions overlapping in time

  - FFS: monitoring of multiple more than one LOs per period e.g. if LP-WUS common to all UEs is supported or in case of eDRX (if supported)

- FFS eDRX, if supported

Agreement

For the case where a UE supports PEI and PEI is configured by the gNB, after the UE receives LP-WUS indicating wake-up, it is up to UE implementation whether to monitor PEI or not.

Agreement

It is supported that the UE monitors the legacy PO after receiving LP-WUS indicating wake-up.

- FFS: support of UE monitoring dynamic PO

Conclusion

For idle/inactive mode, how to map a UE to a subgroup ID for LP-WUS is left to RAN2 to decide.

## RAN1#116bis agreements

Agreement

For multi-beam operation of LP-WUS, UE assumes the same LP-WUS information payload is repeated in all transmitted beams corresponding to LP-WUS

- the selection of the beam(s) for the reception of the LP-WUS is up to UE implementation

Agreement

Each LO consists of N * K LP-WUS MOs, where N is the number of beams corresponding to LP-WUS, and K is the number of LP-WUS MOs for each beam.

- Option 1: K = 1

- Option 2: K can be larger than or equal to 1

  - FFS if more than 1 LP-WUS is transmitted from the same beam, whether the information in these multiple LP-WUS is always the same or can be different

Agreement

From RAN1 perspective, at least the following metrics can be supported for RRM serving cell measurement performed by OOK-based receiver based on LP-SS:

- LP-RSRP

  - LP-RSRP is the linear average of received power of LP-SS in OOK ON symbols.

    - FFS: How to determine the received power of LP-SS in OOK ON symbols

- LP-RSRQ

  - LP-RSRQ = LP-RSRP/LP-RSSI

  - For the definition of LP-RSSI for determination of LP-RSRQ, further consider the following options:

    - Option 1: LP-RSSI is the linear average of total received power in all LP-SS OOK symbols.

    - Option 2: LP-RSSI is the linear average of total received power in LP-SS OOK OFF symbols.

    - Option 3: LP-RSSI is the linear average of total received power in LP-SS OOK ON symbols.

- FFS: LP-SINR, Power ratio of OOK-ON symbol and OOK-OFF symbol

Note: RAN1 will send an LS to RAN2 and RAN4 on the measurement metrics that can be supported from RAN1 perspective, to facilitate RAN2/RAN4 discussions. The exact metrics for OOK-based receiver to be used and defined in the specifications depend on the outcome of [RAN1]/RAN2/RAN4 discussions.

Working Assumption

From RAN1 perspective, for the entry/exit conditions for LP-WUS monitoring in IDLE/inactive mode,

- The UE may start LP-WUS monitoring if

  - the serving cell measurement performed by the MR is above entry threshold(s), if configured by the gNB, and/or

  - FFS other conditions, and if any, whether all or one or some of the conditions need to be satisfied

- If UE starts LP-WUS monitoring, it may stop the legacy PO monitoring before UE receives LP-WUS indicating wake-up

- The UE monitors the legacy PO (and may monitor PEI) and may stop LP-WUS monitoring if

  - the serving cell measurement performed by the LR is below exit threshold(s), if configured by the gNB, and/or

  - FFS other conditions, and if any, whether all or one or some of the conditions need to be satisfied

- FFS the serving cell measurement metrics

- The entry/exit thresholds can be configured separately for different types of LR

- It is left to RAN2 discussion whether the threshold(s) are always configured by the gNB.

- Note: This may be revisited based on the RAN2/RAN4 discussion.

Conclusion

LP-SINR is not considered further as a metric for RRM serving cell measurement for OOK-based receiver.

## RAN1#117 agreements

Conclusion

Regarding the “FFS: How to determine the received power of LP-SS in OOK ON symbols” for LP-RSRP, no additional work in RAN1.

Agreement

It is supported that UEs monitoring the same PO are divided into multiple subgroups, where LP-WUS can provide wake-up indication for each subgroup. Consider the following options:

- Option 1: UEs monitoring the same PO monitor the same LO

- Option 2: UEs corresponding to different POs monitor the same LO

- Option 3: UEs monitoring the same PO are divided into multiple sets of subgroups, with UEs within each set of subgroups monitoring the same LO.

- Combinations of the above options can be considered.

Agreement

For LP-SS based LP-RSRQ, LP-RSRP and LP-RSSI are measured within the same bandwidth.

Agreement

For LP-RSSI definition for LP-RSRQ, the following is agreed

- Option 1: LP-RSSI is the linear average of total received power in ON and OFF LP-SS OOK symbols.

Note: Above does not constrain LP-SS sequence design for OOK

Working Assumption

From RAN1 perspective, for the RRM measurement metrics based on SSS for OFDM-based LP-WUR, use the same definition of SS-RSRP and SS-RSRQ for LP-SSS-RSRP and LP-SSS-RSRQ, respectively.

- Above is applicable for both time-domain processing or frequency-domain processing

- Above does not imply that RAN1 will introduce LP-SSS-RSRP and LP-SSS-RSRQ in the specifications

Agreement

For idle/inactive mode, the maximum number of information bits (excluding CRC) in a LP-WUS is Z, where Z <= [8 or 16].

- FFS the exact value of Z

Agreement

For idle/inactive mode, the maximum number of subgroups per PO is X, where 8 <= X <= 256.

- FFS the exact value of X.

## RAN1#118 agreements

Agreement

The definitions of LP-RSRP and LP-RSSI for LP-RSRQ are updated as follows:

LP-RSRP is the linear average of received power of LP-SS in OOK ON symbols over the frequency resources defined by the number of REs that carry LP-SS.

LP-RSSI is the linear average of total received power in ON and OFF LP-SS OOK symbols over the frequency resources defined by the number of REs that carry LP-SS.

Agreement

At least support 1:1 association between LP-WUS MO(s)/LP-SS transmissions and SSB beams.

Agreement

For the wake-up delay, consider the following options:

Option 2: UE reports one value from X candidate values for the wake-up delay via UE capability reporting.

- FFS: X is 2, 3, 4

Above applies for IDLE/INACTIVE mode.

Definition of wake-up delay: Minimum gap time between LP-WUS reception and MR to start PDCCH monitoring

Agreement

For the maximum number of subgroups per PO (X), down-select from the following options in RAN1#118bis:

- Option 1: X = 8

- Option 2: X = 16

- Option 3: X = 32

- Option 4: X = 64

- Option 5: X = 128

- Option 6: X = 256

For decision in RAN1#118bis, companies are encouraged to provide the maximum number of information bits per LP-WUS (Z), the number of OFDM symbols occupied by LP-WUS per MO, the number of MOs for their preferred option.

Conclusion

There is no consensus in RAN1 on the support of dynamic PO.

Agreement

On the LO configuration for iDRX, offset value(s) between a LO and a reference PO/PF is/are configured.

- FFS: one or multiple offset values, and if multiple offset values are supported, how a UE decides which value to use

- FFS: the exact definition of the reference PF/PO and the detailed procedure for UE to determine the LO(s) corresponding to Option 1, 2, or 3 (if supported)

- (Working Assumption) For each UE, the periodicity of LO is the same as its iDRX cycle.

- If a UE receives a wake-up indication in a LP-WUS, consider the following alternatives

  - Alt 1: it monitors the PO associated with the offset.

  - Alt 2: it monitors the first PO after its reported wake-up delay.

  - Other alternatives are not precluded

Note: The PO mentioned above refers to legacy PO configured for the UE.

Agreement

Send an LS to RAN2 and RAN4 to convey the following

In RAN1, the common understanding is that UE may not support LP-WUS reception on all the bands supported by the UE. Request RAN2 and RAN4 to check if there is any issue and specification support needed for IDLE/INACTIVE UEs.

Final LS in R1-2407559.

Conclusion

RAN1 will not initiate work on entry/exit conditions based on RRM measurement and RRM measurement offloading/relaxation conditions unless triggered by RAN2 and RAN4

## RAN1#118bis agreements

Agreement

Confirm the following working assumption for iDRX: For each UE, the periodicity of LO is the same as its iDRX cycle.

Agreement

For the mapping between LO and PO, supports at least Option 1 (UEs monitoring the same PO monitor the same LO).

Agreement

From RAN1 perspective, when a UE is monitoring LP-WUS (based on the entry/exit condition for LP-WUS), a UE is not required to monitor a PO if

- it does not detect a LP-WUS on the monitored LO

- or the LP-WUS does not indicate a wake-up indication for the UE’s corresponding subgroup

Agreement

For the offset value(s) between an LO and a reference PO/PF, consider the following options:

- Definition: The gap between an LO and a PO is considered to be no less than the wake-up delay a UE supports if the gap between the end of the last LP-WUS MO the UE monitors in the LO and the start of the PO is no less than the wake-up delay.

- Option 1: gNB configures a single offset value.

  - If the gap between an LO and the PO associated with the offset is no less than the wake-up delay a UE supports, the UE monitors the PO associated with the offset after receiving a wake-up indication in a LP-WUS.

  - Otherwise,

    - Option 1-1: the UE follows the legacy paging monitoring procedure.

    - Option 1-2: the UE monitors LP-WUS. If it receives a wake-up indication in a LP-WUS, it monitors the first PO after its reported wake-up delay.

- Option 2: gNB configures one or multiple offset values.

  - For the same PO, each offset corresponds to a LO.

    - This does not preclude the possibility that the same LO may correspond to different POs with different offset values.

  - Option 2A: A UE does not expect that the gap between the LO associated with the largest offset and the corresponding PO is less than the wake-up delay the UE supports. The UE monitors the LO associated with one offset that has a gap between the LO and the corresponding PO no less than the wake-up delay.

    - This implies that the gNB needs to configure at least one offset value that is no less than the largest wake-up delay supported by the UEs.

    - FFS exactly how to choose the offset

  - Option 2B:

    - If the gap between the LO associated with the largest offset and the corresponding PO is no less than the wake-up delay a UE supports, the UE monitors the LO associated with one offset that has a gap between the LO and the PO associated with the offset no less than the wake-up delay.

      - FFS exactly how to choose the offset

    - Otherwise,

      - Option 2B-1: the UE follows the legacy paging monitoring procedure.

      - Option 2B-2: the UE monitors LP-WUS. If it receives a wake-up indication in a LP-WUS, it monitors the first PO after its reported wake-up delay.

        - FFS exactly how to choose the offset

  - FFS the UE shall monitor the LO associated with additional offset(s)

Note: The PO mentioned above refers to legacy PO configured for the UE.

Agreement

At least the following codepoints are supported for LP-WUS:

- One codepoint corresponding to each of the subgroups that can be indicated by LP-WUS

- One codepoint corresponding to all the subgroups that can be indicated by LP-WUS

- FFS: Additional codepoints

Agreement

Support 3 candidate values for the wake-up delay capability a UE reports, two values for ultra-deep sleep state and one value for deep sleep state.

- FFS the values

- Note: Ultra-deep sleep state and deep sleep state are mentioned above solely for the purpose of RAN1 discussions and it is not intended to be captured in specifications from RAN1 perspective

Agreement

Each LP-WUS is QCLed with one SSB. Each LP-SS is QCLed with one SSB.

- FFS QCL Type A or Type C and/or Type D

- FFS implicit QCL determination or some signaling is required

Agreement

The number of beams for LP-SS is the same as the number of beams for the LP-WUS MOs in an LO.

Agreement

When K (K>1) LP-WUS MOs are configured for each beam in an LO, down select between

- Option A: K LP-WUS MOs for a beam are divided into M (M >=1) groups of R LP-WUS MOs. A UE monitors all or some of the MO(s) within the K LP-WUS MOs.

  - For each group of R LP-WUS MOs, the same LP-WUS information is transmitted.

    - FFS how the same LP-WUS information is transmitted in the R LP-WUS MOs

  - Different LP-WUS information can be transmitted in different groups of R LP-WUS MOs.

  - M = 1 and M > 1 is supported.

  - FFS: detailed UE monitoring behavior

  - FFS R=1 or R>= 1

- Option B: K LP-WUS MOs for a beam are divided into G (G >= 1) groups of R*M (M >= 1) LP-WUS MOs. A UE monitors all or some of the MO(s) within one group of R*M LP-WUS MOs based on its subgroup ID.

  - Each group of R*M LP-WUS MOs is further divided into M groups of R LP-WUS MOs.

    - For each group of R LP-WUS MOs, the same LP-WUS information is transmitted.

      - FFS how the same LP-WUS information is transmitted in the R LP-WUS MOs

    - Different LP-WUS information can be transmitted in different groups of R LP-WUS MOs.

    - FFS: detailed UE monitoring behavior

  - M = 1 and M > 1 is supported.

  - FFS R=1 or R>=1

  - Note: this achieves the same purpose as “Option 3: UEs monitoring the same PO are divided into multiple sets of subgroups, with UEs within each set of subgroups monitoring the same LO.”

Working Assumption

The maximum number of subgroups per PO supported in Rel-19 is 32.

## RAN1#119 agreements

Agreement

Confirm the following working assumption with the addition of the note at the bottom:

Working Assumption

From RAN1 perspective, for the entry/exit conditions for LP-WUS monitoring in IDLE/inactive mode,

- The UE may start LP-WUS monitoring if

  - the serving cell measurement performed by the MR is above entry threshold(s), if configured by the gNB

  - FFS other conditions, and if any, whether all or one or some of the conditions need to be satisfied

- If UE starts LP-WUS monitoring, it may stop the legacy PO monitoring before UE receives LP-WUS indicating wake-up

- The UE monitors the legacy PO (and may monitor PEI) and may stop LP-WUS monitoring if

  - the serving cell measurement performed by the LR is below exit threshold(s), if configured by the gNB

  - FFS other conditions, and if any, whether all or one or some of the conditions need to be satisfied

- FFS the serving cell measurement metrics

- The entry/exit thresholds can be configured separately for different types of LR

- It is left to RAN2 discussion whether the threshold(s) are always configured by the gNB.

- Note: This may be revisited based on the RAN2/RAN4 discussion.

Note: this does not intend to impact any agreements and working assumptions made in RAN1/RAN2 after the working assumption.

Agreement

At least support the case that the number of beams for LP-WUS/LP-SS is the same as the number of SSB beams.

Conclusion

Do not support one-to-multiple mapping between the LP-WUS/LP-SS beams and the SSB beams.

Conclusion

No additional RAN1 specification impact specific to support multiple-to-one mapping between the LP-WUS/LP-SS beams and the SSB beams.

Agreement

For the offset value(s) between an LO and a reference PO/PF, down-select between

- Option 1-1

- Configurability between Option 1-1 and Option 1-2

- Option 2B-1

Agreement

At least for the 1:1 LO to PO mapping,

- The maximum value of M for Option A or Option B with G=1 (if either of them is supported) is 4.

Conclusion

There is no RAN1 consensus to support additional codepoints indicating wake-up at least for 1:1 LO to PO mapping.

For future meetings:

Consider the following alternatives for UE capability report on the wake-up delay:

- Alt 1: For the 3 candidate values for the wake-up delay capability report, support {[80ms], [500ms] and [900ms]}.

  - The reported values assume SSB periodicity of 20ms, where [80ms] assumes [3] SSBs needed for synchronization, [500ms] and [900ms] assume [5] SSBs needed for synchronization.

  - FFS: translation of wake-up delay for different SSB periodicities

- Alt 2: For the wake-up delay capability report, UE reports a pair of values for main radio ramp up and the number of SSBs needed for synchronization (D, NSSB). Support the following 3 candidate capabilities: {[(20ms, 3), (400ms, 5), (800ms, 5)]}.

- Alt 3: Modify the previous agreement on UE capability report on the wake-up delay to UE capability report on the main radio ramp up time. The number of SSBs needed for synchronization is defined in specs, and different number of SSBs can be defined for different UE capability on the main radio ramp up time.

  - 3 candidate values for the main radio ramp up time: {[20ms, 400ms, 800ms]}

  - The number of SSBs needed for synchronization is defined as [3, 5, 5] for [20ms, 400ms, 800ms] main radio ramp up time, respectively.

- Alt 4: Modify the previous agreement on UE capability report on the wake-up delay to UE capability report on the main radio ramp up time. The time needed for synchronization will be defined by RAN4 requirements later.

Agreement

Each LP-WUS or LP-SS is QCLed with an SSB with [select one from ‘typeA’, ‘typeC’], and when applicable, ‘typeD’ with the same SSB.

Working Assumption

If LP-WUS design support 32 subgroups within one MO, do not support Option 3 for LO to PO mapping or Option B for MO configuration.

Conclusion

From RAN1 perspective, there is no consensus on the following proposal:

UE monitors the legacy PO (and may monitor PEI) and may stop LP-WUS monitoring if	the UE has not received LP-WUS for a long time.

For future meetings:

Companies are encouraged to consider section 3.5 of R1-2410862 for further discussions in future meetings.

## RAN1#120 agreements

Agreement

The EPRE ratio between LP-WUS/LP-SS and SSB can be configured by the gNB.

- FFS whether there is a common configuration or separate configuration for LP-WUS and LP-SS

- Above is not applicable for the case when all the subcarriers for LP-WUS/LP-SS are transmitted with zero power (from baseband perspective)

- Above does not mandate any UE implementation for different receiver types

- FFS: whether/how to support for the case when there are different number of OOK ON symbols in different OFDM symbols (if supported)

Agreement

For the offset value(s) between an LO and a reference PO/PF, at least a frame-level offset is provided.

- The reference point (reference PO/PF) for the frame-level offset is the start of the PF, or the first PF of the PF(s) (if mapping of POs from multiple PFs to one LO is supported), associated with the LO.

- FFS other offset value(s) to determine the MOs of the LO

Agreement

The previous agreement in RAN1#119 is updated as follows:

- Each LP-WUS or LP-SS is QCLed with an SSB with [select one from ‘typeA’, ‘typeC’], and when applicable, ‘typeD’ with the same SSB.

Agreement

Confirm the following working assumption with the additional text in red:

Working Assumption

From RAN1 perspective, for the RRM measurement metrics based on SSS for OFDM-based LP-WUR, use the same definition of SS-RSRP and SS-RSRQ for LP-SSS-RSRP and LP-SSS-RSRQ, respectively.

- Above is applicable for both time-domain processing or frequency-domain processing

- Above does not imply that RAN1 will introduce LP-SSS-RSRP and LP-SSS-RSRQ in the specifications

Existing metrics SS-RSRP and SS-RSRQ are reused for OFDM-based LP-WUR. No separate metrics (LP-SSS-RSRP and LP-SSS-RSRQ) will be introduced in the specifications.

- The metrics SS-RSRP and SS-RSRQ are applicable for OFDM-based LP-WUR for RRC_IDLE and RRC_INACTIVE serving cell measurement.

- FFS: whether to remove or modify the restriction of using SMTC window for LP-SSS-RSRP/RSSI measurement.

Agreement

For the LO to PO mapping from network perspective, support Option 2 (UEs corresponding to different POs monitor the same LO).

- This should not increase the maximum number of codepoints per LO/LP-WUS compared to Option 1.

- FFS conditions/restrictions for mapping multiple POs to one LO

- Down-select between 2 and 4 for max number of POs per LO.

Agreement

For UE capability report on the wake-up delay:

- Alt 1: For the 3 candidate values for the wake-up delay capability report, support {[70ms], [500ms] and [900ms]}.

  - The reported values assume SSB periodicity of 20ms, where [70ms] assumes [3] SSBs needed for synchronization, [500ms] and [900ms] assume [5] SSBs needed for synchronization.

  - FFS: translation of wake-up delay for different SSB periodicities

  - FFS: different sets of 3 candidate value for different SSB periodicities

- Send an LS to RAN4 after the down-selection to confirm the number of SSBs. Final LS in R1-2501624.

Agreement

For the offset value(s) between an LO and a reference PO/PF, adopt Option 2B-1.

- gNB can configure 1 or 2 offset values.

  - FFS whether gNB can configure 3 offset values

- If multiple offset values are configured and if the gap between the LO associated with the largest offset value and the corresponding PO is no less than the wake-up delay a UE reports, the UE monitors the LO associated with the smallest offset value that has a gap between the LO and the PO no less than the wake-up delay.

  - Note: if a single offset value is configured, UE behaviour is according to Option 1-1.

- All the UEs supporting LP-WUS for idle/inactive mode supports the configuration of 2 offset values (FFS: 3 values).

Agreement

Confirm the following working assumption with the modification:

Working Assumption

The maximum number of subgroups per PO supported in Rel-19 is 31 32.

## RAN1#120bis agreements

Agreement

Confirm the following working assumption with modifications in red:

Working Assumption

If LP-WUS design support 32 subgroups within one MO, do not support Option 3 for LO to PO mapping or Option B for MO configuration.

Agreement

Option A for MO configuration is supported.

Agreement

A LP-WUS MO can span across multiple slots.

- FFS the limitation on the maximum length for an LP-WUS MO

Agreement

Only R = 1 is supported for Option A.

Conclusion

For the offset value(s) between an LO and a reference PO/PF, do not support the configuration of 3 offset values.

Agreement

The maximum number of POs per LO is 4, and the number of POs per LO can be 1, 2 or 4.

Agreement

For Option 2, the maximum value of M (number of LP-WUS MOs per beam) in Option A for MO configuration is 4.

Agreement

For LP-WUS, the N * M LP-WUS MOs in an LO are indexed sequentially in time, from 1 to N*M, where N is the number of beams corresponding to LP-WUS, and M is the number of LP-WUS MOs for each beam.

- The (n*M+m+1)-th LP-WUS MO corresponds to the (n+1)-th beam, where m=0,1,…,M-1, n=0,1,2,…,N-1. (multiple MOs first, beam second)

Note: Above does not change the previous agreement on association between LP-WUS and SSB beams.

Agreement

For OFDM-based LP-WUR, reuse the LP-SS based LP-RSRP/LP-RSRQ definition of OOK-based LP-WUR.

FFS: Whether OFDM receiver can measure LP-SS if overlaid OFDM sequence is not configured (M=1).

Agreement

For LP-SSS-RSRP/RSSI measurement performed by OFDM-based LP-WUR for the serving cell, SMTC window is not applicable.

- Send an LS to RAN4 to confirm the above agreement. Final LS in R1-2503103.

Agreement

LS on the RRM measurement metrics for OFDM-based LP-WUR is agreed. Final LS in R1-2503103.

- Note: RAN1 understanding is existing metrics SS-RSRP and SS-RSRQ are reused for OFDM-based LP-WUR. No separate metrics (LP-SSS-RSRP and LP-SSS-RSRQ) will be introduced in the specifications.

Agreement

For Option 2, at least one codepoint corresponding to each of the subgroups in each PO is supported.

- For codepoint corresponding to more than one subgroups:

  - Alt 2: One codepoint for each PO corresponding to all the subgroups in the PO

Agreement

For Option 2, a common codepoint per PO is always used and the maximum number of subgroups supported per PO is

- 7 for the case where 4 POs are mapped to one LO

- 15 for the case where 2 POs are mapped to one LO

Agreement

Regarding whether there is any restriction on mapping multiple POs to one LO, no additional constraint for mapping multiple POs to one LO

Agreement

UE determines whether a symbol is available for LP-WUS based on:

- Alt 1: Time-domain pattern configured by the gNB

  - Alt 1A: Periodic time-domain pattern

    - E.g. (1) 1-slot periodicity, the pattern indicates the available symbols in each slot; (2) multi-slot or frame-level periodicity with a bitmap indication; (3) Search space set-like pattern; (4) multi-level time-domain patterns; (5) reuse the mechanism of rate matching pattern

  - Alt 1B: Per-MO pattern, applicable for all Mos

- Alt 2: Information from existing configurations available for idle/inactive UEs such as [SSB, CORESET/Type-0 CSS, TDD DL/UL configuration, etc].

- Alt 3: Combination of Alt 1 and Alt 2

- Alt 4: NW ensures LP-WUS configuration without collision with existing signal(s)

Agreement

For the determination of starting time locations of LP-WUS MOs and LP-WUS transmissions in a LO,

- A reference point is the start of a reference frame determined by the frame-level offset from the start of the first PF of the PF(s) associated with the LO.

- The starting time location of the first LP-WUS MO in a LO is indicated by an offset w.r.t. the reference point.

  - FFS slot-level or symbol-level offset

- The starting time locations of the subsequent LP-WUS MOs in a LO are determined based one of the following alternatives:

  - Alt 1: An offset is indicated for each of the subsequent LP-WUS MOs.

    - FFS slot-level or symbol-level offset

  - Alt 2: The start time location of a subsequent LP-WUS MO is determined implicitly at least based on the previous LP-WUS MO.

    - FFS additional configuration to control the subsequent MO locations, e.g.,

      - Alt 2A: configuration of a single gap between the end of the previous MO (or a set of previous MOs) and the start of the next MO

      - Alt 2B: configuration of candidate starting locations for MOs, similar to search space configuration

  - FFS restriction on MO locations, e.g. only on DL slots

  - FFS minimum gap is needed between two MOs to ensure LR processing time

Agreement

Terminology definition

- Nominal MO duration: this includes both the available and unavailable symbols

- Actual LP-WUS duration: the actual number of OFDM symbols used for LP-WUS transmission as assumed by the UE

Nominal MO duration and actual LP-WUS duration, if defined, are determined using one of the following alternatives:

- Alt A: Nominal MO duration is configured. (e.g. in unit of slots)

  - Actual LP-WUS duration is the number of available symbols within the MO.

  - Actual LP-WUS duration can vary from one MO to another MO.

- Alt B: Actual LP-WUS duration is configured. (e.g. in unit of OFDM symbols, or M and L values)

  - From the start of a MO, MO extends until the number of available OFDM symbols reaches the configured actual LP-WUS duration.

    - FFS: Additional termination condition such as time window

- Alt C: Both nominal MO duration and actual LP-WUS duration are configured.

  - If the number of available OFDM symbols within the nominal MO duration is less than the actual LP-WUS duration, the MO is considered as invalid (no LP-WUS monitoring in this MO) and dropped/deferred.

    - FFS UE behavior if there is no valid MO for the beam(s) that the UE monitors

  - Note: the number of available OFDM symbols within a nominal MO duration can be different for different MOs.

- Alt D: Nominal MO duration is configured. The actual LP-WUS duration is determined based on the same pattern for the available symbols for all the MOs (e.g. by using a per-MO pattern), which is the same for all the MOs.

Agreement

On how to handle the UE capability report on the wake-up delay for SSB periodicities other than 20ms, consider the following alternatives for possible down-selection in RAN1#121.

- Alt 1: Do not report for SSB periodicities other than 20ms

  - Note: LP-WUS is not supported for SSB periodicities larger than 20ms

- Alt 2: For UE capability report on the wake-up delay, the UE reports one of the following 3 capabilities (the values in one of the columns):

| SSB periodicity (ms) | Wake-up delay (ms) UE capability 1 | Wake-up delay (ms) UE capability 2 | Wake-up delay (ms) UE capability 3 |
| --- | --- | --- | --- |
| [5] | [x] | [x] | [x] |
| [10] | [x] | [x] | [x] |
| 20 | [70] | [500] | [900] |
| [40] | [x] | [x] | [x] |
| [80] | [x] | [x] | [x] |
| [160] | [x] | [x] | [x] |

- Alt 3: It is up to RAN4

Agreement

Each LP-SS transmission for each beam always occupies consecutive OFDM symbols.