3GPP TSG- RAN WG1 Meeting #120                                          R1-2504341

St Julian’s, Malta, May 19th – 23th, 2025

Agenda Item:	9.10.1

Source:	Apple Inc.

Title:	Discussion on RRM measurement adaptation for XR

Document for:	Discussion/Decision

# 1. Introduction

Regarding XR enhancements, the following Objectives are in the Rel-19 WID:

The Rel-19 XR ph3 objectives are as follows:

- Study and if justified, specify aspects related to multi-modality (intra-UE) (with coordination with SA2/SA4 as needed by LS request). Aim to facilitate efficient and effective support for XR application with Multiple QoS flows with multi-modal inter-dependencies, meeting multi-modal QoS requirements, e.g. synchronization and/or coordination. Efficiency enhancements are expected to be visible in terms of capacity or power consumption. [RAN2].

  - Note: Check in RAN#105 (check also other WG involvement if needed).

- Specify enhancements to enable transmission/reception in gaps/restrictions that are caused by RRM measurements (from inter-frequency RRM measurement gaps, or intra-frequency measurements, or other scheduling restrictions etc). [RAN1, RAN2, RAN4]

- Specify the corresponding measurement gap and scheduling restriction to enable the identified enhancements with RRM performance impact taken into consideration, work being triggered by LS. [RAN4]

- Specify Enhancements for Scheduling, as follows:

  - For the UL, Study and if justified, Specify enhancements using delay/deadline information, for support of UL scheduling to enable high XR capacity while meeting delay requirements/avoiding too late PDUs. [RAN2].

    - Note: LCP implementation complexity need to be taken into account when evaluating solutions.

    - Note: Check in RAN#105

- Specify the following user plane enhancements [RAN2]

  - RLC re-transmission related enhancements for operation of RLC Acknowledged Mode (AM) with small packet delay budget.

  - If justified, define a mechanism for transmitter to inform the receiver of SN gap (or missing SNs) in PDCP.

- Specify Core requirements related to the above objectives as necessary [RAN4]

Note: 	Whether / to what extent network exposure / RAN awareness / e.g. RAN involved rate control, possibly additional info for DL scheduling, parallel with SA2 work, shall be covered in this WI is TBD.

At RAN1 #119, a number of agreements were reached:

Agreement

For explicit indication by DCI, bit value equal to “1” indicates the corresponding gap/restriction occasion is to be “skipped”.

- FFS: Interpretation of bit value equal to “0”.

Agreement

A component of an FG can have Option 1 as a first value and Option 2 as a second value:

- Option 1: For explicit indication by DCI, bit value equal to “0” means UE ignores the indication of this field in the DCI.

- Option 2: For explicit indication by DCI, bit value equal to “0” means UE behaviour for the corresponding gap/restriction occasion is as per legacy behaviour.

Note: UE indicates only one value of this component.

Agreement

For explicit indication by DCI to skip a particular gap/restriction, in addition to DCI formats 0_1/1_1 and 0_2/1_2, one bit indication can be included as a part of DCI formats 0_3/1_3.

- Explicit indication can be configured for each DCI format individually by higher layer.

- The skipping indication corresponds to the first gap/restriction occasion among co-scheduled cells by the DCI (Type-1C field)

Above applies only for intra-band CA cases (same SCS for cells in the set of co-scheduled cells).

In this contribution we provide our views on the Rel-19 XR enhancements.

# RRM measurement adaption

A measurement gap (MG) can be per-UE MG, per-frequency range MG, or other MGs as supported by gapToAddModList-r17. Measurement gap(s) are provided by RRC signaling:

MeasGapConfig ::= SEQUENCE {

gapFR2 SetupRelease { GapConfig } OPTIONAL, -- Need M

...,

[[

gapFR1 SetupRelease { GapConfig } OPTIONAL, -- Need M

gapUE SetupRelease { GapConfig } OPTIONAL-- Need M

]],

[[

gapToAddModList-r17 SEQUENCE (SIZE (1..maxNrofGapId-r17)) OF GapConfig-r17 OPTIONAL, -- Need N

gapToReleaseList-r17 SEQUENCE (SIZE (1..maxNrofGapId-r17)) OF MeasGapId-r17 OPTIONAL, -- Need N

posMeasGapPreConfigToAddModList-r17 PosMeasGapPreConfigToAddModList-r17 OPTIONAL, -- Need N

posMeasGapPreConfigToReleaseList-r17 PosMeasGapPreConfigToReleaseList-r17 OPTIONAL-- Need N

]]

}

GapConfig-r17 ::= SEQUENCE {

measGapId-r17 MeasGapId-r17,

gapType-r17 ENUMERATED {perUE, perFR1, perFR2},

gapOffset-r17 INTEGER (0..159),

mgl-r17 ENUMERATED {ms1, ms1dot5, ms2, ms3, ms3dot5, ms4, ms5, ms5dot5, ms6, ms10, ms20},

mgrp-r17 ENUMERATED {ms20, ms40, ms80, ms160},

mgta-r17 ENUMERATED {ms0, ms0dot25, ms0dot5, ms0dot75},

refServCellIndicator-r17 ENUMERATED {pCell, pSCell, mcg-FR2} OPTIONAL, -- Cond NEDCorNRDC

refFR2-ServCellAsyncCA-r17 ServCellIndex OPTIONAL, -- Cond AsyncCA

preConfigInd-r17 ENUMERATED {true} OPTIONAL, -- Need R

ncsgInd-r17 ENUMERATED {true} OPTIONAL, -- Need R

gapAssociationPRS-r17 ENUMERATED {true} OPTIONAL, -- Need R

gapSharing-r17 MeasGapSharingScheme OPTIONAL, -- Need R

gapPriority-r17 GapPriority-r17 OPTIONAL, -- Need R

...

}

MUSIM-GapConfig-r17 ::= SEQUENCE {

musim-GapToReleaseList-r17 SEQUENCE (SIZE (1..3)) OF MUSIM-GapId-r17 OPTIONAL,-- Need N

musim-GapToAddModList-r17 SEQUENCE (SIZE (1..3)) OF MUSIM-Gap-r17 OPTIONAL,-- Need N

musim-AperiodicGap-r17 MUSIM-GapInfo-r17 OPTIONAL, -- Need N

...

}

MUSIM-Gap-r17 ::= SEQUENCE {

musim-GapId-r17 MUSIM-GapId-r17,

musim-GapInfo-r17 MUSIM-GapInfo-r17

}

PosGapConfig-r17 ::= SEQUENCE {

measPosPreConfigGapId-r17 MeasPosPreConfigGapId-r17,

gapOffset-r17 INTEGER (0..159),

mgl-r17 ENUMERATED {ms1dot5, ms3, ms3dot5, ms4, ms5dot5, ms6, ms10, ms20},

mgrp-r17 ENUMERATED {ms20, ms40, ms80, ms160},

mgta-r17 ENUMERATED {ms0, ms0dot25, ms0dot5},

gapType-r17ENUMERATED {perUE, perFR1, perFR2},

...

}

A MG configuration provides a periodic pattern derived from parameters provided in RRC signaling:

- MGL (measurement gap length): the length of MG in milliseconds.

- MGRP (measurement gap repetition period): indicates the periodicity at which the MG is repeated.

- MGTA (measurement gap timing advance): indicates the timing advance of the MG. The UE can assume the MG starts MGTA milliseconds earlier with respect to the subframe timing.

- gapOffset: indicates the slot offset of the MG pattern. Its value range is from 0 to MGRP-1.

- The UE calculates the SFN and subframe number of the first subframe for each gap.

SS/PBCH block measurement timing configuration (SMTC) can be configured by a number of IEs in RRC signaling including SSB-MTC, SSB-MTC2, SSB-MTC2-LP-r16, SSB-MTC3-r16, SSB-MTC4-r17, SSB-MTC-AdditionalPCI-r17, etc.

In one example, SSB-MTC is configured as:

SSB-MTC ::= SEQUENCE {

periodicityAndOffset CHOICE {

sf5 INTEGER (0..4),

sf10 INTEGER (0..9),

sf20 INTEGER (0..19),

sf40 INTEGER (0..39),

sf80 INTEGER (0..79),

sf160 INTEGER (0..159)

},

duration ENUMERATED { sf1, sf2, sf3, sf4, sf5 }

}

Period, Offset and duration are provided in defining a periodic pattern for SMTC.

Prior to Rel-17,  up to 2 MGs can be configured for a UE:

- gapUE or { gapFR1 and/or gapFR2}

In Rel-17, measurement gaps can be configured through new IEs:

- GapConfig-17 provides a MG configuration which is a ncsg (Network Controlled Small Gap)(ncsgInd-r17=true));

- GapConfig-17 provides a MG configuration which is a preconfigured MG ( preConfigInd-r17==true);

- A MG configuration which is neither ncsg nor a preconfigured MG;

- A MUSIM gap

- A positioning gap

One way to develop a RAN1 solution without touching the exact nature of measure gaps is to index various MG configurations, and the adaptation to their operations is through reference to MG configuration indices. One example is provided below. It should be noted from Figure 2 what is allowed by RAN4 specification (TS 38.133) can be more restrictive than what is allowed by RAN2 specification TS 38.331.  To reduce signaling overhead, it may be possible to consider restrictions from TS 38.133, to obtain a shorter list. However, to simplify the design, and also achieve an agonistic design, the table as shown in Figure 1 is more desirable.

Figure 1 Index of measurement gaps

Figure 2 Restriction on simultaneously configured MGs from TS 38.133

We have

Proposal 1: To facilitate adaptation of legacy MGs, Network Controlled Small Gaps, pre-configured MG, MGs for multi-SIM, MGs for positioning with a unified signaling design, MGs can be indexed. A MG index set consists of one or more MG index, MGs referred by a MG index set can be skipped through a single NW indication.

For the RRM measurement adaptation, several design aspects have been discussed in our previous contributions:

- Prioritization of channels/signals versus De-prioritization/adaptation of MG and/or SMTC scheduling restriction

  - A protection time window mechanism (Type-1 time window)

    - During a protection time window, eligible channel(s)/signal(s) are prioritized over RRM measurement.

    - The protection time window can be over the duration of a specific channel/signal or over a time interval  which may have a duration not fully aligned a specific eligible channel/signal.

    - It can be noted protection time window(s) can be provided in periodic configuration, semi-persistent configuration or MAC-CE or dynamic signaling.

  - A de-prioritization time window mechanism (Type-2 time window)

    - During a de-prioritization time window, a MG instance, a scheduling restriction instance is de-prioritized.

    - A de-prioritization time window can be MG or scheduling restriction specific. As a UE can be configured with multiple MGs and/or SMTC (thus corresponding scheduling restriction), e.g., per-UE MG, per-FR (Frequency Range) MGs, gap configuration configured through gapToAddModList-r17 under MeasGapConfig, or by SSB-MTC, etc.,  a de-prioritization time window can be associated with one or more of them. In one example, a de-prioritization time window configuration is for per-FR1 MG, an instance of the per-FR1 MG enclosed by the de-prioritization time-window is skipped or de-prioritized, and other MGs are not affected thus not skipped. In another example, a periodic pattern is tied with a specific MG or a specific SMTC configuration to provide de-prioritization time windows.

      - Alt. 2b.1.   skip all MG/SMTC occasions  within the time window.

      - Alt. 2b.2    skip  MG/SMTC occasions for specific configuration(s) within the time window. The prescribed configurations can be RRC configured/MAC CE indicated to a code state (or a single bit in a DCI). With Alt. 2, the MG/SMTC configurations not linked to the code state are not skipped over.

    - It can be noted de-prioritization time window(s) can be provided in periodic configuration, semi-persistent configuration or MAC-CE or dynamic signaling.

  - Hybrid mechanism

    - It may be also possible two mechanisms are both activated simultaneously.

  - Thus, the adaptation to prioritize Tx/Rx for XR over RRM measurements can be supported by protection time window mechanism, de-prioritization time window mechanism, or a hybrid mechanism. Then we can use the term “time window” to cover both “protection time window” and “de-prioritization time window”.

- Time-domain property of adaptation:

  - From extensive discussions from previous meetings, and also consider UE implementation complexity,  the adaptation is periodic.

- Time unit for adaptation (protection and/or de-prioritization):

  - Alt. 4.a: duration of/covering a measurement duration such as a MG instance or scheduling restriction due to a SMTC window, e.g., the duration of per-UE MG (gap length “mgl” of per-UE GapConfig) for the de-prioritization time-window mechanism.

  - Alt. 4.b: duration over a time-window

  - Handling of partial overlap between a time-window and one instance of MG/scheduling restriction, e.g., Adjusting a time-window by expanding or contracting.

  - Handling of partial overlap between a time-window and a channel/signal

- Impact to processing timeline

- Impact to CSI reporting

- Impact to RRM measurement for which RAN4 discussion is required.

The RRM measurement adaptation can be pursued in two approaches: in the first approach, channels/signals are prioritized over RRM measurement, MG and/or SMTC scheduling restriction is overridden, the design is mainly about specifying procedure/signaling to derive prioritized channels/signals. This is covered by the protection time-window mechanism (Type-1 time-window).

In the second approach, MG and/or SMTC scheduling restriction are adapted, and the design is about specifying procedure/signaling to switching off temporarily or adapting MG and/or SMTC scheduling restriction. This is covered by the de-prioritization time-window mechanism (Type-2 time-window).

The signaling is for a UE to derive a periodically occurring time window within which channels/signals are prioritized.

For XR traffic, e.g., video stream at 60 frames per second, when the traffic jitter is small, the arrival of XR packets can be predictable, and a time-window (Type-1 time-window or Type-2 time-window) can be allocated to enclose the expected traffic arrival and potential retransmissions including the required PDCCH/PUCCH, and within the time-window, RRM measurement adaptation is conducted. And the time-window is a periodic time-window. The time-window can be parametrized by {period, offset, duration}. The period can be in the units of slots or milliseconds. It may be also desirable to support non-integer periodicity as a ratio of two integers for the time-window, e.g., 50/3, 100/9 or 25/3 milliseconds to handle video streams at 60/90/120 frames per second. Note such non-integer periodicity can be used for both protection time-window mechanism and de-prioritization time-window mechanism.

Figure 3 Periodic/semi-persistent configuration for RRM measurement adaptation

Figure 4 Time-window for RRM measurement adaptation (Choice 1)

A periodic time-window (Type-1 time-window or Type-2 time-window) can be configured/activated by RRC signaling.

As a time-window is periodic, it may not be always possible to avoid partial enclosure of MG/SMTC scheduling restriction by the time-window. In the illustration, MG-2/SMTC-1 are partially covered by the time-window; thus it may be necessary to define a rule to derive an effective time window. In one choice, the time-window is enlarged by the overlapped MG and/or SMTC scheduling restriction (Figure 1). In another choice, the time-window is shrunken by the overlapped MG and/or SMTC scheduling restriction (Figure 2).

Figure 5 Time-window adaptation (Choice 2)

Considering multiple data flows for XR, multiple time-window configurations, e.g., each time-window configuration for a XR data flow can be motivated. They can have the same time-domain property (periodic, semi-persistent, or dynamic), or of different time-domain property. They can be Type-1 time-windows, or Type-1 time-windows, or Type-1/Type-2 time-windows.

It can be noted with periodic time window, both dynamic transmission/reception (DG PDSCH and DG PUSCH) and configured transmission/reception (any or all of SPS PDSCH, CG PUSCH, periodic/semi-persistent CSI-RS, periodic/semi-persistent SRS) can benefit from prioritization. Thus the periodic time window solution covers the XR use case well. With the periodic window solution, RAN1 specification effort is minimal.

At RAN1 #118, the following regarding dynamic signaling based solution was reached:

Working Assumption

For solutions based on triggering/enabling by network signaling to enable Tx/Rx in gaps/restrictions that are caused by RRM measurements select the following option:

- Alt. 1: Dynamic indication to enable Tx/Rx in particular gap/restriction that are caused by RRM measurements.

  - Alt 1-1: Explicit indication by DCI to skip a particular gap/restriction;

    - Indication is included as part of scheduling DCI:

      - Bit-field size is one bit;

        - The bit in the DCI is used to indicate whether to skip the first gap/restriction occasion after a minimum time offset required between the last symbol of the PDCCH carrying the DCI format and the start of corresponding skipped gap/restriction occasion indicated by the DCI.

Send an LS to RAN4 to inform them of the above working assumption and ask them if there is any issue with it. Final LS in R1-2407561.

At RAN1 #119, the following was agreed:

Agreement

For explicit indication by DCI, bit value equal to “1” indicates the corresponding gap/restriction occasion is to be “skipped”.

- FFS: Interpretation of bit value equal to “0”.

Agreement

A component of an FG can have Option 1 as a first value and Option 2 as a second value:

- Option 1: For explicit indication by DCI, bit value equal to “0” means UE ignores the indication of this field in the DCI.

- Option 2: For explicit indication by DCI, bit value equal to “0” means UE behaviour for the corresponding gap/restriction occasion is as per legacy behaviour.

Note: UE indicates only one value of this component.

Agreement

For explicit indication by DCI to skip a particular gap/restriction, in addition to DCI formats 0_1/1_1 and 0_2/1_2, one bit indication can be included as a part of DCI formats 0_3/1_3.

- Explicit indication can be configured for each DCI format individually by higher layer.

- The skipping indication corresponds to the first gap/restriction occasion among co-scheduled cells by the DCI (Type-1C field)

Above applies only for intra-band CA cases (same SCS for cells in the set of co-scheduled cells).

The identification of the first gap after the dynamic signaling need clarification on a few issues:

Figure 6 handling sequence

Processing Order Option 1: the first gap after PDCCH+lead time is identified first, then the first gap is marked as skippable (non-present), then the TS 38.133 specification text for resolving overlap of measurement gaps is applied to the remaining measurement gaps if any to determine surviving measurement gaps if any. For this “RAN1-spec first” approach, the collision handling as specified in TS 38.133 is not run yet, at least for two or MG occasions starting at the same time (and in general for two more overlapping MGs), a rule is needed to determine which one is referred to by the signaling.

Processing Order Option 2: the TS 38.133 specification text for resolving overlap of measurement gaps is applied to the measurement gaps which are enclosed between the end of dynamic signaling plus an offset and the end of a first measurement gap after [the end of dynamic signaling plus an offset] to determine surviving measurement gap. Still using the example, if by TS 38.133, MG-1 is deprioritized, then MG-2 and MG-3 survive out of the TS 38.133 processing, and the dynamic signaling refers to MG-2 in this case. However, if MG-2 and MG-3 are deprioritized in the TS 38.133 processing, MG-1 survives and the dynamic signaling refers to MG-1.

Note in the above, the discussion has been focused on measurement gap only. However, scheduling restriction can be associated with RRM measurement or L1 measurement. Then there are  further combinations to consider.

We also need to point out whether carving out measurement gap/L3-scheduling restriction collision resolution will lead to complications in TS 38.133 specification is not so clear, which points again to the high specification impact with the dynamic signaling based design. For instance, if too many MG/L3-scheduling restriction occasions are adapted, UE may not have enough measurement samples within a measurement period so that accuracy cannot be guaranteed. The potential impact is already being discussed in RAN4.

We have

Observation: changes in RAN1 specifications and RAN4 specifications may be needed to specify the first “instance” to be skipped by dynamic signaling.

Also at RAN1 #118bis, the following was agreed:

Agreement

From RAN1 perspective, the case where an occasion(s) of gap/restrictions that is caused by RRM measurements is cancelled/skipped partially is not supported in Release 19.

| Option 1 behavior |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | DCI-1 | DCI-2 | UE behavior | Open Question |
| Case 1 | 0 | 0 | Neither MG-1 nor MG-2 is skipped | N/A |
| Case 2 | 0 | 1 | MG-2 is skipped | Partial skipping of MG-1 (T2) is needed? |
| Case 3 | 1 | 0 | MG-1 is skipped, but how about T3? | Partial skipping of MG-2(T3) is needed? |
| Case 4 | 1 | 1 | MG-1 and MG-2 are skipped | Should this combination be supported? |

With Option 1,

- With Case 1, no MG is skipped.

- With Case 2, MG-1 is not skipped by the signaling from DCI-1. Also with the RAN1 #118bis agreement, the UE does not expect any of portion of MG-1 is skipped. Then by the signaling form DCI-2, if that is allowed/supported by specification, then MG-2 is skipped. As MG-1 and MG-2 overlap over time interval T2, when MG-2 is skipped in its entirety, a portion of MG-1 is also skipped. Not to complicate UE implementation and also to be consistent with the RAN1 #118bis no partial skipping agreement, we propose Case 2 is not supported.

- With Case 3, by the no partial skipping agreement, skipping extension should be introduced so MG-2 is also skipped by the first signaling with “1”, and the second signaling with “0” does not take any effect with Option 1.

- With Case 4, it should be discussed whether this is supported.

| Option 2 behavior |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | DCI-1 | DCI-2 | UE behavior | Open question |
| Case 1 | 0 | 0 | Neither MG-1 nor MG-2 is skipped | N/A |
| Case 2 | 0 | 1 | MG-2 is skipped | Partial skipping of MG-1 (T2) is needed? |
| Case 3 | 1 | 0 | MG-1 is skipped, but how about T2? | Partial cancellation is MG-1 (T1) is supported? |
| Case 4 | 1 | 1 | MG-1 and MG-2 are skipped | N/A |

With Option 2,

- With Case 1, no MG is skipped.

- With Case 2, MG-1 is not skipped by the signaling from DCI-1. Also with the RAN1 #118bis agreement, the UE does not expect any of portion of MG-1 is skipped. Then by the signaling form DCI-2, if that is allowed/supported by specification, then MG-2 is skipped. As MG-1 and MG-2 overlap over time interval T2, when MG-2 is skipped in its entirety, a portion of MG-1 is also skipped. Not to complicate UE implementation and also to be consistent with the RAN1 #118bis no partial skipping agreement, we propose Case 2 is not supported.

- With Case 3, by the no partial skipping agreement, this combination is not supported.

- With Case 4, it should be discussed whether this is supported.

By comparing Option 1 and Option 2, there are more exclusion cases for Case 2. It is foreseeable for more complicated cases, Option 2 will have more exclusion cases to handle and may even create ambiguous cases which need specification clarifications.

Essentially the UE processing due to TS 38.133 and TS 38.213 need to be handled. TS 38.133 handles the prioritization among MGs and scheduling restriction, and TS 38.213 handles the removal of a MG/scheduling restriction. Assume on a CC a UE receives a skipping signaling (with Option 1 and Option 2) for the first time since the UE has entered the connected mode. There are two choices in defining UE behavior as Processing Order Option 1 and Processing Order Option 2.

If Processing Order Option 1 is taken, then the first gap relevant to the scheduled cell after PDCCH+lead time is identified first, then the first gap is marked as skippable (non-present), afterwards the TS 38.133 specification text for resolving overlap of measurement gaps is applied to the remaining measurement gaps if any to determine surviving measurement gaps if any. For this “RAN1-spec first” approach, the collision handling as specified in TS 38.133 is not run yet, at least for two or MG occasions starting at the same time (and in general for two more overlapping MGs), a rule is needed to determine which one is referred to by the signaling. There can be several alternatives. We propose the following are considered:

Proposal 2: when MGs are eligible for adaptation but L1-scheduling restriction are not eligible for adaptation, consider the following

·       Alt. 1: among MG occasions start at the same time, the MG occasion with the shortest duration is referred by the dynamic signaling (e.g., MG-2)

·       Alt. 2: among MG occasions start at the same time, the MG occasion with the longest duration is referred by the dynamic signaling (e.g., MG-1)

·       Alt. 3: among MG occasions overlapping one another, the MG occasion with the shortest duration is referred by the dynamic signaling;

·       Alt. 4: among MG occasions overlapping one another, the MG occasion with the longest duration is referred by the dynamic signaling;

·       Alt. 5: among MG occasions overlapping one another, the MG occasion which finishes earliest is referred by the dynamic signaling;

·       Alt. 6: among MG occasions overlapping one another, the MG occasion which finishes last is referred by the dynamic signaling;

·       Alt. 7: among MG occasions overlapping one another, the MG occasion which starts earliest is referred by the dynamic signaling;

·       Alt. 8: among MG occasions overlapping one another, the MG occasion which starts last is referred by the dynamic signaling

To avoid partial skipping, consider the following:

Alt. 1:

UE does not expect to receive signaling from NW which would lead to partial skipping of MG/scheduling restriction.

- Consider two cases:

  - A MG/scheduling restriction which is previously indicated as “skipped” cannot be “revived” by a later signaling

  - A MG/scheduling restriction which is not skipped cannot be skipped due to UE’s skipping of an overlapping MG/scheduling restriction.

Alt. 2:

If NW signaling would lead to partial skipping of MG/scheduling restriction, it is up to UE implementation to handle it.

Alt. 3:

- Once a MG/scheduling restriction is indicated as skipping, it cannot be partially un-skipped by a later skipping signaling.

- And a second MG/scheduling restriction which partially/fully overlaps with a first skipped MG/scheduling restriction is also skipped.

- An algorithmic description for Alt. 3 with set S can be found below:

  - The UE starts with a skipped MG/scheduling restriction (instance-1), and a set S consisting of OFDM symbols/time duration of instance-1, If there is any skippable MG/scheduling restriction (designated as instance-2) which overlaps with S, then S <= Union of (S, instance-2), … until S cannot be expanded further. If TS 38.133 processing has not been conducted over S, then TS 38.133 processing is conducted on S. Then S is the time unit for the skipping signaling. Then UE does not expect to receive a signaling to indicate “not skipped” for any duration fully or partially covered by S.

  - Specifically for Option 2, NW signaling cannot revert a MG/scheduling skipping which is covered in a proper superset S.

    - In the example below, SMTC-2 and MG-2 cannot be “revived” by a revert signaling.

    - In an optimization, only MG/scheduling restriction at the beginning of a superset S can be “revived” by a revert signaling (e.g., MG-1 is at the beginning of the superset S, thus eligible for “being revived”).

Figure 7 Example of "set S"

We have

Proposal 3: to handle potential partial skipping situations, consider

Alt. 1:

UE does not expect to receive signaling from NW which would lead to partial skipping of MG/scheduling restriction.

- Consider two cases:

  - A MG/scheduling restriction which is previously indicated as “skipped” cannot be “revived” by a later signaling

  - A MG/scheduling restriction which is not skipped cannot be skipped due to UE’s skipping of an overlapping MG/scheduling restriction.

Alt. 2:

If NW signaling would lead to partial skipping of MG/scheduling restriction, it is up to UE implementation to handle it.

Alt. 3:

- Once a MG/scheduling restriction is indicated as skipping, it cannot be partially un-skipped by a later skipping signaling.

- And a second MG/scheduling restriction which partially/fully overlaps with a first skipped MG/scheduling restriction is also skipped.

Alt. 4 (An algorithmic description for Alt. 3), an operational procedure to derive set S can be found below:

- The UE starts with a skipped MG/scheduling restriction (instance-1), and a set S consisting of OFDM symbols/time duration of instance-1, If there is any skippable MG/scheduling restriction (designated as instance-2) which overlaps with S, then S <= Union of (S, instance-2), … until S cannot be expanded further. If TS 38.133 processing has not been conducted over S, then TS 38.133 processing is conducted on S. Then S is the time unit for the skipping signaling. Then UE does not expect to receive a signaling to indicate “not skipped” for any duration fully or partially covered by S.

- Specifically for Option 2, NW signaling cannot revert a MG/scheduling skipping which is covered in a proper superset S.

  - In the example below, SMTC-2 and MG-2 cannot be “revived” by a revert signaling.

  - In an optimization, only MG/scheduling restriction at the beginning of a superset S can be “revived” by a revert signaling (e.g., MG-1 is at the beginning of the superset S, thus eligible for “being revived”).

# Discussion on “valid downlink slot”

In TS 38.214, regarding CSI reference resource determination, a so-called “valid downlink slot” is defined. A valid downlink slot cannot be inside a measurement gap. Now with the skipping/adaptation of measurement gap, whether a slot which is not a “valid downlink slot” previously due to its overlap with a measurement gap is changed into a “valid downlink slot” needs to be discussed and decided. If dynamic signaling based solution is adopted, to avoid out-of-sync due to missed DCI which triggers RRM measurement adaptation, and to avoid too much impact to modem implementation due to support of the Rel-19 XR feature. During draft revision on TS 38.214 v19 for XR, there were discussions on the treatment of “valid downlink slot”.  We have

- Proposal 4: For the dynamic signaling for RRM measurement adaptation for XR, regarding the designation of “valid downlink slot”, consider

  - Alt. 1: when a slot is designated as not a “valid downlink slot” due to overlap with a configured measurement gap occasion, and that measurement gap occasion is skipped due to RRM measurement adaptation for XR, the slot is NOT converted into a “valid downlink slot”

  - Alt. 2: when a slot is designated as not a “valid downlink slot” due to overlap with a configured measurement gap occasion, and that measurement gap occasion is skipped due to RRM measurement adaptation for XR,   for the basic UE feature supporting RRM measurement adaptation, the slot is NOT converted into a “valid downlink slot” and the slot may be eligible to be designated as “valid downlink slot” subject to additional UE capability.

# Conclusion

Proposal 1: To facilitate adaptation of legacy MGs, Network Controlled Small Gaps, pre-configured MG, MGs for multi-SIM, MGs for positioning with a unified signaling design, MGs can be indexed. A MG index set consists of one or more MG index, MGs referred by a MG index set can be skipped through a single NW indication.

Proposal 2: when MGs are eligible for adaptation but L1-scheduling restriction are not eligible for adaptation, consider the following

·       Alt. 1: among MG occasions start at the same time, the MG occasion with the shortest duration is referred by the dynamic signaling (e.g., MG-2)

·       Alt. 2: among MG occasions start at the same time, the MG occasion with the longest duration is referred by the dynamic signaling (e.g., MG-1)

·       Alt. 3: among MG occasions overlapping one another, the MG occasion with the shortest duration is referred by the dynamic signaling;

·       Alt. 4: among MG occasions overlapping one another, the MG occasion with the longest duration is referred by the dynamic signaling;

·       Alt. 5: among MG occasions overlapping one another, the MG occasion which finishes earliest is referred by the dynamic signaling;

·       Alt. 6: among MG occasions overlapping one another, the MG occasion which finishes last is referred by the dynamic signaling;

·       Alt. 7: among MG occasions overlapping one another, the MG occasion which starts earliest is referred by the dynamic signaling;

·       Alt. 8: among MG occasions overlapping one another, the MG occasion which starts last is referred by the dynamic signaling

Proposal 3: to handle potential partial skipping situations, consider

Alt. 1:

UE does not expect to receive signaling from NW which would lead to partial skipping of MG/scheduling restriction.

- Consider two cases:

  - A MG/scheduling restriction which is previously indicated as “skipped” cannot be “revived” by a later signaling

  - A MG/scheduling restriction which is not skipped cannot be skipped due to UE’s skipping of an overlapping MG/scheduling restriction.

Alt. 2:

If NW signaling would lead to partial skipping of MG/scheduling restriction, it is up to UE implementation to handle it.

Alt. 3:

- Once a MG/scheduling restriction is indicated as skipping, it cannot be partially un-skipped by a later skipping signaling.

- And a second MG/scheduling restriction which partially/fully overlaps with a first skipped MG/scheduling restriction is also skipped.

Alt. 4 (An algorithmic description for Alt. 3), an operational procedure to derive set S can be found below:

- The UE starts with a skipped MG/scheduling restriction (instance-1), and a set S consisting of OFDM symbols/time duration of instance-1, If there is any skippable MG/scheduling restriction (designated as instance-2) which overlaps with S, then S <= Union of (S, instance-2), … until S cannot be expanded further. If TS 38.133 processing has not been conducted over S, then TS 38.133 processing is conducted on S. Then S is the time unit for the skipping signaling. Then UE does not expect to receive a signaling to indicate “not skipped” for any duration fully or partially covered by S.

- Specifically for Option 2, NW signaling cannot revert a MG/scheduling skipping which is covered in a proper superset S.

  - In the example below, SMTC-2 and MG-2 cannot be “revived” by a revert signaling.

  - In an optimization, only MG/scheduling restriction at the beginning of a superset S can be “revived” by a revert signaling (e.g., MG-1 is at the beginning of the superset S, thus eligible for “being revived”).

- Proposal 4: For the dynamic signaling for RRM measurement adaptation for XR, regarding the designation of “valid downlink slot”, consider

  - Alt. 1: when a slot is designated as not a “valid downlink slot” due to overlap with a configured measurement gap occasion, and that measurement gap occasion is skipped due to RRM measurement adaptation for XR, the slot is NOT converted into a “valid downlink slot”

  - Alt. 2: when a slot is designated as not a “valid downlink slot” due to overlap with a configured measurement gap occasion, and that measurement gap occasion is skipped due to RRM measurement adaptation for XR,   for the basic UE feature supporting RRM measurement adaptation, the slot is NOT converted into a “valid downlink slot” and the slot may be eligible to be designated as “valid downlink slot” subject to additional UE capability.

# Reference

- R1-2306379,	LS on new DRX cycles in rational numbers, RAN2, Qualcomm, Rel-18	NR_XR_enh-Core, To: RAN1, RAN4