3GPP TSG RAN WG1 #121			R1-2504359

St Julian’s, Malta, May 19th – 23th, 2025

Agenda Item:	9.15.10

Source:	Apple

Title:	On Rel-19 IoT-NTN TDD mode UE Features

Document for:	Discussion/Decision

# Introduction

According to Rel-19 IoT NTN TDD mode WID [1], RAN1 related objective includes specifying new NB-IoT TDD NTN mode. The following agreements on UE features were made in last RAN1 meeting.

Agreement:

Introduce the following FG for IoT-NTN TDD mode

| Index | Feature group | Components | Prerequisite feature groups | Need for the eNB to know if the feature is supported | Need for the UE to know if the feature is supported (only for V2X WI, where the PC5-RRC capability signalling is delivered between the UEs) | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Capability interpretation for mixture of FDD/TDD | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2-1 | Support of IoT-NTN TDD mode | 1. Support a TDD pattern with periodicity of N=9 radio frames, which consists of D=8 contiguous DL subframes and U=8 contiguous UL subframes. The guard period between the end of the DL subframes and the beginning of the UL subframes is fixed to be 50 ms at the ULSRP 2. The DL subframes of pattern are fixed to subframes [3 4 5 6 7 8 9 0] across two consecutive radio frames. 3. The non-U NB-IoT subframes are not considered by the UE as “NB-IoT UL subframes” 4. The non-D NB-IoT subframes are not considered by the UE as “NB-IoT DL subframes” 5. Support NPSS/NSSS/NPBCH transmissions dropped in non-D NB-IoT subframes 6. Postponement of NPRACH transmissions in non-U NB-IoT subframes until the next U NB-IoT subframe(s), where the unit of postponement is one PRU. [FFS Whether/how to capture components in Rel-17 2-1b, in case the FG is not defined to be the prerequisite one.] [FFS other component(s)/FG(s)] | [Rel-17 2-1b] | Yes | N/A | IoT-NTN TDD mode is not supported | [Per UE or Per band] | [TDD only] | [N/A] | Applicable only for 1616-1626.5 MHz MSS allocated band | FFS |

In this contribution, we share our views on Rel-19 IoT-NTN TDD mode RAN1 UE features.

# Discussion

Based on the progress on Rel-19 IoT-NTN TDD mode UE feature, our views are shared in below table.

| Index | Feature group | Components | Prerequisite feature groups | Need for the eNB to know if the feature is supported | Need for the UE to know if the feature is supported (only for V2X WI, where the PC5-RRC capability signalling is delivered between the UEs) | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Capability interpretation for mixture of FDD/TDD | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2-1 | Support of IoT-NTN TDD mode | 1. Support a TDD pattern with periodicity of N=9 radio frames, which consists of D=8 contiguous DL subframes and U=8 contiguous UL subframes. The guard period between the end of the DL subframes and the beginning of the UL subframes is fixed to be 50 ms at the ULSRP 2. The DL subframes of pattern are fixed to subframes [3 4 5 6 7 8 9 0] across two consecutive radio frames. 3. The non-U NB-IoT subframes are not considered by the UE as “NB-IoT UL subframes” 4. The non-D NB-IoT subframes are not considered by the UE as “NB-IoT DL subframes” 5. Support NPSS/NSSS/NPBCH transmissions dropped in non-D NB-IoT subframes 6. Postponement of NPRACH transmissions in non-U NB-IoT subframes until the next U NB-IoT subframe(s), where the unit of postponement is one PRU. | Rel-17 2-1b | Yes | N/A | IoT-NTN TDD mode is not supported | Per band | TDD only | No | Applicable only for 1616-1626.5 MHz MSS allocated band | Optional with capability signalling For UEs supporting NB-IoT NTN TDD mode, it must indicate this FG is supported |

Proposal 1: Adopt the IoT-NTN TDD mode UE features in above table.

# Conclusion

In this contribution, we provided our views on Rel-19 IoT-NTN TDD mode UE features. Our proposal is as follows:

Proposal 1: Adopt the IoT-NTN TDD mode UE features in above table.

# References

- RP-243293, “Revised WID on introduction of IoT-NTN TDD mode” Dec. 2024

- R1-2503201, “Report of RAN1#120bis meeting”, May 2025