3GPP TSG RAN WG1 #121			R1-2504477

St Julian’s, Malta, May 19th – 23rd, 2025

Source:	Sharp

Title:	Discussion on Reply to LS on simultaneous configuration of SBFD and DC

Agenda Item:	5

Document for:	Discussion and Decision

# Background

RAN1 has received a LS from RAN2 on simultaneous configuration of SBFD and DC [1]. The question is copied below.

RAN2 has discussed whether SBFD and dual connectivity (DC) can be configured simultaneously for an SBFD-aware UE. However, no consensus has been reached. RAN2 would like to seek inputs from RAN1 by asking the following question: Question: Whether SBFD and DC can be configured simultaneously, and whether there is any issue with such configuration for an SBFD-aware UE?

# Discussion

From RAN1 perspective, there are no issues with the simultaneous configuration of SBFD and DC since most DC procedures are transparent to the RAN1 specifications. An exception is power control, where the UE's transmit power is shared between the two cell groups when the UL or flexible symbols of the two groups overlap in time. Although in the legacy, the determination of symbol overlap is based on the semi-static TDD UL/DL configuration, UL sub-band in semi-static DL symbols should also be taken into account for power-sharing. However, we believe this minor adjustment would not hinder support for the simultaneous configuration of SBFD and DC.

On the other hand, in RAN1#120 meeting, RAN1 agreed support for simultaneous configuration of SBFD and CA [2].

Agreement Support SBFD operation on one TDD carrier in multi-carrier scenario. · Note: If the TDD carrier for SBFD operation is an Scell, support UE dedicated signaling for cell-specific configuration of time and frequency location of SBFD subbands for the Scell. · FFS whether/how to handle half-duplex CA case

As in the agreement, RAN1 agreed to limit the number of SBFD carriers to one in multi-carrier scenario. The reason behind that is to reduce UE implementation complexity. Therefore, it is suggested to apply the same limitation for DC, i.e., support SBFD operation on one carrier in DC scenario.

In our view, SBFD can support all the DC options (i.e., EN-DC, NE-DC, and NR-DC) since there is no reasons to restrict the options from RAN1 perspective.

Proposal 1: Support SBFD operation on one carrier in EN-DC, NE-DC, and NR-DC.

Proposal 2: Reply to RAN2 that:

- There is no issue to prevent us from supporting simultaneous configuration of SBFD and DC.

- However, the number of carriers with SBFD configuration should be one across cell groups.

# Conclusion

In this contribution, we have the following proposals:

Proposal 1: Support SBFD operation on one carrier in EN-DC, NE-DC, and NR-DC.

Proposal 2: Reply to RAN2 that:

- There is no issue to prevent us from supporting simultaneous configuration of SBFD and DC.

- However, the number of carriers with SBFD configuration should be one across cell groups.

# References

- R1-2503615, LS on simultaneous configuration of SBFD and DC, RAN2, May 2025

- RAN1 chairman’s note at RAN1#120 meeting, February 2025