3GPP TSG RAN WG1 #121			R1-2504533

St Julian's, Malta, 19 - 23 May, 2025

Source:	NTT DOCOMO, INC.

Title:	Discussion on UE features for Low band carrier aggregation via switching

Agenda Item:	9.15.12

Document for: 	Discussion and Decision

- Introduction

At the RAN#106 meeting, the new WI on low NR band carrier aggregation via switching was approved [1], and based on the WID objective, RAN1 can/should discuss and if necessary define RAN1 UE capabilities on top of RAN4/2 UE capabilities.

Introduce physical layer procedures and requirements to enable low band carrier aggregation via switching according to the following objectives: - Specify UE requirements, including at least switching gap (if needed), and corresponding physical layer procedures to allow switching between {case 1, case 2} [RAN4, RAN1] - Case 1: Tx/Rx on FDD carrier 1 and no Rx on SDL carrier 2 - Case 2: Rx on SDL carrier 2 and no Tx/Rx on FDD carrier 1 - RAN1 to specify only a semi-static switching pattern based on RRC configuration, liaising with RAN2 and RAN4 as necessary - Specify the switching delay and time mask for carrier switching [RAN4] - Specify necessary RRM requirements [RAN4] - Define the corresponding UE capabilities [RAN4, RAN2, RAN1] - Consider the following deployment constraints: - The carrier frequency for all cases is <1 GHz - Co-located and synchronized network deployment for both carriers - Both carriers are in a single TAG - SCS 15KHz on both carriers Note 1: Specify requirements for the feature with the following example band combination: CA_n5A-n29A in this WI, with additional band combinations to be handled via the basket work item approach Note 2: Strive to minimize the RAN1 impact

At the RAN1#120bis meeting, RAN1 made following agreements [2].

Agreement - For Rel-19 low NR band carrier aggregation via switching, support only semi-static configuration of switching pattern to a UE based on UE-specific RRC configuration between the following two cases, - Case 1: Tx/Rx on FDD carrier 1 and no Rx on SDL carrier 2 - Case 2: Rx on SDL carrier 2 and no Tx/Rx on FDD carrier 1 - FDD carrier 1 is PCell and SDL carrier 2 is SCell. - SCS 15KHz on both carriers. Agreement For RRC configuration of semi-static switching pattern in Rel-19 low NR band carrier aggregation via switching, it is not expected that the same slot (1ms) contains both symbol(s) configured as FDD carrier and symbol(s) configured as SDL carrier. Agreement For Rel-19 low NR band carrier aggregation via switching, the semi-static switching pattern and corresponding switching gap are based on the downlink timing (DL reception time at UE side) of Pcell. Agreement For RRC configuration of semi-static switching pattern in Rel-19 low NR band carrier aggregation via switching, the time unit (resolution) of the semi-static switching pattern configuration is slot (1ms for 15kHz SCS). Agreement - For RRC configuration of semi-static switching pattern in Rel-19 low NR band carrier aggregation via switching, support bitmap design - NW configures a periodicity, denoted as P slots, and a bitmap of P bits. Each bit in the bitmap indicates which carrier is configured for the corresponding slot, ‘0’ indicates FDD carrier 1, and ‘1’ indicates SDL carrier 2. - The pattern starts from the beginning of SFN 0 of PCell. - Restriction of maximum X switch(es) within Y slot(s). - FFS: detailed value and relation of X and Y. - FFS: other restriction. Agreement For RRC configuration of semi-static switching pattern in Rel-19 low NR band carrier aggregation via switching, regarding the location of the switching gap, - For switching from SDL carrier to FDD carrier, the switching gap is always assumed at the “switch from” carrier, i.e., SDL carrier. - For switching from FDD carrier to SDL carrier, down-select one of the following solutions: - Alt 1: Switching gap is always assumed at the “switch from” carrier, i.e., FDD carrier. - Alt 2: NW configures whether the switching gap is at the FDD carrier or SDL carrier. - Alt 3: Switching gap is always assumed at the SDL carrier. Agreement For RRC configuration of semi-static switching pattern in Rel-19 low NR band carrier aggregation via switching, semi-static switching pattern is periodic with periodicity of P slots (ms), support P= 40 ms. Agreement For RRC configuration of semi-static switching pattern in Rel-19 low NR band carrier aggregation via switching, the duration of the switching gap is RRC configured by the NW, - Switching gap duration can be different for FDD to SDL switch, and SDL to FDD switch. - Note: NW ensures that the switching gap is enough to cover at least the switching period as in RAN4 LS (R1-2501702) and UL TA (if needed). - For the UL TA, - Switching gap for SDL to FDD switch ends at the end of slot on SDL. - Note: RAN1 assumes that switching gap for SDL to FDD switch includes UL TA. - FFS: whether it has specification impact.

In this contribution, we discuss on UE features for low NR band carrier aggregation via switching from RAN1 perspective.

- Discussions

Since this WI is led by RAN4, RAN1 should discuss whether there is any necessary RAN1 UE feature(s) based on RAN1 agreements. In other words, RAN1 should not discuss any UE capabilities based on RAN2/4 agreements unless requested by RAN2/4.

Based on RAN4 agreements, RAN4/2 will discuss and define UE capabilities to report the support of LBCA via switching, supported switching period, and so on.

On the other hand, following can be observed based on RAN1 agreements made at RAN1#120bis meeting.

- Supported switching pattern would not require UE capability reporting. UE supporting LBCA via switching will support RRC configuration of switching pattern based on RAN1 agreed design and restriction(s), i.e., it should be part of LBCA basic FG to be defined by RAN4/2.

- Regarding switching period location, although RAN1 is still discussing following three alternatives including NW configuration between FDD carrier and SDL carrier for the case of switching from FDD to SDL, we think it would not require UE capability reporting. If RAN1 takes Alt.2, UE supporting LBCA via switching will support such configurability.

- For switching from FDD carrier to SDL carrier, down-select one of the following solutions: - Alt 1: Switching gap is always assumed at the “switch from” carrier, i.e., FDD carrier. - Alt 2: NW configures whether the switching gap is at the FDD carrier or SDL carrier. - Alt 3: Switching gap is always assumed at the SDL carrier.

- Regarding switching period duration, although RAN1 agreed RRC configuration of the duration, we think it would not require additional UE capability reporting on top of supported switching period to be defined by RAN4.

Therefore, we think there is no necessary RAN1 UE feature for LBCA via switching so far according to RAN1 agreements made at RAN1#120bis meeting.

Observation 1: There is no necessary RAN1 UE feature for LBCA via switching so far according to RAN1 agreements made at RAN1#120bis meeting.

Based on some on-going discussions on remaining RAN1 issues, there may be possibility to introduce RAN1 UE features for LBCA via switching. However, we generally prefer to avoid such a situation because of the following reasons.

- WID objective clearly states that “Strive to minimize the RAN1 impact”. So, basically any advanced feature which require separate UE capability from the basic support of LBCA via switching should be discouraged.

- For a certain necessary/essential feature, if RAN1 agrees to introduce multiple options so that UE can report the supported option(s), NW operation would be complicated to handle UEs with different capabilities.

Observation 2: It is generally preferable to have no RAN1 UE feature for LBCA via switching to minimize the RAN1 impact and operational complexity.

Nevertheless, we see some potential RAN1 UE features as below depending on further RAN1 discussion/decision in AI9.12.2.

- For collision handling between semi-static switching pattern and configured/scheduled transmission/reception, there may be possibility to support the collision handling capability as optional (as compromise). In such case, NW configuration restriction can be alleviated for UEs supporting this optional capability, and hence from operational complexity perspective, it may be better than the case where NW configuration restriction to completely avoid the collision applies to all UEs.

- For processing timeline relaxation due to switching, if some UEs can keep existing processing timeline even when switching is performed while some other UEs need the processing timeline relaxation due to switching, there may be possibility to support the processing timeline capability reporting (as compromise). Compared with applying the processing timeline relaxation to all UEs supporting LBCA via switching, reusing existing processing timelines may be good to minimize NW scheduling impact, and it can be a possible choice for NW whether to accommodate UEs requiring the processing timeline relaxation for LBCA operation.

- Conclusion

In this contribution, we discussed on UE features for low NR band carrier aggregation via switching from RAN1 perspective. We provided following observations:

Observation 1: There is no necessary RAN1 UE feature for LBCA via switching so far according to RAN1 agreements made at RAN1#120bis meeting.

Observation 2: It is generally preferable to have no RAN1 UE feature for LBCA via switching to minimize the RAN1 impact and operational complexity.

References

- RP-250247		Revised WID on Low NR band carrier aggregation via switching		AT&T

- RAN1 Chair’s Notes	RAN1#120bis