3GPP TSG RAN WG1 #121						        R1- 2504563

St Julian’s, Malta, May 19th – 23rd, 2025

Agenda Item:	9.11.3

Source: 	LG Electronics

Title: 	Discussion on NR-NTN uplink capacity/throughput enhancement

Document for:	Discussion and decision

- Introduction

In RAN#104, the Rel-19 NTN WI objectives for UL capacity/cell throughput enhancement for FR1-NTN were revised as follows [1]:

2. Uplink Capacity/Cell Throughput Enhancement for FR1-NTN [RAN1, RAN2, RAN4] - Specify Orthogonal Cover Codes (OCC) for DFT-s-OFDM PUSCH at least for multiplexing 2 or 4 UEs when PUSCH repetitions are used - Specify necessary signalling - Update RF requirements accordingly, if needed - Notes for this objective: - The enhancement is not targeting improvements/impacts of MU-MIMO capability - The enhancement is not targeted to PUSCH DMRS - No enhancement for initial access - Enhancements to PRACH are not in scope. - This feature may be applicable for UEs operating in terrestrial networks based on a common design

In RAN1 #119 meeting [2], it was agreed to support OCC length 2/4 with inter-slot OCC to multiplex up to 2/4 UEs. Accordingly, this contribution discusses potential specification aspect(s) when supporting inter-slot OCC for DFT-s-OFDM based PUSCH transmission in FR1 NR NTN FDD system.

- Discussion

  - Performance of OCC Techniques

In RAN1 #120bis meeting [3], there was discussion about the requirements for phase continuity and power consistency within an OCC group and the following agreements were made:

Agreement RAN1 assumes that the UE is required to maintain phase continuity and power consistency for the duration of one OCC group with PUSCH. - FFS: under which conditions the above applies, e.g. under the same conditions that phase continuity applies for DMRS bundling Agreement Send LS to RAN4 on requirements for the phase continuity and power consistency: RAN1 has agreed the following: - RAN1 assumes that the UE is required to maintain phase continuity and power consistency for the duration of one OCC group with PUSCH. - FFS: under which conditions the above applies, e.g. under the same conditions that phase continuity applies for DMRS bundling RAN1 ask RAN4 whether the same phase continuity requirements as for DMRS bundling can be applied for OCC length 2 and/or OCC length 4 under the same conditions as for DMRS bundling, or if new requirements are needed. Agreement The draft LS in R1-2503070 is endorsed. Final LS is in R1-2503071.

Regarding the phase consistency and power consistency for inter-slot OCC with PUSCH repetitions, RAN1 needs to discuss the conditions (denoted as “event” here after) under which the phase consistency and power consistency are not maintained. For example, TS 38.214 section 6.1.7 [5] defines the following events related to DM-RS bundling:

Events which cause power consistency and phase continuity not to be maintained across PUSCH transmissions of PUSCH repetition type A scheduled by DCI format 0_1 or 0_2, or PUSCH repetition Type A with a configured grant, or PUSCH repetition type B or TB processing over multiple slots, or PUCCH transmissions of PUCCH repetition, within the nominal TDW, are: - A downlink slot or downlink reception or downlink monitoring based on tdd-UL-DL-ConfigurationCommon and tdd-UL-DL-ConfigurationDedicated for unpaired spectrum. - The gap between any two consecutive PUSCH transmissions, or the gap between any two consecutive PUCCH transmissions, exceeds 13 symbols for normal cyclic prefix or exceeds 11 symbols for extended cyclic prefix. - The gap between any two consecutive PUSCH transmissions, or the gap between any two consecutive PUCCH transmissions, does not exceed 13 symbols but other uplink transmissions are scheduled between the two consecutive PUSCH transmissions or the two consecutive PUCCH transmissions. - For PUSCH transmissions of PUSCH repetition type A, or PUSCH repetition type B or TB processing over multiple slots, a dropping or cancellation of a PUSCH transmission according to clause 9, clause 11.1 and clause 11.2A of [6, TS 38.213]. - For PUCCH transmissions of PUCCH repetition, a dropping or cancellation of a PUCCH transmission according to clause 9, clause 9.2.6 and clause 11.1 of [6, TS 38.213]. - For any two consecutive PUSCH transmissions of PUSCH repetition type A, or PUSCH repetition type B, and when two SRS resource sets are configured in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with higher layer parameter usage in SRS-ResourceSet set to 'codebook' or 'noncodebook', a different SRS resource set association is used for the two PUSCH transmissions of PUSCH repetition type A, or PUSCH repetition type B, according to Clause 6.1.2.1. - For any two consecutive PUCCH transmissions of PUCCH repetition, and when a PUCCH resource used for repetitions of a PUCCH transmission by a UE includes first and second spatial relations or first and second sets of power control parameters, as described in [10, TS 38.321] and in clause 7.2.1 of [6, TS 38.213], different spatial relations or different power control parameters are used for the two PUCCH transmissions of PUCCH repetition, according to Clause 9.2.6 of [6, TS 38.213]. - Uplink timing adjustment in response to a timing advance command according to clause 4.2 of [6, TS 38.213]. - Frequency hopping. - For reduced capability half-duplex UEs, - a dropping or cancellation of a PUSCH or PUCCH transmission according to clause 17.2 of [6, TS 38.213] or - an overlapping of the gap between two consecutive PUSCH or two consecutive PUCCH transmissions and any symbol of downlink reception or downlink monitoring

When inter-slot OCC is applied for PUSCH repetitions, it is desirable to prevent changes of phase continuity and/or power consistency due to UE operation within an OCC group. This is because the loss of multi-UE throughput due to phase dis-continuity and/or power in-consistency will be greater than the loss of single-UE throughput due to delayed UE operation. For the above UE operations that can change phase continuity and power consistency, UE operations based on the network configurations and/or indications are not under the control of the UE and therefore should be defined as events similar to DM-RS bundling case. On the other hand, UE operations that allow the UE to autonomously adjust its execution time are under the control of the UE and the UE can be restricted from performing such operations within an OCC group to maintain the orthogonality of the OCC. For example, in a NR-NTN scenario, UE specific TA update, common TA update, and UE pre-compensation update (e.g., GNSS position and/or ephemeris update) can be restricted from being performed within in an OCC group.

Proposal #1: To maintain phase continuity and power consistency for OCC with PUSCH, UE does not perform the followings UE operations within an OCC group:

- UE specific and common TA update

- UE pre-compensation update

If RAN4 responds that the phase continuity requirements for DM-RS bundling can be also applied to OCC with PUSCH as well, it may be desirable to define the conditions for maintaining phase continuity and power consistency for OCC similarly to the DM-RS bundling case. This is because the frameworks defined to support DM-RS bundling can be easily extended to OCC as well, reducing the standardization workload. For example, (nominal and/or actual) TDW for OCC can be defined and utilized as for DM-RS bundling case. And the definition of event as in TS 38.214 can be shared between DM-RS bundling case and OCC case. Specifically, if there is no event within an OCC group, UE can maintain phase continuity and power consistency for the duration of one OCC group with PUSCH.

Proposal #2: If the phase continuity requirements for DM-RS bundling can be applied for OCC with PUSCH, reuse the frameworks of DM-RS bundling for OCC with PUSCH.

  - UCI multiplexing

In RAN1 #120bis meeting [3], there was discussion about UCI multiplexing or prioritization for OCC with PUSCH and the following agreements and working assumptions were made:

Agreement For resolving the overlapping PUSCH repetitions with inter-slot OCC and PUCCH with/without repetitions, the legacy timeline conditions for UCI multiplexing or prioritization for dropping PUSCH [/ PUCCH] applies according with the following update: - “the first PUSCH repetition of an OCC group” is used instead of “PUSCH repetition” for the legacy timeline condition, where the legacy PUSCH repetition that overlaps with a PUCCH belongs to the OCC group Note: how to capture the above agreement is up to the spec editor. Agreement If PUCCH without repetitions overlaps with inter-slot OCC with any PUSCH repetitions in an OCC group with a same priority index for PUCCH/PUSCH, the legacy conditions and rules for UCI multiplexing or prioritization for dropping applies with the following updates: - If the UCI is multiplexed on the PUSCH repetition according to legacy rules and the updated timeline conditions for UCI multiplexing are satisfied, UCI is multiplexed on all PUSCH repetitions without A-CSI reports within an OCC group with inter-slot OCC overlaps with the PUCCH. (Option 3-a) - FFS: PUSCH repetition with A-CSI reports - If the PUSCH repetition is dropped according to legacy rules and the updated timeline conditions for PUSCH dropping are satisfied, UCI is transmitted on PUCCH and all PUSCH repetitions within the OCC group that overlaps with the PUCCH are dropped (Option 2) - FFS: if PUCCH is overlap with PUSCH repetition in both time and frequency domain. - UE does not expect there are multiple PUCCHs without repetitions in different PUCCH slots with a same or different UCI types other than SR overlapping with multiple PUSCH repetitions in the same OCC group. - FFS: whether the above applies only when at least one of the overlapping PUCCHs result in a UCI being multiplexed on the PUSCH Note 1: If the UCI on the PUCCH is dropped according to legacy rules and [updated] timeline conditions for UCI dropping are satisfied, there is no [additional] spec impact. (Option 1) Note 2: There can be multiple PUCCHs with same or different UCI types in the same slot (i.e. CSI report and HARQ-ACK) as in the legacy specifications Working assumption 1: The above agreement applies to different priority indexes for PUCCH/PUSCH if no additional specification impact is identified. Working assumption 2: The above agreement applies to PUCCH with repetitions if no additional specification impact is identified.

For UL prioritization between PUSCH and PUCCH, timeline conditions can be also considered for PUCCH dropping case. In TS 38.213 clause 9 [6], when a UE determines overlapping for PUCCH and/or PUSCH transmissions of different priority indexes and the UE is provided uci-MuxWithDiffPrio, the UE can check the timeline conditions and drop PUCCH with smaller priority index according to the following procedure.

When a UE determines overlapping for PUCCH and/or PUSCH transmissions of different priority indexes, other than PUCCH transmissions with SL HARQ-ACK reports, before considering limitations for transmission due to cell DRX operation or as described in clauses 11.1, 11.1.1, 11.2A, 15 and 17.2 including repetitions if any, if the UE is provided uci-MuxWithDiffPrio and the timeline conditions in clause 9.2.5 for multiplexing UCI in a PUCCH or a PUSCH are satisfied … - else - if the UE would transmit the following channels that would overlap in time where, if a channel transmission is with repetitions, the following are applicable per repetition - a first PUCCH transmission of larger priority index and a second PUCCH transmission of smaller priority index - a first PUCCH transmission of larger priority index and a second PUSCH transmission of smaller priority index when the UE cannot simultaneously transmit the first PUCCH and second PUSCH - a first PUCCH transmission of smaller priority index and a second PUSCH transmission of larger priority index when the UE cannot simultaneously transmit the first PUCCH and second PUSCH the UE - transmits the PUCCH or the PUSCH of the larger priority index subject to the limitations for UE transmissions described in clauses 11.1, 11.1.1, 11.2A, and 15 and - does not transmit a PUCCH or a PUSCH of smaller priority index

Proposal #3: The update of the legacy timeline conditions for UCI multiplexing or prioritization can be applied for both of PUSCH dropping and PUCCH dropping.

For UCI multiplexing rule with a same priority index for PUCCH/PUSCH, there was a discussion that UE processing of UCI multiplexing for multiple slots can increase UE complexity. Therefore, it was agreed that UE does not expect that there is multiple UCI from different slots to be multiplexed on the same OCC group. As discussed for draft CR, the above UCI can be identified as HARQ-ACK or CSI, which is the target of UCI multiplexing. The same design principle can be applied to PUSCH repetition with A-CSI report. That is, RAN1 can assume that UE does not expect transmission of PUCCH without repetitions in slots other than the first slot in which an A-CSI report is transmitted for PUSCH repetition with A-CSI report. Then, UCI multiplexing on PUSCH repetitions in an OCC group (Option 3-2a) can be also applied for PUSCH repetitions with A-CSI report.

Proposal #4: Extend the UCI multiplexing rule for PUSCH repetitions with an OCC group to include PUSCH repetitions with A-CSI report as follows:

- If the UCI is multiplexed on the PUSCH repetition according to legacy rules and the updated timeline conditions for UCI multiplexing are satisfied, UCI is multiplexed on all PUSCH repetitions within an OCC group with inter-slot OCC overlaps with the PUCCH.

- For PUSCH repetitions with A-CSI report, UE does not expect there is any PUCCH without repetitions for HARQ-ACK/CSI other than the first slot overlapping with the PUSCH repetitions in the first OCC group.

For the cases of different priority indexes for PUCCH/PUSCH and PUCCH with repetitions, the agreement should be generalized to cover cases where timeline conditions are not taken into account. In TS 38.213 clause 9 [6], when a UE determines overlapping for PUCCH and/or PUSCH transmissions of different priority indexes and the UE is not provided uci-MuxWithDiffPrio, the UE may cancel the second PUSCH or PUCCH of smaller priority index without consideration on the timeline conditions as follows:

When a UE determines overlapping for PUCCH and/or PUSCH transmissions of different priority indexes, other than PUCCH transmissions with SL HARQ-ACK reports, before considering limitations for transmissions including with repetitions, if any, due to cell DRX operation or as described in clauses 11.1, 11.1.1, 11.2A, 15 and 17.2, if the UE is not provided uci-MuxWithDiffPrio, the UE first resolves overlapping for PUCCH and/or PUSCH transmissions of smaller priority index as described in clauses 9.2.5 and 9.2.6. Then, - if a transmission of a first PUCCH of larger priority index scheduled by a DCI format in a PDCCH reception would overlap in time with a repetition of a transmission of a second PUSCH or a second PUCCH of smaller priority index, the UE cancels the repetition of a transmission of the second PUSCH or the second PUCCH before the first symbol that would overlap with the first PUCCH transmission - if a transmission of a first PUSCH of larger priority index scheduled by a DCI format in a PDCCH reception would overlap in time with a repetition of the transmission of a second PUCCH of smaller priority index, the UE cancels the repetition of the transmission of the second PUCCH before the first symbol that would overlap with the first PUSCH transmission where - the overlapping is applicable before or after resolving overlapping among channels of larger priority index, if any, as described in clauses 9.2.5 and 9.2.6 - any remaining PUCCH and/or PUSCH transmission after overlapping resolution is subjected to the limitations for UE transmission due to cell DRX operation or as described in clauses 11.1, 11.1.1, 11.2A, 15 and 17.2 - the UE expects that the transmission of the first PUCCH or the first PUSCH, respectively, would not start before Tproc,2 after a last symbol of the corresponding PDCCH reception - Tproc,2 is the PUSCH preparation time for a corresponding UE processing capability assuming d2,1= d1 [6, TS 38.214], based on μ and N2 as subsequently defined in this clause, and d1 is determined by a reported UE capability

Proposal #5: Confirm the WAs for UCI multiplexing in RAN1 #120bis with the following updates:

If PUCCH with/without repetitions overlaps with inter-slot OCC with any PUSCH repetitions in an OCC group, the legacy conditions and rules for UCI multiplexing or prioritization for dropping applies with the following updates: - If the UCI is multiplexed on the PUSCH repetition according to legacy rules and the updated timeline conditions for UCI multiplexing are satisfied, UCI is multiplexed on all PUSCH repetitions within an OCC group with inter-slot OCC overlaps with the PUCCH. (Option 3-a) - If the PUSCH repetition is dropped according to legacy rules and the updated timeline conditions for PUSCH dropping are satisfied, if any, UCI is transmitted on PUCCH and all PUSCH repetitions within the OCC group that overlaps with the PUCCH are dropped (Option 2) - UE does not expect there are multiple PUCCHs without repetitions in different PUCCH slots with HARQ-ACK or CSI report overlapping with multiple PUSCH repetitions in the same OCC group. - For PUSCH repetitions with A-CSI report, UE does not expect there is any PUCCH without repetitions for HARQ-ACK or CSI report other than the first slot overlapping with the PUSCH repetitions in the first OCC group. Note 1: If the UCI on the PUCCH is dropped according to legacy rules and updated timeline conditions for UCI dropping are satisfied, if any, there is no additional spec impact. (Option 1) Note 2: There can be multiple PUCCHs with same or different UCI types in the same slot (i.e. CSI report and HARQ-ACK) as in the legacy specifications

Moreover, UCI overhead reduction can also be considered. It is worth noting that according to the existing UCI multiplexing rules, there already exists beta offset parameter(s) (e.g., βoffsetPUSCH) that can adjust the number of coded modulation symbols of UCI. For example, beta offset parameter(s) can be semi-statically configured or dynamically indicated by DCI to control UCI overhead on PUSCH. Therefore, for PUSCH with OCC, the UCI overhead can be controlled by adjusting the beta offset parameter(s). The beta offset parameter(s) for PUSCH with OCC can be independent from the beta offset parameter(s) for PUSCH without OCC.

Proposal #6: RAN1 support separate beta offset parameter(s) for OCC with PUSCH.

  - OCC indication/configuration

In RAN1 #120bis meeting [3], there was discussion about OCC indication/configuration for OCC with PUSCH and the following agreements were made:

Agreement OCC length and OCC sequence for OCC with CG PUSCH Type 1 is configured by RRC higher-layers. - Up to RAN2 whether to signal this with one or two RRC parameters - Note: OCC lengths and sequences to be provided in the table of RRC parameters to be prepared by the WI rapporteur Agreement For OCC with CG-PUSCH, the RV sequence applied across OCC groups is RRC configured among the RV sequences defined for legacy CG PUSCH, i.e., [0,2,3,1], [0,3,0,3] or [0,0,0,0]. - Note: no new RRC parameter is needed for the above - FFS: if OCC group dropping is later agreed, how to count RVs may need to be discussed for that case Agreement For the OCC length applied for OCC DG PUSCH and CG PUSCH Type 2, consider the following options: - Option 1: Configured by RRC higher-layer parameter. - Note: this does not preclude jointly configuring OCC sequence and OCC length with the same RRC configuration - Option 2: Max value is configured by RRC higher-layer parameter, and the applied value is implicitly determined from the configured repetition number. - Option 3: Candidates values are configured by RRC higher-layer parameter, and the applied value is indicated by scheduling DCI for DG PUSCH or by activation DCI for CG PUSCH Type 2. - Note: this does not preclude jointly configuring OCC sequence and OCC length with the same RRC configuration - Option 4: Single value is indicated by scheduling DCI for DG PUSCH and by activation DCI for CG PUSCH Type 2 (no RRC configuration of candidate values). - Option 5: Max value is configured by RRC higher-layer parameter, and the applied value is indicated by scheduling DCI for DG PUSCH and by activation DCI for CG PUSCH Type 2. Note: it is not precluded to select a different option for DG PUSCH and CG PUSCH Type 2. Agreement For the OCC sequence applied for OCC DG PUSCH and CG PUSCH Type 2, the sequence is indicated dynamically in DCI - FFS: with a new field or reusing an existing field

For the OCC parameters for DG PUSCH and CG PUSCH Type 2, it should be noted that inter-slot OCC is applicable only when PUSCH repetition Type A is applied. Therefore, it is desirable to configure the OCC parameters in conjunction with the PUSCH repetition Type A settings. Otherwise, exception handling should be considered for cases where the two settings are inconsistent, and this may result in additional specification implications. For example, exception handling may be required when the OCC length is set to 4 but PUSCH repetition type A is not applied or the PUSCH repetition length is shorter than 4. Therefore, for a concise and consistent design, we prefer to indicate the OCC parameters by re-using fields related to PUSCH repetition Type A. Specifically, the OCC index can be implicitly indicated by DM-RS port and the OCC length can be configured and indicated by using the time domain resource allocation (TDRA) list.

Proposal #7: RAN1 support OCC indication/configuration as follows:

- For the OCC sequence applied for DG PUSCH and CG PUSCH Type 2

  - The sequence is implicitly indicated by reusing DM-RS port field.

- For the OCC length applied for DG PUSCH and CG PUSCH Type 2,

  - Candidate values are configured by higher-layer parameter for TDRA list, and the applied value is indicated by reusing TDRA field in scheduling DCI for DG PUSCH or by activation DCI for CG PUSCH Type 2 (Option 3)

Moreover, consideration should be given to dynamically disabling OCC. If the network cannot dynamically control whether OCC is applied or not, when OCC is configured, inefficiencies may arise when performing UCI multiplexing on all slots within an OCC group, even for single-UE case. For example, a single state of DM-RS port field for the OCC parameters can be reserved for OCC disabling.

Proposal #8: RAN1 support dynamic OCC disabling by reserving a single state of DM-RS port field.

- Conclusions

In this contribution, we discussed UL capacity/throughput enhancement by using OCC in Rel-19 NR NTN and the following proposal(s) are made:

Proposal #1: To maintain phase continuity and power consistency for OCC with PUSCH, UE does not perform the followings UE operations within an OCC group:

- UE specific and/or common TA update

- UE pre-compensation update

Proposal #2: If the phase continuity requirements for DM-RS bundling can be applied for OCC with PUSCH, reuse the frameworks of DM-RS bundling for OCC with PUSCH.

Proposal #3: The update of the legacy timeline conditions for UCI multiplexing or prioritization can be applied for both of PUSCH dropping and PUCCH dropping.

Proposal #4: Extend the UCI multiplexing rule for PUSCH repetitions with an OCC group to include PUSCH repetitions with A-CSI report as follows:

- If the UCI is multiplexed on the PUSCH repetition according to legacy rules and the updated timeline conditions for UCI multiplexing are satisfied, UCI is multiplexed on all PUSCH repetitions within an OCC group with inter-slot OCC overlaps with the PUCCH.

- For PUSCH repetitions with A-CSI report, UE does not expect there is any PUCCH without repetitions for HARQ-ACK/CSI other than the first slot overlapping with the PUSCH repetitions in the first OCC group.

Proposal #5: Confirm the WAs for UCI multiplexing in RAN1 #120bis with the following updates:

If PUCCH with/without repetitions overlaps with inter-slot OCC with any PUSCH repetitions in an OCC group, the legacy conditions and rules for UCI multiplexing or prioritization for dropping applies with the following updates: - If the UCI is multiplexed on the PUSCH repetition according to legacy rules and the updated timeline conditions for UCI multiplexing are satisfied, UCI is multiplexed on all PUSCH repetitions within an OCC group with inter-slot OCC overlaps with the PUCCH. (Option 3-a) - If the PUSCH repetition is dropped according to legacy rules and the updated timeline conditions for PUSCH dropping are satisfied, if any, UCI is transmitted on PUCCH and all PUSCH repetitions within the OCC group that overlaps with the PUCCH are dropped (Option 2) - UE does not expect there are multiple PUCCHs without repetitions in different PUCCH slots with HARQ-ACK or CSI report overlapping with multiple PUSCH repetitions in the same OCC group. - For PUSCH repetitions with A-CSI report, UE does not expect there is any PUCCH without repetitions for HARQ-ACK or CSI report other than the first slot overlapping with the PUSCH repetitions in the first OCC group. Note 1: If the UCI on the PUCCH is dropped according to legacy rules and updated timeline conditions for UCI dropping are satisfied, if any, there is no additional spec impact. (Option 1) Note 2: There can be multiple PUCCHs with same or different UCI types in the same slot (i.e. CSI report and HARQ-ACK) as in the legacy specifications

Proposal #6: RAN1 support separate beta offset parameter(s) for OCC with PUSCH.

Proposal #7: RAN1 support OCC indication/configuration as follows:

- For the OCC sequence applied for DG PUSCH and CG PUSCH Type 2

  - The sequence is implicitly indicated by reusing DM-RS port field.

- For the OCC length applied for DG PUSCH and CG PUSCH Type 2,

  - Candidate values are configured by higher-layer parameter for TDRA list, and the applied value is indicated by reusing TDRA field in scheduling DCI for DG PUSCH or by activation DCI for CG PUSCH Type 2 (Option 3)

Proposal #8: RAN1 support dynamic OCC disabling by reserving a single state of DM-RS port field.

Reference

[1] RP-241667, “Revised WID: Non-Terrestrial Networks (NTN) for NR Phase 3,” Thales, CATT, 3GPP TSG RAN Meeting #104, Shanghai, China, June 17-20, 2024.

[2] RAN1 Chair’s Note, RAN1#119.

[3] RAN1 Chair’s Note, RAN1#120bis.

[4] 3GPP TR 38.821: “Solutions for NR to support non-terrestrial networks (NTN)”.

[5] 3GPP TS 38.211 V18.1.0: “NR; Physical channels and modulation”.

[6] 3GPP TS 38.213 V18.6.0: “NR; Physical layer procedures for control”.