3GPP TSG RAN WG1 #121	R1-2504571

# St Julian’s, Malta, May 19th – 23th, 2025

Agenda Item:	9.1.1

Source: 	ASUSTeK

Title:  	Discussion on AIML based beam management

Document for:	Discussion and Decision

- Introduction

In this contribution, we discuss some aspects for AI/ML based beam management.

- Discussion

Performance monitoring

As agreed in RAN1 #119, linkage between CSI-ReportConfig for inference and CSI-ReportConfig for monitoring is constructed based on including CSI-ReportConfigID associated with inference configuration in CSI-ReportConfig for monitoring. Based on this linkage, if there is problem when generating an inference report resulting in no inference report, UE may have issue how to generate a linked monitoring report. In one example, CSI report (re)configuration, serving cell activation, BWP change may cause that UE drop inference report, and has problem to generate a linked monitoring report.

Observation 1: Due to linkage between monitoring and inference, if there is problem when generating an inference report resulting in no inference report, how UE generates a linked monitoring report needs to be specified.

Proposal 1: RAN1 discuss the issue that when there is no inference report transmitted before the CSI reference timing associated with a monitoring report, how UE generates the monitoring report.

A new quantity (e.g., beam accuracy indicator, BAI) was discussed in past RAN1 meeting for performance monitoring. With introduction of BAI, gNB could know some AI/ML model’s performance since BAI corresponds to a number of objects for which a statement or a condition holds for each object. However, typically, if BAI=X and BAI=X+1 makes no much difference considering beam prediction accuracy X/N and (X+1)/N when value of N is larger. In our view, what gNB cares here is rough monitoring performance of AI/ML model. Thus, we would like to propose reporting index of quantization range rather than BAI to save signalling overhead since when N is larger meaning between some code-points to gNB are similar. In one example, in Fig. 1 below, 5 quantization ranges could be configured or specified.

| Reported index | BAI or accuracy percentage |
| --- | --- |
| 0 | 1-st quantization range (i.e., 0 ≤ BAI or accuracy percentage < 1-st value) |
| 1 | 2-nd quantization range (i.e., 1-st value ≤ BAI or accuracy percentage < 2-nd value) |
| 2 | 3-rd quantization range (2-nd value ≤ BAI or accuracy percentage < 3-rd value) |
| 3 | 4-th quantization range (3-rd value ≤ BAI or accuracy percentage < 4-th value) |
| 4 | 5-th quantization range (4-th value ≤ BAI or accuracy percentage < 5-th value) |
| 5~7 | Reserved |

Fig. 1

Proposal 2: Based on a determined BAI, RAN1 decides whether/how to report information of the determined BAI in a monitoring report based on index of quantization range.

Working Assumption For BM-Case 2, at least support to report one beam prediction accuracy for one configured time instance, configured by one CSI-ReportConfig for monitoring, - only one resource set is configured in the CSI-ReportConfig - the one configured time instance (i.e. f-th time instance of the time instance in one inference report) for metric calculation is configured in the CSI-ReportConfig for monitoring - FFS on whether to configure more than one time instance - the performance metric of the f-th time instance is calculated based on N latest transmission occasion(s) of monitoring resource with linked time instance, no later than CSI reference resource corresponding to the CSI report for monitoring - N (N>=1) is configured in the CSI-ReportConfig - FFS on additional rule for counting N linked pair - measurement result of a transmission occasion of the CSI-RS/SSB resources for monitoring is linked with the f-th time instance for prediction, where the f-th time instance has the minimal slot offset to the transmission occasion of the CSI-RS/SSB resources for monitoring. - Wherein, the corresponding inference reports, and the transmission occasions of the CSI-RS/SSB resources for monitoring, [FFS on the f-th time instances] are no later than the CSI reference resource corresponding to the CSI report for monitoring - FFS: whether to introduce a threshold X, and whether it is optionally configured by RRC, where the minimal slot offset k is no larger than X; otherwise, the transmission occasion for monitoring has no linked time instance

According to above working assumption made in previous RAN1 meeting, there are three remaining open issues. From our perspective, 1-st FFS is not needed when report one BAI since more than one instances in one inference report may have different prediction accuracy according to far future time instance or near future time instance. Thus, for reporting one BAI, we think current working assumption is sufficient. 2-nd FFS is related to how to count N linked pair regarding one or more no UEI beam report case. In current CSI framework, there are lots of scenario that the UE drops UEI beam report, thus, we think it could be a general proposal to design N counting is based on available transmitted UEI beam report and available occasion of RS set for performance monitoring. 3-rd FFS, we are fine to introduce a threshold X. Without the threshold, counting a pair with larger gap into N may cause some inaccurate results.

Proposal 3: For BM case-2, when reporting one beam prediction accuracy, NOT support to configure more than one time instances via a bit-map.

Proposal 4: For BM case-2, UE counts a pair of one configured time instance (i.e., f-th in one inference report) and transmission occasion of monitoring resource as one latest N if the one inference report is transmitted and performing reception on the one the transmission occasion of monitoring resource.

Besides, from our perspective, reporting one BAI could be a baseline for performance monitoring for BM case-2. However, if gNB will like to know more information about each time instance of prediction or more than one time instance of prediction for which statement holds or not, it can be up to gNB’s configuration to configure to report more than one BAIs. One remaining issue for configuring more than one time instance via the bit map is that how to design time pattern of monitoring resource set (i.e., set C) since current RS setting in a CSI-ReportConfig for performance monitoring corresponds to periodic manner which cannot configure more than one occasions within one periodicity. From our perspective, one method is to configure burst-like RS set for performance monitoring. The intention is to guarantee that the more than one time instance via the bit map could associate with one occasion of RS set, respectively.

Proposal 5: For BM case-2, when reporting more than one beam prediction accuracy, support to configure more than one time instances via a bit-map.

Proposal 6: For BM case-2, when reporting more than one beam prediction accuracy, a resource set configured in CSI-ReportConfig for performance monitoring is enhanced such that each of more than one time instances maps to one occasion of the resource set

Inference

Agreement

For both BM-Case 1 and BM-Case 2, for UE-sided model for inference, when Set A and Set B are configured within CSI report configuration,

- Two CSI-ResourceConfigId s are configured for Set A and Set B separately

TS 38.331

#### – CSI-ResourceConfig The IE CSI-ResourceConfig defines a group of one or more NZP-CSI-RS-ResourceSet, CSI-IM-ResourceSet and/or CSI-SSB-ResourceSet. CSI-ResourceConfig information element -- ASN1START -- TAG-CSI-RESOURCECONFIG-START CSI-ResourceConfig ::= SEQUENCE { csi-ResourceConfigId CSI-ResourceConfigId, csi-RS-ResourceSetList CHOICE { nzp-CSI-RS-SSB SEQUENCE { nzp-CSI-RS-ResourceSetList SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourceSetsPerConfig)) OF NZP-CSI-RS-ResourceSetId OPTIONAL, -- Need R csi-SSB-ResourceSetList SEQUENCE (SIZE (1..maxNrofCSI-SSB-ResourceSetsPerConfig)) OF CSI-SSB-ResourceSetId OPTIONAL -- Need R }, csi-IM-ResourceSetList SEQUENCE (SIZE (1..maxNrofCSI-IM-ResourceSetsPerConfig)) OF CSI-IM-ResourceSetId }, bwp-Id BWP-Id,

As agreed in RAN1 #119, set A and set B are configured by CSI-ResourceConfig, respectively in one CSI-ReportConfig for inference. According to TS 38.331, a BWP ID to indicate RS set configured by CSI-ResourceConfig belongs to which BWP. Insofar, it’s not clear whether set A and set B could be configured in different BWP. From our perspective, same BWP for set A and set B should be the baseline. On the other hands, benefit of different BWP between set A and set B seems to provide contiguous prediction even BWP change which the gain seems marginal. Thus, we propose Set A and set B configured in CSI-ReportConfig for inference should be configured in same BWP.

Observation 2: In TS 38.331, CSI-ResourceConfig comprises a BWP ID to indicate RS set configured by CSI-ResourceConfig belongs to which BWP, however, it’s not clear whether to support set A and set B configured in different BWP.

Proposal 7: Set A and set B configured in CSI-ReportConfig for inference should be configured in same BWP

UE’s trajectory seems to have more than one time instances to determine. For BM-case2 (i.e., temporal prediction), our view is multiple time instances of set B is needed for one inference report. However, when UE cannot receive sufficient time instance of set B for one inference report, UE may have low confidence to generate an inference report. The inference result in one inference report may be not useful in BM-case2. In this case, whether/how UE triggers a report to inform gNB or drops the inference report may need further discussed.

Observation 3: For an AI/ML model, for BM-case2 (i.e., temporal prediction), insufficient time instance of set B will result in low confidence when generating an inference report.

Proposal 8: When generating an inference report with low confidence, RAN1 discuss how/whether UE inform gNB or drop the inference report

According to TR 38.843, one benefit of AI/ML based beam management is to have lower signalling overhead in temporal domain (i.e., Case B+) as quoted below, and for Set B (N beams) is a subset of Set A (M beams), the RS overhead reduction is 1-N/(YM). However, it’s not clear how to achieve this case by current CSI framework which needs further design. Monitoring pattern of set B or measurement window to achieve could be considered to skip a subset of time instances of set B.

Observation 4: One benefit of AI/ML based beam management is to have lower signalling overhead in temporal domain. How to skip a subset of time instance of set B to achieve signalling overhead reduction needs further design.

Proposal 9: RAN1 confirms that signalling overhead for CSI-RS (i.e., set B) reduction in temporal domain is needed for BM case-2.

Proposal 10: RAN1 study whether/how to leverage monitoring pattern of set B or monitoring window to skip a subset of time instances of set B.

- Conclusion

In this contribution, we have following proposals:

Observation 1: Due to linkage between monitoring and inference, if there is problem when generating an inference report resulting in no inference report, how UE generates a linked monitoring report needs to be specified.

Proposal 1: RAN1 discuss the issue that when there is no inference report transmitted before the CSI reference timing associated with a monitoring report, how UE generates the monitoring report.

Proposal 2: Based on a determined BAI, RAN1 decides whether/how to report information of the determined BAI in a monitoring report based on index of quantization range.

Proposal 3: For BM case-2, when reporting one beam prediction accuracy, NOT support to configure more than one time instances via a bit-map.

Proposal 4: For BM case-2, UE counts a pair of one configured time instance (i.e., f-th in one inference report) and transmission occasion of monitoring resource as one latest N if the one inference report is transmitted and performing reception on the one the transmission occasion of monitoring resource.

Proposal 5: For BM case-2, when reporting more than one beam prediction accuracy, support to configure more than one time instances via a bit-map.

Proposal 6: For BM case-2, when reporting more than one beam prediction accuracy, a resource set configured in CSI-ReportConfig for performance monitoring is enhanced such that each of more than one time instances maps to one occasion of the resource set

Observation 2: In TS 38.331, CSI-ResourceConfig comprises a BWP ID to indicate RS set configured by CSI-ResourceConfig belongs to which BWP, however, it’s not clear whether to support set A and set B configured in different BWP.

Proposal 7: Set A and set B configured in CSI-ReportConfig for inference should be configured in same BWP

Observation 3: For an AI/ML model, for BM-case2 (i.e., temporal prediction), insufficient time instance of set B will result in low confidence when generating an inference report.

Proposal 8: When generating an inference report with low confidence, RAN1 discuss how/whether UE inform gNB or drop the inference report

Observation 4: One benefit of AI/ML based beam management is to have lower signalling overhead in temporal domain. How to skip a subset of time instance of set B to achieve signalling overhead reduction needs further design.

Proposal 9: RAN1 confirms that signalling overhead for CSI-RS (i.e., set B) reduction in temporal domain is needed for BM case-2.

Proposal 10: RAN1 study whether/how to leverage monitoring pattern of set B or monitoring window to skip a subset of time instances of set B.