3GPP TSG RAN WG1 #121	R1-2504576

St. Julian’s, Malta, May 19th-23rd 2025

Agenda item:		9.11.5

Source:	Nordic Semiconductor ASA

Title:					On R19 TDD IoT-NTN

Document for:		Discussion and Decision

# Introduction

In RAN#105, work item in RP-234056 has been approved. Plenary RAN#106 approved update to WID in RP-243293. The main update is the following:

- Support a pattern with a period of 9 radio frames for the target MSS allocated band, where D=U=8 with a fixed guard period.

  - RAN1 to consider whether there is a need for a mechanism to achieve an adjustable guard period for the purpose of allowing deployment with the TDD frame structure of the legacy system operating in the target MSS allocated band.

Restrictions on frame structure of the target MSS band are the following:

Agreement Regarding operation within the same band as the TDD frame structure of legacy system in the 1.6GHz MSS band (shown below), RAN1 work in Rel-19 considers the following design constraints: - At the satellite, all downlink NB-IoT channels/signals in a cell can only use one of the downlink slots in the TDD frame structure (DL1, DL2, DL3 or DL4) across 90ms periods. - The same downlink slot is used in all the 90ms periods. - At the satellite, all uplink NB-IoT channels in a cell can only use one of the uplink slots in the TDD frame structure (UL1, UL2, UL3 or UL4) across 90ms periods. - The same uplink slot is used in all the 90ms periods. - The one uplink slot and one downlink slot in the TDD frame structure have the same index (DL1 & UL1, DL2 & UL2, DL3 & UL3, or DL4 & UL4). - NOTE: this does not imply that the only configuration(s) to be specified are according to these constraints.

# Discussion

## Pre-compensation gaps in UL

Working assumption For precompensation, from RAN1 perspective: - The UE adjusts its time/frequency pre-compensation before the beginning of each set of consecutive 8 uplink subframes. No pre-compensation gap is needed before the beginning of each set of consecutive 8 uplink subframes. - FFS: Whether it is supported to perform segmented pre-compensation within the set of 8 consecutive uplink subframes, and whether in this case a pre-compensation gap is needed. - FFS: whether spec impact is in RAN1, RAN4 or both

For 600km LEO when doppler is16ppm, one sees 15us/s TA drift and at 90 degree elevation doppler change is 0.316ppm/s.  This means 0.12us and 4Hz change at 1.6GHz centre frequency during 8ms. 800km orbit is further relaxed in terms of doppler and TA drifts. If pre-compensation is performed before every 8ms UL burst, system works without performance degradation.

To proof this we ran R17 NTN simulation in an orbit simulator, while measuring UL SNR and length of scheduled NPUSCH, during 3-minute LEO satellite fly. It shows that NPUSCH length 2ms increased to 8,10 or even 20 ms has no impact on the performance of the system.

Figure 1 NPUSCH SNR and NPUSCH length in LEO orbit

| Parameter | Value |
| --- | --- |
| Orbit | 600km LEO orbit 90deg inclination |
| Simulation time | -90 to 90s |
| Channel estimation | Non-ideal |
| SNR @ peak elevation | 16dB |
| # of tones | 12 @ 15kHz SCS |

Regarding spec change:

For a UE in a NTN serving cell, using serving satellite higher-layer ephemeris parameters, if configured, the UE determines NTA,adjUE (defined in [3]) using the serving satellite position and its own position to pre-compensate the two-way transmission delay on the service link for each set of continuously transmitted symbols. To pre-compensate the two-way transmission delay between the uplink time synchronization reference point and the serving satellite, the UE determines NTA,adjcommon (defined in [3]) based on one-way propagation delay Delaycommont which can be obtained as: Delaycommont=12NTAcommon+NTAcommonDrift×t-tepoch+NTAcommonDriftVariation×t-tepoch2 where NTAcommon, NTAcommonDrift, and NTAcommonDriftVariation are given by the higher layer parameters nta-Common, nta-CommonDrift, and nta-CommonDriftVariation respectively, and tepoch is the epoch time given by the higher layer parameter epochTime. Delaycommon(t) provides a distance at time t between the serving satellite and the uplink time synchronization reference point divided by the speed of light. The uplink time synchronization reference point is the point where DL and UL are frame aligned with an offset given by NTA,offset. For a NB-IoT UE communicating over NTN mode other than NTN TDD, time and frequency pre-compensation is adjusted per uplink segment with a transmission duration of Nsegmentprecompensation time units, where the quantity Nsegmentprecompensation is provided by higher layers, as specified in 3GPP TS 36.331 [11].

In RAN1 36.213 specification it is clearly indicated that pre-compensation to happen per transmission segment, if configured. It should be clarified that pre-compensation is performed per each set of continuously transmitted symbols, when pre-compensation segments are not configured. Therefore, a minor clarification would ensure that specification is aligned with WA we made.

Proposal-1: Confirm working assumption as

Working assumption

For precompensation, from RAN1 perspective:

- The UE adjusts its time/frequency pre-compensation before the beginning of each set of consecutive 8 uplink subframes. No pre-compensation gap is needed before the beginning of each set of consecutive 8 uplink subframes.

  - FFS: Whether it is supported to perform segmented pre-compensation within the set of 8 consecutive uplink subframes, and whether in this case a pre-compensation gap is needed.

  - segmented pre-compensations are not supported

- Adopt the specification change above

## Power consumption in R19 TDD NTN IoT

When switching from D to U, up to 50 subframes of gaps are expected. Actual gaps for the UE can be even larger, based on NPDCCH configuration and NPUSCH scheduling. For example, if UE is not scheduled with UL transmission, 82ms is a typical gap time for sleep. In addition, due to satellite mesh delays, it is expected that there will be long periods of inactivity in RRC connected mode until being able to get response for the MO traffic. In these long periods UE just needs to monitor for NPDCCH once 90ms, and would not perform NPDSCH reception or NPUSCH transmission.

Observation-1: It is expected that UEs will spend long time in RRC connected, having long periods of time doing only  monitoring for NPDCCH once 90ms.

This brings lots of possibilities for UE to sleep and even deep sleep during RRC connected mode. However, upon wake-up from deep sleep a UE requires synchronization aid. NRS are sparse, do not contain much energy and therefore UE would benefit from a preamble, such preamble could be an additional NSSS located in SF#3 and transmitted only in UE specific manner in RRC connected mode for the purpose of power saving. NSSS in SF#3 would not disturb legacy initial cell search procedure and would provide UE with time and frequency synchronization possibilities while providing cell-integrity as well.

It is also envisioned that data transmission would be carried in an non-anchor carriers using the same DL-UL pair of slots as on anchor. This because overhead on anchor carrier is large in R19 TDD NTN IoT. On anchor carrier no NPSS or NSSS is present, and thus a NSSS in SF#3 would be essential for re-synchronization to facilitate deep sleep.

XO/PLL are significant contributor for power consumption of a chipset, which could be e.g. few mA. When XO/PLL sleeps power consumption drops to e.g. 10th of uA. Obviously, if XO remain running during large gaps, it is not expected to drift too much and NRS should be sufficient to recover small timing and frequency error. However, to enable XO/PLL to sleep during large gaps, UE can count clocks only using RTC, it is essential that synchronization signal is available upon wake-up.

Proposal-2: If deep sleep shall be supported in R19 TDD IoT NTN system in Connected mode, it is essential that additional synchronization signals (e.g. NSSS) is located in DL burst starting subframe #3, at least on non-anchor.

## DL bitmap

Working assumption DL bitmap (downlinkBitmap / downlinkBitmapNonAnchor) is not present in a NB-IoT NTN TDD cell. - NOTE1: All D NB-IoT subframes (except those carrying NPSS/NSSS/SIB1-NB/NPBCH) are NB-IoT DL subframes. - NOTE2: “D NB-IoT subframes” are subframes contained within the set of D=8 usable consecutive downlink subframes in the TDD structure.

First of all, we believe WA can be confirmed, there are no technical issues. It remained open whether UE may assume that NRS are present in every D subframe. And answer should be yes. UE should have opportunity to obtain as much synchronization signals for reception of SIB1-NB as possible.

Proposal-3: UE may assume that every D subframe (except of NPSS and NSSS) contains NRS.

## DL transmission gap

| Agreement For the issue of handling DL gaps in NB-IoT NTN TDD mode, down-select among the following options: - Option 1: DL gaps do not apply for NB-IoT NTN TDD mode. - Option 2: DL gaps apply based on current specifications. - Option 3: DL gaps apply, where new periodicity(ies) are introduced (either implicity e.g. the periodicity is scaled by a fixed factor F, or explicitly) |  |
| --- | --- |

When NPDSCH is transmitted with large number of repetitions, DL gap gives eNB a possibility to multiplex transmissions of multiple UEs. Firstly, it is unclear what is the maximum number of repetitions to be supported by the NB-IoT NTN TDD. Typically, satellites have large beamforming gain and can use up to 50dBm of transmit power. And NTN is more thermal noise-limited scenario rather than interference limited. Even with small-form antenna deployment, no more than 2/4 repetitions are needed to reach target coverage of -1dB SNR in DL. Even if DL RU is 10 subframes, one UE airtime spans four 90ms DL bursts. More than 4 NPDCCH/NPDSCH repetitions may paralyze anchor carrier, if it needs to serve, SI, paging and initial access with large number of repetitions.

It is unclear whether support of DL gaps is essential. However, if other companies insist that it must be supported, we think that DL gaps can remain as in legacy, but duration would be counted in valid R19 TDD DL subframes only.

Proposal-4: If DL gap support is found essential, the duration is counted in number of valid R19 TDD DL subframes.

## NPDCCH

Agreement For the issue of handling NPDCCH postponing, down-select among the following options: - Option 1: No new periodicities are introduced for NPDCCH search space monitoring - FFS: Changes in postponing / overlapping rules, if any - FFS: Introduction of constraints on the configuration of search space parameters (e.g. Rmax, G…), if any - Option 2: New periodicities are introduced for NPDCCH search space monitoring - FFS: Details of the new periodicity, e.g. the existing periodicity is scaled by a fixed factor F, explicit signalling of the new periodicity, reusing the NB-IoT TN TDD periodicities, etc.

Based on analysis of the spec, it seems that NPDCCH postponing to the next valid DL subframe works out-of-box. It means no specification change is needed. It is anticipated that R19 TDD NTN NB-IoT system would be targeting only CE Level 0 at minimum SNR of -1dB. Therefore, RAN1 should consider to restrict values of R_max to reduce certification testing efforts for this feature. Similarly, small values of StartSF-USS could be also removed, there is no point to configure values like v2 or v4 for N=9 D=8 structure. Finally, small values Rmax* G=pp would not allow configuration of sufficient window/timer length, e.g. for RAR/contention window and SR prohibit timer.

When it comes to introduction of scaling factor for NPDCCH monitoring pattern, we do not think it is essential, but change is sufficiently small and justifiable by scheduling benefits.

NPDCCH-ConfigDedicated-NB-r13 ::= SEQUENCE { npdcch-NumRepetitions-r13 ENUMERATED {r1, r2, r4, r8, r16, r32, r64, r128, r256, r512, r1024, r2048, spare4, spare3, spare2, spare1}, aka Rmax npdcch-StartSF-USS-r13 ENUMERATED {v1dot5, v2, v4, v8, v16, v32, v48, v64}, aka G npdcch-Offset-USS-r13 ENUMERATED {zero, oneEighth, oneFourth, threeEighth} } aka offset

Proposal-5: NPDCCH and NPDSCH channels are postponed to the next valid DL subframe(s).

- In R19 TDD NTN NB-IoT system, minimum configurable T=Rmax* G is 64ms.

  - Maximum Rmax=4

- Scaling factor applied to T can be considered.

## GNSS Gap

Agreement For GNSS measurement gap, RAN1 to further discuss at least the following options: - Option 1: No enhancement on the Rel-18 timeline for the start of GNSS gap - Option 2: The start of the GNSS measurement gap starts at a non-D/non-U subframe - Option 3: GNSS measurement gap does not apply for NB-IoT NTN TDD

When MAC-CE indicates GNSS measurement gap, it configures large enough GNSS measurement GAP to accommodate for the 90ms delay, i.e. Option 1 is sufficient in our opinion. The GNSS measurement gaps configured based on UE capability are possible up to 30s, should be plenty of time to start GNSS measurement and provide new GNSS validity time to eNB to maintain RRC connected mode.

Proposal-6: For GNSS measurement gap, RAN1 to further discuss at least the following options:

- Option 1: No enhancement on the Rel-18 timeline for the start of GNSS gap.

# Conclusions

Proposal-1: Confirm working assumption as

Working assumption

For precompensation, from RAN1 perspective:

- The UE adjusts its time/frequency pre-compensation before the beginning of each set of consecutive 8 uplink subframes. No pre-compensation gap is needed before the beginning of each set of consecutive 8 uplink subframes.

  - FFS: Whether it is supported to perform segmented pre-compensation within the set of 8 consecutive uplink subframes, and whether in this case a pre-compensation gap is needed.

  - segmented pre-compensations are not supported

- Adopt the specification change above

Observation-1: It is expected that UEs will spend long time in RRC connected, having long periods of time doing only  monitoring for NPDCCH once 90ms.

Proposal-2: If deep sleep shall be supported in R19 TDD IoT NTN system in Connected mode, it is essential that additional synchronization signals (e.g. NSSS) is located in DL burst starting subframe #3, at least on non-anchor.

Proposal-3: UE may assume that every D subframe (except of NPSS and NSSS) contains NRS.

Proposal-4: If DL gap support is found essential, the duration is counted in number of valid R19 TDD DL subframes.

Proposal-5: NPDCCH and NPDSCH channels are postponed to the next valid DL subframe(s).

- In R19 TDD NTN NB-IoT system, minimum configurable T=Rmax* G is 64ms.

  - Maximum Rmax=4

- Scaling factor applied to T can be considered.

Proposal-6: For GNSS measurement gap, RAN1 to further discuss at least the following options:

- Option 1: No enhancement on the Rel-18 timeline for the start of GNSS gap.