3GPP TSG RAN WG1 #121			R1-2504615

St. Julians, Malta, May 19th – 23rd, 2025

Source:	WILUS Inc.

Title:	Discussion on SBFD TX/RX/measurement procedures

Agenda item:	9.3.1

Document for:	Discussion

Introduction

From the RAN1#116 and RAN1#120bis meetings [1]-[8], RAN1 agreed under the agenda of SBFD TX/RX/measurement procedures for many technical issues. In this contribution, we provide our views on the remaining issues such as semi-static configuration of SBFD subbands, UE transmission/reception/measurement behavior, procedures in SBFD symbols and/or non-SBFD symbols for SBFD aware UE for SBFD operation and DL/UL collision handling for a SBFD aware UE.

Discussion on subband non-overlapping full duplex

Cell-specific or UE-specific configuration on frequency location of SBFD subbands

At the RAN1#117 meeting, the following was agreed as cell specific indication of SBFD subband frequency location for semi-static frequency indication of SBFD subband. And it was captured at RAN1#118 meeting as working assumption for cell-specific indication of frequency locations of SBFD subbands.

Agreement at RAN1#117 For cell-specific indication of SBFD subband frequency location, adopt the following option. - Option 1: Frequency locations of UL subband and DL subband(s) are explicitly configured. Guardband(s) if any are implicitly derived as the RBs which are not within UL subband or DL subband(s). Agreement at RAN1#118 For configuration of SBFD symbols within a TDD-UL-DL pattern period, the following parameters are supported - A starting slot index - A starting symbol index within the starting slot - An ending slot index - An ending symbol index within the ending slot Agreement at RAN1#119 Confirm the following working assumption made in RAN1#118. Working Assumption at RAN1#118 For cell-specific configuration of frequency locations of SBFD UL subband, for each SCS configuration in SCS-SpecificCarrierList for UL, starting RB and bandwidth of SBFD UL subband are indicated by a RIV-based indication as defined in 38.214 setting NBWPsize=275. For cell-specific configuration of frequency locations of SBFD DL subband(s), for each SCS configuration in SCS-SpecificCarrierList for DL, starting RB and bandwidth of each SBFD DL subband are indicated by a RIV-based indication as defined in 38.214 setting NBWPsize=275. - One or two SBFD DL subbands can be configured

For semi-static indication of frequency locations of SBFD subbands, the following agreement were made in RAN1#116. The remaining issues are to confirm working assumption of cell-specific configuration on frequency location of SBFD subbands and whether to additionally support UE-specific configuration on frequency locations of SBFD subbands.

Agreement at RAN1#116 For RRC connected mode UEs, at least cell-specific configuration on time and frequency(working assumption) location of SBFD subbands is supported within a TDD carrier. - FFS: Additional support of UE-specific configuration on time and/or frequency locations of SBFD subbands

One of the motivations for introducing UE-specific configuration on the frequency position of the SBFD subbands is to reduce the impact of the UE-to-UE CLI by accommodating the UE-specific guardbands. However, as one of the ways to reduce the impact of the UE-to-UE CLI without the UE-specific configuration of the SBFD subband, a gNB can consider not implicitly scheduling some RBs for creating guardbands between UEs if the UE-to-UE CLI between UEs is severely measured and the gNB can avoid strong the UE-to-UE CLI. Therefore, we propose to confirm the working assumption that cell-specific configuration on frequency location of SBFD subbands is supported within a TDD carrier without UE-specific configuration on frequency locations of SBFD subbands.

- Proposal 1: We propose to confirm the working assumption that cell-specific configuration on frequency location of SBFD subbands is supported within a TDD carrier.

  - UE-specific configuration on frequency locations of SBFD subbands is not supported.

Enhancements for UL transmissions and DL receptions

At the TSG-RAN#102 meeting, it was agreed to specify enhancement on resource allocation in frequency domain in SBFD symbols and enhancements to physical channels/signals and procedure for UE transmissions, receptions and measurement across SBFD symbols and non-SBFD symbols in different slots. We provide our views on remaining issues related enhancements to UE transmissions, receptions and measurement for SBFD operation.

    - UL transmission/DL reception across SBFD symbols and non-SBFD symbols in different slots

Regarding frequency resource allocation options for PDSCH receptions across SBFD symbols and non-SBFD symbols in different slots for a SBFD-aware UE, it was captured as working assumption at the RAN1#118 meeting and it was agreed from RAN1#118bis to RAN1#120 meeting for multi-PDSCH scheduled by a single DCI, dynamically scheduled PDSCH repetitions, SPS PDSCH w/ and w/o repetition with Configuration 2 as below.

Working Assumption at RAN1#117 meeting For an SPS PDSCH configuration without repetitions, if the reception occasions are across SBFD symbols and non-SBFD symbols where each reception occasion has either all SBFD or all non-SBFD symbols (i.e. Configuration 2), PDSCH repetitions across SBFD symbols and non-SBFD symbols in different slots where each repetition has either all SBFD or all non-SBFD symbols (i.e. Configuration 2), and for multi-PDSCH scheduled by a single DCI across SBFD symbols and non-SBFD symbols, where each PDSCH within a slot has either all SBFD or all non-SBFD symbols (i.e. Configuration 2), - Option 5: Only the assigned PRBs within DL usable PRBs in SBFD symbols are considered to be valid. - For SPS PDSCH, FFS whether the number of PRBs for TBS determination is based on assigned PRBs or assigned PRBs within DL usable PRBs only - For PDSCH repetitions, FFS whether the number of PRBs for TBS determination is based on assigned PRBs, or based on assigned PRBs within DL usable PRBs only, or based on assigned PRBs within DL usable PRBs of the first repetition. - For multi-PDSCH scheduled by a single DCI, FFS whether the number of PRBs for TBS determination is based on assigned PRBs or assigned PRBs within DL usable PRBs only. Agreement at RAN1#118-bis meeting For multi-PDSCH scheduled by a single DCI with Configuration 2, the number of PRBs for TBS determination for a PDSCH in SBFD symbols is based on assigned PRBs within DL usable PRBs only. The number of PRBs for TBS determination for a PDSCH in non-SBFD symbols is based on assigned PRBs. Agreement at RAN1#119 meeting For dynamically scheduled PDSCH repetitions with Configuration 2, for TBS determination. - Option 1: - If the first repetition occasion indicated by scheduling DCI is in SBFD symbols, the number of PRBs for TBS determination is based on assigned PRBs within DL usable PRBs. - If the first repetition occasion indicated by scheduling DCI is in non-SBFD symbols, the number of PRBs for TBS determination is based on assigned PRBs. Agreement at RAN1#120 meeting For SPS PDSCH with Configuration 2, for TBS determination, - For SPS PDSCH without repetition, - The number of PRBs for TBS determination for an SPS PDSCH in SBFD symbols is based on assigned PRBs within DL usable PRBs only. - The number of PRBs for TBS determination for an SPS PDSCH in non-SBFD symbols is based on assigned PRBs. - For SPS PDSCH with repetition, TBS is determined based on the first repetition occasion. - If the first repetition occasion of a SPS PDSCH is in SBFD symbols, the number of PRBs for TBS determination for the SPS PDSCH is based on assigned PRBs within DL usable PRBs only. - If the first repetition occasion of a SPS PDSCH is in non-SBFD symbols, the number of PRBs for TBS determination for the SPS PDSCH is based on assigned PRBs.

Available slot determination for PUCCH/PUSCH repetition with available slot counting and TBoMS

Based on the following agreements related to Configuration 1 and 2 regarding UL transmission/DL reception across SBFD symbols and non-SBFD symbols in different slots, RAN1 further agreed at RAN1#120bis meeting how to determine available slot(s) for PUCCH repetition, PUSCH repetition with available slot counting, and TBoMS for a SBFD-aware UEs as the following for both cases with Configuration 1 and Configuration 2.

Agreement at RAN1#117 For UL transmissions and DL receptions across SBFD symbols and non-SBFD symbols in different slots (each transmission/reception within a slot has either all SBFD or all non-SBFD symbols) for an SBFD aware UE, the SBFD-aware UE is provided with one of the configurations. - Configuration 1: The transmissions/receptions are restricted to SBFD symbols only or non-SBFD symbols only - Configuration 2: The transmissions/receptions can be in SBFD symbols and non-SBFD symbols - FFS granularity of the configuration, e.g. per UE, per channel/signal etc. - FFS whether support of configuration 2 is subject to UE capability Agreement at RAN1#119 For UL transmissions and DL receptions across SBFD symbols and non-SBFD symbols in different slots (each transmission/reception within a slot has either all SBFD or all non-SBFD symbols) for an SBFD aware UE, - Support of configuration 2 is subject to UE capability. Configuration 1 is the default capability. - SBFD-aware UE can be configured with Configuration 2 on a per UL/DL BWP basis. - The configuration for DL BWP at least applies to PDSCH receptions within the DL BWP. - The configuration for UL BWP applies to PUCCH and PUSCH transmissions within the UL BWP. - For SRS, only Configuration 1 is applicable. Agreement at RAN1#120bis For PUCCH repetition, PUSCH repetition with available counting and TBoMS with Configuration 1, - In case the valid symbol type is SBFD symbol, a slot is determined as available if the symbols allocated for PUCCH/PUSCH in the slot are all SBFD symbols and not include a symbol of an SS/PBCH block with index provided by ssb-PositionsInBurst. - In case the valid symbol type is non-SBFD symbol, a slot is determined as available if the symbols allocated for PUCCH/PUSCH in the slot are all non-SBFD symbols and not include a DL symbol indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated if provided or a symbol of an SS/PBCH block with index provided by ssb-PositionsInBurst. Agreement at RAN1#120bis For PUCCH repetition, PUSCH repetition with available counting and TBoMS with Configuration 2, a slot is determined as available if - the symbols allocated for PUCCH/PUSCH in the slot are all SBFD symbols and not include a symbol of an SS/PBCH block with index provided by ssb-PositionsInBurst, or - the symbols allocated for PUCCH/PUSCH in the slot are all non-SBFD symbols and not include a DL symbol indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated if provided or a symbol of an SS/PBCH block with index provided by ssb-PositionsInBurst.

Actual TDW determination for DMRS bundling

In Rel-17 coverage enhancement WI, DMRS bundling to enable improved channel estimation is introduced. It can be enabled for PUSCH repetition Type A scheduled by DCI format 0_1 or 0_2, for PUSCH repetition Type A with configured grant, for PUSCH repetition Type B, for TB processing over multiple slots PUSCH and for PUCCH repetitions of PUCCH formats 1, 3, and 4.

One or multiple nominal TDWs (time domain windows) which is not longer than the maximum duration determined by UE’s capability, can be configured with duration by gNB. Then, a nominal TDW can be segmented into one or multiple actual TDWs according to events, which result in the loss of power consistency and phase continuity across PUSCH or PUCCH transmissions, such as a downlink slot or downlink reception or downlink monitoring based on tdd-UL-DL-ConfigCommon and tdd-UL-DL-ConfigDedicated, the gap between any two consecutive PUSCH or PUCCH transmissions that exceeds 13 symbols for normal CP and 11 symbols for extended CP, and frequency hopping. Since the downlink slot by tdd-UL-DL-ConfigCommon is one of the events to segment from nominal TDW to actual TDW, joint channel estimation via DMRS bundling is not performed at gNB if there is at least one DL slot in-between PUSCH or PUCCH transmissions.

Recall the SBFD operation, UL subband can be configured in a symbol configured as DL or flexible indicated by tdd-UL-DL-ConfigCommon. In this case, a SBFD-aware UE may not determine PUSCH or PUCCH transmissions across UL subbands in a slot configured as DL by tdd-UL-DL-ConfigCommon as an actual TDW as shown in Figure 1.

Figure 1. Actual TDW determination across PUSCH transmissions in UL subband.

In Figure 1, a SBFD-aware UE can be scheduled to transmit PUSCH over 4 slots with repetition Type A and configured to enable DMRS bundling with nominal TDW duration as 4 slots. Then, 4 actual TDWs in each slot are determined since slot#1, slot#2, and slot#3 are event, which result in the loss of power consistency and phase continuity due to DL symbols configured by tdd-UL-DL-ConfigCommon in the legacy NR system without SBFD operation. As a result, any of PUSCH repetitions cannot be jointly estimated the channel at gNB even if the SBFD-aware UE is configured to enable DMRS bundling.

To address this, RBs within UL subband across SBFD slots or symbols should not be regarded as an event in a slot or symbols configured as DL by tdd-UL-DL-ConfigCommon. The event means the events in TS38.214 which cause power consistency and phase continuity not to be maintained across PUSCH transmissions of PUSCH repetition type A scheduled by DCI format 0_1, 0_2 or 0_3, or PUSCH repetition Type A with a configured grant, or PUSCH repetition type B or TB processing over multiple slots, or PUCCH transmissions of PUCCH repetition, within the nominal TDW. Thus, PUSCH transmissions of PUSCH repetition or TB processing over multiple slots, or PUCCH transmissions of PUCCH repetition within UL subband across SBFD slots or symbols configured as DL by tdd-UL-DL-ConfigCommon can be determined as an actual TDW, and joint channel estimation can be performed at gNB for UL coverage enhancement.

- Proposal 2: PUSCH transmissions of PUSCH repetition or TB processing over multiple slots, or PUCCH transmissions of PUCCH repetition within UL subband across SBFD slots or symbols configured as DL by tdd-UL-DL-ConfigCommon should not be an event which causes power consistency and phase continuity not to be maintained for actual TDW determination for a SBFD-aware UE.

At the RAN1#117 meeting, it was agreed to support separate UL power control for PUSCH/PUCCH/SRS transmission in SBFD symbols and non-SBFD symbols. When a UE is configured with DMRS bundling as enabled, power consistency of PUSCH or PUCCH transmissions cannot be maintained across SBFD and non-SBFD symbols if separate UL power control parameters are applied across SBFD and non-SBFD symbols for Configuration 2. Therefore, separate UL power control parameters across SBFD and non-SBFD symbols for Configuration 2 can be considered as an event which causes power consistency and phase continuity not to be maintained, and different actual TDWs for different symbol types can be made on PUSCH transmissions of PUSCH repetition or PUCCH transmissions of PUCCH repetition across SBFD and non-SBFD symbols if DMRS bundling is configured as enabled for the SBFD-aware UE as shown in Figure 2.

- Proposal 3: If separate UL power control parameters are applied across SBFD and non-SBFD symbols for Configuration 2, it should be considered as an event which causes power consistency and phase continuity not to be maintained and different actual TDWs for different symbol types should be determined for PUSCH transmissions of PUSCH repetition or PUCCH transmissions of PUCCH repetition.

Figure 2. Actual TDW determination on PUSCH transmissions across SBFD and non-SBFD symbols with separate UL power control parameters.

In Figure 2, a SBFD-aware UE can be scheduled to transmit PUSCH repetition Type A over 4 slots with Configuration 2, where P0,SBFD denotes P0 value of PUSCH transmission in SBFD symbols, while P0,normal denotes the P0 value in non-SBFD symbols. When SBFD-Configuration2-Transmission is enabled (i.e., Configuration 2) and the UE is configured to enable DMRS bundling with duration of a nominal TDW as 4 slots, two actual TDWs (actual TDW#1 including PUSCH#1~#3 and actual TDW#2 including PUSCH#4) are determined with respect to an event between PUSCH#3 and PUSCH#4. Considering that the DMRS bundling feature was introduced to enhance UL coverage, it might be further beneficial to determine PUSCH#1~PUSCH#3 and PUSCH#4 as the same actual TDW by maintaining power consistency although separate UL power control parameters are configured/indicated. In other words, DMRS bundling gain can be prioritized over separate UL power control across SBFD symbols and non-SBFD symbols when DMRS bundling is configured as enabled for the SBFD-aware UE with Configuration 2.

HPN determination for multi-slot PUSCH/multi-slot PDSCH

In Rel-17 60GHz WI, multi-slot PUSCHs scheduling with a single DCI was specified, where time domain allocations of multiple PUSCHs can be discontinuous. Those PUSCH transmissions in multi-slots have different TBs, thus different HPNs (HARQ process numbers) are determined for each PUSCH transmission. Because a single HPN value is contained in a DCI, HPN determination rule was specified as follows.

When the UE is scheduled with multiple PUSCHs by a DCI, HPN indicated by DCI applies to the first PUSCH not overlapping with a DL symbol configured by TDD-UL-DL-ConfigCommon or TDD-UL-DL-ConfigDedicated, or a symbol of an SS/PBCH block with index provided by ssb-PositionsInBurst. HPN is then incremented by 1 for each subsequent PUSCH(s) in the scheduled order, with modulo operation. HPN is not incremented if at least one of the symbols for PUSCH or PUCCH transmission overlaps with a DL symbol configured by TDD-UL-DL-ConfigCommon or TDD-UL-DL-ConfigDedicated, or a symbol of an SS/PBCH block with index provided by ssb-PositionsInBurst.

Similar with available slot determination agreed at RAN1#120bis meeting, enhancement on HPN determination of multi-slot PUSCHs scheduled with a single DCI within UL subband is necessary as shown in Figure 3.

Figure 3. HPN determination for multi-slot PUSCHs across SBFD symbols and non-SBFD symbols with Configuration 2.

In the case of Configuration 2 (i.e. transmissions can be in both SBFD symbols and non-SBFD symbols), as shown in Figure 3, a SBFD-aware UE can be scheduled to transmit a multi-slot PUSCH with a single DCI across non-SBFD symbols (slot#1) and SBFD symbols (slot#2) according to the Configuration 2. Here, slot#1 is consisted of all non-SBFD symbols and slot#2 is consisted of all SBFD symbols.

Although, the SBFD-aware UE can transmit PUSCH#2 in the slot#2 according to the Configuration 2, the HPN of PUSCH#2 would not be incremented based on the current specification rules. This can lead to ambiguity, as different PUSCHs with different TBs may share the same HPN, potentially impacting PUSCH HARQ processing when scheduling multi-slot PUSCH transmissions. To resolve this issue, availability in the frequency domain should be further considered in addition to availability in the time domain. Therefore, the HPN of allocated PUSCH should be incremented if its allocated frequency resources are within the UL usable PRBs on SBFD symbols, regardless of the symbol type of the allocated PUSCH is configured as DL or Flexible symbols indicated by tdd-UL-DL-ConfigCommon or tdd-UL-DL-ConfigDedicated. Furthermore, the HPN of allocated PUSCH should not be incremented for any PUSCH that is not transmitted due to a collision with SSB symbols indexed by ssb-PositionsInBurst and one of multiple PUSCHs on SBFD symbols in the time domain as illustrated in Figure 4.

Figure 4. Example of HPN determination in the presence of time-domain collision between SSB symbols and multiple PUSCH transmissions on SBFD symbols.

- Proposal 4: In addition to availability in time domain, availability in frequency domain should be further considered when determining the HPN for multi-slot PUSCHs scheduled by a single DCI for an SBFD-aware UE with Configuration 2.

  - The HPN of allocated PUSCH should be incremented if its allocated frequency resources are within the UL usable PRBs on SBFD symbols regardless of the symbol type of the allocated PUSCH is configured as DL or Flexible symbols, as indicated by tdd-UL-DL-ConfigCommon or tdd-UL-DL-ConfigDedicated.

  - The HPN of allocated PUSCH should not be incremented for a PUSCH that is not transmitted due to a collision between SSB symbols indexed by ssb-PositionsInBurst and one of multiple PUSCHs on SBFD symbols in the time domain.

For the case of Configuration 1 (i.e. transmissions/receptions are restricted to either SBFD symbols only or non-SBFD symbols only), it was agreed as the following to determine the valid symbol type: either SBFD symbols only or non-SBFD symbols only.

Agreement at RAN1#119 It is clarified that ‘the first transmission/reception’ in the following agreement in RAN1#118 refers to the first transmission/reception occasion indicated by scheduling DCI. - It is not expected that the first transmission/reception occasion indicated by scheduling DCI is mapped to both SBFD and non-SBFD symbols. Agreement at RAN1#118 For Configuration 1: The transmissions/receptions are restricted to SBFD symbols only or non-SBFD symbols only, the valid symbol type is determined as following. <Unrelated part omitted> - For dynamically scheduled transmissions/receptions, the valid symbol type is determined based on the symbol type of the first transmission/reception. <Unrelated part omitted>

In the case of multi-slot PUSCH transmissions, when a SBFD-aware UE is scheduled to transmit multi-slot PUSCHs with a single DCI according to the Configuration 1, and the first transmission occasion indicated by the DCI is on SBFD symbols, the SBFD-aware UE is allowed to transmit the multi-slot PUSCHs on SBFD symbols only, as per Configuration 1. Thus, the SBFD-aware UE increments the HPN for PUSCH transmission in the set of symbols within the UL subband on SBFD symbols only. The UE should not increment the HPN for PUSCH(s) not transmitted on non-SBFD UL symbols (i.e. legacy UL symbols), which do not overlap with a DL symbol indicated by tdd-UL-DL-ConfigCommon or tdd-UL-DL-ConfigDedicated, or a symbol of an SS/PBCH block with index provided by ssb-PositionsInBurst.

According to the current TS38.214 specification, the HPN for each subsequent PUSCH(s) in scheduling order on legacy UL symbols after the first transmission occasion is incremented. Therefore, for the case of multi-slot PUSCHs by a single DCI, the rule for HARQ process ID allocation should be revised for a SBFD-aware UE differently to current TS38.214 specification [9].

Similarly, in the case of multi-slot PDSCH transmissions, when a SBFD-aware UE is scheduled to receive multi-slot PDSCHs within a single DCI according to the Configuration 1, and the first reception occasion indicated by the DCI is on SBFD symbols, the SBFD-aware UE is allowed to receive the multi-slot PDSCHs on SBFD symbols only, as per the Configuration 1. Thus, the SBFD-aware UE increments the HPN for PDSCH reception in the set of symbols within the DL subband on SBFD symbols only. The UE should not increment the HPN for PDSCH(s) not received on non-SBFD DL symbols (i.e. legacy DL symbols).

Additionally, the HPN should not be incremented for PDSCH(s) that are not received if at least one of the symbols indicated by the indexed row of the used resource allocation table in the slot overlaps with a UL symbol, as indicated by tdd-UL-DL-ConfigCommon or tdd-UL-DL-ConfigDedicated. Therefore, for the case of multi-slot PDSCHs scheduled by a single DCI, the rule for HARQ process ID allocation should be revised for a SBFD-aware UE differently to current TS38.214 specification [9].

- Proposal 5: For each scheduling case of multi-slot PUSCHs by a single DCI and multi-slot PDSCHs by a single DCI with Configuration 1, the rule for HARQ process ID allocation should be revised for a SBFD-aware UE differently to the current specification.

  - When the first reception occasion indicated by the DCI is on SBFD symbols:

    - In case of multi-slot PDSCHs scheduling, the UE should not increment the HPN for any PDSCH(s) that is not received on non-SBFD DL symbols (i.e. legacy DL symbols).

    - In case of multi-slot PUSCHs scheduling, the UE should not increment the HPN for any PUSCH(s) that is not transmitted on non-SBFD UL symbols (i.e. legacy UL symbols) or on any symbol of an SS/PBCH block with index provided by ssb-PositionsInBurst.

  - When the first reception occasion indicated by the DCI is on non-SBFD symbols:

    - In case of multi-slot PDSCHs scheduling, the UE should not increment the HPN for any PDSCH(s) that is not received if at least one of the symbols indicated by the indexed row of the used resource allocation table in the slot overlaps with an SBFD symbol.

    - In case of multi-slot PUSCHs scheduling, the UE should not increment the HPN for any PUSCH(s) that is not transmitted if at least one of the symbols indicated by the indexed row of the used resource allocation table in the slot overlaps with an SBFD symbol or a symbol of an SS/PBCH block with index provided by ssb-PositionsInBurst.

PUSCH rate-matching in RE-level

In Rel-15, a RE-level rate-matching for PDSCH was specified around REs for LTE CRS (cell-specific reference signal) to guarantee LTE UE’s CRS detection for DL channel estimation in case of spectrum sharing between NR UE and LTE UE. In addition to LTE CRS, PDSCH rate-matching on REs for ZP (zero power) was specified to guarantee other NR UE’s ZP CSI-RS detection for beam management. A UE can be configured with a RE-level resource pattern for LTE CRS or NR ZP CSI-RS, and then the UE assumes that those REs are not available for PDSCH reception. If the UE is indicated to receive PDSCH in REs that overlap with configured RE-level resource pattern, the UE receives PDSCH by rate-matching REs for PDSCH around the REs for LTE CRS or NR ZP CSI-RS. Note that LTE CRS and NR ZP CSI-RS are likely to be configured in the DL symbol indicated by TDD-UL-DL-ConfigCommon or TDD-UL-DL-ConfigDedicated.

For SBFD-aware UEs, UL transmission can be scheduled within UL subband in a symbol configured as a DL symbol indicated by TDD-UL-DL-ConfigCommon or TDD-UL-DL-ConfigDedicated. Then, resources of PUSCH scheduled to the SBFD-aware UE within the UL subband can overlap with REs of LTE CRS or NR ZP CSI-RS. This can be a more critical issue than the Rel-15 PDSCH case due to CLI between DL reception and UL transmission. For this, a RE-level rate matching for PUSCH can be further investigated, and the Rel-15’s RE-level PDSCH rate matching can be further considered as a starting point.

- Proposal 6: For a SBFD-aware UE, RE-level PUSCH scheduled within UL subband in SBFD symbol(s) configured as DL via TDD-UL-DL-ConfigCommon

  - can be rate-matched around REs of LTE CRS configured for LTE UE in case of spectrum sharing between SBFD-aware UE and LTE UE.

  - can be rate-matched around REs of ZP CSI-RS configured for other NR UE.

Uplink cancellation indication

In Rel-16 URLLC WI, UL CI (cancellation indication) was introduced for inter-UE multiplexing in a cell. To reduce latency of UL transmission of higher priority (i.e., URLLC traffic), a UE can be configured to monitor PDCCH including DCI format 2_4 with CRC scrambled by CI-RNTI. An indication of UL CI for a serving cell is applicable to a PUSCH transmission or a SRS transmission in the cell. For the actual UL cancellation, a UE determines reference time-frequency resource region where UL CI is applied. In time domain, DL symbols configured by TDD-UL-DL-ConfigCommon and symbols configured for SSB reception are excluded from the reference time-frequency resource region of UL CI. In other words, UL symbols and symbols not configured for SSB reception can be included as reference time-frequency resource region of UL CI. This is to guarantee finer signaling granularity under the limited payload size of DCI format 2_4 by excluding time domain resources not used for UL transmission.

The UL CI feature can be considered for SBFD operation in consideration that UL latency reduction is one of the key objectives of Rel-19 duplex evolution WI. For example, UL transmission with lower priority (i.e., eMBB traffic) of a SBFD-aware UE within an UL subband can be indicated to cancel by UL CI for another SBFD-aware UE’s UL transmission within the UL subband with higher priority (i.e., URLLC traffic). Since an UL subband can be configured in symbols configured as DL configured by TDD-UL-DL-ConfigCommon or symbols(i.e., DL or Flexible symbols) configured for SSB reception, the SBFD-aware UE excludes those symbols as reference time-frequency resource region that UL CI is applied if legacy rule is used according to current specification. However, as illustrated in Figure 5, it seems reasonable that an UL subband in symbols configured as DL by TDD-UL-DL-ConfigCommon and symbols(i.e., DL or Flexible symbols) not configured for SSB reception should be included as reference time-frequency resource region for UL CI for a SBFD-aware UE in addition to UL symbols and flexible symbols not configured for SSB reception. Therefore, we propose that both non-SBFD symbols configured as DL by TDD-UL-DL-ConfigCommon and symbols configured for SSB reception are excluded and an UL subband in both symbols configured as DL by TDD-UL-DL-ConfigCommon and symbols (i.e., DL or Flexible symbols) not configured for SSB reception is included as reference time-frequency resource region for UL CI for a SBFD-aware UE

Figure 5. Determination of reference time-frequency resource region for UL CI for a SBFD-aware UE

- Proposal 7: For a SBFD-aware UE, we propose that

  - Both non-SBFD symbols configured as DL by TDD-UL-DL-ConfigCommon and symbols configured for SSB reception are excluded as reference time-frequency resource region for UL CI,

  - And an UL subband in both symbols configured as DL by TDD-UL-DL-ConfigCommon and symbols (i.e., DL or Flexible symbols) not configured for SSB reception should be included as reference time-frequency resource region for UL CI in addition to UL symbols and flexible symbols not configured for SSB reception.

When a time-frequency resource with UL subband configured for SBFD-aware UEs is included as a reference time-frequency resource region for UL CI, the reference time-frequency resource region determined by legacy UEs and SBFD-aware UEs may differ. If separate Layer 1 (L1) signalling for UL CI for SBFD-aware UEs is not introduced, the cancellation resources for UL CI should be indicated via DCI format 2_4 using group common signalling for both legacy UEs and SBFD-aware UEs, as in legacy operation. Further discussion is needed in RAN1 how to indicate cancellation of uplink signals/channels scheduled in the UL subband to SBFD-aware UEs using DCI format 2_4 as group common signalling without impacting the existing operation of legacy UEs. As an example, as illustrated in Figure 6, a method using remaining code points (e.g., all '0's) available for indication in UL CI can be considered to indicate the cancellation of uplink signals/channels scheduled in the UL subband while maintaining backward compatibility with legacy UE operation.

Figure 6. An example for common signalling to indicate cancellation resource for Ul CI to both legacy UE and SBFD-aware UE

- Proposal 8 : RAN1 should further discuss how to indicate cancellation of uplink signals/channels scheduled in the UL subband to SBFD-aware UEs using DCI format 2_4 as group common signalling without impacting the existing operation of legacy UEs.

Conclusion

In this contribution, we discussed issues such as semi-static configuration of SBFD subbands, UE transmission/reception/measurement behavior and procedures in SBFD symbols and/or non-SBFD symbols for SBFD aware UE for SBFD operation and summarize our views as the followings:

- Proposal 1: We propose to confirm the working assumption that cell-specific configuration on frequency location of SBFD subbands is supported within a TDD carrier.

  - UE-specific configuration on frequency locations of SBFD subbands is not supported.

- Proposal 2: PUSCH transmissions of PUSCH repetition or TB processing over multiple slots, or PUCCH transmissions of PUCCH repetition within UL subband across SBFD slots or symbols configured as DL by tdd-UL-DL-ConfigCommon should not be an event which causes power consistency and phase continuity not to be maintained for actual TDW determination for a SBFD-aware UE.

- Proposal 3: If separate UL power control parameters are applied across SBFD and non-SBFD symbols for Configuration 2, it should be considered as an event which causes power consistency and phase continuity not to be maintained and different actual TDWs for different symbol types should be determined for PUSCH transmissions of PUSCH repetition or PUCCH transmissions of PUCCH repetition.

- Proposal 4: In addition to availability in time domain, availability in frequency domain should be further considered when determining the HPN for multi-slot PUSCHs scheduled by a single DCI for an SBFD-aware UE with Configuration 2.

  - The HPN of allocated PUSCH should be incremented if its allocated frequency resources are within the UL usable PRBs on SBFD symbols regardless of the symbol type of the allocated PUSCH is configured as DL or Flexible symbols, as indicated by tdd-UL-DL-ConfigCommon or tdd-UL-DL-ConfigDedicated.

  - The HPN of allocated PUSCH should not be incremented for a PUSCH that is not transmitted due to a collision between SSB symbols indexed by ssb-PositionsInBurst and one of multiple PUSCHs on SBFD symbols in the time domain.

- Proposal 5: For each scheduling case of multi-slot PUSCHs by a single DCI and multi-slot PDSCHs by a single DCI with Configuration 1, the rule for HARQ process ID allocation should be revised for a SBFD-aware UE differently to the current specification.

  - When the first reception occasion indicated by the DCI is on SBFD symbols,

    - In case of multi-slot PDSCHs scheduling, the UE should not increment the HPN for any PDSCH(s) that is not received on non-SBFD DL symbols (i.e. legacy DL symbols).

    - In case of multi-slot PUSCHs scheduling, the UE should not increment the HPN for any PUSCH(s) that is not transmitted on non-SBFD UL symbols (i.e. legacy UL symbols) or on any symbol of an SS/PBCH block with index provided by ssb-PositionsInBurst.

  - When the first reception occasion indicated by the DCI is on non-SBFD symbols,

    - In case of multi-slot PDSCHs scheduling, the UE should not increment the HPN for any PDSCH(s) that is not received if at least one of the symbols indicated by the indexed row of the used resource allocation table in the slot overlaps with an SBFD symbol.

    - In case of multi-slot PUSCHs scheduling, the UE should not increment the HPN for any PUSCH(s) that is not transmitted if at least one of the symbols indicated by the indexed row of the used resource allocation table in the slot overlaps with an SBFD symbol or a symbol of an SS/PBCH block with index provided by ssb-PositionsInBurst.

- Proposal 6: For a SBFD-aware UE, RE-level PUSCH scheduled within UL subband in a symbol configured as DL via TDD-UL-DL-ConfigCommon

  - can be rate-matched around REs of LTE CRS configured for LTE UE in case of spectrum sharing between SBFD-aware UE and LTE UE.

  - can be rate-matched around REs of ZP CSI-RS configured for other NR UE.

- Proposal 7: For a SBFD-aware UE, we propose that

  - Both non-SBFD symbols configured as DL by TDD-UL-DL-ConfigCommon and symbols configured for SSB reception are excluded as reference time-frequency resource region for UL CI,

  - And an UL subband in both symbols configured as DL by TDD-UL-DL-ConfigCommon and symbols (i.e., DL or Flexible symbols) not configured for SSB reception should be included as reference time-frequency resource region for UL CI in addition to UL symbols and flexible symbols not configured for SSB reception.

- Proposal 8 : RAN1 should further discuss how to indicate cancellation of uplink signals/channels scheduled in the UL subband to SBFD-aware UEs using DCI format 2_4 as group common signalling without impacting the existing operation of legacy UEs.

References

- Draft Report of 3GPP TSG RAN WG1 #120bis meeting v0.1.0

- Final Report of 3GPP TSG RAN WG1 #120 meeting v1.0.0

- Final Report of 3GPP TSG RAN WG1 #119 meeting v1.0.0

- Final Report of 3GPP TSG RAN WG1 #118-bis meeting v1.0.0

- Final Report of 3GPP TSG RAN WG1 #118 meeting v1.0.0

- Final Report of 3GPP TSG RAN WG1 #117 meeting v1.0.0

- Final Report of 3GPP TSG RAN WG1 #116-bis meeting v1.0.0

- Final Report of 3GPP TSG RAN WG1 #116 meeting v1.0.0

- 3GPP TS 38.214 V18.6.0 (2025-03)