3GPP TSG-RAN WG1 Meeting #121 	R1-2504645

St Julian’s, Malta, May 19th – 23th, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| [DRAFT] CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.212 | CR | xxxx | rev | - | Current version: | 17.11.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on NCJT CSI reporting |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_feMIMO-Core |  |  |  |  |  | Date: |  |  | 2025-05-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Rel-17 NCJT CSI reporting only supports type-I single panel codebook, however, the multi-panel codebook is referred to in mapping order of NCJT CSI reporting. The zero padding bits for NCJT CSI report with Mode 2 is not described when reported in PUSCH. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In table of mapping order of NCJT CSI reporting, the reference of multi-panel codebook is removed. For the zero padding bits for NCJT CSI report with Mode 2, change the reference table to NCJT CSI reporting on PUSCH. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Causes confusion that NCJT reporting also supports multi-panel codebook. And there’s no specification of zero padding bits for NCJT CSI report with Mode 2 when reported in PUSCH. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.1.1.2, 6.3.2.1.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

6.3.1.1.2	CSI only

<Unchanged parts are omitted>

Table 6.3.1.1.2-11B: Mapping order of CSI fields of one CSI report, CSI part 2 subband, csi-ReportMode= Mode 2

| CSI report #n Part 2 subband | PMI subband information fields of all even subbands with increasing order of subband number, from left to right as in Tables 6.3.1.1.2-1, or codebook index for 2 antenna ports associated with the first resource within the CSI-RS resource pair, according to Clause 5.2.2.2.1 in [6, TS38.214] of all even subbands with increasing order of subband number, if pmi-FormatIndicator= subbandPMI and reported part 1 is associated with one CSI-RS resource pair and if reported |
| --- | --- |
|  | PMI subband information fields of all even subbands with increasing order of subband number, from left to right as in Tables 6.3.1.1.2-1, or codebook index for 2 antenna ports associated with the second resource within the CSI-RS resource pair, according to Clause 5.2.2.2.1 in [6, TS38.214] of all even subbands with increasing order of subband number, if pmi-FormatIndicator= subbandPMI and reported part 1 is associated with one CSI-RS resource pair and if reported |
|  | PMI subband information fields of all odd subbands with increasing order of subband number, from left to right as in Tables 6.3.1.1.2-1, or codebook index for 2 antenna ports associated with the first resource within the CSI-RS resource pair, according to Clause 5.2.2.2.1 in [6, TS38.214] of all odd subbands with increasing order of subband number, if pmi-FormatIndicator= subbandPMI and reported part 1 is associated with one CSI-RS resource pair and if reported |
|  | PMI subband information fields of all odd subbands with increasing order of subband number, from left to right as in Tables 6.3.1.1.2-1, or codebook index for 2 antenna ports associated with the second resource within the CSI-RS resource pair, according to Clause 5.2.2.2.1 in [6, TS38.214] of all odd subbands with increasing order of subband number, if pmi-FormatIndicator= subbandPMI and reported part 1 is associated with one CSI-RS resource pair and if reported |
|  | Subband differential CQI for the second TB of all even subbands with increasing order of subband number associated with one CSI-RS resource, as in Tables 6.3.1.1.2-3B, if cqi-FormatIndicator=subbandCQI and reported part 1 is associated with one CSI-RS resource and if reported |
|  | PMI subband information fields of all even subbands with increasing order of subband number, from left to right as in Tables 6.3.1.1.2-1, or codebook index for 2 antenna ports associated with one CSI-RS resource according to Clause 5.2.2.2.1 in [6, TS38.214] of all even subbands with increasing order of subband number, if pmi-FormatIndicator= subbandPMI and reported part 1 is associated with one CSI-RS resource and if reported |
|  | Subband differential CQI for the second TB of all odd subbands with increasing order of subband number associated with one CSI-RS resource, as in Tables 6.3.1.1.2-3B, if cqi-FormatIndicator=subbandCQI and reported part 1 is associated with one CSI-RS resource and if reported |
|  | PMI subband information fields of all odd subbands with increasing order of subband number, from left to right as in Tables 6.3.1.1.2-1, or codebook index for 2 antenna ports associated with one CSI-RS resource according to Clause 5.2.2.2.1 in [6, TS38.214] of all odd subbands with increasing order of subband number, if pmi-FormatIndicator= subbandPMI and reported part 1 is associated with one CSI-RS resource and if reported |

If none of the CSI reports for transmission on a PUCCH is of two parts, the CSI fields of all CSI reports, in the order from upper part to lower part in Table 6.3.1.1.2-12, are mapped to the UCI bit sequence  starting with . The most significant bit of each field is mapped to the lowest order information bit for that field, e.g. the most significant bit of the first field is mapped to.

<Unchanged parts are omitted>

##### 6.3.2.1.2	CSI only

<Unchanged parts are omitted>

Table 6.3.2.1.2-3B: Mapping order of CSI fields of one CSI report, CSI part 1, csi-ReportMode= Mode 2

| CSI report number | CSI fields |
| --- | --- |
| CSI report #n CSI part 1 | CRI as in Tables 6.3.1.1.2-3A, if associated with one CSI-RS resource pair and if reported; CRI as in Tables 6.3.1.1.2-3B, if associated with one CSI-RS resource and if reported |
|  | Rank Combination Indicator as in Tables 6.3.1.1.2-3A, if associated with one CSI-RS resource pair and if reported; Rank Indicator as in Tables 6.3.1.1.2-3B, if associated with one CSI-RS resource and if reported; Zero padding bits OP, if needed |
|  | Wideband CQI for the first TB as in Tables 6.3.1.1.2-3A, if associated with one CSI-RS resource pair and if reported; Wideband CQI for the first TB as in Tables 6.3.1.1.2-3B, if associated with one CSI-RS resource and if reported |
|  | Subband differential CQI for the first TB with increasing order of subband number as in Tables 6.3.1.1.2-3A, if associated with one CSI-RS resource pair and if reported; Subband differential CQI for the first TB with increasing order of subband number as in Tables 6.3.1.1.2-3B, if associated with one CSI-RS resource and if reported |
| Note: Subbands for given CSI report n indicated by the higher layer parameter csi-ReportingBand are numbered continuously in the increasing order with the lowest subband of csi-ReportingBand as subband 0. |  |

The number of zero padding bits OP in Table 6.3.2.1.2-3B is 0 for 1 CSI-RS port and OP=Nmax-Nreported(R) for more than 1 CSI-RS port, where

-	Nmax=maxr∈SRankN(r) . SRank is the set of rank and rank combination values r that are allowed to be reported. Nr is obtained according to Tables 6.3.1.1.2-3A/3B for rank combination indicator and rank indicator respectively.

-	Nreported (R) is obtained according to Tables 6.3.1.1.2-3A for rank combination indicator and R is the reported rank combination

-	Nreported (R) is obtained according to Tables 6.3.1.1.2-3B for rank indicator and R is the reported rank

Table 6.3.2.1.2-4: Mapping order of CSI fields of one CSI report, CSI part 2 wideband

| CSI report number | CSI fields |
| --- | --- |
| CSI report #n CSI part 2 wideband | Wideband CQI for the second TB as in Tables 6.3.1.1.2-3/4/5, if present and reported |
|  | Layer Indicator as in Tables 6.3.1.1.2-3/4/5, if reported |
|  | PMI wideband information fields , from left to right as in Tables 6.3.1.1.2-1/2 or 6.3.2.1.2-1/2, if reported |
|  | PMI wideband information fields , from left to right as in Tables 6.3.1.1.2-1/2 or 6.3.2.1.2-1/2, or codebook index for 2 antenna ports according to Clause 5.2.2.2.1 in [6, TS38.214], if pmi-FormatIndicator= widebandPMI and if reported |

<Unchanged parts are omitted>