3GPP TSG RAN WG1 #121		                                         R1-2504657

St Julian’s, Malta, May 19th – 23th, 2025

Agenda item:	9.15.3

Source: 	Moderator (NTT DOCOMO, INC.)

Title: 	Summary of discussion on Rel-19 NR UE features for SBFD

Document for:	Discussion and Decision

- Introduction

This document summarizes contributions submitted to AI 9.15.3 regarding Rel-19 NR UE features for NR_duplex_evo.

[121-R19-UE_features] Email discussion on Rel-19 UE features – Ralf (AT&T), Naoya (DOCOMO) - To be used for sharing updates on online/offline schedule, details on what is to be discussed in online/offline sessions, tdoc number of the moderator summary for online session, etc

- FGs for SBFD operation

2.1	Basic FG for SBFD

Following inputs are provided in contributions for the RAN1#121 meeting.

| [1] | Huawei, HiSilicon | Basic UE feature group for SBFD The following components were agreed for the basic FG for an SBFD-aware UE 1. Support of cell-common configuration of time and frequency location of SBFD subbands 2. Support of UL transmission in one UL subband and DL reception in one [and two] DL subband in SBFD symbols 3. Support of link direction determination based on configured/scheduled transmissions/receptions and collision handling 4. Support of UL transmissions or DL receptions across SBFD symbols and non-SBFD symbols in different slots, where the transmissions or receptions are restricted to SBFD symbols or non-SBFD symbols (Configuration 1) 5. Support of separate CSI reports for SBFD and non-SBFD symbols FFS: Whether to include two DL subband support in component 2 FFS: other component (which may be separate FG). One remaining issue is whether to include the support of DL reception across two non-contiguous DL subbands into the basic feature group. Our preference is to define it as a separate FG since it has impact to both PDSCH reception and CSI measurement. Another remaining issue is whether the following two components are defined as separate UE features. - Support of separate UL power control for PUSCH/PUCCH/SRS transmissions in SBFD symbols and non-SBFD symbols - Support of separate SRS configurations in SBFD symbols and non-SBFD symbols Our preference is to include both in the basic feature group. Proposal 1: Include the following additional components into FG 60-1 - Support of separate UL power control for PUSCH/PUCCH/SRS transmissions in SBFD symbols and non-SBFD symbols - Support of separate SRS configurations in SBFD symbols and non-SBFD symbols |
| --- | --- | --- |
| [2] | ZTE Corporation, Sanechips | Basic UE feature for SBFD operation In RAN1#120bis, the following agreements were reached on the basic UE feature for SBFD. 60-1; Basic feature for SBFD-aware UE TX/RX/measurement behaviour and procedures; 1. Support of cell-common configuration of time and frequency location of SBFD subbands 2. Support of UL transmission in one UL subband and DL reception in one[/two] DL subband in SBFD symbols 3. Support of link direction determination based on configured/scheduled transmissions/receptions and collision handling 4. Support of UL transmissions or DL receptions across SBFD symbols and non-SBFD symbols in different slots, where the transmissions or receptions are restricted to SBFD symbols or non-SBFD symbols (Configuration 1) 5. Support of separate CSI reports for SBFD and non-SBFD symbols 6. Support of separate PUCCH frequency resource configuration in SBFD symbols and non-SBFD symbols [FFS: 7. Support of separate UL power control parameters based on unified TCI framework in SBFD symbols and non-SBFD symbols] 8. Support of separate PUSCH frequency hopping offset in SBFD symbols and non-SBFD symbols [FFS: 9. Support of separate SRS resource set configuration in SBFD symbols and non-SBFD symbols] FFS: Whether to include two DL subband support in component 2 FFS: other component (which may be separate FG); YES; n/a; Basic feature for SBFD-aware UE TX/RX/ measurement behaviour and procedures is not supported; Per band; TDD only; n/a; n/a; [Note: Only UL transmissions are allowed within UL subband and only DL receptions (except for CLI measurement) are allowed within DL subband(s)] [Note: The maximum number of UL subbands for SBFD operation in an SBFD symbol within a TDD carrier is one.]; Optional with capability signalling. For the basic FG, our views are summarized as below. - Add the following components in the basic FG. - Support of DL reception in two non-contiguous DL subbands. Most of the procedure for PDSCH reception in two non-contiguous DL subbands are not changed compared to legacy, .e.g, VRB-to-PRB mapping and DMRS sequence etc. The main change is TBS determination, which in our view is a basic change for SBFD operation and does not require one additional FG. - Support of separate UL configurations for SBFD and non-SBFD symbols, including component 7 and 9. - Unaligned boundaries between SBFD subband(s) and RBG, CSI reporting subband, CSI-RS resource, PRG - Support of CSI-RS reception in two non-contiguous DL subbands as part of basic FG for SBFD operation. Supported CSI processing timeline for CSI-RS reception in two non-contiguous DL subbands should be reported as component of the basic FG for SBFD. - Two candidate values may be reported: {CSI processing timeline and CPU counting are the same as legacy, scale the CSI processing timeline Z/Z’ by factor of 2 and keep the same CPU counting as legacy}. - Supported maximum number of DL partial PRGs for PDSCH in SBFD symbols should be reported as part of basic FG for SBFD operation - Candidate values for supported maximum number of DL partial PRGs is {2, 4}. If two DL subbands are configured and value 2 is reported by the UE, a PDSCH transmission is valid as long as it only contains up to 2 partial PRGs, no matter the partial PRG are caused by the boundary of BWP as legacy or it is caused by the boundary of DL subband. Otherwise, a PDSCH transmission is valid no matter which and how many partial PRGs the PDSCH contains. - For PUSCH repetition type B, a nominal repetition is segmented into actual repetitions around boundary of SBFD symbols and non-SBFD symbols. - No need of FR1/FR2 differentiation since per band reporting is supported. - To keep the two bullets in the note column. With above, we propose to revise the basic FG per following proposal. Proposal 1: Support the following revision on the basic FG for SBFD operation. 60-1; Basic feature for SBFD-aware UE TX/RX/measurement behaviour and procedures; 1. Support of cell-common configuration of time and frequency location of SBFD subbands 2. Support of UL transmission in one UL subband and DL reception in one/two DL subband in SBFD symbols 3. Support of link direction determination based on configured/scheduled transmissions/receptions and collision handling 4. Support of UL transmissions or DL receptions across SBFD symbols and non-SBFD symbols in different slots, where the transmissions or receptions are restricted to SBFD symbols or non-SBFD symbols (Configuration 1) 5. Support of separate CSI reports for SBFD and non-SBFD symbols 6. Support of separate PUCCH frequency resource configuration in SBFD symbols and non-SBFD symbols 7. Support of separate UL power control parameters based on unified TCI framework in SBFD symbols and non-SBFD symbols 8. Support of separate PUSCH frequency hopping offset in SBFD symbols and non-SBFD symbols 9. Support of separate SRS resource set configuration in SBFD symbols and non-SBFD symbols 10. Unaligned boundaries between SBFD subband(s) and RBG, CSI reporting subband, CSI-RS resource, PRG 11. Support of CSI-RS reception in two non-contiguous DL subbands as part of basic FG for SBFD operation 12. Supported maximum number of DL partial PRGs for PDSCH in SBFD symbols 13. For PUSCH repetition type B, a nominal repetition is segmented into actual repetitions around boundary of SBFD symbols and non-SBFD symbols.; YES; n/a; Basic feature for SBFD-aware UE TX/RX/ measurement behaviour and procedures is not supported; Per band; TDD only; n/a; n/a; For component 11, candidate values: {CSI processing timeline and CPU counting are the same as legacy, scale the CSI processing timeline Z/Z’ by factor of 2 and keep the same CPU counting as legacy} For component 12, candidate values: {2, 4} Note: Only UL transmissions are allowed within UL subband and only DL receptions (except for CLI measurement) are allowed within DL subband(s)] [Note: The maximum number of UL subbands for SBFD operation in an SBFD symbol within a TDD carrier is one.; Optional with capability signalling. |
| [3] | vivo | - SBFD TX/RX/measurement procedures - Following questions on UE features for SBFD TX/RX/measurement procedures were discussed. - Whether ‘support of PDSCH receptions in two non-contiguous DL subbands’ should be part of basic FG for SBFD operation or separate FG - Whether ‘support of CSI-RS receptions in two non-contiguous DL subbands’ should be part of basic FG for SBFD operation or separate FG - Whether supported maximum number of DL partial PRGs for PDSCH in SBFD symbols should be reported as part of basic FG for SBFD operation or part of FG for PDSCH reception in two non-contiguous DL subbands - Regarding the questions, we have the following proposals. Proposal 1: Support of PDSCH receptions in two non-contiguous DL subbands’ should be part of basic FG for SBFD operation. Proposal 2: For CSI-RS receptions in two non-contiguous DL subbands, following alternatives can be considered. - Alt.1: Support of CSI-RS reception in two non-contiguous DL subbands should be part of basic FG for SBFD operation. - Supported CSI processing timeline for CSI-RS reception in two non-contiguous DL subbands should be reported as a component of the basic FG for SBFD. - Two candidate values may be reported: {CSI processing timeline and CPU counting are the same as legacy, scale the CSI processing timeline Z/Z’ by factor of 2 and keep the same CPU counting as legacy}. - Alt.3: Support of CSI-RS reception in two non-contiguous DL subbands with scaled CSI processing timeline Z/Z’ by factor of 2 should be part of the basic FG for SBFD operation. Support of CSI-RS reception in two non-contiguous DL subbands with legacy CSI processing timeline Z/Z’ should be a separate FG. Proposal 3: Supported maximum number of DL partial PRGs for PDSCH in SBFD symbols should be reported as part of basic FG for SBFD operation. - In summary, for SBFD Tx/Rx operation, following FGs are suggested. Proposal 4: Support followings FGs for SBFD operation. 60-1; Basic feature for SBFD-aware UE TX/RX/measurement behaviour and procedures; 1. Support of cell-common configuration of time and frequency location of SBFD subbands 2. Support of UL transmission in one UL subband and DL reception in one/two DL subband in SBFD symbols 3. Support of link direction determination based on configured/scheduled transmissions/receptions and collision handling 4. Support of UL transmissions or DL receptions across SBFD symbols and non-SBFD symbols in different slots, where the transmissions or receptions are restricted to SBFD symbols or non-SBFD symbols (Configuration 1) 5. Support of separate CSI reports for SBFD and non-SBFD symbols 6. Support of separate PUCCH frequency resource configuration in SBFD symbols and non-SBFD symbols 7. Support of separate UL power control parameters based on unified TCI framework in SBFD symbols and non-SBFD symbols 8. Support of separate PUSCH frequency hopping offset in SBFD symbols and non-SBFD symbols 9. Support of separate SRS resource set configuration in SBFD symbols and non-SBFD symbols 10. Support of CSI-RS reception in two non-contiguous DL subbands with [scaled CSI processing timeline Z/Z’ by factor of 2, or legacy CSI processing timeline Z/Z’] 11. Support of DL partial PRG. Maximum number of supported DL partial PRGs in SBFD symbols is [{2,3,4}] FFS: Whether to include two DL subband support in component 2 FFS: other component (which may be separate FG); YES; n/a; Basic feature for SBFD-aware UE TX/RX/ measurement behaviour and procedures is not supported; Per band; TDD only; n/a; n/a; [Note: Only UL transmissions are allowed within UL subband and only DL receptions (except for CLI measurement) are allowed within DL subband(s)] [Note: The maximum number of UL subbands for SBFD operation in an SBFD symbol within a TDD carrier is one.] For component 10, candidate values may be reported: {CSI processing timeline and CPU counting are the same as legacy, scale the CSI processing timeline Z/Z’ by factor of 2 and keep the same CPU counting as legacy}.; Optional with capability signalling. 60-2; Transmission and Reception across SBFD symbols and non-SBFD symbols in different slots (Configuration 2); 1. Support of PDSCH reception across SBFD symbols and non-SBFD symbols (Configuration 2) 2. Support of UL transmission across SBFD symbols and non-SBFD symbols (Configuration 2) FFS: Other component(s) FFS: whether component 2 is defined as separate FG; 60-1; YES; n/a; Transmission and Reception across SBFD symbols and non-SBFD symbols in different slots (Configuration 2) is not supported; Per Band; TDD only; n/a; n/a; Optional with capability signalling. |
| [4] | Nokia | FG 60-1 Basic FG 60-1 for SBFD was updated in RAN1#120b as shown in the Annex. In this section we discuss the remaining issues (highlighted in yellow) related FG 60-1. Regarding remaining issues on the following components: 2. Support of UL transmission in one UL subband and DL reception in one[/two] DL subband in SBFD symbols FFS: 7. Support of separate UL power control parameters based on unified TCI framework in SBFD symbols and non-SBFD symbols] [FFS: 9. Support of separate SRS resource set configuration in SBFD symbols and non-SBFD symbols] FFS: Whether to include two DL subband support in component 2 FFS: other component (which may be separate FG). For component 2, supporting one DL subband in SBFD symbol seems reasonable for a basic FG. However, we are fine to also consider two DL subbands. For component 7, we support including this component. For component 9, this component doesn’t seem necessarily to be in basic feature. However, we are open for more discussion. Regarding the need for FR1/FR2 differentiation, no differentiation is expected. Regarding the note, the note should be captured in the corresponding technical specifications, thus it can be removed from the FG 60-1 description. Proposal 1. For FG 60-1, - Support component 7, - Further discuss the square brackets in component 2, - Further discuss the support of component 9, - Need of FR1/FR2 differentiation: n/a, - Note: Removing the current notes. |
| [5] | MediaTek Inc. | In this section, we provide the list of UE features to be supported for SBFD operation, including features that are basic and supported by all SBFD capable UEs and some addition features that may or may not be supported by a SBFD aware UE. Our views are based on at least agreements made in agenda item 9.3.1 of R19 evolution of NR duplex operation. Agreement: Add the following components in FG 60-1 (basic FG for SBFD operation) - Support of separate PUCCH frequency resource configuration in SBFD symbols and non-SBFD symbols - [FFS: Support of separate UL power control parameters based on unified TCI framework in SBFD symbols and non-SBFD symbols] - Support of separate PUSCH frequency hopping offset in SBFD symbols and non-SBFD symbols - [FFS: Support of separate SRS resource set configuration in SBFD symbols and non-SBFD symbols] Question 2-2-1: Companies are encouraged to provide their views on whether ‘support of PDSCH receptions in two non-contiguous DL subbands’ should be part of basic FG for SBFD operation or separate FG. - Alt.1: Support of PDSCH reception in two non-contiguous DL subbands should be part of basic FG for SBFD operation. - Alt.2: Support of PDSCH reception in two non-contiguous DL subbands should be separate FG. #### Question 2-2-2: Companies are encouraged to provide their views on whether ‘support of CSI-RS receptions in two non-contiguous DL subbands’ should be part of basic FG for SBFD operation or separate FG. - Alt.1: Support of CSI-RS reception in two non-contiguous DL subbands should be part of basic FG for SBFD operation. - Supported CSI processing timeline for CSI-RS reception in two non-contiguous DL subbands should be reported as component of the basic FG for SBFD. - Two candidate values may be reported: {CSI processing timeline and CPU counting are the same as legacy, scale the CSI processing timeline Z/Z’ by factor of 2 and keep the same CPU counting as legacy}. - Alt.2: Support of CSI-RS reception in two non-contiguous DL subbands should be separate FG. - Supported CSI processing timeline for CSI-RS reception in two non-contiguous DL subbands should be reported as component of the FG. - Two candidate values may be reported: {CSI processing timeline and CPU counting are the same as legacy, scale the CSI processing timeline Z/Z’ by factor of 2 and keep the same CPU counting as legacy}. - Note: If Alt.2 in Question 2-2-1 is agreed, whether single or separate FG for PDSCH reception and CSI-RS reception in two DL subbands can be further discussed. - Alt.3: Support of CSI-RS reception in two non-contiguous DL subbands with scaled CSI processing timeline Z/Z’ by factor of 2 should be part of the basic FG for SBFD operation. Support of CSI-RS reception in two non-contiguous DL subbands with legacy CSI processing timeline Z/Z’ should be separate FG. Question 2-2-3: If Alt.2 in Question 2-2-1 is supported and Alt.2 in Question 2-2-2 is supported, companies are encouraged to provide their views on whether separate FGs are necessary for PDSCH reception in two non-contiguous DL subbands and CSI-RS reception in two non-contiguous DL subbands. - Alt.1: Single separate FG for PDSCH reception in two non-contiguous DL subbands and CSI-RS reception in two non-contiguous DL subbands. - Alt.2: Two separate FGs for PDSCH reception in two non-contiguous DL subbands and CSI-RS reception in two non-contiguous DL subbands respectively. Agreement: Introduce following FG for [Transmissions and] Receptions across SBFD and non-SBFD symbols (Configuration 2) as follows, and FFS on whether transmission across SBFD and non-SBFD symbols is defined as separate FG or not. - Alt.1: Configuration 2 for transmission and reception should be defined in a single FG - Alt.2: Configuration 2 for transmission and reception should be defined in separate FGs. In RAN1#120bis, RAN1 discussed some basic UE features as well as some separate UE features for SBFD Tx/Rx/measurement behaviour. We provide our views on the discussed issues below: Support of separate UL configurations for SBFD and non-SBFD symbol There was a discussion on whether ‘Support of separate UL configurations for SBFD and non-SBFD symbols’ should be part of basic FG for SBFD or separate FG(s). In RAN1#120bis, RAN1 agreed to include the following in the basic FG for SBFD operation. - Support of separate PUCCH frequency resource configuration in SBFD symbols and non-SBFD symbols - Support of separate PUSCH frequency hopping offset in SBFD symbols and non-SBFD symbols In addition to the above, we prefer to have the following UL configurations for SBFD and non-SBFD symbols as part of the basic FG for SBFD operation. - Support of separate UL power control parameters based on unified TCI framework in SBFD symbols and non-SBFD symbols. - Support of separate SRS resource set configuration in SBFD symbols and non-SBFD symbols. - Add the following components in FG 60-1 (basic FG for SBFD operation). - Support of separate UL power control parameters based on unified TCI framework in SBFD symbols and non-SBFD symbols. - Support of separate SRS resource set configuration in SBFD symbols and non-SBFD symbols. |
| [6] | Spreadtrum, UNISOC | The following agreements were made in RAN1#120bis. Agreement: Introduce following basic FG for SBFD, and FFS on some potential components (that may be separate FGs). 60-1; Basic feature for SBFD-aware UE TX/RX/measurement behaviour and procedures; 1. Support of cell-common configuration of time and frequency location of SBFD subbands 2. Support of UL transmission in one UL subband and DL reception in one [and two] DL subband in SBFD symbols 3. Support of link direction determination based on configured/scheduled transmissions/receptions and collision handling 4. Support of UL transmissions or DL receptions across SBFD symbols and non-SBFD symbols in different slots, where the transmissions or receptions are restricted to SBFD symbols or non-SBFD symbols (Configuration 1) 5. Support of separate CSI reports for SBFD and non-SBFD symbols FFS: Whether to include two DL subband support in component 2 FFS: other component (which may be separate FG); YES; n/a; Basic feature for SBFD-aware UE TX/RX/ measurement behaviour and procedures is not supported; Per Band; TDD only; n/a; n/a; [Note: Only UL transmissions are allowed within UL subband and only DL receptions (except for CLI measurement) are allowed within DL subband(s)] [Note: The maximum number of UL subbands for SBFD operation in an SBFD symbol within a TDD carrier is one.]; Optional with capability signalling. Add the following components in FG 60-1 (basic FG for SBFD operation) - Support of separate PUCCH frequency resource configuration in SBFD symbols and non-SBFD symbols - [FFS: Support of separate UL power control parameters based on unified TCI framework in SBFD symbols and non-SBFD symbols] - Support of separate PUSCH frequency hopping offset in SBFD symbols and non-SBFD symbols - [FFS: Support of separate SRS resource set configuration in SBFD symbols and non-SBFD symbols]. For component 2, support of DL reception in two DL subband in SBFD symbols should be separate FG. For separate SRS resource set configuration in SBFD symbols and non-SBFD symbols, it should be part of basic FG. It is the basic parameter for UL transmission in SBFD symbols. For separate UL power control parameters based on unified TCI framework in SBFD symbols and non-SBFD symbols, it should be included in separate FG. For the note column, we are fine to keep it. FG for Separate UL power control In RAN1#120bis, whether to add the separate UL power control parameters based on unified TCI framework in SBFD symbols and non-SBFD symbols in basic FG was discussed. If it added in basic FG, FG 23-1-1h (Association between TCI state and UL PC settings for PUCCH, PUSCH, and SRS) should be the prerequisite FG of basic SBFD FG. It means if FG 23-1-1h is not supported, the basic SBFD operation cannot be supported. So separate UL power control parameters based on unified TCI framework in SBFD symbols and non-SBFD symbols should be included in separate FG 60-1c. FG 23-1-1h (Association between TCI state and UL PC settings for PUCCH, PUSCH, and SRS) should be the prerequisite FG of FG 60-1c. |
| [7] | Samsung | Regarding the basic FG for SBFD (60-1), our view is - FFS: Whether to include two DL subband support in component 2 - The basic SBFD feature should be to DU/UD case as it is simpler than DUD case which can be a separate feature - FFS: Support of separate UL power control parameters based on unified TCI framework in SBFD symbols and non-SBFD symbols] - As per symbol type, UE needs to determine UE power control parameter, which requires additional UE complexity. However, the additional complexity is quite marginal since the SBFD symbol determination is semi-static and a UE already have different UL power control parameters associated with TCI state. - FFS: Support of separate SRS resource set configuration in SBFD symbols and non-SBFD symbols - For periodic and semi-persistent SRS, the additional complexity for separate SRS resource sets could be marginal since gNB will configure a proper periodicity and offset. For aperiodic SRS without available slot counting, again gNB will indicated a proper slot for SRS transmission. For aperiodic SRS with available slot counting, a UE requires to find a valid slot for SRS transmission. This is similar as PUSCH repetition type-A or PUCCH repetition in case of configuration 1. Since configuration 1 is already supported in Basic feature 60-1, the additional complexity could be marginal. - FFS: other component (which may be separate FG) - Link direction determination should be supported as basic FG. Otherwise, UE cannot resolve any collisions on SBFD symbol Proposal 1. Support the following for basic FG 60-1 60-1; Basic feature for SBFD-aware UE TX/RX/measurement behaviour and procedures; 1. Support of cell-common configuration of time and frequency location of SBFD subbands 2. Support of UL transmission in one UL subband and DL reception in one [and two] DL subband in SBFD symbols 3. Support of link direction determination based on configured/scheduled transmissions/receptions and collision handling 4. Support of UL transmissions or DL receptions across SBFD symbols and non-SBFD symbols in different slots, where the transmissions or receptions are restricted to SBFD symbols or non-SBFD symbols (Configuration 1) 5. Support of separate CSI reports for SBFD and non-SBFD symbols 6. Support of separate PUCCH frequency resource configuration in SBFD symbols and non-SBFD symbols 7. Support of separate UL power control parameters based on unified TCI framework in SBFD symbols and non-SBFD symbols] 8. Support of separate PUSCH frequency hopping offset in SBFD symbols and non-SBFD symbols 9. Support of separate SRS resource set configuration in SBFD symbols and non-SBFD symbols 10. Link direction determination in SBFD symbols FFS: Whether to include two DL subband support in component 2 FFS: other component (which may be separate FG); YES; n/a; Basic feature for SBFD-aware UE TX/RX/ measurement behaviour and procedures is not supported; Per band; TDD only; n/a; n/a; [Note: Only UL transmissions are allowed within UL subband and only DL receptions (except for CLI measurement) are allowed within DL subband(s)] [Note: The maximum number of UL subbands for SBFD operation in an SBFD symbol within a TDD carrier is one.]; Optional with capability signalling. |
| [8] | Ofinno | FGs for SBFD TX/RX operation To support Configuration 1 during initial access, it is our understanding that configurations related to UL transmission should be included in the basic FG, including power control parameter. On the other hand, FL collected companies' views regarding whether “support of PDSCH receptions in two non-contiguous DL subbands” should be part of the basic FG for SBFD operation or handled as a separate FG in the previous meeting. The following alternatives were presented during the discussion: - Alt.1: Support of PDSCH reception in two non-contiguous DL subbands should be part of the basic FG for SBFD operation. - Alt.2: Support of PDSCH reception in two non-contiguous DL subbands should be a separate FG. In our view, support for downlink reception across two non-contiguous DL subbands, including CSI measurements, is considered a separate FG since it may require additional UE complexity in terms of filtering. When such two subband configurations are used, it should be noted that the UE reports the maximum number of supported PRGs accordingly. Although the term “usable PRBs” is currently used for discussion purposes, it may be helpful to clearly define this term in the specification to avoid potential misunderstanding by future readers. Furthermore, when the PDSCH overlaps with the boundaries of downlink usable PRBs, rate matching is applied. In the legacy specification, a UE can be configured with up to 4 RateMatchPatterns per BWP and per serving cell. For instance, if a UE supports semi-static rate matching, it must support up to 4 patterns per BWP and per cell, as specified in TS 38.214. To limit UE complexity, any such operation should be counted within the existing limits for RateMatchPatterns, and may be treated as a predefined value Proposal 1. Update 60-1 basic FG to include power control, PUCCH resource, and to limit 60-1 to one DL subband. Proposal 2. Support of PDSCH reception in two non-contiguous DL subbands should be a separate FG, as in table in Annex A. Proposal 3. Support to introduce a new FG for rate matching of PDSCH reception within DL subband(s) in SBFD symbols, as in the table in Annex A. |
| [9] | CATT | Basic FG for SBFD - In last meeting, following agreements were made for basic FG for SBFD [1]. Agreement: Adopt “Per band” type for FG 60-1. Agreement: Add the following components in FG 60-1 (basic FG for SBFD operation) - Support of separate PUCCH frequency resource configuration in SBFD symbols and non-SBFD symbols - [FFS: Support of separate UL power control parameters based on unified TCI framework in SBFD symbols and non-SBFD symbols] - Support of separate PUSCH frequency hopping offset in SBFD symbols and non-SBFD symbols - [FFS: Support of separate SRS resource set configuration in SBFD symbols and non-SBFD symbols] Agreement: Update FG 60-1 as follows: - Confirm “cell-common” in component 1 - Confirm “UL transmission in one UL subband and DL reception in one [and two] DL subband in SBFD symbols” - Confirm Component 3: “3. Support of link direction determination based on configured/scheduled transmissions/receptions and collision handling” - Add “Support of separate CSI reports for SBFD and non-SBFD symbols 60-1; Basic feature for SBFD-aware UE TX/RX/measurement behaviour and procedures; 1. Support of cell-common configuration of time and frequency location of SBFD subbands 2. Support of UL transmission in one UL subband and DL reception in one [and two] DL subband in SBFD symbols 3. Support of link direction determination based on configured/scheduled transmissions/receptions and collision handling 4. Support of UL transmissions or DL receptions across SBFD symbols and non-SBFD symbols in different slots, where the transmissions or receptions are restricted to SBFD symbols or non-SBFD symbols (Configuration 1) 5. Support of separate CSI reports for SBFD and non-SBFD symbols FFS: Whether to include two DL subband support in component 2 FFS: other component (which may be separate FG); YES; n/a; Basic feature for SBFD-aware UE TX/RX/ measurement behaviour and procedures is not supported; Per band; TDD only; n/a; n/a; [Note: Only UL transmissions are allowed within UL subband and only DL receptions (except for CLI measurement) are allowed within DL subband(s)] [Note: The maximum number of UL subbands for SBFD operation in an SBFD symbol within a TDD carrier is one.]; Optional with capability signalling. - For component 2, DL reception in two DL subband in SBFD symbols should be included considering that both SBFD operation on one DL subband and on two DL subbands are typical scenario for SBFD operation. - For separate UL power control parameters based on unified TCI framework in SBFD symbols and non-SBFD symbols, separate SRS resource set configuration in SBFD symbols and non-SBFD symbols, these can be included as components in basic FG for SBFD since these are basic parameters for UL transmissions in SBFD symbols. Proposal 1: Support the following basic FG for SBFD. 60-1; Basic feature for SBFD-aware UE TX/RX/measurement behaviour and procedures; 1. Support of cell-common configuration of time and frequency location of SBFD subbands 2. Support of UL transmission in one UL subband and DL reception in one and two DL subband in SBFD symbols 3. Support of link direction determination based on configured/scheduled transmissions/receptions and collision handling 4. Support of UL transmissions or DL receptions across SBFD symbols and non-SBFD symbols in different slots, where the transmissions or receptions are restricted to SBFD symbols or non-SBFD symbols (Configuration 1) 5. Support of separate CSI reports for SBFD and non-SBFD symbols 6. Support of separate UL power control parameters based on unified TCI framework in SBFD symbols and non-SBFD symbols 7. Support of separate SRS resource set configuration in SBFD symbols and non-SBFD symbols; YES; n/a; Basic feature for SBFD-aware UE TX/RX/ measurement behaviour and procedures is not supported; Per band; TDD only; n/a; n/a; Note: Only UL transmissions are allowed within UL subband and only DL receptions (except for CLI measurement) are allowed within DL subband(s)] Note: The maximum number of UL subbands for SBFD operation in an SBFD symbol within a TDD carrier is one.; Optional with capability signalling. |
| [10] | CMCC | Firstly, besides the above components included in the basic FG for SBFD-aware UE TX/RX/measurement behaviour and procedures, the following components should also be supported: - Support of separate UL power control parameters based on unified TCI framework in SBFD symbols and non-SBFD symbols - Support of separate SRS resource set configuration in SBFD symbols and non-SBFD symbols - Support of DL reception in two DL subbands in SBFD symbols - Support of CSI-RS reception in one DL subband - Support of partial PRG(s) at DL BWP boundary Regarding the agreed FG 60-2, we prefer Alt.1: Configuration 2 for transmission and reception should be defined in a single FG. In addition, some additional UE capabilities can be defined based on the previous RAN1 agreements, including: - Support of CSI-RS reception in two non-contiguous DL subbands in SBFD symbols - Support of using the partial PRG(s) inside the DL usable PRBs at subband boundary |
| [11] | Xiaomi | For the basic FG 60-1, one of the remaining issues is whether to include two DL subband support in component 2. From our perspective, we are fine to include two DL subband support for PDSCH receptions considering the limited additional complexity compared with one DL subband. However, for CSI reception across two DL subband, it is proposed to define it as a separate FG considering the additional UE complexity. For the two notes of FG 60-1, we are supportive. Proposal 1: For FG 60-1, include two DL subband support for PDSCH in component 2. CSI-RS reception across two DL subband is defined as a separate FG. Proposal 2: For FG 60-1, agree the following two notes as in the previous agreement: - Note: Only UL transmissions are allowed within UL subband and only DL receptions (except for CLI measurement) are allowed within DL subband(s) - Note: The maximum number of UL subbands for SBFD operation in an SBFD symbol within a TDD carrier is one. Another remaining issue for FG 60-1 is whether to include support of separate UL power control parameters based on unified TCI framework in SBFD symbols and non-SBFD symbols. From our perspective, we think it should be included. Similarly, support of separate SRS resource set configuration in SBFD symbols and non-SBFD symbols should also be included since it is introduced to support separate power control for SRS in SBFD and in non-SBFD symbols. Proposal 3: For FG 60-1, include the following components: - Support of separate UL power control parameters based on unified TCI framework in SBFD symbols and non-SBFD symbols - Support of separate SRS resource set configuration in SBFD symbols and non-SBFD symbols |
| [12] | OPPO | For FG 60-1 In our view, the two subband allocations, “DU” and “DUD”, have no essential difference on UE implementations of related functionalities. Proposal 1: The component 2 of FG 60-1 contains the support of two DL subbands Because FG 60-1 already contains quite a few differentiations of functionalities/configurations between SBFD symbol and non-SBFD symbol, it makes sense to further include the differentiations between SBFD symbol and non-SBFD symbol for uplink power control and SRS configurations. Proposal 2: FG 60-1 contains the component 7 for Support of separate UL power control parameters based on unified TCI framework in SBFD symbols and non-SBFD symbols. Proposal 3: FG 60-1 contains the component 9 for Support of separate SRS resource set configuration in SBFD symbols and non-SBFD symbols. |
| [13] | Apple | Proposal 1: For the Basic feature for SBFD-aware UE, the following components shall be included - Support of DUD and/or DU - Candidate values can be {DUD, DU, both} - Additional transient time from non-SBFD symbol to SBFD symbol for DL reception - Candidate values can be {1, 2, 3} symbols in TDD SCS - A starting symbol index for SBFD symbols - Candidate values can be {0, 2, 7} Features; Index; Feature group; Components; Prerequisite feature groups; Need for the gNB to know if the feature is supported; Applicable to the capability signalling exchange between UEs (Sidelink WI only)”.; Consequence if the feature is not supported by the UE; Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC); Need of FDD/TDD differentiation; Need of FR1/FR2 differentiation; Capability interpretation for mixture of FDD/TDD and/or FR1/FR2; Note; Mandatory/Optional. 60-1; Basic feature for SBFD-aware UE TX/RX/measurement behaviour and procedures; 1. Support of cell-common configuration of time and frequency location of SBFD subbands 2. Support of UL transmission in one UL subband and DL reception in one DL subband in SBFD symbols 3. Support of link direction determination based on configured/scheduled transmissions/receptions and collision handling; YES; n/a; Basic feature for SBFD-aware UE TX/RX/ measurement behaviour and procedures is not supported; [Per Band]; TDD only; n/a; n/a; [Note: Only UL transmissions are allowed within UL subband and only DL receptions (except for CLI measurement) are allowed within DL subband(s)] [Note: The maximum number of UL subbands for SBFD operation in an SBFD symbol within a TDD carrier is one.]; Optional with capability signalling. 60. NR_duplex_evo; 60-1a; Support of SBFD ware UE in RRC connected; - Support of single shot SBFD Tx in SBFD slot - Support of single shot SBFD Rx in SBFD - Support of SBFD measurement in SBFD slot; Yes; n/a; SBFD Tx/Rx/measurement is not supported; Per band; n/a; n/a; n/a; Optional with capability signalling. 60. NR_duplex_evo; 60-1-1; Transition gap between non-SBFD symbols and SBFD symbols and vice versa; 1. number of symbols to switch from non-SBFD symbol to SBFD symbol 2. number of symbols to switch from SBFD symbol to non-SBFD symbol 3. number of symbols to switch from UL sub-band to DL sub-band 4. number of symbols to switch from DL sub-band to UL sub-band; Yes; n/a; Per band; n/a; n/a; n/a; Component 1 candidate values in number of symbols {0, 2, 7, 14, 28} symbols Component 2 candidate values in number of symbols {0, 2, 7, 14, 28} symbols; Optional with capability signalling. 60. NR_duplex_evo; 60-1-2; Number of DL sub-bands in SBFD slot; 1. number of DL sub-bands in SBFD slot; Yes; n/a; UE cannot be scheduled/configured to receive across two DL sub-bands; Per band; n/a; n/a; n/a; Component 1 candidate values are {1, 2}; Optional with capability signalling. 60. NR_duplex_evo; 60-1-3; FDRA Type 0 in SBFD slot; 1. FDRA Type 0 for PDSCH 2. FDRA Type 0 for PUSCH; 5-1; Yes; n/a; UE cannot be scheduled with Type 0 PXSCH in SBFD; Per band; n/a; n/a; n/a; Note: SBFD aware UE must support at least one FDRA type; Optional with capability signalling. 60. NR_duplex_evo; 60-1-3a; # of partial PRG in SBFD; 1. Number of partial PRG for DL reception in SBFD slot; Yes; n/a; UE cannot be scheduled more than 2 partial PRG; Per band; n/a; n/a; n/a; Candidate values are {2,3,4}; Optional with capability signalling. 60. NR_duplex_evo; 60-1-4; FDRA Type 1 in SBFD slot; 1. FDRA Type 1 for PDSCH 2. FDRA Type 1 for PUSCH; 5-1; Yes; n/a; UE cannot be scheduled with Type 1 PXSCH in SBFD; Per band; n/a; n/a; n/a; Note: SBFD aware UE must support at least one FDRA type; Optional with capability signalling. 60. NR_duplex_evo; 60-1-5; Frequency hopping in SBFD slot; 1. intra-slot frequency hopping in SBFD slot 2. Inter-slot frequency hopping in SBFD slots 3. Inter-slot frequency hopping between SBFD and non-SBFD slots; Yes; n/a; UE ignores FH flag for PUSCHs in SBFD; Per band; n/a; n/a; n/a; Optional with capability signalling. 60. NR_duplex_evo; 60-1-5a; RB-Offset for PUSCH in SBFD; 1. Applying RB-Offset for PUSCH in SBFD symbols; Yes; n/a; UE assumes RB-Offset =0; Per band; n/a; n/a; n/a; Optional with capability signalling. 60. NR_duplex_evo; 60-1-6; Separate freq. config for PUCCH-Resource; 1. Support separate configurations of startingPRB 2. Support separate configurations of secondHopPRB; Yes; n/a; No separate PUCCH-Resource config is supported; Per band; n/a; n/a; n/a; Optional with capability signalling. 60. NR_duplex_evo; 60-1-7; Separate SRS-ResourceSets; 1. Support two SRS-Resource set with usage Codebook for SBFD and non-SBFD transmissions 2. Support two SRS-Resource set with usage nonCodebook for SBFD and non-SBFD transmissions; Yes; n/a; No separate SRS-ResourceSets is supported; Per band; n/a; n/a; n/a; Note: if UE supports separate SRS-ResourceSets for Codebook, the two sets have same number of SRS resources Note: if UE supports separate SRS-ResourceSets for nonCodebook, the two sets have same number of SRS antenna ports; Optional with capability signalling. 60. NR_duplex_evo; 60-1-8; Support of Configuration 2 for DL reception; 1. Support Configuration 2 for PDSCH with repetitions 2. Support Configuration 2 for SPS PDSCH 3. Support Configuration 2 for multi-grant PDSCH 4. Support Configuration 2 for CSI-RS measurement; Yes; n/a; No support of Configuration 2 for DL reception; Per band; n/a; n/a; n/a; Optional with capability signalling. 60. NR_duplex_evo; 60-1-9; Support of Configuration 2 for UL reception; 1. Support Configuration 2 for PUSCH with repetitions Type A 2. Support Configuration 2 for PUSCH with repetitions Type B 3. Support Configuration 2 for CG-PUSCH (Type1 or 2) 4. Support Configuration 2 for multi-grant PUSCH 5. Support Configuration 2 for P/SP-PUCCH; Yes; n/a; No support of Configuration 2 for UL transmission; Per band; n/a; n/a; n/a; Optional with capability signalling. 60. NR_duplex_evo; 60-1-10; CSI-RS processing across two DL subbands; 1. CSI processing timeline and CPU counting for CSI processing across DL sub-bands; 60-2; Yes; n/a; CSI processing timeline Z/Z’ is scaled by factor of 2; Per band; n/a; n/a; n/a; Optional with capability signalling. 60. NR_duplex_evo; 60-1-11; Separate UL power control for SBFD and non-SBFD; 1. Support new P0AlphaSets in Uplink-powerControl for PUSCH 2. Support new P0AlphaSets in Uplink-powerControl for PUCCH 3. Support new P0AlphaSets in Uplink-powerControl for SRS; Yes; n/a; Same/single P0AlphaSets is applied to both SBFD and non-SBFD; Per band; n/a; n/a; n/a; Optional with capability signalling. |
| [14] | Qualcomm Incorporated | Basic SBFD-aware UE FG 60-1 A basic UE feature for supporting gNB SBFD operation should include the basic functionalities and scheduling enhancement to support efficient UL transmission and DL reception in SBFD symbols based on at least the following agreements made in the AI 9.3.1 of Rel-19 duplex enhancement (SBFD). The definition of components 1-6 and component 8 for were agreed. Regarding the number of DL subbands for component 2, it is preferred to have two candidates values {1,2} to indicate the number of supported DL subbands rather than a separate UE FG. In addition, we support to confirm component 7 to support separate power control for UL transmission in SBFD symbols and component 9 to support separate configurations of SRS resource sets for SBFD symbols. Proposal 1: Support the following for the basic SBFD-aware UE feature (FG 60-1): - Candidate values of the number of DL subbands for component 2 are {1,2} and remove the square brackets. - Remove FFS of component 7: Support separate PC parameters for UL transmission in SBFD symbols by additional PC parameters configuration within unified TCI state for SRS/PUCCH/PUSCH in SBFD symbols - Remove FFS of component 9 and add a note that separate SRS resources set only applicable for SRS usages ‘codebook ,’non codebook’ and ‘beamManagement’. 60. NR_duplex_evo; 60-1; Basic feature for SBFD-aware UE TX/RX/measurement behaviour and procedures; 1. Support of cell-common configuration of time and frequency location of SBFD subbands 2. Support of UL transmission in one UL subband and DL reception in one/two DL subband in SBFD symbols 3. Support of link direction determination based on configured/scheduled transmissions/receptions and collision handling 4. Support of UL transmissions or DL receptions across SBFD symbols and non-SBFD symbols in different slots, where the transmissions or receptions are restricted to SBFD symbols or non-SBFD symbols (Configuration 1) 5. Support of separate CSI reports for SBFD and non-SBFD symbols 6. Support of separate PUCCH frequency resource configuration in SBFD symbols and non-SBFD symbols 7. Support of separate UL power control parameters based on unified TCI framework in SBFD symbols and non-SBFD symbols 8. Support of separate PUSCH frequency hopping offset in SBFD symbols and non-SBFD symbols 9. Support of separate SRS resource set configuration in SBFD symbols and non-SBFD symbols FFS: other component (which may be separate FG); YES; n/a; Basic feature for SBFD-aware UE TX/RX/ measurement behaviour and procedures is not supported; Per band; TDD only; n/a; n/a; [Note: Only UL transmissions are allowed within UL subband and only DL receptions (except for CLI measurement) are allowed within DL subband(s)] [Note: The maximum number of UL subbands for SBFD operation in an SBFD symbol within a TDD carrier is one.] Component 2 candidate values for the number of DL subbands {1,2} Component 9 only applicable for SRS sets with usage codebook, non-codebook and beam-management.; Optional with capability signalling. |
| [15] | NTT DOCOMO, INC. | Separate UL configurations for SBFD symbols and non-SBFD symbols Considering different CLI level on SBFD and non-SBFD symbols, appropriate UL power control parameters for SBFD and non-SBFD respectively are essential for practical SBFD operation. The support of separate UL power control parameters in SBFD symbols and non-SBFD symbols should be part of the basic FG for SBFD. However, since separate UL power control parameters in SBFD symbols and non-SBFD symbols are specified only for unified TCI framework, separate UL power control parameters in SBFD symbols and non-SBFD symbols are supported if UE reports support of both FG 23-1-1 (unified TCI framework) and FG 60-1. For support of separate SRS resource sets in SBFD symbols and non-SBFD symbols, the main controversial point may be whether support of multiple SRS resource sets is essential for SBFD operation. At the RAN1#119 meeting, it was agreed that only configuration 1 is applicable for SRS, i.e. SRS transmissions occasions of an SRS resource set is restricted in a specific symbol type (SBFD symbols only or non-SBFD symbols only). Moreover, it was made as working assumption that for codebook based PUSCH transmissions, the precoding information is identified by the SRI in the SRS resource set corresponding to same symbol type of the PUSCH transmission. If only one SRS resource set with usage set to ‘codebook’ is configured, e.g. in non-SBFD symbols, the precoding information for PUSCH in SBFD symbols can’t be indicated by this SRS resource set. Therefore, we think separate SRS resource sets in SBFD symbols and non-SBFD symbols should be essential for SBFD operation, and it should be included as part of FG 60-1. Proposal 1: For support of separate UL configurations for SBFD and non-SBFD symbols: - Separate UL power control parameters in SBFD symbols and non-SBFD symbols are supported if UE reports support of both FG 23-1-1 (unified TCI framework) and FG 60-1. - “Support of separate SRS resource set configuration in SBFD symbols and non-SBFD symbols” should be part of FG 60-1. |
| [16] | Ericsson | One of the outstanding questions for FG 60-1 is whether reception in both of the configured DL subbands should be included in the basic feature group. In our view it should, since otherwise it would imply a restriction in relation to legacy operation and the network’s ability to schedule said UE over its whole BWP. RAN1 made the following agreement in RAN1 #118bis [4]: Agreement … DL usable PRBs are determined as intersection between cell-specific DL subband(s) and the DL BWP in SBFD symbols.. RAN1 has neither discussed, nor agreed on any restrictions with respect to the above agreement. It is worth noting that a lower capability UE, e.g., a RedCap UE will not be configured with a BWP spanning both DL subbands, and for that reason would anyway not be affected by this requirement. Include support for DL reception in two non-contiguous DL subbands in component 2 of basic FG 60-1. Separate UL configurations, including power control parameters based on unified TCI framework and SRS resource set configurations, etc., for SBFD and non-SBFD symbols to be part of basic FG 60-1 for an SBFD-aware UE as these are essential components of SBFD operation. Similarly, it is also straightforward for an SBFD-aware UE to generate separate CSI reports for SBFD symbols and non-SBFD symbols based on the CSI report configurations set by the gNB and by leveraging the corresponding CSI-RS receptions. Moreover, RAN1 also reached consensus in the earlier meeting that configuration 1 with transmission or reception on one specific symbol type set in DL-BWP is not applicable to CSI-RS. Altogether, these components shall be part of basic FG 60-1 to avoid further FG fragmentation. - Include support for separate CSI reports for SBFD and non-SBFD symbols, including the determination of subbands for CSI reporting, in basic FG 60-1. Include support for separate UL power control parameters based on unified TCI framework for SBFD and non-SBFD symbols in basic FG 60-1. Include support for separate SRS resource set configuration in SBFD and non-SBFD symbols in basic FG 60-1. Depending on the configuration of DL and UL subbands, PDSCH reception can be subject to partial PRGs. To have better resource efficiency, it is essential to have the support of DL Partial PRG operation as part of basic FG 60-1. And the UE can report about the support of maximum number of DL partial PRGs. - Include support for DL partial PRG in basic FG 60-1. We recall that configuration 1 is default capability of the Rel-19 SBFD-aware UE, which is expected to transmit/receive in SBFD symbols if the gNB configures valid symbol type is SBFD. In that case, it is necessary for the UE to determine the number of PRBs for transport block size based on assigned PRBs within usable PRBs only, by leveraging the FDRA and SBFD configuration. - Include support for determination of transport block size of PDSCH/PUSCH subject to valid symbol type, FDRA in DCI and SBFD configuration, in basic FG 60-1. In RAN1 #120, Configuration 1 and Configuration 2 were introduced as UE-features with components and later modified in RAN1 #120bis, as shown in Table 1. In our view, it is a poor choice of naming to use the same component naming for opposing features, as is shown by the magenta highlighting. The same applies to the column describing the consequences for not supporting the feature. The only way for a reader to be able to differentiate between Component 1 of FG X-1a and FG X-2, respectively, as well as the consequences for not supporting it, is to disregard the main clause of the sentence and focus on the following subordinate clause for Configuration 1, alternatively resort to a definition of ‘Configuration 1’ and ‘Configuration 2’ which are names that may not even be included in the specifications. Table 1: FGs and components for Configuration 1 and Configuration 2, respectively. Index; Feature group; Components; Consequence if the feature is not supported by the UE. 60-1; Basic feature for SBFD-aware UE TX/RX/measurement behaviour and procedures; 4. Support of UL transmissions or DL receptions across SBFD symbols and non-SBFD symbols in different slots, where the transmissions or receptions are restricted to SBFD symbols or non-SBFD symbols (Configuration 1). 60-2; [Transmission and reception] across SBFD symbols and non-SBFD symbols in different slots (Configuration 2); 1. Support of PDSCH reception across SBFD symbols and non-SBFD symbols (Configuration 2) [2. Support of UL transmission across SBFD symbols and non-SBFD symbols (Configuration 2)]; [Transmission and reception] across SBFD symbols and non-SBFD symbols in different slots (Configuration 2) are not supported. Instead, we propose to reformulate the names and descriptions such that they are unmistakenly unique, as presented in Table 2. Here, there is a clear distinction between supporting multi-slot transmissions and receptions in either SBFD or non-SBFD symbols in different slots and supporting multi-slot transmissions and receptions in both SBFD and non-SBFD symbols in different slots. Table 2: Proposed component naming and descriptions for FGs X-1 and X-2. Index; Feature group; Components; Consequence if the feature is not supported by the UE. 60-1; Basic feature for SBFD-aware UE TX/RX/measurement behaviour and procedures; [4. Support of UL transmissions or DL receptions in either SBFD symbols or non-SBFD symbols in different slots. 60-2; [Transmission and reception] across SBFD symbols and non-SBFD symbols in different slots (Configuration 2); [1. Support of PDSCH receptions in both SBFD symbols and non-SBFD symbols in different slots [2. Support of UL transmissions in both SBFD symbols and non-SBFD symbols in different slots]; Transmission and reception in both SBFD symbols and non-SBFD symbols in different slots is not supported. Using the same component names or descriptions for contradictory capabilities in different FGs opens for ambiguous interpretations and should be avoided. For FG 60-1, change Component name to ‘4. Support of UL transmissions or DL receptions in either SBFD symbols or non-SBFD symbols in different slots’. For FG 60-2, change Component names to ‘1. Support of PDSCH reception in both SBFD symbols and non-SBFD symbols in different slots’ and ‘2. Support of UL transmissions in both SBFD symbols and non-SBFD symbols in different slots’. For FG 60-2, change consequence description to ‘Transmission and reception in both SBFD symbols and non-SBFD symbols in different slots is not supported’. |

The following shows the latest status for the relevant FG(s).

| 60-1 | Basic feature for SBFD-aware UE TX/RX/measurement behaviour and procedures | 1. Support of cell-common configuration of time and frequency location of SBFD subbands 2. Support of UL transmission in one UL subband and DL reception in one[/two] DL subband in SBFD symbols 3. Support of link direction determination based on configured/scheduled transmissions/receptions and collision handling 4. Support of UL transmissions or DL receptions across SBFD symbols and non-SBFD symbols in different slots, where the transmissions or receptions are restricted to SBFD symbols or non-SBFD symbols (Configuration 1) 5. Support of separate CSI reports for SBFD and non-SBFD symbols 6. Support of separate PUCCH frequency resource configuration in SBFD symbols and non-SBFD symbols [FFS: 7. Support of separate UL power control parameters based on unified TCI framework in SBFD symbols and non-SBFD symbols] 8. Support of separate PUSCH frequency hopping offset in SBFD symbols and non-SBFD symbols [FFS: 9. Support of separate SRS resource set configuration in SBFD symbols and non-SBFD symbols] FFS: Whether to include two DL subband support in component 2 FFS: other component (which may be separate FG) |  | YES | n/a | Basic feature for SBFD-aware UE TX/RX/ measurement behaviour and procedures is not supported | Per band | TDD only | n/a | n/a | [Note: Only UL transmissions are allowed within UL subband and only DL receptions (except for CLI measurement) are allowed within DL subband(s)] [Note: The maximum number of UL subbands for SBFD operation in an SBFD symbol within a TDD carrier is one.] | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Summary of companies’ views and discussion

- Support of DL reception in two DL subbands in SBFD symbols as a component in basic FG for SBFD: ZTE, vivo (subject to CSI processing timeline), CATT, OPPO, Ericsson, DOCOMO (at least for PDSCH) - Two separate FGs for PDSCH reception in two DL subbands and CSI-RS reception in two DL subbands: SPRD/UNISOC, Apple, QC, - One separate FG for DL reception (including PDSCH and CSI-RS) in two non-contiguous DL subbands:HW/Hisi, Samsung, Ofinno, - One separate FG for CSI-RS in two non-contiguous DL subbands: MediaTek, CMCC, xiaomi, Qualcomm (CPU components), - One separate FG for CSI-RS in two non-contiguous DL subbands without processing timeline relaxation: vivo, CATT, Apple - CSI-RS in two DL subbands using relaxed CSI processing timeline Z/Z’ by factor of 2 as a component in basic FG for SBFD: vivo, CATT - CSI-RS in two DL subbands using relaxed CSI processing timeline Z/Z’ by factor of 2 is included in the FG for DL reception in two non-contiguous DL subbands: Apple - DL partial PRG (or supported max number of DL partial PRGs) as a component in basic FG for SBFD: - Separate FG: HW/Hisi, SPRD/UNISOC, CATT (for more than 2 partial PRGs), Offino, MediaTek, CMCC, Apple, - Maximum number of supported partial PRGs is a component of FG for support of DL reception in two non-contiguous DL subbands: ZTE, vivo, DOCOMO, E///, - Support of separate UL power control parameters based on unified TCI framework in SBFD symbols and non-SBFD symbols as a component in basic FG for SBFD: HW/Hisi, ZTE, Nokia, MTK, Samsung, Ofinno, CATT, CMCC, Xiaomi, OPPO, QC, DCM, E///, - Separate FG: SPRD/UNISOC, Apple, - Support of separate SRS resource set configuration in SBFD symbols and non-SBFD symbols as a component in basic FG for SBFD: HW/Hisi, ZTE, MTK, Samsung, CATT, CMCC, Xiaomi, OPPO, QC (except for SRS resource set for usage antennaSwitching), DCM, E///, - Separate FG: Apple, - Other components - Unaligned boundaries between SBFD subband(s) and RBG, CSI reporting subband, CSI-RS resource, PRG: ZTE - Counting of non-continuous CSI-RS: Samsung, - Support of partial PRG(s) at DL BWP boundary: CMCC - Support for determination of transport block size of PDSCH/PUSCH subject to valid symbol type, FDRA in DCI and SBFD configuration: Ericsson - Other separate FGs - Separate FG for transition gap between non-SBFD symbols and SBFD symbols and vice versa: Apple - Separate FG for support of FDRA Type 0 in SBFD slot: Apple - Separate FG for support of FDRA Type 1 in SBFD slot: Apple - Separate FG for maximum number of DL-UL switching in SBFD slot, and SBFD/TDD period: Qualcomm - Separate FG for the maximum number of DL/UL switching for the SBFD-aware UE within SBFD slots and the SBFD time period: Qualcomm - Note column: - Remove: Nokia, - Confirm: xiaomi - Others - change Component name to ‘4. Support of UL transmissions or DL receptions in either SBFD symbols or non-SBFD symbols in different slots’.: E///

Proposal 2-1-1:

For FG 60-1, regarding FFS for support of separate UL power control parameters based on unified TCI framework in SBFD symbols and non-SBFD symbols,

- Confirm the FFS as it is:

  - “[FFS: 7. Support of separate UL power control parameters based on unified TCI framework in SBFD symbols and non-SBFD symbols]”

- Add the following in Note column

  - “Component 7 applies only to UEs supporting FG 23-1-1”

| Company | Comment |
| --- | --- |
| ZTE | Support |
| DOCOMO | Support the proposal. Separate UL power control for SBFD symbols and non-SBFD symbols is essential for SBFD operation. |
| Ericsson | OK |
| Huawei, HiSilicon | Support |
| Samsung | Support |
| QC | Support |

Proposal 2-1-2:

For FG 60-1, regarding FFS for support of separate SRS resource set configuration in SBFD symbols and non-SBFD symbols,

- Confirm the FFS as it is

  - “[FFS: 9. Support of separate SRS resource set configuration in SBFD symbols and non-SBFD symbols]”

- Add the following note in Note column

  - “Component 9 is only applicable for SRS resource sets with usage codebook, nonCodebook and beamManagement.”

| Company | Comment |
| --- | --- |
| ZTE | Support |
| DOCOMO | Support the proposal. Separate SRS resource set configuration for SBFD and non-SBFD is essential for SBFD operation. |
| Ericsson | OK |
| Huawei, HiSilicon | Support |
| Samsung | Support |
| QC | Support |

Question 2-1-3:

Companies are encouraged to provide their views on whether the following two notes are needed or not.

- [Note: Only UL transmissions are allowed within UL subband and only DL receptions (except for CLI measurement) are allowed within DL subband(s)]

- [Note: The maximum number of UL subbands for SBFD operation in an SBFD symbol within a TDD carrier is one.]

| Company | Comment |
| --- | --- |
| ZTE | Prefer to add the two notes, especially the first one. |
| DOCOMO | Support to confirm the two notes. |
| Ericsson | Not needed, can be captured elsewhere. |
| Huawei, HiSilicon | No strong views but both notes have been captured in the other specs already. |
| Samsung | Prefer to keep both. |
| QC | Notes provide good clarification, good to keep. |

2.2	FG for support of DL reception in two non-contiguous DL subbands

Following inputs are provided in contributions for the RAN1#121 meeting.

| [1] | Huawei, HiSilicon | DL reception in two non-contiguous DL subbands As discussed in section 2.1, an SBFD-aware UE can optionally support DL reception across two non-contiguous DL subbands. This has an impact on PDSCH reception and CSI processing timeline. This FG includes the following components - CSI processing timeline in SBFD symbols for the support of one contiguous CSI-RS resource allocation across two DL subbands - Maximum number of supported partial PRGs in SBFD symbols for the support of PDSCH reception across two DL subbands For component 1, UE can indicate whether CSI processing timeline is the same as legacy or the CSI processing timeline Z/Z’ is scaled by factor of 2. Proposal 2: The support of UL transmission in one UL subband and DL reception in two non-contiguous DL subbands is defined as a separate FG including the following components - CSI processing timeline in SBFD symbols for the support of one contiguous CSI-RS resource allocation across two DL subbands - Maximum number of supported partial PRGs in SBFD symbols for the support of PDSCH reception across two DL subbands |
| --- | --- | --- |
| [2] | ZTE Corporation, Sanechips | n/a |
| [3] | vivo | - SBFD TX/RX/measurement procedures - Following questions on UE features for SBFD TX/RX/measurement procedures were discussed. - Whether ‘support of PDSCH receptions in two non-contiguous DL subbands’ should be part of basic FG for SBFD operation or separate FG - Whether ‘support of CSI-RS receptions in two non-contiguous DL subbands’ should be part of basic FG for SBFD operation or separate FG - Whether supported maximum number of DL partial PRGs for PDSCH in SBFD symbols should be reported as part of basic FG for SBFD operation or part of FG for PDSCH reception in two non-contiguous DL subbands - Regarding the questions, we have the following proposals. Proposal 1: Support of PDSCH receptions in two non-contiguous DL subbands’ should be part of basic FG for SBFD operation. Proposal 2: For CSI-RS receptions in two non-contiguous DL subbands, following alternatives can be considered. - Alt.1: Support of CSI-RS reception in two non-contiguous DL subbands should be part of basic FG for SBFD operation. - Supported CSI processing timeline for CSI-RS reception in two non-contiguous DL subbands should be reported as a component of the basic FG for SBFD. - Two candidate values may be reported: {CSI processing timeline and CPU counting are the same as legacy, scale the CSI processing timeline Z/Z’ by factor of 2 and keep the same CPU counting as legacy}. - Alt.3: Support of CSI-RS reception in two non-contiguous DL subbands with scaled CSI processing timeline Z/Z’ by factor of 2 should be part of the basic FG for SBFD operation. Support of CSI-RS reception in two non-contiguous DL subbands with legacy CSI processing timeline Z/Z’ should be a separate FG. |
| [4] | Nokia |  |
| [5] | MediaTek Inc. | Support of DL reception in two non-contiguous DL subbands There was a discussion on whether on whether ‘Support of DL reception in two non-contiguous DL subbands’ should be part of basic FG for SBFD or separate FG(s) (including whether separate FGs e.g., for PDSCH and CSI-RS are necessary). In our view, we can have separate discussion for PDSCH and CSI-RS. Support for non-contiguous CSI-RS across two DL subbands can be considered as a separate FG, which includes support for support of scaling the CSI processing timeline Z/Z’ by factor of 2. This is because the UE can rely on contiguous CSI-RS within a one DL subband for CSI measurement. Support for non-contiguous PDSCH, on the other hand, can be considered as part of the basic FG since existing FDRA type 0 can be used to achieve non-contiguous PDSCH allocation. - Support of non-contiguous PDSCH across two DL subbands should be part of basic FG. - Support of non-contiguous CSI-RS across two DL subbands should be a separate FG. |
| [6] | Spreadtrum, UNISOC | FGs for DL reception in two non-contiguous DL subbands For DL reception in two non-contiguous DL subbands, it should be multiple separate FGs for PDSCH reception in two non-contiguous DL subbands and for CSI-RS reception in two non-contiguous DL subbands. RA Type 0 for PDSCH is included in FG 5-1(Basic scheduling/HARQ operation), which is mandatory UE feature. So some companies think PDSCH reception in two non-contiguous DL subbands should be part of basic FG. However, for RA Type 1 for PDSCH, only the assigned PRBs within DL usable PRBs are considered to be valid for PDSCH. And the number of PRBs for TBS determination is based on the assigned PRBs within DL usable PRBs only. So the resource mapping and TBS determination is different with legacy. In SBFD operation, only CSI-RS frequency resources within DL usable PRBs are valid for SBFD-aware UE. Non-contiguous CSI-RS resource is derived by excluding frequency resources outside DL usable PRBs, which is different legacy UE. Thus, we prefer separate FG (60-1a) for DL reception in two non-contiguous DL subbands. PDSCH reception in two non-contiguous DL subbands and CSI-RS reception in two non-contiguous DL subbands can be defined in one FG. For CSI processing timeline in SBFD symbols to process the CSI-RS across two DL subbands, there are two candidate value . One value is CSI processing timeline and CPU counting are the same as legacy, and the other value is scale the CSI processing timeline Z/Z’ by factor of 2 and keep the same CPU counting as legacy. In last meeting, some companies argued that gNB cannot know whether UE support one/two DL subband and gNB cannot do appropriate DUD/DU/UD configuration. From our perspective, the configuration of subband location is independent with capability of DL reception. As long as the basic SBFD FG is supported, any subband configuration can be supported. The location of the UL subband will depends on the SBFD carrier location within the NR band and the number of adjacent carriers. For example, if SBFD carrier has two adjacent carriers, gNB would configure the UL subband in the middle of carrier (DUD configuration) to reduce inter-gNB interference. After random access is completed, gNB can schedule DL resource in one DL subband or across two DL subband based on capability of DL reception. Thus, support of DL reception in two non-contiguous DL subbands should be separate FG. |
| [7] | Samsung | For two DL subbands, our views are - Define separate FG to support DL reception across two DL subbands in SBFD symbols. - PDSCH reception across two DL subbands in SBFD symbols and CSI-RS reception across two DL subbands in SBFD symbols can be defined in one FG. Proposal 3. Support the following FG for SBFD to support DL reception across two DL subbands in SBFD symbol 60-A; [Support of DL reception in two non-contiguous DL subbands]; 1. Support of DL reception in two non-contiguous DL subbands 2. Support of CSI-RS reception in two non-contiguous DL subbands 3. CSI processing timeline in SBFD symbols for the support of one contiguous CSI-RS resource allocation across two DL subbands FFS: whether each of components within brackets is separate FG or within a combined FG FFS: other component (which may be separate FG); 60-1; Yes; DL reception in two non-contiguous DL subbands is not supported; Per band; [For component 3, candidate values are {Legacy, Scaled}] [‘Legacy’ means the CSI processing timeline and CPU counting are the same as legacy. ‘Scaled’ means the CSI processing timeline Z/Z’ by factor of 2 and keep the same CPU counting as legacy.]; Optional with capability signalling. How to count of non-contiguous CSI-RS towards the maximum number of active CSI-RS and CSI-RS port per CC and across all CCs should be reflected in UE capability. Similar to the CSI processing timeline, the non-contiguous CSI-RS may be counted as 1 (legacy) or 2 (scaled) towards the maximum number of active CSI-RS and CSI-RS port per CC and across all CCs. That is, some UEs may need to relax the counting of non-contiguous CSI-RS towards the limit of the maximum number of active CSI-RS and CSI-RS port. Proposal 4. Support the following FG for SBFD to take into account counting of non-contiguous CSI-RS across two DL subbands in SBFD symbol 60-B; Counting of non-continuous CSI-RS; 1. Counting of non-contiguous CSI-RS towards the maximum number of active CSI-RS and CSI-RS port per CC and across all CCs (maxNumberSimultaneousNZP-CSI-RS-PerCC, totalNumberPortsSimultaneousNZP-CSI-RS-PerCC, maxNumberSimultaneousNZP-CSI-RS-ActBWP-AllCC totalNumberPortsSimultaneousNZP-CSI-RS-ActBWP-AllCC) is scaled by R; X-3; Yes; Non-continuous CSI-RS across two DL subbands cannot be configured; Per band; Candidate value for R in component 1 is {1, 2}; Optional with capability signalling. |
| [8] | Ofinno |  |
| [9] | CATT | FG for CSI processing timeline in SBFD symbols For CSI processing timeline, the following was agreed in RAN1. Agreement For CSI processing timeline in SBFD symbols to process the CSI-RS across two DL subbands, - Option 3: It is up to UE capability to indicate one of the following: - CSI processing timeline and CPU counting are the same as legacy. - Scale the CSI processing timeline Z/Z’ by factor of 2 and keep the same CPU counting as legacy. Based on the agreement, UE should report whether UE supports legacy CSI processing timeline or scaled CSI processing timeline. A separate FG for CSI-RS resource across two DL subband using legacy CSI processing timeline and CPU counting is introduced. If UE does not report this capability, the scaled CSI processing timeline is used. Proposal 3: Support the following FG for SBFD operation configuration 2. 60-1b; CSI processing timeline to process CSI-RS across two DL subbands in SBFD symbols without relaxation; CSI processing timeline and CPU counting in SBFD symbols to process the CSI-RS across two DL subbands are the same as legacy; 60-1; YES; n/a; For CSI processing timeline in SBFD symbols to process the CSI-RS across two DL subbands, scale the legacy CSI processing timeline Z/Z’ by factor of 2 and keep the same CPU counting as legacy.; Per Band; TDD only; n/a; n/a; Optional with capability signalling. |
| [10] | CMCC |  |
| [11] | Xiaomi |  |
| [12] | OPPO |  |
| [13] | Apple |  |
| [14] | Qualcomm Incorporated | UE FG for non-contiguous CSI-RS across DL subbands The support of non-contiguous CSI-RS across two downlink subbands and corresponding CSI processing timeline should be a separate UE FG, based on the following RAN1. Working Assumption For frequency resource allocation for CSI-RS across downlink subbands for SBFD-aware UEs, support one contiguous CSI-RS resource allocation with non-contiguous CSI-RS resource derived by excluding frequency resources outside DL usable PRBs. - No impact on CSI-RS sequence generation - CSI-RS sequence mapping is applied to CSI-RS resources within DL usable PRBs only (effectively, this is same as the case when the CSI-RS sequence mapped to the RBs outside the DL usable PRBs are punctured) - RAN1 to further study the impact on CSI processing timeline in SBFD symbols to process the CSI-RS across the two DL subbands Agreement For CSI processing timeline in SBFD symbols to process the CSI-RS across two DL subbands, - Option 3: It is up to UE capability to indicate one of the following: - CSI processing timeline and CPU counting are the same as legacy. - Scale the CSI processing timeline Z/Z’ by factor of 2 and keep the same CPU counting as legacy.. Proposal 3: Support a UE feature (FG 60-2b) for supporting non-contiguous CSI-RS resource across two DL subband with one of the following capabilities: - Cap1: using legacy CSI processing timeline and CPU counting are the same as legacy - Cap2: using relaxed CSI processing timeline Z/Z’ by factor of 2 and same CPU counting as legacy. 60. NR_duplex_evo; 60-2b; Non-contiguous CSI-RS in SBFD symbols; Support of non-contiguous CSI-RS resource across two DL subband with one of the following capabilities: Cap1: using legacy CSI processing timeline and CPU counting are the same as legacy Cap2: using relaxed CSI processing timeline Z/Z’ by factor of 2 and same CPU counting as legacy.; YES; n/a; Non-contiguous CSI-RS in SBFD symbols is not supported; Per Band; TDD only; n/a; n/a; Optional with capability signalling. |
| [15] | NTT DOCOMO, INC. | DL reception in two non-contiguous DL subbands and DL partial PRG At the RAN1#120bis meeting, it was discussed whether support of DL reception in two DL subbands should be part of FG 60-1 or a separate FG. We think it should be part of the basic FG at least for PDSCH reception in two non-contiguous DL subbands, since non-contiguous FDRA for PDSCH is basic function of NR. As agreed at previous meetings, UE shall report the capability of the maximum number of supported partial PRGs in SBFD symbols. Assuming PDSCH reception across two non-contiguous DL subbands is part of the basic FG as discussed above, maximum number of supported partial PRGs in SBFD symbols should be reported as part of basic FG. The candidate values should be {2,4}. Proposal 2: “Support of DL reception in two non-contiguous DL subbands” should be part of FG 60-1 at least for PDSCH. - Maximum number of supported partial PRGs in SBFD symbols should be reported as part of FG 60-1. - Candidate values: {2, 4} For CSI-RS reception in two non-contiguous DL subbands, as it is new case, whether it should be part of basic FG or separate FG can be further discussed. To achieve efficient CSI-RS resource configuration/allocation in SBFD operation (i.e., for all SBFD capable UEs), it is preferred to support it as a part of basic FG. Moreover, UE should also report whether UE supports legacy CSI processing timeline for one contiguous CSI-RS resource allocation across two DL subbands or scaled CSI processing timeline for it. Proposal 3: “Support of CSI-RS reception in two non-contiguous DL subbands” should be part of FG 60-1. - Whether UE supports legacy CSI processing timeline or scaled CSI processing timeline for one contiguous CSI-RS resource allocation across two DL subbands should be reported. |
| [16] | Ericsson | One of the outstanding questions for FG 60-1 is whether reception in both of the configured DL subbands should be included in the basic feature group. In our view it should, since otherwise it would imply a restriction in relation to legacy operation and the network’s ability to schedule said UE over its whole BWP. RAN1 made the following agreement in RAN1 #118bis [4]: Agreement … DL usable PRBs are determined as intersection between cell-specific DL subband(s) and the DL BWP in SBFD symbols.. RAN1 has neither discussed, nor agreed on any restrictions with respect to the above agreement. It is worth noting that a lower capability UE, e.g., a RedCap UE will not be configured with a BWP spanning both DL subbands, and for that reason would anyway not be affected by this requirement. - Include support for DL reception in two non-contiguous DL subbands in component 2 of basic FG 60-1. |

Summary of companies’ views and discussion

Whether the support of DL receptions in two non-contiguous DL subbands as separate FG? - Support of DL reception in two DL subbands in SBFD symbols as a component in basic FG for SBFD: ZTE, vivo (subject to CSI processing timeline), CATT, OPPO, Ericsson, DOCOMO (at least for PDSCH) - Two separate FGs for PDSCH reception in two DL subbands and CSI-RS reception in two DL subbands: SPRD/UNISOC, Apple, QC, - One separate FG for DL reception (including PDSCH and CSI-RS) in two non-contiguous DL subbands: HW/Hisi, Samsung, Ofinno, - One separate FG for CSI-RS in two non-contiguous DL subbands: MediaTek, CMCC, xiaomi, Qualcomm (CPU components), Processing timeline for CSI-RS in two non-contiguous DL subbands - Two candidate values in the separate FG for CSI-RS reception in two contiguous DL subbands: SPRD/UNISOC, ZTE, MediaTek, HW/Hisi, Xiaomi, Qualcomm - Two candidate values in the component of FG 60-1: DOCOMO - One separate FG for CSI-RS in two non-contiguous DL subbands without processing timeline relaxation: vivo, CATT, Apple - CSI-RS in two DL subbands using relaxed CSI processing timeline Z/Z’ by factor of 2 as a component in basic FG for SBFD: vivo, CATT - CSI-RS in two DL subbands using relaxed CSI processing timeline Z/Z’ by factor of 2 is included in the FG for DL reception in two non-contiguous DL subbands: Apple Type of the FG - Per band: vivo, SPRD/UNISOC, [ZTE], CATT, CMCC, Samsung, Xiaomi, Apple Prerequisite of the FG - X-1 (basic FG): SPRD/UNISOC, ZTE, CATT, CMCC, HW/Hisi, Samsung, Xiaomi - 2-1: SPRD/UNISOC Other separate FG related wtih DL reception in two non-contiguous DL subbands - Counting of non-continuous CSI-RS: Samsung - Rate matching of PDSCH reception within DL subband(s): Samsung

Proposal 2-2-1:

For FG 60-1, confirm the FFS in component 2 as it is as follows (i.e., support of DL reception in two DL subbands in SBFD symbols is included in FG 60-1 component 2)

| Company | Comment |
| --- | --- |
| ZTE | Support. If this feature is optional, it implies either gNB shall ensure all DL receptions located within one DL subbands or gNB can only configure one DL subband for all UEs. Both are too restrictive. |
| DOCOMO | We support the proposals. |
| Ericsson | Support. |
| Huawei, HiSilicon | We had a preference to separate component 2 out an optional UE capability since it requires a distinct implementation for both PDSCH and CSI measurement. |
| Samsung | We support separate FG for reception across two DL subbands in SBFD symbols. |
| QC | We support to have candidate component values “1,2” for the number of supported DL subband. |

#### Question 2-2-2:

Companies are encouraged to share the views on how to define “support of CSI-RS reception in two non-contiguous DL subbands”:

- Alt-1: Add a new component in FG 60-1 for “Support of CSI-RS reception in two non-contiguous DL subbands, and supported CSI processing timeline”

  - Candidate values: {“with factor-of-2 relaxation”, “without factor-of-2 relaxation}”

- Alt-2: Introduce a new FG for “support of CSI-RS reception in two non-contiguous DL subbands, and supported CSI processing timeline”

  - Candidate values: {“with factor-of-2 relaxation”, “without factor-of-2 relaxation}”

  - Prerequisite FG: 60-1

| 60-1a | CSI-RS reception in two non-contiguous DL subbands | 1. Support of CSI-RS reception in two non-contiguous DL subbands - Candidate values: {“with factor-of-2 relaxation”, “without factor-of-2 relaxation} | 60-1 | YES | n/a | CSI-RS reception in two non-contiguous DL subbands is not supported | Per band | TDD only | n/a | n/a |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comment |
| --- | --- |
| ZTE | Support Alt-1. Support of CSI-RS reception in two non-contiguous DL subbands should be a basic feature for SBFD. |
| DOCOMO | We prefer Alt.1. If CSI-RS reception in two non-contiguous DL subbands should be separate FG, it will not be used in real field at all, as anyway CSI-RS reception in each DL subband is possible. |
| Ericsson | We support Alt-1. |
| Huawei, HiSilicon | Alt-2. See our reply to proposal 2-2-1. |
| Samsung | Prefer Alt-2. |
| QC | Support Alt 2 |

2.3	FG for support of partial PRG

Following inputs are provided in contributions for the RAN1#121 meeting.

| [1] | Huawei, HiSilicon |  |
| --- | --- | --- |
| [2] | ZTE Corporation, Sanechips | - Supported maximum number of DL partial PRGs for PDSCH in SBFD symbols should be reported as part of basic FG for SBFD operation - Candidate values for supported maximum number of DL partial PRGs is {2, 4}. If two DL subbands are configured and value 2 is reported by the UE, a PDSCH transmission is valid as long as it only contains up to 2 partial PRGs, no matter the partial PRG are caused by the boundary of BWP as legacy or it is caused by the boundary of DL subband. Otherwise, a PDSCH transmission is valid no matter which and how many partial PRGs the PDSCH contains. |
| [3] | vivo |  |
| [4] | Nokia |  |
| [5] | MediaTek Inc. |  |
| [6] | Spreadtrum, UNISOC | FG for DL partial PRG Support of DL partial PRG should be separate FG (60-1b), which is aligned with RAN1 agreement. The candidate value can be 2 and 4. The consequence of not reporting this UE FG is partial PRGs in the subband boundary is not supported. |
| [7] | Samsung | Additionally, PDSCH will be rate matched when it crosses the boundaries of DL usable PRBs. In legacy, a UE may be configured with up to 4 RateMatchPattern(s) per BWP and up to 4 RateMatchPattern(s) per serving cell. For example, once a UE supports semi-static rate matching, this UE has to support up to 4 rate matching patters per BWP and per cell where these values are predefined in 38.214. To avoid increasing the UE complexity, such operation should be counted towards the legacy limits on RateMatchPattern(s) per BWP or per cell. It may be counted as a predefined value, e.g., one RateMatchPattern, or it may be counted as β RateMatchPattern(s) where the value of β is indicated via UE capability signaling. Proposal 5. Support the following FG for SBFD to take into account rate-matching of PDSCH reception within DL subband(s) in SBFD symbol 60-C; Rate matching of PDSCH reception within DL subband(s); 1. Counting of PDSCH rate matching within DL subband(s) toward the maximum number of cell-specific RateMatchingPattern(s); X-1; Yes; PDSCH cannot be rate-matched within DL subband(s) when 4 cell-specific RateMatchingPattern(s) are configured; Per band; The maximum number of RateMatchPattern(s) that can be configured per BWP or per cell is 3, when rate matching of PDSCH occurs because of cross the boundaries of DL usable PRBs.; Optional with capability signalling. |
| [8] | Ofinno |  |
| [9] | CATT | FG for support of partial PRG - For DL partial PRG, it was agreed that UE capability reports the maximum number of supported partial PRGs. In legacy NR, the maximum two partial PRG were used. Hence, more than 2 partial PRG can be supported based on separate UE capability. The candidate value can be 3 and 4. For UEs without this capability, the maximum number of partial PRGs supported is 2. Proposal 2: Support the following FG for partial PRG. 60-1a; DL partial PRG in SBFD symbols; Maximum number of supported partial PRGs for PDSCH in SBFD symbols; 60-1; YES; n/a; UE does not support more than 2 partial PRGs for PDSCH in SBFD symbols; Per Band; TDD only; n/a; n/a; Candidate values: {3, 4} UE does not expect to be scheduled with a PDSCH in SBFD symbols with number of partial PRGs to be more than the maximum number of supported partial PRGs in SBFD symbols.; Optional with capability signalling. |
| [10] | CMCC |  |
| [11] | Xiaomi |  |
| [12] | OPPO |  |
| [13] | Apple |  |
| [14] | Qualcomm Incorporated |  |
| [15] | NTT DOCOMO, INC. |  |
| [16] | Ericsson |  |

Summary of companies’ views and discussion

Whether the support of DL partial PRG is reported as separate FG, as component of basic FG? - DL partial PRG (or supported max number of DL partial PRGs) as a component in basic FG for SBFD: - Separate FG: HW/Hisi, SPRD/UNISOC, CATT (for more than 2 partial PRGs), Offino, MediaTek, CMCC, Apple, - Maximum number of supported partial PRGs is a component of FG for support of DL reception in two non-contiguous DL subbands: ZTE, vivo, DOCOMO, E///, Candidate values for maximum number of partial PRGs - {2, 4}: SPRD/UNISOC, ZTE, DCM, - {2, 3, 4}: vivo, Apple, CATT (2 is basic) Type of the FG - Per band: vivo, SPRD/UNISOC, [ZTE], CATT, MediaTek, CMCC, Apple Prerequisite of the basic FG - X-1 (basic FG): CATT, MediaTekss, CMCC - FG X-1a for PDSCH reception in two non-contiguous DL subbands: SPRD/UNISOC, Offino Other potential FGs: - PDSCH rate matching: Samsung,

Proposal 2-3-1:

Add a new component in FG 60-1 for “Supported maximum number of DL partial PRGs for PDSCH in SBFD symbols”

- Candidate values: {2, 4}

| Company | Comment |
| --- | --- |
| ZTE | Support |
| DOCOMO | We support the proposal. DL reception in two DL subbands should be part of FG 60-1. Therefore, the supported maximum number of partial PRGs in SBFD symbols should also be reported as a component of FG 60-1. |
| Ericsson | OK |
| Huawei, HiSilicon | Support |
| Samsung | Support |
| QC | Support |

2.4	FG for transmissions and receptions across SBFD and non-SBFD symbols (configuration 2)

Following inputs are provided in contributions for the RAN1#121 meeting.

| [1] | Huawei, HiSilicon | Transmissions and receptions across SBFD and non-SBFD symbols (Configuration 2) In RAN1#120bis, the following was agreed Agreement: Introduce following FG for [Transmissions and] Receptions across SBFD and non-SBFD symbols (Configuration 2) as follows, and FFS on whether transmission across SBFD and non-SBFD symbols is defined as separate FG or not - Alt.1: Configuration 2 for transmission and reception should be defined in a single FG - Alt.2: Configuration 2 for transmission and reception should be defined in separate FGs. The remaining issue is whether to define transmission and reception in single FG or separate FGs. Alt.2 unnecessarily fragments this feature by separating support of Configuration 2 for DL reception and UL transmission into two FGs. Alt.1 aligns better with the previous agreement. From implementation point of view, Alt-1 is a cleaner approach. Proposal 3: The support of UL transmissions or DL receptions across SBFD symbols and non-SBFD symbols (Configuration 2) is defined as a single FG separate from the basic FG. |
| --- | --- | --- |
| [2] | ZTE Corporation, Sanechips | Configuration 2 for UL transmissions and DL receptions across SBFD symbols and non-SBFD symbols in different slots. For UL transmissions and DL receptions across SBFD symbols and non-SBFD symbols in different slots, it was agreed that Configuration 1 is the default capability and support of Configuration 2 is subject to UE capability. So, support of Configuration 1 is included in the basic FG 60-1 as shown above. A separate UE capability is needed for Configuration 2. Agreement For UL transmissions and DL receptions across SBFD symbols and non-SBFD symbols in different slots (each transmission/reception within a slot has either all SBFD or all non-SBFD symbols) for an SBFD aware UE, - Support of configuration 2 is subject to UE capability. Configuration 1 is the default capability. - SBFD-aware UE can be configured with Configuration 2 on a per UL/DL BWP basis. - The configuration for DL BWP at least applies to PDSCH receptions within the DL BWP. - The configuration for UL BWP applies to PUCCH and PUSCH transmissions within the UL BWP. - For SRS, only Configuration 1 is applicable.. In RAN1#120bis, FG 60-2 was agreed at least for DL reception and it is still pending whether to have separate FG for UL transmission. In our view, the main difference between transmission and reception is that different RRC configurations are applied for some aspects between SBFD and non-SBFD symbols. However, these aspects have already been included in the basic FG. In this context, we have the following proposal. Proposal 2: Support the following revision on FG 60-2. 60-2; Transmission and reception across SBFD symbols and non-SBFD symbols in different slots (Configuration 2); 1. Support of PDSCH reception across SBFD symbols and non-SBFD symbols (Configuration 2) 2. Support of UL transmission across SBFD symbols and non-SBFD symbols (Configuration 2); FG 60-1; YES; n/a; Transmission and reception across SBFD symbols and non-SBFD symbols in different slots (Configuration 2) are not supported; per band; TDD only; n/a; n/a; Optional with capability signalling. |
| [3] | vivo |  |
| [4] | Nokia | FG 60-2 For FG 60-2, the following discussion points are still open: [2. Support of UL transmission across SBFD symbols and non-SBFD symbols (Configuration 2)] FFS: Other component(s) FFS: whether component 2 is defined as separate FG. For component 2, transmissions and receptions across different symbol types should be considered as a same FG. For the FFS on other component, at least the support of resource determination using RB offset for PUSCH should be added as a component. Proposal 2. For FG 60-2, - remove component 2 and the square brackets in the FG name, - add the support of resource determination using RB offset for PUSCH as a component. |
| [5] | MediaTek Inc. | Support of configuration 2 for UL transmission and DL reception In RAN1#120bis, RAN1 agreed to support a separate FG for receptions across SBFD and non-SBFD symbols (Configuration 2). It still needs to be determined whether transmission across SBFD and non-SBFD symbols is defined as separate FG or not. Since different UE behaviors are required for UL and DL, having one report for the support of configuration 2 for UL transmission and another report for the support of configuration 2 for DL reception (Alt.2) is preferred. To limit the number of reported FGs, support of configuration 2 for transmission across SBFD and non-SBFD symbols can be defined as part of the basic FG for SBFD operation. - Support Ccnfiguration 2 for transmission and reception as separate FGs - Support of configuration 2 for transmission across SBFD and non-SBFD symbols added to basic FG fpr SBFD operation. |
| [6] | Spreadtrum, UNISOC | The following agreements were made in RAN1#120b. Agreement: Introduce following FG for [Transmissions and] Receptions across SBFD and non-SBFD symbols (Configuration 2) as follows, and FFS on whether transmission across SBFD and non-SBFD symbols is defined as separate FG or not - Alt.1: Configuration 2 for transmission and reception should be defined in a single FG - Alt.2: Configuration 2 for transmission and reception should be defined in separate FGs 60-2; [Transmission and] Reception across SBFD symbols and non-SBFD symbols in different slots (Configuration 2); 1. Support of PDSCH reception across SBFD symbols and non-SBFD symbols (Configuration 2) [2. Support of UL transmission across SBFD symbols and non-SBFD symbols (Configuration 2)] FFS: Other component(s) FFS: whether component 2 is defined as separate FG; YES; n/a; [Transmission and] Reception across SBFD symbols and non-SBFD symbols in different slots (Configuration 2) is not supported; [Per Band]; TDD only; n/a; n/a; Optional with capability signalling.. From our perspective, it should be two separate FGs for UL transmission and for DL reception, respectively. Based on RAN1 agreement, SBFD-aware UE can be configured with Configuration 2 on a per UL/DL BWP basis. So UE can only support Configuration 2 for UL transmission or DL reception. The UE behaviour is different for DL and UL. For example, TBS determination is same as legacy for UL transmission. But there are some new behaviour for TBS determination for DL reception. Thus, define separate FG from FG for reception across SBFD symbols and non-SBFD symbols in different slots: Support of configuration 2 for UL transmission should be single FG (60-2a). |
| [7] | Samsung | Regarding FG 60-2, our views are - Configuration 2 can be split into two FGs for UL-Tx and DL-Rx. But, we generally fine to define single FGs for UL-Tx and DL-Rx to avoid lots of FGs (Alt 1) - It can be per band Proposal 2. Support the following FG 60-2 for SBFD to support configuration 2. 60-2; [Transmission and] reception across SBFD symbols and non-SBFD symbols in different slots (Configuration 2); 1. Support of PDSCH reception across SBFD symbols and non-SBFD symbols (Configuration 2) [2. Support of UL transmission across SBFD symbols and non-SBFD symbols (Configuration 2)] FFS: Other component(s) FFS: whether component 2 is defined as separate FG; YES; n/a; [Transmission and] Reception across SBFD symbols and non-SBFD symbols in different slots (Configuration 2) is not supported; [Per Band]; TDD only; n/a; n/a; Optional with capability signalling. |
| [8] | Ofinno | For Configuration 2, per band new FG addition to the basic FG is agreed. However, the FG is further divided into two is discussed in the previous meeting with following alternatives. - Alt.1: Configuration 2 for transmission and reception should be defined in a single FG - Alt.2: Configuration 2 for transmission and reception should be defined in separate FGs In our view, so far there was no agreement to support separate Configuration 2 for transmission and reception, and the basic FG has not been defined separately for Tx and Rx either. Therefore, introducing separate FGs for Tx and Rx aspects of this feature does not have strong motivation at this stage. It is our understanding that a unified FG per band for Configuration 2 remains more aligned with the current framework and agreements. |
| [9] | CATT | FG for transmissions and receptions across SBFD and non-SBFD symbols (configuration 2) - Regarding UE features for SBFD operation configuration 2, the following was agreed. Agreement: Introduce following FG for [Transmissions and] Receptions across SBFD and non-SBFD symbols (Configuration 2) as follows, and FFS on whether transmission across SBFD and non-SBFD symbols is defined as separate FG or not - Alt.1: Configuration 2 for transmission and reception should be defined in a single FG - Alt.2: Configuration 2 for transmission and reception should be defined in separate FGs 60-2; [Transmission and] Reception across SBFD symbols and non-SBFD symbols in different slots (Configuration 2); 1. Support of PDSCH reception across SBFD symbols and non-SBFD symbols (Configuration 2) [2. Support of UL transmission across SBFD symbols and non-SBFD symbols (Configuration 2)] FFS: Other component(s) FFS: whether component 2 is defined as separate FG; YES; n/a; [Transmission and] Reception across SBFD symbols and non-SBFD symbols in different slots (Configuration 2) is not supported; [Per Band]; TDD only; n/a; n/a; Optional with capability signalling. - Based on RAN1 agreement, UE behaviour is different for DL and UL in SBFD operation configuration 2. Furthermore, SBFD-aware UE can be configured with Configuration 2 on a per UL/DL BWP basis. Hence, it should be two separate FGs for UL transmission and for DL reception, respectively. Proposal 4: Support the following FGs for SBFD operation configuration 2. 60-2; Reception across SBFD symbols and non-SBFD symbols in different slots (Configuration 2); 1. Support of PDSCH reception across SBFD symbols and non-SBFD symbols (Configuration 2); 60-1; YES; n/a; Reception across SBFD symbols and non-SBFD symbols in different slots (Configuration 2) is not supported; Per Band; TDD only; n/a; n/a; Optional with capability signalling. 60-2a; Transmission across SBFD symbols and non-SBFD symbols in different slots (Configuration 2); 1. Support of UL transmission across SBFD symbols and non-SBFD symbols (Configuration 2)]; 60-1; YES; n/a; Transmission across SBFD symbols and non-SBFD symbols in different slots (Configuration 2) is not supported; Per Band; TDD only; n/a; n/a; Optional with capability signalling. |
| [10] | CMCC | Regarding the agreed FG 60-2, we prefer Alt.1: Configuration 2 for transmission and reception should be defined in a single FG. |
| [11] | Xiaomi | For FG 60-2, the main remaining issue is whether to include transmission across SBFD and non-SBFD symbols. Given that the mechanisms for DL receptions and UL transmissions are quite different for Configuration 2, it is proposed to not include transmission in FG 60-2. Instead, a separate FG is defined for transmission for Configuration 2. It is proposed to adopt “Per band” type for FG 60-2 and the FG for UL transmissions with Configuration 2. Proposal 4: Support of UL transmission across SBFD symbols and non-SBFD symbols (Configuration 2) is defined as a separate FG from FG 60-2. - Adopt “Per band” type for the separate FG. Proposal 5: Adopt “Per band” type for FG 60-2. |
| [12] | OPPO | For FG 60-2 The Configuration 2 involves with a set of relatively more complicated operations. From UE implementation point of view, it should be given the chance to implement just UL transmission or DL reception but not necessarily both. Proposal 4: Split FG 60-2 into two FG’s, one or DL reception and another for UL transmission. - Both split FGs have FG 60-1 as the prerequisite. |
| [13] | Apple |  |
| [14] | Qualcomm Incorporated | UE FG 60-2 for the support of configuration #2 for UL and DL For UL transmission (or DL reception) across SBFD and non-SBFD symbols in different slots, the support of configurations #2 should be separate UE FG. UE should report separately the support of configuration #2 for UL transmission and DL reception as they are also separately configured per UL/DL BWP basis. Agreement For UL transmissions and DL receptions across SBFD symbols and non-SBFD symbols in different slots (each transmission/reception within a slot has either all SBFD or all non-SBFD symbols) for an SBFD aware UE, - Support of configuration 2 is subject to UE capability. Configuration 1 is the default capability. - SBFD-aware UE can be configured with Configuration 2 on a per UL/DL BWP basis. - The configuration for DL BWP at least applies to PDSCH receptions within the DL BWP. - The configuration for UL BWP applies to PUCCH and PUSCH transmissions within the UL BWP. - For SRS, only Configuration 1 is applicable.. Proposal 2: Support a UE feature (FG X-2a) for separately supporting configuration # 2 of UL transmissions across SBFD symbols and non-SBFD symbols in different slots (each transmission/reception within a slot has either all SBFD or all non-SBFD symbols). 60. NR_duplex_evo; 60-2; Reception across SBFD symbols and non-SBFD symbols in different slots (Configuration 2); 1. Support of PDSCH reception across SBFD symbols and non-SBFD symbols (Configuration 2); YES; n/a; Reception across SBFD symbols and non-SBFD symbols in different slots (Configuration 2) is not supported; Per Band]; TDD only; n/a; n/a; Optional with capability signalling. 60. NR_duplex_evo; 60-2a; Transmission across SBFD symbols and non-SBFD symbols in different slots (Configuration 2); 1. Support of PUCCH and PUSCH transmission across SBFD symbols and non-SBFD symbols (Configuration 2); YES; n/a; Transmission across SBFD symbols and non-SBFD symbols in different slots (Configuration 2) is not supported; Per Band; TDD only; n/a; n/a; Optional with capability signalling. |
| [15] | NTT DOCOMO, INC. | Configuration 2 On whether single or separate FGs for UL transmission and DL reception for configuration 2, we think a single separate FG is enough since the complexity is similar. Proposal 4: Configuration 2 for transmission and reception should be defined in a single FG. |
| [16] | Ericsson | - Include support for transmissions and receptions across SBFD and non-SBFD symbols in a single FG for both UL transmission and DL reception in FG 60-2. - For FG 60-2, change Component names to ‘1. Support of PDSCH reception in both SBFD symbols and non-SBFD symbols in different slots’ and ‘2. Support of UL transmissions in both SBFD symbols and non-SBFD symbols in different slots’. - For FG 60-2, change consequence description to ‘Transmission and reception in both SBFD symbols and non-SBFD symbols in different slots is not supported’. |

The following shows the latest status for the relevant FG(s).

| 60-2 | [Transmission and] Reception across SBFD symbols and non-SBFD symbols in different slots (Configuration 2) | 1. Support of PDSCH reception across SBFD symbols and non-SBFD symbols (Configuration 2) [2. Support of UL transmission across SBFD symbols and non-SBFD symbols (Configuration 2)] FFS: Other component(s) FFS: whether component 2 is defined as separate FG |  | YES | n/a | [Transmission and] Reception across SBFD symbols and non-SBFD symbols in different slots (Configuration 2) is not supported | [Per Band] | TDD only | n/a | n/a |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Summary of companies’ views and discussion

Whether the support of configuration 2 is reported as separate FG(s) or as component of basic FG - Single separate FG applied for both DL and UL: ZTE, Nokia, CMCC, HW/Hisi, Samsung, Ofinno, DOCOMO, Ericsson, - Two separate FGs for UL-Tx/DL-Rx: SPRD/UNISOC, CATT, Xiaomi, OPPO, Qualcomm, Apple, - One separate FG for DL, and configuration 2 for UL is a component in the basic FG: MediaTek, Components: - “the support of resource determination using RB offset for PUSCH: Nokia, Type of the FG(s) - Per band: vivo, ZTE, CATT, Samsung, Xiaomi, Apple, QC, Others - change Component names to ‘1. Support of PDSCH reception in both SBFD symbols and non-SBFD symbols in different slots’ and ‘2. Support of UL transmissions in both SBFD symbols and non-SBFD symbols in different slots’.: E/// - change consequence description to ‘Transmission and reception in both SBFD symbols and non-SBFD symbols in different slots is not supported’: E///

Question 2-4-1:

Companies are encouraged to provide their views for how to resolve the FFS about UL transmission in FG 60-2

- Alt.1: Configuration 2 for transmission and reception should be defined in a single FG

- Alt.2: Configuration 2 for transmission and reception should be defined in separate FGs

| Company | Comment |
| --- | --- |
| ZTE | Support Alt.1. No matter from complexity or IODT perspective, Alt.1 is more reasonable to us. |
| DOCOMO | We prefer Alt 1. |
| Ericsson | We support Alt-1. |
| Huawei, HiSilicon | Alt.1. Alt.2 unnecessarily fragments this feature by separating support of Configuration 2 for DL reception and UL transmission into two FGs. From implementation point of view, Alt-1 is a cleaner approach. |
| Samsung | Alt.1 |
| QC | Alt. 2 |

2.5	Potential other FGs

Following inputs are provided in contributions for the RAN1#121 meeting.

| [1] | Huawei, HiSilicon |  |
| --- | --- | --- |
| [2] | ZTE Corporation, Sanechips |  |
| [3] | vivo |  |
| [4] | Nokia |  |
| [5] | MediaTek Inc. |  |
| [6] | Spreadtrum, UNISOC |  |
| [7] | Samsung | Additionally, PDSCH will be rate matched when it crosses the boundaries of DL usable PRBs. In legacy, a UE may be configured with up to 4 RateMatchPattern(s) per BWP and up to 4 RateMatchPattern(s) per serving cell. For example, once a UE supports semi-static rate matching, this UE has to support up to 4 rate matching patters per BWP and per cell where these values are predefined in 38.214. To avoid increasing the UE complexity, such operation should be counted towards the legacy limits on RateMatchPattern(s) per BWP or per cell. It may be counted as a predefined value, e.g., one RateMatchPattern, or it may be counted as β RateMatchPattern(s) where the value of β is indicated via UE capability signaling. Proposal 5. Support the following FG for SBFD to take into account rate-matching of PDSCH reception within DL subband(s) in SBFD symbol 60-C; Rate matching of PDSCH reception within DL subband(s); 1. Counting of PDSCH rate matching within DL subband(s) toward the maximum number of cell-specific RateMatchingPattern(s); X-1; Yes; PDSCH cannot be rate-matched within DL subband(s) when 4 cell-specific RateMatchingPattern(s) are configured; Per band; The maximum number of RateMatchPattern(s) that can be configured per BWP or per cell is 3, when rate matching of PDSCH occurs because of cross the boundaries of DL usable PRBs.; Optional with capability signalling. |
| [8] | Ofinno | Proposal 3. Support to introduce a new FG for rate matching of PDSCH reception within DL subband(s) in SBFD symbols, as in the table in Annex A. |
| [9] | CATT |  |
| [10] | CMCC |  |
| [11] | Xiaomi |  |
| [12] | OPPO |  |
| [13] | Apple |  |
| [14] | Qualcomm Incorporated | UE FG on Maximum number of DL/UL switching As the SBFD-aware UE is half duplex, it may frequently switch between DL and UL mode during SBFD slot and/or the SBFD pattern. This will incur some many DL-UL switching as compared to legacy semi-static TDD operation where only UE would switch once for DDDSU TDD pattern. RAN1 should discuss some limitation on the maximum number of DL/UL switching at UE side as frequency switching will affect UE peak throughput. In current NR specification, component 9 of FG 5-1 puts a limitation of maximum one switching point per flexible slot in TDD deployment as a baseline while FG 5-1b allows for more than one DL/UL switch point in a slot. Then, similarly for an SBFD slot, the maximum number of switching points per SBFD slot should be one. Additionally, there should be a limit on the maximum number of DL/UL switching point per SBFD pattern, otherwise many of the symbols will be wasted for frequency DL-UL switching. Example in the figure below shows a scenario where UE needs to switch in every SBFD slot from DL mode (PDCCH reception) to UL mode (PUSCH transmission) with 1-2 symbols are not utilized per each SBFD slot to accommodate Tx-Rx switching and UE TA advance. Figure 2-1: Example of DL-UL switching per each SBFD slot with 1-2 symbols gap for Rx-Tx switching. Proposal 5: Support a UE feature for reporting the maximum number of DL-UL switching in SBFD slot, and SBFD/TDD period. 60. NR_duplex_evo; 60-2e; Maximum number of DL-UL switching in SBFD slots.; 1. The maximum number of DL-UL switching in SBFD slot. 2. The maximum number of DL-UL switching in SBFD/TDD period; YES; n/a; Per Band; TDD only; n/a; n/a; Optional with capability signalling. UE FG on supported UL/DL SB configurations For DUD pattern where the UL subband is in the middle of the carrier, designing a receiver notch filter to suppress the uplink subband is challenging than a filter for a single DL subband. Additionally, some low tier UEs can’t accommodate DL reception across the two DL subbands. This motivates UE reporting of single DL subband scheduling. In addition, SBFD-aware would require a UE-specific guardband to accommodate the UE receiver filters requirement which is different than gNB cell-specific guardband [1].. Proposal 6: Support a UE feature UE reporting the supported minimum guardband between DL and UL subbands. 60. NR_duplex_evo; 60-2d; Guardband between the UL/DL subband; Minimum guardband between UL and DL subband.; YES; n/a; Per Band; TDD only; n/a; n/a; Optional with capability signalling. |
| [15] | NTT DOCOMO, INC. |  |
| [16] | Ericsson |  |

Summary of companies’ views and discussion

- Rate matching of PDSCH reception within DL subband(s): Samsung, Ofinno, - the maximum number of DL-UL switching in SBFD slot, and SBFD/TDD period: QC, - Guardband b/w UL subband and DL subband(s): QC,

Question 2-5-1:

Companies are encouraged to provide their views on whether each of the following (related to separate configuration for SBFD and non-SBFD symbols) should be part of basic FG for SBFD operation or separate FG.

- Question 2-5-1-1: Whether to define a new FG for “Rate matching of PDSCH reception within DL subband(s)”

  - Note: Proposed in [7] and [8]

- Question 2-5-1-2: Whether to define a new FG for “Maximum number of DL-UL switching in SBFD slots.”

  - Note: Proposed in [14]

- Question 2-5-1-3: Whether to define a new FG for “Guardband between the UL/DL subband”

  - Note: Proposed in [14]

| Company | Comment |
| --- | --- |
| ZTE | Question 2-5-1-1: Seems no need to bundle with rate matching features. Question 2-5-1-2: Ok to discuss. Question 2-5-1-3: No. Need to be decided in AI 9.3.1 first. |
| DOCOMO | Question 2-5-1-1: NOT prefer to define a new FG for “Rate matching of PDSCH reception within DL subband(s)”. Rate matching of PDSCH is already supported as basic feature. Question 2-5-1-2: NOT prefer to define a new FG for “Maximum number of DL-UL switching in SBFD slots.” If maximum number of DL-UL switching in SBFD slots is reported, it’s better to be as a component of FG 60-1. Question 2-5-1-3: NOT prefer to define a new FG for “Guardband between the UL/DL subband”. If guardband between the UL/DL subband is reported, it’s better to be as a component of FG 60-1. |
| Ericsson | Q1: We prefer not to define a new FG for “Rate matching of PDSCH reception within DL subband(s)”. Q2: We prefer not to define a new FG for “Maximum number of DL-UL switching in SBFD slots.” Q3: Regarding guard band, RAN1 has agreed to specify implicit guard bands based on explicit subband configurations. Hence, we prefer not to define a new FG for “Guardband between the UL/DL subband”. |
| Huawei, HiSilicon | We don’t see the need introduce these new FGs. |
| Samsung | Question 2-5-1-1: Need to discuss. Valid PRB determination is considered as cell-specific rate-matching. So, whether to share rate-matching capability or not is needed to be clarified. Question 2-5-1-2: Ok to discuss Question 2-5-1-3: It was discussed under AI9.3.1, and no agreements yet. Need to conclude it on 9.3.1 first. |
| QC | Question 2-5-1-1: basic UE FG Question 2-5-1-2 (at least for #DL-UL switching within SBFD slot): separate FG with candidate values {1,2}. Question 2-5-1-3: separate FG that would help gNB do proper DL/UL scheduling. |

- FGs for SBFD random access operation

3.1	FG for SBFD random access operation based on RACH configuration Option 1 with Alt 1-1

Following inputs are provided in contributions for the RAN1#121 meeting.

| [1] | Huawei, HiSilicon | SBFD random access operation related UE features According to the RAN1#120 agreement, two separate UE capabilities are introduced for the support of SBFD random access operation based on the two different RACH configuration options - Support of SBFD random access operation based on one single RACH configuration in RRC_IDLE/INACTIVE/CONNECTED mode (RACH configuration Option 1 with Alt 1-1) - Support of SBFD random access operation based on two separate RACH configurations, including one legacy RACH configuration and one additional RACH configuration in RRC_IDLE/INACTIVE/CONNECTED mode (RACH configuration Option 2) In RAN1#120, there was a discussion on whether the support of RACH configuration Option 1 or Option 2 should be included in the basic feature group and the following was agreed Agreement: Introduce following FGs for SBFD random access, one based on single RACH configuration (Option 1) and the other based on two RACH configurations (Option 2), and FFS on whether UE supporting SBFD random access needs to support at least Option 1 or Option 2. - Alt.1: UE supporting SBFD random access needs to support at least Option 1 - Alt.2: UE supporting SBFD random access needs to support at least Option 2 - Alt.3: UE supporting SBFD random access supports Option 1 or Option 2 (i.e., no default option). Considering that an SBFD-aware UE can perform random access based on legacy ROs as a legacy UE, it seems not critical to support a default option (Alt.3). One argument for Alt.1 is that UE may not implement either Option 1 or Option 2 if there is no default option. If this is the primary concern, another alternative is to support both Option 1 and Option 2 even through this contradicts with previous agreement. It should be noted that RACH configuration Option 1 and Option 2 have different application scenarios. It does not make sense to prioritize one use case over the other. Proposal 4: The support of RACH configuration Option 1 and Option 2 are defined as separate FGs, i.e., no default option. |
| --- | --- | --- |
| [2] | ZTE Corporation, Sanechips | UE features for RACH procedure In RAN1#120bis, the following agreements were reached on SBFD random access operation. 60-3; SBFD random access operation based on one single RACH configuration in RRC_IDLE/INACTIVE/CONNECTED mode; Support of SBFD random access operation based on one single RACH configuration in RRC_IDLE/INACTIVE/CONNECTED mode (RACH configuration Option 1) FFS: Other component(s) which may be separate FG; 60-1; YES; n/a; SBFD random access operation based on one single RACH configuration in RRC_IDLE/INACTIVE/CONNECTED mode is not supported; Per band; TDD only; n/a; n/a; Optional with capability signalling. 60-4; SBFD random access operation based on two separate RACH configurations, including one legacy RACH configuration and one additional RACH configuration in RRC_IDLE/INACTIVE/CONNECTED mode; Support of SBFD random access operation based on two separate RACH configurations, including one legacy RACH configuration and one additional RACH configuration in RRC_IDLE/INACTIVE/CONNECTED mode (RACH configuration Option 2) FFS: Other component(s) which may be separate FG; [60-1, 60-3]; YES; n/a; SBFD random access operation based on two separate RACH configurations in RRC_IDLE/INACTIVE/CONNECTED mode is not supported; Per band; TDD only; n/a; n/a; Optional with capability signalling. For FG 60-3 and 60-4, our views are summarized as below. - Regarding whether to add additional components, down-select from the following two alternatives. - Alt 1: - Add the following components in FG 60-3. - Support of reinterpretation of frequency offset of lowest RO - Support of separate power control parameters in additional-RO - Support of separate mapping of SSB and additional ROs, and separate determination of the association period and association pattern period. - Add the following components in FG 60-4. - Support of valid RO across SBFD and non-SBFD symbols - Alt 2: Keep FG 60-3 and FG 60-4 concise as it is. Neither add any additional components nor any additional separate FGs. - Note: If legacy FGs, e.g., Msg1 repetition, is reported together with FG 60-3/60-4, the conjunction of Msg1 repetition and SBFD is supported. - FG 60-3 is the prerequisite FG of FG 60-4. With above, we have the following proposal. Proposal 3: For FG 60-3 and FG 60-4, FG 60-3 is the prerequisite FG of FG 60-4, and down-select from the following two alternatives. - Regarding whether to add additional components, down-select from the following two alternatives. - Alt 1: - Add the following components in FG 60-3. - Support of reinterpretation of frequency offset of lowest RO - Support of separate power control parameters in additional-RO - Support of separate mapping of SSB and additional ROs, and separate determination of the association period and association pattern period. - Add the following components in FG 60-4. - Support of valid RO across SBFD and non-SBFD symbols - Alt 2: Keep FG 60-3 and FG 60-4 concise as it is. Neither add any additional components nor any additional separate FGs. - Note: If legacy FGs, e.g., Msg1 repetition, is reported together with FG 60-3/60-4, the conjunction of Msg1 repetition and SBFD is supported. |
| [3] | vivo | - SBFD random access operation - Following questions on UE features for SBFD RACH operation were discussed. - Whether RACH configuration option 1 is default option - Whether ‘support of valid RO across SBFD and non-SBFD symbols’ should be part of FG 60-4 or separate FG - Whether to introduce a separate FG for PRACH repetition in additional ROs, and whether separate FGs for RACH configuration Option 1 and Option 2 - Whether to introduce a separate FG for Msg.3 repetition in SBFD symbols - Regarding the questions, we have the following proposals. Proposal 5: UE supporting SBFD random access supports Option 1 or Option 2 (i.e., no default option) Proposal 6: Support of valid RO across SBFD and non-SBFD symbols should be a separate FG. Proposal 7: Support to introduce a separate FG for PRACH repetition in additional ROs. - This separate FGs is common for RACH configuration Option 1 and Option 2 Proposal 8: Support to introduce a separate FG for Msg.3 repetition in SBFD symbols - For SBFD RACH operation, following FGs are suggested. Proposal 9: Support followings FGs for SBFD RACH operation. 60-3; SBFD random access operation based on one single RACH configuration in RRC_IDLE/INACTIVE/CONNECTED mode; Support of SBFD random access operation based on one single RACH configuration in RRC_IDLE/INACTIVE/CONNECTED mode (RACH configuration Option 1) FFS: Other component(s) which may be separate FG; 60-1; YES; n/a; SBFD random access operation based on one single RACH configuration in RRC_IDLE/INACTIVE/CONNECTED mode is not supported; Per band; TDD only; n/a; n/a; Optional with capability signalling. 60-4; SBFD random access operation based on two separate RACH configurations, including one legacy RACH configuration and one additional RACH configuration in RRC_IDLE/INACTIVE/CONNECTED mode; Support of SBFD random access operation based on two separate RACH configurations, including one legacy RACH configuration and one additional RACH configuration in RRC_IDLE/INACTIVE/CONNECTED mode (RACH configuration Option 2) FFS: Other component(s) which may be separate FG; [60-1, 60-3]; YES; n/a; SBFD random access operation based on two separate RACH configurations in RRC_IDLE/INACTIVE/CONNECTED mode is not supported; Per band; TDD only; n/a; n/a; Optional with capability signalling. 60-4a; RACH occasion across SBFD and non-SBFD symbol; Support additional ROs across SBFD symbols and non-SBFD symbols in the same slot or across slots; 60-4; YES; n/a; UE does not support RACH occasion across SBFD and non-SBFD symbol; [Per Band]; TDD only; n/a; n/a; Optional with capability signalling. 60-5; Preamble repetition for SBFD RA operation; 1. Determination of the set of Npreamblerep valid additional-ROs and the time period of the set(s) of Npreamblerep valid additional-ROs 2. Support separate configuration of rsrp-ThresholdMsg1-RepetitionNum2/4/8 for PRACH transmission with preamble repetitions within additional-ROs and PRACH transmission with preamble repetitions within legacy-ROs.; 54-1, 60-3 or 60-4,; YES; n/a; Preamble repetition for SBFD RA operation is not supported; [Per Band]; TDD only; n/a; n/a; Optional with capability signalling. 60-6; Msg.3 repetition in SBFD symbols; Support of repetition of PUSCH transmission scheduled by RAR UL grant and DCI format 0_0 with CRC scrambled by TC-RNTI in SBFD symbols; [30-6, 60-3 or 60-4]; YES; n/a; Msg.3 repetition in SBFD symbols is not supported; [Per Band]; TDD only; n/a; n/a; Optional with capability signalling. |
| [4] | Nokia | FG 60-3 FG 60-3 for SBFD was agreed in RAN1#120 as shown in the Annex. In this section we discuss the remaining issues (highlighted in yellow) related FG 60-3. Concerning the FFS on other components, the following components can be further added: - Support valid additional ROs determination, including reinterpretation of msg1-FrequencyStart in rach-ConfigCommon. - Support separate association periods and association pattern periods determination for additional ROs and legacy ROs. - Support separate SSB-to-RO mapping for the valid additional ROs. - Support separate power control parameters for PRACH transmission in SBFD symbols and non-SBFD symbols. There was also a discussion on whether a separate FG should be indicated for Msg3 repetition in SBFD symbol. In our view, this feature can be a component of the basic FG for random access. Therefore, it should be part of FG 60-3. Proposal 3. For FG 60-3, - The following components should be added: - Support valid additional ROs determination, including reinterpretation of msg1-FrequencyStart in rach-ConfigCommon. - Support separate association periods and association pattern periods determination for additional ROs and legacy ROs. - Support separate SSB-to-RO mapping for the valid additional ROs. - Support separate power control parameters for PRACH transmission in SBFD symbols and non-SBFD symbols. - Support of repetition of PUSCH transmission scheduled by RAR UL grant and DCI format 0_0 with CRC scrambled by TC-RNTI in SBFD symbols. |
| [5] | MediaTek Inc. | Whether UE supporting SBFD random access needs to support at least Option 1 There was a discussion on whether a UE supporting SBFD random access operation needs to support at least RACH configuration Option 1. According to RAN1 agreement from RAN1#117, a UE is not required to support both RACH configuration options. Therefore, if a UE only supports RACH configuration Option 2, that UE cannot have RACH configuration Option 1 as a default option. Although having a default RACH configuration option may be beneficial for a UE that supports both RACH configuration Options, the RAN1 agreement does not support defining a default RACH configuration Option. - UE supporting SBFD random access supports Option 1 or Option 2 (i.e., no default option). |
| [6] | Spreadtrum, UNISOC | - FG 60-3 Type of the FG: Per band Other components: - Support of reinterpretation of frequency offset of lowest RO - Support of separate power control parameters in additional-ROs. - Support of separate mapping of SSB and additional ROs, separate determination of the association period and association pattern period |
| [7] | Samsung | The following agreements were made in RAN1#120 [3]. <60-3, 60-4> Agreement (RAN1#120) Introduce following FGs for SBFD random access, one based on single RACH configuration (Option 1) and the other based on two RACH configurations (Option 2), and FFS on whether UE supporting SBFD random access needs to support at least Option 1 or Option 2. - Alt.1: UE supporting SBFD random access needs to support at least Option 1 - Alt.2: UE supporting SBFD random access needs to support at least Option 2 - Alt.3: UE supporting SBFD random access supports Option 1 or Option 2 (i.e., no default option) 60-3; SBFD random access operation based on one single RACH configuration in RRC_IDLE/INACTIVE/CONNECTED mode; Support of SBFD random access operation based on one single RACH configuration in RRC_IDLE/INACTIVE/CONNECTED mode (RACH configuration Option 1) FFS: Other component(s) which may be separate FG; X-1; YES; n/a; SBFD random access operation based on one single RACH configuration in RRC_IDLE/INACTIVE/CONNECTED mode is not supported; [Per Band]; TDD only; n/a; n/a; Optional with capability signalling. 60-4; SBFD random access operation based on two separate RACH configurations, including one legacy RACH configuration and one additional RACH configuration in RRC_IDLE/INACTIVE/CONNECTED mode; Support of SBFD random access operation based on two separate RACH configurations, including one legacy RACH configuration and one additional RACH configuration in RRC_IDLE/INACTIVE/CONNECTED mode (RACH configuration Option 2) FFS: Other component(s) which may be separate FG; [X-1, X-3]; YES; n/a; SBFD random access operation based on two separate RACH configurations in RRC_IDLE/INACTIVE/CONNECTED mode is not supported; [Per Band]; TDD only; n/a; n/a; Optional with capability signalling. Agreement: Adopt “Per band” type for FG 60-3. Agreement: Adopt “Per band” type for FG 60-4 In RAN1#120, 2 separate FGs were agreed for UE support of RACH configuration Option 1 or 2, respectively. For the gNB, the choice to implement and/or support one of RACH configuration Option 1 or 2 in the serving cell is not restricted. For certain use cases such as extended coverage/cell range using random access on SBFD resources, RACH configuration Option 2 is needed to configure long preamble formats such as B4/F0 when not possible on the legacy ROs. In other cases, PRACH repetition using either RACH configuration Options 1 or 2 can be sufficient. It is also possible, that random access on SBFD resources in the serving cell is enabled by the gNB only for UEs in RRC_CONNECTED mode. Alt.3, i.e., no default option, has significant risk for feature fragmentation. A UE supporting X-3 entering a cell indicating support for RACH config Option 2 cannot benefit from random access on SBFD resources. Similarly, when the gNB configures use of RACH config Option 1 in SIB1, but the UE is not required to support X-3, there is little incentive to implement such a feature. Our strong preference is therefore at least Alt.1, or alternatively, the new Alt. 4 “UE supporting SBFD random access supports Option 1 and Option 2. Proposal 6. Support the (new) Alt. 4 “UE supporting SBFD random access supports Option 1 and Option 2”. |
| [8] | Ofinno | FGs for random access operation SBFD random access based on a single RACH configuration (Option 1) applies across all RRC states, including CFRA initiated by PDCCH order. This option enables PRACH transmission without repetition and represents the baseline capability for SBFD random access. A UE supporting SBFD random access is required to support at least this option. SBFD random access based on two RACH configurations (Option 2) also applies across all RRC states, including CFRA initiated by PDCCH order. This option allows the use of two separate RACH configurations, for example across SBFD and non-SBFD symbols. Option 2 is defined under a separate FG from Option 1 and requires the UE to support both the SBFD basic FG and Option 1 for RACH configuration as prerequisites. This option also assumes PRACH transmission without repetition. In addition, support for valid ROs that span SBFD and non-SBFD symbols is defined as a separate FG under Option 2. A UE supporting this FG shall indicate support for Option 2-based configuration. Preamble repetition capability is configured independently of the RACH configuration option. If the UE supports only Option 1 or only Option 2, the repetition capability applies to the supported option. If the UE supports both options, the repetition capability applies to both configurations. Proposal 4. Update 60-3 and 60-4 random access features to include CFRA initiated by PDCCH order and introduce new features, as in table in Annex A. RAN1#120 agreed the following with selection between Alt 1 and Alt3. Introduce following FGs for SBFD random access, one based on single RACH configuration (Option 1) and the other based on two RACH configurations (Option 2), and FFS on whether UE supporting SBFD random access needs to support at least Option 1 or Option 2. - Alt.1: UE supporting SBFD random access needs to support at least Option 1 - Alt.2: UE supporting SBFD random access needs to support at least Option 2 - Alt.3: UE supporting SBFD random access supports Option 1 or Option 2 (i.e., no default option). We consider Option 1 should be considered as default. To maximize utilization of SBFD feature during initial access procedure, a default option seems beneficial. Thus, we support Alt. 1. Proposal 5. Adopt Alt 1 for SBFD random access. |
| [9] | CATT | FG for SBFD random access operation based on RACH configuration Option 1 with Alt 1-1 An Open issue is that whether UE supporting SBFD random access needs to support at least Option 1. The following alternatives were agreed for down selection in RAN1#120. - Alt.1: UE supporting SBFD random access needs to support at least Option 1 - Alt.3: UE supporting SBFD random access supports Option 1 or Option 2 (i.e., no default option) According to RAN1 agreement in RAN1#117, a UE is not required to support both RACH configuration options. Therefore, Alt.3 should be supported. UE supporting SBFD random access supports Option 1 or Option 2 (i.e., no default option). |
| [10] | CMCC | Two FGs for RACH configuration Option 1 and Option 2 are agreed, one discussion issue is that whether Option 1 is the default option for SBFD random access operation. As the agreement made in main session, one UE is not required to support both RACH configuration Option 1 and Option 2. Thus, we think Alt.3 can be supported, thus, UE supporting SBFD random access supports Option 1 or Option 2 (i.e., no default option). Regarding the additional components of FG 60-3 and 60-4: - Support of separate power control parameters for additional-RO and legacy-RO should be added into FG 60-3 - Support of PRACH transmission across SBFD symbols and non-SBFD symbols should be added into FG 60-4 Then some additional UE capabilities can be defined based on the previous RAN1 agreements which don’t need to differentiate separate FGs for RACH configuration Option 1 and Option 2, including: - Support of PRACH repetition - Support of Msg3 repetition |
| [11] | Xiaomi | For FG X-4, one remaining issue is whether FG X-3 is the prerequisite. According to our understanding, FG X-4 is a more advanced UE capability compared with FG X-3. The main difference is to support different preamble formats for SBFD and non-SBFD symbols. For UE supporting FG X-4, UE should be able to support FG X-3. Proposal 6: The following UE features are proposed for SBFD random access operation. |
| [12] | OPPO | For FG 60-4 Regarding to whether to have no default option or keep Option-1 as default (i.e., 60-3 is a prerequisite of 60-4), we slightly prefer to have a default on Option-1 in order to ensure the UE to have an advanced random access functionality (comparing to the legacy) being enabled when roaming to different wireless markets, but are also open to having no default. Proposal 5: Treat SBFD random access Option 1 as the default, i.e., FG 60-4 takes {60-1,60-3} as prerequisite. |
| [13] | Apple | 60-3; RACH configuration Option 1 (Use one single RACH configuration and based on the existing parameters of the single RACH configuration); - Support additional-ROs include the ROs in SBFD symbols configured as downlink by tdd-UL-DL-ConfigurationCommon, and the ROs across SBFD symbols configured as downlink and SBFD symbols configured as flexible by tdd-UL-DL-ConfigurationCommon - Support of reinterpretation of frequency offset of lowest RO - Support separate power control parameters for PRACH transmission in SBFD symbols and non-SBFD symbols - Support of separate mapping of SSB and additional ROs, and separate determination of the association period and association pattern period - Support PRACH transmission in additional-ROs for SBFD-aware UEs in RRC_CONNECTED state and RRC_IDLE/INACTIVE state; 60-1; Yes; n/a; Preamble repetition for SBFD RA operation with RACH configuration Option 1 is not supported; Per band; TDD only; n/a; n/a; Optional with capability signalling. 60-4; RACH configuration Option 2 (Use two separate RACH configurations, including one legacy and one additional RACH configuration); - Support additional-ROs in non-SBFD symbols configured by additional RACH configuration - Support a valid RO starting from SBFD symbol and ending in non-SBFD symbol either in the same slot or across different slots - Support of separate mapping of SSB and additional ROs, and separate determination of the association period and association pattern period - Support PRACH transmission in additional-ROs for SBFD-aware UEs in RRC_CONNECTED state and RRC_IDLE/INACTIVE state; 60-1; Yes; n/a; Preamble repetition for SBFD RA operation with RACH configuration Option 2 is not supported; Per band; TDD only; n/a; n/a; Optional with capability signalling. |
| [14] | Qualcomm Incorporated | UE features for supporting random-access in SBFD symbols For random-access in SBFD symbols, CBRA and CFRA using Type-1 random access procedure (4-step RACH) is supported. where two PRACH configuration options were introduced. PRACH configuration option (1) is based on using a single shared legacy RACH configuration where the additional-ROs in SBFD-DL symbols can be valid for SBFD-aware UE. The PRACH configuration options (2) is based on using two separate PRACH configurations, one legacy PRACH configuration indicating legacy-ROs and additional PRACH configuration indicating additional-ROs for random-access in SBFD symbols. For either PRACH configuration option 1 or PRACH configuration option 2, the ROs used for SBFD RA is referred as ‘additional-ROs’. In RAN1 #120, the following UE FGs was agreed for UE supporting Rel-19 SBFD random access with per-band granularity reporting [2]. Agreement: Adopt “Per band” type for FG 60-3. Agreement: Adopt “Per band” type for FG 60-4 Agreement: Introduce following FGs for SBFD random access, one based on single RACH configuration (Option 1) and the other based on two RACH configurations (Option 2), and FFS on whether UE supporting SBFD random access needs to support at least Option 1 or Option 2. - Alt.1: UE supporting SBFD random access needs to support at least Option 1 - Alt.2: UE supporting SBFD random access needs to support at least Option 2 - Alt.3: UE supporting SBFD random access supports Option 1 or Option 2 (i.e., no default option) X-3; SBFD random access operation based on one single RACH configuration in RRC_IDLE/INACTIVE/CONNECTED mode; Support of SBFD random access operation based on one single RACH configuration in RRC_IDLE/INACTIVE/CONNECTED mode (RACH configuration Option 1) FFS: Other component(s) which may be separate FG; X-1; YES; n/a; SBFD random access operation based on one single RACH configuration in RRC_IDLE/INACTIVE/CONNECTED mode is not supported; [Per Band]; TDD only; n/a; n/a; Optional with capability signalling. X-4; SBFD random access operation based on two separate RACH configurations, including one legacy RACH configuration and one additional RACH configuration in RRC_IDLE/INACTIVE/CONNECTED mode; Support of SBFD random access operation based on two separate RACH configurations, including one legacy RACH configuration and one additional RACH configuration in RRC_IDLE/INACTIVE/CONNECTED mode (RACH configuration Option 2) FFS: Other component(s) which may be separate FG; [X-1, X-3]; YES; n/a; SBFD random access operation based on two separate RACH configurations in RRC_IDLE/INACTIVE/CONNECTED mode is not supported; [Per Band]; TDD only; n/a; n/a; Optional with capability signalling.. Per RAN1 agreement, it is up to UE capability reporting to support either or both PRACH configuration. In addition, the support of PRACH transmission with preamble repetitions within additional-ROs of either PRACH configuration should be reported separately for PRACH configuration option 1 and PRACH configuration option 2. Finally, msg3 repetition in SBFD symbols based on configuration #1 should be reported separately. Proposal 7. Support the following list of UE capabilities for random access in SBFD symbols Index; Feature group; Components; Consequence if the feature is not supported by the UE; Type; Note; Mandatory/Optional. 60-3; SBFD random access operation based on one single RACH configuration in RRC_IDLE/INACTIVE/CONNECTED mode; 1. Support of SBFD random access operation based on one single RACH configuration in RRC_IDLE/INACTIVE/CONNECTED mode (RACH configuration Option 1) 2. Support of PRACH transmission without repetition within additional valid-RO in SBFD symbols configured as downlink by tdd-UL-DL-ConfigurationCommon 3. Support of separate PC parameters for PRACH transmission in the additional-ROs. 4. Separate mapping of SSB to the additional ROs and separate determination of the association period and association pattern period 5. Support of CBRA switching procedures between PRACH transmission in additional-ROs and legacy-ROs. 6. Support of explicit indication of RO-type (legacy-RO or additional-RO) for CFRA by PDCCH order.; 60-1; SBFD random access operation based on one single RACH configuration in RRC_IDLE/INACTIVE/CONNECTED mode is not supported; Per band; Optional with capability signalling. |
| [15] | NTT DOCOMO, INC. | RACH configuration option 1 and option 2 For SBFD random access operation, RACH configuration option 1 (i.e. use single PRACH configuration) and RACH configuration option 2 (i.e. use additional PRACH configuration for SBFD) are supported, and network would like to use either option 1 or 2 according to operation policy. If some UEs support only RACH configuration option 1 and some other UEs support only RACH configuration option 2, network may need to handle them separately e.g., in different cells to avoid complex operation. Fragmented support of RACH configuration options will cause practical issue for SBFD operation. Therefore, taking option 1 as the default option should be supported, i.e. Alt 1. For RACH configuration option 2, it is preferred that support of valid RO across SBFD and non-SBFD symbols is part of FG 60-4. Proposal 5: For FG 60-3 and FG 60-4 for SBFD random access operation: - UE supporting SBFD random access needs to support at least Option 1, i.e. 60-3 should be prerequisite of 60-4. - Support of valid RO across SBFD and non-SBFD symbols should be part of FG 60-4. In our understanding, there is no additional UE complexity to support Msg 3 PUSCH repetitions in SBFD symbols on top of FG 30-6 and FG 60-3/60-4. Msg 3 PUSCH repetitions in SBFD symbols can be supported by the support of FG 30-6 and FG 60-3/60-4. Therefore, Alt 1 is preferred. Proposal 7: Regarding Msg.3 repetition in SBFD symbols, clarify in FG 60-3/4 Note column that “Note: When a UE supports this FG and FG 30-6, the UE also support Msg.3 repetition in SBFD symbol”. |
| [16] | Ericsson | RAN1 #119 agreed to support Configuration 1 for Msg3 repetitions [6]: Agreement For an SBFD aware UE, if addition RO is selected for Msg1 transmission, only Configuration 1 is supported for Msg3 repetition.. In our view, the above agreement leaves no room for reinterpretation and should be adhered to. Hence, a UE supporting Msg3 repetitions must support at least Configuration 1 and only supporting Configuration 2 is not an alternative for SBFD RACH. Apart from Msg3 repetitions, in our view there is no additional need to support either Configuration 1 or Configuration 2 with respect to the PRACH preamble or RAR. RAN1 has also not discussed these configurations in any other context in AI 9.3.2. Both PRACH preamble transmission and RAR reception are unique operations during which time the UE has very little else to do. Hence, it should not be a problem for a UE to either transmit PRACH or receive RAR independently of whether it supports Configuration 1 or Configuration 2. Considering that RAN1 has already agreed to support Configuration 1 for Msg3 repetitions, and that Configuration 1 is the default configuration for all other signals and channels, it is reasonable that it is also the configuration that is assumed during the remainder of the RACH procedure. RAN1 agreements clearly states that Configuration 1 is used for Msg3 repetitions. Configurations 1 and 2 does not apply to PRACH or RAR. A UE supporting the single RACH configuration needs to support at least Configuration 1 (Alt. 1). RAN1 also discussed whether Option 1 or Option 2, if any, should be the default feature. In our view, the only thing Option 1 and Option 2 have in common is that they are both specified withing the SBFD WI. Apart from that, they address completely different use cases and even different UE types. Whereas Option 2 can be found in eMBB UEs, Option 1 addresses IoT and MTC UEs. This view is also supported by the agreement on the two options where it is clearly stated that a “UE is not required to support both options.” Based on that, it is not possible to claim that one option should have precedence over the other. Option 1 and Option 2 target different UE types and therefore, per agreement, a UE is not expected to support both options. Support separate FGs for the single and additional RACH configurations. Another point up for discussion is how to handle the network configuration where a valid RO spans both SBFD and non-SBFD symbols. For that particular use case, to perform initial access with an extended range, it makes little sense to have this component as a separate UE-feature. In that case, the SBFD-aware UE supporting Option 2 would still not be able to connect to the network due to the network being configured with ROs across both SBFD and non-SBFD symbols. That kind of functionality mismatch should surely be avoided, and since RAN1 agreed that this feature is configurable on the network side, it should be mandatory on the UE side, for UEs supporting Option 2. Support for ROs across both SBFD and non-SBFD symbols is a network configuration but should not be a separate UE-capability to avoid capability mismatch. Support of a valid RO across SBFD and non-SBFD symbols is a component of the FG for SBFD random access based on the additional RACH configurations (Option 2) (Alt. 1). |

The following shows the latest status for the relevant FG(s).

| 60-3 | SBFD random access operation based on one single RACH configuration in RRC_IDLE/INACTIVE/CONNECTED mode | Support of SBFD random access operation based on one single RACH configuration in RRC_IDLE/INACTIVE/CONNECTED mode (RACH configuration Option 1) FFS: Other component(s) which may be separate FG | 60-1 | YES | n/a | SBFD random access operation based on one single RACH configuration in RRC_IDLE/INACTIVE/CONNECTED mode is not supported | Per band | TDD only | n/a | n/a |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Summary of companies’ views and discussion

Whether RACH configuration option 1 is default option? - Alt.1: UE supporting SBFD random access needs to support at least Option 1 - ZTE, Offino, Nokia, OPPO, Xiaomi, DOCOMO, - Alt.3: UE supporting SBFD random access supports Option 1 or Option 2 (i.e., no default option) - Vivo, SPRD/UNISOC, CATT, MediaTek, CMCC, HW/Hisi, Ericsson, Apple, Qualcomm, - Alt.4: UE supporting SBFD random access supports both Option 1 and Option 2 - Samsung, Components of the FG 60-3 - Support of reinterpretation of frequency offset of lowest RO: ZTE, SPRD/UNISOC, Nokia, Apple, MediaTek - Support of separate power control parameters in additional-ROs: ZTE, SPRD/UNISOC, Nokia, MediaTek, CMCC, Apple, Qualcomm - Support of separate mapping of SSB and additional ROs: ZTE, SPRD/UNISOC, Nokia, MediaTek, Apple, Qualcomm - Support of separate determination of the association period and association pattern period: SPRD/UNISOC, Nokia, MediaTek, Apple, Qualcomm - Support of CFRA initiated by PDCCH order: Offino - Support of Msg 3 repetitions in SBFD symbols: HW, Nokia. DOCOMO, - Support of PRACH transmission without repetition within additional valid-RO in SBFD symbols configured as downlink by tdd-UL-DL-ConfigurationCommon: Qualcomm - Support of CBRA switching procedures between PRACH transmission in additional-ROs and legacy-ROs: Qualcomm - Support of explicit indication of RO-type (legacy-RO or additional-RO) for CFRA by PDCCH order: Qualcomm Others: - Explicitly capture only SBFD Configuration 1 for PRACH preamble transmission and RAR: E///

Proposal 3-1-1:

Add the following component in FG 60-3.

- Support of reinterpretation of frequency offset of lowest RO

- Support of separate power control parameter (i.e., preambleReceivedTargetPower) in additional-RO

- Support of separate mapping of SSB and additional ROs, and separate determination of the association period and association pattern period.

| Company | Comment |
| --- | --- |
| ZTE | We are supportive of this proposal as shown in our contribution. While, if companies want to add more components to FG 60-3, we are also ok to not include any additional components to FG 60-3 so as to keep this RACH related FG as concise as legacy RACH related features. |
| DOCOMO | Support the proposal. |
| Ericsson | Support. |
| Huawei, HiSilicon | As we commented before, we don't see the need to capture every detailed change into the FG, especially when UE does not report the candidate value for the feature component anyway. |
| Samsung | We are ok not to include. Component description for FG 60-3 doesn’t need to be so detailed. |
| QC | Support. They follow RAN1 agreements and provide necessary clarification for the FG. |

Proposal 3-1-2:

Remove “FFS: Other component(s) which may be separate FG” in FG 60-3.

| Company | Comment |
| --- | --- |
| Moderator | Note that when the need for any new component is identified and supported by companies until ASN.1 freeze (while strongly preferred to be done by August RAN1), that could be added even if we remove this FFS. |
| ZTE | OK |
| DOCOMO | Support. |
| Ericsson | Support. |
| Huawei, HiSilicon | Okay |
| Samsung | Support |
| QC | Support |

3.2	FG for SBFD random access operation based on RACH configuration Option 2

Following inputs are provided in contributions for the RAN1#121 meeting.

| [1] | Huawei, HiSilicon | SBFD random access operation related UE features According to the RAN1#120 agreement, two separate UE capabilities are introduced for the support of SBFD random access operation based on the two different RACH configuration options - Support of SBFD random access operation based on one single RACH configuration in RRC_IDLE/INACTIVE/CONNECTED mode (RACH configuration Option 1 with Alt 1-1) - Support of SBFD random access operation based on two separate RACH configurations, including one legacy RACH configuration and one additional RACH configuration in RRC_IDLE/INACTIVE/CONNECTED mode (RACH configuration Option 2) In RAN1#120, there was a discussion on whether the support of RACH configuration Option 1 or Option 2 should be included in the basic feature group and the following was agreed Agreement: Introduce following FGs for SBFD random access, one based on single RACH configuration (Option 1) and the other based on two RACH configurations (Option 2), and FFS on whether UE supporting SBFD random access needs to support at least Option 1 or Option 2. - Alt.1: UE supporting SBFD random access needs to support at least Option 1 - Alt.2: UE supporting SBFD random access needs to support at least Option 2 - Alt.3: UE supporting SBFD random access supports Option 1 or Option 2 (i.e., no default option). Considering that an SBFD-aware UE can perform random access based on legacy ROs as a legacy UE, it seems not critical to support a default option (Alt.3). One argument for Alt.1 is that UE may not implement either Option 1 or Option 2 if there is no default option. If this is the primary concern, another alternative is to support both Option 1 and Option 2 even through this contradicts with previous agreement. It should be noted that RACH configuration Option 1 and Option 2 have different application scenarios. It does not make sense to prioritize one use case over the other. Proposal 4: The support of RACH configuration Option 1 and Option 2 are defined as separate FGs, i.e., no default option. |
| --- | --- | --- |
| [2] | ZTE Corporation, Sanechips | UE features for RACH procedure In RAN1#120bis, the following agreements were reached on SBFD random access operation. 60-3; SBFD random access operation based on one single RACH configuration in RRC_IDLE/INACTIVE/CONNECTED mode; Support of SBFD random access operation based on one single RACH configuration in RRC_IDLE/INACTIVE/CONNECTED mode (RACH configuration Option 1) FFS: Other component(s) which may be separate FG; 60-1; YES; n/a; SBFD random access operation based on one single RACH configuration in RRC_IDLE/INACTIVE/CONNECTED mode is not supported; Per band; TDD only; n/a; n/a; Optional with capability signalling. 60-4; SBFD random access operation based on two separate RACH configurations, including one legacy RACH configuration and one additional RACH configuration in RRC_IDLE/INACTIVE/CONNECTED mode; Support of SBFD random access operation based on two separate RACH configurations, including one legacy RACH configuration and one additional RACH configuration in RRC_IDLE/INACTIVE/CONNECTED mode (RACH configuration Option 2) FFS: Other component(s) which may be separate FG; [60-1, 60-3]; YES; n/a; SBFD random access operation based on two separate RACH configurations in RRC_IDLE/INACTIVE/CONNECTED mode is not supported; Per band; TDD only; n/a; n/a; Optional with capability signalling. For FG 60-3 and 60-4, our views are summarized as below. - Regarding whether to add additional components, down-select from the following two alternatives. - Alt 1: - Add the following components in FG 60-3. - Support of reinterpretation of frequency offset of lowest RO - Support of separate power control parameters in additional-RO - Support of separate mapping of SSB and additional ROs, and separate determination of the association period and association pattern period. - Add the following components in FG 60-4. - Support of valid RO across SBFD and non-SBFD symbols - Alt 2: Keep FG 60-3 and FG 60-4 concise as it is. Neither add any additional components nor any additional separate FGs. - Note: If legacy FGs, e.g., Msg1 repetition, is reported together with FG 60-3/60-4, the conjunction of Msg1 repetition and SBFD is supported. - FG 60-3 is the prerequisite FG of FG 60-4. With above, we have the following proposal. Proposal 3: For FG 60-3 and FG 60-4, FG 60-3 is the prerequisite FG of FG 60-4, and down-select from the following two alternatives. - Regarding whether to add additional components, down-select from the following two alternatives. - Alt 1: - Add the following components in FG 60-3. - Support of reinterpretation of frequency offset of lowest RO - Support of separate power control parameters in additional-RO - Support of separate mapping of SSB and additional ROs, and separate determination of the association period and association pattern period. - Add the following components in FG 60-4. - Support of valid RO across SBFD and non-SBFD symbols - Alt 2: Keep FG 60-3 and FG 60-4 concise as it is. Neither add any additional components nor any additional separate FGs. - Note: If legacy FGs, e.g., Msg1 repetition, is reported together with FG 60-3/60-4, the conjunction of Msg1 repetition and SBFD is supported. |
| [3] | vivo |  |
| [4] | Nokia | FG 60-4 FG 60-4 for SBFD was agreed in RAN1#120 as shown in the Annex. In this section we discuss the remaining issues (highlighted in yellow) related FG 60-4. Concerning prerequisite feature groups, FG 60-1 and FG 60-3 could be prerequisite of FG 60-4. Although a UE is not expected to support both FGs 60-3 and 60-4, the UE (in case it cannot support both) should support FG 60-3. If we do not consider a default FG, there exists two UE types in network. In this case, network needs to either support both UE types at a same type, which significantly increases the complexity at the NW to handle all RO validation and SSB-to-RO mapping for both Options, or network simply picks one of the two Options, which leads to the waste of capability of the unpicked UE type in the network. Concerning the FFS on other components, the following component can be further added (assuming FG 60-3, and thus its components, are prerequisite for FG 60-4): - Support an RO spanning across SBFD and non-SBFD symbols. Proposal 4. For FG 60-4, - prerequisite feature groups: FG 60-1 and FG 60-3. - The following components should be added: - Support an RO spanning across SBFD and non-SBFD symbols. |
| [5] | MediaTek Inc. | Whether the support of valid RO across SBFD and non-SBFD symbols is separate FG There was a discussion on whether the support of valid RO across SBFD and non-SBFD symbols should be part of FG 60-4 or separate FG. Given that an RO across SBFD and non-SBFD symbols is only considered valid in RACH configuration Option 2, it is preferred to have support of valid RO across SBFD and non-SBFD symbols as a component of FG 60-4 (i.e., Alt 1). - Support of valid RO across SBFD and non-SBFD symbols is a component of of FG 60-4. |
| [6] | Spreadtrum, UNISOC | - FG 60-4 Prerequisite of the FG: 60-1. For FFS on whether UE supporting SBFD random access needs to support at least Option 1, Alt 3 is preferred. UE supporting SBFD random access supports Option 1 or Option 2 (i.e., no default option). Type of the FG: Per band Other components: - Support of separate power control parameters in additional-ROs. - Support of separate mapping of SSB and additional ROs, separate determination of the association period and association pattern period |
| [7] | Samsung | The following agreements were made in RAN1#120 [3]. <60-3, 60-4> Agreement (RAN1#120) Introduce following FGs for SBFD random access, one based on single RACH configuration (Option 1) and the other based on two RACH configurations (Option 2), and FFS on whether UE supporting SBFD random access needs to support at least Option 1 or Option 2. - Alt.1: UE supporting SBFD random access needs to support at least Option 1 - Alt.2: UE supporting SBFD random access needs to support at least Option 2 - Alt.3: UE supporting SBFD random access supports Option 1 or Option 2 (i.e., no default option) 60-3; SBFD random access operation based on one single RACH configuration in RRC_IDLE/INACTIVE/CONNECTED mode; Support of SBFD random access operation based on one single RACH configuration in RRC_IDLE/INACTIVE/CONNECTED mode (RACH configuration Option 1) FFS: Other component(s) which may be separate FG; X-1; YES; n/a; SBFD random access operation based on one single RACH configuration in RRC_IDLE/INACTIVE/CONNECTED mode is not supported; [Per Band]; TDD only; n/a; n/a; Optional with capability signalling. 60-4; SBFD random access operation based on two separate RACH configurations, including one legacy RACH configuration and one additional RACH configuration in RRC_IDLE/INACTIVE/CONNECTED mode; Support of SBFD random access operation based on two separate RACH configurations, including one legacy RACH configuration and one additional RACH configuration in RRC_IDLE/INACTIVE/CONNECTED mode (RACH configuration Option 2) FFS: Other component(s) which may be separate FG; [X-1, X-3]; YES; n/a; SBFD random access operation based on two separate RACH configurations in RRC_IDLE/INACTIVE/CONNECTED mode is not supported; [Per Band]; TDD only; n/a; n/a; Optional with capability signalling. Agreement: Adopt “Per band” type for FG 60-3. Agreement: Adopt “Per band” type for FG 60-4 In RAN1#120, 2 separate FGs were agreed for UE support of RACH configuration Option 1 or 2, respectively. For the gNB, the choice to implement and/or support one of RACH configuration Option 1 or 2 in the serving cell is not restricted. For certain use cases such as extended coverage/cell range using random access on SBFD resources, RACH configuration Option 2 is needed to configure long preamble formats such as B4/F0 when not possible on the legacy ROs. In other cases, PRACH repetition using either RACH configuration Options 1 or 2 can be sufficient. It is also possible, that random access on SBFD resources in the serving cell is enabled by the gNB only for UEs in RRC_CONNECTED mode. Alt.3, i.e., no default option, has significant risk for feature fragmentation. A UE supporting X-3 entering a cell indicating support for RACH config Option 2 cannot benefit from random access on SBFD resources. Similarly, when the gNB configures use of RACH config Option 1 in SIB1, but the UE is not required to support X-3, there is little incentive to implement such a feature. Our strong preference is therefore at least Alt.1, or alternatively, the new Alt. 4 “UE supporting SBFD random access supports Option 1 and Option 2. Proposal 6. Support the (new) Alt. 4 “UE supporting SBFD random access supports Option 1 and Option 2”. |
| [8] | Ofinno | FGs for random access operation SBFD random access based on a single RACH configuration (Option 1) applies across all RRC states, including CFRA initiated by PDCCH order. This option enables PRACH transmission without repetition and represents the baseline capability for SBFD random access. A UE supporting SBFD random access is required to support at least this option. SBFD random access based on two RACH configurations (Option 2) also applies across all RRC states, including CFRA initiated by PDCCH order. This option allows the use of two separate RACH configurations, for example across SBFD and non-SBFD symbols. Option 2 is defined under a separate FG from Option 1 and requires the UE to support both the SBFD basic FG and Option 1 for RACH configuration as prerequisites. This option also assumes PRACH transmission without repetition. In addition, support for valid ROs that span SBFD and non-SBFD symbols is defined as a separate FG under Option 2. A UE supporting this FG shall indicate support for Option 2-based configuration. Preamble repetition capability is configured independently of the RACH configuration option. If the UE supports only Option 1 or only Option 2, the repetition capability applies to the supported option. If the UE supports both options, the repetition capability applies to both configurations. Proposal 4. Update 60-3 and 60-4 random access features to include CFRA initiated by PDCCH order and introduce new features, as in table in Annex A. RAN1#120 agreed the following with selection between Alt 1 and Alt3. Introduce following FGs for SBFD random access, one based on single RACH configuration (Option 1) and the other based on two RACH configurations (Option 2), and FFS on whether UE supporting SBFD random access needs to support at least Option 1 or Option 2. - Alt.1: UE supporting SBFD random access needs to support at least Option 1 - Alt.2: UE supporting SBFD random access needs to support at least Option 2 - Alt.3: UE supporting SBFD random access supports Option 1 or Option 2 (i.e., no default option). We consider Option 1 should be considered as default. To maximize utilization of SBFD feature during initial access procedure, a default option seems beneficial. Thus, we support Alt. 1. Proposal 5. Adopt Alt 1 for SBFD random access. |
| [9] | CATT | FG for SBFD random access operation based on RACH configuration Option 2 - Regarding SBFD RACH configuration 2, there was a discussion on whether the support of valid RO across SBFD and non-SBFD symbols is separate FG. It is preferred that support of valid RO across SBFD and non-SBFD symbols as a separate FG. |
| [10] | CMCC | Regarding the additional components of FG 60-3 and 60-4: - Support of separate power control parameters for additional-RO and legacy-RO should be added into FG 60-3 - Support of PRACH transmission across SBFD symbols and non-SBFD symbols should be added into FG 60-4 |
| [11] | Xiaomi |  |
| [12] | OPPO | For FG 60-4 Regarding to whether to have no default option or keep Option-1 as default (i.e., 60-3 is a prerequisite of 60-4), we slightly prefer to have a default on Option-1 in order to ensure the UE to have an advanced random access functionality (comparing to the legacy) being enabled when roaming to different wireless markets, but are also open to having no default. Proposal 5: Treat SBFD random access Option 1 as the default, i.e., FG 60-4 takes {60-1,60-3} as prerequisite. |
| [13] | Apple | 60-3; RACH configuration Option 1 (Use one single RACH configuration and based on the existing parameters of the single RACH configuration); - Support additional-ROs include the ROs in SBFD symbols configured as downlink by tdd-UL-DL-ConfigurationCommon, and the ROs across SBFD symbols configured as downlink and SBFD symbols configured as flexible by tdd-UL-DL-ConfigurationCommon - Support of reinterpretation of frequency offset of lowest RO - Support separate power control parameters for PRACH transmission in SBFD symbols and non-SBFD symbols - Support of separate mapping of SSB and additional ROs, and separate determination of the association period and association pattern period - Support PRACH transmission in additional-ROs for SBFD-aware UEs in RRC_CONNECTED state and RRC_IDLE/INACTIVE state; 60-1; Yes; n/a; Preamble repetition for SBFD RA operation with RACH configuration Option 1 is not supported; Per band; TDD only; n/a; n/a; Optional with capability signalling. 60-4; RACH configuration Option 2 (Use two separate RACH configurations, including one legacy and one additional RACH configuration); - Support additional-ROs in non-SBFD symbols configured by additional RACH configuration - Support a valid RO starting from SBFD symbol and ending in non-SBFD symbol either in the same slot or across different slots - Support of separate mapping of SSB and additional ROs, and separate determination of the association period and association pattern period - Support PRACH transmission in additional-ROs for SBFD-aware UEs in RRC_CONNECTED state and RRC_IDLE/INACTIVE state; 60-1; Yes; n/a; Preamble repetition for SBFD RA operation with RACH configuration Option 2 is not supported; Per band; TDD only; n/a; n/a; Optional with capability signalling. 60-5; Preamble repetition for SBFD RA operation with RACH configuration Option1; - Determination of the set of Npreamblerep valid additional-ROs and the time period of the set(s) of Npreamblerep valid additional-ROs - Support separate configuration of rsrp-ThresholdMsg1-RepetitionNum2/4/8 for PRACH transmission with preamble repetitions within additional-ROs and PRACH transmission with preamble repetitions within legacy-ROs.; 54-1, 60-3; Yes; n/a; Per band; TDD only; n/a; n/a; Optional with capability signalling. 60-5a; Preamble repetition for SBFD RA operation with RACH configuration Option1; - Determination of the set of Npreamblerep valid additional-ROs and the time period of the set(s) of Npreamblerep valid additional-ROs - Support separate configuration of rsrp-ThresholdMsg1-RepetitionNum2/4/8 and msg1-RepetitionNum for PRACH transmission with preamble repetitions within additional-ROs and PRACH transmission with preamble repetitions within legacy-ROs; 54-1, 60-4; Yes; n/a; Per band; TDD only; n/a; n/a; Optional with capability signalling. 60-6; Msg.3 repetition in SBFD symbols; - Support of repetition of PUSCH transmission scheduled by RAR UL grant and DCI format 0_0 with CRC scrambled by TC-RNTI in SBFD symbols; 30-6, 60-3 or 60-4; Yes; n/a; Msg.3 repetition in SBFD symbols is not supported; Per band; TDD only; n/a; n/a; Optional with capability signalling. |
| [14] | Qualcomm Incorporated | UE features for supporting random-access in SBFD symbols For random-access in SBFD symbols, CBRA and CFRA using Type-1 random access procedure (4-step RACH) is supported. where two PRACH configuration options were introduced. PRACH configuration option (1) is based on using a single shared legacy RACH configuration where the additional-ROs in SBFD-DL symbols can be valid for SBFD-aware UE. The PRACH configuration options (2) is based on using two separate PRACH configurations, one legacy PRACH configuration indicating legacy-ROs and additional PRACH configuration indicating additional-ROs for random-access in SBFD symbols. For either PRACH configuration option 1 or PRACH configuration option 2, the ROs used for SBFD RA is referred as ‘additional-ROs’. In RAN1 #120, the following UE FGs was agreed for UE supporting Rel-19 SBFD random access with per-band granularity reporting [2]. Agreement: Adopt “Per band” type for FG 60-3. Agreement: Adopt “Per band” type for FG 60-4 Agreement: Introduce following FGs for SBFD random access, one based on single RACH configuration (Option 1) and the other based on two RACH configurations (Option 2), and FFS on whether UE supporting SBFD random access needs to support at least Option 1 or Option 2. - Alt.1: UE supporting SBFD random access needs to support at least Option 1 - Alt.2: UE supporting SBFD random access needs to support at least Option 2 - Alt.3: UE supporting SBFD random access supports Option 1 or Option 2 (i.e., no default option) X-3; SBFD random access operation based on one single RACH configuration in RRC_IDLE/INACTIVE/CONNECTED mode; Support of SBFD random access operation based on one single RACH configuration in RRC_IDLE/INACTIVE/CONNECTED mode (RACH configuration Option 1) FFS: Other component(s) which may be separate FG; X-1; YES; n/a; SBFD random access operation based on one single RACH configuration in RRC_IDLE/INACTIVE/CONNECTED mode is not supported; [Per Band]; TDD only; n/a; n/a; Optional with capability signalling. X-4; SBFD random access operation based on two separate RACH configurations, including one legacy RACH configuration and one additional RACH configuration in RRC_IDLE/INACTIVE/CONNECTED mode; Support of SBFD random access operation based on two separate RACH configurations, including one legacy RACH configuration and one additional RACH configuration in RRC_IDLE/INACTIVE/CONNECTED mode (RACH configuration Option 2) FFS: Other component(s) which may be separate FG; [X-1, X-3]; YES; n/a; SBFD random access operation based on two separate RACH configurations in RRC_IDLE/INACTIVE/CONNECTED mode is not supported; [Per Band]; TDD only; n/a; n/a; Optional with capability signalling.. Per RAN1 agreement, it is up to UE capability reporting to support either or both PRACH configuration. In addition, the support of PRACH transmission with preamble repetitions within additional-ROs of either PRACH configuration should be reported separately for PRACH configuration option 1 and PRACH configuration option 2. Finally, msg3 repetition in SBFD symbols based on configuration #1 should be reported separately. Proposal 7. Support the following list of UE capabilities for random access in SBFD symbols 60-4; SBFD random access operation based on two separate RACH configurations, including one legacy RACH configuration and one additional RACH configuration in RRC_IDLE/INACTIVE/CONNECTED mode; 1. Support of SBFD random access operation based on two separate RACH configurations, including one legacy RACH configuration and one additional RACH configuration in RRC_IDLE/INACTIVE/CONNECTED mode (RACH configuration Option 2) 2. Support of PRACH transmission without repetition within the valid-RO in SBFD symbols configured by the SBFD-specific PRACH configuration. 3. Support separate mapping of SSB to the additional ROs and separate determination of the association period and association pattern period 4. Support of CBRA switching procedures between PRACH transmission in additional-ROs (SBFD-dedicated PRACH configuration) and legacy-ROs (common PRACH configuration) 5. Support of explicit indication of RO-type for CFRA triggered by PDCCH order.; 60-1; SBFD random access operation based on two separate RACH configurations in RRC_IDLE/INACTIVE/CONNECTED mode is not supported; Per band; Optional with capability signalling. 60-4b; RO across SBFD and non-SBFD symbols; Support of PRACH transmission in RO across SBFD and non-SBFD symbols; 60-4; UE doesn’t support PRACH transmission in RO across SBFD and non-SBFD symbols; Per band; Optional with capability signalling. |
| [15] | NTT DOCOMO, INC. | RACH configuration option 1 and option 2 For SBFD random access operation, RACH configuration option 1 (i.e. use single PRACH configuration) and RACH configuration option 2 (i.e. use additional PRACH configuration for SBFD) are supported, and network would like to use either option 1 or 2 according to operation policy. If some UEs support only RACH configuration option 1 and some other UEs support only RACH configuration option 2, network may need to handle them separately e.g., in different cells to avoid complex operation. Fragmented support of RACH configuration options will cause practical issue for SBFD operation. Therefore, taking option 1 as the default option should be supported, i.e. Alt 1. For RACH configuration option 2, it is preferred that support of valid RO across SBFD and non-SBFD symbols is part of FG 60-4. Proposal 5: For FG 60-3 and FG 60-4 for SBFD random access operation: - UE supporting SBFD random access needs to support at least Option 1, i.e. 60-3 should be prerequisite of 60-4. - Support of valid RO across SBFD and non-SBFD symbols should be part of FG 60-4. In our understanding, there is no additional UE complexity to support Msg 3 PUSCH repetitions in SBFD symbols on top of FG 30-6 and FG 60-3/60-4. Msg 3 PUSCH repetitions in SBFD symbols can be supported by the support of FG 30-6 and FG 60-3/60-4. Therefore, Alt 1 is preferred. Proposal 7: Regarding Msg.3 repetition in SBFD symbols, clarify in FG 60-3/4 Note column that “Note: When a UE supports this FG and FG 30-6, the UE also support Msg.3 repetition in SBFD symbol”. |
| [16] | Ericsson | RAN1 #119 agreed to support Configuration 1 for Msg3 repetitions [6]: Agreement For an SBFD aware UE, if addition RO is selected for Msg1 transmission, only Configuration 1 is supported for Msg3 repetition.. In our view, the above agreement leaves no room for reinterpretation and should be adhered to. Hence, a UE supporting Msg3 repetitions must support at least Configuration 1 and only supporting Configuration 2 is not an alternative for SBFD RACH. Apart from Msg3 repetitions, in our view there is no additional need to support either Configuration 1 or Configuration 2 with respect to the PRACH preamble or RAR. RAN1 has also not discussed these configurations in any other context in AI 9.3.2. Both PRACH preamble transmission and RAR reception are unique operations during which time the UE has very little else to do. Hence, it should not be a problem for a UE to either transmit PRACH or receive RAR independently of whether it supports Configuration 1 or Configuration 2. Considering that RAN1 has already agreed to support Configuration 1 for Msg3 repetitions, and that Configuration 1 is the default configuration for all other signals and channels, it is reasonable that it is also the configuration that is assumed during the remainder of the RACH procedure. RAN1 agreements clearly states that Configuration 1 is used for Msg3 repetitions. Configurations 1 and 2 does not apply to PRACH or RAR. A UE supporting the single RACH configuration needs to support at least Configuration 1 (Alt. 1). RAN1 also discussed whether Option 1 or Option 2, if any, should be the default feature. In our view, the only thing Option 1 and Option 2 have in common is that they are both specified withing the SBFD WI. Apart from that, they address completely different use cases and even different UE types. Whereas Option 2 can be found in eMBB UEs, Option 1 addresses IoT and MTC UEs. This view is also supported by the agreement on the two options where it is clearly stated that a “UE is not required to support both options.” Based on that, it is not possible to claim that one option should have precedence over the other. Option 1 and Option 2 target different UE types and therefore, per agreement, a UE is not expected to support both options. Support separate FGs for the single and additional RACH configurations. Another point up for discussion is how to handle the network configuration where a valid RO spans both SBFD and non-SBFD symbols. For that particular use case, to perform initial access with an extended range, it makes little sense to have this component as a separate UE-feature. In that case, the SBFD-aware UE supporting Option 2 would still not be able to connect to the network due to the network being configured with ROs across both SBFD and non-SBFD symbols. That kind of functionality mismatch should surely be avoided, and since RAN1 agreed that this feature is configurable on the network side, it should be mandatory on the UE side, for UEs supporting Option 2. Support for ROs across both SBFD and non-SBFD symbols is a network configuration but should not be a separate UE-capability to avoid capability mismatch. Support of a valid RO across SBFD and non-SBFD symbols is a component of the FG for SBFD random access based on the additional RACH configurations (Option 2) (Alt. 1). |

The following shows the latest status for the relevant FG(s).

| 60-4 | SBFD random access operation based on two separate RACH configurations, including one legacy RACH configuration and one additional RACH configuration in RRC_IDLE/INACTIVE/CONNECTED mode | Support of SBFD random access operation based on two separate RACH configurations, including one legacy RACH configuration and one additional RACH configuration in RRC_IDLE/INACTIVE/CONNECTED mode (RACH configuration Option 2) FFS: Other component(s) which may be separate FG | [60-1, 60-3] | YES | n/a | SBFD random access operation based on two separate RACH configurations in RRC_IDLE/INACTIVE/CONNECTED mode is not supported | Per band | TDD only | n/a | n/a |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Summary of companies’ views and discussion

Whether RACH configuration option 1 is default option? - Alt.1: UE supporting SBFD random access needs to support at least Option 1 - ZTE, Offino, Nokia, OPPO, Xiaomi, DOCOMO, - Alt.3: UE supporting SBFD random access supports Option 1 or Option 2 (i.e., no default option) - Vivo, SPRD/UNISOC, CATT, MediaTek, CMCC, HW/Hisi, Ericsson, Apple, Qualcomm, - Alt.4: UE supporting SBFD random access supports both Option 1 and Option 2 - Samsung, Whether the support of valid RO across SBFD and non-SBFD symbols is reported as a separate FG, or component of basic FG? - Separate FG on valid RO across SBFD and non-SBFD symbols: vivo, SPRD/UNISOC, CATT, Offino, Xiaomi, Qualcomm - Support of valid RO across SBFD and non-SBFD symbols is component of FG 60-4: ZTE, Nokia, MediaTek, CMCC, Ericsson, Apple, DOCOMO Components of the FG 60-4 - Support of separate power control parameters in additional-ROs: Nokia, SPRD/UNISOC - Support of separate mapping of SSB and additional ROs: Nokia, SPRD/UNISOC, Apple, Qualcomm - Support separate determination of the association period and association pattern period: Nokia, SPRD/UNISOC, Qualcomm - Support of Msg 3 repetitions in SBFD symbols: HW, Nokia. DOCOMO, - Support of CFRA initiated by PDCCH order: Offino - Support additional-ROs in non-SBFD symbols configured by additional RACH configuration: Apple - Support PRACH transmission in additional-ROs for SBFD-aware UEs in RRC_CONNECTED state and RRC_IDLE/INACTIVE state: Apple - Support repetitions in SBFD symbols: Apple - Support of PRACH transmission without repetition within the valid-RO in SBFD symbols configured by the SBFD-specific PRACH configuration: Qualcomm - Support of CBRA switching procedures between PRACH transmission in additional-ROs (SBFD-dedicated PRACH configuration) and legacy-ROs (common PRACH configuration): Qualcomm - Support of explicit indication of RO-type (legacy-RO or additional-RO) for CFRA triggered by PDCCH order: Qualcomm Others: - Explicitly capture only SBFD Configuration 1 for PRACH preamble transmission and RAR: E///

Proposal 3-2-1:

Add a component ‘support of valid RO across SBFD and non-SBFD symbols’ in FG 60-4.

| Company | Comment |
| --- | --- |
| ZTE | Support |
| DOCOMO | Support the proposal. |
| Ericsson | Support. |
| Huawei, HiSilicon | Support |
| Samsung | Support |
| QC | We prefer a separate UE FG. |

Question 3-2-2:

Companies are encouraged to share views on whether FG 60-3 should be a prerequisite FG for FG 60-4 or not (i.e., whether FG 60-3 should be default for SBFD RA operation or not)

- Alt-1: FG 60-3 should be a prerequisite FG for FG 60-4 (UE supporting SBFD random access needs to support at least Option 1)

- Alt-2: FG 60-3 should NOT be a prerequisite FG for FG 60-4 (UE supporting SBFD random access supports Option 1 or Option 2 (i.e., no default option))

| Company | Comment |
| --- | --- |
| Moderator | Although a number of companies prefer Alt-1, it seems that the following WA made in 9.3.2 implies Alt-2. Working Assumption For SBFD-aware UEs in RRC CONNECTED state, both RACH configuration Option 1 with Alt 1-1 (i.e., use one single RACH configuration, and only based on the existing parameters of the single RACH configuration) and RACH configuration Option 2 (i.e., Use two separate RACH configurations, including one legacy RACH configuration and one additional RACH configuration) are supported. Enabling both options at the same time for a UE is not supported. - For Option 1 with Alt 1-1, FFS whether/how to reinterpret msg1-FrequencyStart in rach-ConfigCommon, RO validation rules and SSB-RO mapping rules, etc. - For Option 2, FFS the RO validation rules, SSB-RO mapping rules, whether all the parameters currently in rach-ConfigCommon are necessary to be included in the additional RACH configuration, etc. UE is not required to support both options.. Meanwhile there is also the following conclusion: Conclusion There is no consensus in RAN1 to support the following in Rel-19: - Enabling both RACH configuration Option 1 and Option 2 for SBFD random access operation in a cell from network side.. It seems configuring both in a cell is to be prohibited even from NW perspective. It might imply that in a SBFD cell, even when both UEs supporting Option 1 and Option 2 are there, only either ones could enjoy the benefit of this feature? With the above in mind, I would like to get a bit more inputs for this issue to see how we should proceed. |
| ZTE | The above WA and Conclusion from moderator do not preclude Alt.1, which intends to define a default UE capability, i.e., Option 1. If not all UEs are able to support Option 2, this allows NW to configure Option 1 to let all UEs enjoy RACH in SBFD symbols. |
| DOCOMO | We prefer Alt-1. If some UEs support only RACH configuration option 1 and some other UEs support only RACH configuration option 2, network may need to handle them sepa-rately e.g., in different cells to avoid complex operation. Fragmented support of RACH configuration options will cause practical issue for SBFD operation. Therefore, we strongly prefer to define option 1 as default option to avoid fragmented support and complex SBFD operation. |
| Ericsson | Support Alt. 2. Option 1 and Option 2 address very different network types. Option 1 is targeting Industrial and Enterprise networks whereas Option 2 is targeting larger, outdoor cells where range may be an issue. Different UE types will cater to the different networks, why, in practice, there is very little risk of overlap with a certain UE ending up in the other type of network. If there is, legacy RACH can still be used for a UE supporting the other Option. |
| Huawei, HiSilicon | Considering that an SBFD-aware UE can perform random access based on legacy ROs as a legacy UE, it seems not critical to support a default option. Even we go with Alt.1, it does not mandate FG 60-3 to be implemented for SBFD-aware UE since it is not in the basic FG. If this is the primary concern is that only one set of UEs can benefit from SBFD RACH since the NW will only operate with either Option 1 or Option 2, another alternative is to support both Option 1 and Option 2 even through this contradicts with previous agreement. |
| Samsung | Our preference is that the UE should support both FG 60-3 and 60-4, i.e., must support both RACH config Option 1 and 2. We can support Alt.2 (no default option) if majority view. |
| QC | Support Alt. 2, per RAN1 agreement. |

Proposal 3-2-3:

Add the following components in FG 60-4:

- Support of separate power control parameters in additional-ROs

- Support of separate mapping of SSB and additional ROs

- Support separate determination of the association period and association pattern period

| Company | Comment |
| --- | --- |
| ZTE | Similar comment as for Proposal 3-1-1. |
| DOCOMO | We think “separate power control parameters in additional-ROs” is not necessary to be included as component of FG 60-4. As separate RACH configuration is introduced, separate power control parameters is natural for FG 60-4. |
| Ericsson | Support. |
| Huawei, HiSilicon | As we commented before, we don't see the need to capture every detailed change into the FG, especially when UE does not report the candidate value for the feature component anyway. |
| Samsung | Similar comment as for Proposal 3-1-1. |
| QC | Support |

3.3	FG for preamble repetition within additional ROs

Following inputs are provided in contributions for the RAN1#121 meeting.

| [1] | Huawei, HiSilicon | Preamble repetition within additional-ROs For preamble repetition within additional-ROs, two separate UE capabilities can be defined for RACH configuration Option 1 and RACH configuration Option 2 respectively. For preamble repetition within additional-ROs based on RACH configuration Option 1, the following components are included - Determination of the set of Npreamblerep valid additional-ROs and the time period of the set(s) of Npreamblerep valid additional-ROs - For RACH configuration Option 1, support separate configuration of rsrp-ThresholdMsg1-RepetitionNum2/4/8 for PRACH transmission with preamble repetitions within additional-ROs and PRACH transmission with preamble repetitions within legacy-ROs. For preamble repetition within additional-ROs based on RACH configuration Option 2, the following components are included - Determination of the set of Npreamblerep valid additional-ROs and the time period of the set(s) of Npreamblerep valid additional-ROs - For RACH configuration Option 2, support separate configuration of rsrp-ThresholdMsg1-RepetitionNum2/4/8 and msg1-RepetitionNum for PRACH transmission with preamble repetitions within additional-ROs and PRACH transmission with preamble repetitions within legacy-ROs. |
| --- | --- | --- |
| [2] | ZTE Corporation, Sanechips |  |
| [3] | vivo | Proposal 7: Support to introduce a separate FG for PRACH repetition in additional ROs. - This separate FGs is common for RACH configuration Option 1 and Option 2 |
| [4] | Nokia | FG 60-5 In RAN1#120 the following proposal from Moderator was discussed: Proposal 3-3: Introduce following FG (as FFS) for PRACH repetition in additional ROs, and FFS on whether separate FGs for RACH configuration Option 1 and Option 2 are necessary. - Alt.1: separate FGs for RACH configuration Option 1 and Option 2 are NOT necessary - Alt.2: separate FGs for RACH configuration Option 1 and Option 2 are necessary - Alt.3: If a UE reports legacy Rel-18 FG for PRACH repetition, and also reports support of FG X-3/4, it means the UE supports RACH repetition in SBFD operation for Option 1/2 respectively. X-5; Preamble repetition within additional-ROs; 1. Determination of the set of Npreamblerep valid additional-ROs and the time period of the set(s) of Npreamblerep valid additional-ROs [2. For RACH configuration Option 1, support separate configuration of rsrp-ThresholdMsg1-RepetitionNum2/4/8 for PRACH transmission with preamble repetitions within additional-ROs and PRACH transmission with preamble repetitions within legacy-ROs.] [3. For RACH configuration Option 2, support separate configuration of rsrp-ThresholdMsg1-RepetitionNum2/4/8 and msg1-RepetitionNum for PRACH transmission with preamble repetitions within additional-ROs and PRACH transmission with preamble repetitions within legacy-ROs.] FFS: separate FGs for RACH configuration Option 1 and Option 2 are necessary; [X-3 or X-4, 54-1]; YES; n/a; Preamble repetition within additional-ROs is not supported; [Per Band]; TDD only; n/a; n/a; Optional with capability signalling.. In either Option 1 or Option 2, Msg1 repetition is performed in only one RO type. Therefore, from Msg1 repetition functionality point of view, there’s no difference between supporting Msg1 repetition for Option1 and Option 2. Proposal 5. Introduce a separate FG for PRACH repetition in additional ROs. Separate FGs of PRACH repetition for RACH configuration Option 1 and Option 2 are NOT necessary. |
| [5] | MediaTek Inc. | Whether to support separate FGs for PRACH repetition in additional ROs for RACH configuration Option 1 and Option 2 There was a discussion on whether to introduce separate FG for PRACH repetition in additional ROs for RACH configuration Option 1 and Option 2. According to RAN1 agreement from RAN1#117, “enabling both options at the same time for a UE is not supported”. As such, a single FG can be supported, which applies to the PRACH configuration option which is enabled at any given time. If UE supports the FG for PRACH repetition in additional ROs and RACH configuration Option 1 is enabled, UE supports PRACH repetition in additional ROs based on RACH configuration Option 1. On the other hand, if UE supports the FG for PRACH repetition in additional ROs and RACH configuration Option 2 is enabled, UE supports PRACH repetition in additional ROs based on RACH configuration Option 2. - Support of separate FGs for PRACH repetition in additional ROs for RACH configuration Option 1 and Option 2 is NOT necessary. |
| [6] | Spreadtrum, UNISOC | - Other FG: PRACH repetitions Two FGs: Separate FGs for PRACH repetition in additional RO for RACH configuration option 1 and option 2 Type of the FGs: Per band Prerequisite of the FG of RACH configuration Option 1: 60-3 and 54-1 Prerequisite of the FG of RACH configuration Option 2: 60-4 and 54-1 |
| [7] | Samsung | Several more FL proposals for random access on SBFD resources were discussed in RAN1#120. Proposal 3-3 (RAN1#120): Introduce following FG (as FFS) for PRACH repetition in additional ROs, and FFS on whether separate FGs for RACH configuration Option 1 and Option 2 are necessary. - Alt.1: separate FGs for RACH configuration Option 1 and Option 2 are NOT necessary - Alt.2: separate FGs for RACH configuration Option 1 and Option 2 are necessary - Alt.3: If a UE reports legacy Rel-18 FG for PRACH repetition, and also reports support of FG X-3/4, it means the UE supports RACH repetition in SBFD operation for Option 1/2 respectively. X-5; Preamble repetition within additional-ROs; 1. Determination of the set of Npreamblerep valid additional-ROs and the time period of the set(s) of Npreamblerep valid additional-ROs [2. For RACH configuration Option 1, support separate configuration of rsrp-ThresholdMsg1-RepetitionNum2/4/8 for PRACH transmission with preamble repetitions within additional-ROs and PRACH transmission with preamble repetitions within legacy-ROs.] [3. For RACH configuration Option 2, support separate configuration of rsrp-ThresholdMsg1-RepetitionNum2/4/8 and msg1-RepetitionNum for PRACH transmission with preamble repetitions within additional-ROs and PRACH transmission with preamble repetitions within legacy-ROs.] FFS: separate FGs for RACH configuration Option 1 and Option 2 are necessary; [X-3 or X-4, 54-1]; YES; n/a; Preamble repetition within additional-ROs is not supported; [Per Band]; TDD only; n/a; n/a; Optional with capability signalling. Proposal 3-4 (RAN1#120): Introduce following FG (as FFS) for Msg.3 repetition in SBFD symbols. X-5; Msg.3 repetition in SBFD symbols; Support of repetition of PUSCH transmission scheduled by RAR UL grant and DCI format 0_0 with CRC scrambled by TC-RNTI in SBFD symbols; [X-1 or X-3 or X-4, 30-6]; YES; n/a; Msg.3 repetition in SBFD symbols is not supported; [Per Band]; TDD only; n/a; n/a; Optional with capability signalling.. Regarding proposal 3-3 (PRACH repetition), we prefer Alt.2 “separate FGs for RACH configuration Option 1 and Option 2” to indicate support for PRACH repetition. PRACH repetition based on Rel-18 principles is already readily supported for the case of RACH configuration Option 2. PRACH repetition with RACH configuration Option 1 is a delta which the UE implementation may or may not choose to support. We support proposal 3-4 (PUSCH msg.3 repetition”). Proposal 7. For proposal 3-3 (PRACH repetition) support Alt.2: separate FGs for RACH configuration Option 1 and Option 2 are necessary. Proposal 8. Support/introduce the FG as by proposal 3-4 (PUSCH msg.3 repetition). |
| [8] | Ofinno |  |
| [9] | CATT | FG for preamble repetition within additional ROs For PRACH repetition within additional-ROs, whether single FG or separate FGs for RACH configuration option 1 and option 2 was discussed. Considering that separate UE capabilities are defined for RACH configuration Option 1 and RACH configuration Option 2, two separate UE capabilities can be defined for PRACH repetition within additional ROs for RACH configuration Option 1 and RACH configuration Option 2 respectively. |
| [10] | CMCC | Then some additional UE capabilities can be defined based on the previous RAN1 agreements which don’t need to differentiate separate FGs for RACH configuration Option 1 and Option 2, including: - Support of PRACH repetition - Support of Msg3 repetition |
| [11] | Xiaomi | Index; Feature group; Components; Prerequisite feature groups; Need for the gNB to know if the feature is supported; Applicable to the capability signalling exchange between UEs (Sidelink WI only)”.; Consequence if the feature is not supported by the UE; Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC); Need of FDD/TDD differentiation; Need of FR1/FR2 differentiation; Capability interpretation for mixture of FDD/TDD and/or FR1/FR2; Note; Mandatory/Optional. X-4; SBFD random access operation based on two separate RACH configurations, including one legacy RACH configuration and one additional RACH configuration in RRC_IDLE/INACTIVE/CONNECTED mode; Support of SBFD random access operation based on two separate RACH configurations, including one legacy RACH configuration and one additional RACH configuration in RRC_IDLE/INACTIVE/CONNECTED mode (RACH configuration Option 2); X-1, X-3; YES; n/a; SBFD random access operation based on two separate RACH configurations in RRC_IDLE/INACTIVE/CONNECTED mode is not supported; Per Band; TDD only; n/a; n/a; Optional with capability signalling. X-4-1; SBFD random access operation based on two separate RACH configurations with valid RO across SBFD and non-SBFD symbols; Support of SBFD random access operation based on two separate RACH configurations with or without valid RO starting from SBFD symbol and ending in non-SBFD symbol either in the same slot or across different slots; X-4; YES; n/a; SBFD random access operation based on two separate RACH configurations with valid RO starting from SBFD symbol and ending in non-SBFD symbol either in the same slot or across different slots is not supported.; Per Band; TDD only; n/a; n/a; Optional with capability signalling. X-5; Support of preamble repetitions in SBFD symbols; 1. Support of preamble repetitions within additional-ROs 2. Support of preamble repetitions within legacy-ROs; 60-2-1; YES; n/a; preamble repetitions in SBFD symbols are not supported; Per band; TDD only; n/a; n/a; Optional with capability signalling. X-6; Support of repetition of PUSCH transmission scheduled by RAR UL grant and DCI format 0_0 with CRC scrambled by TC-RNTI in SBFD symbols; Support of repetition of PUSCH transmission scheduled by RAR UL grant and DCI format 0_0 with CRC scrambled by TC-RNTI with Configuration 1; 30-6 60-2-1; YES; n/a; Msg3 PUSCH repetitions in SBFD symbols are not supported; Per band; TDD only; n/a; n/a; Optional with capability signalling. |
| [12] | OPPO |  |
| [13] | Apple | 60-5; Preamble repetition for SBFD RA operation with RACH configuration Option1; - Determination of the set of Npreamblerep valid additional-ROs and the time period of the set(s) of Npreamblerep valid additional-ROs - Support separate configuration of rsrp-ThresholdMsg1-RepetitionNum2/4/8 for PRACH transmission with preamble repetitions within additional-ROs and PRACH transmission with preamble repetitions within legacy-ROs.; 54-1, 60-3; Yes; n/a; Per band; TDD only; n/a; n/a; Optional with capability signalling. 60-5a; Preamble repetition for SBFD RA operation with RACH configuration Option1; - Determination of the set of Npreamblerep valid additional-ROs and the time period of the set(s) of Npreamblerep valid additional-ROs - Support separate configuration of rsrp-ThresholdMsg1-RepetitionNum2/4/8 and msg1-RepetitionNum for PRACH transmission with preamble repetitions within additional-ROs and PRACH transmission with preamble repetitions within legacy-ROs; 54-1, 60-4; Yes; n/a; Per band; TDD only; n/a; n/a; Optional with capability signalling. |
| [14] | Qualcomm Incorporated | Proposal 7. Support the following list of UE capabilities for random access in SBFD symbols 60-3; SBFD random access operation based on one single RACH configuration in RRC_IDLE/INACTIVE/CONNECTED mode; 1. Support of SBFD random access operation based on one single RACH configuration in RRC_IDLE/INACTIVE/CONNECTED mode (RACH configuration Option 1) 2. Support of PRACH transmission without repetition within additional valid-RO in SBFD symbols configured as downlink by tdd-UL-DL-ConfigurationCommon 3. Support of separate PC parameters for PRACH transmission in the additional-ROs. 4. Separate mapping of SSB to the additional ROs and separate determination of the association period and association pattern period 5. Support of CBRA switching procedures between PRACH transmission in additional-ROs and legacy-ROs. 6. Support of explicit indication of RO-type (legacy-RO or additional-RO) for CFRA by PDCCH order.; 60-1; SBFD random access operation based on one single RACH configuration in RRC_IDLE/INACTIVE/CONNECTED mode is not supported; Per band; Optional with capability signalling. 60-3a; PRACH repetition in additional-RO of PRACH configuration option 1; 1. Support of PRACH transmission with preamble repetition in the additional ROs using the same number of repetitions {2,4,8} for PRACH transmission with preamble repetitions within legacy-ROs. 2. Support separate configuration of RSRP thresholds for PRACH transmission with preamble repetitions within additional-ROs; 60-3; UE doesn’t support PRACH transmission with preamble repetition in additional ROs based on legacy PRACH configuration (PRACH configuration option 1); Per band; Optional with capability signalling. 60-4; SBFD random access operation based on two separate RACH configurations, including one legacy RACH configuration and one additional RACH configuration in RRC_IDLE/INACTIVE/CONNECTED mode; 1. Support of SBFD random access operation based on two separate RACH configurations, including one legacy RACH configuration and one additional RACH configuration in RRC_IDLE/INACTIVE/CONNECTED mode (RACH configuration Option 2) 2. Support of PRACH transmission without repetition within the valid-RO in SBFD symbols configured by the SBFD-specific PRACH configuration. 3. Support separate mapping of SSB to the additional ROs and separate determination of the association period and association pattern period 4. Support of CBRA switching procedures between PRACH transmission in additional-ROs (SBFD-dedicated PRACH configuration) and legacy-ROs (common PRACH configuration) 5. Support of explicit indication of RO-type for CFRA triggered by PDCCH order.; 60-1; SBFD random access operation based on two separate RACH configurations in RRC_IDLE/INACTIVE/CONNECTED mode is not supported; Per band; Optional with capability signalling. 60-4a; PRACH repetition in additional-RO of PRACH configuration option 2; 1. Support of PRACH transmission with preamble repetition in the additional ROs of SBFD PRACH configuration (PRACH configuration option 2). 2. support separate configuration of RSRP thresholds and repetition numbers {2,4,8} for PRACH transmission with preamble repetitions within additional-ROs; 60-4; UE doesn’t support PRACH transmission with preamble repetition in additional ROs based on SBFD PRACH configuration (PRACH configuration option 2).; Per band; Optional with capability signalling. |
| [15] | NTT DOCOMO, INC. | We think that single separate FG for PRACH repetition in additional ROs is sufficient. If UE supports this FG and FG 60-3, the UE supports PRACH repetition in additional ROs based on RACH configuration option 1. If UE supports this FG and FG 60-4, the UE supports PRACH repetition in additional ROs based on RACH configuration option 2. Proposal 6: Support of PRACH repetition in additional ROs should be single separate FG (e.g., as FG 60-5), without further separation between RACH configuration option 1 and option 2. |
| [16] | Ericsson | A common design aspect of the single RACH configuration (Option 1) is that it should not introduce any additional parameters. In our view, that is also the case for UE-features. Hence, for Option 1, PRACH preamble repetitions and Msg3 repetitions can be determined by the corresponding legacy feature in FG 30-6. Additionally, there is in our view no technical merit to why an SBFD-aware UE also supporting preamble repetitions or Msg3 repetitions in legacy RACH should not be able to also support it for Option 1. In contrast, for the additional RACH configuration (Option 2), a separate RACH configuration will be provided. However, the fundamental capability for the UE to support repetitions within the additional configuration should not differ. Hence, also for this FG, we propose to make it dependent on the legacy capability for repetitions. - PRACH preamble repetitions and Msg3 repetitions for both the single and additional RACH configurations are determined by the corresponding legacy UE-features for PRACH preamble and Msg3 repetitions. |

Summary of companies’ views and discussion

FG structure - Single FG for PRACH repetition in additional RO for both configuration option 1 and option 2: vivo, Offino, Nokia, MediaTek, CMCC, Xiaomi, DOCOMO - Separate two FGs for PRACH repetition in additional RO for each of configuration option 1 and option 2: SPRD/UNISOC, CATT, HW/Hisi, Samsung, Apple, QC, - No separate FG for preamble repetition (support is determined by the legacy 54-1): Ericsson Components: - Option 1: - Determination of the set of Npreamblerep valid additional-ROs and the time period of the set(s) of Npreamblerep valid additional-Ros: HW/Hisi (Option 1), Apple, - For RACH configuration Option 1, support separate configuration of rsrp-ThresholdMsg1-RepetitionNum2/4/8 for PRACH transmission with preamble repetitions within additional-ROs and PRACH transmission with preamble repetitions within legacy-Ros: HW/Hisi, Apple, QC, - Option 2: - Determination of the set of Npreamblerep valid additional-ROs and the time period of the set(s) of Npreamblerep valid additional-Ros: HW/Hisi (Option 2), Apple, - For RACH configuration Option 2, support separate configuration of rsrp-ThresholdMsg1-RepetitionNum2/4/8 and msg1-RepetitionNum for PRACH transmission with preamble repetitions within additional-ROs and PRACH transmission with preamble repetitions within legacy-ROs.: HW/Hisi, Apple, QC, Type of the FG(s) - Per band: SPRD/UNISOC, CATT, Offino, MediaTek, CMCC, Xiaomi, Apple, DOCOMO, Qualcomm Prerequisite of the FG(s) - X-3 (RACH configuration option 1) or X-4 (RACH configuration option 2): SPRD/UNISOC, CATT, Offino, CMCC, HW/Hisi, Apple, Qualcomm - X-1 (basic FG for SBFD): CATT - 54-1 (PRACH repetition): SPRD/UNISOC, CMCC, Apple - 60-2-1: Xiaomi

Proposal 3-3-1:

Introduce following FG for PRACH repetition in additional ROs, and FFS on whether separate FGs for RACH configuration Option 1 and Option 2 are necessary.

- Alt.1: separate FGs for RACH configuration Option 1 and Option 2 are NOT necessary

- Alt.2: separate FGs for RACH configuration Option 1 and Option 2 are necessary

| 60-5 | Preamble repetition within additional-ROs | 1. Determination of the set of Npreamblerep valid additional-ROs and the time period of the set(s) of Npreamblerep valid additional-ROs [2. For RACH configuration Option 1, support separate configuration of rsrp-ThresholdMsg1-RepetitionNum2/4/8 for PRACH transmission with preamble repetitions within additional-ROs and PRACH transmission with preamble repetitions within legacy-ROs.] [3. For RACH configuration Option 2, support separate configuration of rsrp-ThresholdMsg1-RepetitionNum2/4/8 and msg1-RepetitionNum for PRACH transmission with preamble repetitions within additional-ROs and PRACH transmission with preamble repetitions within legacy-ROs.] FFS: separate FGs for RACH configuration Option 1 and Option 2 are necessary | [60-3 or 60-4, 54-1] | YES | n/a | Preamble repetition within additional-ROs is not supported | [Per Band] | TDD only | n/a | n/a |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comment |
| --- | --- |
| ZTE | Prefer not to introduce a new FG for PRACH repetition in additional ROs. |
| DOCOMO | We are ok with the proposal and prefer Alt.1. Support of FG60-3 and/or 60-4 can represent support for option 1 and/or option 2. |
| Ericsson | Prefer Alt. 1. UEs are most likely anyway going to support one of the two Options, hence, separate FGs are not needed. Even if the UE is supporting both Options, supporting PRACH repetitions in both should not be an overly complex implementation. |
| Huawei, HiSilicon | We are fine either way. Alt.2 is a cleaner approach if RACH option 1 and Option 2 are separate UE features. |
| Samsung | Alt.2 |
| QC | Support Alt.2. It is double complexity, timeline to support PRACH repetitions in legacy and additional-ROs. We can’t assume UEs support 54-1 should be able by default to support 60-3 and/or 60-4. |

3.4	FG for Msg3 repetitions in SBFD symbols

Following inputs are provided in contributions for the RAN1#121 meeting.

| [1] | Huawei, HiSilicon | Msg3 repetition in SBFD symbols For Msg3 repetition in SBFD symbols, there was a discussion on whether to introduce to separate FG. Our view is that it can be included into the basic FG. Note that the support of Configuration 1 (UL transmissions or DL receptions across SBFD symbols and non-SBFD symbols in different slots, where the transmissions or receptions are restricted to SBFD symbols or non-SBFD symbols) is included is the basic FG. A UE supporting Msg3 repetition (FG 30-6) and the basic SBFD FG should be have the capability to support Msg3 repetition in SBFD symbols without much additional complexity. There is no need to introduce a separate UE capability for this. Proposal 5: The support of Msg3 repetition in SBFD symbols is not defined as a separate FG. |
| --- | --- | --- |
| [2] | ZTE Corporation, Sanechips |  |
| [3] | vivo |  |
| [4] | Nokia |  |
| [5] | MediaTek Inc. |  |
| [6] | Spreadtrum, UNISOC | - Other FG: MSG3 repetitions in SBFD symbols One FG for Msg.3 repetition in SBFD symbols Type of the FG: Per band Prerequisite of the basic FG: 60-3/60-4 and 30-6 |
| [7] | Samsung | Several more FL proposals for random access on SBFD resources were discussed in RAN1#120. Proposal 3-3 (RAN1#120): Introduce following FG (as FFS) for PRACH repetition in additional ROs, and FFS on whether separate FGs for RACH configuration Option 1 and Option 2 are necessary. - Alt.1: separate FGs for RACH configuration Option 1 and Option 2 are NOT necessary - Alt.2: separate FGs for RACH configuration Option 1 and Option 2 are necessary - Alt.3: If a UE reports legacy Rel-18 FG for PRACH repetition, and also reports support of FG X-3/4, it means the UE supports RACH repetition in SBFD operation for Option 1/2 respectively. X-5; Preamble repetition within additional-ROs; 1. Determination of the set of Npreamblerep valid additional-ROs and the time period of the set(s) of Npreamblerep valid additional-ROs [2. For RACH configuration Option 1, support separate configuration of rsrp-ThresholdMsg1-RepetitionNum2/4/8 for PRACH transmission with preamble repetitions within additional-ROs and PRACH transmission with preamble repetitions within legacy-ROs.] [3. For RACH configuration Option 2, support separate configuration of rsrp-ThresholdMsg1-RepetitionNum2/4/8 and msg1-RepetitionNum for PRACH transmission with preamble repetitions within additional-ROs and PRACH transmission with preamble repetitions within legacy-ROs.] FFS: separate FGs for RACH configuration Option 1 and Option 2 are necessary; [X-3 or X-4, 54-1]; YES; n/a; Preamble repetition within additional-ROs is not supported; [Per Band]; TDD only; n/a; n/a; Optional with capability signalling. Proposal 3-4 (RAN1#120): Introduce following FG (as FFS) for Msg.3 repetition in SBFD symbols. X-5; Msg.3 repetition in SBFD symbols; Support of repetition of PUSCH transmission scheduled by RAR UL grant and DCI format 0_0 with CRC scrambled by TC-RNTI in SBFD symbols; [X-1 or X-3 or X-4, 30-6]; YES; n/a; Msg.3 repetition in SBFD symbols is not supported; [Per Band]; TDD only; n/a; n/a; Optional with capability signalling.. Regarding proposal 3-3 (PRACH repetition), we prefer Alt.2 “separate FGs for RACH configuration Option 1 and Option 2” to indicate support for PRACH repetition. PRACH repetition based on Rel-18 principles is already readily supported for the case of RACH configuration Option 2. PRACH repetition with RACH configuration Option 1 is a delta which the UE implementation may or may not choose to support. We support proposal 3-4 (PUSCH msg.3 repetition”). Proposal 7. For proposal 3-3 (PRACH repetition) support Alt.2: separate FGs for RACH configuration Option 1 and Option 2 are necessary. Proposal 8. Support/introduce the FG as by proposal 3-4 (PUSCH msg.3 repetition). |
| [8] | Ofinno |  |
| [9] | CATT | Regarding Msg3 repetitions in SBFD symbols, a separate FG can be defined. |
| [10] | CMCC | Then some additional UE capabilities can be defined based on the previous RAN1 agreements which don’t need to differentiate separate FGs for RACH configuration Option 1 and Option 2, including: - Support of PRACH repetition - Support of Msg3 repetition |
| [11] | Xiaomi | Index; Feature group; Components; Prerequisite feature groups; Need for the gNB to know if the feature is supported; Applicable to the capability signalling exchange between UEs (Sidelink WI only)”.; Consequence if the feature is not supported by the UE; Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC); Need of FDD/TDD differentiation; Need of FR1/FR2 differentiation; Capability interpretation for mixture of FDD/TDD and/or FR1/FR2; Note; Mandatory/Optional. X-4; SBFD random access operation based on two separate RACH configurations, including one legacy RACH configuration and one additional RACH configuration in RRC_IDLE/INACTIVE/CONNECTED mode; Support of SBFD random access operation based on two separate RACH configurations, including one legacy RACH configuration and one additional RACH configuration in RRC_IDLE/INACTIVE/CONNECTED mode (RACH configuration Option 2); X-1, X-3; YES; n/a; SBFD random access operation based on two separate RACH configurations in RRC_IDLE/INACTIVE/CONNECTED mode is not supported; Per Band; TDD only; n/a; n/a; Optional with capability signalling. X-4-1; SBFD random access operation based on two separate RACH configurations with valid RO across SBFD and non-SBFD symbols; Support of SBFD random access operation based on two separate RACH configurations with or without valid RO starting from SBFD symbol and ending in non-SBFD symbol either in the same slot or across different slots; X-4; YES; n/a; SBFD random access operation based on two separate RACH configurations with valid RO starting from SBFD symbol and ending in non-SBFD symbol either in the same slot or across different slots is not supported.; Per Band; TDD only; n/a; n/a; Optional with capability signalling. X-5; Support of preamble repetitions in SBFD symbols; 1. Support of preamble repetitions within additional-ROs 2. Support of preamble repetitions within legacy-ROs; 60-2-1; YES; n/a; preamble repetitions in SBFD symbols are not supported; Per band; TDD only; n/a; n/a; Optional with capability signalling. X-6; Support of repetition of PUSCH transmission scheduled by RAR UL grant and DCI format 0_0 with CRC scrambled by TC-RNTI in SBFD symbols; Support of repetition of PUSCH transmission scheduled by RAR UL grant and DCI format 0_0 with CRC scrambled by TC-RNTI with Configuration 1; 30-6 60-2-1; YES; n/a; Msg3 PUSCH repetitions in SBFD symbols are not supported; Per band; TDD only; n/a; n/a; Optional with capability signalling. |
| [12] | OPPO |  |
| [13] | Apple | 60-6; Msg.3 repetition in SBFD symbols; - Support of repetition of PUSCH transmission scheduled by RAR UL grant and DCI format 0_0 with CRC scrambled by TC-RNTI in SBFD symbols; 30-6, 60-3 or 60-4; Yes; n/a; Msg.3 repetition in SBFD symbols is not supported; Per band; TDD only; n/a; n/a; Optional with capability signalling. |
| [14] | Qualcomm Incorporated | Proposal 7. Support the following list of UE capabilities for random access in SBFD symbols 60-5; MSG3 transmission with repetition in SBFD symbols; 1. Support of Repetition of msg3 PUSCH transmission scheduled by RAR UL grant and DCI format 0_0 with CRC scrambled by TC-RNTI in SBFD symbols with separate power control parameters.; 60-3 or 60-4; UE doesn’t support the Repetition of PUSCH transmission scheduled by RAR UL grant and DCI format 0_0 with CRC scrambled by TC-RNTI in SBFD symbols; Per band; Optional with capability signalling. |
| [15] | NTT DOCOMO, INC. | In our understanding, there is no additional UE complexity to support Msg 3 PUSCH repetitions in SBFD symbols on top of FG 30-6 and FG 60-3/60-4. Msg 3 PUSCH repetitions in SBFD symbols can be supported by the support of FG 30-6 and FG 60-3/60-4. Therefore, Alt 1 is preferred. Proposal 7: Regarding Msg.3 repetition in SBFD symbols, clarify in FG 60-3/4 Note column that “Note: When a UE supports this FG and FG 30-6, the UE also support Msg.3 repetition in SBFD symbol”. |
| [16] | Ericsson | A common design aspect of the single RACH configuration (Option 1) is that it should not introduce any additional parameters. In our view, that is also the case for UE-features. Hence, for Option 1, PRACH preamble repetitions and Msg3 repetitions can be determined by the corresponding legacy feature in FG 30-6. Additionally, there is in our view no technical merit to why an SBFD-aware UE also supporting preamble repetitions or Msg3 repetitions in legacy RACH should not be able to also support it for Option 1. In contrast, for the additional RACH configuration (Option 2), a separate RACH configuration will be provided. However, the fundamental capability for the UE to support repetitions within the additional configuration should not differ. Hence, also for this FG, we propose to make it dependent on the legacy capability for repetitions. - PRACH preamble repetitions and Msg3 repetitions for both the single and additional RACH configurations are determined by the corresponding legacy UE-features for PRACH preamble and Msg3 repetitions. |

Summary of companies’ views and discussion

FG structure - Separate FG for Msg.3 repetition in SBFD symbols: vivo, SPRD/UNISOC, CATT, Offino, CMCC, Samsung, CMCC, Xiaomi, Apple, Qualcomm - No separate FG for Msg3 repetition (support is determined by the legacy 30-6): Ericsson, HW/Hisi, Nokia, DOCOMO Type of the FG (for Msg.3 repetition in SBFD symbols) - Per band: SPRD/UNISOC, CATT, Offino, CMCC, Xiaomi Prerequisite of the basic FG - X-3 (RACH configuration option 1) or X-4 (RACH configuration option 2): SPRD/UNISOC, Offino, CMCC, Qualcomm - X-1 (basic FG for SBFD): CATT - 30-6 (Msg.3 repetition): SPRD/UNISOC, CATT, CMCC, Xiaomi - 60-2-1: Xiaomi

Question 3-4-1:

Companies are encouraged to share views for how to capture Msg.3 repetition in SBFD symbols:

- Alt-1: Add as a component in FG 60-3 and 60-4, for UE supporting FG 60-3 or 60-4 and supporting FG30-6 as well

- Alt-2: Introduce a new FG, e.g., as follows

| 60-6 | Msg.3 repetition in SBFD symbols | Support of repetition of PUSCH transmission scheduled by RAR UL grant and DCI format 0_0 with CRC scrambled by TC-RNTI in SBFD symbols | [60-3 or 60-4, 30-6] | YES | n/a | Msg.3 repetition in SBFD symbols is not supported | [Per Band] | TDD only | n/a | n/a |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comment |
| --- | --- |
| ZTE | Similar to PRACH repetition, no need to introduce a new FG. We are ok with Alt-1 if needed. |
| DOCOMO | We prefer Alt-1. |
| Ericsson | Support Alt. 1. |
| Huawei, HiSilicon | We would like to propose another alternative. Add the support of Msg.3 repetition in SBFD symbols as a component in the basic FG. The reason is that the support of Configuration 1 is included is the basic FG. A UE supporting Msg3 repetition (FG 30-6) and the basic SBFD FG should be have the capability to support Msg3 repetition in SBFD symbols without much additional complexity. |
| Samsung | Alt-2 |
| QC | Alt-2. |

ss

3.5	Potential other FGs

Following inputs are provided in contributions for the RAN1#121 meeting.

| [1] | Huawei, HiSilicon |  |
| --- | --- | --- |
| [2] | ZTE Corporation, Sanechips |  |
| [3] | vivo | Proposal 6: Support of valid RO across SBFD and non-SBFD symbols should be a separate FG. |
| [4] | Nokia |  |
| [5] | MediaTek Inc. |  |
| [6] | Spreadtrum, UNISOC | - Other FG: RO across SBFD/non-SBFD A separate FG for support of valid RO across SBFD and non-SBFD symbols Type of the FG(s): Per band Prerequisite of the FG: 60-1, 60-4 |
| [7] | Samsung |  |
| [8] | Ofinno |  |
| [9] | CATT |  |
| [10] | CMCC |  |
| [11] | Xiaomi |  |
| [12] | OPPO |  |
| [13] | Apple |  |
| [14] | Qualcomm Incorporated |  |
| [15] | NTT DOCOMO, INC. |  |
| [16] | Ericsson |  |

Summary of companies’ views and discussion

- Support of valid RO across SBFD and non-SBFD symbols should be a separate FG.: vivo, SPRD/UNISOC,

- FGs for CLI handling

4.1	FGs for UL resource muting

Following inputs are provided in contributions for the RAN1#121 meeting.

| [1] | Huawei, HiSilicon | UL resource muting For the support of UL resource muting, FG 60-7 and FG 60-7a were introduced for CP-OFDM waveform and DFT-S-OFDM waveform respectively. One remaining issue is whether a UE shall support one UL resource muting pattern or multiple UL resource muting patterns for the basic support of UL resource muting. In another words, whether the semi-statically configured time location and frequency location of UL resource muting for DG-PUSCH and Type-2 CG PUSCH and one or multiple Type-1 CG PUSCH configurations should be the same. It should be noted that the following has already been agreed in RAN1#120. Agreement For the semi-statically configured time location and frequency location of UL muting symbol(s) for PUSCH, - Separate configurations of up to two UL muting symbols can be provided per UE for DG PUSCH/Type 2 CG PUSCH and Type 1 CG PUSCH - Separate configuration for each of multiple Type 1 CG PUSCH configurations is supported. Given that there is not much additional UE implementation effort to support different UL muting patterns for DG-PUSCH and Type-2 CG PUSCH and Type-1 CG PUSCH, we suggest to remove the FFS from FG60-7 and FG60-7a. Proposal 6: For both FG 60-7 and FG 60-7a, UE supports different UL resource muting for DG/Type 2 CG PUSCH and Type 1 CG PUSCH. |
| --- | --- | --- |
| [2] | ZTE Corporation, Sanechips | UL resource muting In RAN1#120bis, the following FGs were agreed for UL resource muting. 60-7; UL resource muting for CP-OFDM waveform; 1. Support semi-static configuration of time location and frequency location of UL resource muting for CP-OFDM waveform - Up to two UL muting symbols within a slot for PUSCH - A configurable comb offset {0, 1} for the UL muting symbol [- FFS: Whether this FG only support one common pattern of UL muting for DG/Type 2 CG PUSCH and Type 1 CG PUSCH] 2. Support dynamic on/off indication of the configured UL muting symbols by TDRA field in DCI for DG-PUSCH and Type-2 CG PUSCH 3. Support semi-static determination of UL muting symbols for Type-1 CG PUSCH FFS: other potential component/FG/additional restriction; [FFS]; YES; n/a; UL resource muting for CP-OFDM waveform is not supported; [Per band]; TDD only; n/a; n/a; Optional with capability signalling. 60-7a; UL resource muting for DFTS-OFDM waveform; 1. Support semi-static configuration of time location and frequency location of UL resource muting for DFTS-OFDM waveform - Up to two UL muting symbols within a slot for PUSCH - A configurable comb offset {0, 1} for the UL muting symbol [- FFS: Whether this FG only support one common pattern of UL muting for DG/Type 2 CG PUSCH and Type 1 CG PUSCH] 2. Support dynamic on/off indication of the configured UL muting symbols by TDRA field in DCI for DG-PUSCH and Type-2 CG PUSCH 3. Support semi-static determination of UL muting symbols for Type-1 CG PUSCH FFS: other potential component/FG/additional restriction; [FFS]; YES; n/a; UL resource muting for DFTS-OFDM waveform is not supported; [Per Band]; TDD only; n/a; n/a; Optional with capability signalling. Regarding the yellow highlighted pending parts, our views are summarized as below. - Support different UL muting patterns between DG/Type 2 CG PUSCH and Type 1 CG PUSCH - Prefer NOT to add FG 60-1 as the prerequisite - OK with per band reporting Proposal 4: For UL resource muting, support the following for FG 60-7 and 60-7a. - Support different UL muting patterns between DG/Type 2 CG PUSCH and Type 1 CG PUSCH - NOT to add FG 60-1 as the prerequisite - Reporting granularity: per band |
| [3] | vivo | Proposal : For basic FG for UL muting, separate configuration between DG/CG Type 2 PUSCH and CG Type 1 PUSCH is supported. Proposal : For basic FG for UL muting, separate configuration for each of the multiple CG Type 1 PUSCH configurations is supported. |
| [4] | Nokia | The following open issues are common for both 60-7 and 60-7a: [- FFS: Whether this FG only support one common pattern of UL muting for DG/Type 2 CG PUSCH and Type 1 CG PUSCH] FFS: other potential component/FG/additional restriction. Given that the muting pattern is configured per slot, we are fine to support common pattern of UL muting for DG/Type 2 CG vs. Type 1 CG PUSCH. Although, benefit may exist if separate patterns are considered for Type 1, which has specific semi-statically configured resource and periodicity. Proposal 6. Support common pattern of UL muting for DG/Type 2 CG vs. Type 1 CG PUSCH. |
| [5] | MediaTek Inc. | Whether to support one common pattern of UL muting for DG/Type 2 CG PUSCH and Type 1 CG PUSCH In RAN1#120bis, there was discussion on whether one common pattern of UL muting is supported for DG/Type 2 CG PUSCH and Type 1 CG PUSCH. According to RAN1 agreement in RAN1#120, separate configurations of up to two UL muting symbols can be provided per UE for DG PUSCH/Type 2 CG PUSCH and Type 1 CG PUSCH. From the agreement it is clear that a common UL muting pattern is defined for DG PUSCH and Type 2 CG PUSCH, which is separate from the muting pattern defined for Type 2 CG PUSCH. Therefore, separate UL muting patterns for DG/Type 2 CG PUSCH and Type 1 CG PUSCH is added to FG 60-7 and FG 60-7a. - Support of separate UL muting patterns for DG/Type 2 CG PUSCH and Type 1 CG PUSCH is added to FG 60-7 and FG 60-7a |
| [6] | Spreadtrum, UNISOC | FGs for UL resource muting In RAN1#120bis, following agreement was achieved. Agreement: Introduce the following FGs for UL resource muting 60-7; UL resource muting for CP-OFDM waveform; 1. Support semi-static configuration of time location and frequency location of UL resource muting for CP-OFDM waveform - Up to two UL muting symbols within a slot for PUSCH - A configurable comb offset {0, 1} for the UL muting symbol [- FFS: Whether this FG only support one common pattern of UL muting for DG/Type 2 CG PUSCH and Type 1 CG PUSCH] 2. Support dynamic on/off indication of the configured UL muting symbols by TDRA field in DCI for DG-PUSCH and Type-2 CG PUSCH 3. Support semi-static determination of UL muting symbols for Type-1 CG PUSCH FFS: other potential component/FG/additional restriction; [FFS]; YES; n/a; UL resource muting for CP-OFDM waveform is not supported; [Per Band]; TDD only; n/a; n/a; Optional with capability signalling. 60-7a; UL resource muting for DFTS-OFDM waveform; 1. Support semi-static configuration of time location and frequency location of UL resource muting for DFTS-OFDM waveform - Up to two UL muting symbols within a slot for PUSCH - A configurable comb offset {0, 1} for the UL muting symbol [- FFS: Whether this FG only support one common pattern of UL muting for DG/Type 2 CG PUSCH and Type 1 CG PUSCH] 2. Support dynamic on/off indication of the configured UL muting symbols by TDRA field in DCI for DG-PUSCH and Type-2 CG PUSCH 3. Support semi-static determination of UL muting symbols for Type-1 CG PUSCH FFS: other potential component/FG/additional restriction; [FFS]; YES; n/a; UL resource muting for DFTS-OFDM waveform is not supported; [Per Band]; TDD only; n/a; n/a; Optional with capability signalling.. For basic FG for UL resource muting, single UL muting pattern should be configured for DG PUSCH, Type 1 CG PUSCH and Type 2 CG PUSCH. There is no need to support separate configurations of up to two UL muting symbols for DG PUSCH/Type 2 CG PUSCH and Type 1 CG PUSCH. Separate configurations should be separate FG. So FFS part in component 1 should be confirmed. From our perspective, UL muting resource can not be applied to non-SBFD symbols. So the prerequisite FG of 60-7/60-7a is FG 60-1. The granularity of the FG 60-7/60-7a is per Band. |
| [7] | Samsung | The following agreements on UE features for UL muting were made in RAN1#120bis [2][3]. <60-7, 60-7a> Agreement (RAN1#120bis): Introduce the following FGs for UL resource muting 60-7; UL resource muting for CP-OFDM waveform; 1. Support semi-static configuration of time location and frequency location of UL resource muting for CP-OFDM waveform - Up to two UL muting symbols within a slot for PUSCH - A configurable comb offset {0, 1} for the UL muting symbol [- FFS: Whether this FG only support one common pattern of UL muting for DG/Type 2 CG PUSCH and Type 1 CG PUSCH] 2. Support dynamic on/off indication of the configured UL muting symbols by TDRA field in DCI for DG-PUSCH and Type-2 CG PUSCH 3. Support semi-static determination of UL muting symbols for Type-1 CG PUSCH FFS: other potential component/FG/additional restriction; [FFS]; YES; n/a; UL resource muting for CP-OFDM waveform is not supported; [Per Band]; TDD only; n/a; n/a; Optional with capability signalling. 60-7a; UL resource muting for DFTS-OFDM waveform; 1. Support semi-static configuration of time location and frequency location of UL resource muting for DFTS-OFDM waveform - Up to two UL muting symbols within a slot for PUSCH - A configurable comb offset {0, 1} for the UL muting symbol [- FFS: Whether this FG only support one common pattern of UL muting for DG/Type 2 CG PUSCH and Type 1 CG PUSCH] 2. Support dynamic on/off indication of the configured UL muting symbols by TDRA field in DCI for DG-PUSCH and Type-2 CG PUSCH 3. Support semi-static determination of UL muting symbols for Type-1 CG PUSCH FFS: other potential component/FG/additional restriction; [FFS]; YES; n/a; UL resource muting for DFTS-OFDM waveform is not supported; [Per Band]; TDD only; n/a; n/a; Optional with capability signalling.. Regarding the “FFS: Whether this FG only support one common pattern of UL muting for DG/Type 2 CG PUSCH and Type 1 CG PUSCH” – It is our understanding that RRC should restrict the possible configuration of muting symbols to one/same set of either 1 or 2 symbol positions for the DG. However, at least one different/separate (from DG) common pattern of UL muting is required for Type 2 CG PUSCH and Type 1 CG. One reason is that from the gNB perspective, it should be possible to estimate the inter-gNB CLI at least for one set of alternative UL muting positions when compared to the DG based PUSCH allocations for which the UL muting can be dynamically indicated on/off by the TDRA field. We also consider that the support of one different/separate set of UL muting symbols for the CG allocations would not result in undue UE complexity. Proposal 9: For FGs 60-7/60-7a and FFS: Whether this FG only support one common pattern of UL muting for DG/Type 2 CG PUSCH and Type 1 CG PUSCH Support one common pattern of UL muting for DG and one common pattern for Type 2 CG PUSCH and Type 1 CG PUSCH. Regarding the FFS: other potential component/FG/additional restriction - We propose to add the following component to 60-7/60-7A: “Support 1 or 2 UL muting symbols on PUSCH > 7 symbols for PUSCH mapping type A”. Support for UL muting on short PUSCH, e.g., <8 symbols is not justified. The gNB can create symbol level scheduling gaps during back-to-back scheduling for CLI estimation/equalization. Proposal 10: For FG 60-7/60-7A, add component “Support 1 or 2 UL muting symbols on PUSCH > 7 symbols for PUSCH mapping type A”. We propose to add the following component to 60-7/60-7A: “For PUSCH mapping type A, support 1 UL muting symbol before the first FL DMRS and 1 UL muting symbol after the first FL DMRS”. The reason is that the original motivation for support of up to 2 UL muting symbols is using one UL muting symbol for CLI equalization in the data region of the slot, i.e., in the middle of the slot with respect to PDSCH-PUSCH, but the other second UL muting symbol for CLI equalization in the control region of the slot, i.e., on the first or second symbol with respect to DL-UL interference incurred from PDCCH. Therefore, for PUSCH mapping type B, the UE only needs to support 1 UL muted symbol. Proposal 11: For FG 60-7/60-7A, add component “For PUSCH mapping type A, support 1 UL muting symbol before the first FL DMRS and 1 UL muting symbol after the first FL DMRS” Proposal 12: For FG 60-7/60-7A, add component “For PUSCH mapping type B, support 1 UL muting symbol”. In our view, mandatory UE side support of UL muting on PUSCH symbols that can carry UCI is not required. For Rel-19 UEs not supporting such a functionality, the gNB can still configure time-domain locations of UL muting symbols based on the maximum UCI payload assumption and accounting for the beta offsets. When the UE not supporting UL muting on PUSCH symbols that can carry UCI determines presence of UCI on such a symbol resulting from the control/data multiplexing for PUSCH, the UE does not apply the UL muting the in the symbol. It is even more important to exclude specific PUSCH allocation cases such as UCI only payload from UL muting. Even with a combined CSI and A/N payload size of ~2000 bits, the resulting PUSCH allocation would typically not exceed a few symbols over the PUSCH allocation bandwidth. For UCI-only PUSCH, a transparent muting symbol can easily be created by the gNB as scheduling gap following the UCI-only PUSCH similar to the case of a short PUSCH allocations. It should also be considered that in absence of a data payload on PUSCH, the resulting UCI decoding performance is harder to control by the gNB. Proposal 13: For FG 60-7/60-7A, add the additional restriction that UL resource muting is not applied to a PUSCH which only carries a UCI payload but no data payload. |
| [8] | Ofinno |  |
| [9] | CATT | FGs for UL resource muting For UE features on UL resource muting, following agreements [1] were achieved in the last meeting. Agreement: Introduce the following FGs for UL resource muting 60-7; UL resource muting for CP-OFDM waveform; 1. Support semi-static configuration of time location and frequency location of UL resource muting for CP-OFDM waveform - Up to two UL muting symbols within a slot for PUSCH - A configurable comb offset {0, 1} for the UL muting symbol [- FFS: Whether this FG only support one common pattern of UL muting for DG/Type 2 CG PUSCH and Type 1 CG PUSCH] 2. Support dynamic on/off indication of the configured UL muting symbols by TDRA field in DCI for DG-PUSCH and Type-2 CG PUSCH 3. Support semi-static determination of UL muting symbols for Type-1 CG PUSCH FFS: other potential component/FG/additional restriction; [FFS]; YES; n/a; UL resource muting for CP-OFDM waveform is not supported; [Per Band]; TDD only; n/a; n/a; Optional with capability signalling. 60-7a; UL resource muting for DFTS-OFDM waveform; 1. Support semi-static configuration of time location and frequency location of UL resource muting for DFTS-OFDM waveform - Up to two UL muting symbols within a slot for PUSCH - A configurable comb offset {0, 1} for the UL muting symbol [- FFS: Whether this FG only support one common pattern of UL muting for DG/Type 2 CG PUSCH and Type 1 CG PUSCH] 2. Support dynamic on/off indication of the configured UL muting symbols by TDRA field in DCI for DG-PUSCH and Type-2 CG PUSCH 3. Support semi-static determination of UL muting symbols for Type-1 CG PUSCH FFS: other potential component/FG/additional restriction; [FFS]; YES; n/a; UL resource muting for DFTS-OFDM waveform is not supported; [Per Band]; TDD only; n/a; n/a; Optional with capability signalling. For FG 60-7/7a, based on the following agreement, both common and separate pattern of UL muting for DG/Type 2 CG PUSCH and Type 1 are supported. The only difference between one common and separate pattern of UL muting for DG/Type 2 CG PUSCH and Type 1 CG PUSCH is the number of UL muting patterns configured by gNB. There is no difference on UE performing UL muting. So we prefer that FG 60-7/7a includes separate patterns of UL muting for DG/Type 2 CG PUSCH and Type 1 CG PUSCH and separate patterns of UL muting for each of multiple Type 1 CG PUSCH configurations as the sub-bullet of component 1. Agreement For the semi-statically configured time location and frequency location of UL muting symbol(s) for PUSCH, - Separate configurations of up to two UL muting symbols can be provided per UE for DG PUSCH/Type 2 CG PUSCH and Type 1 CG PUSCH - Separate configuration for each of multiple Type 1 CG PUSCH configurations is supported. Proposal 6: Support the following FGs for UL resource muting. 60-7; UL resource muting for CP-OFDM waveform; 1. Support semi-static configuration of time location and frequency location of UL resource muting for CP-OFDM waveform - Up to two UL muting symbols within a slot for PUSCH - A configurable comb offset {0, 1} for the UL muting symbol - Separate configurations of up to two UL muting symbols can be provided per UE for DG PUSCH/Type 2 CG PUSCH and Type 1 CG PUSCH - Separate configuration for each of multiple Type 1 CG PUSCH configurations is supported 2. Support dynamic on/off indication of the configured UL muting symbols by TDRA field in DCI for DG-PUSCH and Type-2 CG PUSCH 3. Support semi-static determination of UL muting symbols for Type-1 CG PUSCH; FFS; YES; n/a; UL resource muting for CP-OFDM waveform is not supported; Per Band; TDD only; n/a; n/a; Optional with capability signalling. 60-7a; UL resource muting for DFTS-OFDM waveform; 1. Support semi-static configuration of time location and frequency location of UL resource muting for DFTS-OFDM waveform - Up to two UL muting symbols within a slot for PUSCH - A configurable comb offset {0, 1} for the UL muting symbol - Separate configurations of up to two UL muting symbols can be provided per UE for DG PUSCH/Type 2 CG PUSCH and Type 1 CG PUSCH - Separate configuration for each of multiple Type 1 CG PUSCH configurations is supported 2. Support dynamic on/off indication of the configured UL muting symbols by TDRA field in DCI for DG-PUSCH and Type-2 CG PUSCH 3. Support semi-static determination of UL muting symbols for Type-1 CG PUSCH; FFS; YES; n/a; UL resource muting for DFTS-OFDM waveform is not supported; Per Band; TDD only; n/a; n/a; Optional with capability signalling. |
| [10] | CMCC | Regarding whether UE only supports one UL resource muting pattern or multiple UL resource muting patterns, as the agreement in main session, separate configuration for each of multiple Type 1 CG PUSCH configurations is supported, thus we think the UL resource muting FG implies multiple UL resource muting pattern configurations for multiple Type-1 CG PUSCH configurations. |
| [11] | Xiaomi | For FG 60-9, 60-9a, 60-8, 60-9b, 60-7 and 60-7a, it is proposed to adopt “Per band” type. Proposal 7: Adopt “Per band” type for FG 60-9, 60-9a, 60-8, 60-9b, 60-7 and 60-7a. |
| [12] | OPPO | For FG 60-7 & FG 60-7a We support to maintain common muting pattern among DG, Type-1 CG and Type-2 CG, as a baseline UE capability. Proposal 6: Both FG 60-7 and FG 60-7a only support one common pattern of UL muting for DG/Type 2 CG PUSCH and Type 1 CG PUSCH. |
| [13] | Apple | Features; Index; Feature group; Components; Prerequisite feature groups; Need for the gNB to know if the feature is supported; Applicable to the capability signalling exchange between UEs (Sidelink WI only)”.; Consequence if the feature is not supported by the UE; Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC); Need of FDD/TDD differentiation; Need of FR1/FR2 differentiation; Capability interpretation for mixture of FDD/TDD and/or FR1/FR2; Note; Mandatory/Optional. 60. NR_duplex_evo; 60-3-4; Support of non-transparent UL resource muting; - CP-OFDM - DFT-S-OFDM; Yes; n/a; Non-transparent UL resource muting is not supported; Per band; n/a; n/a; n/a; Optional with capability signalling. |
| [14] | Qualcomm Incorporated | UE features for supporting UL resource muting on PUSCH For UL resource muting, RAN1 agreed to support indication/determination of UL resource muting for PUSCH based on semi-static configuration, assuming comb-2 for both DFT-S-OFDM and CP-OFDM in each allocated PRB and up to 2 symbols in time domain. To determine the time location of UL muting symbols in a slot for a DG PUSCH and type 2 CG PUSCH, RAN1 agreed on that the time location of each of one or two UL muting symbols is semi-statically configured, and muting all of the semi-statically configured time location of UL muting symbol(s) can be dynamically turned ON/OFF by TDRA field in DCI. And for frequency allocation, if UL resource muting is applied on a symbol for a PUSCH, a comb-2 pattern is applied for all of the allocated PRBs of the PUSCH on that symbol. And a comb offset {0,1} is configured for CP-OFDM and further discussion for DFT-s-OFDM. And 3dB power boosting is assumed for REs in the PUSCH symbol not containing PT-RS with UL resource muting. In RAN1 #120bis meeting, RAN1 agreed that: Agreement Introduce the following FGs for UL resource muting 60-7; UL resource muting for CP-OFDM waveform; 1. Support semi-static configuration of time location and frequency location of UL resource muting for CP-OFDM waveform - Up to two UL muting symbols within a slot for PUSCH - A configurable comb offset {0, 1} for the UL muting symbol [- FFS: Whether this FG only support one common pattern of UL muting for DG/Type 2 CG PUSCH and Type 1 CG PUSCH] 2. Support dynamic on/off indication of the configured UL muting symbols by TDRA field in DCI for DG-PUSCH and Type-2 CG PUSCH 3. Support semi-static determination of UL muting symbols for Type-1 CG PUSCH FFS: other potential component/FG/additional restriction; [FFS]; YES; n/a; UL resource muting for CP-OFDM waveform is not supported; [Per Band]; TDD only; n/a; n/a; Optional with capability signalling. 60-7a; UL resource muting for DFTS-OFDM waveform; 1. Support semi-static configuration of time location and frequency location of UL resource muting for DFTS-OFDM waveform - Up to two UL muting symbols within a slot for PUSCH - A configurable comb offset {0, 1} for the UL muting symbol [- FFS: Whether this FG only support one common pattern of UL muting for DG/Type 2 CG PUSCH and Type 1 CG PUSCH] 2. Support dynamic on/off indication of the configured UL muting symbols by TDRA field in DCI for DG-PUSCH and Type-2 CG PUSCH 3. Support semi-static determination of UL muting symbols for Type-1 CG PUSCH FFS: other potential component/FG/additional restriction; [FFS]; YES; n/a; UL resource muting for DFTS-OFDM waveform is not supported; [Per Band]; TDD only; n/a; n/a; Optional with capability signalling.. Proposal 8: Support UE feature for SBFD-aware UE of the following list of UE capabilities for the RE level UL resource muting with comb-2 pattern for PUSCH. Index; Feature group; Components; Prerequisite FG; Consequence if the feature is not supported by the UE; Type; Note; Mandatory/Optional. 60-7; UL resource muting for CP-OFDM waveform; 1. Support semi-static configuration of time location and frequency location of UL resource muting for CP-OFDM waveform - Up to two UL muting symbols within a slot for PUSCH - A configurable comb offset {0, 1} for the UL muting symbol - Support one common pattern of UL muting for DG/Type 2 CG PUSCH and Type 1 CG PUSCH 2. Support dynamic on/off indication of the configured UL muting symbols by TDRA field in DCI for DG-PUSCH and Type-2 CG PUSCH 3. Support semi-static determination of UL muting symbols for Type-1 CG PUSCH; N/A; YES; n/a; UL resource muting for CP-OFDM waveform is not supported; Per Band. 60-7a; UL resource muting for DFTS-OFDM waveform; 1. Support semi-static configuration of time location and frequency location of UL resource muting for DFTS-OFDM waveform - Up to two UL muting symbols within a slot for PUSCH - A configurable comb offset {0, 1} for the UL muting symbol - Support one common pattern of UL muting for DG/Type 2 CG PUSCH and Type 1 CG PUSCH 2. Support dynamic on/off indication of the configured UL muting symbols by TDRA field in DCI for DG-PUSCH and Type-2 CG PUSCH 3. Support semi-static determination of UL muting symbols for Type-1 CG PUSCH; N/A; YES; n/a; UL resource muting for DFTS-OFDM waveform is not supported; Per Band. |
| [15] | NTT DOCOMO, INC. | It needs further discussion on whether FG 60-7/60-7a only support one common pattern of UL muting for DG/Type 2 CG PUSCH and Type 1 CG PUSCH. It was agreed that UL muting symbols can be separately configured for DG/Type 2 CG PUSCH and Type 1 CG PUSCH. And UL muting pattern for per Type-1 CG PUSCH configuration can be separately configured. We think it is the basic mechanism for UL muting symbol configuration. Therefore, separate configurations of UL muting pattern for DG/Type 2 CG PUSCH and Type 1 CG PUSCH should be part of the FG 60-7/60-7a, and separate configurations of UL muting pattern for per Type-1 CG PUSCH should also be part of FG 60-7/7a. Regarding the reporting granularity of FGs, we think per band should be sufficient. Proposal 8: For FG 60-7 and FG 60-7a: - Separate configurations of UL muting pattern for DG/Type 2 CG PUSCH and Type 1 CG PUSCH should be part of FG 60-7 and FG 60-7a. - Separate configurations of UL muting pattern for per Type-1 CG PUSCH should be part of FG 60-7 and FG 60-7a. - Reporting type is per band. |
| [16] | Ericsson |  |

The following shows the latest status for the relevant FG(s).

| 60-7 | UL resource muting for CP-OFDM waveform | 1. Support semi-static configuration of time location and frequency location of UL resource muting for CP-OFDM waveform - Up to two UL muting symbols within a slot for PUSCH - A configurable comb offset {0, 1} for the UL muting symbol [- FFS: Whether this FG only support one common pattern of UL muting for DG/Type 2 CG PUSCH and Type 1 CG PUSCH] 2. Support dynamic on/off indication of the configured UL muting symbols by TDRA field in DCI for DG-PUSCH and Type-2 CG PUSCH 3. Support semi-static determination of UL muting symbols for Type-1 CG PUSCH FFS: other potential component/FG/additional restriction | [FFS] | YES | n/a | UL resource muting for CP-OFDM waveform is not supported | [Per band] | TDD only | n/a | n/a |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 60-7a | UL resource muting for DFTS-OFDM waveform | 1. Support semi-static configuration of time location and frequency location of UL resource muting for DFTS-OFDM waveform - Up to two UL muting symbols within a slot for PUSCH - A configurable comb offset {0, 1} for the UL muting symbol [- FFS: Whether this FG only support one common pattern of UL muting for DG/Type 2 CG PUSCH and Type 1 CG PUSCH] 2. Support dynamic on/off indication of the configured UL muting symbols by TDRA field in DCI for DG-PUSCH and Type-2 CG PUSCH 3. Support semi-static determination of UL muting symbols for Type-1 CG PUSCH FFS: other potential component/FG/additional restriction | [FFS] | YES | n/a | UL resource muting for DFTS-OFDM waveform is not supported | [Per Band] | TDD only | n/a | n/a |  | Optional with capability signalling |

Summary of companies’ views and discussion

FG structure and components - FFS: Whether this FG only support one common pattern of UL muting for DG/Type 2 CG PUSCH and Type 1 CG PUSCH - Separate configurations as a component of the basic FG for UL muting: HW/Hisi, vivo, ZTE, MTK, Samsung, CATT, CMCC, DOCOMO, - One pattern of UL muting for DG/Type 1/2 CG PUSCH in the component of the FG: Nokia, SPRD/UNISOC, OPPO, Qualcomm - FFS: Whether this FG only support one common pattern of UL muting across different Type 1 CG PUSCH configurations: - Separate configurations as a component of the basic FG: vivo, CATT, CMCC, DOCOMO, - Other components in the FG - Support 1 or 2 UL muting symbols on PUSCH > 7 symbols for PUSCH mapping type A: Samsung, - For PUSCH mapping type A, support 1 UL muting symbol before the first FL DMRS and 1 UL muting symbol after the first FL DMRS: Samsung, - For PUSCH mapping type B, support 1 UL muting symbol: Samsung, - 3dB power boosting for a symbol not containing PT-RS with UL resource muting: MediaTek - Supprted maximum number of muting symbols: OPPO - (restriction) UL resource muting is not applied to a PUSCH which only carries a UCI payload but no data payload: Samsung, Type of the FG(s) - Per band: ZTE, SPRD/UNISOC, xiaomi, CATT, Offino, MediaTek, CMCC, Xiaomi, Apple, DOCOMO, Qualcomm Prerequisite of the FG(s) - 60-1 (basic FG for SBFD): SPRD/UNISOC, CATT, MediaTek, CMCC, Xiaomi - No: ZTE, QC,.

Proposal 4-1-1:

Adopt “Per band” for FG 60-7 and 60-7a

| Company | Comment |
| --- | --- |
| ZTE | Support |
| DOCOMO | Support. |
| Ericsson | Support |
| Huawei, HiSilicon | Support |
| Samsung | Support |
| QC | Support |

Question 4-1-2:

Companies are encouraged to provide their view on the following two FFSs put in FG 60-7 and 60-7a:

[- FFS: Whether this FG only support one common pattern of UL muting for DG/Type 2 CG PUSCH and Type 1 CG PUSCH]

- Alt-1: FG 60-7 and 60-7a contain the support of:

  - Only one common pattern of UL muting between DG/Type 2 CG PUSCH and Type 1 CG PUSCH

  - Only one common pattern of UL muting among multiple Type 1 CG PUSCH configurations

- Alt-2: FG 60-7 and 60-7a contain the support of:

  - Same or different pattern of UL muting between DG/Type 2 CG PUSCH and Type 1 CG PUSCH

  - Only one common pattern of UL muting among multiple Type 1 CG PUSCH configurations

- Alt-3: FG 60-7 and 60-7a contain the support of:

  - Same or different pattern of UL muting between DG/Type 2 CG PUSCH and Type 1 CG PUSCH

  - Same or different pattern of UL muting among multiple Type 1 CG PUSCH configurations

| Company | Comment |
| --- | --- |
| ZTE | Alt-3 |
| DOCOMO | We prefer Alt-3. We think Alt-3 is aligned with the agreements that UL muting symbol configuration is common between DG PUSCH and Type 2 CG PUSCH, while UL muting symbol configuration for Type-1 CG PUSCH is provided for each CG configuration. |
| Ericsson | Alt-3 |
| Huawei, HiSilicon | Alt.3. Agree with DCM that it aligns with the agreement. |
| Samsung | Alt-3 |
| QC | Alt-1. The way we understood the prior agreement on separate configuration for type 1 CG PUSCH is to configure or not for UL muting, similar as TDRA field on/off. But common pattern shall be assumed for DG/Type 2 CG PUSCH and Type 1 CG PUSCH. Same discussion in RAN1 happened during down selection of Option 2 of common pattern for DG/Type 2 CG PUSCH. Same philosophy shall apply to Type 1 CG PUSCH as basic UE feature. No need to repeat the discussion again. |

4.2	FGs for L1-CLI-RSSI measurement

Following inputs are provided in contributions for the RAN1#121 meeting.

| [1] | Huawei, HiSilicon | L1-CLI-RSSI measurement and reporting It was agreed that support of semi-persistent L1 SRS-RSRP measurement resource and aperiodic reporting is defined as a separate UE capability. Similarly, the support of semi-persistent L1 L1-RSSI measurement resource and aperiodic reporting should also be defined as a separate UE capability. Proposal 7: The support of semi-persistent L1 L1-RSSI measurement resource and aperiodic reporting is defined as a separate UE capability. The second issue is whether to include the support of periodic L1 CLI-RSSI reporting and periodic CLI-RSSI measurement resource in FG 60-8. To align with the existing UE capability definitions, we prefer to include the support of periodic L1 CLI-RSSI reporting and periodic CLI-RSSI measurement resource in FG 60-8. For L1 CLI-RSSI measurement and reporting, the following components are included (update in red) - Aperiodic L1 CLI-RSSI reporting on PUSCH and periodic L1 CLI-RSSI reporting on PUCCH - Maximum number of periodic CSI report setting per BWP for L1-CLI-RSSI report - Maximum number of aperiodic CSI report setting per BWP for L1-CLI-RSSI report - Maximum number of configured aperiodic CSI triggering states in CSI-AperiodicTriggerStateList per CC - Support of CLI-RSSI measurement resource configured in one UL subband only, in one DL subband only, or across two DL subbands only. - Periodic and aperiodic CLI-RSSI measurement resource - Maximum number of CLI-RSSI measurement resources (sum of aperiodic/periodic/semi-persistent) across all CCs configured to measure L1-CLI-RSSI |
| --- | --- | --- |
| [2] | ZTE Corporation, Sanechips | L1-based CLI measurement and reporting In RAN1#120bis, the following FGs were agreed for L1-based UE-UE CLI handling. 60-8; L1 CLI-RSSI measurement and [aperiodic] reporting; 1. Aperiodic L1 CLI-RSSI reporting on PUSCH [and periodic L1 CLI-RSSI reporting on PUCCH] 2. Support of CLI-RSSI measurement resource configured in one UL subband only, in one DL subband only, or across two DL subbands only. 3. [Periodic and] aperiodic CLI-RSSI measurement resource FFS: whether each of components within brackets is separate FG or require reporting FFS: other potential component/FG; FFS; YES; n/a; [Aperiodic] L1 CLI-RSSI measurement and reporting is not supported; [Per band]; TDD only; [n/a]; [n/a]; Optional with capability signalling. 60-9; L1 SRS-RSRP measurement and aperiodic reporting; 1. Aperiodic L1 SRS-RSRP reporting on PUSCH, and [periodic and] aperiodic SRS-RSRP measurement resource FFS: whether each of components within brackets is separate FG or require reporting FFS: other potential component/FG; FFS; YES; n/a; [Aperiodic] L1 SRS-RSRP measurement and reporting is not supported; [Per band]; TDD only; n/a; n/a; Optional with capability signalling. 60-9a; FDM-ed reception of DL signals/channels and L1 SRS-RSRP measurement resource; 1. Support of FDMed reception of DL signals/channels and L1 SRS-RSRP measurement resource; 60-9; YES; n/a; FDMed reception of DL signals/channels and L1 SRS-RSRP measurement resource is not supported; [Per band]; TDD only; n/a; n/a; Optional with capability signalling. 60-9b; Support of semi-persistent L1 SRS-RSRP measurement resource and aperiodic reporting; 1. Support of semi-persistent L1 SRS-RSRP measurement resource and aperiodic reporting; 60-9; YES; n/a; Semi-persistent L1 SRS-RSRP measurement resource and aperiodic reporting is not supported; [Per Band]; TDD only; n/a; n/a; Optional with capability signalling. Regarding the yellow highlighted pending parts, our views are summarized as below. - Prefer to include periodic and aperiodic measurement and reporting in one FG - Similar to FG 60-9b, we are ok to add one separate FG for semi-persistent L1 CLI-RSSI measurement resource and aperiodic reporting - Prefer NOT to add FG 60-1 as the prerequisite for FG 60-8 and 60-9 - OK with per band reporting Proposal 5: Support the following revisions on L1-based UE-UE measurement and reporting related FGs. 60-8; L1 CLI-RSSI measurement and aperiodic reporting; 1. Aperiodic L1 CLI-RSSI reporting on PUSCH and periodic L1 CLI-RSSI reporting on PUCCH 2. Support of CLI-RSSI measurement resource configured in one UL subband only, in one DL subband only, or across two DL subbands only. 3. Periodic and aperiodic CLI-RSSI measurement resource; YES; n/a; Aperiodic L1 CLI-RSSI measurement and reporting is not supported; Per band; TDD only; n/a; n/a; Optional with capability signalling. 60-8a; Support of semi-persistent L1 CLI-RSSI measurement resource and aperiodic reporting; 1. Support of semi-persistent L1 CLI-RSSI measurement resource and aperiodic reporting; 60-8; YES; n/a; Semi-persistent L1 S CLI-RSSI measurement resource and aperiodic reporting is not supported; Per Band; TDD only; n/a; n/a; Optional with capability signalling. 60-9; L1 SRS-RSRP measurement and aperiodic reporting; 1. Aperiodic L1 SRS-RSRP reporting on PUSCH, and periodic and aperiodic SRS-RSRP measurement resource; YES; n/a; Aperiodic L1 SRS-RSRP measurement and reporting is not supported; Per band; TDD only; n/a; n/a; Optional with capability signalling. 60-9a; FDM-ed reception of DL signals/channels and L1 SRS-RSRP measurement resource; 1. Support of FDMed reception of DL signals/channels and L1 SRS-RSRP measurement resource; 60-9; YES; n/a; FDMed reception of DL signals/channels and L1 SRS-RSRP measurement resource is not supported; Per band; TDD only; n/a; n/a; Optional with capability signalling. 60-9b; Support of semi-persistent L1 SRS-RSRP measurement resource and aperiodic reporting; 1. Support of semi-persistent L1 SRS-RSRP measurement resource and aperiodic reporting; 60-9; YES; n/a; Semi-persistent L1 SRS-RSRP measurement resource and aperiodic reporting is not supported; Per Band; TDD only; n/a; n/a; Optional with capability signalling. |
| [3] | vivo | Proposal : For L1-CLI-RSSI measurement and reporting, define the following reporting (related to maximum number of CLI-RSSI measurement resources in various units): - Maximum number of configured L1 CLI-RSSI measurement resources (sum of aperiodic/semi-persistent/periodic) per CC - Maximum number of simultaneous L1 CLI-RSSI measurement resources with resource type #1/#2 (sum of aperiodic/semi-persistent/periodic) per CC |
| [4] | Nokia | FG 60-8 The following open points can be noted for FG 60-8: 1. Aperiodic L1 CLI-RSSI reporting on PUSCH [and periodic L1 CLI-RSSI reporting on PUCCH] 3. [Periodic and] aperiodic CLI-RSSI measurement resource FFS: whether each of components within brackets is separate FG or require reporting FFS: other potential component/FG. The main discussion point here is whether periodic CLI-RSSI measurement and reporting should be included in FG 60-8 or it could be a separate FG. We do not see any technical difference for supporting both periodic and aperiodic resource measurement and reporting. Therefore, our first preference is to keep them in one FG. Proposal 7. For FG 60-8, support keeping periodic and aperiodic CLI-RSSI measurement and reporting in a same FG, i.e., support removing the square brackets in 60-8. |
| [5] | MediaTek Inc. | Whether support of periodic L1 CLI-RSSI reporting on PUSCH is a separate FG In RAN1#120bis, an FG 60-8 was agreed on support for L1 CLI-RSSI measurement reporting. Aperiodic L1 CLI-RSSI reporting on PUSCH was agreed as a component of this FG. It needs to be determined whether periodic L1 CLI-RSSI reporting on PUCCH is part of this FG or is defined as a separate FG. In our view, support for periodic L1 CLI-RSSI reporting on PUCCH should be component of FG 60-8 (i.e., periodic L1 CLI-RSSI reporting on PUCCH is not a separate FG). - Support periodic L1 CLI-RSSI reporting on PUCCH is part of FG 60-8. Whether support of periodic CLI-RSSI measurement resource is a separate FG In RAN1#120bis, an FG 60-8 was agreed on support for L1 CLI-RSSI measurement reporting. Aperiodic CLI-RSSI measurement resource was agreed as a component of this FG. It needs to be determined whether periodic CLI-RSSI measurement resource is part of this FG or is defined as a separate FG. In our view, support for periodic CLI-RSSI measurement resource should be part of FG 60-8 (i.e., periodic CLI-RSSI measurement resource is not a separate FG). - Support periodic CLI-RSSI measurement resource is part of FG 60-8. |
| [6] | Spreadtrum, UNISOC | For FG for L1 CLI-RSSI measurement/reporting, potential component as following: In RAN1#120bis, following agreement was achieved. Agreement: Introduce following FG for L1 CLI-RSSI measurement/reporting. 60-8; L1 CLI-RSSI measurement and [aperiodic] reporting; 1. Aperiodic L1 CLI-RSSI reporting on PUSCH [and periodic L1 CLI-RSSI reporting on PUCCH] 2. Support of CLI-RSSI measurement resource configured in one UL subband only, in one DL subband only, or across two DL subbands only. 3. [Periodic and] aperiodic CLI-RSSI measurement resource FFS: whether each of components within brackets is separate FG or require reporting FFS: other potential component/FG; FFS; YES; n/a; [Aperiodic] L1 CLI-RSSI measurement and reporting is not supported; [Per Band]; TDD only; [n/a]; [n/a]; Optional with capability signalling.. Periodic and aperiodic CLI-RSSI measurement resource should be part of basic FG for L1 CLI-RSSI measurement. It reuses same principle in R16 L1-RSRP/SINR UE feature. So the FFS part in component 3 should be confirmed. For semi-persistent CLI-RSSI measurement resource and aperiodic reporting, it should be included in separate FG. For periodic L1 CLI-RSSI reporting on PUCCH, it can be included in basic FG for L1 CLI-RSSI measurement/reporting. So FFS part in component 1 can be confirmed. The maximum number of CLI-RSSI measurement resources should be separate FG, which is similar as L3-CLI-RSSI capability. Maximum number of CLI resource per resource set in Rel-16 can be reused, i.e. maxNrofCLI-RSSI-Resources-r16 = 64. In legacy CLI-RSSI measurement, only total number of the measurement resources that the UE can support needs to be reported. There is no need to report maximum number of L1 CLI-RSSI measurement resources per CC. So maximum number of L1 CLI-RSSI measurement resources (sum of aperiodic/semi-persistent/periodic) across all CCs configured to measure L1-CLI-RSSI should be separate FG. The prerequisite FG of L1 CLI-RSSI measurement/reporting is FG 2-32 (Basic CSI feedback). The granularity of the FG 60-8 is per Band. |
| [7] | Samsung | The following agreements on UE features for UE L1-based CLI measurement and reporting were made in RAN1#120bis [2][3]. <60-8>, <60-9a> Agreement (RAN1#120bis): Introduce following FG for L1 SRS-RSRP measurement/reporting. 60-9; L1 SRS-RSRP measurement and aperiodic reporting; 1. Aperiodic L1 SRS-RSRP reporting on PUSCH, and [periodic and] aperiodic SRS-RSRP measurement resource FFS: whether each of components within brackets is separate FG or require reporting FFS: other potential component/FG; FFS; YES; n/a; [Aperiodic] L1 SRS-RSRP measurement and reporting is not supported; [Per Band]; TDD only; n/a; n/a; Optional with capability signalling. Agreement (RAN1#120bis): Introduce a new FG for FDMed reception of DL signals/channels and L1 SRS-RSRP measurement resource as follows 60-9a; FDM-ed reception of DL signals/channels and L1 SRS-RSRP measurement resource; 1. Support of FDMed reception of DL signals/channels and L1 SRS-RSRP measurement resource; 60-9; YES; n/a; FDMed reception of DL signals/channels and L1 SRS-RSRP measurement resource is not supported; [Per Band]; TDD only; n/a; n/a; Optional with capability signalling. Agreement (RAN1#120bis): Introduce following FG for L1 CLI-RSSI measurement/reporting. 60-8; L1 CLI-RSSI measurement and [aperiodic] reporting; 1. Aperiodic L1 CLI-RSSI reporting on PUSCH [and periodic L1 CLI-RSSI reporting on PUCCH] 2. Support of CLI-RSSI measurement resource configured in one UL subband only, in one DL subband only, or across two DL subbands only. 3. [Periodic and] aperiodic CLI-RSSI measurement resource FFS: whether each of components within brackets is separate FG or require reporting FFS: other potential component/FG; FFS; YES; n/a; [Aperiodic] L1 CLI-RSSI measurement and reporting is not supported; [Per Band]; TDD only; [n/a]; [n/a]; Optional with capability signalling. Agreement (RAN1#120bis): Define the following FG 60-9b; Support of semi-persistent L1 SRS-RSRP measurement resource and aperiodic reporting; 1. Support of semi-persistent L1 SRS-RSRP measurement resource and aperiodic reporting; 60-9; YES; n/a; Semi-persistent L1 SRS-RSRP measurement resource and aperiodic reporting is not supported; [Per Band]; TDD only; n/a; n/a; Optional with capability signalling.. Regarding the FFS on separate FG/components with respect to aperiodic and periodic measurement and configuration modes in FGs 60-8, 60-9, 60-9a - In our view, it is important to prevent UE feature fragmentation in Rel-19 for the L1-based CLI reporting. It needs to be considered that based on the RAN1#120 and RAN1#120bis agreement, the additional (to aperiodic) support of periodic L1-CLI RSSI reporting comes at little delta for the UE modem implementation. Therefore, aperiodic L1 SRS-RSRP reporting on PUSCH using periodic or aperiodic SRS-RSRP measurement resources, and aperiodic L1 CLI-RSSI reporting on PUSCH and periodic L1 CLI-RSSI reporting on PUCCH should be supported by the UE Proposal 14: For FG 60-8, support component 1 “Aperiodic L1 CLI-RSSI reporting on PUSCH and periodic L1 CLI-RSSI reporting on PUCCH, and component 3 Periodic and aperiodic CLI-RSSI measurement resource. Proposal 15: For FG 60-9, support component 1 “Aperiodic L1 SRS-RSRP reporting on PUSCH, and periodic and aperiodic SRS-RSRP measurement resource. |
| [8] | Ofinno |  |
| [9] | CATT | FGs for L1-CLI-RSSI measurement - Regarding UE features for L1-CLI-RSSI measurement, the following was agreed. Agreement: Introduce following FG for L1 CLI-RSSI measurement/reporting. 60-8; L1 CLI-RSSI measurement and [aperiodic] reporting; 1. Aperiodic L1 CLI-RSSI reporting on PUSCH [and periodic L1 CLI-RSSI reporting on PUCCH] 2. Support of CLI-RSSI measurement resource configured in one UL subband only, in one DL subband only, or across two DL subbands only. 3. [Periodic and] aperiodic CLI-RSSI measurement resource FFS: whether each of components within brackets is separate FG or require reporting FFS: other potential component/FG; FFS; YES; n/a; [Aperiodic] L1 CLI-RSSI measurement and reporting is not supported; [Per Band]; TDD only; [n/a]; [n/a]; Optional with capability signalling. For FG 60-8, one FFS issue is whether separate FGs are defined for aperiodic L1 CLI-RSSI reporting on PUSCH and periodic L1 CLI-RSSI reporting on PUCCH. In the existing spec, separate FGs are defined for different report types for L1 beam report. Referring to L1 beam report, we prefer to define separate FGs for aperiodic L1 CLI-RSSI reporting on PUSCH and periodic L1 CLI-RSSI reporting on PUCCH. Proposal 7: Separate FGs are defined for aperiodic L1 CLI-RSSI reporting on PUSCH and periodic L1 CLI-RSSI reporting on PUCCH. Another open issue is whether separate FG is defined for periodic and aperiodic CLI measurement resource for CLI-RSSI reporting. Except the difference on resource configuration, there is no difference on UE CLI measurement. So we prefer to define one FG for periodic and aperiodic CLI measurement resource. Proposal 8: Support the following FGs for L1-CLI-RSSI measurement. 60-8; L1 CLI-RSSI measurement and aperiodic reporting; 1. Aperiodic L1 CLI-RSSI reporting on PUSCH 2. Support of CLI-RSSI measurement resource configured in one UL subband only, in one DL subband only, or across two DL subbands only. 3. Periodic and aperiodic CLI-RSSI measurement resource; FFS; YES; n/a; Aperiodic L1 CLI-RSSI measurement and reporting is not supported; Per Band; TDD only; n/a; n/a; Optional with capability signalling. 60-8a; L1 CLI-RSSI measurement and periodic reporting; 1. Periodic L1 CLI-RSSI reporting on PUCCH 2. Support of CLI-RSSI measurement resource configured in one UL subband only, in one DL subband only, or across two DL subbands only. 3. Periodic and aperiodic CLI-RSSI measurement resource; FFS; YES; n/a; Periodic L1 CLI-RSSI measurement and reporting is not supported; Per Ban; TDD only; n/a; n/a; Optional with capability signalling. Based on the agreement achieved in the last meeting, for L1 SRS-RSRP aperiodic reporting, semi-persistent L1 SRS-RSRP measurement resource was defined in a separate FG with periodic and aperiodic SRS-RSRP measurement resource. From our view, new introduced MAC CE for semi-persistent L1 SRS-RSRP measurement resource activation/deactivation is one of the factors. Based on the same consideration, a separate FG is proposed for semi-persistent L1 CLI-RSSI measurement resource and aperiodic reporting. Proposal 9: Define a separate FG for semi-persistent L1 CLI-RSSI measurement resource and aperiodic reporting. 60-8b; Support of semi-persistent L1 CLI-RSSI measurement resource and aperiodic reporting; 1. Support of semi-persistent L1 CLI-RSSI measurement resource and aperiodic reporting; 60-8; YES; n/a; Semi-persistent L1 CLI-RSSI measurement resource and aperiodic reporting is not supported; Per Band; TDD only; n/a; n/a; Optional with capability signalling. |
| [10] | CMCC | First, for L1 CLI-RSSI measurement and reporting, support of semi-persistent L1 SRS-RSRP measurement resource and aperiodic reporting is defined as a separate UE capability, and a new FG for FDMed reception of DL signals/channels and L1 SRS-RSRP measurement resource is also introduced. Similarly, the following two additional UE capabilities can be defined for L1 CLI-RSSI measurement and reporting: - Support of semi-persistent L1 CLI-RSSI measurement resource and aperiodic reporting - Support of FDMed reception of DL signals/channels and L1 CLI-RSSI measurement resource Then, in the existing CSI measurement and reporting framework, the basic FG (2-32) includes both the periodic CSI report on PUCCH and aperiodic CSI report on PUSCH, and for the aperiodic CSI report, periodic and aperiodic CSI-RS resources are supported in the basic FG. Since Rel-19 L1 based UE-to-UE CLI measurement and reporting is designed based on the existing CSI framework, the same principles should be applied. That is, it is preferred to include periodic CLI-RSSI measurement resource and periodic L1 CLI-RSSI reporting on PUCCH as part of FG 60-8, and include periodic SRS-RSRP measurement resource as part of FG 60-9. |
| [11] | Xiaomi | For FG 60-9, 60-9a, 60-8, 60-9b, 60-7 and 60-7a, it is proposed to adopt “Per band” type. Proposal 7: Adopt “Per band” type for FG 60-9, 60-9a, 60-8, 60-9b, 60-7 and 60-7a. |
| [12] | OPPO | For FG 60-8 & FG 60-9 From UE implementation point of view, there is no much difference on complexity between periodic CLI measurement and aperiodic CLI measurement, but the difference between periodic reporting and aperiodic reporting is more obvious. Proposal 7: FG 60-8 is split into two FG3, one for periodic reporting on PUCCH, another for aperiodic reporting on PUSCH. Proposal 8: FG 60-9 component 1 contains both periodic and aperiodic SRS-RSRP measurement resources. |
| [13] | Apple | 60. NR_duplex_evo; 60-3-1; Support of L1 based CLI measurement method within the active DL BWP; - Method#1: UE measures RSSI within DL subband - Method#2: UE measures RSRP of aggressor UE within UL subband - Method#3: UE measures RSSI within UL subband; Yes; n/a; L1 based CLI measurement and report is not supported; Per band; n/a; n/a; n/a; Optional with capability signalling. 60. NR_duplex_evo; 60-3-2; Support of CLI-RSSI measurement resource type; - Measurement resource type#1: One CLI-RSSI measurement resource is configured within a DL subband - Measurement resource type#2: One CLI-RSSI measurement resource is configured across two DL subbands; Yes; n/a; Per band; n/a; n/a; n/a; Optional with capability signalling. 60. NR_duplex_evo; 60-3-3; Support of L1 CLI-RSSI and L1-SRS-RSRP measurement; - The max number of L1-CLI-RSSI/L1-SRS-RSRP resources (sum of aperiodic/periodic/semipersistent) across all CCs configured to measure L1-CLI-RSSI or L1-SRS-RSRP within a slot shall not exceed M - The max number of aperiodic L1-CLI-RSSI resources across all CCs configured to measure CLI-RSSI shall not exceed MA_1 - The max number of aperiodic L1-SRS-RSRP resources across all CCs configured to measure SRS-RSRP shall not exceed MB_1; Yes; n/a; Per band; n/a; n/a; n/a; Optional with capability signalling. |
| [14] | Qualcomm Incorporated | UE features for L1-CLI measurements and reporting For L1 CLI measurement and reporting, RAN1 agreed on details of configuration of aperiodic, periodic and semi-persistent SRS-RSRP and CLI-RSSI measurement resources, configuration/determination of QCL typed per aperiodic, periodic and semi-persistent SRS-RSRP and CLI-RSSI measurement resource, and also on details of configuration of at least aperiodic reporting. RAN1 also agreed on Method #1 that UE measures RSSI within DL subband, Method #2 that UE measures RSRP of aggressor UE within UL subband, and Method #3 for CLI measurement that UE measures RSSI within UL subband. We also support have a UE FG for minimum slot offset configuration for aperiodic CLI-RSSI-MeasurementResourceSet and please refer to our contribution for CLI handling for the reasoning. In RAN1 #120bis meeting, RAN1 agreed that: Agreement: Introduce following FG for L1 SRS-RSRP measurement/reporting. 60-9; L1 SRS-RSRP measurement and aperiodic reporting; 1. Aperiodic L1 SRS-RSRP reporting on PUSCH, and [periodic and] aperiodic SRS-RSRP measurement resource FFS: whether each of components within brackets is separate FG or require reporting FFS: other potential component/FG; FFS; YES; n/a; [Aperiodic] L1 SRS-RSRP measurement and reporting is not supported; [Per Band]; TDD only; n/a; n/a; Optional with capability signalling. Agreement: Introduce a new FG for FDMed reception of DL signals/channels and L1 SRS-RSRP measurement resource as follows 60-9a; FDM-ed reception of DL signals/channels and L1 SRS-RSRP measurement resource; 1. Support of FDMed reception of DL signals/channels and L1 SRS-RSRP measurement resource; 60-9; YES; n/a; FDMed reception of DL signals/channels and L1 SRS-RSRP measurement resource is not supported; [Per Band]; TDD only; n/a; n/a; Optional with capability signalling. Agreement: Introduce following FG for L1 CLI-RSSI measurement/reporting. 60-8; L1 CLI-RSSI measurement and [aperiodic] reporting; 1. Aperiodic L1 CLI-RSSI reporting on PUSCH [and periodic L1 CLI-RSSI reporting on PUCCH] 2. Support of CLI-RSSI measurement resource configured in one UL subband only, in one DL subband only, or across two DL subbands only. 3. [Periodic and] aperiodic CLI-RSSI measurement resource FFS: whether each of components within brackets is separate FG or require reporting FFS: other potential component/FG; FFS; YES; n/a; [Aperiodic] L1 CLI-RSSI measurement and reporting is not supported; [Per Band]; TDD only; [n/a]; [n/a]; Optional with capability signalling. Agreement: Define the following FG 60-9b; Support of semi-persistent L1 SRS-RSRP measurement resource and aperiodic reporting; 1. Support of semi-persistent L1 SRS-RSRP measurement resource and aperiodic reporting; 60-9; YES; n/a; Semi-persistent L1 SRS-RSRP measurement resource and aperiodic reporting is not supported; [Per Band]; TDD only; n/a; n/a; Optional with capability signalling.. Proposal 9: Support UE feature for SBFD-aware UE of the following list of UE capabilities for L1 UE-to-UE CLI measurement and reporting: Index; Feature group; Components; Prerequisite FG; Consequence if the feature is not supported by the UE; Type; Note; Mandatory/Optional. X-7; L1 based CLI-RSSI aperiodic reporting; The support of aperiodic CLI-RSSI reporting using aperiodic CLI-RSSI resources.; 2-35,2-32; UE does not support L1 CLI-RSSI measurement and reporting; Per band; Note: CLI-RSSI reports are counted towards the maximum number CSI report (the simultaneousCSI-ReportsPerCC); Optional with capability signalling. X-7a; Slot offset; The support of minimum slot offset configuration for aperiodic CLI-RSSI-MeasurementResourceSet; X-4-1; candidate values: [1,32] The candidate value shall be per frequency range based.; Optional with capability signalling. X-7b; L1 based CLI -RSSI resources; 1. The maximum number of configured CLI-RSSI measurement resources per CC. 2. The maximum number of configured CLI-RSSI measurement resources across all CCs. 3. The maximum number of simultaneous CLI-RSSI resources per CC 4. The maximum number of simultaneous CLI-RSSI resources in active BWPs across all CCs 5. The maximum number of CLI-RSSI measurements resource per slot per CC 6. The maximum number of CLI-RSSI measurement resource per slot across all CC; X-4-1; UE does not support L1 CLI-RSSI measurement and reporting; Per band; Components 1-6 candidate values: TBD; Optional with capability signalling. X-7c; Periodic CLI-RSSI resource; Support of periodic CLI resources for L1-based aperiodic CLI reporting; X-4-1; UE does not support periodic CLI-RSSI resources; Per band; Optional with capability signalling. X-7d; Semi-persistent CLI-RSSI resource; Support of semi-persistent CLI resources for L1-based aperiodic CLI reporting; X-4-1; UE does not support semi-persistent CLI-RSSI resources; Per band; Optional with capability signalling. X-7e; Periodic CLI-RSSI resource; Support of periodic CLI resources for L1-based periodic CLI reporting; X-4-1; UE does not support periodic CLI-RSSI resources for periodic reporting; Per band; Optional with capability signalling. X-9; FDM DL reception and L1 CLI-RSSI measurement (Method #3); Simultaneous reception of DL signals/channels and CLI-RSSI measurement resource; X-4-1; UE doesn’t support Simultaneous reception of DL signals/channels and CLI RSSI measurement resource; Per band; Optional with capability signalling. |
| [15] | NTT DOCOMO, INC. | For L1 CLI-RSSI measurement and reporting, periodic L1 CLI-RSSI reporting on PUCCH should be included as part of FG 60-8, since both periodic and aperiodic beam reporting (2-21/22) are mandatory. And periodic L1 CLI-RSSI measurement resource should be part of FG 60-8. In legacy, UE reports maximum number of measurement resources across all CCs for L1 beam reporting in FG 2-24 (SSB/CSI-RS for beam management). Similarly, maximum number of configured L1 CLI-RSSI measurement resources (sum of periodic/semi-persistent/aperiodic) across all CCs should be reported. It can be included as part of FG 60-8. Similar to FG 60-9b for L1 SRS-RSRP measurement resource for aperiodic reporting, support of semi-persistent L1 CLI-RSSI measurement resource for aperiodic L1 CLI-RSSI reporting should be a separate FG. Proposal 9: For L1 CLI-RSSI measurement and reporting, - Periodic L1 CLI-RSSI reporting on PUCCH should be included as part of FG 60-8. - Periodic L1 CLI-RSSI measurement resource should be part of FG 60-8. - Supported maximum number of configured L1 CLI-RSSI measurement resources (sum of periodic/semi-persistent/aperiodic) across all CCs should be part of FG 60-8. - Define a separate FG for support of semi-persistent L1 CLI-RSSI measurement resource for aperiodic L1 CLI-RSSI reporting. |
| [16] | Ericsson | One of the FFS points is whether the components within the brackets are a separate FG. In our view, for SBFD L1 UE-UE CLI measurement was proposed to identify CLI interference that occur sporadically and randomly when two UEs are close to each other and are scheduled in an SBFD symbol/slot. Therefore, it makes sense that aperiodic reporting is defined as the basic capability. We don’t think periodic reporting is as useful; hence, we think this should be defined as a separate FG. FGs 60-8 and 60-9 contain only aperiodic reporting. Periodic reporting is moved to separate (new) FGs. Another discussion point is regarding the type of the FG. We think that all the FGs related to UL resource muting and CLI measurements need to be “Per band “and thus the square brackets can be removed. Support “Per band” type for all FGs related to UL resource muting and CLI measurement. For FG 60-8, component 2 “Support of CLI-RSSI measurement resource configured in one UL subband only, in one DL subband only, or across two DL subbands only.” conflicts with the following agreement from RAN1 #120bis: Agreement Configuration of Method 1 or Method 3 are not explicitly provided for L1 CLI-RSSI measurement and reporting. Configuration of Measurement resource type #1 or Measurement resource type #2 are not explicitly provided for L1 CLI-RSSI measurement. - CLI-RSSI measurement resource is configured in one UL subband only, in one DL subband only, or across two DL subbands only. - If CLI-RSSI measurement resource is configured across two DL subbands, the frequency resources of CLI-RSSI measurement resource are derived by excluding the frequency resources outside DL usable PRBs. This agreement means that the CLI-RSSI measurement resource and reporting framework does not need to explicitly provide whether the resource is configured in one UL subband only, one DL subband only or across two DL subbands. The UE simply performs measurements based on the configured resources. Hence we propose to remove Component 2 from the FG description For FG 60-8 delete component 2 “Support of CLI-RSSI measurement resource configured in one UL subband only, in one DL subband only, or across two DL subbands only” according to the latest agreement from RAN1 #120bis. We also think that CLI measurement is a general tool that is not dependent on whether the UE is “SBFD aware.” For example, Rel-19 UEs that are not SBFD-aware can still perform CLI-RSSI measurements if configured to do so, e.g., for a UE in a legacy TDD system that may be interfered in a DL slot by a UE in an SBFD system transmitting UL in an SBFD slot that overlaps the DL slot in the TDD system. Hence, we think that neither FGs 60-8 nor 60-9 should have FG 60-1 as a pre-requisite. FGs 60-8 and 60-9 are not defined with pre-requisite FGs. |

The following shows the latest status for the relevant FG(s).

| 60-8 | L1 CLI-RSSI measurement and [aperiodic] reporting | 1. Aperiodic L1 CLI-RSSI reporting on PUSCH [and periodic L1 CLI-RSSI reporting on PUCCH] 2. Support of CLI-RSSI measurement resource configured in one UL subband only, in one DL subband only, or across two DL subbands only. 3. [Periodic and] aperiodic CLI-RSSI measurement resource FFS: whether each of components within brackets is separate FG or require reporting FFS: other potential component/FG | FFS | YES | n/a | [Aperiodic] L1 CLI-RSSI measurement and reporting is not supported | [Per band] | TDD only | [n/a] | [n/a] |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Summary of companies’ views and discussion

FG structure and components - Whether periodic reporting should be part of the FG? - Separate FGs for periodic and aperiodic reporting of L1 CLI-RSSI: CATT, OPPO, vivo, Qualcomm, E///, - Periodic reporting as component(s) of the FG: HW/Hisi, ZTE, Nokia, MTK, SRPD/UNISOC, Samsung, Offino, CMCC, DCM, - Whether periodic resource should be part of the basic FG? - Separate FGs for periodic resource: QC, - Part of the basic FG: HW/Hisi, ZTE, Nokia, MTK, SPRD/UNISOC, Samsung, CATT, CMCC, DCM, - Whether semi-persistent resource should be part of the basic FG? - Separate FG: ZTE, SPRD/UNISOC, CATT, CMCC, QC, DCM, - Max. number of report setting config per BWP: HW/Hisi - Per P/SP/AP report: HW/Hisi - Maximum configured number of CLI-RSSI measurement resources across all CCs: HW/Hisi, SPRD/UNISOC (separate), Apple, QC (separate), DCM, - Maximum configured number of CLI-RSSI measurement resources per CC: vivo, QC (separate), - Maximum number of simultaneous CLI-RSSI measurement resources across all CCs: vivo, QC (separate), - Maximum number of simultaneous CLI-RSSI measurement resources per CC: QC (separate), - Maximum number of CLI-RSSI measurement resources per slot across all CCs: QC (separate), - Maximum number of CLI-RSSI measurement resources per slot per CC: QC (separate), Type of the FG(s) - Per band: ZTE, SPRD/UNISOC, CATT, Offino, MediaTek, CMCC, Xiaomi, Apple, DCM, E/// Prerequisite of the FG(s) - X-1 (basic FG for SBFD): ZTE, CATT, MediaTek, CMCC, HW/Hisi, OPPO, Xiaomi - 2-32 (CSI report framework): SPRD/UNISOC - No FG 60-1: ZTE, E///, Others - A separate FG to report the support of minimum slot offset configuration for aperiodic CLI-RSSI-MeasurementResourceSet: Qualcomm - A separate FG to report the support of FDM DL reception and L1 CLI-RSSI measurement (Method #3): Qualcomm - Remove component 2: E///.

Proposal 4-2-1:

Adopt “Per band” for FG 60-8

| Company | Comment |
| --- | --- |
| ZTE | Support |
| DOCOMO | Support. |
| Ericsson | Support |
| Huawei, HiSilicon | Support |
| Samsung | Support |
| QC | Support |

Proposal 4-2-2:

Confirm “Periodic and” in Component 3 of FG 60-8

| Company | Comment |
| --- | --- |
| ZTE | Support |
| DOCOMO | Support. |
| Ericsson | Periodic resources can be a separate |
| Huawei, HiSilicon | Support |
| Samsung | Support |
| QC | Same view as Ericsson, Periodic resources can be a separate |

Question 4-2-3:

Companies are encouraged to share views on whether “[and periodic L1 CLI-RSSI reporting on PUCCH]” in Component 1 of FG 60-8 should be confirmed or defined as a separate FG from FG 60-8

- Alt-1: “[and periodic L1 CLI-RSSI reporting on PUCCH]” in Component 1 of FG 60-8 should be confirmed

- Alt-2: “[and periodic L1 CLI-RSSI reporting on PUCCH]” in Component 1 of FG 60-8 should be removed, and define a separate FG for “support of periodic L1 CLI-RSSI reporting on PUCCH”

| Company | Comment |
| --- | --- |
| ZTE | Prefer Alt-1 |
| DOCOMO | We prefer Alt-1. Periodic L1 CLI-RSSI reporting on PUCCH should be included as part of FG 60-8, since both periodic and aperiodic beam reporting (2-21/22) are mandatory. |
| Ericsson | Alt-2. We think that the aperiodic reporting should be the main use for CLI reporting as CLI is sporadic and random. Periodic reporting can be a separate FG. |
| Huawei, HiSilicon | Alt.1. This is in line with the existing UE capability definitions |
| Samsung | Alt-1 |
| QC | Alt 2 |

Proposal 4-2-4:

Add a component for “Maximum number of configured CLI-RSSI measurement resources (sum of aperiodic/periodic/semi-persistent) across all CCs” in FG 60-8

| Company | Comment |
| --- | --- |
| ZTE | Fine |
| DOCOMO | Support. |
| Ericsson | Support in principle. However, We should discuss the need for Maximum number of configured resources across all CCs or per CC which is discussed in the next question. |
| Huawei, HiSilicon | Support |
| Samsung | Not needed. |
| QC | Fine |

Question 4-2-5:

Companies are encouraged to provide their views on the need to define the following reporting (related to maximum number of CLI-RSSI measurement resources in various units), and if needed, whether to define new FG to report them:

- Q4-2-5-1: Maximum number of configured L1 CLI-RSSI measurement resources (sum of aperiodic/semi-persistent/periodic) per CC

- Q4-2-5-2: Maximum number of simultaneous L1 CLI-RSSI measurement resources (sum of aperiodic/semi-persistent/periodic) across all CCs

- Q4-2-5-3: Maximum number of simultaneous L1 CLI-RSSI measurement resources (sum of aperiodic/semi-persistent/periodic) per CC

- Q4-2-5-4: Maximum number of L1 CLI-RSSI measurement resources (sum of aperiodic/semi-persistent/periodic) per slot across all CCs

- Q4-2-5-5: Maximum number of L1 CLI-RSSI measurement resources (sum of aperiodic/semi-persistent/periodic) per slot per CC

| Company | Comment |
| --- | --- |
| Moderator | If we follow legacy CLI FG and/or L1-RSRP reporting, it seems Proposal 4-2-4 would be sufficient. Proponents for the above potential components are encouraged to clarify the technical reason why it is needed. |
| ZTE | Agree with moderator |
| DOCOMO | We prefer not to define any reporting in Q4-2-5-1/Q4-2-5-2/Q4-2-5-3/Q4-2-5-4/Q4-2-5-5. We think L1 CLI reporting should follow legacy L1-RSRP reporting. |
| Ericsson | What is the motivation to have “configured” and “simultaneous” resources ? we do not see the need for having this fine granularity as well. |
| Huawei, HiSilicon | We don’t see the need of these capabilities. We would like to propose the following since these are reported separately for CSI and L1-beam reporting. We think they should be reported for L1 CLI-RSSI as well. - Maximum number of periodic CSI report setting per BWP for L1-CLI-RSSI report - Maximum number of aperiodic CSI report setting per BWP for L1-CLI-RSSI report - Maximum number of configured aperiodic CSI triggering states in CSI-AperiodicTriggerStateList per CC |
| Samsung | Agree with moderator. |
| QC | Support the proposal, which follows the UE FG for CSI framework. Proposal 4-2-4 only follows L3 CLI. |

Proposal 4-2-6:

Define new FG for support of semi-persistent L1 CLI-RSSI measurement resource

- Component: Support of semi-persistent CLI-RSSI measurement resource

- Prerequisite FG: FG 60-8

| 60-8a | Support of semi-persistent L1 CLI-RSSI measurement resource | 1. Support of semi-persistent L1 CLI-RSSI measurement resource | 60-8 | YES | n/a | Semi-persistent L1 CLI-RSSI measurement resource is not supported | [Per Band] | TDD only | n/a | n/a |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comment |
| --- | --- |
| ZTE | Support |
| DOCOMO | Support the proposal. |
| Ericsson | Support the proposal. This will be a new MAC CE, so better to have a separate FG. |
| Huawei, HiSilicon | Support |
| Samsung | Support |
| QC | Support |

4.3	FGs for L1-SRS-RSRP measurement

Following inputs are provided in contributions for the RAN1#121 meeting.

| [1] | Huawei, HiSilicon | L1-SRS-RSRP measurement and reporting For L1-SRS-RSRP measurement and reporting, the remaining issue is whether to include the support of periodic SRS-RSRP measurement resource in FG 60-9. Similar to the discussion above, we prefer to include the support of periodic SRS-RSRP measurement resource in FG 60-9. In addition, we suggest to separate the component into reporting and measurement resource similar to L1-CLI-RSSI measurement reporting since the reported values are defined separately. For L1-SRS-RSRP measurement and reporting, the following components are included (update in red) - Aperiodic L1 SRS-RSRP reporting on PUSCH - Maximum number of aperiodic CSI report setting per BWP for L1-SRS-RSRP report - Maximum number of configured aperiodic CSI triggering states in CSI-AperiodicTriggerStateList per CC - Periodic and aperiodic SRS-RSRP measurement resource - Maximum number of SRS-RSRP measurement resources (sum of aperiodic/periodic/semi-persistent) across all CCs configured to measure L1-SRSR-RSRP within a slot - Maximum number of SRS-RSRP measurement resources (sum of aperiodic/periodic/semi-persistent) across all CCs configured to measure L1-SRSR-RSRP - Maximum number of aperiodic SRS-RSRP measurement resources across all CCs configured to measure L1-SRS-RSRP |
| --- | --- | --- |
| [2] | ZTE Corporation, Sanechips | L1-based CLI measurement and reporting In RAN1#120bis, the following FGs were agreed for L1-based UE-UE CLI handling. 60-8; L1 CLI-RSSI measurement and [aperiodic] reporting; 1. Aperiodic L1 CLI-RSSI reporting on PUSCH [and periodic L1 CLI-RSSI reporting on PUCCH] 2. Support of CLI-RSSI measurement resource configured in one UL subband only, in one DL subband only, or across two DL subbands only. 3. [Periodic and] aperiodic CLI-RSSI measurement resource FFS: whether each of components within brackets is separate FG or require reporting FFS: other potential component/FG; FFS; YES; n/a; [Aperiodic] L1 CLI-RSSI measurement and reporting is not supported; [Per band]; TDD only; [n/a]; [n/a]; Optional with capability signalling. 60-9; L1 SRS-RSRP measurement and aperiodic reporting; 1. Aperiodic L1 SRS-RSRP reporting on PUSCH, and [periodic and] aperiodic SRS-RSRP measurement resource FFS: whether each of components within brackets is separate FG or require reporting FFS: other potential component/FG; FFS; YES; n/a; [Aperiodic] L1 SRS-RSRP measurement and reporting is not supported; [Per band]; TDD only; n/a; n/a; Optional with capability signalling. 60-9a; FDM-ed reception of DL signals/channels and L1 SRS-RSRP measurement resource; 1. Support of FDMed reception of DL signals/channels and L1 SRS-RSRP measurement resource; 60-9; YES; n/a; FDMed reception of DL signals/channels and L1 SRS-RSRP measurement resource is not supported; [Per band]; TDD only; n/a; n/a; Optional with capability signalling. 60-9b; Support of semi-persistent L1 SRS-RSRP measurement resource and aperiodic reporting; 1. Support of semi-persistent L1 SRS-RSRP measurement resource and aperiodic reporting; 60-9; YES; n/a; Semi-persistent L1 SRS-RSRP measurement resource and aperiodic reporting is not supported; [Per Band]; TDD only; n/a; n/a; Optional with capability signalling. Regarding the yellow highlighted pending parts, our views are summarized as below. - Prefer to include periodic and aperiodic measurement and reporting in one FG - Similar to FG 60-9b, we are ok to add one separate FG for semi-persistent L1 CLI-RSSI measurement resource and aperiodic reporting - Prefer NOT to add FG 60-1 as the prerequisite for FG 60-8 and 60-9 - OK with per band reporting Proposal 5: Support the following revisions on L1-based UE-UE measurement and reporting related FGs. 60-8; L1 CLI-RSSI measurement and aperiodic reporting; 1. Aperiodic L1 CLI-RSSI reporting on PUSCH and periodic L1 CLI-RSSI reporting on PUCCH 2. Support of CLI-RSSI measurement resource configured in one UL subband only, in one DL subband only, or across two DL subbands only. 3. Periodic and aperiodic CLI-RSSI measurement resource; YES; n/a; Aperiodic L1 CLI-RSSI measurement and reporting is not supported; Per band; TDD only; n/a; n/a; Optional with capability signalling. 60-8a; Support of semi-persistent L1 CLI-RSSI measurement resource and aperiodic reporting; 1. Support of semi-persistent L1 CLI-RSSI measurement resource and aperiodic reporting; 60-8; YES; n/a; Semi-persistent L1 S CLI-RSSI measurement resource and aperiodic reporting is not supported; Per Band; TDD only; n/a; n/a; Optional with capability signalling. 60-9; L1 SRS-RSRP measurement and aperiodic reporting; 1. Aperiodic L1 SRS-RSRP reporting on PUSCH, and periodic and aperiodic SRS-RSRP measurement resource; YES; n/a; Aperiodic L1 SRS-RSRP measurement and reporting is not supported; Per band; TDD only; n/a; n/a; Optional with capability signalling. 60-9a; FDM-ed reception of DL signals/channels and L1 SRS-RSRP measurement resource; 1. Support of FDMed reception of DL signals/channels and L1 SRS-RSRP measurement resource; 60-9; YES; n/a; FDMed reception of DL signals/channels and L1 SRS-RSRP measurement resource is not supported; Per band; TDD only; n/a; n/a; Optional with capability signalling. 60-9b; Support of semi-persistent L1 SRS-RSRP measurement resource and aperiodic reporting; 1. Support of semi-persistent L1 SRS-RSRP measurement resource and aperiodic reporting; 60-9; YES; n/a; Semi-persistent L1 SRS-RSRP measurement resource and aperiodic reporting is not supported; Per Band; TDD only; n/a; n/a; Optional with capability signalling. |
| [3] | vivo |  |
| [4] | Nokia | FGs 60-9, 60-9a, and 60-9b For FG 60-9, main open issue is the following: Aperiodic L1 SRS-RSRP reporting on PUSCH, and [periodic and] aperiodic SRS-RSRP measurement resource FFS: whether each of components within brackets is separate FG or require reporting FFS: other potential component/FG. Similar to FG 60-8, our first preference is to keep periodic and aperiodic SRS-RSRP measurement resource in a same FG. Proposal 8. For FG 60-9, support keeping periodic and aperiodic SRS-RSRP measurement resource in a same FG, i.e., support removing the square brackets in 60-9. |
| [5] | MediaTek Inc. | Whether support of periodic SRS-RSRP measurement resource is a separate FG In RAN1#120bis, an FG 60-9 was agreed on support for L1 SRS-RSRP measurement and aperiodic reporting. Aperiodic SRS-RSRP measurement resource was agreed as a component of this FG. It needs to be determined whether periodic SRS-RSRP measurement resource is part of this FG or is defined as a separate FG. In our view, support for periodic SRS-RSRP measurement resource should be part of FG 60-9 (i.e., periodic CLI-RSSI measurement resource is not a separate FG). - Support periodic SRS-RSRP measurement resource is part of FG 60-9. |
| [6] | Spreadtrum, UNISOC | In RAN1#120bis, following agreement was achieved. Agreement: Introduce following FG for L1 SRS-RSRP measurement/reporting. 60-9; L1 SRS-RSRP measurement and aperiodic reporting; 1. Aperiodic L1 SRS-RSRP reporting on PUSCH, and [periodic and] aperiodic SRS-RSRP measurement resource FFS: whether each of components within brackets is separate FG or require reporting FFS: other potential component/FG; FFS; YES; n/a; [Aperiodic] L1 SRS-RSRP measurement and reporting is not supported; [Per Band]; TDD only; n/a; n/a; Optional with capability signalling. Agreement: Introduce a new FG for FDMed reception of DL signals/channels and L1 SRS-RSRP measurement resource as follows 60-9a; FDM-ed reception of DL signals/channels and L1 SRS-RSRP measurement resource; 1. Support of FDMed reception of DL signals/channels and L1 SRS-RSRP measurement resource; 60-9; YES; n/a; FDMed reception of DL signals/channels and L1 SRS-RSRP measurement resource is not supported; [Per Band]; TDD only; n/a; n/a; Optional with capability signalling. Agreement: Define the following FG 60-9b; Support of semi-persistent L1 SRS-RSRP measurement resource and aperiodic reporting; 1. Support of semi-persistent L1 SRS-RSRP measurement resource and aperiodic reporting; 60-9; YES; n/a; Semi-persistent L1 SRS-RSRP measurement resource and aperiodic reporting is not supported; [Per Band]; TDD only; n/a; n/a; Optional with capability signalling.. Periodic and aperiodic SRS-RSRP measurement resource should be part of basic FG for L1 CLI-RSSI measurement. So the FFS part in component 1 of FG 60-9 can be confirmed. Max number of L1 SRS-RSRP measurement resource (sum of aperiodic/semi-persistent/periodic) across all CCs should be separate FG. Max number of CLI resource per resource set in Rel-16 can be reused, i.e. maxNrofCLI-SRS-Resources-r16 = 32. The prerequisite FG of FG 6-9 is FG 2-32 (Basic CSI feedback). The granularity of the FG 60-9/60-9a/60-9b is per Band. |
| [7] | Samsung | Proposal 15: For FG 60-9, support component 1 “Aperiodic L1 SRS-RSRP reporting on PUSCH, and periodic and aperiodic SRS-RSRP measurement resource. |
| [8] | Ofinno | FGs for CLI handling In the previous meeting, following FG is agreed for FDMed reception of DL signal/channels and CLI measurement resources. Agreement: Introduce a new FG for FDMed reception of DL signals/channels and L1 SRS-RSRP measurement resource as follows 60-9a; FDM-ed reception of DL signals/channels and L1 SRS-RSRP measurement resource; 1. Support of FDMed reception of DL signals/channels and L1 SRS-RSRP measurement resource; 60-9; YES; n/a; FDMed reception of DL signals/channels and L1 SRS-RSRP measurement resource is not supported; [Per Band]; TDD only; n/a; n/a; Optional with capability signalling. As discussed in our companion contribution [R1-2503733], UE behavior on this feature needs to be further clarified. L1-based CLI reporting is based on the CSI framework and triggered in an aperiodic manner, but delivered via UCI, requiring clear UE behavior for consistency. One key issue is the interaction with PRS, which is scheduled by the LMF rather than the gNB. Since PRS scheduling may conflict with CLI measurement resources, it is important to clarify that the DL signals referenced by the FG include PRS. To handle conflicts, the UE should measure both PRS and CLI when frequency-division multiplexed (FDM), otherwise, treat the CLI measurement resource as a DL signal and apply PRS prioritization rules. This approach ensures reliable and unambiguous CLI reporting. Proposal 6. Update 60-9a to include PRS as a DL signals/channels. |
| [9] | CATT | FGs for L1-SRS-RSRP measurement Regarding UE features for L1-SRS-RSRP measurement, the following was agreed. Agreement: Introduce following FG for L1 SRS-RSRP measurement/reporting. 60-9; L1 SRS-RSRP measurement and aperiodic reporting; 1. Aperiodic L1 SRS-RSRP reporting on PUSCH, and [periodic and] aperiodic SRS-RSRP measurement resource FFS: whether each of components within brackets is separate FG or require reporting FFS: other potential component/FG; FFS; YES; n/a; [Aperiodic] L1 SRS-RSRP measurement and reporting is not supported; [Per Band]; TDD only; n/a; n/a; Optional with capability signalling. Agreement: Introduce a new FG for FDMed reception of DL signals/channels and L1 SRS-RSRP measurement resource as follows 60-9a; FDM-ed reception of DL signals/channels and L1 SRS-RSRP measurement resource; 1. Support of FDMed reception of DL signals/channels and L1 SRS-RSRP measurement resource; 60-9; YES; n/a; FDMed reception of DL signals/channels and L1 SRS-RSRP measurement resource is not supported; [Per Band]; TDD only; n/a; n/a; Optional with capability signalling. Agreement: Define the following FG 60-9b; Support of semi-persistent L1 SRS-RSRP measurement resource and aperiodic reporting; 1. Support of semi-persistent L1 SRS-RSRP measurement resource and aperiodic reporting; 60-9; YES; n/a; Semi-persistent L1 SRS-RSRP measurement resource and aperiodic reporting is not supported; [Per Band]; TDD only; n/a; n/a; Optional with capability signalling. Similar as FG 60-8, one FG for periodic and aperiodic CLI measurement resource is preferred for L1 SRS-RSRP measurement and aperiodic reporting. Proposal 10: Support the following FG for L1 SRS-RSRP measurement. 60-9; L1 SRS-RSRP measurement and aperiodic reporting; 1. Aperiodic L1 SRS-RSRP reporting on PUSCH, and periodic and aperiodic SRS-RSRP measurement resource; FFS; YES; n/a; Aperiodic L1 SRS-RSRP measurement and reporting is not supported; Per Band; TDD only; n/a; n/a; Optional with capability signalling. |
| [10] | CMCC |  |
| [11] | Xiaomi | For FG 60-9, 60-9a, 60-8, 60-9b, 60-7 and 60-7a, it is proposed to adopt “Per band” type. Proposal 7: Adopt “Per band” type for FG 60-9, 60-9a, 60-8, 60-9b, 60-7 and 60-7a. |
| [12] | OPPO | For FG 60-8 & FG 60-9 From UE implementation point of view, there is no much difference on complexity between periodic CLI measurement and aperiodic CLI measurement, but the difference between periodic reporting and aperiodic reporting is more obvious. Proposal 7: FG 60-8 is split into two FG3, one for periodic reporting on PUCCH, another for aperiodic reporting on PUSCH. Proposal 8: FG 60-9 component 1 contains both periodic and aperiodic SRS-RSRP measurement resources. |
| [13] | Apple | 60. NR_duplex_evo; 60-3-1; Support of L1 based CLI measurement method within the active DL BWP; - Method#1: UE measures RSSI within DL subband - Method#2: UE measures RSRP of aggressor UE within UL subband - Method#3: UE measures RSSI within UL subband; Yes; n/a; L1 based CLI measurement and report is not supported; Per band; n/a; n/a; n/a; Optional with capability signalling. 60. NR_duplex_evo; 60-3-2; Support of CLI-RSSI measurement resource type; - Measurement resource type#1: One CLI-RSSI measurement resource is configured within a DL subband - Measurement resource type#2: One CLI-RSSI measurement resource is configured across two DL subbands; Yes; n/a; Per band; n/a; n/a; n/a; Optional with capability signalling. 60. NR_duplex_evo; 60-3-3; Support of L1 CLI-RSSI and L1-SRS-RSRP measurement; - The max number of L1-CLI-RSSI/L1-SRS-RSRP resources (sum of aperiodic/periodic/semipersistent) across all CCs configured to measure L1-CLI-RSSI or L1-SRS-RSRP within a slot shall not exceed M - The max number of aperiodic L1-CLI-RSSI resources across all CCs configured to measure CLI-RSSI shall not exceed MA_1 - The max number of aperiodic L1-SRS-RSRP resources across all CCs configured to measure SRS-RSRP shall not exceed MB_1; Yes; n/a; Per band; n/a; n/a; n/a; Optional with capability signalling. |
| [14] | Qualcomm Incorporated | UE features for L1-CLI measurements and reporting For L1 CLI measurement and reporting, RAN1 agreed on details of configuration of aperiodic, periodic and semi-persistent SRS-RSRP and CLI-RSSI measurement resources, configuration/determination of QCL typed per aperiodic, periodic and semi-persistent SRS-RSRP and CLI-RSSI measurement resource, and also on details of configuration of at least aperiodic reporting. RAN1 also agreed on Method #1 that UE measures RSSI within DL subband, Method #2 that UE measures RSRP of aggressor UE within UL subband, and Method #3 for CLI measurement that UE measures RSSI within UL subband. We also support have a UE FG for minimum slot offset configuration for aperiodic CLI-RSSI-MeasurementResourceSet and please refer to our contribution for CLI handling for the reasoning. In RAN1 #120bis meeting, RAN1 agreed that: Agreement: Introduce following FG for L1 SRS-RSRP measurement/reporting. 60-9; L1 SRS-RSRP measurement and aperiodic reporting; 1. Aperiodic L1 SRS-RSRP reporting on PUSCH, and [periodic and] aperiodic SRS-RSRP measurement resource FFS: whether each of components within brackets is separate FG or require reporting FFS: other potential component/FG; FFS; YES; n/a; [Aperiodic] L1 SRS-RSRP measurement and reporting is not supported; [Per Band]; TDD only; n/a; n/a; Optional with capability signalling. Agreement: Introduce a new FG for FDMed reception of DL signals/channels and L1 SRS-RSRP measurement resource as follows 60-9a; FDM-ed reception of DL signals/channels and L1 SRS-RSRP measurement resource; 1. Support of FDMed reception of DL signals/channels and L1 SRS-RSRP measurement resource; 60-9; YES; n/a; FDMed reception of DL signals/channels and L1 SRS-RSRP measurement resource is not supported; [Per Band]; TDD only; n/a; n/a; Optional with capability signalling. Agreement: Introduce following FG for L1 CLI-RSSI measurement/reporting. 60-8; L1 CLI-RSSI measurement and [aperiodic] reporting; 1. Aperiodic L1 CLI-RSSI reporting on PUSCH [and periodic L1 CLI-RSSI reporting on PUCCH] 2. Support of CLI-RSSI measurement resource configured in one UL subband only, in one DL subband only, or across two DL subbands only. 3. [Periodic and] aperiodic CLI-RSSI measurement resource FFS: whether each of components within brackets is separate FG or require reporting FFS: other potential component/FG; FFS; YES; n/a; [Aperiodic] L1 CLI-RSSI measurement and reporting is not supported; [Per Band]; TDD only; [n/a]; [n/a]; Optional with capability signalling. Agreement: Define the following FG 60-9b; Support of semi-persistent L1 SRS-RSRP measurement resource and aperiodic reporting; 1. Support of semi-persistent L1 SRS-RSRP measurement resource and aperiodic reporting; 60-9; YES; n/a; Semi-persistent L1 SRS-RSRP measurement resource and aperiodic reporting is not supported; [Per Band]; TDD only; n/a; n/a; Optional with capability signalling.. Proposal 9: Support UE feature for SBFD-aware UE of the following list of UE capabilities for L1 UE-to-UE CLI measurement and reporting: Index; Feature group; Components; Prerequisite FG; Consequence if the feature is not supported by the UE; Type; Note; Mandatory/Optional. X-8; L1 based CLI SRS-RSRP aperiodic reporting; The support of aperiodic SRS-RSRP reporting using aperiodic SRS-RSRP resources.; 2-35,2-32; UE does not support L1 SRS-RSRP measurement and reporting.; Note: CLI SRS-RSPR reports are counted towards the maximum number CSI report (the simultaneousCSI-ReportsPerCC); Optional with capability signalling. X-8a; Slot offset; The support of minimum slot offset configuration for aperiodic SRS-RSRP-MeasurementResourceSet; X-4-2; candidate values: [1,32] The candidate value shall be per frequency range based.; Optional with capability signalling. X-8b; L1 based CLI-SRS-RSRP resources; 1. The maximum number of configured CLI-SRS-RSRP measurement resources per CC. 2. The maximum number of configured CLI-SRS-RSRP measurement resources across all CCs. 3. The maximum number of simultaneous CLI-SRS-RSRP resources per CC 4. The maximum number of simultaneous CLI-SRS-RSRP resources in active BWPs across all CCs 5. The maximum number of CLI-SRS-RSRP measurements resource per slot per CC 6. The maximum number of CLI-SRS-RSRP measurement resource per slot across all CC; X-4-2; UE does not support L1 SRS-RSRP measurement and reporting.; Per band; Components 1-6 candidate values: TBD; Optional with capability signalling. X-8c; Periodic SRS-RSRP resource; Support of Periodic CLI SRS-RSRP resources for L1-based aperiodic CLI reporting; X-4-2; UE does not support periodic CLI SRS-RSRP resources; Per band; Optional with capability signalling. X-8d; Semi-persistent SRS-RSRP resource; Support of semi-persistent CLI SRS-RSRP resources for L1-based aperiodic CLI reporting; X-4-2; UE does not support semi-persistent CLI SRS-RSRP resources; Per band; Optional with capability signalling. X-10; FDM DL reception and L1 SRS-RSRP measurement (Method #2); Simultaneous reception of DL signals/channels and SRS-RSRP measurement resource; X-4-2; UE doesn’t support Simultaneous reception of DL signals/channels and SRS-RSRP measurement resource; Per band; Optional with capability signalling. |
| [15] | NTT DOCOMO, INC. | For L1 SRS-RSRP measurement and reporting, periodic L1 SRS-RSRP measurement resource should be part of FG 60-9. Similarly, maximum number of configured L1 SRS-RSRP measurement resources (sum of periodic/semi-persistent/aperiodic) across all CCs should be reported as part of FG 60-9. Proposal 10: For L1 SRS-RSRP measurement and reporting, - Periodic L1 SRS-RSRP measurement resource should be part of FG 60-9. - Supported maximum number of configured L1 SRS-RSRP measurement resources (sum of periodic/semi-persistent/aperiodic) across all CCs should be part of FG 60-9. |
| [16] | Ericsson | One of the FFS points is whether the components within the brackets are a separate FG. In our view, for SBFD L1 UE-UE CLI measurement was proposed to identify CLI interference that occur sporadically and randomly when two UEs are close to each other and are scheduled in an SBFD symbol/slot. Therefore, it makes sense that aperiodic reporting is defined as the basic capability. We don’t think periodic reporting is as useful; hence, we think this should be defined as a separate FG. FGs 60-8 and 60-9 contain only aperiodic reporting. Periodic reporting is moved to separate (new) FGs. Another discussion point is regarding the type of the FG. We think that all the FGs related to UL resource muting and CLI measurements need to be “Per band “and thus the square brackets can be removed. Support “Per band” type for all FGs related to UL resource muting and CLI measurement. For FG 60-8, component 2 “Support of CLI-RSSI measurement resource configured in one UL subband only, in one DL subband only, or across two DL subbands only.” conflicts with the following agreement from RAN1 #120bis: Agreement Configuration of Method 1 or Method 3 are not explicitly provided for L1 CLI-RSSI measurement and reporting. Configuration of Measurement resource type #1 or Measurement resource type #2 are not explicitly provided for L1 CLI-RSSI measurement. - CLI-RSSI measurement resource is configured in one UL subband only, in one DL subband only, or across two DL subbands only. - If CLI-RSSI measurement resource is configured across two DL subbands, the frequency resources of CLI-RSSI measurement resource are derived by excluding the frequency resources outside DL usable PRBs. This agreement means that the CLI-RSSI measurement resource and reporting framework does not need to explicitly provide whether the resource is configured in one UL subband only, one DL subband only or across two DL subbands. The UE simply performs measurements based on the configured resources. Hence we propose to remove Component 2 from the FG description For FG 60-8 delete component 2 “Support of CLI-RSSI measurement resource configured in one UL subband only, in one DL subband only, or across two DL subbands only” according to the latest agreement from RAN1 #120bis. We also think that CLI measurement is a general tool that is not dependent on whether the UE is “SBFD aware.” For example, Rel-19 UEs that are not SBFD-aware can still perform CLI-RSSI measurements if configured to do so, e.g., for a UE in a legacy TDD system that may be interfered in a DL slot by a UE in an SBFD system transmitting UL in an SBFD slot that overlaps the DL slot in the TDD system. Hence, we think that neither FGs 60-8 nor 60-9 should have FG 60-1 as a pre-requisite. FGs 60-8 and 60-9 are not defined with pre-requisite FGs. |

The following shows the latest status for the relevant FG(s).

| 60-9 | L1 SRS-RSRP measurement and aperiodic reporting | 1. Aperiodic L1 SRS-RSRP reporting on PUSCH, and [periodic and] aperiodic SRS-RSRP measurement resource FFS: whether each of components within brackets is separate FG or require reporting FFS: other potential component/FG | FFS | YES | n/a | [Aperiodic] L1 SRS-RSRP measurement and reporting is not supported | [Per band] | TDD only | n/a | n/a |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 60-9a | FDM-ed reception of DL signals/channels and L1 SRS-RSRP measurement resource | 1. Support of FDMed reception of DL signals/channels and L1 SRS-RSRP measurement resource | 60-9 | YES | n/a | FDMed reception of DL signals/channels and L1 SRS-RSRP measurement resource is not supported | [Per band] | TDD only | n/a | n/a |  | Optional with capability signalling |
| 60-9b | Support of semi-persistent L1 SRS-RSRP measurement resource and aperiodic reporting | 1. Support of semi-persistent L1 SRS-RSRP measurement resource and aperiodic reporting | 60-9 | YES | n/a | Semi-persistent L1 SRS-RSRP measurement resource and aperiodic reporting is not supported | [Per Band] | TDD only | n/a | n/a |  | Optional with capability signalling |

Summary of companies’ views and discussion

FG structure and components - Whether to include periodic resource in the basic FG? - Basic FG: HW/Hisi, ZTE, Nokia, MTK, SPRD/UNISOC, Samsung, CATT, OPPO, DCM, - Separate FG: QC, - Max. number of aperiodic CSI report setting per BWP: Hw/Hisi, - Max. number of SRS-RSRP measurement resources across all CCs: Hw/Hisi, SPRD/UNISOC (separate), Apple, QC, DCM, - Within a slot: Hw/Hisi, - Aperiodic resource: HW/Hisi, Type of the FG(s) - Per band: ZTE, SPRD/UNISOC, CATT, Offino, MediaTek, Xiaomi, Apple, QC, DOCOMO - Per UE: DOCOMO Prerequisite of the FG(s) - X-1 (basic FG for SBFD): CATT, MediaTek, CMCC, HW/Hisi, OPPO, Xiaomi - 2-35 (CSI report framework): SPRD/UNISOC - No FG 60-1: ZTE, E///, Others - Update FG 60-9a to include PRS as a DL signals/channels: Ofinno, - A separate FG to report the support of minimum slot offset configuration for aperiodic SRS-RSRP-MeasurementResourceSet: Qualcomm

Proposal 4-3-1:

- Confirm [Periodic and] in Component 1 of FG 60-9.

- Confirm “60-9” as a prerequisite FG for FG 60-9b

| Company | Comment |
| --- | --- |
| ZTE | Support |
| DOCOMO | We support to confirm [Periodic and] in Component 1 of FG 60-9, and we support to confirm “60-9” as a prerequisite FG for FG 60-9b. |
| Ericsson | Support |
| Huawei, HiSilicon | Support |
| Samsung | Support |
| QC | We think similar as RSSI resource, periodic resource can be a separate FG. |

Proposal 4-3-2:

Add a component for “Maximum number of SRS-RSRP measurement resources across all CCs” in FG 60-9

| Company | Comment |
| --- | --- |
| ZTE | Support |
| DOCOMO | Support the proposal. |
| Ericsson | OK. Same discussion needed as CLI-RSSI measurement. |
| Huawei, HiSilicon | Support Similar to L3 based SRS-RSRP measurement, we think UE shall also report the maximum number of SRS-RSRP measurement resources per slot - Maximum number of SRS-RSRP measurement resources (sum of aperiodic/periodic/semi-persistent) across all CCs configured to measure L1-SRSR-RSRP within a slot Similar to L1 beam reporting, we think UE shall also report the max number of aperiodic SRS-RSRP measurement resources across all CCs configured to measure L1-SRS-RSRP - Maximum number of aperiodic SRS-RSRP measurement resources across all CCs configured to measure L1-SRS-RSRP Besides, we would like to propose the following since these are reported separately for CSI and L1-beam reporting. We think they should be reported for L1 SRS-RSRP as well. - Maximum number of aperiodic CSI report setting per BWP for L1-SRS-RSRP report - Maximum number of configured aperiodic CSI triggering states in CSI-AperiodicTriggerStateList per CC |
| Samsung | Not needed |
| QC | Support. And also support additional UE FG as proposal 4-2-5. |

Proposal 4-3-3:

Adopt “Per band” for FG 60-9/9a/9b

| Company | Comment |
| --- | --- |
| ZTE | Support |
| DOCOMO | Support. |
| Ericsson | Support. |
| Huawei, HiSilicon | Support |
| Samsung | Support |
| QC | Support |

- Conclusions

TBD

References

- R1-2503264	UE features for evolution of NR duplex operation	Huawei, HiSilicon

- R1-2503333	Discussion on UE features for Rel-19 SBFD	ZTE Corporation, Sanechips

- R1-2503387	Discussion on UE features for evolution of NR duplex operation: SBFD	vivo

- R1-2503401	SBFD UE features	Nokia

- R1-2503444	Discussion on UE features for evolution of NR duplex operation	MediaTek Inc.

- R1-2503533	Discussion on UE features for evolution of NR duplex operation: SBFD	Spreadtrum, UNISOC

- R1-2503593	UE features for Rel-19 Duplex Evolution for NR	Samsung

- R1-2503742	Discussion on Rel-19 NR UE features for evolution of NR duplex operation	Ofinno

- R1-2503783	UE features for SBFD	CATT

- R1-2503852	Discussion on UE features for evolution of NR duplex operation: SBFD	CMCC

- R1-2503902	UE features for SBFD	Xiaomi

- R1-2504212	Discussion on UE features for Rel-19 NR Duplex	OPPO

- R1-2504354	Views on UE features for Rel-19 SBFD	Apple

- R1-2504421	UE features for evolution of NR duplex operation	Qualcomm Incorporated

- R1-2504525	Discussion on UE features for evolution of NR duplex operation	NTT DOCOMO, INC.

- R1-2504614	UE features for SBFD	Ericsson

Appendix: SBFD UE feature list after RAN1#120bis

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 60. NR_duplex_evo | 60-1 | Basic feature for SBFD-aware UE TX/RX/measurement behaviour and procedures | 1. Support of cell-common configuration of time and frequency location of SBFD subbands 2. Support of UL transmission in one UL subband and DL reception in one[/two] DL subband in SBFD symbols 3. Support of link direction determination based on configured/scheduled transmissions/receptions and collision handling 4. Support of UL transmissions or DL receptions across SBFD symbols and non-SBFD symbols in different slots, where the transmissions or receptions are restricted to SBFD symbols or non-SBFD symbols (Configuration 1) 5. Support of separate CSI reports for SBFD and non-SBFD symbols 6. Support of separate PUCCH frequency resource configuration in SBFD symbols and non-SBFD symbols [FFS: 7. Support of separate UL power control parameters based on unified TCI framework in SBFD symbols and non-SBFD symbols] 8. Support of separate PUSCH frequency hopping offset in SBFD symbols and non-SBFD symbols [FFS: 9. Support of separate SRS resource set configuration in SBFD symbols and non-SBFD symbols] FFS: Whether to include two DL subband support in component 2 FFS: other component (which may be separate FG) |  | YES | n/a | Basic feature for SBFD-aware UE TX/RX/ measurement behaviour and procedures is not supported | Per band | TDD only | n/a | n/a | [Note: Only UL transmissions are allowed within UL subband and only DL receptions (except for CLI measurement) are allowed within DL subband(s)] [Note: The maximum number of UL subbands for SBFD operation in an SBFD symbol within a TDD carrier is one.] | Optional with capability signalling |
| 60. NR_duplex_evo | 60-2 | [Transmission and] Reception across SBFD symbols and non-SBFD symbols in different slots (Configuration 2) | 1. Support of PDSCH reception across SBFD symbols and non-SBFD symbols (Configuration 2) [2. Support of UL transmission across SBFD symbols and non-SBFD symbols (Configuration 2)] FFS: Other component(s) FFS: whether component 2 is defined as separate FG |  | YES | n/a | [Transmission and] Reception across SBFD symbols and non-SBFD symbols in different slots (Configuration 2) is not supported | [Per Band] | TDD only | n/a | n/a |  | Optional with capability signalling |
| 60. NR_duplex_evo | 60-3 | SBFD random access operation based on one single RACH configuration in RRC_IDLE/INACTIVE/CONNECTED mode | Support of SBFD random access operation based on one single RACH configuration in RRC_IDLE/INACTIVE/CONNECTED mode (RACH configuration Option 1) FFS: Other component(s) which may be separate FG | 60-1 | YES | n/a | SBFD random access operation based on one single RACH configuration in RRC_IDLE/INACTIVE/CONNECTED mode is not supported | Per band | TDD only | n/a | n/a |  | Optional with capability signalling |
| 60. NR_duplex_evo | 60-4 | SBFD random access operation based on two separate RACH configurations, including one legacy RACH configuration and one additional RACH configuration in RRC_IDLE/INACTIVE/CONNECTED mode | Support of SBFD random access operation based on two separate RACH configurations, including one legacy RACH configuration and one additional RACH configuration in RRC_IDLE/INACTIVE/CONNECTED mode (RACH configuration Option 2) FFS: Other component(s) which may be separate FG | [60-1, 60-3] | YES | n/a | SBFD random access operation based on two separate RACH configurations in RRC_IDLE/INACTIVE/CONNECTED mode is not supported | Per band | TDD only | n/a | n/a |  | Optional with capability signalling |
| 60. NR_duplex_evo | 60-7 | UL resource muting for CP-OFDM waveform | 1. Support semi-static configuration of time location and frequency location of UL resource muting for CP-OFDM waveform - Up to two UL muting symbols within a slot for PUSCH - A configurable comb offset {0, 1} for the UL muting symbol [- FFS: Whether this FG only support one common pattern of UL muting for DG/Type 2 CG PUSCH and Type 1 CG PUSCH] 2. Support dynamic on/off indication of the configured UL muting symbols by TDRA field in DCI for DG-PUSCH and Type-2 CG PUSCH 3. Support semi-static determination of UL muting symbols for Type-1 CG PUSCH FFS: other potential component/FG/additional restriction | [FFS] | YES | n/a | UL resource muting for CP-OFDM waveform is not supported | [Per band] | TDD only | n/a | n/a |  | Optional with capability signalling |
| 60. NR_duplex_evo | 60-7a | UL resource muting for DFTS-OFDM waveform | 1. Support semi-static configuration of time location and frequency location of UL resource muting for DFTS-OFDM waveform - Up to two UL muting symbols within a slot for PUSCH - A configurable comb offset {0, 1} for the UL muting symbol [- FFS: Whether this FG only support one common pattern of UL muting for DG/Type 2 CG PUSCH and Type 1 CG PUSCH] 2. Support dynamic on/off indication of the configured UL muting symbols by TDRA field in DCI for DG-PUSCH and Type-2 CG PUSCH 3. Support semi-static determination of UL muting symbols for Type-1 CG PUSCH FFS: other potential component/FG/additional restriction | [FFS] | YES | n/a | UL resource muting for DFTS-OFDM waveform is not supported | [Per Band] | TDD only | n/a | n/a |  | Optional with capability signalling |
| 60. NR_duplex_evo | 60-8 | L1 CLI-RSSI measurement and [aperiodic] reporting | 1. Aperiodic L1 CLI-RSSI reporting on PUSCH [and periodic L1 CLI-RSSI reporting on PUCCH] 2. Support of CLI-RSSI measurement resource configured in one UL subband only, in one DL subband only, or across two DL subbands only. 3. [Periodic and] aperiodic CLI-RSSI measurement resource FFS: whether each of components within brackets is separate FG or require reporting FFS: other potential component/FG | FFS | YES | n/a | [Aperiodic] L1 CLI-RSSI measurement and reporting is not supported | [Per band] | TDD only | [n/a] | [n/a] |  | Optional with capability signalling |
| 60. NR_duplex_evo | 60-9 | L1 SRS-RSRP measurement and aperiodic reporting | 1. Aperiodic L1 SRS-RSRP reporting on PUSCH, and [periodic and] aperiodic SRS-RSRP measurement resource FFS: whether each of components within brackets is separate FG or require reporting FFS: other potential component/FG | FFS | YES | n/a | [Aperiodic] L1 SRS-RSRP measurement and reporting is not supported | [Per band] | TDD only | n/a | n/a |  | Optional with capability signalling |
| 60. NR_duplex_evo | 60-9a | FDM-ed reception of DL signals/channels and L1 SRS-RSRP measurement resource | 1. Support of FDMed reception of DL signals/channels and L1 SRS-RSRP measurement resource | 60-9 | YES | n/a | FDMed reception of DL signals/channels and L1 SRS-RSRP measurement resource is not supported | [Per band] | TDD only | n/a | n/a |  | Optional with capability signalling |
| 60. NR_duplex_evo | 60-9b | Support of semi-persistent L1 SRS-RSRP measurement resource and aperiodic reporting | 1. Support of semi-persistent L1 SRS-RSRP measurement resource and aperiodic reporting | 60-9 | YES | n/a | Semi-persistent L1 SRS-RSRP measurement resource and aperiodic reporting is not supported | [Per Band] | TDD only | n/a | n/a |  | Optional with capability signalling |