pat3GPP TSG RAN WG1 #121							   R1-2504682

St Julian’s, Malta, May 19th – 23th, 2025

Source:	Moderator (MediaTek)

Title:	FL summary 4 for on-demand SIB1 in idle/inactive mode

Agenda item:	9.5.2

Document for:	Discussion and Decision

Introduction

This document is used to assist the discussions for “9.5.2 On-demand SIB1 for idle/inactive mode UEs” of the Rel-19 working item on “Enhancements of network energy savings for NR” based on proposals from companies.

This contribution provides discussion points in Section 2, resulted RAN1 conclusion/agreement in Section 3, and reference (companies tdoc list) in Section 4.

Discussion points

- Issue 1: Whether/how to support transmission for on-demand SIB1 with the association of SSB(s) based on a received UL-WUS

Background

In RAN1 #120-bis, it was agreed that:

Agreement

The following agreement is updated as follows:

The UE assumes that, in the OD-SIB1 window, PDCCH for an OD-SIB1 message is transmitted in PDCCH monitoring occasions corresponding only to at least the SSB associated with the transmitted PRACH for UL-WUS if this is indicated via UL WUS configuration by a 1-bit indication.

- If the 1-bit indication is included in UL WUS config and indicate as TRUE, the UE assumes that, in the OD-SIB1 window, PDCCH for an OD-SIB1 message is transmitted in PDCCH monitoring occasions corresponding only to the SSB associated with the PRACH for UL-WUS.

- If the 1-bit indication is not included in UL WUS config or the 1-bit indication is FALSE, the UE assumes the same as legacy, i.e., in the OD-SIB1 window, PDCCH for an OD-SIB1 message is transmitted in at least one PDCCH monitoring occasion corresponding to each transmitted SSB.

- Further study possible down selection from the following options to indicate additional information related to SSBs associated with PDCCH monitoring occasions.

  - Alt1: The information is provided in UL-WUS config

  - Alt2: The information is provided in RAR in response to UL-WUS transmission

  - Alt3: No additional information is provided

For this topic, companies’ views in RAN1 #121 are collected below:

China Telecom: Alt 3

MTK: Alt 3

Lenovo: Alt 3

ETRI: Alt 3

Transsion: Alt 3

OPPO:

Proposal 5: RAN1 to adopt TP#2 below.

--------------- TP#2 of TS 38.213 in Clause 23 -----------------------

If the UE identifies a RAPID associated with a corresponding PRACH transmission from the UE in a PDSCH reception scheduled by the DCI format 1_0 with CRC scrambled by the RA-RNTI, the UE can be indicated by higher layers to monitor PDCCH on the second cell to detect a DCI format 1_0 with CRC scrambled by the SI-RNTI according to a Type0-PDCCH CSS set provided by SearchSpaceZero. If the UE is provided XYZ, the UE monitors PDCCH only in monitoring occasions associated with the SS/PBCH block. The UE starts monitoring PDCCH to detect the DCI format 1_0 with CRC scrambled by the SI-RNTI after a number of slots provided by od-sib1-windowStartOffset from the starting slot of the window controlled by ra_ResponseWindow, and for a number of slots provided by od-sib1-WindowDuration.

--------------- end of TP#2-------------------------------------------------------

Panasonic: Alt 3

LG:

Proposal #3: Support gNB to provide the additional information on which SSBs are currently being transmitted by the UL WUS configuration or by RAR in response to the UL WUS.

Proposal #4: Support gNB to configure SSB group each of which includes one or multiple SSBs by UL WUS configuration. If the UE transmits WUS corresponding to an SSB, the UE assumes that, in the OD-SIB1 window, PDCCH for an OD-SIB1 message is transmitted in PDCCH monitoring occasions corresponding only to SSB(s) associated with SSB group including the SSB.

Apple:

Proposal 3: Regarding indicating additional information related to SSBs associated with PDCCH monitoring occasions, do not support Alt 1 The information is provided in UL-WUS config, while Alt 2 the information is provided in RAR in response to UL-WUS transmission can be considered.

QC:

Proposal 1: For PDCCH monitoring for OD-SIB1, if the 1-bit indication is included in UL WUS config or the 1-bit indication is TRUE, the UE is provided with an indication of a list of the SSBs for monitoring PDCCH for OD-SIB1 in RAR in response to UL WUS transmission.

DENSO: Alt 3

DCM: Alt 3

CAICT:

Proposal 1: The information provided in RAR in response to UL-WUS transmission can indicate additional information related to SSBs associated with PDCCH monitoring occasions.

CEWiT:

Proposal 1: Support Alt 1: “The information is provided in UL-WUS config” to indicate additional information

related to SSBs associated with PDCCH monitoring occasions.

FUTUREWEI

Proposal 1: Support Alt 3: No additional information is provided to indicate additional information related to SSBs associated with PDCCH monitoring occasions.

Huawei, HiSilicon

Proposal 2: Support Alt3, i.e., no additional information is provided to indicate additional information related to SSBs associated with PDCCH monitoring occasions.

ZTE, Sanechips

Observation 1: The motivation to indicate additional information related to SSBs associated with PDCCH monitoring occasions is not clear.

Observation 2: The gNB does not know which beams other than the beam corresponding to UL WUS have better performance.

Observation 3: Alt 1 leads to frequent UL WUS configuration updates, increases the interaction between NES cell and cell A and the implementation complexity.

Observation 4: There is no clear gain observed from Alt 2. Alt2 is excluded by RAN2 agreement.

Proposal 1: For additional information related to SSBs associated with PDCCH monitoring occasions, Alt 3 (No additional information is provided) is supported.

vivo

Proposal 7: Indication of additional information related to SSBs associated with PDCCH monitoring occasions is not supported, i.e., Alt.3.

Nokia

Observation-3: Practically, the SSB beam can be narrow, e.g. for UE with mobility and frequency range with FR2, where the UE can be very likely located at the beam-edge between two narrow SSB beams.

Proposal-5: To improve the reliability of UE reception of OD-SIB1, it is beneficial for gNB transmission of OD-SIB1 also in the neighbouring SSB beams of the SSB beam associated with the UL WUS transmission.

Proposal-6: gNB can indicate in the UL WUS configuration on which set of neighbouring SSB beams the OD-SIB1 are transmitted together with the SSB beam associated with the UL WUS transmission.

Proposal-7: RAN1 to discuss on how to indicate the set of neighbouring SSB beams with OD-SIB1 transmission.

NEC

Proposal 4: From the options to indicate additional information related to SSBs associated with PDCCH monitoring occasions, support Alt3: No additional information is provided.

Spreadtrum, UNISOC

Proposal 1 Support option 3 for indication of OD-SIB1 Tx beams.

- Option 3: No additional information is provided.

Samsung

Proposal 3: Support Alt 3 (No additional information is provided)

Ofinno

Proposal 1: No additional information related to SSBs associated with PDCCH monitoring occasions is provided.

CATT

Proposal 5: For indicating additional information related to SSBs associated with PDCCH monitoring occasions, support Alt3.

- Alt3: No additional information is provided.

CMCC

Proposal 2: For additional information related to SSBs associated with PDCCH monitoring occasions, support one of the followings:

- Alt 1: The information is provided in UL-WUS config.

- Alt 3: No additional information is provided.

Xiaomi

Proposal 1: For associated SSB(s) for on-demand SIB1 transmission, Alt.3 is adopted.

- Alt. 3: No additional information is provided.

InterDigital

Proposal 1: No additional information is provided on the SSB beams for receiving PDCCH for OD-SIB1 (aside from the 1-bit indication in UL-WUS configuration)

Fujitsu

Proposal 3. Regarding additional information related to SSBs associated with PDCCH monitoring occasions, support Alt 3.

- Alt 3: No additional information is provided.

Ericsson

Observation 1 If additional information related to SSBs associated with PDCCH monitoring occasions is provided in UL-WUS (Alt1), it is semi-static and must account for all RACH occasions where the UE can send UL WUS.

Observation 2 If additional information related to SSBs associated with PDCCH monitoring occasions is provided in RAR (Alt2), it is dynamic and can be tailored to the PRACH resource used for UL WUS.

Proposal 1 Support additional information related to SSBs associated with PDCCH monitoring occasions provided in RAR in response to UL-WUS transmission (Alt2) or no additional information is provided (Alt3).

Proposal 2 If Alt2 is adopted, the additional information in RAR consist of a subset of SSBs indicated with a bitmap or a list of SSB indices.

For this topic, companies’ views in RAN1 #121 are summarized below:

On whether/how to indicate additional information related to SSBs associated with PDCCH monitoring occasions.

- Alt1: The information is provided in UL-WUS config

  - Supported by LG, CEWiT, Nokia, CMCC,

- Alt2: The information is provided in RAR in response to UL-WUS transmission

  - Supported by LG, Apple, QC, CAICT, Ericsson

- Alt3: No additional information is provided

  - Supported by China Telecom, MTK, Lenovo, ETRI, Transsion, Panasonic, DENSO, DCM, FUTUREWEI, Huawei/HiSilicon, ZTE/Sanechips, vivo, NEC, Spreadtrum/UNISOC, Samsung, Ofinno, CATT, CMCC, Xiaomi, InterDigital, Fujitsu, Ericsson

- FL Proposal 1-1

On whether/how to indicate additional information related to SSBs associated with PDCCH monitoring occasions, adopt the following:

- Alt3: No additional information is provided

| Company | Support or not | Comment |
| --- | --- | --- |
| MTK | Support | This seems to be vast majority view. |
| Google | Support |  |
| Sharp | Support | For NES gain, current 1 bit indication seems having enough flexibility, and the change of detectable SSB should be solved by retransmission procedure. |
| CMCC | Support | Okay |
| ZTE, Sanechips | Support | 1-bit indication is enough. The motivation to indicate additional information related to SSBs associated with PDCCH monitoring occasions is not clear. |
| OPPO | support |  |
| Apple |  | We think Alt 2 would provide both NES gain and flexibility for NW to provide additional SSB information to assist UE in detecting OD-SIB1. However, given the majority view, we are OK to compromise to Alt 3. |
| Futurewei | Support |  |
| Nokia/NSB |  | Considering the majorities’ view, we can agree with Alt3 at this late stage of very last meeting |
| Samsung | Support |  |
| Fujitsu | Support |  |
| Sony | Support |  |
| Xiaomi | Support |  |
| NEC | Support |  |
| LG |  | We think Alt 1 and Alt 2 can improve the reliability of UE reception of OD-SIB1 but we are OK to compromise to Alt 3. |
| DOCOMO | Support |  |
| ETRI | Support |  |
| Spreadtrum | Support |  |
| Panasonic | Support |  |
| vivo | Support |  |
| CATT | Support |  |
| Tejas | Support |  |
| China Telecom | Support |  |
| Huawei, HiSilicon 1 | Support |  |
| Ofinno | Support |  |
| Lenovo | Support |  |

- FL Proposal 1-2 (OPPO)

RAN1 adopts TP#2 below.

--------------- TP#2 of TS 38.213 in Clause 23 -----------------------

If the UE identifies a RAPID associated with a corresponding PRACH transmission from the UE in a PDSCH reception scheduled by the DCI format 1_0 with CRC scrambled by the RA-RNTI, the UE can be indicated by higher layers to monitor PDCCH on the second cell to detect a DCI format 1_0 with CRC scrambled by the SI-RNTI according to a Type0-PDCCH CSS set provided by SearchSpaceZero. If the UE is provided XYZ, the UE monitors PDCCH only in monitoring occasions associated with the SS/PBCH block. The UE starts monitoring PDCCH to detect the DCI format 1_0 with CRC scrambled by the SI-RNTI after a number of slots provided by od-sib1-windowStartOffset from the starting slot of the window controlled by ra_ResponseWindow, and for a number of slots provided by od-sib1-WindowDuration.

--------------- end of TP#2-------------------------------------------------------

Reason of change: In RAN1 agreements, there is no intention to change/restrict the UE monitoring behavior, but only the UE assumption on the SIB1 beams. The description of the UE assumption on the beams can be captured in TS 38.331 as legacy for OSI.

| Company | Support or not | Comment |
| --- | --- | --- |
| MTK | No | The XYZ in the red deleted text seems can be used to reflect the 1-bit indication in previous RAN1 agreement: Agreement The following agreement is updated as follows: The UE assumes that, in the OD-SIB1 window, PDCCH for an OD-SIB1 message is transmitted in PDCCH monitoring occasions corresponding only to the SSB associated with the transmitted PRACH for UL-WUS if this is indicated via UL WUS configuration by a 1-bit indication. |
| Google | No | This is not aligned with the agreement. |
| Sharp | OK | We are OK to remove if the function of the 1-bit indication is clarified enough in TS 38.331. |
| CMCC | No | Share similar view with MTK. |
| ZTE, Sanechips |  | The XYZ in the red deleted text is used to reflect the 1-bit indication which is useful. Considering that the OPPO’s concern, the wording can be modified as following. --------------- TS 38.213 in Clause 23 ----------------------- If the UE identifies a RAPID associated with a corresponding PRACH transmission from the UE in a PDSCH reception scheduled by the DCI format 1_0 with CRC scrambled by the RA-RNTI, the UE can be indicated by higher layers to monitor PDCCH on the second cell to detect a DCI format 1_0 with CRC scrambled by the SI-RNTI according to a Type0-PDCCH CSS set provided by SearchSpaceZero. If the UE is provided XYZ, the UE is expected to monitors PDCCH only in monitoring occasions associated with the SS/PBCH block. The UE starts monitoring PDCCH to detect the DCI format 1_0 with CRC scrambled by the SI-RNTI after a number of slots provided by od-sib1-windowStartOffset from the starting slot of the window controlled by ra_ResponseWindow, and for a number of slots provided by od-sib1-WindowDuration. --------------- end------------------------------------------------------- |
| OPPO | Support | The RAN1 agreement is only on the UE assumption. It does not mean to mandate the UE behavior. This assumption can be captured in 38.331 instead of the 38.213, where UE behavior is defined. |
| Apple | No | Same understanding as MTK. |
| Futurewei | Support | Same understanding as Oppo |
| Nokia/NSB | No | Agree the comments from MTK |
| Samsung | Support | Share same view as OPPO. Capturing the UE assumption in RRC parameter is enough without any UE behaviour in TS 38.213. |
| Fujitsu | Not support | Same view as MTK. |
| Xiaomi |  | Same understanding as MTK. |
| LG | No | Agree with MTK. |
| DOCOMO | No |  |
| ETRI | No | Same understanding as MTK. |
| Panasonic |  | Although we had the agreement of the 1-bit indication, the red font text is not aligned with it. We are okay to either remove it or to change it to “UE assumes that…”. Because to mandate UE to only monitor certain beams is not the same with the behaviour in the agreement. |
| vivo |  | Better to follow the saying in agreement to specify UE assumption instead of UE behavior |
| Tejas | No |  |
| China Telecom | No |  |
| Huawei, HiSilicon 1 |  | Open to discuss. However, we do not see a strong need. |
| Ericsson | Support | Same understanding as Sharp, Oppo, Samsung – this should be clear from 331. |
| Ofinno | No | Same view as MTK |
| Lenovo | No |  |
| OPPO2 |  | comment to MTK, Apple, CMCC, Nokia, Fujitsu, LG, ETRI, Ofinno: Could you please provide evidence that the UE monitoring behavior was agreed in such a way it was captured in the draft CR? RAN1 only agrees that the UE assumes how the NW will transmit the OD-SIB1. We didn’t agree how the UE would monitor. |

- Issue 2: Contents of the RAR for UL WUS

Background

In RAN1 #118, the following is agreed that UE is expected to receive RAR for UL WUS:

Agreement

RAN1 assumes the UE is expected to receive the RAR responding to the preamble transmission for Msg1-based on-demand SIB1 procedure, as the baseline.

In RAN1 #118b, the following is agreed

Agreement

RAN1 to discuss the contents of RAR in response to UL WUS using the legacy RAR for OSI as a starting point

In RAN1 #119, the following is agreed

Agreement

At least the contents of RAR in response to SI request are included in RAR in response to UL WUS

In RAN2 #129b, the following is agreed

Agreement

Align with legacy RAR for OSI for OD-SIB1 operation. Legacy RAR MAC PDU subheader with RAPID only to be used as NW acknowledgement for OD-SIB1 request.

Companies’ view in RAN1 #121 are collected below.

Transsion:

Proposal 3  The contents of RAR in response to SI request follows legacy operation.

CAICT:

Proposal 2:

On contents of the RAR for UL WUS, do not support additional parameters on top of contents of RAR in

response to SI request.

Spreadtrum, UNISOC

Proposal 3 Don’t support additional contents in RAR for OD-SIB1 other than contents of RAR for SI request.

Xiaomi

Proposal 2: Only the contents of RAR in response to SI request are included in RAR in response to UL WUS.

Proposal 4: Adopt text proposal#1 below for clause 23 of TS 38.213.

23 UE procedure to request SIB1 reception -----------------------omitted text----------------------- [If the SS/PBCH block on the second cell indicates kSSB>23 for FR1 or kSSB>11 for FR2,] the UE is not required to monitor PDCCH on the second cell to detect the DCI format 1_0 with CRC scrambled by the SI-RNTI prior to a reception of a PDSCH carrying a RAPID associated with a corresponding PRACH transmission scheduled by the DCI format 1_0 with CRC scrambled by the RA-RNTI.

CMCC

Proposal 3: For the specific content in RAR in response to UL WUS, support one of the followings:

- Do not include other parameter in RAR (reuse RAPID-only MAC subPDU in TS 38.321).

- Up to RAN2 to decide whether other parameter can be included in RAR, and how to deal with the potential misalignment for legacy UE.

Companies’ view in RAN1 #121 are summarized below.

On contents of the RAR for UL WUS, whether to support additional parameters on top of contents of RAR in response to SI request

- Yes:

- No: Transsion, CAICT, Spreadtrum/UNISOC, Xiaomi, CMCC

Moderator hence has the following proposal:

- FL Proposal 2-1

On contents of the RAR for UL WUS, do not support additional parameters on top of contents of RAR in response to SI request.

| Company | Support or not | Comment |
| --- | --- | --- |
| MTK | Support | This also fits the following RAN2 #129b agreement: Agreement Align with legacy RAR for OSI for OD-SIB1 operation. Legacy RAR MAC PDU subheader with RAPID only to be used as NW acknowledgement for OD-SIB1 request. |
| Google | Support |  |
| Sharp | Support | It aligns the RAN2 agreement. |
| CMCC | Support | Introduce other parameter in RAR will strongly impact legacy UE and should be up to RAN2 to solve such issue. Since RAN2 just agreed to align with legacy, RAN1 should align with RAN2 either. |
| ZTE, Sanechips | Support |  |
| Futurewei | Support |  |
| Nokia/NSB | Support | Agree the comments from MTK |
| Samsung | Support | To align with RAN2 agreement, additional parameter shouldn’t be included in RAR for UL WUS. |
| Fujitsu | Support |  |
| Sony | Support |  |
| Xiaomi | Support |  |
| LG | Support |  |
| DOCOMO | Support |  |
| ETRI | Support |  |
| Spreadtrum | Support |  |
| Panasonic | Support |  |
| vivo | Support |  |
| CATT | Support |  |
| Tejas | Support |  |
| China Telecom | Support |  |
| Huawei, HiSilicon 1 |  | fine |
| Ericsson |  | This will be the outcome if no parameter is agreed to be added. |
| Ofinno | Support |  |
| Lenovo | Support |  |

- FL Proposal 2-2 (Xiaomi)

RAN1 adopts TP below for clause 23 of TS 38.213.

23 UE procedure to request SIB1 reception -----------------------omitted text----------------------- [If the SS/PBCH block on the second cell indicates kSSB>23 for FR1 or kSSB>11 for FR2,] the UE is not required to monitor PDCCH on the second cell to detect the DCI format 1_0 with CRC scrambled by the SI-RNTI prior to a reception of a PDSCH carrying a RAPID associated with a corresponding PRACH transmission scheduled by the DCI format 1_0 with CRC scrambled by the RA-RNTI.

Reason of change: If RAN1 can agree only RAPID is included in RAR in response to UL WUS.

| Company | Support or not | Comment |
| --- | --- | --- |
| MTK | Support | This is the corresponding TP of FL Proposal 2-1. |
| Google |  | OK, but it may be a bit redundant. |
| Sharp | Support |  |
| CMCC |  | Okay to make it clearer. |
| OPPO |  | The TP is technically correct! But we have a general question on this paragraph. Given the current wording in draft CR suggested by editor, it seems that the it is already clear that only the ‘if’ condition is met, the UE will monitor type0-PDCCH. Therefore, we think that the whole paragraph in FL proposal 2-2 can be removed. If the UE identifies a RAPID associated with a corresponding PRACH transmission from the UE in a PDSCH reception scheduled by the DCI format 1_0 with CRC scrambled by the RA-RNTI, the UE can be indicated by higher layers to monitor PDCCH on the second cell to detect a DCI format 1_0 with CRC scrambled by the SI-RNTI according to a Type0-PDCCH CSS set provided by SearchSpaceZero |
| Apple | Support |  |
| Futurewei | Support |  |
| Nokia/NSB |  | The original version without adding can be also fine for us |
| Fujitsu |  | We think whether a PDSCH carrying a RAPID can be captured on RAN2 spec. |
| Xiaomi | Support. | I appreciate the detail comments from OPPO. We would like to point out that the text excerpted by OPPO comes from a parallel paragraph. The TP we propose is the follow-up paragraph with the same level with the previous paragraph. We don’t think the ‘if condition’ covers the TP. It seems all companies agree with the principle, we propose to take this TP and leave it to editor to decide whether/how to proceed it. |
| NEC | Support |  |
| LG | Support |  |
| ETRI | OK | Current specification seems already clear, but fine with the proposal. |
| Spreadtrum | Support |  |
| vivo | Support |  |
| Tejas |  | We suggest to add the following to the above paragraph based on the agreement made in the RAN1#120-bis meeting. 23 UE procedure to request SIB1 reception -----------------------omitted text----------------------- [If the SS/PBCH block on the second cell indicates kSSB>23 for FR1 or kSSB>11 for FR2,] the UE is not required to monitor PDCCH on the second cell to detect the DCI format 1_0 with CRC scrambled by the SI-RNTI prior to a reception of a PDSCH carrying a RAPID associated with a corresponding PRACH transmission scheduled by the DCI format 1_0 with CRC scrambled by the RA-RNTI. If the SS/PBCH block on the second cell indicates kSSB>23 for FR1 or kSSB>11 for FR2, it is up to UE to monitor PDCCH on the second cell to detect the DCI format 1_0 with CRC scrambled by the SI-RNTI prior to PRACH transmission. Agreement (RAN1#120-bis) If a UE has SIB1 request configuration of a cell and before transmitting UL WUS, - If the UE detects a SSB where K_SSB>=24 for FR1 or K_SSB>=12 for FR2, select the following: - Alt. 3: It is up to UE implementation on whether to monitor Type 0 PDCCH for SIB1 transmission. |
| China Telecom | Support. |  |
| Huawei, HiSilicon 1 |  | Seems not essential. |
| Ericsson | Support |  |
| Ofinno | Support |  |

- Issue 3: Parameters/contents inside the UL WUS configuration

Background

In RAN1 #117, it is agreed to use the Table I below from [R1-2405106, Ericsson] as a starting point to discuss the required parameters/contents inside the UL WUS configuration.

Table I.

| Purpose | Parameters |  |  |
| --- | --- | --- | --- |
| To which cell does the config applies | NES-CellId | PhysCellId |  |
|  |  | ARFCN-ValueNR |  |
| WUS transmission | SIB1-RequestConfig | ss-PBCH-BlockPower |  |
|  |  | rach-OccasionsSIB1 | Prach-ConfigurationIndex |
|  |  |  | msg1-FDM |
|  |  |  | msg1-FrequencyStart |
|  |  |  | zeroCorrelationZoneConfig |
|  |  |  | preambleReceivedTargetPower |
|  |  |  | preambleTransMax |
|  |  |  | powerRampingStep |
|  |  |  | ra-ResponseWindow |
|  |  |  | ssb-perRACH-Occasion |
|  |  | sib1-RequestPeriod |  |
|  |  | sib1-RequestResource | ra-PreambleStartIndex |
|  |  |  | ra-AssociationPeriodIndex |
|  |  |  | ra-ssb-OccasionMaskIndex |
|  |  | rsrp-ThresholdSSB |  |
|  |  | prach-RootSequenceIndex |  |
|  |  | msg1-SubcarrierSpacing |  |
|  |  | restrictedSetConfig |  |
|  |  | tdd-UL-DL-ConfigurationCommon |  |
|  | frequencyInfoUL | frequencyBandList |  |
|  |  | absoluteFrequencyPointA |  |
|  |  | offsetToCarrier |  |
|  |  | p-Max |  |
|  |  | frequencyShift7p5khz |  |
| SIB1 reception | pdcch-ConfigSIB1 | ssb-SubcarrierOffset |  |
|  |  | controlResourceSetZero |  |
|  |  | searchSpaceZero |  |
| RAR Reception | pdcch-ConfigOD-SIB1-RAR | controlResourceSet |  |
|  |  | monitoringSlotPeriodicityAndOffset |  |
|  |  | Duration |  |
|  |  | monitoringSymbolsWithinSlot |  |
|  |  | aggregationLevels |  |

In RAN1 #118-bis, the following are agreed:

Agreement

For the purpose of “to which cell does the configuration applies”, at least the following parameters are included in the UL-WUS configuration.

| Purpose | Parameters |  |
| --- | --- | --- |
| To which cell does the configuration applies | NES-CellId | PhysCellId |
|  |  | ARFCN-ValueNR |

Note: ARFCN-ValueNR is used to indicate the absolute radio frequency channel number (ARFCN) for SSB of NES cell.

Agreement

For the purpose of “WUS transmission”, at least the following parameters are included in the UL-WUS configuration:

- rsrp-ThresholdSSB

- prach-RootSequenceIndex

- msg1-SubcarrierSpacing

- restrictedSetConfig

Note: In legacy spec, the parameters above are under the IE RACH-ConfigCommon

Agreement

For the purpose of “WUS transmission”, at least the following parameters to indicate “frequency information for UL-WUS transmission” are included in the UL-WUS configuration.

| Purpose | Parameters |
| --- | --- |
| WUS transmission | frequencyBandList |
|  | absoluteFrequencyPointA |
|  | offsetToCarrier |
|  | p-Max |
|  | frequencyShift7p5khz (working assumption)ULSubCarrierSpacing |

Agreement

For the purpose of “WUS transmission”, at least the following parameters are included in the UL-WUS configuration.

| Purpose | Parameters |  |  |
| --- | --- | --- | --- |
| WUS transmission | SIB1-RequestConfig | ss-PBCH-BlockPower |  |
|  |  | SSB-positionInBurst |  |
|  |  | tdd-UL-DL-ConfigurationCommon |  |
|  |  | rach-OccasionsSIB1 | Prach-ConfigurationIndex |
|  |  |  | msg1-FDM |
|  |  |  | msg1-FrequencyStart |
|  |  |  | zeroCorrelationZoneConfig |
|  |  |  | preambleReceivedTargetPower |
|  |  |  | preambleTransMax |
|  |  |  | powerRampingStep |
|  |  |  | ra-ResponseWindow |
|  |  |  | ssb-perRACH-Occasion |
|  |  | sib1-RequestPeriod |  |
|  |  | sib1-RequestResources | ra-PreambleStartIndex |
|  |  |  | ra-AssociationPeriodIndex |
|  |  |  | ra-ssb-OccasionMaskIndex |

Note:

- In legacy spec, ss-PBCH-BlockPower/SSB-positionInBurst/tdd-UL-DL-ConfigurationCommon are under the IE ServingCellConfigCommon/ServingCellConfigCommonSIB

- Msg1-FrequencyStart is with respect to the first RB of the UE carrier determined by offsetToCarrier and absoluteFrequencyPointA

Agreement

Search space for RAR in response to UL WUS on a NES Cell can be provided by the UL WUS configuration. If not, follow the search space zero for the NES Cell.

In RAN1 #119, the following are agreed:

Agreement

The parameter “n-TimingAdvanceOffset” is included in the UL-WUS configuration.

Agreement

For parameters agreed for UL WUS, whether it is mandatory or optional is for further discussion.

Agreement

At least for RO validation determination for WUS transmission, the parameter “ssb-PeriodicityServingCell” is included in the UL-WUS configuration at least for TDD system.

- FFS: FDD system

Agreement

Confirm the working assumption below from RAN1 #118-bis to include it in the UL-WUS configuration.

- (working assumption) ULSubCarrierSpacing

Agreement

CORESET0 of NES cell is used for RAR CORESET of that NES cell.

Agreement

At least for the case where SSB is transmitted on sync-raster, the indication of the quantity K_SSB (defined in TS 38.211 as subcarrier offset from subcarrier 0 in common resource block NCRBSSB to the lowest-numbered subcarrier of the SS/PBCH block) is included in the UL-WUS configuration for FDD and TDD NES cell.

Agreement

The duration of the time window for on-demand SIB1 is indicated in the UL WUS configuration.

- Unit of the duration is in slot

- FFS: Starting time

In RAN1 #120, the following is concluded:

Conclusion

- The parameter “ssb-PeriodicityServingCell” is not included in the UL-WUS configuration for FDD system.

- RA-RNTI is not included in the UL-WUS configuration.

- Cell barring information is not included in the UL-WUS configuration.

- Cell selection information is not included in the UL-WUS configuration.

- Accessible UE types is not included in the UL-WUS configuration.

- CarrierBandwidth (for MPR calculation) is not included in the UL-WUS configuration.

- InitialUplinkBWP is not included in the UL-WUS configuration.

- TotalNumberOfRA-Preambles is not included in the UL-WUS configuration.

- SIB1-RequestResourceRedCap is not included in the UL-WUS configuration.

- SupplementaryUL is not included in the UL-WUS configuration

In RAN1 #120-bis, the following are agreed:

Agreement

Indication of K_SSB value in the UL-WUS configuration should be 5 bits for FR1 and 4 bits for FR2.

- Note: there is no change to current PBCH structure.

- UE does not expect K_SSB to be larger than 23 for FR1 and larger than 11 for FR2.

Agreement

The value of the time offset between the reference time point and the starting time for the time window of OD-SIB1 can be either 0 or positive value.

Agreement

In the UL WUS configuration, how to support multiple NES-CellId(s) is up to RAN2 discussion.

Agreement

From RAN1 perspective, for agreed UL WUS parameters, regarding their mandatory or optional presence and applicability to TDD and/or FDD, adopt the followings:

- PhysCellId and ARFCN-ValueNR are mandatory

- frequencyBandList and absoluteFrequencyPointA are present in IE FrequencyInfoUL for FDD (as in the legacy specification)

- K_SSB is mandatory

- searchSpaceZero and controlResourceSetZero are mandatory

- ra-PreambleStartIndex, od-sib1-duration, offsetToTimeWindow are mandatory

Agreement

The parameter ra-SearchSpace in the UL-WUS configuration is of type SearchSpace.

Agreement

Replace the current parameter name offsetToTimeWindow with od-sib1-windowStartOffset (to make the naming more comprehensive).

Agreement

Include CarrierBandwidth and locationAndBandwidth in the UL-WUS configuration.

- Previous RAN1 #120 conclusion is reverted

Agreement

The following previous RAN1 #118-bis agreement is updated as:

- Msg1-FrequencyStart is with respect to the first RB of the UE carrier InitialUplinkBWP determined by locationAndBandwidth offsetToCarrier and absoluteFrequencyPointA (same as legacy)

Agreement

RAN1 to discuss the value range for od-sib1-duration. For example,

- Option 1: Use the value-range for si-windowLength (from 5 slots to 5120 slot) as starting point.

- Option 2: The duration of the time window can be a multiple of 160ms (i.e. legacy SIB1 periodicity).

- Option 3: The duration of the time window can be a multiple of the on-demand SIB1 repetition periodicity (if supported)

- Other options are not precluded.

Agreement

RAN1 clarifies frequencyBandList in UL WUS configuration refers to the IE within FrequencyInfoUL-SIB.

- Value range for od-sib1-duration:

On the value range of od-sib1-duration, companies’ views in RAN1 #121 are collected below:

China Telecom:

Proposal 2: Support Option 1, i.e., use the value range for si-windowLength (from 5 slots to 5120slots) as a

starting point for the value range of od-sib1-duration.

Lenovo:

Proposal 3: Use the value-range for si-windowLength (from 5 slots to 5120 slot) for configuring od-sib1-duration.

ETRI:

Proposal 1: Use the value range of si-windowLength for the OD-SIB1 window duration (Option 1).

- FFS: Whether/how to guarantee valid Type 0 PDCCH monitoring occasion(s) within the window

Transsion:

Proposal 2  For the value range for od-sib1-duration, option 2 or option 3 can be supported.

OPPO:

Proposal 1: the value range for is (40 ms, 60 ms, 80 ms,..., 250 ms) with granularity of 20 ms.

Proposal 2: the value range for the time offset between the reference time point and the starting time of the OD-SIB1 window is (0 ms, 20 ms, 40 ms).

Proposal 3: If RAN1 agreement on OD-SIB1 duration can be reverted, we suggest that the OD-SIB1 duration should be configured in terms of the number of type0-PDCCH monitoring occasions.

Panasonic:

Proposal 2: The value range of od-sib1-durati on is defi ned as the duration of the time window of a multiple of 160ms (i.e. legacy SIB1 periodicity).

MTK: Option 2: multiple of 160ms

Apple:

Proposal 1: For the value range of od-sib1-duration, consider the following two options:

- Option 1: Use the value-range for si-windowLength (from 5 slots to 5120 slot) as starting point.

- Option 4: The duration of the time window can be a multiple of 20ms

QC:

Proposal 2: The value range of the PDCCH monitoring window od-sib1-duration is the same value range as si-windowLength (i.e., Option 1).

DENSO:

Proposal 2: The value range for the duration of the on-demand SIB1 window is specified as a multiple of the

on-demand SIB1 repetition periodicity (Option 3).

Proposal 3: ULSubcarrierSpacing is used for the subcarrier spacing for PRACH baseband signal generation.

DCM:

Proposal 3: The duration of OD-SIB1 window could be multiple of 160ms, e.g., [160ms, 320ms, 640ms, 1280ms].

CEWiT:

Proposal 2: Support Option 3 “The duration of the time window can be a multiple of the on-demand SIB1

repetition periodicity” for the value range for si-windowLength.

FUTUREWEI

Proposal 2: Support Option 2: The value range for od-sib1-duration can be a multiple of 160ms.

Huawei, HiSilicon

Proposal 3: Support to use the value-range for si-windowLength (from 5 slots to 5120 slots) as the value range for odsib1-duration.

ZTE, Sanechips

Proposal 2: For the value range for od-sib1-duration, both option 2 and option 3 can be considered.

vivo

Proposal 6: Support option1 for the value range for od-sib1-duration, i.e., use the value-range for si-windowLength (from 5 slots to 5120 slot) as starting point.

Nokia

Observation-2: Based on TS38.212, UE is not expecting to change of MIB payload within one MIB TTI of 80 ms.

Proposal-1: RAN1 to discuss that the duration of OD-SIB1 window, od-sib1-WindowDuration, is a multiple of MIB TTI of 80 ms to avoid PBCH combining issue at UE.

Proposal-3: For OD-SIB1 operation, RAN1 to discuss whether the duration of OD-SIB1 window, od-sib1- WindowDuration shall be larger/smaller than or equal to the transmission periodicity of OD-SIB1, od-sib1- periodicity.

NEC

Proposal 3: For od-sib1-duration value range, support Option-2 The duration of the time window can be a multiple of 160ms (i.e. legacy SIB1 periodicity)

Spreadtrum, UNISOC

Proposal 2 Support option 1 for od-sib1-duration.

- Option 1: Use the value-range for si-windowLength (from 5 slots to 5120 slot) as starting point.

Samsung

Proposal 2: Support Option 2 (The duration of the time window can be a multiple of 160ms (i.e. legacy SIB1 periodicity)) for both Rel-19 NES capable UE and legacy UE.

- Minimum value of od-sib1-duration is 160 ms.

Tejas

Proposal 3: The value range of OD-SIB1 window duration should be multiple of SSB periodicity of the NES CELL.

Ofinno

Proposal 2. Define the value range for od-sib1-duration in terms of number of slots from 5 slots to 5120 slots.

CMCC

Proposal 1: For the value range for od-sib1-duration, support Option 3 (The duration of the time window can be a multiple of the on-demand SIB1 repetition periodicity).

- 20ms can be taken as the starting point for on-demand SIB1 repetition periodicity.

Xiaomi

Proposal 3: For the value range of the time window duration for on-demand SIB1, Option 1 is adopted.

- Option 1: Use the value-range for si-windowLength (from 5 slots to 5120 slot) as starting point

InterDigital

Proposal 2: For the duration of OD-SIB1, support the value-range of si-windowLength (from 5 slots to 5120 slot) as starting point (Option 1)

Fujitsu

Proposal 1. Regarding od-sib1-duration, use the value range for si-windowLength (from 5 slots to 5120 slot) as the starting point.

Ericsson

Proposal 5 It is clarified that od-sib1-duration corresponds to the time duration where the NES Cell is guaranteed to transmit OD-SIB1 after an OD-SIB1 request.

Observation 4 The granularity of od-sib1-duration in terms of slots is unnecessarily fine and the range, i.e., up to 5120 slots, is unnecessarily long.

Observation 5 The duration of the time window being a multiple of 160ms (Option 2) is a reasonable compromise between flexibility and simplicity.

Observation 6 Relating the duration of the time window with the optional property of repetition periodicity can be error prone and it is best to be avoided.

Proposal 6 Support the duration of the time window is a multiple of 160ms (Option 2). Additionally, support the duration 80ms.

Companies’ views for value range of od-sib1-duration in RAN1 #121 are summarized below:

For the value range for od-sib1-duration

- Option 1: Use the value-range for si-windowLength (from 5 slots to 5120 slot).

  - Supported by China Telecom, Lenovo, ETRI, Apple, Qualcomm, Huawei/HiSilicon, vivo, Spreadtrum/UNISOC, Ofinno, Xiaomi, InterDigital, Fujitsu

- Option 2: The duration of the time window can be a multiple of 160ms (i.e. legacy SIB1 periodicity).

  - Supported by Transsion, Panasonic, MTK, DCM, FUTUREWEI, ZTE/Sanechips, NEC, Samsung, Ericsson (add 80ms),

- Option 2a: {20, 40, 80, 160, 320,...}ms

  - Supported by Apple, vivo

- Option 3: The duration of the time window can be a multiple of the on-demand SIB1 repetition periodicity (if supported)

  - Supported by Transsion, DENSO, CEWiT, ZTE/Sanechips, CMCC (Ex. 20ms),

- Option 4: The value range for od-sib1-duration is (40 ms, 60 ms, 80 ms, ..., 240 ms) with granularity of 20 ms.

  - Supported by OPPO

- Option 5: The duration of the time window can be configured in terms of the number of type0-PDCCH monitoring occasions.

  - Supported by OPPO

- Option 6: Multiple of 20ms

  - Supported by Apple

- Option 7: Multiple of 80ms

  - Supported by Nokia

- Option 8: Multiple of SSB periodicity of the NES cell

  - Supported by Tejas

- FL Proposal 3-1

On the value range for od-sib1-duration, adopt the following:

- Option 1: Use the value-range for si-windowLength (from 5 slots to 5120 slot)

| Company | Support or not | Comment |
| --- | --- | --- |
| MTK | No, but willing to discuss | We prefer Option 2 (multiple of 160ms) as the OD-SIB1 operating mechanism (only transmitted in the time window and not being periodic) is different from legacy OD-OSI. Note: An example of legacy OD-OSI is shown below (ref) |
| Google |  | The range of 5-5120 slot seems to be too big. We would like to understand the motivation for this. |
| CMCC | No | If option 1 is adopted, the minimum value should be equal or larger than 20ms (i.e. the typical periodicity of SIB1 reception) |
| ZTE, Sanechips | No | The si-windowLength is no greater than one SIB1 transmission periodicity which cannot guarantee that the UE can receive SIB1 within the time window. We suggest take option 2 or option 3 as the starting point. |
| OPPO | No | We have two concerns:1) the granularity of 1 slot is not justified, this seems to be over-design. The type0-PDCCH monitoring granularity is 20 ms. Thus, 20 ms can be used as the window length granularity; 2) the window length needs to ensure at least one monitoring occasion can be covered within the window. Thus, we think that 5 slots as minimum value is too small. We need to check the the monitoring duration corresponding to all the SSB beam directions. |
| Apple |  | Agree with MTK that the OD-SIB1 monitoring window is a different concept from legacy OD-OSI monitoring window. A middle point between Opiton1 and Option2 may be a value range including: {20, 40, 80, 160, 320,...}ms |
| Futurewei | No | Prefer Option 2, multiple of 160ms |
| Nokia/NSB | No, but willing to discuss as well | We can be OK with 80ms and 160ms, but it is more sensible to be with 80ms considering of MIB update periodicity. |
| Samsung | Not support | We prefer Option 2. For the value range for od-sib1-duration, it would be desirable to define the value based on legacy SIB1 periodicity. Since on-demand SIB1 is intended for both Rel-19 NES-capable UEs and legacy UEs, it would be preferable to use a multiple of the legacy SIB1 periodicity for od-sib1-duration. Furthermore, the minimum value of od-sib1-duration should be 160 ms to guarantee the SIB1 transmission within duration. - Updated FL Proposal 3-1 On the value range for od-sib1-duration, adopt the following: - Option 2: The duration of the time window can be a multiple of 160ms (i.e. legacy SIB1 periodicity). - Minimum value of od-sib1-duration is 160 ms. |
| Fujitsu | Support | Regarding the example provided by MTK, our understanding is that it is the start time of a legacy OD-OSI time window, rather than the window length. |
| Xiaomi | Support | Option 1 is the legacy configuration value for SI window duration. Indeed, it is a superset of option 2 and option 3. We believe gNB can provide a proper configuration based on option 1 to achieve better NES gain. |
| NEC |  | For the od-sib1-duration value range, we support Option 2, where the duration of the time window is a multiple of 160ms (i.e., legacy SIB1 periodicity). We believe this approach is favored due to implementation simplicity and its alignment with existing NR structures, facilitating a smoother introduction of on-demand SIB1. |
| LG |  | It seems that the minimum value is needed to be discussed. |
| DOCOMO |  | Support Option 2 |
| ETRI |  | The unit can be a slot as agreed previously, but too small values can be excluded. |
| Spreadtrum | Support | Fine with the proposal. Option 1 is the legacy configuration value for SI window and for some configured values, option 1 can cover option 2 and option 3. |
| Panasonic | No | As we explained in our tdoc that legacy si-windowLength works differently from our current context. Also agree with the explanation from MTK. |
| vivo |  | First we think X ms can be used for SIB1 duration indication and the value of X can be directly determined. In our mind, it can be {80ms, 160ms, 320ms, 640ms}. |
| CATT | OK |  |
| Tejas |  | From TS 38.331 <Unwanted text omitted> - the SIB1 is transmitted on the DL-SCH with a periodicity of 160 ms and variable transmission repetition periodicity within 160 ms as specified in TS 38.213 [13], clause 13. The default transmission repetition periodicity of SIB1 is 20 ms but the actual transmission repetition periodicity is up to network implementation. For SSB and CORESET multiplexing pattern 1, SIB1 repetition transmission period is 20 ms. For SSB and CORESET multiplexing pattern 2/3, SIB1 transmission repetition period is the same as the SSB period (TS 38.213 [13], clause 13). SIB1 includes information regarding the availability and scheduling (e.g. mapping of SIBs to SI message, periodicity, SI-window size) of other SIBs with an indication whether one or more SIBs are only provided on-demand and, in that case, the configuration needed by the UE to perform the SI request. SIB1 is cell-specific SIB; <Unwanted text omitted> Updated Proposal: On the value range for od-sib1-duration, adopt the following: - Option 1: Use the value-range for si-windowLength (from 5 slots to 5120 slot) For SSB and CORESET multiplexing pattern 1, od-sib1-duration is multiple of 20 ms. For SSB and CORESET multiplexing pattern 2/3, od-sib1-duration is multiple of the SSB periodicity of NES CELL. |
| China Telecom |  | We support the principle of Option 1, since we don’t think it is necessary to constrain the window length to some specific values, e.g., multiple of 160ms. But we are OK to further discussion the value range in Option 1. |
| Huawei, HiSilicon 1 | Support | Option 1 is flexible enough and it is consistent with legacy SI window size si-WindowLength ENUMERATED {s5, s10, s20, s40, s80, s160, s320, s640, s1280, s2560-v1710, s5120-v1710 }, and we do not see a need for different values. |
| Ofinno | Support option 1 | But we are ok to find a middle ground between option 1 and option 2. First smallest value can be 20 ms. But granularity of 160 ms is too large. We are fine with Apple proposal. |
| Lenovo | Support |  |
| Moderator |  | We can have a further discussion of Option 1 v.s. 2 during online sessions. From moderator’s perspective, legacy on-demand SI is provided in a periodic way, so the small values say 5 slots in Option 1 can work. However, for on-demand SIB1, it is only contained in one window, so the window length may need to be multiple of 20ms or multiple of 160ms. See example figures below. - Legacy OD OSI - The UE shall receive the PDCCH scrambled with SI-RNTI from the start of the SI-window and continue until the end of the SI-window (given by si-WindowLength) or until the SI message was received. - If the SI message was not received by the end of the SI-window, repeat reception at the next SI-window occasion for the concerned SI. - R19 OD SIB1 |

Vivo: Unit is ms first

- FL Proposal 3-1-2

On the value range for od-sib1-duration, adopt the following:

- Option 2: The duration of the time window can be a multiple of 160ms (i.e. legacy SIB1 periodicity).

| Company | Support or not | Comment |
| --- | --- | --- |
| Ericsson | Support | For example the values {160ms, 320ms, 480ms} would suffice. If the time window duration is less than 160ms we have an issue, see figure below: - SIB1 transmissions within 160ms period is up to gNB. Hence the monitoring window may not include any transmission occasions |

- FL Proposal 3-1-3 (Apple)

On the value range for od-sib1-duration, adopt the following:

- {20, 40, 80, 160, 320, ...} ms

- FL Proposal 3-2 (Ericsson)

RAN1 confirms that od-sib1-duration corresponds to the time duration where the NES Cell is guaranteed to transmit OD-SIB1 after an OD-SIB1 request.

- Note: It is up to the NES Cell to decide whether to prolong it

| Company | Support or not | Comment |
| --- | --- | --- |
| MTK | Support | We tend to think this is already implied in previous RAN1 agreements, but fine to further clarify. |
| Google | Support |  |
| CMCC |  | Okay |
| OPPO | Question | Does this proposal tries to clarify the UE only monitors type0-PDCCH within the OD-SIB1 window? it is not clear about the intention of this proposal |
| Apple | Support |  |
| Futurewei | Support |  |
| Nokia/NSB | OK |  |
| Samsung | Not support | Agree with intention of proposal for SIB1 duration, but additional confirmation is not needed. |
| Fujitsu | Support |  |
| Xiaomi |  | Same feeling as Qualcomm. Further clarification is appreciated. |
| NEC | Support | We agree with the clarification that od-sib1-duration defines the time during which the NES cell guarantees OD-SIB1 transmission following a request. |
| LG |  | We share the view with OPPO. |
| DOCOMO |  | Whether there is any UE behaviour update? If no, it is just implementation. |
| ETRI |  | The intention of the note is not clear to us. |
| Spreadtrum | Support |  |
| Panasonic |  | We may have some sympathy on the intention but the wording is a bit not clear. In principle, network may choose to transmit or not to transmit OD-SIB1 after an OD-SIB1 request. But once network transmit, we agree it should at least last for this duration. So our proposed amendment is: RAN1 confirms that od-sib1-duration corresponds to the time duration is guaranteed where if the NES Cell is guaranteed to transmit OD-SIB1 after an OD-SIB1 request. - Note: It is up to the NES Cell to decide whether to prolong it |
| vivo |  | We support the intention. But there is no spec impact. |
| Tejas | Support |  |
| China Telecom | Support |  |
| Huawei, HiSilcon 1 | not needed | We don’t need to agree on cells behaviours. what does the UE expect if this is agreed? |
| Ericsson |  | If this is already clear to companies then we are also ok to drop the proposal. |
| Ofinno | Support |  |

- OD-SIB1 transmission/repetition periodicity:

On OD-SIB1 transmission/repetition periodicity, companies’ views in RAN1 #121 are collected below:

Nokia

Observation-1: For legacy OD-SI operation, as specified in TS38.331, the network always configures si-WindowLength to be shorter than or equal to the si-Periodicity.

Proposal-2: RAN1 to discuss what the transmission periodicity of OD-SIB1, od-sib1-periodicity, is. Is it the same as legacy SIB1 transmission periodicity of 160 ms or is it aligned with the legacy OD-SI transmission periodicity, where the value range can be from 80 ms to 5120 ms.

Proposal-4: RAN1 to discuss the NW indication of repetition periodicity or periodicity of OD-SIB1 transmission in UL WUS configuration.

Fujitsu

Proposal 4. Regarding the UE assumption on on-demand SIB1 repetitions within the time window, UE assumes that the SIB1 transmitted within the time window follows the legacy SIB repetition mechanism, i.e., repetition within a 160 ms period.

Ericsson

Observation 3 If the UE knows the SIB1 transmission pattern within a 160 ms period, then it can avoid blind detection in Type0 PDCCH CSS occasions which are not used.

Proposal 3 Include assistance parameters in the UL WUS configuration that informs the UE of the SIB1 transmission/repetition occasions. For example:

a) SIB1 periodicity and offset, or

b) SIB1 search space mask indicating which of the occasions are used by the NES cell.

Proposal 4 The assistance parameters in the WUS configuration that informs the of the SIB1 transmission/repetition occasions are also valid after the on-demand SIB1 acquisition procedure.

- FL Proposal 3-3 (Nokia, Fujitsu)

RAN1 to clarify the transmission periodicity of OD-SIB1 is

- Option 1: same as legacy SIB1 transmission periodicity of 160 ms

- Option 2: aligned with the legacy OD-SI transmission periodicity, where the value range can be from 80 ms to 5120 ms.

| Company | Option 1 or Option 2 or other opinions | Comment |
| --- | --- | --- |
| MTK | Option 1 | With the following previous RAN1 agreement, we tend to think Option 1 (i.e., same as legacy) is a good fit. RAN1 #118b Agreement For the repetition periodicity of SIB1 within the time window of on-demand SIB1: - Up to NW implementation (no change to existing specification) Note: 38.331 Clause 5.2.1 has the following text on SIB1 repetition periodicity - the SIB1 is transmitted on the DL-SCH with a periodicity of 160 ms and variable transmission repetition periodicity within 160 ms as specified in TS 38.213 [13], clause 13. The default transmission repetition periodicity of SIB1 is 20 ms but the actual transmission repetition periodicity is up to network implementation. For SSB and CORESET multiplexing pattern 1, SIB1 repetition transmission period is 20 ms. For SSB and CORESET multiplexing pattern 2/3, SIB1 transmission repetition period is the same as the SSB period (TS 38.213 [13], clause 13). Meanwhile, it seems worthwhile to do further clarification as suggested in Fujitsu proposal below (same as Option 1): - Fujitsu: “Regarding the UE assumption on on-demand SIB1 repetitions within the time window, UE assumes that the SIB1 transmitted within the time window follows the legacy SIB repetition mechanism, i.e., repetition within a 160 ms period.” |
| Google | Option 1 |  |
| CMCC |  | We think same as the default transmission repetition periodicity of SIB1, i.e., 20 ms, should be better. |
| OPPO |  | this is not RAN1 issue, can be left to RAN2 to discuss. RAN1 only needs to discuss the UE monitoring behavior |
| Apple | Option 1 |  |
| Futurewei | Option 1 |  |
| Nokia/NSB |  | It is related to Proposal 3-1, and we are open for discussions. We can be OK with 80ms and 160ms, but it is more sensible to be with 80ms considering of MIB update periodicity. |
| Samsung | Not support | We don’t think this proposal is necessary. The FL proposal 3-1 is enough for OD-SIB1 operation, and the transmission periodicity of OD-SIB1 is up to implementation. |
| Fujitsu | Option 1 |  |
| Xiaomi |  | We share the same view with OPPO. The transmission periodicity of SIB1 is defined in RAN1. Same thing for OD-SIB1. |
| NEC | Option 1 | We are of the view that aligning with the legacy SIB1 transmission periodicity of 160ms (Option 1) for on-demand SIB1 operations offers consistency and leverages established mechanisms. Our proposal for od-sib1-duration being a multiple of 160ms is based on this principle. |
| LG | Option 1 | Option 1 is aligned with the agreement. |
| DOCOMO | Option 1 | No specification impact. |
| ETRI |  | The meaning of the “transmission periodicity” in the proposal is not clear. There can be two interpretations: one is repetition periodicity (=160 ms), another is actual transmission periodicity (up to gNB implementation, i.e., at least once per every 160 ms). We think we don’t need to discuss this issue in RAN1. RAN2 can discuss if needed. |
| Spreadtrum | option 1 |  |
| Panasonic | Option 1 | To allow legacy UE camping and simplify the implementation of both gNB and UE. |
| vivo |  | Agree with Samsung, there is no need to discuss this. |
| Tejas | Option 1 | Agreement(RAN2#129-bis) When the cell supporting on demand SIB1 is broadcasting SIB1 (e.g. upon SIB1 request), legacy UE can camp on the cell if the legacy UE is able to acquire the broadcasted SIB1. No specification impact is foreseen. From TS 38.331 <Unwanted text omitted> - the SIB1 is transmitted on the DL-SCH with a periodicity of 160 ms and variable transmission repetition periodicity within 160 ms as specified in TS 38.213 [13], clause 13. The default transmission repetition periodicity of SIB1 is 20 ms but the actual transmission repetition periodicity is up to network implementation. For SSB and CORESET multiplexing pattern 1, SIB1 repetition transmission period is 20 ms. For SSB and CORESET multiplexing pattern 2/3, SIB1 transmission repetition period is the same as the SSB period (TS 38.213 [13], clause 13). SIB1 includes information regarding the availability and scheduling (e.g. mapping of SIBs to SI message, periodicity, SI-window size) of other SIBs with an indication whether one or more SIBs are only provided on-demand and, in that case, the configuration needed by the UE to perform the SI request. SIB1 is cell-specific SIB; <Unwanted text omitted> When NES CELL is transmitting SIB1 due to trigger from release 19 UE, from RAN2 agreement legacy UE can be allowed to camp on the cell. Hence, to allow legacy UE to camp on NES CELL, we support option 1 |
| China Telecom | Option 1. |  |
| Huawei, HiSilcon 1 |  | Seems RAN2 discussion and decision with no RAN1 impact . option 1 or 2 could lead to different UE assumptions on cell barring, etc... |
| Ericsson | Not support | Agree with Samsung, Option 1 is already clear from past agreements. |
| Ofinno | Option 1 |  |
| Lenovo | Option 1 |  |

- FL Proposal 3-3-2 (Fujitsu)

RAN1 to draw the following conclusion:

- Regarding the UE assumption on on-demand SIB1 repetition/transmission within the time window, UE assumes that the SIB1 transmitted within the time window follows the legacy SIB1 repetition mechanism, i.e., repetition within a 160 ms period.

- FL Proposal 3-3-3 (LG)

RAN1 to draw the following conclusion:

- The legacy behavior below is applied for OD-SIB1.

  - The SIB1 is transmitted on the DL-SCH with a periodicity of 160 ms and variable transmission repetition periodicity within 160 ms.

- FL Proposal 3-4 (Nokia, Ericsson)

Support NW indication of repetition periodicity or periodicity of OD-SIB1 transmission in UL WUS configuration.

FFS more parameters in the UL WUS configuration:

- SIB1 offset

- SIB1 search space mask

- Note: These parameters are also valid after the on-demand SIB1 acquisition procedure.

| Company | Support or not | Comment |
| --- | --- | --- |
| MTK | No | We tend to think there is no need to additionally indicate the transmission or repetition periodicity of OD-SIB1. All definitions can follow legacy. See previous RAN1 agreement below. RAN1 #118b Agreement For the repetition periodicity of SIB1 within the time window of on-demand SIB1: - Up to NW implementation (no change to existing specification) Note: 38.331 Clause 5.2.1 has the following text on SIB1 repetition periodicity - the SIB1 is transmitted on the DL-SCH with a periodicity of 160 ms and variable transmission repetition periodicity within 160 ms as specified in TS 38.213 [13], clause 13. The default transmission repetition periodicity of SIB1 is 20 ms but the actual transmission repetition periodicity is up to network implementation. For SSB and CORESET multiplexing pattern 1, SIB1 repetition transmission period is 20 ms. For SSB and CORESET multiplexing pattern 2/3, SIB1 transmission repetition period is the same as the SSB period (TS 38.213 [13], clause 13). |
| Google |  | OK |
| CMCC | No | Follow the legacy seems enough. |
| OPPO |  | this enhancement may not be needed in this release |
| Futurewei |  | We do not think is needed |
| Nokia/NSB |  | We are open for discussions. Regarding the indication of transmission periodicity of OD-SIB1 in WUS configuration, practically if the NW does not repeat OD-SIB1 transmission in every 20ms, the UE would still need to monitor and try to receive in every 20ms, which is a waste of UE power consumption for OD-SIB1 monitoring and reception. Thus, the indication can benefit for UE power consumption as it allows UE to determine how many occasions are available for OD-SIB1 reception. |
| Samsung | No |  |
| Fujitsu | Not support | Regarding the repetition periodicity, as RAN1 already supported that it is up to NW implementation to determine the repetition periodicity, it is not necessary to indicate repetition periodicity in UL WUS configuration. Regarding the periodicity, it is related to FL Proposal 3-3. If the legacy SIB1 periodicity is supported, then it is also not necessary to indicate the value in UL WUS configuration, |
| Xiaomi |  | Same view with FL. |
| NEC | No | We have agreed that parameters like od-sib1-duration and od-sib1-windowStartOffset are part of the UL WUS configuration. Regarding additional parameters for SIB1 repetition periodicity or search space masks within the on-demand SIB1 window, we should carefully consider the trade-off between providing more detailed assistance to the UE and maintaining a lean and friendlier implementation UL WUS framework. Our general preference is for solutions that minimize implementation complexity. |
| LG | No | We don’t see the necessity to support the additional parameters. |
| DOCOMO | No | Current parameters achieve enough flexibility. |
| ETRI | No | Same understanding as MTK. |
| Spreadtrum | No | We don’t see the necessity. |
| vivo | No | Following legacy is enough. |
| Tejas | No |  |
| China Telecom | No |  |
| Huawei, HiSilcon 1 | Not needed | This was discussed before and according to our understanding was concluded as not needed . given the time left we do not think it is needed to reopen this discussion again. |
| Ericsson | Support | Although not strictly needed for the feature, it simplifies UE monitoring for SIB1. |
| Ofinno | No | Agree with MTK |

- Parameters inside UL-WUS configuration other than od-sib1-duration:

On parameters inside UL-WUS configuration other than od-sib1-duration, companies’ views in RAN1 #121 are collected below:

OPPO:

Proposal 4:	RAN1 adopts TP#1 below.

--------------- TP#1 of TS 38.213 in Clause 23 -----------------------

Unless otherwise mentioned, the higher layer parameters in this clause and in referenced clauses are provided by SIB1-RequestConfig on a first cell.

A UE can be provided, by NES_CellId, a physical cell identity of a second cell and an ARFCN by ARFCN-ValueNR for SS/PBCH block receptions on the second cell. When

-	the UE receives an SS/PBCH block on the second cell, and

-	kSSB>23 for FR1 or kSSB>11 for FR2 is indicated by the SS/PBCH block on the second cell, and

-	conditions for PRACH transmission associated with the SS/PBCH block on the second cell are satisfied [12, TS 38.331],

the UE can transmit a PRACH associated with the SS/PBCH block on the second cell based on:

-	a timing adjustment indicated by n-TimingAdvanceOffset, if provided, as described in Clause 4.2

-	a power determined as described in Clause 7.4

-	a procedure determined as in Clause 8.1 based on Type-1 random access procedure

where kSSB for determining the common resource block [4, TS 38.211] is provided by k-ssb. A CORESET for Type0-PDCCH CSS and Type1-PDCCH CSS is provided by controlResourceSetZero.

--------------- end of TP#1 -------------------------------------------------------

ZTE, Sanechips

Proposal 6: Two options for UE to obtain the UL point A for TDD should be considered.

- Option 1: Configure offsetToPointA in the UL WUS configuration at least for TDD.

- Option 2: The absoluteFrequencyPointA is also present in IE FrequencyInfoUL for TDD.

vivo

Proposal 1: The design principle for WUS configuration is to minimize the size of mandatory parameters in WUS configuration as much as possible.

Observation 1: UE needs to distinguish whether the received WUS configuration applies to the serving cell or other NES cells.

Observation 2: UE can distinguish the received WUS configuration applies to the serving cell or other NES cells based on PhysCellId which is mandatory configured in WUS configuration.

Proposal 2: If a cell indicates a WUS configuration applying to serving cell itself, some parameters can follow the corresponding values in the SIB1 of serving cell itself to further reduce the payload of WUS configuration.

Proposal 3: Some parameters in WUS configuration can follow the value of the counterpart of cellA to further reduce the payload of WUS configuration.

Proposal 4: Include offsetToPointA in the WUS configuration that is mandatorily present for TDD (to derive UL point A as in legacy).

Proposal 5: frequencyBandList is mandatorily present in WUS configuration for TDD system, which refers to the IE within FrequencyInfoDL-SIB.

Proposal 11: support to change the following names to align with RAN2 running CR.

rsrp-ThresholdSSB -> rsrp-SIB1ThresholdSSB,

ra-PreambleStartIndex -> ra-SIB1PreambleStartIndex,

ra-AssociationPeriodIndex -> ra-AssociationPeriodIndexSib1

NEC

Proposal 1: If WUS resources are configured independent of PRACH resources for other usages, discuss the cases of time and/or frequency overlap of WUS resources and RACH resources for other usages.

Tejas

Proposal 1: The parameters ‘absoluteFrequencyPointA’, ‘offsetToCarrier’ and ‘locationAndBandwidth’ need to be mandatorily present in the UL-WUS configuration for both FDD and TDD system.

Google

Proposal 2: Support to configure the bandwidth for downlink to improve the performance for RAR and ODSIB1.

- FL Proposal 3-6 (OPPO)

RAN1 to adopt TP#1 below:

--------------- TP#1 of TS 38.213 in Clause 23 -----------------------

Unless otherwise mentioned, the higher layer parameters in this clause and in referenced clauses are provided by SIB1-RequestConfig on a first cell.

A UE can be provided, by NES_CellId, a physical cell identity of a second cell and an ARFCN by ARFCN-ValueNR for SS/PBCH block receptions on the second cell. When

-	the UE receives an SS/PBCH block on the second cell, and

-	kSSB>23 for FR1 or kSSB>11 for FR2 is indicated by the SS/PBCH block on the second cell, and

-	conditions for PRACH transmission associated with the SS/PBCH block on the second cell are satisfied [12, TS 38.331],

the UE can transmit a PRACH associated with the SS/PBCH block on the second cell based on:

-	a timing adjustment indicated by n-TimingAdvanceOffset, if provided, as described in Clause 4.2

-	a power determined as described in Clause 7.4

-	a procedure determined as in Clause 8.1 based on Type-1 random access procedure

where kSSB for determining the common resource block [4, TS 38.211] is provided by k-ssb. A CORESET for Type0-PDCCH CSS and Type1-PDCCH CSS is provided by controlResourceSetZero.

--------------- end of TP#1 -------------------------------------------------------

Reason of change: The CORESET#0 configuration is not included in Clause 23 of current R19 draft spec. Thus, the UE cannot know that the configuration of CORESET#0 is provided by SIB1-RequestConfig if not specified.

| Company | Support or not | Comment |
| --- | --- | --- |
| MTK | Yes |  |
| Google | No | This seems to be redundant. |
| Sharp | Yes |  |
| CMCC | Yes |  |
| ZTE, Sanechips | OK |  |
| OPPO | Support |  |
| Apple | Yes |  |
| Futurewei | OK |  |
| Nokia/NSB | OK |  |
| Samsung | Support with updated TP. | Support the OPPO’s TP. For further clarification, we suggest the following updated TP: RAN1 to adopt TP#1 below: --------------- TP#1 of TS 38.213 in Clause 23 ----------------------- Unless otherwise mentioned, the higher layer parameters in this clause and in referenced clauses are provided by SIB1-RequestConfig on a first cell. A UE can be provided, by NES_CellId, a physical cell identity of a second cell and an ARFCN by ARFCN-ValueNR for SS/PBCH block receptions on the second cell. When - the UE receives an SS/PBCH block on the second cell, and - kSSB>23 for FR1 or kSSB>11 for FR2 is indicated by the SS/PBCH block on the second cell, and - conditions for PRACH transmission associated with the SS/PBCH block on the second cell are satisfied [12, TS 38.331], the UE can transmit a PRACH associated with the SS/PBCH block on the second cell based on: - a timing adjustment indicated by n-TimingAdvanceOffset, if provided, as described in Clause 4.2 - a power determined as described in Clause 7.4 - a procedure determined as in Clause 8.1 based on Type-1 random access procedure where kSSB for determining the common resource block [4, TS 38.211] is provided by k-ssb. A CORESET for Type0-PDCCH CSS and Type1-PDCCH CSS is provided by controlResourceSetZero. The UE determines a CORESET of the Type0-PDCCH CSS from controlResourcesetZero provided in [uplink-WUS-Config] and determines Type0-PDCCH monitoring occasions from searchSpaceZero provided in [uplink-WUS-Config] --------------- end of TP#1 ------------------------------------------------------- |
| Fujitsu | Support |  |
| Sony | Support |  |
| Xiaomi |  | Same feeling as Google. RAN1 has the following agreement on the search space and CORESET used for PDCCH monitoring of OD-SIB1. UE is clear about everything on OD-SIB1 PDCCH monitoring. We don’t need to explicitly capture it in 213. Agreement For type 0 PDCCH monitoring occasions of on demand SIB1, searchSpaceZero and controlResourceSetZero for on-demand SIB1 are provided from UL WUS configuration if SSB on NES cell is on sync raster and K_SSB is not equal to 30 in FR1 or 14 in FR2. |
| NEC | Support |  |
| LG | Support |  |
| ETRI | Support |  |
| Panasonic | Support |  |
| vivo | Support |  |
| Tejas | Support |  |
| Huawei, HiSilcon 1 | Not needed | Can be discussed later. |
| Ericsson | Support | Support Samsungs TP. |
| Lenovo | Support |  |
| OPPO2 |  | Samsung’s modified TP is better and clearer. We support Samsung’s modified TP. |

- FL Proposal 3-6-1 (Samsung)

RAN1 to adopt TP#1 below:

--------------- TP#1 of TS 38.213 in Clause 23 -----------------------

Unless otherwise mentioned, the higher layer parameters in this clause and in referenced clauses are provided by SIB1-RequestConfig on a first cell.

A UE can be provided, by NES_CellId, a physical cell identity of a second cell and an ARFCN by ARFCN-ValueNR for SS/PBCH block receptions on the second cell. When

-	the UE receives an SS/PBCH block on the second cell, and

-	kSSB>23 for FR1 or kSSB>11 for FR2 is indicated by the SS/PBCH block on the second cell, and

-	conditions for PRACH transmission associated with the SS/PBCH block on the second cell are satisfied [12, TS 38.331],

the UE can transmit a PRACH associated with the SS/PBCH block on the second cell based on:

-	a timing adjustment indicated by n-TimingAdvanceOffset, if provided, as described in Clause 4.2

-	a power determined as described in Clause 7.4

-	a procedure determined as in Clause 8.1 based on Type-1 random access procedure

where kSSB for determining the common resource block [4, TS 38.211] is provided by k-ssb. The UE determines a CORESET of the Type0-PDCCH CSS from controlResourcesetZero provided in [uplink-WUS-Config] and determines Type0-PDCCH monitoring occasions from searchSpaceZero provided in [uplink-WUS-Config]

--------------- end of TP#1 -------------------------------------------------------

Reason of change: The CORESET#0 configuration is not included in Clause 23 of current R19 draft spec. Thus, the UE cannot know that the configuration of CORESET#0 is provided by SIB1-RequestConfig if not specified.

| Company | Support or not | Comment |
| --- | --- | --- |

- FL Proposal 3-7 (ZTE, vivo, Tejas)

RAN1 to adopt one of the following two options for UE to obtain the UL point A for TDD system.

- Option 1: Configure offsetToPointA in the UL WUS configuration at least for TDD system.

- Option 2: Configure absoluteFrequencyPointA under the IE FrequencyInfoUL also for TDD system.

| Company | Support or not | Comment |
| --- | --- | --- |
| MTK | Support | We think both options can work. Slightly prefer Option 1. |
| Google | Support |  |
| CMCC | Support | Option 1 and Option 2 are both OK for us. BTW, we already have the following agreement: Thus, seems the IE in Option 2 is already introduced. |
| ZTE, Sanechips | Support | Both options can be considered and option 1 is more in line with the legacy specification. |
| OPPO |  | Option 1 seems to be following the legacy definition |
| Apple |  | Option 1 is preferred following legacy. |
| Futurewei | OK | Option 1 is preferred |
| Samsung |  | Support FL’s proposal, we prefer Option 2 since absoluteFrequencyPointA already has been introduced in WUS configuration. |
| Fujitsu | Support option 1 |  |
| Xiaomi |  | Prefer Option 1. |
| LG | Option 1 |  |
| vivo | Support | Option 1 is preferred since it is more aligned with legacy behavior |
| Tejas |  | absoluteFrequencyPointA should be sufficient, which is common to both FDD and TDD system. It is already agreed in previous RAN1 meeting as CMCC pointed out. |
| Ericsson |  | Option 2 is preferred since absoluteFrequencyPointA already has been introduced in WUS configuration. |
| Ofinno | Support | We prefer option 1 |
| Moderator |  | The following agreement can be considered along with the discussions: Agreement From RAN1 perspective, for agreed UL WUS parameters, regarding their mandatory or optional presence and applicability to TDD and/or FDD, adopt the followings: - PhysCellId and ARFCN-ValueNR are mandatory - frequencyBandList and absoluteFrequencyPointA are present in IE FrequencyInfoUL for FDD (as in the legacy specification) - K_SSB is mandatory - searchSpaceZero and controlResourceSetZero are mandatory - ra-PreambleStartIndex, od-sib1-duration, offsetToTimeWindow are mandatory |

- FL Proposal 3-8 (vivo)

If a cell indicates a WUS configuration applying to serving cell itself, some parameters can follow the same values in the SIB1 of the cell itself to reduce the payload of WUS configuration.

| Company | Support or not | Comment |
| --- | --- | --- |
| MTK |  | This kind of enhancement can be left to RAN2. |
| Google |  | We also think this is a RAN2 issue |
| CMCC |  | Support but final decision may left to RAN2. |
| ZTE, Sanechips | No | The WUS configurations for a NES cell can be carried by more than one cells. In order to avoid the ambiguity, the WUS configurations for one cell among different cells should be consistent. And the WUS configuration can be cell specific or area specific. Especially for area specific WUS configuration, the restrictions are more stringent. |
| OPPO |  | OK to leave for RAN2 |
| Apple |  | We prefer to discuss case by case as suggested by vivo |
| Futurewei |  | OK to leave to RAN2 |
| Samsung |  | Same view as MTK |
| Fujitsu |  | This can be left to RAN2. |
| Sony |  | This issue is up to RAN2. |
| Xiaomi |  | OK to leave to RAN2. |
| NEC |  | Can be left to RAN2 |
| LG |  | It is up to RAN2 discussion. |
| ETRI |  | We already discussed similar issue in a previous meeting and concluded to leave it to RAN2. |
| Spreadtrum |  | Can leave to RAN2. |
| vivo |  | We can discuss the general principle in RAN1 since the content of WUS configuration for camping cell, signalling the same parameters twice is not necessary. |
| Tejas |  | Should be left to RAN2 |
| Huawei, HiSilcon 1 | Not needed | Seems RAN2 decision to take |
| Ericsson | No | The UL WUS configuration need to be the same regardless of which cell that provides it because otherwise the systemInformationAreaId + ValueTag cannot be used. |
| Ofinno | No | Should be left for RAN2 |

- FL Proposal 3-9 (vivo)

Some parameters in WUS configuration can follow the values of cell A to reduce the payload of WUS configuration.

| Company | Support or not | Comment |
| --- | --- | --- |
| MTK |  | This kind of enhancement can be left to RAN2. |
| Google |  | We also think this is a RAN2 issue |
| CMCC |  | Support but final decision may left to RAN2. |
| ZTE, Sanechips |  | Similar as FL proposal 3-8. |
| OPPO |  | OK to leave for RAN2 |
| Apple |  | We prefer to discuss case by case as suggested by vivo |
| Futurewei |  | OK to leave it to RAN2 |
| Samsung |  | Same view as MTK |
| Fujitsu |  | Same view as in FL Proposal 3-8 |
| Sony |  | This issue is up to RAN2. |
| xiaomi |  | OK to leave to RAN2. |
| NEC |  | Can be left to RAN2 |
| LG |  | It is up to RAN2 discussion. |
| ETRI |  | RAN2 can discuss if needed. |
| vivo |  | We can discuss the general principle in RAN1 since the content of WUS configuration for camping cell, signalling the same parameters twice is not necessary. |
| Tejas |  | Should be left to RAN2 |
| Huawei, HiSilcon 1 | Not needed | Seems RAN2 decision to take |
| Ericsson |  | RAN2 issue. And System-Information update procedure will become really complicated. |
| Ofinno |  | Leave it for RAN2 to decide. |

- FL Proposal 3-10 (vivo)

The frequencyBandList is mandatorily present in WUS configuration for TDD system, which refers to the IE within FrequencyInfoDL-SIB.

| Company | Support or not | Comment |
| --- | --- | --- |
| MTK | Yes |  |
| Google | Support |  |
| CMCC | Support |  |
| OPPO |  | Does this proposal impact FL proposal 3-7? |
| Apple | Yes |  |
| Futurewei | OK |  |
| Samsung | Support |  |
| Xiaomi | Yes |  |
| NEC | Support |  |
| vivo | Support |  |
| Tejas | Support |  |
| Ofinno | Support |  |

- FL Proposal 3-11 (Tejas)

The parameters ‘absoluteFrequencyPointA’, ‘offsetToCarrier’ and ‘locationAndBandwidth’ are mandatorily present in the UL-WUS configuration for both FDD and TDD system.

| Company | Support or not | Comment |
| --- | --- | --- |
| MTK | Yes | For the “absoluteFrequencyPointA” part, it’s the same topic as in FL Proposal 3-6. |
| Google | Support |  |
| CMCC |  | Same view as MTK |
| ZTE, Sanechips |  | Can be discussed after FL Proposal 3-7. |
| OPPO | No | It depends the outcome of FL proposal 3-7 |
| Futurewei |  | OK in principle. We agreed that should be discussed after the Proposal 3-7. |
| Samsung | Support |  |
| vivo | Not support |  |
| Tejas | Support |  |
| Ofinno | Support |  |

- FL Proposal 3-11-2 (Tejas)

The parameters ‘absoluteFrequencyPointA’, ‘offsetToCarrier’ and ‘locationAndBandwidth’ are mandatorily present in the UL-WUS configuration for both FDD and TDD system.

| Company | Support or not | Comment |
| --- | --- | --- |

- FL Proposal 3-12 (Google)

Configure the bandwidth for downlink in the UL WUS configuration (to improve the performance for RAR and OD-SIB1).

| Company | Support or not | Comment |
| --- | --- | --- |
| Google | Support |  |
| OPPO | No | no needed. The RAR performance is not worse than legacy system |
| Futurewei |  | We think that is not needed |
| ETRI |  | We think enhancing the RAR reception performance is not a NES-specific issue and it would be the out of scope. |
| vivo | No |  |

- Issue 4: UL-WUS on NES cell overlapping with paging occasion on normal cell

Background

It is agreed in RAN2 #129b that:

Agreement

Leave this issue (how to handle paging interruption during OD-SIB1 acquisition) to RAN4.

Companies’ views for this topic in RAN1 #121 are collected below:

MTK

Proposal 2: A potential issue of paging dropping may arise if UL-WUS on NES cell overlapped with paging occasion on Cell A as shown in Figure 1. UE should prioritize paging if the overlapping happens.

Figure 1. An Example of UL-WUS on NES cell overlapped with paging occasion on normal cell

Transsion:

Proposal 4  There is no need to resolve the issue of UL WUS on NES cell overlapping with paging occasion on normal cell.

CAICT:

Proposal 3:  It is up to UE implementation to prioritize receiving the paging occasion from Cell A or transmitting the UL WUS to NES cell, if the UL-WUS occasions on NES cell overlapped with the paging occasion on Cell A.

ZTE, Sanechips

Proposal 5: UE behavior in the case that the resources of UL WUS in NES cell and paging occasions on cell A are overlapped can be up to UE implementation.

Ofinno

Proposal 3: The impact of UL WUS in the NES cell on paging in the normal cell is up to RAN4 decision.

Fujitsu

Proposal 5. How to handle the collision issue between UL WUS occasions in NES cell and paging occasions in Cell A is left to RAN4.

For this topic, companies’ views in RAN1 #121 are summarized below

What should UE do if UL-WUS on NES cell overlapped with paging occasion on Cell A

- Up to UE implementation: Transsion, CAICT, ZTE/Sanechips,

- UE prioritizes receiving the paging occasion from Cell A: MTK

- Left to RAN4: Ofinno, Fujitsu, MTK

- FL Proposal 4-1

It is up to RAN4 to discuss whether to prioritize paging if the UL-WUS occasions on NES cell overlapped with the paging occasion on Cell A.

Figure example of UL-WUS on NES cell overlapped with paging occasion on Cell A from [29, MediaTek]

| Company | Support or not | Comment |
| --- | --- | --- |
| MTK | Yes | It seems RAN4 is discussing the paging interruption and this can be left to them (which is aligned with RAN2’s agreement below). RAN2 129b Agreement Leave this issue (how to handle paging interruption during OD-SIB1 acquisition) to RAN4. |
| Google | Support |  |
| CMCC | Yes | And we also think this can be solved via implantation, e.g., by properly configure the resource of PO and RO for UL-WUS. |
| OPPO | Yes, but | RAN1 needs to send an LS to inform RAN4 about this issue |
| Apple | Yes | RAN4 is already discussing this issue. |
| Futurewei | OK |  |
| Samsung | Support |  |
| Fujitsu | Support |  |
| Sony | Support |  |
| NEC | Support | We agree that this is a matter best addressed by RAN4, which handles UE requirements related to paging and interruptions. |
| LG | Support |  |
| DOCOMO | Yes | It seems not a RAN1 issue. |
| Spreadtrum | Support |  |
| vivo | Support |  |
| CATT | OK |  |
| Tejas | Support |  |
| Huawei, HiSilcon |  | Not needed nor clear why it is up to RAN4? |
| Ericsson |  | Agree with CMCC that this can be solved via implementation. An LS to RAN4 is not needed. In fact, the proposal is not needed. |
| Ofinno | Support | Agree RAN4 is discussing and let them decide. |

- Issue 5: UE to monitor the RAR for SIB1 requests sent by other UEs

Background

The following proposals are brought up in RAN1 #121:

LG:

Proposal #2: To support UEs to monitor the RAR for SIB1 requests sent by other UEs, UE may monitor DCI scrambled by RA-RNTI which is associated with RO allocated for on-demand SIB1 but not associated with RO containing PRACH transmitted by UE.

- FL Proposal 5-1 (LG)

UE monitors DCI scrambled by RA-RNTI which is associated with RO allocated for on-demand SIB1 but not associated with RO containing PRACH transmitted by UE.

- To support UEs to monitor the RAR for SIB1 requests sent by other UEs,

| Company | Support or not | Comment |
| --- | --- | --- |
| MTK | No | We tend to think this can be left to UE implementation. As the probability of multiple UEs requesting on-demand SIB1 at the same time is low, the benefits for UEs to monitor the RAR for SIB1 requests sent by other UEs seems low to us. |
| Google |  | We think this can be up to UE implementation |
| CMCC | No | We do not observe the need and benefit. |
| ZTE, Sanechips | No | No motivations are observed. |
| OPPO | No |  |
| Apple | No | We also think this can be left for UE implementation. |
| Futurewei |  | We think that is a good to have, but may be left for the implementation |
| Nokia/NSB |  | Different from LG’s view, to our understanding, the RO used for OD-SIB1 PRACH transmission is common for all the UEs. And with this assumption, the UE could and can monitor the RAR for SIB1 requests send by other UEs. |
| Samsung | Not support | We think it is up to UE implementation. |
| Fujitsu |  | It can be up to UE implementation. |
| xiaomi |  | We also think this can be up to UE implementation |
| NEC |  | The potential for a UE to monitor RARs intended for other UEs is an optimization that could be considered, but its benefits versus complexity would need careful evaluation, and it might be best left to UE implementation. |
| LG |  | Given the majority views, we are OK with left it to UE implementation. |
| DOCOMO | No |  |
| ETRI |  | Can be up to UE implementation. |
| Spreadtrum | Not | Up to UE implementation |
| vivo | No |  |
| Tejas |  | Up to UE implementation. |
| Huawei , Hisilicon 1 |  | up to UE implementation this is same as we agreed for OD-SIB1 scheduling DCI monitoring before sending WUS |
| Ofinno | No |  |
| Lenovo | No |  |

- Issue 6: Relation between OD-SIB1 monitoring window and RAR window

Background

In RAN1 #120, it is agreed that:

Agreement

The reference time point to determine the window starting time for on-demand SIB1 is based on the starting slot of the RAR window for UL WUS wherein UE successfully received a RAR.

In RAN2 #129, it is agreed that:

Agreement

Upon reception of RAR, the UE monitors OD-SIB1 in the window agreed by RAN1.

In RAN1 #120-bis, it is agreed that:

Agreement

After transmitting a UL-WUS, UE is not required to monitor Type0-PDCCH occasion (for OD-SIB1) before UE successfully received its RAR in response to the UL WUS.

Companies’ views for this topic in RAN1 #121 are collected below:

China Telecom:

Proposal 3: It should be up to UE’s implement to monitor OD-SIB1 during the RAR window before the OD-SIB1 window starts.

LG:

Proposal #1: RAN1 to discuss whether to specify the UE behaviour if an OD-SIB1 PDCCH is detected in Type0-PDCCH occasion before the UE receives the RAR after the UL WUS transmission.

Apple:

Proposal 2: UE starts monitoring Type-0 PDCCH occasions within the OD-SIB1 monitoring window, from the

first PDCCH monitoring occasion that is at least N slots from the ending slot of RAR reception.

DCM:

Proposal 4:

- UE monitors from the first type 0 PDCCH monitoring occasion associated with one or all SSB(s) according to 1-bit indication in UL-WUS from the starting of the slot which is n slot after the slot of PDSCH of RAR reception.

  - where, n=3Nslotsubframe,μ

Fig.1 Starting time of PDCCH monitoring occasion

Fujitsu

Observation 2. When the on-demand SIB1 time window overlaps with the RAR window, skipping Type0- PDCCH occasions can lead to PDCCH reception failure and increased network energy consumption. Therefore, it is crucial to strictly specify when the UE shall start monitoring Type0-PDCCH occasion for on-demand SIB1.

Proposal 2. The UE starts the PDCCH monitoring at the first symbol of the earliest CORESET the UE is configured to receive PDCCH for Type0-PDCCH CSS set, after whichever is later between the time offset relative to the starting slot of the RAR window and the RAR reception.

Sharp:

Proposal 2: If ra-SearchSpace is not provided by WUS-Config, allow the UE to monitor the PDCCH according to a Type1-PDCCH CSS set provided by searchSpaceZero for DCI format 1_0 with CRC scrambled not only by RA-RNTI but also by SI-RNTI during the ra_ResponseWindow.

Lenovo:

Proposal 1: Adopt the following TP for TS38.213,

<Unrelated part omitted> The UE starts monitoring PDCCH to detect the DCI format 1_0 with CRC scrambled by the SI-RNTI after a number of slots provided by od-sib1-windowStartOffset from the starting slot of the window controlled by ra_ResponseWindow, and for a number of slots provided by od-sib1-WindowDuration. [If the SS/PBCH block on the second cell indicates kSSB>23 for FR1 or kSSB>11 for FR2,] After the PRACH transmission, the UE is not required to monitor PDCCH on the second cell to detect the DCI format 1_0 with CRC scrambled by the SI-RNTI prior to a reception of a PDSCH scheduled by the DCI format 1_0 with CRC scrambled by the RA-RNTI. <Unrelated part omitted>

OPPO:

Proposal 6: RAN1 to adopt TP#3 below.

--------------- TP#3 of TS 38.213 in Clause 23 -----------------------

If the UE identifies a RAPID associated with a corresponding PRACH transmission from the UE in a PDSCH reception scheduled by the DCI format 1_0 with CRC scrambled by the RA-RNTI, the UE can be indicated by higher layers to monitor PDCCH on the second cell to detect a DCI format 1_0 with CRC scrambled by the SI-RNTI according to a Type0-PDCCH CSS set provided by SearchSpaceZero. If the UE is provided XYZ, the UE monitors PDCCH only in monitoring occasions associated with the SS/PBCH block. The UE starts monitoring PDCCH to detect the DCI format 1_0 with CRC scrambled by the SI-RNTI in the next monitoring occasion after a number of slots provided by od-sib1-windowStartOffset from the starting slot of the window controlled by ra_ResponseWindow, and for a number of slots provided by od-sib1-WindowDuration.

--------------- end of TP#3 -------------------------------------------------------

Proposal 7: RAN1 to adopt TP#4 below.

--------------- TP#4 of TS 38.213 in Clause 23 -----------------------

[If the SS/PBCH block on the second cell indicates kSSB>23 for FR1 or kSSB>11 for FR2,] the UE is not required to monitor PDCCH on the second cell to detect the DCI format 1_0 with CRC scrambled by the SI-RNTI prior to a reception of a PDSCH scheduled by the DCI format 1_0 with CRC scrambled by the RA-RNTI.

--------------- end of TP#4 -------------------------------------------------------

Tejas

Proposal 2: The reference point to determine the starting slot of on-demand SIB1 window is the start of the slot containing the starting symbol of the RAR window.

Huawei, HiSilicon

Proposal 1: Adopt TP#1 to clarify the reference time point to determine the starting slot of on-demand SIB1 window is the slot containing the starting symbol of RAR window.

- FL Proposal 6-1 (LG)

RAN1 to discuss whether to specify the UE behaviour if an OD-SIB1 PDCCH is detected in Type0-PDCCH occasion before the UE receives the RAR after the UL WUS transmission.

| Company | Support or not | Comment |
| --- | --- | --- |
| MTK | No | It can be up to UE implementation. No need to further specify UE behavior. |
| Google |  | We think it can be up to UE implementation |
| CMCC | No | Same view as MTK. |
| OPPO |  | Please see our comment in FL proposal 2-2 |
| Apple | No |  |
| Futurewei |  | It can be up to UE implementation |
| Samsung | Not | Share same view as MTK. |
| Fujitsu | No | Same view as MTK. |
| Xiaomi | No | Same view as MTK. |
| NEC |  | Left to UE implementation |
| LG | Support | We understand that UE does not require to monitor OD-SIB1 PDCCH until receiving RAR. However, if UE detect the OD-SIB1 by the implementation, the OD-SIB1 reception delay and the power consumption of gNB/UE can be reduced by terminating monitor RAR in the OD-SIB1 procedure and receiving OD-SIB1 directly. |
| DOCOMO | No | Same view as FL. |
| ETRI |  | We think the specification needs not differentiate UE behavior between the two cases where the OD-SIB1 was received before and after the RAR. This can be handled by UE implementation. |
| Spreadtrum | Not | Up to UE implementation |
| Panasonic | No |  |
| vivo | No |  |
| Tejas |  | Up to UE implementation |
| Huawei, HiSilicon 1 |  | What to discuss is not clear? The UE in this case can receive OD-SIB1 directly which is the ultimate goal of the UE in this feature. Then it is up to the UE if it wants to monitor the DCI of the RAR and receive the RAR or not, if it want to waste its energy . |
| Ericsson |  | Same view as MTK. |
| Ofinno | No | Agree with MTK |

- FL Proposal 6-2 (Apple, DOCOMO, Fujitsu)

After transmitting the UL WUS, UE starts Type 0 PDCCH monitoring from the later one between N slots after RAR reception and offsetToTimeWindow slots after the reference time point.

- [N= 3Nslotsubframe,μ]

Figure to illustrate RAR processing from [34, vivo]

| Company | Support or not | Comment |
| --- | --- | --- |
| MTK | Yes | This seems can be used to accommodate RAR processing time for UE side. The N value (3Nslotsubframe,μ) proposed from DOCOMO can be fine to us. |
| Google | Support | At least 28 symbols are necessary, similar to PDCCH monitoring after UE receives the response for PCell BFR |
| CMCC |  | Okay |
| ZTE, Sanechips | No | The RAR processing time does not affect the end time of the time window. And when the UE starts to monitor Type 0 PDCCH for OD-SIB1 can be up to UE implementation. No additional offset is needed. |
| OPPO | No | In our understanding, for OSI when UE receives RAR, UE starts to monitor SI, there is no such RAR processing time defined. Thus, we cannot agree with the necessity of the proposal. |
| Apple | Yes | We think this is important to ensure that UE can at least monitor one Type0 PDCCH monitoring occasion especially when the OD-SIB1 monitoring window is short. |
| Futurewei |  | We do not see it necessary. Same understanding as Oppo. |
| Nokia/NSB | No | Not need to specify such parameter |
| Fujitsu | Support | We think it is crucial to strictly specify when the UE shall start monitoring Type0-PDCCH occasion for on-demand SIB1 to avoid PDCCH reception failure and unnecessary network energy consumption. |
| Samsung |  | We’d like to further clarify the meaning of value N. From our point of view, the value of N reflects processing time of RAR. To be clear, we suggest updated proposal as below: - Updated FL Proposal 6-2 (Apple, DOCOMO, Fujitsu) After transmitting the UL WUS, UE starts Type 0 PDCCH monitoring from the later one between N slots after the slot where UE receives RAR in response to UL WUS after RAR reception and offsetToTimeWindow slots after the reference time point. - [N= 3Nslotsubframe,μ] Figure to illustrate RAR processing from [34, vivo] |
| xiaomi |  | We don’t see the necessity of this proposal either. |
| NEC | Support | This respects both the configured window start and the UE's need for RAR processing time, and aligns with the agreement that monitoring isn't required before successful RAR reception. |
| LG | Support with the modifications | We suggest the following modification to account for the UE starting PDCCH monitoring on the first symbol of the CORESET0: After transmitting the UL WUS, UE starts Type 0 PDCCH monitoring at the first symbol of the earliest CORESET the UE is configured to receive PDCCH for Type 0 PDCCH CSS set, from the later one between N slot after RAR reception and offsetToTimeWindow slots after reference time point. |
| DOCOMO | Yes |  |
| ETRI |  | Same understanding as OPPO. |
| vivo | Support |  |
| Tejas | Support |  |
| Huawei, HiSilicon 1 | Do not support | Agree with ZTE |
| Ericsson | Not support |  |
| Ofinno | Support |  |

- FL Proposal 6-3 (Sharp)

If ra-SearchSpace is not provided by WUS-Config, UE monitors the PDCCH according to a Type1-PDCCH CSS set provided by searchSpaceZero for DCI format 1_0 with CRC scrambled not only by RA-RNTI but also by SI-RNTI during the ra_ResponseWindow.

| Company | Support or not | Comment |
| --- | --- | --- |
| MTK | No | Seems like an enhancement with marginal benefits to us. |
| Google | No | Agree with FL |
| CMCC | No | Same view as MTK. |
| OPPO | No | If the UE monitors SI-RNTI, what is the reason to ask UE also monitor RA-RNTI? |
| Apple |  | We have a previous agreement in RAN1 #118bis as following which is already reflecting the above proposal. Agreement Search space for RAR in response to UL WUS on a NES Cell can be provided by the UL WUS configuration. If not, follow the search space zero for the NES Cell. |
| Futurewei |  | Same understanding as Apple |
| Samsung | Not support |  |
| Fujitsu | Not support | Same view as MTK. |
| Sharp | Yes | According to the RAN#118bis agreements, Agreement (RAN#118bis) Search space for RAR in response to UL WUS on a NES Cell can be provided by the UL WUS configuration. If not, follow the search space zero for the NES Cell. It can be observed that the UE monitors PDCCH in the same search space (i.e., searchSpaceZero) for the same DCI format (i.e., DCI format 1_0) both before and after RAR reception if ra-SearchSpace is not provided by WUS-Config. Therefore, OD-SIB1 reception latency can be reduced with minimal UE overhead by allowing the UE to monitor the PDCCH according to a Type1-PDCCH CSS set provided by searchSpaceZero for a DCI format with CRC scrambled by both RA-RNTI and SI-RNTI during the ra_ResponseWindow. Moreover, as it is possible that some UE does not monitor Type 0 PDCCH for SIB1 transmission before transmitting UL WUS (based on following agreement), Agreement (RAN#120bis) If a UE has SIB1 request configuration of a cell and before transmitting UL WUS, - If the UE detects a SSB where K_SSB>=24 for FR1 or K_SSB>=12 for FR2, select the following: - Alt. 3: It is up to UE implementation on whether to monitor Type 0 PDCCH for SIB1 transmission we think it is possible to help those UEs reducing OD-SIB1 reception latency only with very little effort as those UE anyway needs to monitor PDCCH candidate for RAR. |
| Xiaomi |  | Same understanding as Apple. |
| LG | No |  |
| DOCOMO | No |  |
| ETRI | Not support |  |
| Spreadtrum | Not |  |
| vivo | No |  |
| Ofinno | Not support |  |

- FL Proposal 6-4 (Lenovo)

Adopt the following TP for TS38.213 Clause 23.

<Unrelated part omitted> The UE starts monitoring PDCCH to detect the DCI format 1_0 with CRC scrambled by the SI-RNTI after a number of slots provided by od-sib1-windowStartOffset from the starting slot of the window controlled by ra_ResponseWindow, and for a number of slots provided by od-sib1-WindowDuration. [If the SS/PBCH block on the second cell indicates kSSB>23 for FR1 or kSSB>11 for FR2,] After the PRACH transmission, the UE is not required to monitor PDCCH on the second cell to detect the DCI format 1_0 with CRC scrambled by the SI-RNTI prior to a reception of a PDSCH scheduled by the DCI format 1_0 with CRC scrambled by the RA-RNTI. <Unrelated part omitted>

Reason of change: To better align with previous RAN1 agreement:

- “After transmitting a UL-WUS, UE is not required to monitor Type0-PDCCH occasion (for OD-SIB1) before UE successfully received its RAR in response to the UL WUS.”

| Company | Support or not | Comment |
| --- | --- | --- |
| MTK | Support |  |
| Google | No | It is agreed UE needs to monitor the PDCCH for SIB1 as legacy when k_SSB is smaller than 23 for FR1 and smaller than 11 for FR2 |
| Sharp | Support |  |
| CMCC | Support |  |
| ZTE, Sanechips | Support |  |
| OPPO | No | Please see our comment in FL proposal 2-2 |
| Apple |  | We think the original CR is clear. |
| Nokia/NSB | Support |  |
| Fujitsu | Support |  |
| NEC | Support |  |
| LG | Support |  |
| DOCOMO | Support | To align with the agreement. |
| ETRI | OK |  |
| Huawei, HiSilicon 1 | No | We have the two agreements below which are captured by the original CR and change the wording to capture only one is not ok. Noting that “UE is not required to” and “It is up to UE implementation” has the same meaning. Agreement@120bis After transmitting a UL-WUS, UE is not required to monitor Type0-PDCCH occasion (for OD-SIB1) before UE successfully received its RAR in response to the UL WUS. Agreement@120bis If a UE has SIB1 request configuration of a cell and before transmitting UL WUS, - If the UE detects a SSB where K_SSB>=24 for FR1 or K_SSB>=12 for FR2, select the following: - Alt. 3: It is up to UE implementation on whether to monitor Type 0 PDCCH for SIB1 transmission |
| Ericsson | Support |  |
| Ofinno | Support |  |

- FL Proposal 6-5 (OPPO)

RAN1 to adopt TP#3 below.

--------------- TP#3 of TS 38.213 in Clause 23 -----------------------

If the UE identifies a RAPID associated with a corresponding PRACH transmission from the UE in a PDSCH reception scheduled by the DCI format 1_0 with CRC scrambled by the RA-RNTI, the UE can be indicated by higher layers to monitor PDCCH on the second cell to detect a DCI format 1_0 with CRC scrambled by the SI-RNTI according to a Type0-PDCCH CSS set provided by SearchSpaceZero. If the UE is provided XYZ, the UE monitors PDCCH only in monitoring occasions associated with the SS/PBCH block. The UE starts monitoring PDCCH to detect the DCI format 1_0 with CRC scrambled by the SI-RNTI in the next monitoring occasion after a number of slots provided by od-sib1-windowStartOffset from the starting slot of the window controlled by ra_ResponseWindow, and for a number of slots provided by od-sib1-WindowDuration.

--------------- end of TP#3 -------------------------------------------------------

Reason of change: Current spec text may cause confusion that the UE starts monitoring after a number of slots provided by od-sib1-windowStartOffset from the starting slot of the window controlled by ra_ResponseWindow, even without PDCCH monitoring occasions available in the slot.

| Company | Support or not | Comment |
| --- | --- | --- |
| MTK | Support | Fine to clarify this. |
| Google |  | OK |
| Sharp | Support |  |
| OPPO | Support |  |
| Futurewei | OK |  |
| Fujitsu |  | It is related to FL Proposal 6-2. We suggest to discuss it after FL Proposal 6-2. |
| NEC | Support |  |
| LG |  | Support in principle. However, the terminology may be needed to be modified to align with the current specification. |
| DOCOMO |  | It can be discussed after discussion of Proposal 6-2. |
| ETRI |  | Since the sentence states that “The UE starts monitoring ~ after a number of slots”, the current wording may also be okay. |
| Tejas | Support |  |
| Ericsson |  | Propose the rewording below: --------------- TP of TS 38.213 in Clause 23 ----------------------- The UE starts monitoring monitors PDCCH occasions to detect the DCI format 1_0 with CRC scrambled by the SI-RNTI after a number of slots provided by od-sib1-windowStartOffset from the starting slot of the window controlled by ra_ResponseWindow, and for a number of slots provided by od-sib1-WindowDuration. --------------- end of TP ------------------------------------------------------- |
| Ofinno | Support |  |

- FL Proposal 6-5-1 (Ericsson, QC)

RAN1 to adopt TP below.

--------------- TP of TS 38.213 in Clause 23 -----------------------

If the UE identifies a RAPID associated with a corresponding PRACH transmission from the UE in a PDSCH reception scheduled by the DCI format 1_0 with CRC scrambled by the RA-RNTI, the UE can be indicated by higher layers to monitor PDCCH on the second cell to detect a DCI format 1_0 with CRC scrambled by the SI-RNTI according to a Type0-PDCCH CSS set provided by SearchSpaceZero. If the UE is provided XYZ, the UE monitors PDCCH only in monitoring occasions associated with the SS/PBCH block. The UE starts monitoring monitors PDCCH to detect the DCI format 1_0 with CRC scrambled by the SI-RNTI after a number of slots provided by od-sib1-windowStartOffset from the starting slot of the window controlled by ra_ResponseWindow, and for a number of slots provided by od-sib1-WindowDuration.

--------------- end of TP -------------------------------------------------------

Reason of change: Considering the RAR processing time and OD-SIB1 window starting time may fall into the middle of SSB burst and/or SIB1 transmission time corresponding to one SSB burst, when to start the PDCCH monitoring in the OD-SIB1 window is up to UE implementation.

| Company | Support or not | Comment |
| --- | --- | --- |

- FL Proposal 6-6 (OPPO)

Proposal 7: RAN1 to adopt TP#4 below.

--------------- TP#4 of TS 38.213 in Clause 23 -----------------------

The UE starts monitoring PDCCH to detect the DCI format 1_0 with CRC scrambled by the SI-RNTI after a number of slots provided by od-sib1-windowStartOffset from the starting slot of the window controlled by ra_ResponseWindow, and for a number of slots provided by od-sib1-WindowDuration.

[If the SS/PBCH block on the second cell indicates kSSB>23 for FR1 or kSSB>11 for FR2,] the UE is not required to monitor PDCCH on the second cell to detect the DCI format 1_0 with CRC scrambled by the SI-RNTI prior to a reception of a PDSCH scheduled by the DCI format 1_0 with CRC scrambled by the RA-RNTI.

--------------- end of TP#4 -------------------------------------------------------

Reason of change: This part is duplicated with the paragraph above it.

| Company | Support or not | Comment |
| --- | --- | --- |
| MTK | No | The deleted red part seems to reflect the following previous RAN1 agreement: - “After transmitting a UL-WUS, UE is not required to monitor Type0-PDCCH occasion (for OD-SIB1) before UE successfully received its RAR in response to the UL WUS.” |
| Google | No | We need to clarify the condition for not monitoring PDCCH. By default (without any text in spec), UE needs to monitor all SSs. |
| CMCC | No | Same view as MTK. |
| OPPO | Support | Please see our comment in FL proposal 2-2 |
| Apple | No |  |
| Samsung | Not support | Same view as MTK |
| Fujitsu | Not support | Same view as MTK. |
| Xiaomi | No | Same view as MTK |
| LG | Not support | Same view as MTK |
| DOCOMO | No | Same view as FL. |
| vivo | Not support |  |
| Tejas | No |  |
| Ofinno | Not support |  |
| Lenovo | No |  |
| OPPO2 |  | @Google, MTK: After UE sending WUS, when the UE starts to monitor type0-PDCCH is clearly defined by the following draft CR text. It says that after UE sending WUS, the UE needs to monitor RAR. If UE successfully receives RAR, UE can start to monitor type0-PDCCH. Therefore, it implies that before the UE receives RAR, the UE can only monitor RAR not type0-PDCCH. This is why we think the last paragraph is redundant. In response to the PRACH transmission, the UE monitors PDCCH on the second cell to detect a DCI format 1_0 with CRC scrambled by a corresponding RA-RNTI during a window controlled by ra_ResponseWindow, as described in Clause 8.2. The UE monitors PDCCH according to a Type1-PDCCH CSS set provided by ra-SearchSpace, if provided; else provided by SearchSpaceZero as described in Clause 10.1. If the UE identifies a RAPID associated with a corresponding PRACH transmission from the UE in a PDSCH reception scheduled by the DCI format 1_0 with CRC scrambled by the RA-RNTI, the UE can be indicated by higher layers to monitor PDCCH on the second cell to detect a DCI format 1_0 with CRC scrambled by the SI-RNTI according to a Type0-PDCCH CSS set provided by SearchSpaceZero |

- FL Proposal 6-7 (Tejas, Huawei/HiSilicon)

The reference point to determine the starting slot of on-demand SIB1 window is the start of the slot containing the starting symbol of the RAR window.

| Company | Support or not | Comment |
| --- | --- | --- |
| MTK | Yes | This was discussed in last RAN1 meeting. Fine to further do this clarification. |
| Google |  | OK |
| CMCC |  | Okay |
| ZTE, Sanechips |  | Follow legacy specification is preferred. |
| OPPO | Yes | This is a conflicting definition between legacy and draft CR. from the starting slot of the window controlled by ra_ResponseWindow, |
| Apple | Yes |  |
| Futurewei | OK |  |
| Fujitsu | OK |  |
| Sony | Yes |  |
| LG | Not support | We don’t see the necessity of this proposal. |
| DOCOMO |  | It seems the same meaning as the agreed CR. The modification is not necessary. |
| vivo | OK |  |
| Tejas | Support |  |
| Ofinno | Support |  |

- Issue 7: UE behaviors inside the OD-SIB1 time window

Background

In RAN1 #118b, it is agreed that:

Agreement

For the repetition periodicity of SIB1 within the time window of on-demand SIB1:

- Up to NW implementation (no change to existing specification)

Moderator Note (to Fujitsu Proposal 3 below): 38.331 Clause 5.2.1 has the following text on SIB1 repetition periodicity

- the SIB1 is transmitted on the DL-SCH with a periodicity of 160 ms and variable transmission repetition periodicity within 160 ms as specified in TS 38.213 [13], clause 13. The default transmission repetition periodicity of SIB1 is 20 ms but the actual transmission repetition periodicity is up to network implementation. For SSB and CORESET multiplexing pattern 1, SIB1 repetition transmission period is 20 ms. For SSB and CORESET multiplexing pattern 2/3, SIB1 transmission repetition period is the same as the SSB period (TS 38.213 [13], clause 13).

In RAN1 #120, it is agreed that:

Agreement

The reference time point to determine the window starting time for on-demand SIB1 is based on the starting slot of the RAR window for UL WUS wherein UE successfully received a RAR.

Agreement

The time offset between the reference time point and the starting time for the time window of on-demand SIB1 is indicated in the UL WUS configuration.

- Unit of the time offset is in slot

Figure. Reference time point and time offset to OD-SIB1 window (revised from [R1-2408856, Qualcomm])

Companies’ views for this topic in RAN1 #121 are collected below:

MTK:

Proposal 4: When OD-SIB1 window starting time falls into the middle of SSB burst, it is up to UE implementation to handle this scenario.

Figure from [R1-2401292, MTK]

Nokia

Observation-4: As specified in Clause 13 of TS38.213, legacy UE determines the slot to monitor Type0- PDCCH based on the SS/PBCH block index, since legacy UE obtains the Type0-PDCCH configurations from PBCH/MIB carried by SS/PBCH block.

Observation-5: For legacy operation, the time reference for UE monitoring of Type0-PDCCH is the SSB block slot.

Proposal-8: RAN1 to discuss how UE determines the slot number and symbols for OD-SIB1 monitoring occasions, i.e. the time resources (slots, symbols) for Type 0 PDCCH monitoring occasions within the SIB1 transmission window.

Proposal-9: RAN1 to clarify whether or not the index of starting slot n0 in TS38.213, for UE to monitor Type0-PDCCH of OD-SIB1 should incorporate with RAR reception slot index.

Proposal-10: RAN1 to clarify if UE should consider that the index of starting slot n0 in TS38.213, for UE to monitor Type0-PDCCH of OD-SIB1 is based on SS/PBCH block received after RAR reception.

ZTE, Sanechips

Proposal 3: Whether the OD-SIB1 can be updated within the time window should follow legacy spec. There is no need to restrict the payloads of the OD-SIB1 transmitted in the time window should be the same.

Proposal 4: The transmission of the updated SIB1 and the updated WUS configuration on the NES cell should be within a time duration.

- FL Proposal 7-1 (MTK)

When the OD-SIB1 window starting time falls into the middle of SSB burst and/or SIB1 transmission time corresponding to one SSB burst, it is up to UE implementation to monitor

- from the first Type 0 PDCCH monitoring occasion inside the OD-SIB1 window, or

- from the first Type 0 PDCCH monitoring occasion inside the OD-SIB1 window corresponding to the next SSB burst.

| Company | Support or not | Comment |
| --- | --- | --- |
| MTK | Yes | When one SSB burst consists of multiple beams, the SSB burst and SIB1 transmission time can last multiple slots. The OD-SIB1 window starting time may falls into the middle of SSB burst and/or SIB1 transmission time. Hence, the corresponding UE behavior may need to be clarified (see figures below). Figure from [R1-2401292, MTK] Figure from ref |
| Google | Support |  |
| CMCC | Support |  |
| ZTE, Sanechips | OK |  |
| OPPO |  | We don’t see the issue, the UE only follows the location of the search space no matter whether its relative location to SSB burst. |
| Futurewei |  | We do not think that a spec text is necessary |
| Samsung | Fine, but not essential | In our point of view, it is up to UE implementation. And also, it is not a critical issue. No need to further clarify the monitoring occasions. |
| Fujitsu | OK |  |
| Xiaomi | Support |  |
| LG |  | Agree with Samsung |
| vivo |  | Prefer no spec impact. |
| Tejas | Support |  |
| Huawei , HiSilicon |  | gNB implementation can solve this issue. And we agree with Samsung comment. |
| Ericsson |  | The first bullet is good, but the second bullet can cause strange behaviour when the 1-bit indicator that SIB1 PDCCH is provided in SSBs associated with UL WUS is used: If the starting time for the monitoring window comes after the first SSB index, but before the SSB index associated with UL WUS, then the UE should not wait until the next SSB burst. |
| Ofinno | Support |  |

- FL Proposal 7-2 (Nokia)

RAN1 to clarify:

- how UE determines the slot number and symbols for OD-SIB1 monitoring occasions, i.e. the time resources (slots, symbols) for Type 0 PDCCH monitoring occasions within the SIB1 transmission window

- whether or not the index of starting slot n0 in TS38.213, for UE to monitor Type0-PDCCH of OD-SIB1 should incorporate with RAR reception slot index

- if UE should consider that the index of starting slot n0 in TS38.213, for UE to monitor Type0-PDCCH of OD-SIB1 is based on SS/PBCH block received after RAR reception

| Company | Support or not | Comment |
| --- | --- | --- |
| MTK |  | We tend to think the points above are kind of clear from the figure below (revised from [R1-2408856, Qualcomm]) - UE determines the slot number and symbols for OD-SIB1 monitoring occasions from the searchSpaceZero and controlResourceSetZero configured in the UL WUS configuration. UE only monitors the slots that indicated by searchSpaceZero and inside the OD-SIB1 transmission window. - The starting slot for UE to monitor Type0-PDCCH of OD-SIB1 only depends on the starting slot of the RAR window. |
| Google |  | Agree with FL |
| OPPO |  | We fail to understand the issue from this proposal. More elaboration can be discussed. |
| Xiaomi |  | Agree with FL. |
| LG |  | Agree with oppo. |
| vivo |  | Agree with FL |
| Tejas |  | Open to discuss |

- FL Proposal 7-3 (ZTE)

Whether the OD-SIB1 can be updated within the time window should follow legacy spec, i.e. no restriction on the payloads of the transmitted OD-SIB1 being the same in the time window.

| Company | Support or not | Comment |
| --- | --- | --- |
| MTK | Yes | The OD-SIB1 contents periodicity should be 160ms as legacy. Hence, if the time window is longer than 160ms, the payloads of the transmitted OD-SIB1 can be different. Note: 38.331 Clause 5.2.1 has the following text on SIB1 repetition periodicity - the SIB1 is transmitted on the DL-SCH with a periodicity of 160 ms and variable transmission repetition periodicity within 160 ms as specified in TS 38.213 [13], clause 13. The default transmission repetition periodicity of SIB1 is 20 ms but the actual transmission repetition periodicity is up to network implementation. For SSB and CORESET multiplexing pattern 1, SIB1 repetition transmission period is 20 ms. For SSB and CORESET multiplexing pattern 2/3, SIB1 transmission repetition period is the same as the SSB period (TS 38.213 [13], clause 13). |
| Google | Support |  |
| Futurewei | ok |  |
| Fujitsu | Support |  |
| Xiaomi | Support. |  |
| LG |  | This topic seems to be overlapped with Proposal 3-3 |
| DOCOMO | Yes | Same understanding considering following agreements. Agreement For The repetition periodicity of SIB1 within the time window of on-demand SIB1: - Up to NW implementation (no change to existing specification) Agreement If a UE has SIB1 request configuration of a cell and before transmitting UL WUS, - If the UE detects a SSB where K_SSB>=24 for FR1 or K_SSB>=12 for FR2, select the following: Alt. 3: It is up to UE implementation on whether to monitor Type 0 PDCCH for SIB1 transmission |
| vivo | Support and without spec impact |  |
| Ericsson |  | System information update procedure is a RAN2 question. |
| Ofinno | Support |  |

- FL Proposal 7-4 (ZTE)

Proposal 4: The transmission of the updated SIB1 and the updated WUS configuration on the NES cell should be within a time duration.

| Company | Support or not | Comment |
| --- | --- | --- |
| MTK |  | This seems to be a RAN2 topic. |
| Google |  | We also think this is a RAN1 issue |
| ZTE, Sanechips | Support | As shown in the figure, the OB-SIB1/SIBx transmission behavior due to the SI change is not clear. In order to obtain the NES gain, the OB-SIB1/SIBx transmission behavior due to the SI change should restricted. |
| Futurewei |  | OK to leave it RAN2 |
| Fujitsu |  | Same view as MTK that it is a RAN2 issue. |
| Sony |  | This issue is up to RAN2. |
| NEC |  | Left to RAN2/RAN3 discussion |
| LG |  | This issue is up to RAN2. |
| DOCOMO |  | We consider that something may be related to RAN1, e.g., whether WUS transmission is needed or not for SIB1/WUS configuration update. It is better to clarify “should be within a time duration”, is it means that sending a WUS signal do not needed to update the SIB1/WUS configuration? |
| ZTE, Sanechips 2 |  | Answer to DOCOMO: “should be within a time duration” means that the updated SIB1/WUS configurations can be transmitted within a time duration. For example, when the SI change is received, the updated SIB1/WUS configuration can be broadcasted from the start of the next modification period and only be transmitted within one modification period (the length of the duration can be discussed). |

- Issue 8: Whether other features such as RedCap and NR-U can be combined with OD-SIB1 (vivo)

Background

The following proposals are brought up in RAN1 #121:

FUTUREWEI

Proposal 3: In the absence of supporting performance evaluations for NR-U, and considering LBT complexity, we recommend excluding OD-SIB1 for NR-U in Rel-19.

Observation 1: The R19 OD-SIB1 feature requires changes of the legacy RedCap initial access.

Proposal 4: If RAN1 supports R19 OD-SIB1 feature for RedCap UEs, provide in UL WUS configuration information on RedCap barring access to the NES cell.

vivo

Observation 3: Both network energy and UE power can be saved if there is an advance indication that whether redcap feature is not supported or not.

Proposal 9: Add a parameter such as RedCapAllowed in WUS configuration to indicate whether NES cell supports RedCap UEs to request OD-SIB1.

Proposal 10: Add LBT type in WUS configuration to support NES cell in unlicensed band.

Ofinno

Proposal 4: Do not study whether the OD-SIB1 can be made applicable to RedCap features in Rel-19 NES WI.

- FL Proposal 8-1 (vivo, FUTURWEI)

Add a parameter such as RedCapAllowed in WUS configuration to indicate whether NES cell supports RedCap UEs to request OD-SIB1.

| Company | Support or not | Comment |
| --- | --- | --- |
| MTK | Support |  |
| Google |  | We would like to understand the necessity first |
| CMCC |  | Same view as Google |
| OPPO | No | We have a general question. If the WUS configuration is provided in the initial DL BWP of Cell A which RedCap UE cannot support, does it mean that the RedCap is not allowed already? |
| Apple | OK |  |
| Futurewei | OK | To Oppo, yes, I agree with you. The proposal is for the case when Cell A supports a RedCap. |
| Samsung | Support |  |
| Fujitsu |  | In our understanding, it belongs to cell access information and is currently under discussion in RAN2. It can be left to RAN2 |
| Xiaomi |  | With the understanding from OPPO, we doubt the necessity to have the proposal. gNB already has ways to preclude RedCap UE with configuring a large BWP that RedCap UE cannot support. |
| LG | Not support | It is out of scope. |
| vivo | Support | Agree with Futurewei on OPPO’s question. If Cell A supports Redcap and NES doesn’t support Redcap, Redcap UEs may send UL WUS to NES cells which is unnecessary so that NES cell can not sleep that results in NES gain reduction. |
| Ericsson | No | This seems not necessary, the UE will figure out if RedCap is allowed from SIB1. UL WUS configuration should provide the means to request SIB1, not replace SIB1. |
| Ofinno | Not support |  |
| OPPO2 |  | @Futurewei: If the Cell A wants to inform the RedCap UE that the WUS configuration is not allowed for RedCap UE, the Cell A can simply decide not to provide WUS configuration in the RedCap dedicated initial DL BWP. There is no need to introduce a new parameter to bar RedCap UE. This was my question. The new parameter seems not necessary, the Cell A can already achieve this by implementation. |

- FL Proposal 8-2 (vivo)

Add LBT type in WUS configuration to support NES cell in unlicensed band.

| Company | Support or not | Comment |
| --- | --- | --- |
| Google | No | We failed to see the necessity to further enhance NR-U |
| CMCC | No | Same view as Google |
| OPPO | No | We don’t think NRU case deserves to be included in this release |
| Futurewei | No |  |
| Xiaomi | No |  |
| NEC | No | The applicability of on-demand SIB1 to specific UE types like RedCap or deployment scenarios like NR-U requires careful consideration of the associated complexities and use cases. For NR-U, the implications of features like LBT on the on-demand SIB1 procedure would need thorough study which may not be possible in the time left. |
| LG | Not support | It is out of scope. |
| vivo | Yes | We think NR unlicensed cell is always a capacity cell which is better to be in NES state. |
| Ofinno | Not support | It is out of scope. |

- FL Proposal 8-3 (FUTUREWEI)

Exclude support of OD-SIB1 for NR-U in Rel-19.

| Company | Support or not | Comment |
| --- | --- | --- |
| MTK | Support |  |
| Google | No | We failed to see the necessity to further enhance NR-U |
| Sharp | Support |  |
| CMCC | Support |  |
| OPPO | Support |  |
| Apple | Support |  |
| Futurewei | Support |  |
| Fujitsu | Support |  |
| Xiaomi | Support |  |
| NEC | Support |  |
| LG | Support |  |
| Ofinno | Support |  |

- Issue 9: RO selection based on rsrp-ThresholdSSB

Background

The following proposals are brought up in RAN1 #121:

DCM:

Proposal 2:

- It is up to RAN2 to decide the SSB and PRACH resource selection criterion for UE sending a WUS.

- FL Proposal 9-1 (DOCOMO)

It is up to RAN2 to decide the SSB and PRACH resource selection criterion for UE sending a WUS.

| Company | Support or not | Comment |
| --- | --- | --- |
| MTK | Support |  |
| Google |  | OK |
| Sharp | Support |  |
| CMCC |  | Okay |
| OPPO | Support |  |
| Apple | Support |  |
| Futurewei | Support |  |
| Samsung | Support |  |
| Fujitsu | Support |  |
| Sony | Support |  |
| Xiaomi | Support. |  |
| NEC | Support |  |
| LG | Support |  |
| DOCOMO | Support |  |
| vivo | Support |  |
| Ofinno | Support |  |

- Issue 10: Whether/how to indicate the OD-SIB1 transmission status to UE

Background

In RAN2 #127b, the following is agreed

Agreement

A cell for which SIB1 request configuration is available, can periodically broadcast SIB1.

Agreement

If UE has SIB1 request configuration of a cell, UE needs to check if SIB1 is currently being broadcasted or provided on demand for that cell before requesting SIB1 of that cell.

In RAN1 #120, it was agreed that:

Agreement

If a UE has SIB1 request configuration of a cell and before transmitting UL WUS,

- If the UE detects a SSB where K_SSB>=24 for FR1 or K_SSB>=13 for FR2, down select from the following:

  - Alt. 1: Only if K_SSB=30 for FR1 and K_SSB=14 for FR2, UE monitors Type 0 PDCCH for SIB1 transmission; Otherwise, UE doesn’t monitor Type 0 PDCCH

    - FFS: Whether monitoring time window of Type 0 PDCCH is up to UE implementation or specified.

  - Alt. 2: UE monitors Type 0 PDCCH for SIB1 transmission

    - FFS: Whether monitoring time window of Type 0 PDCCH is up to UE implementation or specified.

  - Alt. 3: It is up to UE implementation on whether to monitor Type 0 PDCCH for SIB1 transmission

  - Alt. 4: UE doesn’t monitor Type 0 PDCCH for SIB1 transmission.

  - Alt. 5: UE monitors Type 0 PDCCH for SIB1 based on some repurposed parameters in MIB or PBCH

Note: If the UE detects a SSB where K_SSB<=23 for FR1 or K_SSB<=12 for FR2, UE monitors Type 0 PDCCH for SIB1 transmission as legacy.

Note: From Ericsson point view, Alt 3 and Alt 4 may be inconsistent with existing RAN2 agreements

Note: The above cases are for SSB on the sync raster.

In RAN1 #120b, it was agreed that:

Agreement

If a UE has SIB1 request configuration of a cell and before transmitting UL WUS,

- If the UE detects a SSB where K_SSB>=24 for FR1 or K_SSB>=123 for FR2, select the following:

  - Alt. 3: It is up to UE implementation on whether to monitor Type 0 PDCCH for SIB1 transmission

Companies’ views for this topic in RAN1 #121 are collected below:

Sony:

Proposal 1: The information indicating the transmission status of SIB1 (e.g., whether the SIB1 of the NES cell is currently being broadcasted or provided on demand) should be included in UL WUS configuration.

CEWiT:

Proposal 3: Support indicating the transmission status of on-demand SIB1 using repurposed parameters in MIB or PBCH of NES cell for idle UEs.

Proposal 4: RAN1 to further study how UE determines the following states for the NES cell.

- State B: SIB1 is currently being periodically broadcasted (like legacy UE)

- State C1: SIB1 is provided on demand within a time window (no SIB1 transmission and UE has to sent SIB1 request)

vivo

Proposal 8: UE monitors type 0 PDCCH for SIB1 acquisition using the kssb and pdcch-ConfigSIB1 in the latest stored WUS configuration after receiving Short Message indicating SI change notification.

Samsung

Proposal 1: Make a conclusion in RAN1 that it’s up to gNB implementation to set the kSSB value to be <24 in

FR1 or <12 in FR2, such that legacy UE can camp on the cell when on-demand SIB1 is broadcasted.

CATT

Observation 1: If NES cell transmits SSB and the SSB is with 23< kSSB < kSSB

Proposal 1: For on-demand SIB1, the kSSB in PBCH payload of NES cell should support identifying the SIB1 transmission states, i.e. SIB1 is being broadcasted or SIB1 is not being broadcasted thus it can be requested by UL WUS.

Proposal 2: If NES cell transmits SSB and the kSSB = 30 for FR1 or kSSB = 14 for FR2, on demand SIB1 is currently being broadcasted.

Observation 2: There is a conflict between the agreements of RAN1 and RAN2. The agreement of RAN1 allows the UE implementation to determine whether to monitor Type 0 PDCCH for SIB1 transmission or not by implementation. In contrast, the agreement of RAN2 requires the UE to monitor Type 0 PDCCH before requesting SIB1.

Proposal 3: Update RAN1#120-b agreement to support RAN2 agreement.

Proposal 4: If a UE has SIB1 request configuration of a cell and before transmitting UL WUS,

- If the UE detects a SSB where K_SSB>=24 for FR1 or K_SSB>=13 for FR2, support Alt. 1:

  - Alt. 1: Only if K_SSB=30 for FR1 and K_SSB=14 for FR2, UE monitors Type 0 PDCCH for SIB1 transmission; Otherwise, UE doesn’t monitor Type 0 PDCCH

- FL Proposal 10-1 (Sony, CEWIT, CATT)

Support indicating the transmission status of on-demand SIB1 using repurposed parameters in MIB or PBCH or UL WUS configuration of NES cell.

| Company | Support or not | Comment |
| --- | --- | --- |
| MTK | No | This topic has been discussed multiple RAN1 meetings and we already have an agreement below. We do not think reopening the discussion can help progress. RAN1 #120b Agreement If a UE has SIB1 request configuration of a cell and before transmitting UL WUS, - If the UE detects a SSB where K_SSB>=24 for FR1 or K_SSB>=123 for FR2, select the following: - Alt. 3: It is up to UE implementation on whether to monitor Type 0 PDCCH for SIB1 transmission |
| Google |  | Agree with FL |
| Sharp | No | Same view with MTK. |
| CMCC | No | Same view with MTK |
| Futurewei | No | Agree with FL |
| Samsung | Not support | Same view as MTK |
| Fujitsu | Not support | Same view as MTK |
| Sony |  | We have a question that how to deal with the inconsistency between RAN1 (up to UE implementation) and RAN2 agreement (UE needs to check if SIB1 is currently being broadcasted or provided on demand for that cell before requesting SIB1 of that cell)? |
| Xiaomi |  | Agree with FL. |
| NEC | No |  |
| LG |  | We support indicating the transmission status of on-demand SIB1 using UL WUS configuration. |
| DOCOMO | No |  |
| Ericsson | Not support |  |
| Ofinno | No | Agree with MTK |

- FL Proposal 10-2 (vivo)

UE monitors type 0 PDCCH for SIB1 acquisition using the kssb and pdcch-ConfigSIB1 in the latest stored WUS configuration after receiving Short Message indicating SI change notification.

| Company | Support or not | Comment |
| --- | --- | --- |
| MTK |  | Seems like a RAN2 topic. |
| Google |  | This is a RAN2 issue |
| OPPO | Open to discuss | This is RAN1 issue as this is related to type0-PDCCH monitoring assumption. |
| Futurewei |  | Agree with FL, is a RAN2 issue |
| Xiaomi |  | Agree with FL. |
| NEC |  | Up to RAN2 decision |
| LG |  | Up to RAN2 decision |
| DOCOMO | Support | It may be related to Proposal 7-4. We consider that something may be related to RAN1, e.g., whether WUS transmission is needed or not for SIB1/WUS configuration update. The approach of vivo proposed is one of the solution. |
| vivo |  | This is the Type 0 PDCCH monitoring behavior after receiving paging short message indicating SIB1 change. It can be discussed in RAN1. |

- Issue 11: Relation between OD-SIB1 procedure and existing on-demand SI procedure

Background

The following proposals are brought up in RAN1 #121:

LG:

Proposal #5: Discuss how to guarantee on-demand SIB1 transmission while the UE receives SIBx (other than SIB1) or performs the existing on-demand SI procedure.

- FL Proposal 11-1 (LG)

Discuss how to guarantee on-demand SIB1 transmission while the UE receives SIBx (other than SIB1) or performs the existing on-demand SI procedure.

| Company | Support or not | Comment |
| --- | --- | --- |
| MTK | No | This has been discussed in last meeting and we do not think it is good to discuss further optimization between OD-SIB1 and legacy on-demand SI procedure in the last RAN1 meeting. |
| Google | No | Agree with FL |
| CMCC | No | It is out of scope. |
| OPPO |  | I think due to the limited time, LG may provide directly open issues, if they have identified. To us, the UE may send WUS for requesting OD-SIB1 when the UE finds necessary. Is there any issue? |
| Futurewei | No | Agree with FL |
| Samsung | Not support | No further optimization is needed for OD-SIB1. Below RAN2 agreement seems enough. RAN2 agreement from RAN2#129bis: 9. If UE has not received the PDCCH scheduling SIB1 upon the expiry of the SIB1 monitoring window, UE may consider the cell as being barred. |
| Fujitsu | Not support | Same view as MTK. |
| Xiaomi | No | Agree with FL. |
| NEC | No |  |
| LG |  | Given the majority views, we are OK with no further optimization. |
| DOCOMO | No |  |
| vivo | No |  |
| Ofinno | No | Agree with MTK |

- Issue 12: MAC subPDU location identification for RAR of OD-SIB1 request

Background

The following proposals are brought up in RAN1 #121:

Sharp:

Proposal 1: To solve the issue on MAC subPDU location identification for RAR of OD-SIB1 request,

RAN1 should consider following options:

- Option 1: RACH occasion for OD-SIB1 request is a dedicated resource and is not used for other purpose

- Option 2: New RNTI is introduced for RAR of OD-SIB1 request

- Option 3: OD-SIB1 request configuration includes configuration of OD-OSI request (RAPID

information)

- FL Proposal 12-1 (Sharp)

RAN1 should consider following options to solve the issue on MAC subPDU location identification for RAR of OD-SIB1 request

- Option 1: RACH occasion for OD-SIB1 request is a dedicated resource and is not used for other purpose

- Option 2: New RNTI is introduced for RAR of OD-SIB1 request

- Option 3: OD-SIB1 request configuration includes configuration of OD-OSI request (RAPID

information)

| Company | Support or not | Comment |
| --- | --- | --- |
| MTK |  | Seems like a RAN2 topic. |
| Google |  | This is clearly a RAN2 issue |
| Sharp | Support | Although we are fine that the details would be determined in RAN2, we would like to know the companies’ view whether RO for OD-SIB1 request can be shared with other usage (e.g. PDCCH order). If it can be shared, this issue occurs, and some solutions may impact on the contents on OD-SIB1 request configuration or RNTI for OD-SIB1 RAR. |
| CMCC |  | Can be up to RAN2. |
| OPPO |  | Agree that this should not be discussed in RAN1 |
| Futurewei |  | Agree with FL |
| Sharp2 |  | By the offline comment from other company, we understand this issue has been solved by the RAN2 in last April meeting even if RO for OD-SIB1 request is shared with other usage. We don’t see the issue on it anymore, then we would like to withdraw this proposal. We apologize for taking your time. |
| Xiaomi |  | Agree with FL. |
| LG |  | Agree with MTK |
| Ofinno | Do not support | It is RAN2 issue. |

- Issue 13: PRACH transmission for SIB1 request

Background

The following proposals are brought up in RAN1 #121:

Google:

Proposal 1: Endorse the following TP for 38.211 to clarify the PRACH transmission for SIB1 request

--------------------------------- start of TP for 38.211-------------------------------------

5.3.2	OFDM baseband signal generation for PRACH

<omitted text>

-	Δf is the subcarrier spacing of the initial uplink bandwidth part during initial access. If the PRACH transmission is for a candidate cell Δf is provided by ltm-PRACH-SubcarrierSpacing in EarlyUL-SyncConfig. If the PRACH transmission is for SIB1 request, Δf is provided by SIB1-RequestConfig. Otherwise,  is the subcarrier spacing of the active uplink bandwidth part;

-	μ0 is the largest μ value among the subcarrier spacing configurations by the higher-layer parameter scs-SpecificCarrierList;

-	NBWP,istart is the lowest numbered resource block of the initial uplink bandwidth part and is derived by the higher-layer parameter initialUplinkBWP or initialUplinkBWP-RedCap during initial access and from the higher-layer parameters bwp-GenericParameters in EarlyUL-SyncConfig if the PRACH transmission is for a candidate cell and from the higher-layer parameters SIB1-RequestConfig if the PRACH transmission is for SIB1 request. Otherwise, NBWP,istart is the lowest numbered resource block of the active uplink bandwidth part and is derived by the higher-layer parameter BWP-Uplink;

<omitted text>

6.3.3.1	Sequence generation

<omitted text>

There are 64 preambles defined in each time-frequency PRACH occasion, enumerated in increasing order of first increasing cyclic shift  of a logical root sequence, and then in increasing order of the logical root sequence index, starting with the index obtained from the higher-layer parameter prach-RootSequenceIndex or rootSequenceIndex-BFR or by msgA-PRACH-RootSequenceIndex if configured and a type-2 random-access procedure is initiated as described in clause 8.1 of [5, TS 38.213] or by prach-RootSequenceIndex in EarlyUL-SyncConfig if the PRACH transmission is for a candidate cell or by prach-RootSequenceIndex in SIB1-RequestConfig if the PRACH transmission is for SIB1 request. Additional preamble sequences, in case 64 preambles cannot be generated from a single root Zadoff-Chu sequence, are obtained from the root sequences with the consecutive logical indexes until all the 64 sequences are found. The logical root sequence order is cyclic; the logical index 0 is consecutive to LRA-2. The sequence number  is obtained from the logical root sequence index according to Tables 6.3.3.1-3 to 6.3.3.1-4B.

--------------------------------- start of TP for 38.211-------------------------------------

- FL Proposal 13-1 (Google)

Adopt the following TP for 38.211 to clarify the PRACH transmission for SIB1 request.

--------------------------------- start of TP for 38.211-------------------------------------

5.3.2	OFDM baseband signal generation for PRACH

<omitted text>

-	Δf is the subcarrier spacing of the initial uplink bandwidth part during initial access. If the PRACH transmission is for a candidate cell Δf is provided by ltm-PRACH-SubcarrierSpacing in EarlyUL-SyncConfig. If the PRACH transmission is for SIB1 request, Δf is provided by SIB1-RequestConfig. Otherwise,  is the subcarrier spacing of the active uplink bandwidth part;

-	μ0 is the largest μ value among the subcarrier spacing configurations by the higher-layer parameter scs-SpecificCarrierList;

-	NBWP,istart is the lowest numbered resource block of the initial uplink bandwidth part and is derived by the higher-layer parameter initialUplinkBWP or initialUplinkBWP-RedCap during initial access and from the higher-layer parameters bwp-GenericParameters in EarlyUL-SyncConfig if the PRACH transmission is for a candidate cell and from the higher-layer parameters SIB1-RequestConfig if the PRACH transmission is for SIB1 request. Otherwise, NBWP,istart is the lowest numbered resource block of the active uplink bandwidth part and is derived by the higher-layer parameter BWP-Uplink;

<omitted text>

6.3.3.1	Sequence generation

<omitted text>

There are 64 preambles defined in each time-frequency PRACH occasion, enumerated in increasing order of first increasing cyclic shift  of a logical root sequence, and then in increasing order of the logical root sequence index, starting with the index obtained from the higher-layer parameter prach-RootSequenceIndex or rootSequenceIndex-BFR or by msgA-PRACH-RootSequenceIndex if configured and a type-2 random-access procedure is initiated as described in clause 8.1 of [5, TS 38.213] or by prach-RootSequenceIndex in EarlyUL-SyncConfig if the PRACH transmission is for a candidate cell or by prach-RootSequenceIndex in SIB1-RequestConfig if the PRACH transmission is for SIB1 request. Additional preamble sequences, in case 64 preambles cannot be generated from a single root Zadoff-Chu sequence, are obtained from the root sequences with the consecutive logical indexes until all the 64 sequences are found. The logical root sequence order is cyclic; the logical index 0 is consecutive to LRA-2. The sequence number  is obtained from the logical root sequence index according to Tables 6.3.3.1-3 to 6.3.3.1-4B.

--------------------------------- end of TP for 38.211-------------------------------------

| Company | Support or not | Comment |
| --- | --- | --- |
| Google | Support | This is to implement RAN1’s agreement |
| OPPO |  | We understand the intention. But according to editor’s comment, all the parameters provided by SIB1-RequestConfig should be captured in Clause 23. So we may make editor aware of this issue and leave editor find a way to capture this in Clause 23. |
| Apple | Support |  |
| LG |  | Agree with OPPO |
| vivo |  | Agree with OPPO |

- Issue 14: Other topics

Discussion place holder for issues not included above.

- FL Proposal 14-1

Companies are invited to bring up other proposals which (you think) should be treated in this meeting. Moderator would try to devise further discussion based on received comments.

- Proposal receiving support/attention from multiple companies would be visited first.

| Company | Comment |
| --- | --- |

Resulted RAN1 conclusion/agreement

The following is achieved in this meeting:

Conclusion

On whether/how to indicate additional information related to SSBs associated with PDCCH monitoring occasions, no additional information is provided

Conclusion

For the purpose of finalizing Rel-19 NES, RAN1 assumes the following UE behavior for OD-SIB1.

- The SIB1 is transmitted on the DL-SCH with a periodicity of 160 ms and variable transmission repetition periodicity within 160 ms.

Agreement

On the value range for od-sib1-duration, adopt the following:

- {20, 40, 80, 160, 320} ms

Agreement

For UE to obtain the UL point A for TDD system, configure offsetToPointA in the UL WUS configuration for TDD system.

- No RAN1 specification impact is expected

Conclusion

There is no consensus on the support of OD-SIB1 for NR-U in Rel-19.

Agreement

RAN1 to adopt TP below.

--------------- TP of TS 38.213 in Clause 23 -----------------------

If the UE identifies a RAPID associated with a corresponding PRACH transmission from the UE in a PDSCH reception scheduled by the DCI format 1_0 with CRC scrambled by the RA-RNTI, the UE can be indicated by higher layers to monitor PDCCH on the second cell to detect a DCI format 1_0 with CRC scrambled by the SI-RNTI according to a Type0-PDCCH CSS set provided by SearchSpaceZero. If the UE is provided XYZ, the UE monitors PDCCH only in monitoring occasions associated with the SS/PBCH block. The UE starts monitoring monitors PDCCH to detect the DCI format 1_0 with CRC scrambled by the SI-RNTI after a number of slots provided by od-sib1-windowStartOffset from the starting slot of the window controlled by ra_ResponseWindow, and for a number of slots provided by od-sib1-WindowDuration.

--------------- end of TP -------------------------------------------------------

Reason of change: Considering the RAR processing time and OD-SIB1 window starting time may fall into the middle of SSB burst and/or SIB1 transmission time corresponding to one SSB burst, when to start the PDCCH monitoring in the OD-SIB1 window is up to UE implementation.

Agreement

The parameters ‘absoluteFrequencyPointA’, ‘offsetToCarrier’ and ‘locationAndBandwidth’ are mandatorily present in the UL-WUS configuration for both FDD and TDD system.

Agreement

The frequencyBandList is mandatorily present in WUS configuration for TDD system, which refers to the IE within FrequencyInfoDL-SIB.

Agreement

The reference point to determine the starting slot of on-demand SIB1 window is the start of the slot containing the starting symbol of the RAR window.

Agreement

RAN1 adopts TP below for clause 23 of TS 38.213.

23 UE procedure to request SIB1 reception -----------------------omitted text----------------------- [If the SS/PBCH block on the second cell indicates kSSB>23 for FR1 or kSSB>11 for FR2,] the UE is not required to monitor PDCCH on the second cell to detect the DCI format 1_0 with CRC scrambled by the SI-RNTI prior to a reception of a PDSCH carrying a RAPID associated with a corresponding PRACH transmission scheduled by the DCI format 1_0 with CRC scrambled by the RA-RNTI.

- 4 References (all from RAN1 #121)

- R1-2503234	Discussion of on-demand SIB1 for idle/inactive mode UEs	FUTUREWEI

- R1-2503268	Discussion on on-demand SIB1 for eNES	Huawei, HiSilicon

- R1-2503316	Discussion on on-demand SIB1 for NES	ZTE Corporation, Sanechips

- R1-2503366	Remaining issues on-demand SIB1 for idle/inactive mode UEs   vivo

- R1-2503413	On-demand SIB1 for Idle/Inactive mode UEs	Nokia, Nokia Shanghai Bell

- R1-2503417	Discussion on on-demand SIB1 for UEs in idle/inactive mode	NEC

- R1-2503520	Discussion on on-demand SIB1 for idle/inactive mode UEs	Spreadtrum, UNISOC

- R1-2503571	On-demand SIB1 for idle/inactive mode UEs	Samsung

- R1-2503717	OD-SIB1 for idle/inactive UEs	Tejas Network Limited

- R1-2503738	Discussion on on-demand SIB1 for idle/inactive mode UEs	Ofinno

- R1-2503798	Discussion on on-demand SIB1	CATT

- R1-2503836	Discussion on on-demand SIB1 for UEs in idle/inactive mode	CMCC

- R1-2503887	Discussion on on-demand SIB1 for idle/inactive mode UEs	Xiaomi

- R1-2503973	Discussion on on-demand SIB1 for idle/inactive mode UEs	InterDigital, Inc.

- R1-2503999	Discussion on on-demand SIB1 transmission for network energy savings	Fujitsu

- R1-2504016	On-demand SIB1 for UEs in idle/inactive mode for NES	Ericsson

- R1-2504031	On-demand SIB1 for Idle/Inactive Mode UE	 Google

- R1-2504051	Discussion on on-demand SIB1 for idle/inactive mode UEs	China Telecom

- R1-2504065	On-demand SIB1 for idle/inactive mode UEs	Sony

- R1-2504108	On-demand SIB1 for idle/inactive mode UEs	Lenovo

- R1-2504142	On-demand SIB1 for idle/inactive mode UEs	ETRI

- R1-2504177	Discussion on on-demand SIB1 transmission for idle/inactive mode UEs Transsion Holdings

- R1-2504193	Discussion on the enhancement to support on demand SIB1 for idle/inactive mode UE OPPO

- R1-2504236	Discussion on on-demand SIB1 for idle/inactive mode UEs	Panasonic

- R1-2504248	On-demand SIB1 for idle/inactive mode UEs	LG Electronics

- R1-2504263	On-demand SIB1 for idle or inactive mode UEs	MediaTek Inc.

- R1-2504326	On On-demand SIB1 for IDLE/INACTIVE mode UEs	Apple

- R1-2504399	On-demand SIB1 procedure	Qualcomm Incorporated

- R1-2504457	Discussion on on-demand SIB1 for idle/inactive mode UEs	DENSO CORPORATION

- R1-2504467	Discussion on on-demand SIB1 transmission for idle UEs	Sharp

- R1-2504506	Discussion on on-demand SIB1 for idle/inactive mode UEs	NTT DOCOMO, INC.

- R1-2504556	Discussion on on-demand SIB1 in idle/inactive mode	CAICT

- R1-2504603	Discussion on  on-demand SIB1	CEWiT

- R1-2501811	Discussions on on-demand SIB1 for idle/inactive mode UEs	vivo (RAN1 #120b)