3GPP TSG RAN WG1 #121							R1-2504729

St Julian, Malta, May 19th – 23rd, 2025

Agenda item:	9.2.2

Source: 	Moderator (Samsung)

Title: 	Moderator Summary for Monday Offline Session on Rel-19 CSI enhancements

Document for:	Discussion and Decision

## Introduction

The following proposals were discussed.

## Summary of proposals

### Issue 1 (WID objective 2a and 2b): Type-I and Type-II codebook refinement for up to 128 CSI-RS ports

| 1.2.1 | [120bis] Agreement For the Rel-19 Type-I and Type-II (except for Type-II Doppler) codebook refinement for 48 , 64, and 128 CSI-RS ports, when the K>1 aggregated NZP aperiodic CSI-RS resources for CMR are within two consecutive slots, the triggering offset of the associated aperiodic CSI-IM follows the associated NZP CSI-RS resource(s) for CMR in the first slot. - FFS: How to resolve this for Rel-19 Type-II Doppler Conclusion 1.B.1: For the Rel-19 Type-I and Type-II (except for Type-II Doppler) codebook refinement for 48, 64, and 128 CSI-RS ports, when the K>1 aggregated NZP aperiodic CSI-RS resources for CMR are within two consecutive slots, following the legacy spec, the aperiodic triggering offset configured with the NZP CSI-RS resource set for interference measurement is the same as that of the associated NZP CSI-RS resource set for CMR. FL assessment: This proposal is technically sound and a simple extension of the previous agreement for AP CSI-IM to NZP CSI-RS for IM except for Type-II Doppler. However, Nokia argues that this can be handled via NW implementation since NZP CSI-RS for IM (unlike CSI-IM) has its own triggering offset. Nokia’s argument is valid. | Support/fine: Samsung, Google, NTT DOCOMO, MediaTek, Spreadtrum, HONOR, CATT, Apple, NEC, Qualcomm, vivo, Sharp, Huawei/HiSi (open), Rakuten, Fraunhofer IIS/HHI, Tejas, Lenovo/MotM, Not support (NW implementation): Nokia, OPPO, ZTE, CMCC, ETRI, Fujitsu, Ericsson, Sony, |
| --- | --- | --- |

Arguments from opponents: Unlike CSI-IM, NZP CSI-RS for IM includes its own triggering offset. So the NW configure this offset to align with the associated CMR in the first slot.

### Issue 2 (WID objective 2c): CRI-based CSI for hybrid beamforming (HBF)

| 2.2 | Proposal 2.B: For the Rel-19 CRI-based CSI refinement for up to 128 CSI-RS ports, regarding packing order of NRep CSI reports, - For reports with non-M CRI based CSI reports (Legacy CSI reports till Rel-18) should have higher priority over M CRIs based CSI reports. - For reports configured with MR resources should have higher priority over reports with non-MR based CSI reports. - For reports with lower M values should have higher priority over higher M CRIs based CSI reports. PriiCSIy,k,c,s,m,M, MR=2∙Ncells∙Ms∙Mm∙y+Ncells∙Ms∙Mm∙k+Ms∙Mm∙c+s+Ms∙(M-MR)∙m where - m = 0 for non-M CRI based CSI reports. Mm= 1 (legacy CSI reports up to Rel-18), - m = 1 for M CRI based CSI reports. Mm is the maximum number of CRIs configured for multi-CRI CSI reports not carrying L1-RSRP or L1-SINR Note: It is up to the editor how this is captured in the specification FL assessment: This proposal is from RAN1#120 round-1. But since the main proponent was not present in Wuhan due to visa issues, the final decision for this proposal is deferred to RAN1#121 Malta. This proposal seems aligned with the legacy principle and is technically sound. | Support/fine: Tejas, Samsung, Huawei/HiSi (open), MediaTek (open), Spreadtrum (open), Apple (open), IDC, CMCC, Not support: Google, Apple, ZTE, ETRI, Fujitsu, Lenovo/MotM, OPPO, Ericsson (concern), Nokia/NSB (concern), NTT DOCOMO, NTT CORP, TCL, HONOR, NEC, |
| --- | --- | --- |

Arguments from opponents: This can be achieved based on the current priority rules + NW implementation

### Issue 3 (WID objective 3): CJT calibration reporting for non-ideal synchronization and backhaul

--

# References