3GPP TSG-RAN WG1 #121	R1-2504803

St Julian's, Malta, 19th -23rd May, 2025

Agenda Item:	5

Source:	MediaTek Inc., Qualcomm

Title:	Discussion on reply LS on CB-msg3-EDT

Document for:	Discussion & Decision

# 1. Introduction

In RAN#107, a revised WID on IoT NTN enhancements was endorsed for Release 19 with an objective on enhancements to reduce the necessary uplink and downlink signaling to complete an EDT transaction as shown below [1].  This feature is now referred to as CB-Msg3-EDT procedure (tentatively) in RAN2.

- Study and specify, if beneficial the following enhancements to reduce the necessary uplink and downlink signaling to complete an EDT transaction [RAN2]:

  - Msg3 transmission without msg1/RAR

  - Efficient delivery (reduced overhead) of msg4 / RRCEarlyDataComplete

  - Study and specify RRM requirement, if identified [RAN4]

RAN2 has sent an LS to RAN1on L1 parameters for on CB-msg3-EDT [2]. RAN2 has agreed to introduce the shared resource configuration in system information for CB-Msg3-EDT procedure. This configuration is used for the UE to transmit Msg3 on a PUSCH channel in a contention-based manner and to receive Msg4. RAN2 assumes that the configuration introduced for PUR could be reused as much as possible in the shared resource configuration for both eMTC and NB-IoT. RAN2 also assumes that the shared resource configuration is per CE level. Since the configuration is mainly L1 parameters, RAN2 kindly request RAN1 help to finalize the signaling design.

This contribution summarizes companies views and proposed replies to  RAN2 questions Q1, Q2, Q3, Q4, Q5, Q6, Q7 and Q8 to RAN1 which are related to eMTC NTN and NB-IoT NTN.

# 2. Question Q1

Q1: For both eMTC and NB-IoT, RAN2 assumes power ramping should be supported for CB-msg3-EDT. RAN2 would like to confirm with RAN1 on whether to have power ramping on this Msg3 (re-)transmission. If RAN1 confirms, please provide the control parameters for power ramping.

RAN2 asked question Q1 for both eMTC and NB-IoT on power ramping should be supported for CB-msg3-EDT:

Power ramping for CB-msg3-EDT is preferable as msg1 preamble in RACH procedure is skipped. Always transmitting msg3 at maximum power increases device power consumption and results in a higher level of interference.

- 7 companies indicated support

- 2 companies indicated not support

Moderator view:

- In case power ramping is not supported, Open Loop based on Path Loss can be used similarly to Rel-16 PUR, per CE level. The UL power control parameters p0-UE-NPUSCH-r16 and alpha-r16 for NB-IoT NTN are not needed.

- In case power ramping is supported, RAN1 need to align understanding on these parameters.   Alpha-r16 is a HL parameter in PUR-Config-NB provided by higher layers for serving cell c and corresponds to αc(j) with j=3. Likewise, p0-UE-NPUSCH-r16 is a parameter provided by higher layers for serving cell c and corresponds to P0_UE_NPUSCH,c(j) with j= 3. These parameters are specified in in TS 36.213, Clause 16.2.1.1.1 for UE Transmit power for a Narrowband Physical Uplink Shared Channel (NPUSCH) transmission. Whether to use the specified values or new values should be discussed..

More consensus is needed in RAN1 on whether to support power ramping for CB-msg3-EDT in eMTC NTN and NB-IoT NTN. One way to proceed would be to capture impact on RAN1 specifications to power ramping and reply to RAN2 accordingly.

Proposal reply to Q1: For open loop power control the following the following UL power control parameters can be reused for CB-msg3-EDT

- p0-UE-NPUSCH-r16 and alpha-r16 for NB-IoT NTN

- p0-UE-PUSCH-r16 and alpha-r16 for eMTC NTN

If RAN2 decides to introduce power ramping, RAN1 recommends to reuse the same power ramping stepsize as for (N)PRACH (0, 2, 4 and 6dB)

Companies are encouraged to provides comments on RAN1 reply to Q1 below

| Companies | Comments |
| --- | --- |
| Ericsson | We are not ok with the sentence at the bottom since there is no consensus to recommend power ramping nor any component around it. We can state that: “If RAN2 decides to introduce power ramping, then the “powerRampingStep” parameter would need to be included in the pre-configuration, and RAN1 would have to reflect this into the technical specifications”. In our view, it is sufficient to rely on the initial transmit power formula using the control parameters already present in “PUR-Config”/ “PUR-Config-NB”. |
| Xiaomi | According to the Q1, maybe it should also be discussed in RAN1 whether to support power ramping for CB-Msg3 EDT. Regarding the power control parameters, we are fine with these parameters mentioned in FL’s proposal reply to Q1. |
| Vivo1 | Our understanding of the situation is that RAN2 LS was intended to ask RAN1 to make the decision on whether power ramping is supported or not. Since power ramping is not supported for PUR and RAN1 has not evaluated its performance, if RAN1 decides to let RAN2 to make the final decision, we suggest that the LS reply reflects the RAN1 situation. For example: “RAN1 specification does not support power ramping for CB-msg3-EDT. RAN1 has not yet evaluated the potential performance of power ramping for CB-msg3-EDT, and it is likely that there will not be sufficient time to address this topic within the R19 timeframe” |
| CMCC2 | We are fine to reuse the open loop power control parameter for CB-msg3-EDT. - p0-UE-NPUSCH-r16 and alpha-r16 for NB-IoT NTN - p0-UE-PUSCH-r16 and alpha-r16 for eMTC NTN |
| NEC | Agree with VIVO on power ramping. |
| Apple | The proposed answer is ok. It’s up to RAN2 to decide whether to support power ramping |
| LGE | We are fine with the vivo’s proposal. Meanwhile, if RAN1 confirm the power ramping for CB-MSG3-EDT, on top of the power ramping parameter, we also need to reuse the initial target power for NPRACH. |
| Lenovo | We share the similar view as vivo that we are not ready to support the power ramping if no benefits are identified by the groups. |
| OPPO | It seems that we should firstly discuss whether to support power ramping for CB-Msg3 EDT transmission. Based on the majority views of companies collected by FL, we can reply to RAN2 that RAN1 confirms to have power ramping on Msg3 transmission (7/9). Regarding the parameters for power ramping, we support the proposal that the p0-UE-[N]PUSCH-r16 and alpha-r16 are reused to determine the MSG3_RECEIVED_TARGET_POWER, and the power ramping step size for [N]PRACH can be reused. |
| Huawei, HiSilicon | We do not think the power ramping on msg3 is necessary |
| Nokia, NSB | We think it is similar as power ramping for PRACH as both are contention based. RAN2 assume power ramping is used and RAN2 is just asking how to define the parameter for power ramping. Then power ramping should be between CB-Msg3 window for CB-Msg3 reattempt, while no CE level changing during the power ramping. For the second sentence, we propose to update as “If RAN2 decides to introduce power ramping, RAN1 recommends to reuse the same power ramping stepsize as for (N)PRACH (0, 2, 4 and 6dB) between CB-Msg3 windows.” |
| ZTE | For this aspect, there are at least following issues need to be decided: - General power control parameter: It’s more for the open-loop control, e.g., alpha. For this aspect, reusing the parameter for PUR can be a good starting point. Other details for the closed loop control can be discussed later, however, simplified approach, i.e., open-loop only is preferred since it’s still in idle state. - Whether to support the power ramping: In our view, it’s beneficial to enable this feature. The details to support power ramping, e.g., including the power ramping step size, mechanism, etc. Without confirmation of the 2nd issue, it’s hard to push any recommendation in the reply LS. |

## 2.1 Proposed reply to Q1

Proposal reply to Q1:

- RAN1 specification does not support power ramping for CB-msg3-EDT. RAN1 has not yet evaluated the potential performance of power ramping for CB-msg3-EDT, and it is likely that there will not be sufficient time to address this topic within the R19 timeframe

- For open loop power control the following the following UL power control parameters can be reused for CB-msg3-EDT

  - p0-UE-NPUSCH-r16 and alpha-r16 for NB-IoT NTN

  - p0-UE-PUSCH-r16 and alpha-r16 for eMTC NTN

- If RAN2 decides to introduce power ramping, RAN1 recommends to reuse the same power ramping stepsize as for (N)PRACH (0, 2, 4 and 6dB)

# 3. Question Q2

RAN2 asked question Q2 for eMTC on MPDCCH configuration for shared resource configuration for CB-msg3-EDT

Q2: For eMTC, RAN2 agrees to have MPDCCH configuration for shared resource configuration and decides to reuse the parameters from PUR MPDCCH configuration (in IE PUR-MPDCCH-Config-r16, as below) as baseline. RAN2 think TDD parameter is not needed and maybe narrowband configuration should be defined as a set, not one single narrow band. RAN2 would like to check with RAN1 on how to define the detail MPDCCH configuration.

PUR-MPDCCH-Config-r16 ::=		SEQUENCE {

mpdcch-FreqHopping-r16			BOOLEAN,

mpdcch-Narrowband-r16			INTEGER (1..maxAvailNarrowBands-r13),

mpdcch-PRB-PairsConfig-r16		SEQUENCE{

numberPRB-Pairs-r16				ENUMERATED {n2, n4, n6, spare1},

resourceBlockAssignment-r16		BIT STRING (SIZE(4))

},

mpdcch-NumRepetition-r16		ENUMERATED {r1, r2, r4, r8, r16, r32, r64, r128, r256},

mpdcch-StartSF-UESS-r16			CHOICE {

fdd								ENUMERATED {v1, v1dot5, v2, v2dot5, v4, v5, v8, v10},

tdd							ENUMERATED {v1, v2, v4, v5, v8, v10, v20, spare1}

},

mpdcch-Offset-PUR-SS-r16	ENUMERATED {zero, oneEighth, oneQuarter,

threeEighth, oneHalf, fiveEighth,

threeQuarter, sevenEighth}

}

The majority of the companies agree to support reusing the MPDCCH configuration with the following comments:

- The search space should be a CSS, not USS.

- The TDD parameters can be removed.

- From RAN1 perspective, there is no need to define multiple narrowbands for the response.

Proposal reply to Q2: From RAN1 perspective, the MPDCCH PUR configuration can be reused, with the following modifications:

- The TDD parameters are not needed.

- The search space will be a common search space (CSS) instead of UE-specific search space (USS).

- From RAN1 perspective there is no need to define the set of narrowbands as a set.

| Companies | Comments |
| --- | --- |
| Ericsson | We are ok with the “Proposal reply to Q2”. |
| Xiaomi | Agree with FL that the search space should be a CSS. |
| Vivo1 | agree |
| CMCC2 | Fine with the 1st two bullets. And open to discuss whether to define the set of narrowbands as a set. It seems that without extending the MPDCCH narrowbands, it is can work for MPDCCH. |
| Apple | OK with the proposed answer. |
| LGE | OK. |
| Lenovo | We are general fine with the response. It seems RAN2 asks RAN1 on how to define the detail MPDCCH configuration, it is straightforward to suggest configuring the MPDCCH configuration in broadcast channel instead of mentioning the search space is common search space (does RAN2 care about this?) |
| OPPO | Agree with FL and xiaomi that the search space should be a CSS instead of USS. In this case, the MPDCCH configuration for Type2-MPDCCH common search space should be taken as baseline: mpdcch-startSF-CSS-RA-r13 CHOICE { fdd-r13 ENUMERATED {v1, v1dot5, v2, v2dot5, v4, v5, v8, v10}, tdd-r13 ENUMERATED {v1, v2, v4, v5, v8, v10, v20, spare} } OPTIONAL, -- Cond MP mpdcch-NarrowbandsToMonitor-r13 SEQUENCE (SIZE(1..2)) OF INTEGER (1..maxAvailNarrowBands-r13), mpdcch-NumRepetition-RA-r13 ENUMERATED {r1, r2, r4, r8, r16, r32, r64, r128, r256}, |
| Huawei, HiSilicon | support |
| Nokia, NSB | We do not support the third bullet. For MPDCCH PUR, narrowband configuration is defined as a set. CB-Msg3 is reusing the PUR configuration and should have similar processing as RAN2 view in the LS, in which 2 narrowbands are used to distribute the MPDCCH which should be also benefit for more MPDCCH capacity. In this case, we propose to update the third bullet as - From RAN1 perspective, similar as PUR, there can is no need to define the set of narrowbands as a set, and UE determines which MPDCCH narrowband to monitor. For the second bullet, we think the CSS should be also as per CE level and it should be replied to RAN2, as update as following: - The search space will be a common search space (CSS) per CE level instead of UE-specific search space (USS). |
| ZTE | Fine with the proposal. |

## 3.1 Proposed reply to Q2

Proposal reply to Q2: From RAN1 perspective, the MPDCCH PUR configuration can be reused, with the following modifications:

- The TDD parameters are not needed.

- The search space will be a common search space (CSS) instead of UE-specific search space (USS).

- From RAN1 perspective there is no need to define the set of narrowbands as a set.

# 4. Question Q3

RAN2 asked question Q2 for eMTC on PUSCH configuration for shared resource configuration for CB-msg3-EDT

Q3: For eMTC, RAN2 agrees to have PUSCH configuration for shared resource configuration and decides to reuse the parameters from PUR PUSCH configuration (in IE PUR-PUSCH-Config-r16, as below) as baseline. RAN2 has some questions on some parameters:

- Whether pusch-CyclicShift-r16 and pusch-NB-MaxTBS-r16 are needed

- Whether prb-AllocationInfo should be defined as a “set” format with intention to provide a set of shared frequency-domain resources

Note that:

- CE mode B related parameters will not be included as RAN2 agreed to preclude CE mode B.

- The power related parameters (p0-UE-PUSCH-r16 and alpha-r16) should be updated based on the result of Q1.

RAN2 would like to check with RAN1 on how to define the detail PUSCH configuration.

PUR-PUSCH-Config-r16 ::=		SEQUENCE {

pur-GrantInfo-r16				CHOICE {

ce-ModeA						SEQUENCE {

numRUs-r16						BIT STRING (SIZE(2)),

prb-AllocationInfo-r16			BIT STRING (SIZE(10)),

mcs-r16							BIT STRING (SIZE(4)),

numRepetitions-r16				BIT STRING (SIZE(3))

},

ce-ModeB						SEQUENCE {

subPRB-Allocation-r16			BOOLEAN,

numRUs-r16						BOOLEAN,

prb-AllocationInfo-r16			BIT STRING (SIZE(8)),

mcs-r16							BIT STRING (SIZE(4)),

numRepetitions-r16				BIT STRING (SIZE(3))

}

}	OPTIONAL,	-- Need ON

pur-PUSCH-FreqHopping-r16		BOOLEAN,

p0-UE-PUSCH-r16					INTEGER (-8..7),

alpha-r16						Alpha-r12,

pusch-CyclicShift-r16			ENUMERATED {n0, n6},

pusch-NB-MaxTBS-r16				BOOLEAN,

locationCE-ModeB-r16			INTEGER (0..5)	OPTIONAL – Cond SubPRB

}

The majority of the companies propose to not support pusch-NB-MaxTBS-r16 and pusch-CyclicShift-r16. All companies agree that the PRB allocation should be allocated as a set.

Proposal reply to Q3: From RAN1 perspective:

- pusch-NB-MaxTBS-r16 and pusch-CyclicShift-r16 are not needed to be signaled.

- prb-AllocationInfo should be defined as a “set” format with intention to provide a set of shared frequency-domain resources

| Companies | Comments |
| --- | --- |
| Ericsson | We can be ok with the “Proposal reply to Q3” upon including the following statement: To be confirmed by RAN2 whether to support both multi-PRB allocation and sub-PRB allocation, or Multi-PRB allocation only, or sub-PRB allocation only. In our view, using sub-PRB for LTE-MTC will facilitate the design, will minimize the specification impact, and would allow achieving an equivalent solution like the one that can achieved for NB-IoT with 15 kHz SCS. |
| Xiaomi | For the prb-AllocationInfo, we share the similar view as Ericsson that further clarification on the resource allocation granularity is necessary for the resource set configuration. By the way, we just wonder it should be a list of prb-AllocationInfo to configure a set of resources, rather than use a single RRC parameter prb-AllocationInfo to configure FDM-ed TOs, right? |
| Vivo1 | agree |
| CMCC2 | Fine with the proposal. |
| Apple | Both parameter prb-AllocationInfo and pusch-CyclicShift-r16 can improve the Msg3 PUSCH capacity ,multiple values of pusch-CyclicShift-r16 can be signalled. If pusch-CyclicShift is not siganalled, what DMRS cyclic shift is assumed ? |
| LGE | I have one thing to clarify. So, if the cyclic shift parameter is not introduced, is it correct understanding the following part will be used? Otherwise, the cyclic shift in a slot is given as with where the value of is given by Table 5.5.2.1.1-2 according to the parameter cyclicShift provided by higher layers. For non-BL/CE UEs is given using the most recent uplink-related DCI TS 36.212 [3] for the transport block associated with the corresponding PUSCH transmission, except for subslot-PUSCH/slot-PUSCH, as follows: For BL/CE UEs, a cyclic shift field of '000' shall be assumed when determining from Table 5.5.2.1.1-1.. If yes, we are fine with the proposal. |
| Lenovo | We are fine with the proposal |
| OPPO | We basically support the proposal. In addition, in our understanding, the PUR frequency hopping is an optional UE capability, so it is unnecessary to mandate the CB-Msg3-EDT PUSCH is transmitted with frequency hopping. Thus, the pur-PUSCH-FreqHopping is also not needed. |
| Huawei, HiSilicon | support |
| Nokia, NSB | To have quick progress, less impact to spec and implementation is needed, thus better to reuse pusch-NB-MaxTBS-r16 and pusch-CyclicShift-r16. We are fine for the second bullet. |
| ZTE | Fine with the proposal. Additionally, within this “set”, each entry corresponds to one configured resource in the same manner as existing definition. |

## 4.1 Proposed reply to Q3

Proposal reply to Q3: From RAN1 perspective:

- pusch-NB-MaxTBS-r16 and pusch-CyclicShift-r16 are not needed to be signaled.

- prb-AllocationInfo should be defined as a “set” format with intention to provide a set of shared frequency-domain resources

# 5. Question Q4

RAN2 asked question Q2 for eMTC on PUSCH configuration for shared resource configuration for CB-msg3-EDT

Q4: For eMTC, RAN2 has discussed the PDSCH configuration for shared resource configuration. Some companies think PUR PDSCH configuration is configured according to UE capability, which is not the case for CB-Msg3/Msg4 configuration. So, RAN2 does not know whether the parameters of PUR PDSCH configuration (in pur-PDSCH-FreqHopping and pur-PDSCH-maxTBS within IE PUR-Config-r16) is needed. RAN2 would like to check with RAN1 on whether and how to define the detail PDSCH configuration.

PUR-Config-r16 ::=		SEQUENCE {

<Skip unrelated Part>

pur-PDSCH-FreqHopping-r16		BOOLEAN,

<Skip unrelated Part>

...,

[[	pur-PDSCH-maxTBS-r17		BOOLEAN						OPTIONAL	-- Need ON

]]

}

For this question, the majority of the companies (6) propose to not support frequency hopping and larger TBS. One companies propose to keep these parameters.

Proposal reply to Q4: From RAN1 perspective:

- pur-PDSCH-FreqHopping and pur-PDSCH-maxTBS are not needed to be signaled.

| Companies | Comments |
| --- | --- |
| Ericsson | We are not sure yet on whether “pur-PDSCH-FreqHopping” is not needed. |
| Xiaomi | Support. |
| Vivo1 | agree |
| CMCC2 | Support |
| Apple | OK |
| LGE | OK |
| Lenovo | OK |
| OPPO | Support |
| Huawei, HiSilicon | support |
| Nokia, NSB | We do not support this proposal reply. From network flexibility, pur-PDSCH-FreqHopping and pur-PDSCH-maxTBS should be supported, as UE will not report its capability prior to CB-Msg3 transmission. |
| ZTE | Fine with the proposal |

## 5.1 Proposed response to Q4

Proposal reply to Q4: From RAN1 perspective:

- pur-PDSCH-FreqHopping and pur-PDSCH-maxTBS are not needed to be signaled.

# 6. Question Q5

RAN2 asked question Q2 for eMTC on PUCCH configuration for shared resource configuration for CB-msg3-EDT

Q5: For eMTC, RAN2 agrees to have PUCCH configuration for shared resource configuration and decides to reuse the parameters from PUR PUCCH configuration (in IE PUR-PUCCH-Config-r16, as below) as baseline. RAN2 would like to check with RAN1 on how to define the detailed PUCCH configuration.

PUR-PUCCH-Config-r16 ::=			SEQUENCE {

n1PUCCH-AN-r16						INTEGER (0..2047)			OPTIONAL,	-- Need ON

pucch-NumRepetitionCE-Format1-r16	ENUMERATED {n1, n2, n4, n8}	OPTIONAL	-- Need ON

}

The majority of the companies propose to reuse the current parameters. Nokia, however, proposes to and n1PUCCH to the successRAR in case RAN2 introduces a broadcast response (similar to successRAR)

Moderator’s view: In the current specification, the HARQ-ACK resource is defined based on n1PUCCH + some additional parameters depending on the scheduling:

In case RAN2 provides a broadcast response to CB-msg3-EDT, then additional parameter may be needed in the broadcast response (similar to 2-step RACH).

Proposal reply to Q5:  n1PUCCH-AN and pucch-NumRepetitionCE-Format1 can be reused as in PUR-PUCCH-Config. Additionally, if RAN2 introduces a broadcast response to CB-msg3-EDT (e.g. similar to successRAR), additional parameters may need to be included in the successRAR to avoid collision between PUCCH of multiple UEs.

| Companies | Comments |
| --- | --- |
| Ericsson | We are ok with the “Proposal reply to Q4”. |
| Xiaomi | Cell-specific RRC parameters with UE-specific factors to determine the PUCCH resources is necessary. Maybe the logic of PUCCH resources configuration solution for HARQ feedback of Msg4 could reused (if exists). |
| Vivo1 | agree |
| CMCC2 | Fine with the proposal. |
| Apple | Ok with this Proposal. |
| LGE | OK |
| Lenovo | OK |
| OPPO | OK |
| Huawei, HiSilicon | We hope to see RAN2 progress on the use of PUCCH for multiple UE at first |
| Nokia, NSB | In April, RAN2 agreed “RAN2 confirms the working assumption that one CB-Msg4 can target multiple UEs simultaneously. FFS how the multiplexing is organized.” Thus, the broadcast response is already agreed, and we can update the proposal reply to Q4 accordingly (change the “if RAN2 introduces” to “since RAN2 introduces”). |
| ZTE | For the parameter configuration, e.g., n1PUCCH-AN and pucch-NumRepetitionCE-Format1, it’s fine to reuse it. Regarding the avoidance of the PUCCH collision for multiple UEs, it can be up to RAN2’s decision on whether and how to support it. If majority in RAN1 prefer to provide the suggestion, we can start with the view that “avoidance is beneficial from RAN1 perspective”. Once the issue is confirmed by RAN2, we can work on the details in next stage. |

## 6.1 Proposed reply to Q5

Proposal reply to Q5:  n1PUCCH-AN and pucch-NumRepetitionCE-Format1 can be reused as in PUR-PUCCH-Config. Additionally, if RAN2 introduces a broadcast response to CB-msg3-EDT (e.g. similar to successRAR), additional parameters may need to be included in the successRAR to avoid collision between PUCCH of multiple UEs.

# 7. Question Q6

RAN2 asked question Q6 for NB-IoT on physical layer parameters pur-PhysicalConfig-r16 in PUR-Config-NB as baseline:

Question Q6: For NB-IoT, RAN2 agrees to reuse the physical layer parameters pur-PhysicalConfig-r16 in PUR-Config-NB as baseline. RAN2 assumes to have below parameters:

- Number of resource units for NPUSCH (as in npusch-NumRUsIndex-r16)

- Number of repetitions for NPUSCH (as in npusch-NumRepetitionsIndex-r16)

- Set of subcarriers (similar to npusch-SubCarrierSetIndex but change it to a “set”), FFS whether subcarriers are provided as a contiguous set.

- MCS configuration for NPUSCH (as in npusch-MCS-r16).

Note that the power parameters (p0-UE-NPUSCH-r16 and alpha-r16) should be updated according to the result of Q1.

RAN2 would like check with RAN1 on how to define the detail /physical layer parameters.

pur-PhysicalConfig-r16				SEQUENCE {

carrierConfig-r16					CarrierConfigDedicated-NB-r13,

npusch-NumRUsIndex-r16			INTEGER (0..7),

npusch-NumRepetitionsIndex-r16		INTEGER (0..7),

npusch-SubCarrierSetIndex-r16		CHOICE {

khz15							INTEGER (0..18),

khz3dot75							INTEGER (0..47)

},

npusch-MCS-r16					CHOICE {

singleTone						INTEGER (0..10),

multiTone						INTEGER (0..13)

},

p0-UE-NPUSCH-r16				INTEGER (-8..7),

alpha-r16						ENUMERATED {al0, al04, al05, al06, al07, al08, al09, al1},

npusch-CyclicShift-r16				ENUMERATED {n0, n6},

npdcch-Config-r16				NPDCCH-ConfigDedicated-NB-r13

}	OPTIONAL,	-- Need ON

There is good consensus on RAN1 to re-use pur-PhysicalConfig-r16 configuration for CB-msg3-EDT as baseline..

- npusch-SubCarrierSetIndex-r16 with khz15 and khz3dot75:

  - 9 companies support to re-use this field.

  - RAN1 can discuss whether the value should be changed to a “set”; whether to support khz15 and khz3dot75 also need further discussion.

- npusch-NumRUsIndex-r16:

  - 7 companies support to re-use this field.

- npusch-NumRepetitionsIndex-r16:

  - 6 companies support to re-use this field.

- npusch-MCS-r16 with singleTone and multitone:

  - 6 companies support to re-use this field.

- npdcch-Config-r16:

  - 7 companies support to re-use this field.

- TBS per CE level:

  - 2 companies support to re-use this field. RAN1 can further discuss whether to use TBS per CE level, or ask RAN2 to further clarify whether this field need to be specified as it was not included in the list.

- npusch-CyclicShift-r16:

  - 3 companies indicated not needed (E///, ZTE, OPPO)

  - 1 company support if RAN2 agrees 1 PRB = 2 separate CB-msg3-EDT resources

- carrierConfig-r16:

  - 1 company support

Both npusch-SubCarrierSetIndex-r16, both khz15 and khz3dot75 sub fields could be supported, or a single sub field could be supported. RAN1 can further discuss, or ask RAN2 to make a further agreement of whether to support one subfield of both subfields.

The values for HL parameters npusch-NumRUsIndex-r16, npusch-NumRepetitionsIndex-r16, npusch-SubCarrierSetIndex-r16 and npusch-MCS-r16 with singleTone and multiTone are consistent with Release 16 PUR configuration and should be used as baseline for CB-Msg3 EDT configuration (details of Release 16 PUR configuration for these parameters can be found in the Appendix B).

Alpha-r16 and p0-UE-NPUSCH-r16 areHL  parameters relevant for power ramping if supported as discussed in reply to Q1.

npusch-CyclicShift-r16 is a HL parameter that allows up to 2 UEs to use same time and frequency resource to transmit NPUSCH in Release-16 PUR. For PUR it is known from dedicated config. For CB-msg3-EDT, RAN1 can discuss whether this parameter is configured via common signalling and UE behaviour to choose one value.

carrierConfig-r16 is a HL parameter configured via dedicated signalling in Release-16 PUR. RAN1 can discuss whether this parameter should be configured via common signalling for CB-msg3-EDT. Since RAN2 agreed to introduce a new IE (e.g. CB-Msg3-ConfigSIB-NB-r19) for shared resources configuration of CB-Msg3 in SIB2-NB and SIB22-NB for non-anchor carrier in for Release 19 NB-IoT NTN, this HL parameter carrierConfig-r16 chould be configured on SIB  for CB-msg3-EDT.

Proposal reply to Q6: RAN1 agrees to re-use pur-PhysicalConfig-r16 configuration for CB-msg3-EDT as baseline. RAN1 discussed the following fields in CB-msg3-EDT configuration for NB-IoT NTN:

- The following parameters can be supported:

  - npusch-SubCarrierSetIndex-r16 with khz15 and khz3dot75 (To be confirmed by RAN2 whether to support both values, or a single value)

  - npusch-NumRUsIndex-r16

  - npusch-NumRepetitionsIndex-r16

  - npusch-SubCarrierSetIndex-r16 (but defining this as a set)

  - npusch-MCS-r16	 with singleTone and multitone

  - npdcch-Config-r16

- RAN1 can further discuss whether the following fields can be supported:

  - npusch-CyclicShift-r16 if RAN2 agrees 1 PRB = 2 separate CB-msg3-EDT resources

  - carrierConfig-r16 if configured on SIB2-NB for anchor carrier and SIB22-NB for non anchor carrier

Companies are encouraged to provides comments on RAN1 reply to Q6 below

| Companies | Comments |
| --- | --- |
| Ericsson | We are ok with the proposal upon incorporating the following comments: - npusch-MCS-r16 with singleTone and multitone (To be confirmed by RAN2 whether to support both singleTone and multitone, or singleTone only). In our understanding the list of parameters above are meant to be related with npusch, therefore it seems that “npdcch-Config-r16” should be removed from the list. For the second bullet, we have the following suggestions to keep it simpler: - Depending on RAN2 design decisions on CB-Ms3-EDT, RAN1 may further discuss whether the following fields should be supported: - npusch-CyclicShift-r16 if RAN2 agrees 1 PRB = 2 separate CB-msg3-EDT resources - carrierConfig-r16 if configured on SIB2-NB for anchor carrier and SIB22-NB for non anchor carrier |
| Xiaomi | Share the similar view with Ericsson that npdcch-config-r16 should be removed in the list. Regarding whether to support 15kHz SCS NPUSCH format 1 with single-tone, there is the following agreement in RAN2 according to the incoming LS from RAN2(R1-2503613): We specify support for NB-IoT with 15kHz with no specific enhancements, leaving to NW implementation whether to implement this or not, accepting potential performance degradation.. Based on above, we suppose that if a set of NPUSCH resources are configured, whether 3.75kHz SCS and 15kHz SCS could be simultaneously configured are determined by the eNB. Besides, the set of resources maybe include the following three types simultaneously: single-tone 15kHz SCS, single-tone 3.75 kHz SCS, multi-tone 15 kHz SCS. |
| Vivo1 | NPDCCH is discussed in Q7, there is no need to consider it in Q6 We are fine with the suggestion that “RAN1 further discusses carrierConfig.” Alternatively, the decision can be left to RAN2. Regarding npusch-CyclicShift, we are wondering why it is handled differently compared to pusch-CyclicShift for MPUSCH in Q3(i.e., not supported). |
| CMCC2 | We do not support to reuse npusch-CyclicShift-r16 for the CB-msg3. The npusch-CyclicShift-r16 will introduce UE multiplexing for the unknown UE during the initial access. It will complicate the gNB’s processing and increase the interference, which will lower the success rate for UE initial access through CB-msg3. |
| EC | Current RAN1 specficiation doesn’t support using npusch-SubCarrierSetIndex to configure a set of subcarriers for 3.75 kHz SCS. How to define a set of subcarriers still need RAN1 work. |
| Apple | Again, if npusch-CyclicShift is not supported, what is the DMRS cyclic shift assumption from UE side? |
| LGE | Regarding the cyclic shift, the current specification specifies the default cyclic shift only for 12-tone case. The cyclic shift for and is derived from higher layer parameters threeTone-CyclicShift and sixTone-CyclicShift, respectively, as defined in Table 10.1.4.1.2-3. For , if npusch-CyclicShift in PUR-Config-NB is configured for NPUSCH (re)transmission corresponding to preconfigured uplink resource it provides the value of ncs and the cyclic shift α in a slot is given as α=2πncs/12, otherwise .. Meanwhile, it seems that threeTone-CyclicShift and sixTone-CyclicShift, are the common parameter. So, depending on the number of tones, the UE will determine the cyclic shift value to be threeTone-CyclicShift, sixTone-CyclicShift, or 0 for 3-tone, 6-tone, 12-tone, respectively. We also think that the NPDCCH can be handled in another question. |
| OPPO | Regarding the 1st subbullet in the proposal, i.e.., npusch-SubCarrierSetIndex-r16 with khz15 and khz3dot75 (To be confirmed by RAN2 whether to support both values, or a single value), RAN2 already made an agreement to support 15kHz SCS for CB-Msg3 EDT, so the text in the bracket can be removed. “We specify support for NB-IoT with 15kHz with no specific enhancements, leaving to NW implementation whether to implement this or not, accepting potential performance degradation.” In addition, similar to eMTC, npusch-CyclicShift is also not needed because the NPUSCH configuration related to CB-Msg3-EDT is cell-specific parameter, and signaling 1 cyclic shift anyway does not allow UE multiplexing. |
| Huawei, HiSilicon | We are fine with the first part of proposal to reuse the existing parameters. Suggest to merge the two subbullet on “npusch-SubCarrierSetIndex-r16” together. For the RAN1 study part, we think it is over optimized. |
| Nokia, NSB | We are OK for this proposal reply. |
| ZTE | Regarding npusch-NumRepetitionsIndex-r16, we think RAN1 should clarify that the repetition number is applied for each replica, since it is mainly determined based on coverage performance and should not be shared by different replicas. |

## 7.1 Proposed reply to Q6

Proposal reply to Q6: RAN1 agrees to re-use pur-PhysicalConfig-r16 configuration for CB-msg3-EDT as baseline. RAN1 discussed the following fields in CB-msg3-EDT configuration for NB-IoT NTN:

- The following parameters can be supported:

  - npusch-SubCarrierSetIndex-r16

  - npusch-NumRUsIndex-r16

  - npusch-NumRepetitionsIndex-r16

  - npusch-SubCarrierSetIndex-r16 (but defining this as a set)

  - npusch-MCS-r16	 with singleTone and multitone

  - npdcch-Config-r16

- Depending on RAN2 design decisions on CB-Ms3-EDT, RAN1 may further discuss whether the following fields should be supported:

- npusch-CyclicShift-r16

- carrierCnfig-r16

Note 1: To be confirmed by RAN2 whether to also support khz3dot75, since RAN2 already agreed to support  khz15

Note 2: To be confirmed by RAN2 whether to support both singleTone and multitone, or singleTone only for HL parameter npusch-MCS-r16.

# 8. Question Q7

RAN2 asked question Q7 for NB-IoT on PUR PDCCH configuration (in IE NPDCCH-ConfigDedicated-NB-r13) as baseline:

Q7: For NB-IoT, RAN2 agrees to have NPDCCH configuration for shared resource configuration and decides to reuse the parameters from PUR PDCCH configuration (in IE NPDCCH-ConfigDedicated-NB-r13, as below) as baseline. RAN2 would like to check with RAN1 on how to define the detail NPDCCH configuration.

NPDCCH-ConfigDedicated-NB-r13 ::=	SEQUENCE {

npdcch-NumRepetitions-r13			ENUMERATED {r1, r2, r4, r8, r16, r32, r64, r128, r256, r512, r1024, r2048,

spare4, spare3, spare2, spare1},

npdcch-StartSF-USS-r13			ENUMERATED {v1dot5, v2, v4, v8, v16, v32, v48, v64},

npdcch-Offset-USS-r13				ENUMERATED {zero, oneEighth, oneFourth, threeEighth}

}

NPDCCH-ConfigDedicated-NB-v1530 ::=	SEQUENCE {

npdcch-StartSF-USS-v1530			ENUMERATED {v96, v128}

}

There is good consensus in RAN1. There are 7 companies that support re-use  PUR PDCCH configuration (in IE NPDCCH-ConfigDedicated-NB-r13) as baseline.

RAN2 agrees to have NPDCCH configuration for shared resource configuration and decides to reuse the parameters from PUR PDCCH configuration (in IE NPDCCH-ConfigDedicated-NB-r13, as below) as baseline . NPDCCH-ConfigDedicated-NB-r13 is used to configure npdcch-Config-r16 in PUR-Config-NB-r16 IE. This is used for HARQ re-transmission of NPUSCH with PUR-RNTI. For NPDCCH UE-specific search space, the aggregation and repetition levels defining the search spaces and the corresponding NPDCCH candidates associated with PUR-RNTI are given by higher layer parameter npdcch-NumRepetitions in PUR-Config-NB (TS 36.213 Clause 16.6). Similar way can be used for NPDCCH configuration for indication of msg4 on NPDSCH for CB-msg3-EDT.

RAN2#129bis captured note in the summary on L1 parameters for CB-msg3-EDT

cb-Msg3-PhysicalConfig-r19

- npdcch-NumRepetitions-r19/npdcch-StartSF-CSS-r19/npdcch-Offset-r19: 3 parameters copied from NPDCCH-ConfigDedicated-NB-r13. Note hear it should be CSS instead of USS.

npdcch-StartSF-USS-r13 and npdcch-Offset-USS-r13 should be with CSS as USS cannot be configured for CB-msg3-EDT. Hence, this parameter could be renamed as npdcch-StartSF-CSS-r19 and npdcch-Offset-CSS-r19. RAN1 can discussed the details of CSS for NPDCCH for indication of msg4.

Proposal reply to Q7: NPDCCH-ConfigDedicated-NB-r13 IE can be used as baseline for NPDCCH configuration for indication of msg4 on NPDSCH for CB-msg3-EDT. RAN1 assumes this configuration will be provided over broadcast RRC signalling.

Companies are encouraged to provides comments on RAN1 reply to Q7 below

| Companies | Comments |
| --- | --- |
| Ericsson | We are ok with the “Proposal to reply to Q7”. |
| Xiaomi | Just one comment on the name of RRC parameter: Is it reasonable to be named as “NPDCCH-ConfigDedicated-NB-r13” for a cell-specific configuration? |
| CMCC2 | Fine with the proposal. |
| Apple | ok |
| LGE | OK. It seems that npdcch-NumRepetitions-RA-r13, npdcch-StartSF-CSS-RA-r13 and npdcch-Offset-RA-r13 have the same set of candidate values compared to USS. |
| Lenovo | OK |
| OPPO | OK |
| Huawei, HiSilicon | support |
| Nokia, NSB | One comment for this proposal reply: the NPDCCH configuration shall be per CE level. This enables UEs in various coverage conditions to reliably receive the DCI. i.e. updated as following: NPDCCH-ConfigDedicated-NB-r13 IE can be used as baseline for NPDCCH configuration for indication of msg4 on NPDSCH for CB-msg3-EDT, with per CE level. RAN1 assumes this configuration will be provided over broadcast RRC signalling. |
| ZTE | OK |

## 8.1 Proposed reply to Q7

Proposal reply to Q7: NPDCCH-ConfigDedicated-NB-r13 IE can be used as baseline for NPDCCH configuration for indication of msg4 on NPDSCH for CB-msg3-EDT. RAN1 assumes this configuration will be provided over broadcast RRC signalling.

# 9. Question Q8

RAN2 asked question Q8 for both eMTC and NB-IoT on PUR PDCCH configuration (in IE NPDCCH-ConfigDedicated-NB-r13) as baseline:

Q8: For both eMTC and NB-IoT, any other L1 parameters are needed for CB-Msg3-EDT procedure in additional to previous discussion?

In addition, RAN2 agrees that:

Introduce a new RNTI (i.e. CB-RNTI) for CB-Msg4 monitoring and CB-Msg3 scrambling. We include this agreement in the LS to RAN1

RAN2 would like to inform RAN1 on the new RNTI for CB-Msg4 monitoring and CB-Msg3 scrambling. RAN2 has not yet made agreements on how to determine the RNTI, but RAN2 assumes there may be some potential RAN1 specification impact on this.

More discussion may be needed in RAN1 on Ran1 specifications impact:

- 1 company indicated there is no impact, wait for RAN2.

- 2 companies indicated support

Proposal reply to Q8: The following additional L1 parameters may need to be signaled:

- OCC ON/OFF (if RAN2 decides to introduce support for CB-msg3 with OCC)

Companies are encouraged to provides comments on RAN1 reply to Q8 below

| Companies | Comments |
| --- | --- |
| Ericsson | In our understanding, this LS is for CB-Msg3-EDT without OCC, since to our best knowledge RAN2 has a separate discussion on the potential support of OCC for CB-msg3. |
| Xiaomi | For every resource in the set of FDMed transmission occasions, we think the OCC enabling parameter should be individually configured. As our comments in Q6, the set of resources configured by the eNB maybe include the following three types simultaneously: single-tone 15kHz SCS, single-tone 3.75 kHz SCS, multi-tone 15 kHz SCS. Different resource may have different relationship with OCC multiplexing. Furthermore, considering that some UEs may not support OCC multiplexing, flexible configuration on the combination of CB-MSG3-EDT and OCC multiplexing is more necessary. Furthermore, one further clarification may be necessary for the use of the frequency domain resource set: Between several replicas transmissions for a same TB for a given UE, whether the frequency domain resources could be changed? For example, the first replica chooses the frequency-domain Resource #1, the second replica selects the frequency-domain Resource #3. |
| Vivo1 | Same view as Ericsson, the CB-msg3-EDT with OCC is a separate discussion and is handled by RAN2. No need to discuss this |
| CMCC2 | Not support. Our thinking is that it is RAN1’s evaluation and decision on whether support OCC for CB-msg3. Similar as introducing the cyclic shift for CB-msg3, we do not thinking it is reasonable to support multiple UE multiplexing for the CB-msg3. |
| NEC | Same as VIVO and Ericsson. |
| Apple | We are open to discuss whether support OCC in CB-Msg3 transmission. |
| Lenovo | We are open to discuss whether to support OCC in CB-Msg3. |
| Huawei, HiSilicon | OCC for CB msg3 is low prioritized in RAN2. We can conclude not support it or up to RAN2. |
| Nokia, NSB | We don’t think it is necessary to mention the OCC in this reply. |
| ZTE | We are open to discuss the combination between OCC and CB-Msg3. |

## 9.1 Proposed reply to Q8

Proposal reply to Q8: RAN1 can further discuss on additional L1 parameters that may need to be signaled:

# 10. Conclusions:

TBA

# 11. Appendix A

In RAN2#129

RAN2 #129 Agreements:

1.	RAN2 assumes that at least the following will be part of the shared resources configuration for CB-msg3 (FFS on other aspects)

-	Time domain resources for (N)PUSCH occasions: periodicity and start time (e.g., start subframe, start SFN)

-	Frequency domain resources for (N)PUSCH occasions

-	repetition number

-	(N)PDCCH resource

-	MCS

6.	As Signalling design Baseline RAN2 assumes the PUR config and the NPRACH config for shared (N)PUSCH config can be used and some of the parameters can be included in a new CB EDT config.

In RAN2#129bis

Agreements;

4.	We don’t introduce support for eMTC CE mode B case (it will not be possible to signal resources to be used for this case)

5.	We specify support for NB-IoT with 15kHz with no specific enhancements, leaving to NW implementation whether to implement this or not, accepting potential performance degradation.

7.	The start of CB-msg3 EDT transmission window is aligned with the start of time domain (N)PUSCH resource.

8.	The CB-msg3 EDT transmission window length and periodicity may be different. FFS on possible signalling optimization in case the length and periodicity are the same.

9.	RAN2 assumes power ramping should be supported for CB-msg3-EDT (for both eMTC and NB-IoT) should be supported and will ask RAN1 for confirmation and in case which parameters should apply

(CB-Msg3-EDT configuration for eMTC)

10.	For eMTC, introduce a new IE (e.g. CB-Msg3-ConfigSIB-r19) for shared resources configuration of CB-Msg3 in SIB2.

11.	For eMTC, introduce MPDCCH configuration in shared resources configuration. The fields in IE PUR-MPDCCH-Config-r16 could be reused as baseline. Confirm with RAN1 on the detail parameters (e.g. whether additional narrow band is needed).

12.	We will not support TDD related parameters.

13.	For eMTC, introduce PUSCH configuration in shared resources configuration. The fields in IE PUR-PUSCH-Config-r16 could be reused as baseline. Confirm with RAN1 on the detail parameters. (e.g. whether pusch-CyclicShift-r16, pusch-NB-MaxTBS-r16 are needed, whether prb-AllocationInfo should be defined as a “set” format with intention to provide a set of shared frequency-domain resources).

14.	For eMTC, check with RAN1 if anything is needed for PDSCH configuration in shared resources configuration

15.	For eMTC, introduce PUCCH configuration in shared resources configuration. The fields in IE PUR-PUCCH-Config-r16 could be reused as baseline. Confirm with RAN1 on the detail parameters.

(CB-Msg3 configuration for NB-IoT)

16.	For NB-IoT, introduce a new IE (e.g. CB-Msg3-ConfigSIB-NB-r19) for shared resources configuration of CB-Msg3 in SIB2-NB and SIB22-NB for non-anchor carrier.

17.	For NB-IoT, introduce below physical layer parameters in shared resources configuration as below:

-	Number of resource units for NPUSCH (as in npusch-NumRUsIndex-r16)

-	Number of repetitions for NPUSCH (as in npusch-NumRepetitionsIndex-r16)

-	Set of subcarriers (similar to npusch-SubCarrierSetIndex but change it to a “set”), FFS whether subcarriers are provided as a contiguous set.

-	MCS configuration for NPUSCH (as in npusch-MCS-r16).

-	PDCCH parameters (as in NPDCCH-ConfigDedicated-NB-r13)

-	The non-anchor carrier index for monitoring Msg4. If this field is absent, anchor carrier is assumed to be used.

NOTE: confirm with RAN1 is needed

Agreements (part3):

1.	The CB-msg3-EDT configuration (e.g., number of replicas, number of time resources and number of frequency resources) is CE level specific.

# 12. Appendix B

According to the specifications in TS 36.213 Clause 16.5.1.1 Resource allocation

- The resource allocation information in uplink DCI format N0 for NPUSCH transmission or configured by higher layers for NPUSCH transmission using preconfigured uplink resource indicates to a scheduled UE

- a set of contiguously allocated subcarriers () of a resource unit determined by the Subcarrier indication field, or by the higher layer parameter npusch-SubCarrierSetIndex in PUR-Config-NB

- a number of resource units () determined by the resource assignment field according to Table 16.5.1.1-2, or by the higher layer parameter npusch-NumRUsIndex in PUR-Config-NB

- a repetition number () determined by the repetition number field according to Table 16.5.1.1-3, and for a NPUSCH transmission using preconfigured uplink resource, the UE shall use the repetition number configured by higher layers; except for NPUSCH with 16QAM where NRep=1.

- For NPUSCH transmission with subcarrier spacing, where  is the subcarrier indication field and is reserved, or nsc is configured by higher layers parameter npusch-SubCarrierSetIndex in PUR-Config-NB for NPUSCH transmissions using preconfigured uplink resources.

- For NPUSCH transmission with subcarrier spacing, the subcarrier indication field () in the DCI or npusch-SubCarrierSetIndex in PUR-Config-NB for NPUSCH transmissions using preconfigured uplink resources determines the set of contiguously allocated subcarriers () according to Table 16.5.1.1-1.

Table 16.5.1.1-1: Allocated subcarriers for NPUSCH with .

| Subcarrier indication field () | Set of Allocated subcarriers () |
| --- | --- |
| 0 – 11 |  |
| 12-15 |  |
| 16-17 |  |
| 18 |  |
| 19-63 | Reserved |

Table 16.5.1.1-2: Number of resource units () for NPUSCH.

| 0 | 1 |
| --- | --- |
| 1 | 2 |
| 2 | 3 |
| 3 | 4 |
| 4 | 5 |
| 5 | 6 |
| 6 | 8 |
| 7 | 10 |

Table 16.5.1.1-3: Number of repetitions () for NPUSCH.

| 0 | 1 |
| --- | --- |
| 1 | 2 |
| 2 | 4 |
| 3 | 8 |
| 4 | 16 |
| 5 | 32 |
| 6 | 64 |
| 7 | 128 |

# 13. References

- R2-2503185, Reply LS on soft satellite switch with re-sync, 3GPP TSG RAN WG1 #120bis, Wuhan, China, 07th – 11th April 2025

- R1-2501606, Reply LS on soft satellite switch with re-sync, 3GPP TSG RAN WG1 Meeting #120, Athens, Greece, February 17th-21st, 2025

- R2-2502773, Moderator (MediaTek), Report of [Post129][307][R19 IoT NTN] CB-msg3/CB-msg4, RAN2#129bis, Wuhan, China, 07th – 11th April 2025