3GPP TSG RAN WG1 #121                                                                                     R1-2504876

St Julian’s, Malta, May 19th – 23rd, 2025

Source:	Moderator (CATT)

Title:	Summary#3 on LS on differentiation of sDCI based mTRP and sTRP

Agenda Item:	7

Document for:	Discussion

Introduction

In RAN1#121 meeting, the following conclusion is achieved on the LS sent from RAN2 to ask for suggestions from RAN1 on differentiation of sDCI mTRP, mDCI mTRP and sTRP [1]:

Conclusion

RAN1 has no consensus on the down-selection between the alternatives of the below agreement:

Agreement To differentiate the cells operated as sTRP vs sDCI mTRP, down-select one of the options: - Alt-1: It is RAN1 consensus that RRC parameter(s) is used to indicate whether a serving cell is in sDCI mTRP operation or sTRP operation. - Option 1: RAN1 agrees that whether RRC parameter applyIndicatedTCI-State is configured in ControlResourceSet can be used to indicate whether a serving cell is in sDCI mTRP operation or sTRP operation. However, for each list, the network ensures that either at least one of the following RRC parameters is configured for all BWPs of all serving cells or none of the following RRC parameters is configured in any BWP of any serving cell. - “applyIndicatedTCI-State-r18” in ConfiguredGrantConfig, ControlResourceSet, PUSCH-Config, SRS-ResourceSet, PDCCH-ConfigCommon, PUCCH-ResourceExt-v1610, or - “applyIndicatedTCI-State-r18” and “applyIndicatedTCI-State2-r18” in CSI-AssociatedReportConfigInfo, or - “applyIndicatedTCI-StateDCI-1-0-r18” in TCI-InDCI-r18. - mappingPattern-r17”, “multipanelSchemeSDM-r18”, “multipanelSchemeSFN-r18” in PUSCH-Config - “repetitionSchemeConfig-r16” in PDSCH-Config - “searchSpaceLinkingId-r17” in searchSpace - “sfnSchemePDCCH-r17” and “sfnSchemePDSCH-r17” in MIMOParam-r17 - “cjt-Scheme-PDSCH-r18” in ServingCellConfig - Two SRS resource sets configured in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with higher layer parameter usage in SRS-ResourceSet set to 'codebook' or ‘noncodebook’ - Option 2 (No concern): RAN1 agrees that whether RRC parameter applyIndicatedTCI-State is configured in ControlResourceSet can be used to indicate whether a serving cell is in sDCI mTRP operation or sTRP operation. - Alt 2: It is RAN1 consensus that the number of indicated TCI states can be used to indicate whether a serving cell is in sDCI mTRP operation or sTRP operation.

This moderator summary aims at collecting the comments from companies regarding whether/how we capture the conclusion in a draft LS.

Discussion

## 2.1 Draft reply LS

Based on the conclusion we have, I have prepared the below draft reply LS:

- Overall description

RAN1 would like to thank RAN2 for the LS on differentiation of mTRP and sTRP. RAN1 provides the corresponding replies.

- sTRP vs sDCI mTRP

To separate the cells operated as sTRP and sDCI mTRP, it is proposed to add the following highlight sentence. imultaneousU-TCI-UpdateList1, simultaneousU-TCI-UpdateList2, simultaneousU-TCI-UpdateList3, simultaneousU-TCI-UpdateList4 List of serving cells for which the Unified TCI States Activation/Deactivation MAC CE applies simultaneously, as specified in TS 38.321 [3] clause 6.1.3.47. The different lists shall not contain same serving cells. Network only configures in these lists serving cells that are configured with unifiedTCI-StateType. Network should not configure serving cells that are configured with a BWP with different number of coresetPoolIndexes in the same list. For each list, the network ensures that either applyIndicatedTCI-State is configured for all ControlResourceSets in all BWPs of all serving cells or applyIndicatedTCI-State is not configured in any ControlResourcesSet of any BWP of any serving cell.. But some companies understand that applyIndicateTCI-State with value “none” configured for ControlResourceSet indicates sTRP operation and different channels on a BWP may operate differently.

Question 2a: RAN2 would like to ask if RAN1 has any concern on the above highlight sentence?

Question 2b: If there is any concern on Question 2a, what’s RAN1’s suggestion on how to separate the cells operated as sTRP and sDCI mTRP?

[RAN1’s reply]

Question 2a): Yes.

Question 2b): RAN1 has reached to the following conclusion:

Conclusion

RAN1 has no consensus on the down-selection between the alternatives of the below agreement:

Agreement To differentiate the cells operated as sTRP vs sDCI mTRP, down-select one of the options: - Alt-1: It is RAN1 consensus that RRC parameter(s) is used to indicate whether a serving cell is in sDCI mTRP operation or sTRP operation. - Option 1: RAN1 agrees that whether RRC parameter applyIndicatedTCI-State is configured in ControlResourceSet can be used to indicate whether a serving cell is in sDCI mTRP operation or sTRP operation. However, for each list, the network ensures that either at least one of the following RRC parameters is configured for all BWPs of all serving cells or none of the following RRC parameters is configured in any BWP of any serving cell. - “applyIndicatedTCI-State-r18” in ConfiguredGrantConfig, ControlResourceSet, PUSCH-Config, SRS-ResourceSet, PDCCH-ConfigCommon, PUCCH-ResourceExt-v1610, or - “applyIndicatedTCI-State-r18” and “applyIndicatedTCI-State2-r18” in CSI-AssociatedReportConfigInfo, or - “applyIndicatedTCI-StateDCI-1-0-r18” in TCI-InDCI-r18. - mappingPattern-r17”, “multipanelSchemeSDM-r18”, “multipanelSchemeSFN-r18” in PUSCH-Config - “repetitionSchemeConfig-r16” in PDSCH-Config - “searchSpaceLinkingId-r17” in searchSpace - “sfnSchemePDCCH-r17” and “sfnSchemePDSCH-r17” in MIMOParam-r17 - “cjt-Scheme-PDSCH-r18” in ServingCellConfig - Two SRS resource sets configured in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with higher layer parameter usage in SRS-ResourceSet set to 'codebook' or ‘noncodebook’ - Option 2 (No concern): RAN1 agrees that whether RRC parameter applyIndicatedTCI-State is configured in ControlResourceSet can be used to indicate whether a serving cell is in sDCI mTRP operation or sTRP operation. - Alt 2: It is RAN1 consensus that the number of indicated TCI states can be used to indicate whether a serving cell is in sDCI mTRP operation or sTRP operation.

- Actions

To RAN2

ACTION: 	RAN1 respectfully asks RAN2 to take the information into consideration.

From moderator’s assessment, although a few companies argue that we do not have to send an LS to RAN2 regarding the LS, the other few companies believe we should reply to the answers raised by RAN2. As the moderator, I tend to agree that we should send the reply to RAN2 as they have made changes on the spec and seek the advice from RAN1. We should reply to the questions 2A and 2B raised from RAN2. And of course, please provide your views on whether/how we capture the conclusion in the reply LS.

  - Q1: Do you agree to have a reply LS? If yes, do you agree with the draft reply LS?

| Company | Agree or Not Agree | Comments |
| --- | --- | --- |

Proposal for online discussion

Since I did not receive any negative comments on sending the reply LS to RAN2, I suggest that we provide such a LS.

Proposal 1: Provide the reply LS to RAN2 for the discussion regarding the differentiation of sDCI based mTRP and sTRP. Final LS is in R1-2504885.

Reference

- R1-2500013	LS on differentiation of sDCI mTRP, mDCI mTRP and sTRP	                         CATT

- R1-25003019	Summary on LS on differentiation of sDCI mTRP and sTRP	                         CATT

- R1-2503344	Draft reply LS on differentiation of sDCI mTRP, mDCI mTRP and sTRP	vivo

- R1-2503676	Discussion on differentiation of sDCI mTRP, mDCI mTRP and sTRP	ZTE Corporation, Sanechips

- R1-2503768	Discussion on LS on differentiation of sDCI based mTRP and sTRP	CATT

- R1-2503769	Draft reply LS on differentiation of sDCI based mTRP and sTRP	CATT

- R1-2503871	Discussion on LS on differentiation of sDCI mTRP and sTRP	Xiaomi

- R1-2504186	Discussion on LS on differentiation of sDCI mTRP, mDCI mTRP and sTRP	OPPO

- R1-2504452	Discussion on LS on differentiation of sTRP and sDCI mTRP	Ericsson

- R1-2503544	Discussion on differentiation of sDCI mTRP, mDCI mTRP and sTRP	Samsung