3GPP TSG RAN WG1 #121			R1-240xxxx

St Julian’s, Malta, May 19th – 23th, 2025

Agenda Item:	9.13

Source:	Ad-Hoc Chair (Huawei)

Title:	Session notes for 9.13 LTE-based 5G Broadcast Phase 2

Document for:	Discussion, Decision

## LTE-based 5G Broadcast Phase 2

Please refer to RP-250794 for detailed scope of the WI for LTE-based 5G Broadcast Phase 2.

Rapporteur to provide initial input on higher layer signalling under agenda item 9.13. For input on higher layer signalling from any other source, please include it as part of your tdoc to relevant sub-agenda items.

[121-R19-Broadcast] Email discussion on Rel-19 LTE-bad 5G Broadcast Phase 2 – Jordi (EBU)

- To be used for sharing updates on online/offline schedule, details on what is to be discussed in online/offline sessions, tdoc number of the moderator summary for online session, etc

R1-2503609	Further discussion on Time-frequency interleaver for 5G Broadcast Phase 2	Shanghai Jiao Tong University

R1-2504652	Higher layer parameters for Rel-19 LTE-based 5G Broadcast after RAN1#120	EBU

### Time-frequency interleaver design for MBMS dedicated cells

R1-2503249	On time-frequency interleavers for LTE-based 5G Broadcast	Huawei, HiSilicon

R1-2503332	Discussion on time-frequency interleaver design for LTE-based 5G Broadcast	ZTE Corporation, Sanechips

R1-2503589	Further details on Time and Frequency Interleaving for 5G Broadcast phase-2	Samsung

R1-2504241	Further discussion on open issues regarding TFI for LTE-based 5G Broadcast	EBU

R1-2504417	Time and Frequency Interleaving for 5G Broadcast	Qualcomm Incorporated

R1-2504654	Time and Frequency Interleaving for 5G Broadcast	Qualcomm Incorporated

R1-2504732

Agreement

For each CB of the TB, k0n=2RsubblockTC+rvnEmin, where Emin=NLQmG'/C (as per TS 36.212), and rvn∈{0…N-1} is the redundancy version of the n-th subframe, where NL=1, and, rvn values are mapped sequentially to subframes, i.e., rvn=n.

Agreement

Time interleaving parameters are indicated per PMCH configuration, e.g., via a configuration of M,N in pmch-Config. Specify the following set of new MSI periodicities: {rf7, rf14, rf53, rf56, rf112, rf424}.

- Additionally down-select among {rf27 or rf28}, {rf106 or rf108}, {rf211 or rf212}.

- FFS actions on the residual mismatch after applying the new MSI periodicities in conjunction with the configuration of M,N

  - Option 1: Fill the residual space with a service with different TFI configuration (M,N), e.g., via an additional (addressing the residual space) configuration via pmch-Config

Agreement

The soft buffer size for one time-interleaved TB is down-selected from

- Option 1:  NIR=βNsoftM, where:

  - Option 1a: Nsoft is the soft buffer size pertaining to a reference UE category, M is the number of TBs for time-interleaving and β is a scaling factor. FFS: exact value/values/configuration of β.

  - Option 1b: Nsoft is configured in PMCH-Config from {1827072, 3654144, 5481216, 7308288, 9744384, 12789504, 14616576, 17052672, 19488768, 24360960, 29233152, 34105344, 36541440, 38977536, 42631680, 47431680, 303562752}, 1β is configured in PMCH-Config from: {1, 3/2, 6/5, 8/5, 2, 12/5, 8/3, 3, 5, 32};  and M is the number of TBs for time-interleaving.

- Option 2 : The soft buffer size for the code block is denoted as Ncb=min(TBSLBRMC∙RLBRM,Kw), where C and Kw are same as defined in TS 36.212 clause 5.1. TBSLBRM is calculated based on, e.g., nPRB,LBRM, ITBS, N. RLBRM=2/3.

  - FFS: Details of determining TBSLBRM, nPRB,LBRM, ITBS (e.g., based on a notion of maximum coding rate) may need to be considered, to provide the flexibilities afforded by the LTE specifications, keeping in mind the LTE UE categories.

- FFS: impact to soft buffer size for UE receiving unicast considering the case where UE prioritizes receiving broadcast.

Agreement

Confirm the working assumption in the following agreement:

Agreement

The number of columns, C, of the frequency interleaver is C= #CBs Nsymb ×m, where #CBs denotes the number of CBs in the TB (including scaling, if any), Nsymb denotes the number of OFDM symbols of the PMCH,

- Working assumption:

  - m = Nsymbgcd(Nsymb,#CBs), and gcd(.) denotes the operation to compute the greatest common divisor.

Agreement

For PMCH with time interleaving, the maximum TBS a UE supports for the scaled TB is down-selected from the following:

- Option 1: the largest TBS among the multiple maximum TBS of a DL-SCH as per TS 36.306 set by UE category.

- Option 2: the smallest TBS among the multiple maximum TBS of a DL-SCH as per TS 36.306 set by UE category.

- Option 3: signalled by the UE from the multiple maximum TBS of a DL-SCH as per TS 36.306 set by UE, e.g. using a new UE capability