3GPP TSG RAN WG1 #121			R1-240xxxx

St Julian’s, Malta, May 19th – 23th, 2025

Agenda Item:	9.4

Source:	Ad-Hoc Chair (Huawei)

Title:	Session notes for 9.4 Solutions for Ambient IoT in NR

Document for:	Discussion, Decision

## Solutions for Ambient IoT (Internet of Things) in NR

Please refer to RP-250796 for detailed scope of the WI.

[121-R19-A-IoT] Email discussion on Rel-19 Ambient IoT – Jingwen (CMCC)

- To be used for sharing updates on online/offline schedule, details on what is to be discussed in online/offline sessions, tdoc number of the moderator summary for online session, etc

R1-2503830	TP for A-IoT physical layer functions for TS 38.300	CMCC, Huawei, HiSilicon

### Physical channels design – modulation aspects

Including discussions on CP handling for R2D.

R1-2503220	Modulation aspects for Ambient IoT	Ericsson

R1-2503224	Discussion on modulation aspects for A-IoT physical channel	FUTUREWEI

R1-2503294	Physical channels design on modulation	Huawei, HiSilicon

R1-2503299	AIoT Physical channels design - modulation aspects	Nokia

R1-2503311	Discussion on Ambient IoT modulation	ZTE Corporation, Sanechips

R1-2503358	Remaining issues on Modulation Aspects of Physical Channels Design	vivo

R1-2503515	Discussion on modulation aspects of physical channels design for Ambient IoT	Spreadtrum, UNISOC

R1-2503536	Discussion on modulation aspects for Ambient IoT physical design	TCL

R1-2503566	Views on Physical channels design – modulation aspects	Samsung

R1-2503703	Discussion on modulation aspects for A-IoT physical channel	Tejas Network Limited

R1-2503725	Discussion on Physical Channel Design and Modulation Aspects for Ambient-IoT	EURECOM

R1-2503734	Views on Physical channels design – modulation aspects for AIoT	Ofinno

R1-2503793	Ambient IoT physical channel design and modulation	CATT

R1-2503831	Discussion on modulation aspects of physical channel design	CMCC

R1-2503882	Discussion on modulation aspects for Ambient IoT	Xiaomi

R1-2503924	Discussion on modulation aspects of ambient IoT	NEC

R1-2504007	A-IoT Physical Channels Design on Modulation Aspects	Panasonic

R1-2504046	Discussion on physical channels design about modulation aspects for Ambient IoT	China Telecom

R1-2504088	Modulation for R2D	Fujitsu

R1-2504098	Discussion on Physical channels design for Ambient IoT-modulation aspects	HONOR

R1-2504205	Discussion on modulation aspects of A-IoT	OPPO

R1-2504243	A-IoT PHY layer design - waveform and modulation aspects	LG Electronics

R1-2504287	Remaining issues on modulation aspects for Ambient IoT	InterDigital, Inc.

R1-2504318	On remaining modulation aspects for Ambient IoT	Apple

R1-2504394	Physical channels design – modulation aspects	Qualcomm Incorporated

R1-2504433	Discussion on modulation aspects	Sharp

R1-2504482	Discussion on Modulation Aspects for Ambient IoT	Indian Institute of Tech (M)

R1-2504501	Discussion on modulation aspects of physical channel design for Ambient IoT	NTT DOCOMO, INC.

R1-2504585	Discussion on physical channels design for A-IoT	China Unicom

R1-2504617	Discussion on modulation aspects of physical channels design for Ambient IoT	WILUS Inc.

R1-2504633	Discussion on modulation related aspects of AIoT	IIT Kanpur

### Physical channels design – line coding, FEC, CRC, repetition aspects

Including discussions on small frequency shift

R1-2503221	Coding aspects for Ambient IoT	Ericsson

R1-2503225	Coding aspects for A-IoT physical channel	FUTUREWEI

R1-2503295	Physical channel design on channel coding	Huawei, HiSilicon

R1-2503300	AIoT Physical channels design - line coding, FEC, CRC, repetition aspects	Nokia

R1-2503312	Discussion on Ambient IoTcoding and SFS	ZTE Corporation, Sanechips

R1-2503359	Remaining issues on line coding, FEC, CRC and repetition for A-IoT	vivo

R1-2503516	Discussion on line coding, FEC, CRC, repetition aspects for Ambient IoT	Spreadtrum, UNISOC

R1-2503537	Discussion on other aspects for Ambient IoT physical design	TCL

R1-2503567	Views on Physical channels design – line coding, FEC, CRC, repetition aspects	Samsung

R1-2503618	Discussion on Physical Channel Designs for A-IoT	Panasonic

R1-2503794	Ambient IoT channel coding and small frequency shift	CATT

R1-2503832	Discussion on coding aspects of physical channel design	CMCC

R1-2503883	Discussion on line coding, FEC, CRC and repetition aspects for Ambient IoT	Xiaomi

R1-2503925	Physical layer design – line coding, FEC, CRC, repetition aspects	NEC

R1-2504047	Discussion on physical channels design about line coding, FEC, CRC, repetition aspects for Ambient IoT	China Telecom

R1-2504089	Discussion on coding aspects	Fujitsu

R1-2504099	Discussion on Physical channels design for Ambient IoT-other aspects	HONOR

R1-2504206	Discussion on physical channels design for A-IoT	OPPO

R1-2504244	A-IoT PHY layer design - line coding, FEC, CRC and repetition aspects	LG Electronics

R1-2504261	A-IoT - PHY line coding, FEC, CRC, repetition aspects	MediaTek Inc.

R1-2504288	Remaining issues on coding aspects for Ambient IoT	InterDigital, Inc.

R1-2504319	On remaining coding aspects for Ambient IoT	Apple

R1-2504395	Physical channels design – line coding, FEC, CRC, repetition aspects	Qualcomm Incorporated

R1-2504434	Discussion on coding aspects	Sharp

R1-2504474	Discussion on A-IoT Physical channels design	ASUSTeK

R1-2504502	Discussion on coding and CRC aspects of physical channel design for Ambient IoT	NTT DOCOMO, INC.

R1-2504634	Discussion on other aspects of Phy design for AIoT	IIT Kanpur

### Timing acquisition and synchronization

Including discussions on the necessity/functionalities of preamble/midamble/postamble for R2D and D2R, respectively.

R1-2503222	Timing acquisition and synchronization for Ambient IoT	Ericsson

R1-2503226	Discussion on timing acquisition and synchronization aspects for A-IoT	FUTUREWEI

R1-2503296	Physical signals design	Huawei, HiSilicon

R1-2503301	AIoT Timing acquisition and synchronization	Nokia

R1-2503313	Discussion on Ambient IoT signals	ZTE Corporation, Sanechips

R1-2503360	Remaining issues on Timing acquisition and synchronization for AIoT	vivo

R1-2503420	Discussion on timing acquisition and synchronization	NEC

R1-2503517	Discussion on  timing acquisition and synchronization for Ambient IoT	Spreadtrum, UNISOC

R1-2503538	Discussion on timing acquisition and synchronization functionalities for Ambient IoT	TCL

R1-2503568	Views on timing acquisition and synchronization	Samsung

R1-2503610	Discussion on timing acquisition and synchronization	InterDigital, Inc.

R1-2503660	Discussion on timing acquisition and synchronization for Ambient IoT	Lenovo

R1-2503704	Discussion on timing acquisition and synchronization of A-IoT	Tejas Network Limited

R1-2503715	Discussion on timing acquisition and synchronisation for Ambient IoT	Lekha Wireless Solutions

R1-2503735	Views on Timing acquisition and synchronization	Ofinno

R1-2503795	Ambient IoT Timing and synchronization	CATT

R1-2503833	Discussion on timing acquisition and synchronization	CMCC

R1-2503884	Discussion on timing acquisition and synchronization for Ambient IoT	Xiaomi

R1-2504008	A-IoT Timing acquisition and synchronization	Panasonic

R1-2504048	Discussion on timing acquisition and synchronization for Ambient IoT	China Telecom

R1-2504090	Discussion on timing acquisition and synchronization	Fujitsu

R1-2504111	Discussion on timing acquisition and synchronization for Ambient-IOT	Fraunhofer IIS, Fraunhofer HHI

R1-2504139	Discussion on timing acquisition and synchronization	ETRI

R1-2504207	Discussion on timing acquisition and synchronization for A-IoT	OPPO

R1-2504245	Timing acquisition and synchronization for A-IoT	LG Electronics

R1-2504320	On remaining timing acquisition & synchronization aspects for Ambient IoT	Apple

R1-2504396	Timing acquisition and synchronization	Qualcomm Incorporated

R1-2504435	Discussion on timing acquisition and synchronization	Sharp

R1-2504483	Discussion on Timing acquisition and synchronization for Ambient IoT	Indian Institute of Tech (M)

R1-2504503	Discussion on timing acquisition and synchronization for Ambient IoT	NTT DOCOMO, INC.

R1-2504600	Discussion on timing acquisition and synchronization aspects for Ambient IoT	CEWiT

R1-2504635	Discussion on timing and synchronization aspects for AIoT	IIT Kanpur

R1-2504322	FL Summary#1 on timing acquisition and synchronization for Ambient IoT	Moderator (Apple)

HP Proposal 2-1a

For SIP of R-TAS, SIP pattern with Alt 2-4 is adopted, i.e. ON-OFF-ON-OFF with a ratio of 1:1:1:3

- Note: Detection method of SIP presence at the device is not specified

Agreement

Confirm the working assumption in the following agreement from RAN1#120bis:

Agreement

- For D2R preamble/midamble, base sequence is generated from m-sequence, where the length of the sequence is 2n-1

  - Value(s) of n

    - Long preamble/midamble is generated based on n = 5

    - Working assumption: Short preamble/midamble is generated based on n=3

- Only 1-part preamble/midamble are supported for D2R

- Preamble immediately precedes the PDRCH without any gap

- Both long and short preamble and midamble are supported based on the working assumption on n

  - when midamble is present at least the following cases are supported and reader explicitly indicates one of the following cases for PDRCH:

    - Short preamble and short midamble

    - Long preamble and long midamble

Note: the case of short preamble and long midamble will not be supported

  - When midamble is not present the reader explicitly indicates short or long preamble for PDRCH

Agreement

M = {2,6,12,24} are adopted for CAP and same M value is used for CAP and PRDCH in an R2D transmission.

Proposed Conclusion 4-1

R2D postamble is not specified in Rel-19.

R1-2504323	FL Summary#2 on timing acquisition and synchronization for Ambient IoT	Moderator (Apple)

R1-2504324	FL Summary#3 on timing acquisition and synchronization for Ambient IoT	Moderator (Apple)

### Other aspects incl. multiplexing/multiple access, scheduling information, and timing relationships

Including discussions on L1 control information and any other L1 procedural aspects.

R1-2503223	Other aspects for Ambient IoT	Ericsson

R1-2503227	Multiple access, scheduling and timing aspects for A-IoT	FUTUREWEI

R1-2503297	Multiplexing, scheduling, and other physical-layer procedures	Huawei, HiSilicon

R1-2503302	AIoT Other aspects incl. multiplexing/multiple access, scheduling information, and timing relationships	Nokia

R1-2503314	Discussion on Ambient IoT multiple access and timing	ZTE Corporation, Sanechips

R1-2503361	Remaining issues on other aspects for Rel-19 Ambient IoT	vivo

R1-2503518	Discussion on other aspects for Ambient IoT	Spreadtrum, UNISOC

R1-2503569	Views on aspects including multiplexing/multiple access, scheduling information, and timing relationships	Samsung

R1-2503611	Discussion on multiplexing/multiple access, scheduling information, and timing relationships	InterDigital, Inc.

R1-2503619	Discussion on other aspects of A-IoT	Panasonic

R1-2503661	Discussion on multiple access, scheduling and timing aspects of Ambient IoT	Lenovo

R1-2503705	Discussion on multiple access, scheduling and timing aspects for A-IoT	Tejas Network Limited

R1-2503736	Views on other aspects for AIoT	Ofinno

R1-2503796	Ambient IoT frame structure, system control and resource allocation	CATT

R1-2503834	Discussion on access, scheduling and timing relationships	CMCC

R1-2503885	Discussion on other aspects for Ambient IoT	Xiaomi

R1-2503926	Discussion on control and other aspects of ambient IoT	NEC

R1-2504049	Discussion on other aspects for Ambient IoT	China Telecom

R1-2504064	Multiple access and timing relationships for Ambient IoT	Sony

R1-2504091	Discussion on other aspects of Ambient IoT	Fujitsu

R1-2504100	Discussion on L1 control information and L1 procedural aspects	HONOR

R1-2504140	Discussion on other aspects for Ambient IoT	ETRI

R1-2504208	Discussions on other aspects of A-IoT communication	OPPO

R1-2504246	Other aspects for A-IoT	LG Electronics

R1-2504299	Discussion on multiplexing, multiple access, scheduling information, and timing relationships	Google

R1-2504321	On remaining multiple access, scheduling and control aspects for Ambient IoT	Apple

R1-2504397	Discussion on other aspects for Rel-19 Ambient IoT	Qualcomm Incorporated

R1-2504436	Discussion on other aspects	Sharp

R1-2504475	Discussion on control information and procedural aspects	ASUSTeK

R1-2504504	Discussion on other aspects for Ambient IoT	NTT DOCOMO, INC.

R1-2504542	Discussion on other aspects of Ambient IoT	KT Corp.

R1-2504589	Discussion on multiple access, scheduling information and timing relationships for Ambient IoT	TCL

R1-2504601	Discussion on multiple access, scheduling and timing aspects for Ambient IoT	CEWiT

R1-2504618	Discussion on multiplexing/multiple access and timing relationships for Ambient IoT	WILUS Inc.

R1-2503362	FL summary #1 on other aspects for Rel-19 Ambient IoT	Moderator (vivo)

R1-2503363	FL summary #2 on other aspects for Rel-19 Ambient IoT	Moderator (vivo)

R1-2503364	FL summary #3 on other aspects for Rel-19 Ambient IoT	Moderator (vivo)

FL1 Proposal 3.2.1-1a

Select one option from the following for Toffset1, where Toffset1 is the time interval from the end of the R2D transmission triggering random access to the starting time of the first Msg1 time domain resource for X=1 or X=2, from the device perspective.

- Option 1-1: Toffset1 is predefined in the spec as one fixed value of 30us

  - The fixed value is applicable regardless of the use of FEC.

- Option 1-2: Toffset1 is predefined in the spec as two fixed values

  - The fixed value of 30 us is used when FEC is not used

  - The fixed value of Z us is used when FEC is used (determine the value of Z within RAN1#121)

- Option 2: Toffset1 is derived based on the R2D chip duration TchipR2D and D2R chip duration TchipD2R indicated in the R2D transmission triggering random access

  - Toffset1=maxa×TchipR2D, b×TchipD2R+t1, where a=4, b=20 and t1≥0

  - Determine t1 within this meeting considering at least the FEC impact, if applicable

- Option 3: Toffset1 is indicated explicitly in the R2D transmission triggering random access from a set of predefined values

  - The predefined values are {30, 40, 50, 60}us and determine whether and what additional value(s) is/are needed within this meeting

If the end of the R2D transmission is defined as before the start of padding, the length of padding is additionally added to Toffset1.

Agreement

For Toffset1, where Toffset1 is the time interval from the end of the R2D transmission triggering random access to the starting time of the first Msg1 time domain resource for X=1 or X=2,

- Toffset1 is not smaller than 30 us for CBRA.

FL1 Proposal 3.2.1-2a

Confirm the working assumption for determination of the starting time for the second Msg1 time domain resource for X=2 from device perspective, that is derived based on the starting time of the first Msg1 time domain resource plus Toffset2

FL1 Proposal 3.2.1-2b

Select one option from the following for Toffset2:

- Option 1: Toffset2=αToffset1+βTmsg1, where

  - Tmsg1 is the duration of the 1st Msg1 time domain resource

  - Determine α and β within this meeting

- Option 2: Toffset2 is indicated explicitly in the R2D transmission triggering random access from a set of predefined values

  - The predefined values are {60, 90, 120, 180, 240, 300, 360, 420}us and determine whether and what additional value(s) is/are needed within this meeting