3GPP TSG RAN WG1 #121			R1-240xxxx

St Julian’s, Malta, May 19th – 23th, 2025

Agenda Item:	9.4

Source:	Ad-Hoc Chair (Huawei)

Title:	Session notes for 9.4 Solutions for Ambient IoT in NR

Document for:	Discussion, Decision

## Solutions for Ambient IoT (Internet of Things) in NR

Please refer to RP-250796 for detailed scope of the WI.

[121-R19-A-IoT] Email discussion on Rel-19 Ambient IoT – Jingwen (CMCC)

- To be used for sharing updates on online/offline schedule, details on what is to be discussed in online/offline sessions, tdoc number of the moderator summary for online session, etc

R1-2503830	TP for A-IoT physical layer functions for TS 38.300	CMCC, Huawei, HiSilicon

### Physical channels design – modulation aspects

Including discussions on CP handling for R2D.

R1-2503220	Modulation aspects for Ambient IoT	Ericsson

R1-2503224	Discussion on modulation aspects for A-IoT physical channel	FUTUREWEI

R1-2503294	Physical channels design on modulation	Huawei, HiSilicon

R1-2503299	AIoT Physical channels design - modulation aspects	Nokia

R1-2503311	Discussion on Ambient IoT modulation	ZTE Corporation, Sanechips

R1-2503358	Remaining issues on Modulation Aspects of Physical Channels Design	vivo

R1-2503515	Discussion on modulation aspects of physical channels design for Ambient IoT	Spreadtrum, UNISOC

R1-2503536	Discussion on modulation aspects for Ambient IoT physical design	TCL

R1-2503566	Views on Physical channels design – modulation aspects	Samsung

R1-2503703	Discussion on modulation aspects for A-IoT physical channel	Tejas Network Limited

R1-2503725	Discussion on Physical Channel Design and Modulation Aspects for Ambient-IoT	EURECOM

R1-2503734	Views on Physical channels design – modulation aspects for AIoT	Ofinno

R1-2503793	Ambient IoT physical channel design and modulation	CATT

R1-2503831	Discussion on modulation aspects of physical channel design	CMCC

R1-2503882	Discussion on modulation aspects for Ambient IoT	Xiaomi

R1-2503924	Discussion on modulation aspects of ambient IoT	NEC

R1-2504007	A-IoT Physical Channels Design on Modulation Aspects	Panasonic

R1-2504046	Discussion on physical channels design about modulation aspects for Ambient IoT	China Telecom

R1-2504088	Modulation for R2D	Fujitsu

R1-2504098	Discussion on Physical channels design for Ambient IoT-modulation aspects	HONOR

R1-2504205	Discussion on modulation aspects of A-IoT	OPPO

R1-2504243	A-IoT PHY layer design - waveform and modulation aspects	LG Electronics

R1-2504287	Remaining issues on modulation aspects for Ambient IoT	InterDigital, Inc.

R1-2504318	On remaining modulation aspects for Ambient IoT	Apple

R1-2504394	Physical channels design – modulation aspects	Qualcomm Incorporated

R1-2504433	Discussion on modulation aspects	Sharp

R1-2504482	Discussion on Modulation Aspects for Ambient IoT	Indian Institute of Tech (M)

R1-2504501	Discussion on modulation aspects of physical channel design for Ambient IoT	NTT DOCOMO, INC.

R1-2504585	Discussion on physical channels design for A-IoT	China Unicom

R1-2504617	Discussion on modulation aspects of physical channels design for Ambient IoT	WILUS Inc.

R1-2504633	Discussion on modulation related aspects of AIoT	IIT Kanpur

R1-2504710

Agreement

For R2D, the minimum Btx,R2D # of PRBs associated to each agreed M value (i.e. M = 2, 6, 12 and 24) is specified as per TR38.769.

[Open][H] Proposal 3.2a-v2:

For the further down-select between Option 1 and Option 3 for M=24 on CP handling, update the below agreement on the followings:

Agreement

For further down-selection among CP handing which retains subcarrier orthogonality, at least for PRDCH, at least Method Type 1 is supported

- For supported M values <= 12

  - RAN1 will not further pursue additional CP handling design

- For supported M values > 12

  - RAN1 will further down-select one from the followings

    - Option 1: Candidate 3 of M2-1-1 (as per agreements from RAN1#120)

      - Insert padding chips only at the end OOK chips of OFDM symbol

        - The last 2 out of M OOK chips at the end of an OFDM symbol are always ‘ON’

        - For the OFDM symbol contains CAP, RAN1 down-select one option from the followings in RAN1#121:

          - Option A2: The last 2 out of M OOK chips at the end of the OFDM symbol are always ‘OFF’

          - Option A3: The last 2 out of M OOK chips at the end of the OFDM symbol are always ‘ON’

    - Option 3: RAN1 will not further pursue additional CP handling design

Agreement

For M=24 on CP handling, update the below agreement as follows:

Agreement

For further down-selection among CP handing which retains subcarrier orthogonality, at least for PRDCH, at least Method Type 1 is supported

- For supported M values <= 12

  - RAN1 will not further pursue additional CP handling design

- For supported M values > 12

  - RAN1 will further down-select one from the followings

    - Option 1: Candidate 3 of M2-1-1 (as per agreements from RAN1#120)

      - Insert padding chips only at the end OOK chips of OFDM symbol

        - The last 2 out of M OOK chips at the end of an OFDM symbol are always ‘ON’

        - For the OFDM symbol contains CAP:

          - Option A2: The last 2 out of M OOK chips at the end of the OFDM symbol are always ‘OFF’

          - The last 2 out of M OOK chips at the end of the OFDM symbol are always ‘ON’

    - Option 3: RAN1 will not further pursue additional CP handling design

Agreement

For R2D chip duration, for CP handling that retains sub-carrier orthogonality, the length of a R2D chip duration (denoted as ‘C’) in one OFDM symbol is defined as:

- C = 1/(SCS*M)

- Note: the total length of M OOK chips is equal to 1/SCS

Working assumption

When padding is used in R2D, padding is present right after postamble (if postamble is supported).

[Open][H] Proposal 5.2b-v2:

When the generated number of chips for the R2D transmission does not fully occupy the last OFDM symbol, padding is used.

- Padding is to be down-selected as the following:

  - Alt 1a: The content of padding is up to reader implementation and transparent to device.

    - Note: the timeline determination of any timing relationship refers to the end of padding.

    - Note: it implies the device should be aware of the duration of padding or the last OFDM symbol boundary by implementation. FFS how accurately the device can be aware.

  - Alt 1b: The content of padding is up to reader implementation and transparent to device.

    - The Toffset between R2D and the following D2R shall include the padding duration which depends on the number of padding chips and M values.

    - The TR2D_R2D_min (if any) between R2D transmissions shall include the padding duration which depends on the number of padding chips and M values.

    - Note: the timeline determination of any timing relationship refers to the start of the padding.

  - Alt 2: Specify the detailed content of padding, down-select one option from the followings

    - Option 3: The end chips of the last OFDM symbol including the padding content is [OFF, ON, ON] for M = 24, and [OFF ON] or [ON OFF] for M = 2, 6 and 12. The other chips of padding do not require specification.

    - Option 6: Alternation of zero-chip and one-chip padding

    - Note: the timeline determination of any timing relationship refers to the end of padding.

  - Note: the end chip(s) of the padding content shall follow the CP handling solution determined in RAN1, and may be affected by other agreements.

  - Note: the chip(s) of the padding content shall avoid to result in SIP pattern.

[Open][H] Proposal 5.2b-v2:

When the generated number of chips for the R2D transmission does not fully occupy the last OFDM symbol, padding is used.

- The content of padding is up to reader implementation and transparent to device.

### Physical channels design – line coding, FEC, CRC, repetition aspects

Including discussions on small frequency shift

R1-2503221	Coding aspects for Ambient IoT	Ericsson

R1-2503225	Coding aspects for A-IoT physical channel	FUTUREWEI

R1-2503295	Physical channel design on channel coding	Huawei, HiSilicon

R1-2503300	AIoT Physical channels design - line coding, FEC, CRC, repetition aspects	Nokia

R1-2503312	Discussion on Ambient IoTcoding and SFS	ZTE Corporation, Sanechips

R1-2503359	Remaining issues on line coding, FEC, CRC and repetition for A-IoT	vivo

R1-2503516	Discussion on line coding, FEC, CRC, repetition aspects for Ambient IoT	Spreadtrum, UNISOC

R1-2503537	Discussion on other aspects for Ambient IoT physical design	TCL

R1-2503567	Views on Physical channels design – line coding, FEC, CRC, repetition aspects	Samsung

R1-2503618	Discussion on Physical Channel Designs for A-IoT	Panasonic

R1-2503794	Ambient IoT channel coding and small frequency shift	CATT

R1-2503832	Discussion on coding aspects of physical channel design	CMCC

R1-2503883	Discussion on line coding, FEC, CRC and repetition aspects for Ambient IoT	Xiaomi

R1-2503925	Physical layer design – line coding, FEC, CRC, repetition aspects	NEC

R1-2504047	Discussion on physical channels design about line coding, FEC, CRC, repetition aspects for Ambient IoT	China Telecom

R1-2504089	Discussion on coding aspects	Fujitsu

R1-2504099	Discussion on Physical channels design for Ambient IoT-other aspects	HONOR

R1-2504206	Discussion on physical channels design for A-IoT	OPPO

R1-2504244	A-IoT PHY layer design - line coding, FEC, CRC and repetition aspects	LG Electronics

R1-2504261	A-IoT - PHY line coding, FEC, CRC, repetition aspects	MediaTek Inc.

R1-2504288	Remaining issues on coding aspects for Ambient IoT	InterDigital, Inc.

R1-2504319	On remaining coding aspects for Ambient IoT	Apple

R1-2504395	Physical channels design – line coding, FEC, CRC, repetition aspects	Qualcomm Incorporated

R1-2504434	Discussion on coding aspects	Sharp

R1-2504474	Discussion on A-IoT Physical channels design	ASUSTeK

R1-2504502	Discussion on coding and CRC aspects of physical channel design for Ambient IoT	NTT DOCOMO, INC.

R1-2504634	Discussion on other aspects of Phy design for AIoT	IIT Kanpur

R1-2504749

Agreement

For D2R transmission,

- The minimum Tb is 1.39μs.

- Support at least the following values of Tb, Tchip, and R from the table agreed in RAN1#120bis.

|  | Tchip (μs) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Tb (μs) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 133.33 | 66.67 | 33.33 | 16.67 | 11.11 | 8.33 | 5.56 | 4.17 | 2.78 | 2.08 | 1.39 | 1.04 | 0.69 | 0.52 |
| 266.67 | R=1 | R=2 | R=4 | R=8 | R=12 | R=16 | R=24 | R=32 | R=48 | R=64 | R=96 | R=128 |  | R=256 |
| 133.33 |  | R=1 | R=2 | R=4 | R=6 | R=8 | R=12 | R=16 | R=24 | R=32 | R=48 | R=64 | R=96 | R=128 |
| 66.67 |  |  | R=1 | R=2 |  | R=4 | R=6 | R=8 | R=12 | R=16 | R=24 | R=32 | R=48 | R=64 |
| 33.33 |  |  |  | R=1 |  | R=2 |  | R=4 | R=6 | R=8 | R=12 | R=16 | R=24 | R=32 |
| 22.22 |  |  |  |  | R=1 |  | R=2 |  | R=4 |  | R=8 |  | R=16 |  |
| 16.67 |  |  |  |  |  | R=1 |  | R=2 |  | R=4 | R=6 | R=8 | R=12 | R=16 |
| 11.11 |  |  |  |  |  |  | R=1 |  | R=2 |  | R=4 |  | R=8 |  |
| 8.33 |  |  |  |  |  |  |  | R=1 |  | R=2 |  | R=4 | R=6 | R=8 |
| 5.56 |  |  |  |  |  |  |  |  | R=1 |  | R=2 |  | R=4 |  |
| 4.17 |  |  |  |  |  |  |  |  |  | R=1 |  | R=2 |  | R=4 |
| 2.78 |  |  |  |  |  |  |  |  |  |  | R=1 |  | R=2 |  |
| 2.08 |  |  |  |  |  |  |  |  |  |  |  | R=1 |  | R=2 |
| 1.39 |  |  |  |  |  |  |  |  |  |  |  |  | R=1 |  |
| 1.04 |  |  |  |  |  |  |  |  |  |  |  |  |  | R=1 |

Note: This does not imply the device to pre-store the table.

Agreement

For bit collection after FEC, the output bits for each input bit are arranged sequentially in accordance with the input bits, e.g., for input bits c0,c1,…,cK-1, the output of the FEC is d00,d01,d02,d10,d11,d12,…,dK-10,dK-11,dK-12, with the code rate of 1/3.

Agreement

For attaching the CRC, the parity bits are appended to the end of the input bits, according to the order in TS38.212.

### Timing acquisition and synchronization

Including discussions on the necessity/functionalities of preamble/midamble/postamble for R2D and D2R, respectively.

R1-2503222	Timing acquisition and synchronization for Ambient IoT	Ericsson

R1-2503226	Discussion on timing acquisition and synchronization aspects for A-IoT	FUTUREWEI

R1-2503296	Physical signals design	Huawei, HiSilicon

R1-2503301	AIoT Timing acquisition and synchronization	Nokia

R1-2503313	Discussion on Ambient IoT signals	ZTE Corporation, Sanechips

R1-2503360	Remaining issues on Timing acquisition and synchronization for AIoT	vivo

R1-2503420	Discussion on timing acquisition and synchronization	NEC

R1-2503517	Discussion on  timing acquisition and synchronization for Ambient IoT	Spreadtrum, UNISOC

R1-2503538	Discussion on timing acquisition and synchronization functionalities for Ambient IoT	TCL

R1-2503568	Views on timing acquisition and synchronization	Samsung

R1-2503610	Discussion on timing acquisition and synchronization	InterDigital, Inc.

R1-2503660	Discussion on timing acquisition and synchronization for Ambient IoT	Lenovo

R1-2503704	Discussion on timing acquisition and synchronization of A-IoT	Tejas Network Limited

R1-2503715	Discussion on timing acquisition and synchronisation for Ambient IoT	Lekha Wireless Solutions

R1-2503735	Views on Timing acquisition and synchronization	Ofinno

R1-2503795	Ambient IoT Timing and synchronization	CATT

R1-2503833	Discussion on timing acquisition and synchronization	CMCC

R1-2503884	Discussion on timing acquisition and synchronization for Ambient IoT	Xiaomi

R1-2504008	A-IoT Timing acquisition and synchronization	Panasonic

R1-2504048	Discussion on timing acquisition and synchronization for Ambient IoT	China Telecom

R1-2504090	Discussion on timing acquisition and synchronization	Fujitsu

R1-2504111	Discussion on timing acquisition and synchronization for Ambient-IOT	Fraunhofer IIS, Fraunhofer HHI

R1-2504139	Discussion on timing acquisition and synchronization	ETRI

R1-2504207	Discussion on timing acquisition and synchronization for A-IoT	OPPO

R1-2504245	Timing acquisition and synchronization for A-IoT	LG Electronics

R1-2504320	On remaining timing acquisition & synchronization aspects for Ambient IoT	Apple

R1-2504396	Timing acquisition and synchronization	Qualcomm Incorporated

R1-2504435	Discussion on timing acquisition and synchronization	Sharp

R1-2504483	Discussion on Timing acquisition and synchronization for Ambient IoT	Indian Institute of Tech (M)

R1-2504503	Discussion on timing acquisition and synchronization for Ambient IoT	NTT DOCOMO, INC.

R1-2504600	Discussion on timing acquisition and synchronization aspects for Ambient IoT	CEWiT

R1-2504635	Discussion on timing and synchronization aspects for AIoT	IIT Kanpur

R1-2504322	FL Summary#1 on timing acquisition and synchronization for Ambient IoT	Moderator (Apple)

HP Proposal 2-1a

For SIP of R-TAS, SIP pattern with Alt 2-4 is adopted, i.e. ON-OFF-ON-OFF with a ratio of 1:1:1:3

- Note: Detection method of SIP presence at the device is not specified

Agreement

Confirm the working assumption in the following agreement from RAN1#120bis:

Agreement

- For D2R preamble/midamble, base sequence is generated from m-sequence, where the length of the sequence is 2n-1

  - Value(s) of n

    - Long preamble/midamble is generated based on n = 5

    - Working assumption: Short preamble/midamble is generated based on n=3

- Only 1-part preamble/midamble are supported for D2R

- Preamble immediately precedes the PDRCH without any gap

- Both long and short preamble and midamble are supported based on the working assumption on n

  - when midamble is present at least the following cases are supported and reader explicitly indicates one of the following cases for PDRCH:

    - Short preamble and short midamble

    - Long preamble and long midamble

Note: the case of short preamble and long midamble will not be supported

  - When midamble is not present the reader explicitly indicates short or long preamble for PDRCH

Agreement

M = {2,6,12,24} are adopted for CAP and same M value is used for CAP and PRDCH in an R2D transmission.

Proposed Conclusion 4-1

R2D postamble is not specified in Rel-19.

R1-2504323	FL Summary#2 on timing acquisition and synchronization for Ambient IoT	Moderator (Apple)

Proposal 2-3A

For SIP of R-TAS, following design is adopted (based on combination of Alt 1-2 and Alt 2-4) with following two parts:

- A fixed duration part of 1 OFDM symbol duration with ON-OFF chips and corresponding ratio of 1:3

  - M = 4 for the fixed part chips

- A variable duration part with ON-OFF chips and corresponding ratio of 1:1 and it precedes the OFDM symbol with the fixed duration part

  - Duration of variable part is up to reader implementation, with minimum allowed value of 0 and a maximum allowed value of 1 OFDM symbol

  - It is up to reader’s implementation to align with the start of OFDM symbol boundary for the symbol with variable part

- Note: Detection method of SIP presence at the device is not specified

Agreement

For D2R ambles,

- For n = 3, adopt m-sequence with following:

  - Polynomial: x³ + x² + 1

  - Initial State: Down-select between 010 or 100

  - Resulting Sequence: Down-select between 0 1 0 0 1 1 1 (for 010 initial state) or 1 0 0 1 1 1 0 (for 100 initial state)

- For n = 5, adopt m-sequence with following:

  - Polynomial: x⁵ + x³ + 1

  - Initial State: 01001

  - Resulting Sequence: 0 1 0 0 1 0 0 0 0 1 0 1 0 1 1 1 0 1 1 0 0 0 1 1 1 1 1 0 0 1 1

R1-2504324	FL Summary#3 on timing acquisition and synchronization for Ambient IoT	Moderator (Apple)

### Other aspects incl. multiplexing/multiple access, scheduling information, and timing relationships

Including discussions on L1 control information and any other L1 procedural aspects.

R1-2503223	Other aspects for Ambient IoT	Ericsson

R1-2503227	Multiple access, scheduling and timing aspects for A-IoT	FUTUREWEI

R1-2503297	Multiplexing, scheduling, and other physical-layer procedures	Huawei, HiSilicon

R1-2503302	AIoT Other aspects incl. multiplexing/multiple access, scheduling information, and timing relationships	Nokia

R1-2503314	Discussion on Ambient IoT multiple access and timing	ZTE Corporation, Sanechips

R1-2503361	Remaining issues on other aspects for Rel-19 Ambient IoT	vivo

R1-2503518	Discussion on other aspects for Ambient IoT	Spreadtrum, UNISOC

R1-2503569	Views on aspects including multiplexing/multiple access, scheduling information, and timing relationships	Samsung

R1-2503611	Discussion on multiplexing/multiple access, scheduling information, and timing relationships	InterDigital, Inc.

R1-2503619	Discussion on other aspects of A-IoT	Panasonic

R1-2503661	Discussion on multiple access, scheduling and timing aspects of Ambient IoT	Lenovo

R1-2503705	Discussion on multiple access, scheduling and timing aspects for A-IoT	Tejas Network Limited

R1-2503736	Views on other aspects for AIoT	Ofinno

R1-2503796	Ambient IoT frame structure, system control and resource allocation	CATT

R1-2503834	Discussion on access, scheduling and timing relationships	CMCC

R1-2503885	Discussion on other aspects for Ambient IoT	Xiaomi

R1-2503926	Discussion on control and other aspects of ambient IoT	NEC

R1-2504049	Discussion on other aspects for Ambient IoT	China Telecom

R1-2504064	Multiple access and timing relationships for Ambient IoT	Sony

R1-2504091	Discussion on other aspects of Ambient IoT	Fujitsu

R1-2504100	Discussion on L1 control information and L1 procedural aspects	HONOR

R1-2504140	Discussion on other aspects for Ambient IoT	ETRI

R1-2504208	Discussions on other aspects of A-IoT communication	OPPO

R1-2504246	Other aspects for A-IoT	LG Electronics

R1-2504299	Discussion on multiplexing, multiple access, scheduling information, and timing relationships	Google

R1-2504321	On remaining multiple access, scheduling and control aspects for Ambient IoT	Apple

R1-2504397	Discussion on other aspects for Rel-19 Ambient IoT	Qualcomm Incorporated

R1-2504436	Discussion on other aspects	Sharp

R1-2504475	Discussion on control information and procedural aspects	ASUSTeK

R1-2504504	Discussion on other aspects for Ambient IoT	NTT DOCOMO, INC.

R1-2504542	Discussion on other aspects of Ambient IoT	KT Corp.

R1-2504589	Discussion on multiple access, scheduling information and timing relationships for Ambient IoT	TCL

R1-2504601	Discussion on multiple access, scheduling and timing aspects for Ambient IoT	CEWiT

R1-2504618	Discussion on multiplexing/multiple access and timing relationships for Ambient IoT	WILUS Inc.

R1-2503362	FL summary #1 on other aspects for Rel-19 Ambient IoT	Moderator (vivo)

FL1 Proposal 3.2.1-1a

Select one option from the following for Toffset1, where Toffset1 is the time interval from the end of the R2D transmission triggering random access to the starting time of the first Msg1 time domain resource for X=1 or X=2, from the device perspective.

- Option 1-1: Toffset1 is predefined in the spec as one fixed value of 30us

  - The fixed value is applicable regardless of the use of FEC.

- Option 1-2: Toffset1 is predefined in the spec as two fixed values

  - The fixed value of 30 us is used when FEC is not used

  - The fixed value of Z us is used when FEC is used (determine the value of Z within RAN1#121)

- Option 2: Toffset1 is derived based on the R2D chip duration TchipR2D and D2R chip duration TchipD2R indicated in the R2D transmission triggering random access

  - Toffset1=maxa×TchipR2D, b×TchipD2R+t1, where a=4, b=20 and t1≥0

  - Determine t1 within this meeting considering at least the FEC impact, if applicable

- Option 3: Toffset1 is indicated explicitly in the R2D transmission triggering random access from a set of predefined values

  - The predefined values are {30, 40, 50, 60}us and determine whether and what additional value(s) is/are needed within this meeting

If the end of the R2D transmission is defined as before the start of padding, the length of padding is additionally added to Toffset1.

Agreement

For Toffset1, where Toffset1 is the time interval from the end of the R2D transmission triggering random access to the starting time of the first Msg1 time domain resource for X=1 or X=2,

- Toffset1 is not smaller than 30 us for CBRA.

FL1 Proposal 3.2.1-2a

Confirm the working assumption for determination of the starting time for the second Msg1 time domain resource for X=2 from device perspective, that is derived based on the starting time of the first Msg1 time domain resource plus Toffset2

FL1 Proposal 3.2.1-2b

Select one option from the following for Toffset2:

- Option 1: Toffset2=αToffset1+βTmsg1, where

  - Tmsg1 is the duration of the 1st Msg1 time domain resource

  - Determine α and β within this meeting

- Option 2: Toffset2 is indicated explicitly in the R2D transmission triggering random access from a set of predefined values

  - The predefined values are {60, 90, 120, 180, 240, 300, 360, 420}us and determine whether and what additional value(s) is/are needed within this meeting

R1-2503363	FL summary #2 on other aspects for Rel-19 Ambient IoT	Moderator (vivo)

Agreement

For Toffset1, where Toffset1 is the time interval from the end of the R2D transmission triggering random access to the starting time of the first Msg1 time domain resource for X=1 or X=2, from the device perspective:

- Toffset1 is from a set of predefined values.

  - FFS: the predefined values

    - E.g. values within the range [2666.8, 1333.4, 666.6, 333.4, 222.2, 166.6, 133.34, 111.2, 83.4, 55.6, 44.44, 41.6, 30] us

  - FFS: which values of Toffset1 correspond to which values of the following factors (to be selected with future agreement):

    - R2D chip length, D2R chip length, padding length, whether FEC is applied

Agreement

Toffset2=αToffset1+βTmsg1, where

  - Tmsg1 is the duration of the 1st Msg1 time domain resource

  - α=0.25 and β=1.25

Agreement

Define Toffset3, which is the time interval from the end of a R2D transmission for Msg2 to the starting time of the corresponding Msg3 time domain resource, from the device perspective.

Agreement

Define Toffset4, which is the time interval from the end of a R2D transmission to the starting time of the corresponding D2R time domain resource except for Msg1 and Msg3 transmission, from the device perspective.

Agreement

Confirm the working assumption with following updates

Working assumption

- For indicating the payload size (i.e. TBS-like) for PDRCH transmission with variable size except for Msg1 and Msg3 transmission in CBRA and 1st D2R Message in CFRA:

  - 7 bits for byte-level D2R payload size indication

- Regarding the TBS of Msg3 in CBRA, and 1st D2R Message in CFRA

  - From RAN1 perspective, it is up to other WGs to decide the actual payload value set and how device knows the actual payload size

Agreement

It is up to RAN4 to define the value(s) for TD2R_min.

  - Note: RAN1 expects that the value(s) for TD2R_min will be defined in RAN4 specifications

Agreement

For CBRA, for FDMA of multiple Msg1 transmissions in response to a R2D transmission triggering random access, the bit duration Tb, the sequence length for D2R preamble, the block repetition number, the channel coding information, the sequence length and the interval bits for D2R midamble insertion, if applicable, provided in the paging message are the same for all the FDMed Msg1 transmissions.

Agreement

For CBRA, for TDMA of X=2 Msg1 transmissions in response to a R2D transmission triggering random access, the bit duration Tb, the sequence length for D2R preamble, the block repetition number, the channel coding information, the sequence length and the interval bits for D2R midamble insertion, if applicable, provided in the paging message are the same for all the TDMed Msg1 transmissions.

FL2 Proposal 3.3-3

For the case that the payload size for Msg3 transmissions is the same for all devices responding to a PRDCH for Msg2, down-select one option from the following:

- Option 1: the bit duration Tb, the sequence length for D2R preamble, the block repetition number, the channel coding information, the sequence length and the interval bits for D2R midamble insertion, if applicable, is the same for the FDMed Msg3 transmissions

- Option 2: the bit duration Tb, the sequence length for D2R preamble, the block repetition number, the channel coding information, the sequence length and the interval bits for D2R midamble insertion, if applicable, can be different for the FDMed Msg3 transmissions

R1-2503364	FL summary #3 on other aspects for Rel-19 Ambient IoT	Moderator (vivo)