3GPP TSG RAN WG1 #121	R1-2505083

# St Julian’s, Malta, May 19 – 23, 2025

Agenda Item:	9

Source:	Ericsson

Title:	Editor’s summary on draft CR 38.211 for NR_LPWUS

Document for:	Discussion, Decision

# 1	Introduction

This document is intended to facilitate the review process of the draft CR 38.211 for NR_LPWUS.

# 2	Discussion – first round

Please provide your comments on the latest version of the draft CR on 38.211 available in this folder.

| Company | Comment |
| --- | --- |
| FW | Thanks for the great effort on the draft CR, please find couple of minor comments below. In Section 7.4.4.1.1, the text implies that both sequence root values and number of sequence roots are configured by RRC, but only the sequence root values should be configured. We suggest the following edits for clarity: “The root sequence number q is obtained as entry cmP∈{0,1} of the Nroot∈1, 2 root sequences numbers configured by the higher-layer parameter XXX and the cyclic shift ncs is given by ncs=cm mod PNZCP P=NseqNroot where - Nseq is the number of sequences configured by the higher-layer parameter XXX - Nrootϵ1,2 is the number of root sequence numbers configured by the higher-layer parameter XXX” Further, also in Section 7.4.4.1.1, the equation to derive the sequence number cm only applies to the case when L>1, so we suggest the following edit: “The sequence number cm is 0 when L=1 is configured. Otherwise, cm is given by” |
| Huawei, HiSilicon | Thank editor for the draft CR and your great efforts. Comment#1 In our understanding, all editor notes in the draft CR will be deleted before the coming CR approval of RAN plenary. Regarding the DC centering issue of ZC sequences, the current equations are not in line with the agreement because the time-domain sequences are not ZC sequence any more and causes unnecessary correlation property loss for time-domain correlation processing of low power receiver. As a neutral solution, could you please add a parameter k0 as below and make such parameter open? Or could you please share any better plan? Agreement For overlaid OFDM sequence(s) for LP-WUS in time or frequency domain, down-selection from the following: - Option 1-1: overlaid sequence(s) are the sequence(s) of an OOK on symbol before DFT/LS processing - The length of overlaid sequence(s) depends on the number of REs used for LP-WUS and the value of M - Option 1-2: overlaid sequence(s) are the sequence(s) of an OFDM symbol before DFT/LS processing - The length of overlaid sequence(s) depends on the number of REs used for LP-WUS - Option 2: overlaid sequence(s) are the sequence(s) of an OFDM symbol before IFFT processing - The length of overlaid sequence(s) depends on the number of REs used for LP-WUS - Option 3: overlaid sequence(s) are the sequence(s) of an OOK on symbol in time domain after IFFT processing - The length of overlaid sequence(s) depends on IFFT size and the value of M FFS: same or different options are applied for OOK-1 and OOK-4 M>1. Working Assumption For overlaid OFDM sequences for LP-WUS in time or frequency domain, for OOK-1 and OOK-4 M=1 - Option 1-1(compromised): overlaid sequence(s) are the sequence(s) of an OOK on symbol before DFT/LS processing for OOK-4 with M=1. - ZC sequence in time domainDFT/LSIFFT to result in a frequency domain sequence that is ZC sequence or a sequence that is close to ZC sequence, no additional operation - It doesn’t preclude additional operation for OOK-4 with M>1. - DFT size is 2^n Note1: OOK-1 is considered as specific case of OOK-4 with M=1. Note: Concerns were raised by Ericsson and CATT that the above is not beneficial in terms of implementation complexity Suggested changes: ##### 7.4.4.1.2 Generation of rWUS(n) The block of complex-valued symbols rWUS(0),…,rWUS(MbitMZC-1) is defined by rWUSlNscWUS+k=1NscWUSi=0NscWUS-1rWUSlNscWUS+ie-j2πik-k0kNscWUS k=0,1,…,NscWUS-1 l=0,1,…,MbitMWUS-1 NscWUS=11NscRB …. ##### 7.4.5.1.3 Generation of rLPSS(n) The block of complex-valued symbols rLPSS(0),…,rLPSS(NOOKMZC-1) is defined by rLPSSlNscWUS+k=1NscWUSi=0NscWUS-1rLPSSlNscWUS+ie-j2πik-k0kNscWUS k=0,1,…,NscWUS-1 l=0,1,…,NOOKMLPSS-1 NscWUS=11NscRB |
| Ericsson1 | On the change suggested by Huawei/HiSilicon, adding an undefined offset k0 at this stage is not preferable to us. We prefer to keep current text. The DFT shift issue was discussed in FL summary in previous meeting (R1-2504908, section 4.5) but there was no opportunity to discuss online and no agreement was made to introduce DFT shift (we are open to discuss the issue during maintenance if proponents see the need). |

Editor’s response:

- I have clarified as suggested by Futurewei

- On DC centering it seems that RAN1 has not yet concluded whether to include that feature or not. For the time being I will leave the equation as is (we typically do not add a lot of unspecified variables just in case 3GPP might want to take a decision in the future; there are also other open issues in RAN1 on LPWUS), but the intention of the draft CR is not to state any preference in either direction. We can update the specification in the maintenance phase should RAN1 conclude on supporting DC centering. I think it should be clear from e.g. this summary that the question is still on the table and that RAN1 is not at all ‘prohibited’ from discussing DC centering.

# 3	Discussion – second round

Please provide your comments on the latest version of the draft CR on 36.211 available in this folder.

| Company | Comment |
| --- | --- |
| vivo | Thank editor for the great effort on the CR. Just one comment on a minor typo in section 7.4.4.1.1 as shown below. The root sequence number q is obtained as entry cmP∈0,1 of the root sequence numberss configured by the higher-layer parameter XXX and the cyclic shift ncs is given by… Regarding DFT operation, we share same view with E/// and editor, and we support the version provided by editor. |
| ZTE, Sanechips | Thanks for the draft CR and above discussion. Some minor comments are as following - Some typos numberss-->numbers; quantiy-->quantity - clarify the meaning of parameter L In our understanding, L and Nseq should have the same meaning, and they also should share the same parameter XXX. If so, we may not need to define two separate parameters. - the meaning of parameter q - in 7.4.4.1.1 The parameter q should be clarified whether the range is 1~NZC-1 - in 7.4.4.1.1 In other places as shown in the above and following, q refers to a root value for ZC sequences. But in this paragraph, q refers to an index for two root values. To avoid any confusion since the parameter q is used in other places and have different meaning as this one, we’d like to suggest to remove q like following in 7.4.4.1.1 The root sequence number q is obtained as entry cmP∈0,1 of the root sequence numberss configured by the higher-layer parameter XXX and the cyclic shift ncs is given by. - in 7.4.5.1.2 In this case, if q=0, xq(i)=1. I guess there is no RAN1 agreement indicating that and it is still under discussion. if there is no overlaid sequence, we need to further clarify how to deal with it. Therefore, we’d like to suggest to remove q=0 firstly and further discuss the case when overlaid sequence is not configured. - DFT shift Regarding the DFT shift issue, we’d like to further discuss. We agree with the intention from Huawei, but k0 is a little bit aggressive at this moment. Further discussion is needed. |

Editor’s response:

Thanks for the comments. I have generally addressed them in the latest draft, except the comment on L vs N_seq mentioned by ZTE. I think the comment is correct, but I would not like to destabilize the draft CR in the last minute and hence leave it for the maintenance phase.