3GPP TSG RAN WG1 #122			      R1-2505437

Bengaluru, India, Aug 25th – 29th, 2025

Agenda item:	8.11.1

Source:	Xiaomi

Title:	Remaining issues on NR-NTN downlink coverage enhancement

Document for:	Discussion and Decision

- Introduction

Agreement

The enabling/disabling of SIB1 PDSCH repetition is implicitly indicated by the enabling/disabling of Type-0 CSS PDCCH repetition.

Agreement

For the activation/deactivation of Msg4 PDSCH repetition:

- Alt 1: UE specific PDSCH with Msg4 repetition activation indicated via DCI Format 1_0:

  - Signaling uses re-interpretation of 1 MSB in MCS field in DCI.

  - A UE capable of Msg4 PDSCH repetition may report its capability/request in Msg3 PUSCH.

    - Note: RAN1 considers there is no difference between capability and request

    - FFS: whether to specify condition(s) for the UE to report its capability/request. Such conditions may be discussed in RAN1 or other WGs.

  - The aggregation factor is configured in SIB1, with possible value 2 or 4

    - When the aggregation factor is configured in SIB1, the PDSCH MSG4 repetition is enabled.

Agreement

The draft LS in R1-2504935 is endorsed with the following revision:

- RAN1 has agreed to support for Msg4 PDSCH repetition

Final LS is approved in R1-2504936.

Agreement

For PDCCH repetition for Type0 PDCCH CSS of searchSpaceZero configured within MIB pdcch-ConfigSIB1, the solution agreed for M = 2 is also applied to M = 1 and M = 1/2.

Agreement

Revise the RAN1#120bis agreement as follows:

Agreement

For PDCCH repetition for Type0 PDCCH CSS of searchSpaceZero configured within MIB pdcch-ConfigSIB1:

- Enabling/disabling using a reserved bit(s) (i.e aA+7) in PBCH payload

No UE behavior is defined for UE in connected mode specifically for the case where the network changes its signaling between enabling and disabling PDCCH repetition for Type0 PDCCH CSS.

In this contribution, we provide our consideration on 6GR channel coding focusing on design target and candidate 6GR coding schemes, evaluation metrics and methodologies.

Agreement

For PDCCH CSS other than Type-0 CSS and other than Type-3 CSS for common search spaces other than SearchSpaceZero, support intra-slot repetition based

Agreement

For PDCCH CSS other than Type-0 CSS and other than Type-3 CSS for common search spaces other than SearchSpaceZero, support intra-slot repetition based on:

- The starting symbol of monitoring occasion of the second SS is located right after the ending symbol of monitoring occasion of the first SS.

- BD is counted as one or two, subject to UE capability, in RRC connected mode

  - UE assumes that a DCI Format with the same content is repeated on two PDCCH candidates.

  - Note: From RAN1 perspective UE is expected to deliver performance no worse than soft combining

- PDCCH repetition is applicable to RNTI of the CSS.

- Repeated PDCCH candidates within the same CORESET repeated in the slot, and share the same aggregation level (AL), coded bits and same candidate index.

  - Up to editor how to capture this in writing the relevant RAN1 specification.

Working assumption

Inter-slot Type-0 CSS PDCCH repetition is only applicable to the SI-RNTI, and the following rule for BD counting is defined:

- 1 BD in first slot.

- In the second slot: 2 BD in RRC connected mode

  - One BD for Type-0 CSS PDCCH repetition with SI-RNTI and one BD for other PDCCH

Conclusion

It can be discussed in UE feature session whether a dedicated PDSCH repetition capability (e.g. FG5-17a and FG16-2b-5) is a pre-requisite UE feature for Msg4 PDSCH repetition.

In RAN1#121 meeting, the aforementioned agreements were made and the core part for Rel-19 DL coverage enhancement was accomplished. In this paper, we provide some further views on some remaining issue and provide corresponding TPs as per guidelines from meeting agenda.

- Discussion

  - BD counting for inter-slot type-0 CSS PDCCH repetition

The working assumption made for inter-slot Type-0 CSS PDCCH repetition means for type 0 PDCCH repetition implies UE counts two BDs covering both SI-RNTI scrambling CSS PDCCH and C-RNTI scrambled USS PDCCH. Since 2DB is needed, our understanding is the repetition for USS PDCCH using search space 0 is not supported. This working assumption is aligned with WID and overall discussion across Rel-19, thus we propose to confirm the working assumption.

Proposal 1. Confirm the below WA on BD counting

Working assumption

Inter-slot Type-0 CSS PDCCH repetition is only applicable to the SI-RNTI, and the following rule for BD counting is defined:

- 1 BD in first slot.

- In the second slot: 2 BD in RRC connected mode

  - One BD for Type-0 CSS PDCCH repetition with SI-RNTI and one BD for other PDCCH

Moreover, as per RAN plenary decision, the agreed CSS repetition mechanism is also agreed to be extended to TN. Consequently, the following TP to TS 38.213 is needed to reflect the confirmed WA.

*** TS 38.213 10.1*** Denote by MScss(i)(L), 0≤i<Icss, the number of counted PDCCH candidates for monitoring for CSS set Scss(i) and by MSuss(j)(L), 0≤j<Juss, the number of counted PDCCH candidates for monitoring for search space set Suss(j). If a UE indicates numBD-twoPDCCH-r17 with value of 3 and is provided searchSpaceLinkingId with same value for search space sets sj and si, with si<sj, set MScss(j)(L)=2∙MScss(j)(L) if si and sj are CSS sets or set MSuss(j)(L)=2∙MSuss(j)(L) if si and sj are USS sets. For FR1, if aA+7 in PBCH payload has value 1, set MScss(j)(L)=2∙MScss(j)(L) in the second slot where the UE monitors the repeated PDDCH candidates. *** TS 38.213 10.1***

Proposal 2. Adopt the following TP on BD counting for inter-slot type-0 CSS PDCCH repetition

*** TS 38.213 10.1***

Denote by MScss(i)(L), 0≤i<Icss, the number of counted PDCCH candidates for monitoring for CSS set Scss(i) and by MSuss(j)(L), 0≤j<Juss, the number of counted PDCCH candidates for monitoring for search space set Suss(j). If a UE indicates numBD-twoPDCCH-r17 with value of 3 and is provided searchSpaceLinkingId with same value for search space sets sj and si, with si<sj, set MScss(j)(L)=2∙MScss(j)(L) if si and sj are CSS sets or set MSuss(j)(L)=2∙MSuss(j)(L) if si and sj are USS sets. For FR1, if  aA+7 in PBCH payload has value 1, set MScss(j)(L)=2∙MScss(j)(L) in the second slot where the UE monitors the repeated PDDCH candidates.

*** TS 38.213 10.1***

  - Intra-slot repetition for other than Type-0 CSS and Type-3 CSS

Our understanding of the previous agreement on intra-slot repetition for other than type-0 and type-3 CSS is editor will capture in the specification whether the search space is configured as a same search space with symbol level offset or two different search spaces coinciding in the same slot with different searchSpaceLinkingId similar to mTRP feature.

However, the BD part to our understanding does not have any controversies and thus we provide a TP to reflect the agreed bullet on BD counting based on UE capability, which is the same framework as mTRP case.

We provide hereafter two possible TPs corresponding to the two alternatives of defining search spaces respectively where Alt 1 refers to two different search space definition and Alt 2 refers to a single search space definition.

Alt 1: Two different search spaces

*** TS 38.213 10.1***

Denote by MScss(i)(L), 0≤i<Icss, the number of counted PDCCH candidates for monitoring for CSS set Scss(i) and by MSuss(j)(L), 0≤j<Juss, the number of counted PDCCH candidates for monitoring for search space set Suss(j). If a UE indicates numBD-twoPDCCH-r19 with value of 1 and is provided searchSpaceLinkingId with same value for search space sets sj and si, with si<sj, set MScss(i)(L)=0. If a UE indicates numBD-twoPDCCH-r17 with value of 3 and is provided searchSpaceLinkingId with same value for search space sets sj and si, with si<sj, set MScss(j)(L)=2∙MScss(j)(L) if si and sj are CSS sets or set MSuss(j)(L)=2∙MSuss(j)(L) if si and sj are USS sets.

*** TS 38.213 10.1***

Alt 2: A same search space

*** TS 38.213 10.1***

Denote by MScss(i)(L), 0≤i<Icss, the number of counted PDCCH candidates for monitoring for CSS set Scss(i) and by MSuss(j)(L), 0≤j<Juss, the number of counted PDCCH candidates for monitoring for search space set Suss(j). If a UE indicates numBD-twoPDCCH-r19 with value of 2 and is monitoring a search space configured with symbol offset, with si<sj, set MScss(i)(L)=2. If a UE indicates numBD-twoPDCCH-r17 with value of 3 and is provided searchSpaceLinkingId with same value for search space sets sj and si, with si<sj, set MScss(j)(L)=2∙MScss(j)(L) if si and sj are CSS sets or set MSuss(j)(L)=2∙MSuss(j)(L) if si and sj are USS sets.

*** TS 38.213 10.1***

Since our position is to re-use the Rel-17 mTRP mechanism to define intra-slot PDCCH repetitions, we propose the following.

Proposal 3. Adopt the following TP on BD counting for intra-slot other than Type-0 CSS and Type-3 CSS

*** TS 38.213 10.1***

Denote by MScss(i)(L), 0≤i<Icss, the number of counted PDCCH candidates for monitoring for CSS set Scss(i) and by MSuss(j)(L), 0≤j<Juss, the number of counted PDCCH candidates for monitoring for search space set Suss(j). If a UE indicates numBD-twoPDCCH-r19 with value of 1 and is provided searchSpaceLinkingId with same value for search space sets sj and si, with si<sj, set MScss(i)(L)=0. If a UE indicates numBD-twoPDCCH-r17 with value of 3 and is provided searchSpaceLinkingId with same value for search space sets sj and si, with si<sj, set MScss(j)(L)=2∙MScss(j)(L) if si and sj are CSS sets or set MSuss(j)(L)=2∙MSuss(j)(L) if si and sj are USS sets.

*** TS 38.213 10.1***

- Conclusions

In this contribution, we provide the following proposals.

Proposal 1. Confirm the below WA on BD counting

Working assumption

Inter-slot Type-0 CSS PDCCH repetition is only applicable to the SI-RNTI, and the following rule for BD counting is defined:

- 1 BD in first slot.

- In the second slot: 2 BD in RRC connected mode

  - One BD for Type-0 CSS PDCCH repetition with SI-RNTI and one BD for other PDCCH

Proposal 2. Adopt the following TP on BD counting for inter-slot type-0 CSS PDCCH repetition

*** TS 38.213 10.1***

Denote by MScss(i)(L), 0≤i<Icss, the number of counted PDCCH candidates for monitoring for CSS set Scss(i) and by MSuss(j)(L), 0≤j<Juss, the number of counted PDCCH candidates for monitoring for search space set Suss(j). If a UE indicates numBD-twoPDCCH-r17 with value of 3 and is provided searchSpaceLinkingId with same value for search space sets sj and si, with si<sj, set MScss(j)(L)=2∙MScss(j)(L) if si and sj are CSS sets or set MSuss(j)(L)=2∙MSuss(j)(L) if si and sj are USS sets. For FR1, if  aA+7 in PBCH payload has value 1, set MScss(j)(L)=2∙MScss(j)(L) in the second slot where the UE monitors the repeated PDDCH candidates.

*** TS 38.213 10.1***

Proposal 3. Adopt the following TP on BD counting for intra-slot other than Type-0 CSS and Type-3 CSS

*** TS 38.213 10.1***

Denote by MScss(i)(L), 0≤i<Icss, the number of counted PDCCH candidates for monitoring for CSS set Scss(i) and by MSuss(j)(L), 0≤j<Juss, the number of counted PDCCH candidates for monitoring for search space set Suss(j). If a UE indicates numBD-twoPDCCH-r19 with value of 1 and is provided searchSpaceLinkingId with same value for search space sets sj and si, with si<sj, set MScss(i)(L)=0. If a UE indicates numBD-twoPDCCH-r17 with value of 3 and is provided searchSpaceLinkingId with same value for search space sets sj and si, with si<sj, set MScss(j)(L)=2∙MScss(j)(L) if si and sj are CSS sets or set MSuss(j)(L)=2∙MSuss(j)(L) if si and sj are USS sets.

*** TS 38.213 10.1***

References

  - RAN1#121 Chair’s notes