3GPP TSG RAN WG1 Meeting #122                                                                             R1-2505477

Bengaluru, India, August 25th – 29rd, 2025

Source:	TCL

Title:	Discussion  on inference related aspects for CSI compression

Agenda Item:	10.1.1.1

Document for:	Discussion and Decision

- Introduction

CSI compression is a sub-use case of AI/ML for CSI feedback enhancement. Its architecture comprises a two-sided AI/ML model, featuring an encoder implemented on the UE side and a decoder on the NW side. As a study item In Release 19, CSI compression shows its capability to optimize the balance between performance and complexity/overhead.

Although the AI/ML CSI compression exhibits significant improvement on overhead and throughput, there are several remaining issues that includes training collaboration types, two-sided model pairing mechanisms, performance monitoring procedures, intermediate KPIs calculation, CSI quantization, and resource/report configuration for model inference, etc.

In Release 20, CSI compression remains as a work item to provide specification support for the CSI feedback enhancement (two-sided model) use case. It seeks to address and resolve a couple of challenges in line with the objectives of the new WID on AI/ML for the NR air interface proposed in RAN#109 [1]. The following areas will be studied as part of this project:

CSI feedback enhancement, encompassing (two-sided model) [RAN1, RAN2]: - CSI spatial/frequency compression without temporal aspects (“Case 0”), - Specify necessary signalling/mechanism(s) as per the identified potential specification impacts in FS_NR_AIML_Air , including: - Model pairing procedure including ID and applicability reporting - Inference aspects including target CSI type, measurement and report configuration, CQI RI determination, payload determination, quantization configuration codebook, UCI mapping, CSI processing criteria and timeline, priority rules for CSI reports - NW and UE side data collection for training, - Target CSI format - Note The framework defined in BM and CSI prediction use cases could be reused - Specify performance monitoring

This contribution aims to provide our perspectives and solutions on these open issues of the work item concerning the CSI compression from the RAN1 viewpoint.

- Specification support for CSI compression model inference

The execution flow for the CSI compression based on two-sided model is shown in Figure 1.

Figure 1 CSI compression procedures

In the figure, w represents the measured CSI information; w represents the recovered CSI information, which is the output of the decoderdecoder. Therefore, the degree of similarity between w and w determines the system performance of AI/ML-based CSI compression. In conventional CSI compression, w is obtained from direct CSI measurement. However, for overhead reduction purpose we may further utilize the time domain correlation of CSI to improve the CSI compression efficiency.

2.1.3 Specification Impact on UCI loss

Conclusion Under 10% UCI loss, Case 2 shows small to large performance drop (by Comparing Scenario B and Scenario A). Signaling support to mitigate the UCI loss can improve the performance and helps preserving the gain of Case 2 over Case 0 (by Comparing Scenario C and Scenario A and benchmark schemes). Agreement Investigate the following approaches for signaling support for mitigating the impact of UCI loss - NW-signaling to reset of historical CSI information at UE - NW-triggered CSI retransmission

The retransmission of compressed CSI can be configured in the form of event triggering. Since the UCI loss is the main cause of CSI recovery failure problem and it is episodic and cannot be predicted, it can be used as a condition for triggering the gNB to trigger the reset or retransmission behavior at the UE side.

Since the UCI resources are configured by the gNB, if the gNB does not receive the reported CSI data on the specified time slots and RE resources or fails to parse the CSI data, it can be determined that a UCI loss event has occurred. Once a UCI loss event occurs, actually at this time the gNB cannot get any update of CSI information on the current time slot, in consequence the following two actions are required:

- Firstly, due to the lack of CSI information on the current time slot, the gNB can only choose to use the CSI at the moment t-1 for the pre-coding of data in the subsequent PDSCH/PDCCH transmission.

- Secondly, since the UE side CSI has already updated, while the gNB cannot, the gNB needs to trigger the cache CSI synchronization mechanism. At this time, the gNB can choose to send a RRC signaling of CSI synchronization, where the CSI synchronization can be either a signaling to reset the CSI information or a trigger of CSI retransmission.

Proposal 1: Study the triggering mechanism based on UCI loss event for CSI information reset or retransmission.

- Option 1: UCI loss event is determined by NW

- Option 2: UCI loss event is determined by UE

Moreover, the UCI loss often occurs when the channel condition experience significant change, that means the historical CSI information may be out-of-date. We may add more options on how to reset the CSI information when facing UCI loss.

Proposal 2: Investigate the CSI information synchronization for mitigating the impact of UCI loss.

- Option 1: Reset the CSI information to pre-defined values.

- Option 2: NW configs or UE initials CSI information retransmission.

Moreover, the UCI loss often occurs when the channel condition experience significant change, that means the historical CSI information may be out-of-date. We may add more options on how to reset the CSI information when facing UCI loss.

2.1 Resource Set (RS) and CSI Report Configuration

The following consensus was reached regarding the CSI-RS resource and CSI report configuration for the CSI compression sub-use case [3][4]:

Agreement in RAN1#112bis-e The study of AI/ML based CSI compression should be based on the legacy CSI feedback signaling framework. Further study potential specification enhancement on - CSI-RS configurations (No discussion on CSI-RS pattern design enhancements) - CSI reporting configurations - CSI report UCI mapping/priority/omission - CSI processing procedures. - Other aspects are not precluded. Agreement in RAN1#113 In CSI compression using two-sided model use case, further study the applicability and potential specification impact for CSI configuration and report: - For network to indicate CSI reporting related information, gNB can indicate the UE with the one or more of following information: - Information indicating CSI payload size - Information indicating quantization method/granularity. - Rank restriction - Other payload related aspects - For UE determination/reporting of the actual CSI payload size, UE reports related information as configured by the NW Agreement In Options 3a-1 and 4-1, the exchanged dataset or the model parameters can be associated with an ID for pairing related discussion, then - The same ID can be used for UE to collect UE-side target CSI for UE-side training - The same ID can be used for applicability inquiry and reporting - The same ID can be used for inference configuration - FFS: whether ID/even same ID is needed for monitoring configuration - FFS: where the ID is assigned - Note: whether the purpose for pair will be specified will be discussed separately.

The CSI framework, as established in NR, encompasses the setting of CSI-RS resources and report settings, as well as the configuration of the UE with CSI-ReportConfig via RRC signaling. In the use case of CSI compression using a two-sided model, three types of CSI-RS resources associated with the reportQuantity are defined for CSI measurement. These include channel measurement with the first CSI-RS resource configuration, interference measurement with the second CSI-IM resource configuration, and another form of interference executed by the NZP-CSI-RS resource configuration. The information required by the AI/ML model would significantly differ from the traditional CSI framework. Moreover, the conventional CSI-RS resource configuration methodology has not defined the AI or other usage scenarios. Therefore, the method of pairing the AI/ML model with the reference signal configuration needs to be elucidated.

To cater to the specific feedback requirements of the AI/ML model, several methods can be employed to differentiate the AI/ML CSI compression requirement:

- A dedicated reporting quantity has been defined, which is utilized for the operation of the AI/ML-specific model.

- A dedicated reporting configuration is in place, which incorporates information specific to the AI/ML model.

- A dedicated measurement configuration is established, which includes information specific to the AI/ML model.

In prior meetings, several proposals were put forth suggesting that the UE employs multiple AI/ML models to compress the CSI information. For each AI/ML model used for CSI compression, the corresponding CSI-RS resource configuration and CSI reporting configuration would be clarified to align the UE with LCM operations.

Proposal 3: In the case of CSI compression using a two-sided model, the design of an AI/ML-specific CSI-RS resource and CSI reporting configuration that may be compatible with the traditional CSI reporting scenario should be considered in the following aspects:

- AI/ML-specific CSI-RS resource configuration for CSI compression

- AI/ML-specific fields in CSI-ReportConfig IE

- Dedicated report quantities and report configurations for AI

2.2 Priority rules concerning CSI report collision

The following agreements were reached on the priority rules pertaining to CSI report collision for the CSI compression sub-use case:

Agreement In CSI compression using two-sided model use case, further study the feasibility and methods to support the legacy CSI reporting principles including at least: - The priority rule regarding CSI collision handling and CSI omission - Codebook subset restriction - CSI processing Unit Agreement The study of AI/ML based CSI compression should be based on the legacy CSI feedback signaling framework. Further study potential specification enhancement on - CSI-RS configurations (No discussion on CSI-RS pattern design enhancements) - CSI reporting configurations - CSI report UCI mapping/priority/omission - CSI processing procedures. - Other aspects are not precluded.

At times, a UE device may be triggered with multiple CSI reports based on the CSI report configurations. In such instances, it becomes necessary for the UE device to ascertain the priority of these multiple CSI reports. This is to determine whether some CSI reports or portions of the CSI report part 2 should be disregarded when the uplink symbols/slots for multiple CSI reports collide in the temporal domain. In other words, when the UE device is triggered with multiple AI/ML-based CSI reports and non-AI/ML CSI reports, the priority for such AI/ML-based and non-AI/ML-based CSI reports also needs to be established.

In the NR system, a priority rule for CSI reporting has been established, which is associated with multiple factors such as temporal domain behavior (periodic/semi-persistent/aperiodic), CSI report content (whether or not carrying L1-RSRP or L1-SINR), serving cell index, reporting config ID, and so on. Two CSI reports are considered to be in collision if the time occupancy of the physical channels scheduled to carry CSI reports overlaps in at least one OFDM symbol and is transmitted on the same carrier. When a UE is configured to transmit two colliding CSI reports to the gNB, the priority rule will be applied as follows:

- If the y values differ between the two colliding CSI reports, the following rules apply, except in the case where one of the y values is 2 and the other is 3. Here, the parameter y denotes different types of CSI reports, such as y=0 indicating aperiodic CSI reports to be carried on PUSCH, y=1 indicating semi-persistent CSI reports to be carried on PUSCH, y=2 indicating semi-persistent CSI reports to be carried on PUCCH, and y=3 indicating periodic CSI reports to be carried on PUCCH.

- The CSI report with the higher priority value should not be transmitted by the UE.

- Alternatively, the two CSI reports are either multiplexed or one is dropped, depending on the priority values, as outlined in [TS38.213].

At present, AI/ML-based CSI compression has been deliberated multiple times in RAN1/RAN2. For AI/ML models to function properly, dedicated CSI reporting, including AI/ML model life cycle management (LCM), is required. Both non-AI/ML based CSI compression and AI/ML-based CSI compression or related data collection would coexist for a specific radio link. Given the constraints on UCI payload, a new priority rule might be necessary to manage AI/ML and/or non-AI/ML based CSI compression. Consequently, an amendment to the existing CSI reporting priority rule is anticipated.

The establishment of an AI/ML-specific priority rule is essential within the system, distinct from the non-AI/ML specific CSI reporting priority rule currently employed in the NR system. In addition to the factors influencing the value of legacy CSI reporting priority, certain supplementary information may be taken into account as determinants for the value of AI/ML-specific CSI reporting priority. These factors include:

- AI/ML LCM procedures, signifying that CSI reporting associated with different procedures possess varying priority values, such as model monitoring, inference, and training.

- AI/ML characteristics for CSI compression, indicating that CSI reporting associated with different features carries different priority values, such as the model structure for CSI compression and quantization.

- AI/ML CSI reporting contents, suggesting that different CSI reporting information holds varying priority values, such as Rank and CQI, and the codebook subset.

Observation 1: Certain additional information may impact the priority of AI/ML CSI reporting for CSI compression:

- AI/ML LCM procedure

- AI/ML characteristics for CSI compression

- AI/ML reporting contents

- Other factors that may be pertinent to the value of AI/ML-specific priority should not be precluded.

Proposal 4: The definition of AI/ML-specific priority for CSI reporting in relation to CSI compression should be considered in comparison with the traditional CSI priority rules.

Given that both AI/ML CSI reporting and non-AI/ML CSI reporting for CSI compression are likely to coexist in the NR system, when an uplink resource collision occurs, the final priority between AI/ML-based and non-AI/ML-based should be determined. The approach to this determination is as follows:

- Option 1: The priority of AI/ML CSI is always higher than that of non-AI/ML CSI reporting for CSI compression. This implies that non-AI/ML can only be transmitted after the AI/ML CSI, and the CSI priority is determined by the specific priority value within either AI/ML CSI reporting or non-AI/ML CSI reporting.

- Option 2: The priority of AI/ML CSI is always lower than that of non-AI/ML CSI reporting for CSI compression. This implies that AI/ML can only be transmitted after the non-AI/ML CSI, and the CSI priority is determined by the specific priority value within either AI/ML CSI reporting or non-AI/ML CSI reporting.

- Option 3: If both AI/ML CSI and non-AI/ML CSI share the same priority value, the AI/ML CSI will be transmitted first. However, if the priority values differ, the transmission priority is determined by the respective priority value.

- Option 4: If both AI/ML CSI and non-AI/ML CSI share the same priority value, the non-AI/ML CSI will be transmitted first. However, if the priority values differ, the transmission priority is determined by the respective priority value.

Proposal 5: When the UE supports both AI/ML and non-AI/ML CSI reporting, it is necessary to redefine the priority rule considering different types of CSI reporting.

2.3 Mechanism for CSI Compression Processing with Limited UE Capability and Dedicated Timing

For traditional CSI measurement processing criteria, the concept of a CSI Processing Unit (CPU) is introduced to denote the capability of supported simultaneous CSI calculations implemented in a UE. This means that if a UE supports simultaneous CSI calculations, it is said to possess CSI processing units for processing CSI reports. Similarly, for AI/ML models implementing CSI measurement operations, the concept of an AI/ML-specific CPU can also be introduced. However, the requirements for an AI/ML-specific CPU differ significantly from those of a legacy CPU due to the distinct requirements of UE capabilities to implement AI/ML-specific and traditional CSI measurement operations. When AI/ML models are applied to the CSI measurement process for CSI prediction, channel information feedback, beam management, and positioning, the resources (including computing units, memory, storage, and other hardware resources) consumed by AI/ML models differ from those required by traditional CSI computation methods. Consequently, the current CPU and Occupied CPU quantified based on non-AI/ML models are unsuitable for AI/ML models. It is essential to redefine the relevant parameters specifically for AI/ML models.

The utilization of AI/ML models for CSI measurements and calculations may pose challenges to the capabilities of a UE to implement AI/ML models for inference. Firstly, the size of AI/ML models may be substantial due to the complex AI/ML model structures and a large number of AI/ML proprietary parameters, including hyperparameters, learnable parameters, and other types of parameters specific to an AI/ML model. Consequently, to facilitate the running of AI/ML models for inference in a UE, a significant amount of the UE's memory resources must be occupied, which may surpass the available memory resources of the UE specific to AI/ML model operation. Secondly, the computation requirements (e.g., FLOPs) of AI/ML models may be too high for a UE to support. This implies that running AI/ML models for inference in a UE may consume computational resources to a large extent. Lastly, running AI/ML models for inference in a UE may lead to substantial power consumption, which may exceed the tolerable limit of the UE's power-saving requirements.

Observation 2: The memory and computational resources as well as the power consumption may become bottlenecks for UE when implementing AI/ML models for CSI compression and restoration.

Proposal 6: The study of how to describe the capabilities of a UE to implement AI/ML models for inference on CSI compression and calculations should be undertaken.

Upon receipt of the CSI request, which instigates CSI measurements and calculations in the UE via DCI transmitted through PDCCH, the UE commences these measurements and calculations. It then conveys the CSI measurement results to the gNB via the PUSCH/PUCCH, the time slot for which has been predetermined. To ensure the most recent CSI measurement data is reported to the gNB, the CSI measurements and calculations must be completed prior to the first uplink symbol that carries the corresponding CSI report(s), taking into account the CSI computation time.

In the case of CSI measurements and calculations that employ AI/ML models for inference, the estimation of CSI computation time necessitates a reconfiguration for AI/ML. This differs from traditional CSI measurement and calculation operations, which can be executed immediately without any preliminary steps. AI/ML models, on the other hand, require activation within the UE before the implementation of inference, a process that may consume a certain amount of time. Moreover, the size of the AI/ML models may affect the model inference time.

The total operation time for the AI/ML model in CSI computation comprises two durations: the time taken for model activation (if applicable) and the time taken for model inference. This could result in different outcomes for the handling of CSI measurements and calculations (either providing a valid CSI report or disregarding the scheduling request) compared to the time duration from the last symbol of the DCI carrying the CSI request to the first uplink symbol carrying the corresponding CSI report(s).

Observation 3: The computational time for CSI restoration at NW side should be estimated and reconfigured to the UE. This may include the time duration for both model activation and model inference.

2.5 Overhead Reduction Issues in CSI Compression

The following two agreements were reached during RAN1 #116 and #116bis meetings respectively:

Agreement Further study following monitoring options in Rel-19, including the necessity and feasibility, - NW-side monitoring, considering overhead, latency, complexity, monitoring accuracy, UE capability - Based on the target CSI reported by the UE via legacy eT2 codebook or eT2-like high-resolution codebook (Case 1) - SRS-based monitoring - UE-side monitoring, considering overhead, latency, complexity, monitoring accuracy, UE capability - Based on the output of the CSI reconstruction model at the UE (Case 2-1) - Note: CSI reconstruction model at the UE-side can be the same as the actual CSI reconstruction model used at the NW-side, a reference model provided by NW, or a proxy model developed by the UE side. - Via direct estimation of intermediate KPI (e.g., SGCS) without reconstructing a target CSI (Case 2-2) - Via estimation of monitoring output other than intermediate KPI without reconstructing a target CSI - Based on precoded RS (e.g., CSI-RS, DMRS) transmitted from NW based on the output of the CSI reconstruction model - Based on the output of the CSI reconstruction model indicated by the NW via legacy eT2 codebook or eT2-like high-resolution codebook Regarding monitoring metrics: - Monitoring accuracy also includes generalization considerations, if applicable. - Complexity also includes LCM complexity, if applicable. - Monitoring overhead, latency, complexity, and accuracy analysis may have to consider using at N>1 CSI feedback occasions. - Testability of UE reported metrics Discussion may include the following aspects: - Consideration of Options 1-5 and their sub-options for alleviating / resolving the issues related to inter-vendor training collaboration - Temporal domain aspects of CSI compression - How the above monitoring approaches or combination of them may help identifying the cause (e.g., NW side, UE side, data drift) of the performance degradation Note: for UE-side monitoring, the final reported monitoring output, if specified, may be different, e.g., be further derived based on the output of the above approaches. Note: implementation-based monitoring solutions can be considered in assessing the necessity of the above monitoring approaches.

The model monitoring in AI/ML-based CSI compression relies on the computation of intermediate KPIs, which relies on obtaining the original CSI w and the recovered CSI w. However, since w is obtained at the UE side while w is obtained at the NW side, it is necessary to design the corresponding signaling and data transmission to enable the network element who performs the model monitoring obtains both w and w at the same time.

In order to enable the model monitoring, w or w needs to be transferred to the NW or UE for the NW-sided or UE-sided model monitoring, respectively. Both w and w may be high dimensional matrix with very huge data volume due to multiple antenna ports, polarized antennas and large bandwidth in a massive MIMO system. Therefore, transferring the raw CSI, i.e., uncompressed CSI w or w, for the model monitoring purpose may exceed the capacity of the traditional L1 or L3 layer signalling. Therefore, the protocol needs to build a common understanding on the quantization and compression of raw CSI which enables the exchange of the CSI for monitoring in a reduced overhead.

Even if the raw CSI is post processed to a smaller volume, its data size is still relatively large, usually reaching several hundred bits or thousands of bits. An alternative way instead of directly transmit the CSI is to transfer other low-dimensional parameters as a precoding matrix indicator to compute the intermediate KPIs, which may significantly reduce the overhead while guaranteeing the reliability of the model monitoring.

Observation 4: The CSI report or indication for model monitoring in AI/ML based CSI compression introduces considerable overhead, RAN 1 should strive for an efficient signaling and overhead reduction mechanism for the CSI transmission.

Proposal 7: Ran1 should consider whether/how to transmit the raw CSI for monitoring purpose.

- FFS: The overhead reduction scheme based on spectral or temporal processing.

Furthermore, transmitting the quantized and compressed CSI introduces quantization error, which results in the deviation of the calculated intermediate KPIs from the actual values, and makes the model monitoring results less reliable. Therefore, the deviation of the calculated model monitoring metrics caused by quantification errors needs to be considered to determine whether the results of model monitoring are reliable.

Observation 5: The reliability of the overhead reduction scheme for CSI transferring for model monitoring in AI/ML based CSI compression should be guaranteed.

- Conclusions

This contribution has led to the following observations:

Observation 1: Certain additional information may impact the priority of AI/ML CSI reporting for CSI compression:

- AI/ML LCM procedure

- AI/ML characteristics for CSI compression

- AI/ML reporting contents

- Other factors that may be pertinent to the value of AI/ML-specific priority should not be precluded.

Observation 2: The memory and computational resources as well as the power consumption may become bottlenecks for UE when implementing AI/ML models for CSI compression and restoration.

Observation 3: The computational time for CSI restoration at NW side should be estimated and reconfigured to the UE. This may include the time duration for both model activation and model inference.

Observation 4: The CSI report or indication for model monitoring in AI/ML based CSI compression introduces considerable overhead, RAN 1 should strive for an efficient signaling and overhead reduction mechanism for the CSI transmission.

Observation 5: The reliability of the overhead reduction scheme for CSI transferring for model monitoring in AI/ML based CSI compression should be guaranteed.

Furthermore, the following proposals have been put forward:

Proposal 1: Study the triggering mechanism based on UCI loss event for CSI information reset or retransmission.

- Option 1: UCI loss event is determined by NW

- Option 2: UCI loss event is determined by UE

Proposal 2: Investigate the CSI information synchronization for mitigating the impact of UCI loss.

- Option 1: Reset the CSI information to pre-defined values.

- Option 2: NW configs or UE initials CSI information retransmission.

Proposal 3: In the case of CSI compression using a two-sided model, the design of an AI/ML-specific CSI-RS resource and CSI reporting configuration that may be compatible with the traditional CSI reporting scenario should be considered in the following aspects:

- AI/ML-specific CSI-RS resource configuration for CSI compression

- AI/ML-specific fields in CSI-ReportConfig IE

- Dedicated report quantities and report configurations for AI

Proposal 4: The definition of AI/ML-specific priority for CSI reporting in relation to CSI compression should be considered in comparison with the traditional CSI priority rules.

Proposal 5: When the UE supports both AI/ML and non-AI/ML CSI reporting, it is necessary to redefine the priority rule considering different types of CSI reporting.

Proposal 6: The study of how to describe the capabilities of a UE to implement AI/ML models for inference on CSI compression and calculations should be undertaken.

Proposal 7: Ran1 should consider whether/how to transmit the raw CSI for monitoring purpose.

- FFS: The overhead reduction scheme based on spectral or temporal processing.

References

- RP-251870 New SID: New WI on AIML for NR air interface Ph 2