3GPP TSG RAN WG1 #122	R1-2505489

Bengaluru, India, Aug 25th – 29th, 2025

Title: 	Discussion on UE features for AI/ML for NR Air Interface

Source: 	ZTE Corporation, Sanechips

Agenda item:	9.1

Document for:	Discussion/Decision

Introduction

In RAN1#121 meeting, a set of UE features for AI /ML use cases were agreed [1]. In this document, we provide our analysis and proposals for the remaining set of UE features for AI/ML use cases for air interfaces based on the list agreed in RAN1#121.

Discussion

General principles

The Rel-19 UE features for AI/ML use cases for air interfaces will be the first set of AI/ML UE features in 3GPP for physical layer. The related UE features should be carefully designed in order to facilitate the potential commercialization of AI/ML use cases for physical layer and establish a solid foundation for 6G discussion. Below we provide two general principles for designing the UE features for AI/ML use cases in physical layer.

Principle#1: Balance between UE capability report and applicability report

RAN2 has designed detailed procedure for applicability report for AI beam prediction and AI POS enhancement [1].

RAN2 further has made following agreements and signalling procedure (see the attached figure) on applicable functionality reporting for beam management UE-sided model: - Step 1: Network sends UECapabilityEnqiry message to initiate the procedure to a UE reporting its AI/ML supported functionalities. - Step 2: UE sends UECapablityInformation message to network, containing supported functionalities at the UE side. - “Step 3”: Following configurations are provided from NW to UE: 1) UE is allowed to do UAI reporting via OtherConfig. 2) Network may provide NW-side additional condition. FFS on the RRC signalling and whether it is mandatory or optional. 3) FFS on configuration (e.g. inference configuration) of supported functionalities. FFS on the content of configuration. - (Between “Step 3” and “Step 4”) UE decides the applicable functionalities based on NW-side additional conditions (if provided), UE-side additional conditions (internally known by UE) and model availability in device. FFS whether other configuration can considered by UE (e.g. inference configuration). FFS how the applicable functionality is decided if NW-side additional condition is not provided in step 3. - “Step 4”: UE reports applicable functionality in the following scenarios: 1) Upon being configured to provide applicable functionality and upon change of applicable functionality via UAI 2) As response to NW-side additional condition requesting applicable functionality reporting in step 3, FFS other network configuration (e.g. inference configuration). - Step 5: 1) Network configures inference configuration to UE after applicable functionality reporting, if inference configuration based on supported functionality is not provided in Step 3 (i.e. inference configuration is provided in Step 5). 2) If inference configuration based on supported functionality is provided in Step 3, it is up to network implementation whether to provide an updated configuration or not.

Basically, the UE feature report is more like static reporting, while the applicability report is more like semi-static reporting. On easier example to better understand the difference between UE feature report and applicability report is hardware capability and software capability, although it is not 100% accurate. UE feature report is more related to hardware capability, while applicability report is more related to software capability that may be subject to infrequent update.

Designing UE features for AI/ML use cases for physical layer should consider the balance between UE feature report and applicability report. If the design leans towards the UE capability reporting, it will result with UE capability with extreme finer granularity. If the design leans towards the applicability reporting, it will result with frequent applicability reporting.

Principle#2: Balance between UE implementation flexibility and UE implementation fragmentation

The UE feature design should always prioritize the balance between UE implementation flexibility and UE implementation fragmentation. Finer granularity of UE capability reporting leads to higher UE implementation flexibility at the cost of higher UE implementation fragmentation, which is not friendly for the network implementation and commercialization. Rel-19 will be the first release for AI/ML use cases for physical layer, it is essential to prioritize the balance between UE implementation flexibility and UE implementation fragmentation to set solid foundation for future commercialization.

Proposal 1: Consider the following two principles for Rel-19 UE feature design for AI/ML use cases for physical layer

- Principle#1: Balance between UE capability report and applicability report

- Principle#2: Balance between UE implementation flexibility and UE implementation fragmentation

CSI report framework

In RAN1#121 meeting, the following UE features for CSI report framework for UE-side inference were agreed.

| Features | Index | Feature group | Components | Prerequisite feature groups | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- |
| 58. NR_AIML_air | 58-0-1 | CSI report framework for UE-side inference | 1. Maximum number of APUs for all types of UE-sided inference for CSI report(s) for simultaneously in a CC 2. Maximum number of APUs for all types of UE-sided inference for CSI report(s) simultaneously across all CCs | FFS | Component 1 candidate values: FFS Component 2 candidate values: FFS | Optional with capability signalling |

Regarding FG58-0-1, based on our understanding, it can apply similar design as the legacy CPU. For the legacy CPU, the following value ranges are applied.

- simultaneousCSI-ReportsAllCC INTEGER (5..32)

- simultaneousCSI-ReportsPerCC INTEGER (1..8)

Thus, the same value range can be applied for the APU here.

Proposal 2: Regarding FG58-0-1, the value range for Component 1 (APU per CC) is (1..8), and the value range for Component 2 (APU for all CC) is (5..32).

AI beam prediction

Overall, the following 6 UE features have been agreed in RAN1#121:

- FG58-1-1: Increased number of reported RSs for beam management

- FG58-1-2: UE-side beam prediction for BM Case1 (without predicted RSRP)

- FG58-1-3: UE-side beam prediction for BM Case1 (with predicted RSRP)

- FG58-1-4: UE-side beam prediction for BM Case2 (without predicted RSRP)

- FG58-1-5: UE-side beam prediction for BM Case2 (with predicted RSRP)

- FG58-1-7: Data collection for UE-side beam prediction

Regarding the following component of FG58-1-2, basically, there are two different alternatives to let the UE report the supported combinations of set A and set B. The first one is to report this info via UE capability, while the other one is to report this info via UE applicability report defined by RAN2. These two methods represent two different flexibilities. Typically, reporting supported combinations of the number of RS in set B and Set A via UE capability has less flexibility, while reporting supported combinations of the number of RS in set B and Set A via applicability report has higher flexibility since UE can update the applicability report whenever it has new AI models.

[7. Supported combinations of the number of resources for Set B and the number of resources for Set A]

[7a: Supported maximum number of resources for Set B]

[7b: Supported maximum number of resources for Set A]

Thus, we propose to discuss these two different methods.

Proposal 3: Regarding FG58-1-2, discuss the following two methods to report the supported combinations of number of beams in set A and number of beams in set B considering the flexibility of different methods.

- Method.1: Report the supported combinations of the number of resources for Set B and the number of resources for Set A via UE capability.

- Method.2: Report the maximum number of resources for Set B/Set A via UE capability, and report the supported combinations of the number of resources for Set B and the number of resources for Set A via applicability report.

Regarding the components of FG58-1-2~FG58-1-5, the “FFS: CPU/AIMLPU related information” can be addressed by allowing the UE to report the occupied CPU and/or AIMLPU.

Proposal 4: Regarding the components of FG58-1-2~FG58-1-5, UE reports the occupied CPU and/or AIMLPU.

Regarding the following component of FG58-1-2, unlike AI CSI prediction, the additional cache overhead for performance monitoring for AI beam prediction at the UE side is not that large. If UE supports UE side AI beam prediction but doesn’t support performance monitoring, network doesn’t know whether the prediction can be trusted or not. Alternatively, network may trigger UE to report measurement results of Set A periodically to check the prediction accuracy, which will negate the overhead reduction of AI beam prediction. Thus, we propose to introduce performance monitoring in the basic UE feature of AI beam prediction.

[10. Supported options for performance monitoring for beam case 1 with UE side model]

Proposal 5: Regarding FG58-1-2, add the following component 10 for performance monitoring.

- Component 10: Supported options for performance monitoring for beam case 1 with UE side model

Regarding FG58-1-7, we propose the following updates.

- We propose to remove “[for BM case 1]” in the name and component 1. No matter whether it is Beam case 1 or Beam case 2, the UE behaviour is the same, i.e., measuring reference signal and derive the RSRP.

- We propose to remove the component 3. Similar logic as above, UE behaviour is the same no matter whether Set B is a subset of the Set A or not.

Proposal 6: Regarding FG58-1-7, we propose the following

- Remove the “[for BM case 1]” in the name and component 1

- Remove component 3

AI POS enhancement

At the initial stage of Rel-19, 5 sub use cases are defined for AI POS enhancement, i.e.,

- Direct AI/ML positioning:

  - (1st priority) Case 1: UE-based positioning with UE-side model, direct AI/ML positioning

  - (2nd priority) Case 2b: UE-assisted/LMF-based positioning with LMF-side model, direct AI/ML positioning

  - (1st priority) Case 3b: NG-RAN node assisted positioning with LMF-side model, direct AI/ML positioning

- AI/ML assisted positioning

  - (2nd priority) Case 2a: UE-assisted/LMF-based positioning with UE-side model, AI/ML assisted positioning

  - (1st priority) Case 3a: NG-RAN node assisted positioning with gNB-side model, AI/ML assisted positioning

During RAN#107 meeting, it is agreed that the 2nd priority use cases won’t be pursued in Rel-19. In other words, only the following three AI POS enhancement use cases will be specified in Rel-19.

- Direct AI/ML positioning:

  - (1st priority) Case 1: UE-based positioning with UE-side model, direct AI/ML positioning

  - (1st priority) Case 3b: NG-RAN node assisted positioning with LMF-side model, direct AI/ML positioning

- AI/ML assisted positioning

  - (1st priority) Case 3a: NG-RAN node assisted positioning with gNB-side model, AI/ML assisted positioning

Among these three 1st priority use cases, case 3a is with gNB side model and case 3b is with LMF side model. The UE side operation is the same as what we have, i.e., UE only needs to transmit SRS to the base station. From this perspective, there is no need to define any new UE feature for AI POS enhancement case 3a and case 3b.

Proposal 7: New UE feature for AI POS enhancement case 1 is needed, while no need to define any new UE feature for AI POS enhancement case 3a and case 3b.

As agreed in RAN2, AI positioning case 1 is a new positioning method, where the UE features in Rel-16/17/18 can be extended to AI positioning case 1, overall, we think the following UE feature design can be considered.

Proposal 8: Consider the following UE features for AI POS enhancement in Rel-19.

| 58. NR_AIML_Air | 58-2-7 | Support of PRS measurement in RRC_INACTIVE state for case 1 | Support of PRS measurement in RRC_INACTIVE state for case 1 - location server | 58-2-3, 27-6 | No |  | PRS measurement in RRC_INACTIVE state for case 1 is not supported | per band | n/a | n/a | n/a | Need for location server to know if the feature is supported. Note: PRS capabilities for case 1 described in FGs in 58-2-3, 58-2-3a, 58-2-3b are the same for RRC Inactive. Support of PRS processing measurement in RRC_INACTIVE state does not imply that LMF is aware of or controlling UE RRC state |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

AI CSI prediction

In RAN1#121 meeting, the following UE features for AI CSI prediction were agreed. Most of UE features defined in Rel-18 Doppler MIMO are reused for Rel-19 AI CSI prediction.

| Features | Index | Feature group | Components | Prerequisite feature groups | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- |
| 58. NR_AIML_Air | 58-3-1 | CSI prediction for UE-sided inference when N4=1 | 1. Support of CSI prediction for UE-sided inference when N4=1 2. Support for reporting predicted PMI with N4=1 3. A list of supported combinations, each combination is { Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} across all CCs in a band when reported per band, and across all CCs in a band combination when reported per BC simultaneously 4. Support of Rel-16 eType-II regular codebook refinement for predicted PMI with PMI subband R=1 5. Support parameter combinations with L=2,4 6. Support for rank = 1,2 7. Support for the size of DD-basis, N4=1 8. Support X=1 CQI based on the first/earliest slot of the CSI reporting window and the first/earliest predicted PMI (TDCQI=’1-1’) [9. Value for CPU occupation, when P/SP-CSI-RS is configured for CMR] [10. Value for CPU occupation, when A-CSI-RS is configured for CMR] 11. Scaling factor for active resource counting Kp | 2-35 |  | Optional with capability signalling |
| 58. NR_AIML_Air | 58-3-1a-1 | DD unit size when A-CSI-RS is configured for CMR N4>1 for UE side inference of CSI prediction | Value of d=1 for the DD unit size when A-CSI-RS for inference is configured for CMR | 58-3-1 |  | Optional with capability signaling |
| 58. NR_AIML_Air | 58-3-1b | Maximum number of aperiodic CSI-RS resources that can be configured in the same CSI report setting for Rel-16-based doppler measurement for UE side inference of CSI prediction | 1. Maximum number of aperiodic CSI-RS resources for inference that can be configured in the same CSI report setting for Rel-16-based doppler measurement | 58-3-1 | Component 1 candidate values: {4, 8, 12} | Optional with capability signaling |
| 58. NR_AIML_Air | 58-3-1-2 | Support R=2 for Rel-16-based doppler codebook for UE side inference of CSI prediction | A list of supported combinations {Max # of Tx ports in one resource for inference, Max # of resources for inference and total # of Tx ports}, across all CCs simultaneously, with R=2 | 58-3-1 | Candidate values for component 1: - Maximum 16 triplets - Max # of Tx ports in one resource: {4,8,12,16,24,32} - Max # resources: {1 to 64} - Max # total ports: {4 to 256} | Optional with capability signaling |
| 58. NR_AIML_Air | 58-3-1-3 | Support X=1 based on first and last slot of WCSI, for Rel-16-based doppler codebook for UE side inference of CSI prediction | Support of TDCQI = ‘1-2’ | 58-3-1 |  | Optional with capability signaling |
| 58. NR_AIML_Air | 58-3-1-3a | Support X=2 CQI based on 2 slots for Rel-16-based doppler codebook for UE side inference of CSI prediction | Support of TDCQI = ‘2’ | 58-3-1 |  | Optional with capability signaling |
| 58. NR_AIML_Air | 58-3-1-4 | support of l = (n – nCSI,ref ) for CSI reference slot for Rel-16 based doppler codebook for UE side inference of CSI prediction | 1. Support of l = (n – nCSI,ref ) for CSI reference slot when N4=1 and d>1 2. Support of l = (n – nCSI,ref ) for CSI reference slot when N4>1 | 58-3-1 |  | Optional with capability signaling |
| 58. NR_AIML_Air | 58-3-1-5 | Support of L=6 for Rel-16 based doppler codebook for UE side inference of CSI prediction | Support of L=6 for Rel-16 based doppler codebook | 58-3-1 |  | Optional with capability signaling |
| X. NR_AIML_Air | 58-3-1-6 | Support of rank equals 3 and 4 for Rel-16 based doppler codebook for UE side inference of CSI prediction | Support of rank equals 3 and 4 for Rel-16 based doppler codebook | 58-3-1 |  | Optional with capability signaling |
| 58. NR_AIML_Air | 58-3-1-7 | Active CSI-RS resources and ports for mixed R16 based doppler codebook for CSI prediction via UE side model with other codebooks in any slot | 1. List of codebook combinations of two types 2. List of {max number of ports per resource, max number of resources, max number of total ports} for each codebook combination | 23-9-1, 16-3a, 2-36, 2-40, 2-41, 23-9-2, 23-9-4, 40-3-2-1, 40-3-2-1a, 40-3-2-2, X-1-1, X-1-1a, X-1-2 | Component 1 candidate values: FFS Component 2 candidate values: FFS Note: if a UE reports one or more codebook combinations in 58-3-1-7, then usage of active CSI-RS resources and ports for multiple codebooks in any slot is allowed only within those combinations Note: For coexisting of mixed codebooks in any slot, gNB need to honor 16-8, 23-9-5, 58-3-1-7 and per-codebook capability 2-36/40/41, 16-3a, and 23-9-1/23-9-2/23-9-4. [Note: Up to 4 combinations for component 1] | Optional with capability signaling |
| 58. NR_AIML_Air | 58-3-2 | CSI prediction for UE-sided inference when N4>1 | 1. Support of CSI prediction for UE-sided inference when N4>1 2. Support for reporting predicted PMI with N4>1 3. A list of supported combinations, each combination is {Max N4, Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} across all CCs in a band when reported per band, and across all CCs in a band combination when reported per BC simultaneously 4. Value of d=m for the DD unit size when A-CSI-RS is configured for CMR 5. Support for the size of DD-basis, N4>1 7. A list of supported combinations, each combination is {Max N4, Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} for one CSI report setting [8. Supported values of the maximum number of observation number] | 58-3-1 | FFS: CPU/AIMLPU related information Candidate values: FFS | Optional with capability signalling |
| 58. NR_AIML_Air | 58-3-4 | UE side data collection for CSI prediction | 1. Support of data collection for CSI prediction | FFS |  | Optional with capability signalling |

Proposal 9: Regarding FG58-3-1 and FG58-3-2, UE reports the occupied CPU and/or AIMLPU.

Conclusion

In this contribution, we provide our analysis and proposals for the UE feature design for Rel-19 AI/ML use cases for air interface.

General principle

Proposal 1: Consider the following two principles for Rel-19 UE feature design for AI/ML use cases for physical layer

- Principle#1: Balance between UE capability report and applicability report

- Principle#2: Balance between UE implementation flexibility and UE implementation fragmentation

CSI report framework

Proposal 2: Regarding FG58-0-1, the value range for Component 1 (APU per CC) is (1..8), and the value range for Component 2 (APU for all CC) is (5..32).

AI beam prediction

Proposal 3: Regarding FG58-1-2, discuss the following two methods to report the supported combinations of number of beams in set A and number of beams in set B considering the flexibility of different methods.

- Method.1: Report the supported combinations of the number of resources for Set B and the number of resources for Set A via UE capability.

- Method.2: Report the maximum number of resources for Set B/Set A via UE capability, and report the supported combinations of the number of resources for Set B and the number of resources for Set A via applicability report.

Proposal 4: Regarding the components of FG58-1-2~FG58-1-5, UE reports the occupied CPU and/or AIMLPU.

Proposal 5: Regarding FG58-1-2, add the following component 10 for performance monitoring.

- Component 10: Supported options for performance monitoring for beam case 1 with UE side model

Proposal 6: Regarding FG58-1-7, we propose the following

- Remove the “[for BM case 1]” in the name and component 1

- Remove component 3

AI POS enhancement

Proposal 7: New UE feature for AI POS enhancement case 1 is needed, while no need to define any new UE feature for AI POS enhancement case 3a and case 3b.

Proposal 8: Consider the following UE features for AI POS enhancement in Rel-19.

| 58. NR_AIML_Air | 58-2-7 | Support of PRS measurement in RRC_INACTIVE state for case 1 | Support of PRS measurement in RRC_INACTIVE state for case 1 - location server | 58-2-3, 27-6 | No |  | PRS measurement in RRC_INACTIVE state for case 1 is not supported | per band | n/a | n/a | n/a | Need for location server to know if the feature is supported. Note: PRS capabilities for case 1 described in FGs in 58-2-3, 58-2-3a, 58-2-3b are the same for RRC Inactive. Support of PRS processing measurement in RRC_INACTIVE state does not imply that LMF is aware of or controlling UE RRC state |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

AI CSI prediction

Proposal 9: Regarding FG58-3-1 and FG58-3-2, UE reports the occupied CPU and/or AIMLPU.

- Reference

- R1-2504673, Updated RAN1 UE features list for Rel-19 NR after RAN1 #121, Moderators (AT&T, NTT DOCOMO, INC.), RAN1#121