3GPP TSG RAN WG1 Meeting #122		R1-2505733

Bengaluru, India, Aug. 25th-29th, 2025

Source: 	OPPO

Title:	On specification for AI/ML-based CSI prediction

Agenda Item:	8.1.3

Document for:	Discussion and Decision

Remaining issues

In Rel-19, most of basic specifications support for AI/ML-based CSI prediction has been finished to make this use case workable in 5G-A. However, some issues are not toughed in Rel-19, which will be further discussed in this contribution.

Agreement

For CSI prediction using UE-side model, to calculate the inference report using Doppler codebook,

- For PU occupancy, support

  - Dedicated AI/ML PU (OAPU) and/or legacy CPU (OCPU) are occupied,

    - OAPU= 0 or N is reported by UE

    - OCPU=0 or M is reported by UE

    - Note: Detailed values of N and M can be further discussed in UE feature.

    - Note: Combination of OAPU= 0 and OCPU=0 is not allowed

    - Note: if any of the unoccupied PU cannot satisfy the corresponding required PU by the CSI report, the CSI report will follow the legacy behavior of exceeding the CPU limit, neither of the PUs are occupied

- For occupancy duration of CPU and APU,

  - the same occupancy duration is used if both CPU and APU are reported non-zero value

  - if associated monitoring report is not configured, reuse following legacy occupancy duration

    - For semi-persistent CSI report on PUSCH with P/SP CSIRS CMR, occupation starts from the first symbol of KP-th latest consecutive P/SP-CSI-RS occasions no later than CSI reference resource, until the last symbol of the PUSCH carrying the report, where KP is indicated by UE capability.

Note: Detailed values of KP can be further discussed in UE feature.

    - Aperiodic CSI report occupies PU(s) from the first symbol after the PDCCH triggering the CSI report until the last symbol of the scheduled PUSCH carrying the report.

- The total number of dedicated AI/ML PU for AI/ML is reported by UE capability

As agreed in RAN1#121, the CPU/APU occupancy reporting and occupancy duration have been approved for inference report using Doppler codebook. For UE-assisted performance monitoring with SGCS report, UE should calculate the SGCS between ground-truth CSI and inference of CSI prediction model, wherein legacy CPU are occupied for the PMI calculation and SGCS calculation. During this procedure of monitoring result calculation, no additional APU is occupied.

Proposal 1: Support OCPU=Mmonitoring and OAPU=0 reported by UE to calculate the UE-assisted performance monitoring report, where M can be discussed in UE feature.

Then for the PU occupancy duration of CPU and APU, if associated monitoring report is configured, the additional CPU occupancy duration should be considered. For semi-persistent CSI inference report on PUSCH with P/SP CSI-RS CMR, the additional Mmonitoring CPU occupation starts from the first symbol of Kp-th latest consecutive P/SP-CSI-RS occasions no later than CSI reference resource, until the last symbol of the PUSCH carrying the monitoring report, where Kp is indicated by UE capability. For aperiodic CSI inference report, additional Mmonitoring CPU occupation starts from the first symbol after the PDCCH triggering the CSI report, until the last symbol of the scheduled PUSCH carrying the monitoring report.

Proposal 2: For occupancy duration of additional CPU for UE-assisted performance monitoring:

- For semi-persistent CSI inference report on PUSCH with P/SP CSI-RS CMR, the additional Mmonitoring CPU occupation starts from the first symbol of Kp-th latest consecutive P/SP-CSI-RS occasions no later than CSI reference resource, until the last symbol of the PUSCH carrying the monitoring report, where Kp is indicated by UE capability.

- For aperiodic CSI inference report, additional Mmonitoring CPU occupation starts from the first symbol after the PDCCH triggering the CSI report, until the last symbol of the scheduled PUSCH carrying the monitoring report.

Conclusion

In this contribution, we provide our views on some remained issues for AI/ML based UE-sided CSI prediction. Proposals are listed as following:

Proposal 1: Support OCPU=Mmonitoring and OAPU=0 reported by UE to calculate the UE-assisted performance monitoring report, where M can be discussed in UE feature.

Proposal 2: For occupancy duration of additional CPU for UE-assisted performance monitoring:

- For semi-persistent CSI inference report on PUSCH with P/SP CSI-RS CMR, the additional Mmonitoring CPU occupation starts from the first symbol of Kp-th latest consecutive P/SP-CSI-RS occasions no later than CSI reference resource, until the last symbol of the PUSCH carrying the monitoring report, where Kp is indicated by UE capability.

- For aperiodic CSI inference report, additional Mmonitoring CPU occupation starts from the first symbol after the PDCCH triggering the CSI report, until the last symbol of the scheduled PUSCH carrying the monitoring report.

References

- R1-2504939, Summary #6 of CSI prediction, Moderator (LG Electronics), St.Julians, Malta, May 19th-23rd, 2025.