3GPP TSG RAN WG1 #122	              	     R1-2505737

Bengaluru, India, Aug 25th – 29th, 2025

Source:	OPPO

Title:	Discussion on 3-antenna-port uplink transmissions

Agenda Item:	8.2.3

Document for:	Discussion and Decision

Introduction

In 3GPP RAN #105 meeting, a revised work item on further enhancement of MIMO was approved [1], which includes 3-antenna-port codebook-based transmissions:

- Specify non-coherent UL codebook to facilitate 3-antenna-port codebook-based transmissions, enhancement(s) to enable 3T6R SRS antenna switching, as well as UE capability signaling for 3T3R antenna switching and 3-antenna-port non-codebook-based transmissions, without enhancement on UL full power transmission and without enhancement on SRS resource

Note: UL full power transmission mode 1 and 2 are not supported.

Note: Other than UE capability signaling, no other enhancement is specified for 3T3R SRS antenna switching.

In this contribution, we discuss the remaining issues of the specification 38.214-j00.

Discussion

TPs for the draft CR

Reason for change: As stated in the agreement, for a 3Tx UE, only PUSCH antenna ports 1000, 1001, and 1002 are utilized. Another agreement agrees that DCI format 0_3 is not supported for codebook-based PUSCH transmission by a 3Tx UE. However, the current version of 38.214-j00 contains a description of UE PT-RS transmission for PUSCH transmission using three ports in section 6.2.3.1 that is not aligned with the aforementioned agreements.

Agreement For codebook-based transmission by a 3TX UE, - Only PUSCH antenna ports 1000, 1001, 1002 are used - Option- 2: Subject to UE capability, up to 2 PTRS ports may be configured in PTRS-UplinkConfig, - FFS whether a single bit or 2 bits are used for PTRS-DMRS association indication. Above is only for single panel transmission. Conclusion For codebook-based PUSCH transmission by a 3TX UE, PUSCH scheduling by DCI format 0_3 is not supported. - Agreement: Adopt the following TP in red text for TS 38.214 section 6.1.1.1. -------------------------------------------Unchanged parts are omitted---------------------------------- For codebook based transmission with three antenna ports, the UE determines its codebook subsets based on TPMI(s) and upon the reception of higher layer parameter codebookSubset in pusch-Config for PUSCH associated with DCI format 0_1 or 0_3 and codebookSubsetDCI-0-2in pusch-Config for PUSCH associated with DCI format 0_2 which may be configured with 'nonCoherent'. -----------------------------------------Unchanged parts are omitted------------------------------------------.

Summary of change: Suggest the following modifications to section 6.2.3.1 of 38.214-j00: clarify that only PUSCH antenna ports 1000, 1001, and 1002 are used for PUSCH transmission with three ports, and remove the description for DCI format 0-3 to ensure alignment with the agreements.

Consequences if not approved: The agreement is not correctly captured for PTRS with 3 antenna ports.

Text proposal 1 for the endorsed 38.214 draft CR:

6.2.3.1 UE PT-RS transmission procedure when transform precoding is not enabled <<unchanged parts omitted>> For partial-coherent and non-coherent codebook-based UL transmission with 2 or 4 antenna ports, or non-coherent codebook-based UL Transmission with 3 antenna ports, or when the higher layer parameter CodebookTypeUL is set to ‘codebook2’, ‘codebook3’, or ‘codebook4’ with 8 antenna ports, the actual number of UL PT-RS port(s) is determined based on TPMI(s) and/or number of layers which are indicated by 'Precoding information and number of layers' field(s) in DCI format 0_1, 0_2 or 0_3 or configured by higher layer parameter precodingAndNumberOfLayers: - if the UE is configured with the higher layer parameter maxNrofPorts in PTRS-UplinkConfig set to 'n2', the actual UL PT-RS port(s) and the associated transmission layer(s) are derived from indicated TPMI(s) as: - for PUSCH transmission with 2 or 4 ports, PUSCH antenna port 1000 and 1002 in indicated TPMI(s) share PT-RS port 0, and PUSCH antenna port 1001 and 1003 in indicated TPMI(s) share PT-RS port 1. - UL PT-RS port 0 is associated with the UL layer 'x' of layers which are transmitted with PUSCH antenna port 1000 and PUSCH antenna port 1002 in indicated TPMI(s), and UL PT-RS port 1 is associated with the UL layer 'y' of layers which are transmitted with PUSCH antenna port 1001 and PUSCH antenna port 1003 in indicated TPMI(s), where 'x' and/or 'y' are given by DCI parameter 'PTRS-DMRS association' as shown in DCI format 0_1, 0_2 and 0_3 described in Clause 7.3.1 of [5, TS 38.212]. - for PUSCH transmission with 3 ports, PUSCH antenna port 1000 and 1002 in indicated TPMI(s) share PT-RS port 0, and PUSCH antenna port 1001 in indicated TPMI(s) share PT-RS port 1. - UL PT-RS port 0 is associated with the UL layer 'x' of layers which are transmitted with PUSCH antenna port 1000 and PUSCH antenna port 1002 in indicated TPMI(s), and UL PT-RS port 1 is associated with the UL layer 'y' of layers which are transmitted with PUSCH antenna port 1001 in indicated TPMI(s), where 'x' and/or 'y' are given by DCI parameter 'PTRS-DMRS association' as shown in DCI format 0_1 and 0_2 described in Clause 7.3.1 of [5, TS 38.212]. - for PUSCH transmission with 8 ports, PUSCH antenna port 1000, 1001, 1004 and 1005 in indicated TPMI(s) share PT-RS port 0, and PUSCH antenna port 1002, 1003, 1006 and 1007 in indicated TPMI(s) share PT-RS port 1. - UL PT-RS port 0 is associated with the UL layer 'x' of layers which are transmitted with one or more of PUSCH antenna port 1000, 1001, 1004 and 1005 in indicated TPMI(s), and UL PT-RS port 1 is associated with the UL layer 'y' of layers which are transmitted with one or more of PUSCH antenna port 1002, 1003, 1006 and 1007 in indicated TPMI(s), where 'x' and/or 'y' are given by DCI parameter 'PTRS-DMRS association' as shown in DCI format 0_1 and DCI format 0_2 described in Clause 7.3.1 of [5, TS 38.212]. <<unchanged parts omitted>>

Conclusions

In this contribution, we provide one text proposal for 3-antenna-port uplink transmission.

References

- RAN Meeting#105, RP-242394, WID revision: NR MIMO Phase 5

- 38.214-j00, Physical layer procedures for data, V19.0.0.