3GPP TSG RAN WG1 #122									R1-2505932

Bengaluru, India, Aug 25th – 29th, 2025

Agenda item:	11.6

Source:	NEC

Title: 	Discussion on AIML in 6GR interface

Document for:	Discussion and Decision

- Introduction

After intensive discussions in RAN plenary meeting in June, the following objectives are proposed for study of AI/ML air interface in 6GR [1]. In this contribution, we provide our views on AIML in 6GR interface. More specifically, in Section 2, the discussion is on use case selection (i.e., 8.a), and in Section 3, the discussion is on AI/ML framework (i.e., 8.b).

The detailed objectives of the study related to AI/ML in 6GR are [1]: - AI/ML for 6GR and Radio Access Network, leveraging 5G AI/ML framework, as appropriate [See TR38.843] [RAN1, RAN2, RAN3, RAN4] - Identify Use Case(s) of interest (either existing or new) with compelling trade-off between e.g., performance, complexity, etc… Coordinated discussion needs to be ensured with related design areas, where needed (e.g., MIMO, Mobility, etc…) NOTE: lead WG depends on the use case. - AI/ML framework: Extensible AI/ML enablers based on the identified Use Case(s), including - LCM procedures [RAN2, RAN1, RAN3, RAN4] - Data collection and data management, in coordination with SA WGs [RAN2, RAN3, RAN1] Note: NW for AI is assumed to be covered by new services 6GR and RAN design shall ensure that the 6G System can also operate without AI/ML

- Discussion on Use Case Selection

In 6GR air interface, AI/ML can be integrated seamlessly across all components of the communication system. As a starting point, RAN1 can select use cases by

- Identifying existing 5GA use cases and their extensions to be supported in 6G, such as AI/ML BM for MTRP, CSI compression with time-domain aspects

- Studying new use cases on performance and cost, such as AI/ML based RS overhead reduction

- Studying new use cases with which the traditional communication block can be enhanced or replaced by AI/ML models, such as AI/ML based channel estimation

- Studying new use cases by considering joint AI/ML processing of multiple functionalities, such as joint modulation and precoding, joint channel estimation and demodulation

In 6GR air interface, existing use cases specified in Rel-19 and to-be-specified in Rel-20 can be considered for Day 1 support, due to their proven performance gain and reasonable tradeoff between performance and complexity. It is also expected that less spec effort is required to have them in 6G since the existing use cases are well defined and investigated.

However, to make full advantage of AI/ML, some extensions are needed. For example, for AI/ML based BM, only measurement and report for prediction is specified in Rel-19, but how to apply the prediction results is not discussed. RRC reconfiguration seems inevitable to reconfigure the BM RS or TCI state corresponding to the predicted beams, which undermines the potential of AI/ML prediction. To our understanding, fast TCI state activation/indication with the predicted beam(s) could be a very basic extension for beam management. In addition, Rel-19 AI/ML based BM only works under a very basic assumption: single TRP, intra-cell, and Tx-beam prediction. It is not sufficient for AI/ML based BM to be used in more practical FR2 scenarios with BFR, MTRP, inter-cell and Tx-Rx alignment requirements. The following can be considered to enhance AI/ML based BM.

- Beam failure detection and new beam identification based on spatial/temporal beam prediction can solve the blockage issue more efficiently

- MTRP is a more typical deployment scenario and beam group prediction requires more designs on Set B/Set A pattern

- Inter-cell beam prediction can be done jointly with AI/ML mobility

- Tx-Rx beam alignment is a fundamental and realistic requirement and it needs more designs to enable Rx beam prediction

Another example of 5GA use case extension is the CSI compression with time domain aspects, e.g., spatial/temporal/frequency CSI compression, CSI compression plus CSI prediction or “Case 2” and “Case 3” identified during Rel-19 Study. According to the WID [2], only CSI spatial/frequency compression without temporal aspects (“Case 0”) will be specified in Rel-20. However, based on the study, time domain aspects are essential to achieve the promised performance gain. Therefore, built upon the upcoming Rel-20 spec of Case 0, it is good to have Case 2 and 3 as further extension in 6G.

Identify existing 5GA use cases and their extensions to be supported in 6G, for example,

AI/ML based BM with further enhancements on fast TCI state activation/indication with the predicted beam(s), BFR, MTRP, inter-cell scenarios and Tx-Rx beam pair prediction

CSI compression with further enhancements on time domain aspects such as CSI prediction

Other than 5GA use cases and their extensions, in 6GR air interface, the ambitious expectation is that AI/ML can be integrated for each component as well as across multiple components of the communication system, for example, coding, modulation, channel estimation, MIMO, etc., and ultimately AI/ML based Tx-Rx chain and transceiver. However, studies are needed to investigate the performance gain, as well as the cost like complexity, power consumption, etc.

One clear direction to embrace the gain is to apply AI/ML to reduce RS overhead. The existing 5GA use cases can also be seen as good examples of overhead reduction by AI/ML, which cover

- Specific domains: spatial domain beam prediction can be used for spatial-domain RS overhead reduction; time domain beam prediction and CSI prediction can be used for time-domain RS overhead reduction

- Specific RS types: SSB and CSI-RS overhead reduction

- Specific procedures: beam management procedure and CSI acquisition procedure

In 6GR, using AI/ML models for RS overhead reduction can be a general use case which consists of various sub use cases covering different procedures, domains, RS types and scenarios.

AI/ML models for RS overhead reduction can be useful in initial access procedure. SSB and RACH overhead reduction is critical in initial access. Both spatial and time domain prediction is useful to reduce SSB and RACH overhead especially in FR2. And the AI/ML model for beam prediction from Rel-19 can fit initial access procedure with minimal adjustment. In another example, DMRS overhead reduction can be achieved by using AI/ML in channel estimation procedure as well, which we will elaborate in the later part of this section.

Moreover, in addition to spatial domain and time domain, frequency domain prediction can be considered to reduce RS density, e.g., using measurement results based on reduced RS transmission in frequency domain to predict channel assuming a full RS transmission. It can be suitable for both CSI-RS and DMRS and may have impact on the CSI report, PRG/subband determination, demodulation, etc. For spatial domain overhead reduction, in addition to BM, it is also very useful to reduce the port number, e.g., by measurement on a smaller set of ports to predict channels on a larger set of ports.

Furthermore, current AI/ML use cases are mainly for DL enhancement, and the UL counterpart shall have equal importance. Further investigations on UL beam prediction, AI/ML based SRS overhead reduction are needed.

Based on above discussions, for RS overhead reduction, we have the following proposal

Study AI/ML to reduce RS overhead in 6GR air interface, including the following aspects.

Procedures: initial access, BM/CSI, channel estimation

Domains: time, frequency, and spatial domain

RS types: DMRS, SRS, CSI-RS, and SSB

Scenarios: both DL and UL

A more holistic approach to select use case is to consider the core physical layer receiver chain, which traditionally includes blocks for channel estimation, channel equalization, demodulation, and decoding. For 6GR, it is essential to study which of these blocks can be enhanced or replaced by AI/ML functionalities. AI/ML-based channel estimation, AI/ML-based channel equalization and modulation/demodulation techniques in particular have been shown to provide significant performance benefits in various studies.

For AI/ML-based channel estimation, which has the potential of improved accuracy, reduced latency and also reduced RS overhead as mentioned above. It could be useful for both UE-sided DL DMRS based channel estimation and NW-sided UL DMRS based channel estimation. It could also be possible that a two sided model is used for channel estimation. A further study can be done on non-orthogonal data and pilot signal. More specifically, to support AI/ML-based channel estimation, the study may be needed on the following aspects:

- DMRS pattern: new DL and UL DMRS patterns are needed for AI/ML based channel estimation, which shall be with less overhead compared to the non-AI/ML DMRS patterns, e.g., sparser at least in frequency domain. Meanwhile, since 6GR system shall work without AI/ML, the co-existence issue with non-AI/ML DMRS is to be solved

- Data channel processing requirement: a short processing time is expected if AI/ML model is applied for PDSCH and PUSCH

- HARQ ACK: it may require to indicate the root cause of decoding failure, for example, whether NACK is caused by failure of AI/ML model inference

- Power allocation: including power boosting opportunities if less REs carrying DMRS

Furthermore, channel equalization can be viewed as a logical extension of channel estimation. The channel estimates, along with the actual received symbols, can be fed into an AI/ML function to output the equalized symbols directly. This opens two distinct study paths: Option 1, which involves AI/ML-based channel estimation followed by a traditional channel equalizer, and Option 2, which pursues an AI/ML-based model for combined channel estimation and equalization.

Based on above discussions, for AI/ML based channel estimation, we have the following proposals

Study AI/ML based channel estimation in 6GR air interface, including the following aspects

new DL and UL DMRS patterns, and co-existence with non-AI/ML DMRS

processing requirement for PDSCH and PUSCH

HARQ ACK enhancement

Power allocation

Study AI/ML based channel estimation together with channel equalization, including the following options:

Option 1: AI/ML-based channel estimation with a traditional channel equalization block.

Option 2: A joint AI/ML model for combined channel estimation and equalization.

For AI/ML-based modulation and demodulation, many studies have considered replacing traditional receivers (e.g., MMSE) with AI/ML-based detection/demodulation mechanisms. Some studies have also shown the benefits of joint AI/ML-based modulation and demodulation, which may require a two-sided model running at both the transmitter and receiver. Such a joint scheme can allow for modulation strategies that adapt to the channel conditions experienced by the link. Hence, we should consider both of the following options for further study: Option 1, studying an AI/ML-based demodulation scheme that works with a traditional modulation scheme at the transmitter, and Option 2, studying a two-sided AI/ML-based modulation and demodulation scheme.

Study AI/ML based modulation and demodulation, including the following options for further study:

Option 1: Study an AI/ML-based demodulation scheme at the receiver, assuming a traditional modulation scheme is used by the transmitter.

Option 2: Study a two-sided AI/ML-based modulation and demodulation scheme where both transmitter and receiver utilize AI/ML models.

In addition to integrating AI/ML to each individual communication blocks, in 6GR, it is reasonable to consider joint AI/ML processing of multiple blocks, for example, joint channel estimation and equalization as mentioned above, joint beam and CSI prediction, joint CSI prediction and compression, etc. A new use case can be the one realized by coupling multiple independent AI/ML models that are used for different use cases or a powerful AI/ML model addressing multiple communication blocks, which provides some new ideas for the selection of use cases.

This joint processing of AI/ML functionalities approach offers several key advantages:

- Reduced Latency: By executing related functions simultaneously, joint processing minimizes signalling delays and accelerates decision-making.

- Improved Prediction Accuracy: Combining models allows for richer context and better-informed predictions, enhancing reliability in dynamic environments.

- Better Resource Utilization: Coordinated AI tasks optimize the use of spectrum, compute, and scheduling resources, leading to more efficient network operation.

- Enhanced User Experience: Smarter, faster decisions translate to smoother handovers, consistent QoS, and lower drop rates—ultimately improving service quality for end users.

As mentioned above, any AI/ML-based component in Tx-Rx chain can be considered to be processed jointly with each other, such as AI/ML-based modulation, AI/ML-based layer mapping/precoding, AI/ML-based channel estimation/detection, and AI/ML-based demodulation. To start with, there could be the following examples

- Joint CSI compression and prediction, can be achieved by coupling CSI compression and CSI prediction

- Joint modulation and precoding, can be implemented by coupling AI/ML-based modulation and AI/ML-based layer mapping/precoding

- Joint channel estimation and demodulation, can be achieved by coupling AI/ML-based channel estimation/detection and AI/ML-based demodulation

Therefore, in identifying new use cases (or functionalities), we can consider whether they can be realized by coupling other independent functionalities.

Study joint AI/ML processing of multiple functionalities in 6GR and identify the use cases that can be achieved through coupling two or more independent functionalities, e.g., joint CSI compression and prediction, joint modulation and precoding, joint channel estimation and demodulation.

- Discussion on Unified Framework

In 5GA, SA and RAN have studied AI/ML framework separately and coordination among WGs is generally missing. The relative lack of coordination was coming from the fact that the AI/ML involves both air interface performance improvements (e.g., AI/ML for PHY, mobility led by RAN1/2, AI/ML for NG-RAN led by RAN3) and network management functionality (e.g., AI/ML management led by SA5). Both aspects could be independently discussed to focus on what we want for each and with coordination after deciding some technical details.

Since AI/ML framework provides high-level principle on workflows and functional components for AI/ML model, in order to generate a unified AI/ML tool in 3GPP that can help enhance system performance and maintain operation cost-effectiveness, coordination on this among different WGs from the beginning, including between SA and RAN and also RAN internally, is essential and important. Therefore, we suggest the following.

Study 6G AI/ML framework by coordinating with SA from the beginning.

In particular, lessons learnt from 5GA indicate that the support of AI/ML enabled functionality, especially when UE side is involved, e.g., data collection/data transfer/model transfer to/from UE, requires corresponding SA solutions being ready at first. Moreover, as more use cases become deeply integrated with AI/ML, a unified data collection framework across all use cases is essential to reduce overhead and complexity associated with data generation, storage, and transfer.

Moreover, even within RAN1, LCM framework is designed per use case for UE sided AI/ML functionalities such as spatial-domain and time-domain beam prediction, CSI prediction and positioning. In Rel-20, LCM framework will be designed for CSI compression with two sided AI/ML model. It can be expected that LCM for two sided models will be different from the LCM for one sided model. In addition, RAN1 LCM is for UE-sided model, and different WGs (for example, RAN1 and RAN3, RAN WGs and SA WGs) have different LCM frameworks. It could be a huge and unnecessary workload if LCM will be discussed and re-designed for each single use case. Therefore, in 6GR, a unified framework is preferred, especially considering the possible AI/ML node which manages all AI/ML related functionalities, models, datasets. Given this context, it may be worthwhile to consider where such an AI/ML node should be located, whether in the RAN domain, CN domain, or elsewhere. Depending on its placement, the node could play a central role in coordinating AI/ML functionalities and facilitating the unified framework.

Further, in Rel-19, AI/ML processing units are introduced to reflect the AI/ML processing capacity for AI/ML based BM/CSI, i.e., MIMO cases involving CSI report. The same concept of arranging AI/ML processing capability is needed for other use cases not involving CSI report, such as the AI/ML based RS overhead reduction, AI/ML based channel estimation, etc., as discussed before. And it is nature that AI/ML MIMO and non-MIMO functionalities may be implemented simultaneously on the same UE. Studies are needed to assign the limited AI/ML processing units to different use cases, especially the enhancement of the APU occupation rules for use cases not involving CSI report.

Building on this, the framework for handling AI/ML processing units must be inherently dynamic. Considering the possibility of multiple 3GPP-defined AI/ML applications running concurrently at the UE, it is crucial to have a dynamic view of the available processing units, as these applications may not occupy resources continuously. Furthermore, this dynamic view must also account for the resource consumption of non-3GPP AI/ML applications running on the device. While the proprietary nature of these applications need not be specified, the framework should be able to recognize and adapt to fluctuations in the UE's processing capability caused by the non-continuous operation of these internal applications. This ensures an accurate assessment of the UE's real-time ability to execute 3GPP-defined functionalities.

Based on above discussions, for unified AI/ML framework, we have the following proposal

Study unified framework for AI/ML for 6GR, ensuring high performance, security, and adaptability to all use cases in 6G, including

Unified data collection framework

Unified LCM framework

Study a unified AI processing capability among multiple AI/ML functionalities, including:

Mechanisms to manage processing resources for multiple, concurrently running 3GPP-defined AI/ML functionalities, considering their non-continuous activity.

Methods to account for the dynamic impact of non-3GPP applications on the UE's available processing capability for 3GPP-defined functionalities.

- Conclusion

In this contribution, we provided our views on 6GR AI/ML for air-interface, and we have the following proposals:

Proposal 1:	Identify existing 5GA use cases and their extensions to be supported in 6G, for example,

−	AI/ML based BM with further enhancements on fast TCI state activation/indication with the predicted beam(s), BFR, MTRP, inter-cell scenarios and Tx-Rx beam pair prediction

−	CSI compression with further enhancements on time domain aspects such as CSI prediction

Proposal 2:	Study AI/ML to reduce RS overhead in 6GR air interface, including the following aspects.

−	Procedures: initial access, BM/CSI, channel estimation

−	Domains: time, frequency, and spatial domain

−	RS types: DMRS, SRS, CSI-RS, and SSB

−	Scenarios: both DL and UL

Proposal 3:	Study AI/ML based channel estimation in 6GR air interface, including the following aspects

−	new DL and UL DMRS patterns, and co-existence with non-AI/ML DMRS

−	processing requirement for PDSCH and PUSCH

−	HARQ ACK enhancement

−	Power allocation

Proposal 4:	Study AI/ML based channel estimation together with channel equalization, including the following options:

−	Option 1: AI/ML-based channel estimation with a traditional channel equalization block.

−	Option 2: A joint AI/ML model for combined channel estimation and equalization.

Proposal 5:	Study AI/ML based modulation and demodulation, including the following options for further study:

−	Option 1: Study an AI/ML-based demodulation scheme at the receiver, assuming a traditional modulation scheme is used by the transmitter.

−	Option 2: Study a two-sided AI/ML-based modulation and demodulation scheme where both transmitter and receiver utilize AI/ML models.

Proposal 6:	Study joint AI/ML processing of multiple functionalities in 6GR and identify the use cases that can be achieved through coupling two or more independent functionalities, e.g., joint CSI compression and prediction, joint modulation and precoding, joint channel estimation and demodulation.

Proposal 7:	Study 6G AI/ML framework by coordinating with SA from the beginning.

Proposal 8:	Study unified framework for AI/ML for 6GR, ensuring high performance, security, and adaptability to all use cases in 6G, including

−	Unified data collection framework

−	Unified LCM framework

Proposal 9:	Study a unified AI processing capability among multiple AI/ML functionalities, including:

−	Mechanisms to manage processing resources for multiple, concurrently running 3GPP-defined AI/ML functionalities, considering their non-continuous activity.

−	Methods to account for the dynamic impact of non-3GPP applications on the UE's available processing capability for 3GPP-defined functionalities.

- References

- RP-251881, New SID: Study on 6G Radio, 3GPP TSG RAN Meeting #108, Prague, Czech Republic, June 9-13, 2025

- RP-251870, New WI: Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface enhancements, 3GPP TSG RAN Meeting #108, Prague, Czech Republic, June 9-13, 2025