3GPP TSG RAN WG1 #122	R1-2505935

Bengaluru, India, Aug 25th – 29th, 2025

Agenda item:	10.1.1.1

Source:	NEC

Title:              	Discussion on inference related aspects of CSI compression

Document for:	Discussion and Decision

- Introduction

Regarding CSI feedback enhancement, in Release 18 and Release 19, extensive studies on CSI compression with two-sided model have been conducted, covering trade-off between performance and complexity, inter-vendor training collaboration and potential specification impact. Based on these comprehensive research findings, it was agreed in RAN#108 [1] that CSI spatial/frequency compression without temporal aspects (“Case 0”) would be included in Release 20 WID, which primarily includes normative work in areas such as model pairing, inference aspects, NW and UE side data collection for training and performance monitoring. In this contribution, we provide our views on the potential specification impacts related to inference aspects.

CSI feedback enhancement, encompassing (two-sided model) [RAN1, RAN2]: - CSI spatial/frequency compression without temporal aspects (“Case 0”), - Specify necessary signalling/mechanism(s) as per the identified potential specification impacts in FS_NR_AIML_Air , including: - Model pairing procedure including ID and applicability reporting - Inference aspects including target CSI type, measurement and report configuration, CQI RI determination, payload determination, quantization configuration codebook, UCI mapping, CSI processing criteria and timeline, priority rules for CSI reports - NW and UE side data collection for training, - Target CSI format - Note The framework defined in BM and CSI prediction use cases could be reused - Specify performance monitoring

- Discussion

  - Target CSI type

For CSI spatial/frequency compression without temporal aspects, based on our understanding, the target CSI serves as the input of the encoder. Moreover, the input for CSI spatial/frequency compression can be a precoding matrix (or eigenvector), and the dimension of the input should correspond to the number of antenna ports and the number of subbands, a point that has been widely acknowledged and validated by most companies during the evaluation phase. Therefore, we believe that for CSI spatial/frequency compression without temporal aspects, the target CSI type should inherently be a precoding matrix in spatial/frequency domain, rather than the precoding matrix in angular-delay domain. As for using the raw channel as the target CSI, we believe this would significantly degrade compression efficiency and increase computational complexity.

Proposal 1: In CSI spatial/frequency compression without temporal aspects, support precoding matrix (or eigenvector) in spatial-frequency domain as target CSI type.

  - Measurement and report configuration

In CSI spatial/frequency compression without temporal aspects, since AI/ML-based CSI report (i.e., CSI report based on CSI compression) and non-AI/ML-based CSI report differ in terms of CSI computation, reporting mechanism, UCI mapping, processing criteria/timeline and priority/omission rule, it is necessary to introduce new report quantity at least for the compressed PMI in the AI/ML-based CSI-ReportConfig, such as ‘cri-RI-cPMI-CQI’.

Proposal 2: In CSI spatial/frequency compression, introduce new reportQuantity for compressed PMI in CSI-ReportConfig.

A quantization configuration codebook is a structured reference used in CSI compression to map continuous-valued channel data into discrete quantized levels for efficient transmission. It defines a set of quantization levels, each associated with a specific bit pattern, enabling both the transmitter and receiver to interpret compressed CSI consistently. By standardizing how CSI values are encoded and decoded, the quantization codebook plays a crucial role in balancing compression efficiency, feedback overhead, and channel fidelity in wireless communication systems. In CSI spatial/frequency compression, it is recommended to incorporate support for quantization codebook configurations to enhance flexibility and performance.

Non-AI/ML-based CSI report is codebook-based, with the codebook configuration (i.e., codebookConfig) provided by NW. For AI/ML-based CSI report, the compressed bits (or output of CSI generation part) need to be converted into a form transmissible over the air interface using specific quantization approaches, e.g., SQ, VQ. In principle, the codebook in non-AI/ML-based CSI report is itself a specific quantization approach. Therefore, since the quantization at UE side and the dequantization at NW side must be aligned, NW may need to provide UE with quantization (e.g., SQ, VQ) related configuration, which could be achieved by introducing a new codebook.

Proposal 3: In CSI spatial/frequency compression, introduce new codebookConfig in CSI-ReportConfig to provide quantization (e.g., SQ, VQ) related configuration.

  - CQI/RI determination

Agreement In CSI compression using two-sided model use case, further study the following options for CQI determination in CSI report, if CQI in CSI report is configured. - Option 1: CQI is NOT calculated based on the output of CSI reconstruction part from the realistic channel estimation, including - Option 1a: CQI is calculated based on target CSI with realistic channel measurement - Option 1b: CQI is calculated based on target CSI with realistic channel measurement and potential adjustment - Option 1c: CQI is calculated based on legacy codebook - Option 2: CQI is calculated based on the output of CSI reconstruction part from the realistic channel estimation, including - Option 2a: CQI is calculated based on CSI reconstruction output, if CSI reconstruction model is available at the UE and UE can perform reconstruction model inference with potential adjustment - Note: CSI reconstruction part at the UE can be different comparing to the actual CSI reconstruction part used at the NW. - Option 2b: CQI is calculated using two stage approach, UE derive CQI using precoded CSI-RS transmitted with a reconstructed precoder. - Other options are not precluded - Note1: feasibility of different options should be evaluated - Note2: Gap analyses between the UE side CQI calculation results and the NW side results, as well as the impact on the scheduling performance should be evaluated - Note3: Complexity of CQI calculation needs to be evaluated, including the computing complexity and potential RS/signaling overhead

Regarding CQI determination in CSI report, multiple options mentioned above were agreed upon in Release 18. However, no progress has been made thus far in down-selection.

For legacy codebook based CSI feedback, UE determines CQI in CSI report based on the determined the precoding matrix indicated by the PMI. For CSI compression based CSI feedback, if the (proxy) CSI reconstruction part at UE side is available, and it is associated with the CSI reconstruction part at NW side (e.g., NW transfers a proxy CSI reconstruction part to UE, or UE/UE-side trains a proxy CSI reconstruction part based on the CSI generation part related model structure plus parameter or dataset transferred by NW), a CQI determination method similar to legacy codebook based CSI feedback can be used, i.e., Option 2a. If the (proxy) CSI reconstruction part at UE side is not available, the CQI may not be determined based on the output of CSI reconstruction part from the realistic channel estimation, i.e., Option 1. Therefore, if the proxy CSI reconstruction part at UE side is proven to be feasible, we should at least support Option 2a; if not, support Option 1.

For Option 1, Options 1a and 1b utilize the target CSI, which is naturally available at the UE, and do not cause additional delay, overhead, or computation. The main limitation is that the target CSI may not exactly match the reconstructed CSI at the gNB. Option 1c requires the UE to compute both the AI/ML-based compressed CSI feedback and a legacy codebook entry for CQI calculation, which is considered undesirable due to the added computational burden.

For Option 2, Option 2b requires the NW to transmit precoded CSI-RS, which not only increases signaling overhead but also introduces additional delay in CSI processing, as the UE needs to calculate and report CQI based on the received precoded CSI-RS. Therefore, Option 2b should be deprioritized.

Proposal 4: For CQI determination in CSI report, if a proxy CSI reconstruction part at UE side is proven to be feasible, support Option 2a; if not, support Option 1a/1b.

Proposal 5:For CQI determination, Options 1c and Option 2b can be deprioritized due to the respective disadvantages in computational efficiency and signaling overhead.

  - Payload determination

For CSI spatial/frequency compression, the CSI payload is inherently associated with the spatial-domain and frequency-domain configuration for the CSI report. For instance, since the input dimensions correspond to antenna port and subband, the CSI payload should naturally be associated with the number of antenna ports and the number of subbands.

Furthermore, the CSI payload must undergo quantization, implying that the quantization configuration also influences its determination. Specifically, quantization significantly influences the size and efficiency of the CSI payload in spatial/frequency compression. Lower bit depths, such as 2 or 4 bits per element, result in smaller payloads and reduced feedback overhead, making them ideal for bandwidth-constrained scenarios. However, this comes at the cost of reduced precision, which can degrade the accuracy of channel reconstruction and impact downstream tasks like beamforming. Conversely, higher bit depths, such as 8 or 16 bits, preserve more channel detail and improve reconstruction fidelity but increase the payload size and transmission burden. Therefore, selecting the appropriate quantization level is a critical design choice that balances compression efficiency with performance requirements. The determination of most suitable quantization may also be dynamic and related to the channel quality.

Additionally, the dynamic feedback information within the CSI report must be considered when defining the CSI payload. Specifically, the PMI—whether traditional or compressed—is inherently dependent on the RI for its derivation. Regarding the CQI, our understanding suggests that a lower CQI, indicative of poor SNR conditions, may necessitate careful consideration of the compression ratio. That is, if the compressed CSI employs an excessively high compression ratio (resulting in an insufficient bit allocation), the NW may fail to reconstruct the CSI with adequate accuracy. In other words, in low SNR regime, even assuming same quantization, more bits are required (e.g., a lower compression ratio) to achieve the same SGCS performance than that required in high SNR regime. Consequently, the CQI may also play a role in determining the appropriate CSI payload. A dynamic compression ratio adaptive to the channel quality measured at UE side can achieve better overhead-accuracy trade-off for compressed CSI report.

Proposal 6: In CSI spatial/frequency compression, determination of CSI payload (i.e., compressed CSI) for a CSI report should be based on the following:

- Configuration of spatial/frequency domain for the CSI report, e.g., number of antenna ports, number of subbands.

- Configuration of quantization for the CSI report.

- Dynamic compression ratio and quantization determination based on channel quality in the CSI report, e.g., RI, CQI.

  - Quantization configuration codebook

As mentioned in Section 2.2, by standardizing how CSI values are encoded and decoded, the quantization codebook plays a crucial role in balancing compression efficiency, feedback overhead, and channel fidelity in wireless communication systems. In the study phase, scalar quantization (SQ) and vector quantization (VQ) were identified as two promising quantization schemes. SQ offers the advantages of a simple codebook and low computational complexity; however, its compression efficiency is relatively limited due to its failure to account for data correlation. In contrast, VQ achieves higher compression efficiency by fully leveraging data correlations, but at the cost of increased complexity in codebook design and higher computational overhead. Consequently, a balanced approach would be to support both SQ and VQ, with further research needed to determine the optimal selection criteria between them. Additionally, given that CSI compression is performed on a per-layer basis, the strategy of quantization per layer could be explored to maximize compression accuracy and efficiency while minimizing feedback overhead.

Proposal 7: In CSI spatial/frequency compression, for quantization schemes, support both SQ and VQ.

Proposal 8: In CSI spatial/frequency compression, support quantization per layer for compressed bits.

  - UCI mapping

Agreement In CSI compression using two-sided model use case, for the study of UCI format, consider the legacy CSI reporting principle with CSI Part 1 and Part 2 as a starting point, where Part 1 has a network configured fixed size and Part 2 size is dynamic, determined by information in Part 1. Agreement - In CSI compression using two-sided model use case, for CSI report format, when output-CSI-UE and input-CSI-NW is precoding matrix, CSI part 1 includes at least CQI for first codeword, RI, and information representing the part 2 size. CSI part 2 includes at least the content of CSI generation part output. - Other CSI report formats are not precluded

Two-part (i.e., CSI Part 1 and CSI Part 2) is used for legacy CSI report for CSI acquisition. The CSI Part 1 includes information with fixed bitwidth, such as CRI, RI, and CQI (for first codeword), while the CSI Part 2 includes dynamically sized information, such as PMI. For AI/ML-based CSI report, the two-part can be fully reused as the UCI/report format. The only difference is that the CSI Part 2 includes compressed PMI instead of legacy PMI. Additionally, Part 1 may incorporate certain information determining the payload size of the compressed PMI, which would depend on the outcome of the discussion in Section 2.4 Payload determination.

Proposal 9: In CSI spatial/frequency compression, support two-part as CSI report format. The CSI Part 1 includes CRI, RI, CQI (for first codeword) and information determining the payload size of the compressed PMI, while CSI Part 2 includes compressed PMI.

- Whether/which information determining the payload size of the compressed PMI is included in the CSI Part 1 depends on the outcome of discussion about ‘payload determination’.

  - CSI processing criteria and timeline

Agreement Introduce a dedicated AI/ML PU for AI/ML features for UE, - The AI/ML PU is used at least for quantifying the simultaneous processing of multiple CSI reports subject to CSI-related AI/ML use case(s), e.g., CSI compression (if supported), CSI prediction, BM spatial prediction, BM temporal prediction. Agreement For CSI prediction using UE-side model, to calculate the inference report using Doppler codebook, - For PU occupancy, support - Dedicated AI/ML PU (OAPU) and/or legacy CPU (OCPU) are occupied, - OAPU= 0 or N is reported by UE - OCPU=0 or M is reported by UE - Note: Detailed values of N and M can be further discussed in UE feature. - Note: Combination of OAPU= 0 and OCPU=0 is not allowed - Note: if any of the unoccupied PU cannot satisfy the corresponding required PU by the CSI report, the CSI report will follow the legacy behaviour of exceeding the CPU limit, neither of the PUs are occupied - For occupancy duration of CPU and APU, - the same occupancy duration is used if both CPU and APU are reported non-zero value - if associated monitoring report is not configured, reuse following legacy occupancy duration - For semi-persistent CSI report on PUSCH with P/SP CSIRS CMR, occupation starts from the first symbol of KP-th latest consecutive P/SP-CSI-RS occasions no later than CSI reference resource, until the last symbol of the PUSCH carrying the report, where KP is indicated by UE capability. - Note: Detailed values of KP can be further discussed in UE feature. - Aperiodic CSI report occupies PU(s) from the first symbol after the PDCCH triggering the CSI report until the last symbol of the scheduled PUSCH carrying the report. - The total number of dedicated AI/ML PU for AI/ML is reported by UE capability Agreement For UE-sided model, regarding a CSI report with CSI-ReportConfig for inference for BM-Case1, - Rel-15 CPU occupation time is reused for CPU occupation time of the CSI report - Rel-15 CPU occupation time is reused for AI/ML PU occupation time of the CSI report - Note: this is applicable to all types of CSI reports (i.e., AP/SP/P CSI report) Agreement For UE-sided model, regarding a CSI report with CSI-ReportConfig for inference for BM-Case1 and BM-Case 2, when applicable, extend legacy Z3/Z3’ to Z3+d / Z3’+d’, where d and d’ are reported by UE per SCS for BM-Case 1 and BM-Case 2 respectively - Detailed values of d and d’ can be further discussed in UE feature.

In Rel-19, extensive discussions were conducted regarding the CSI processing criteria and timeline for beam prediction and CSI prediction, culminating in the aforementioned agreement. From our understanding, for CSI spatial/frequency compression, at least for model inference:

- For PU occupancy, the PU occupancy for CSI prediction can be as a starting point.

- For CPU and/or APU occupancy, the CPU occupancy time for CSI report with reportQuantity set to 'cri-RI-PMI-CQI' can be reused.

- For CSI computation timeline, the CSI computation timeline for BM-Case1 in Rel-19 can be as a starting point. For instance, extend legacy Z2/Z2’ to Z2+d / Z2’+d’, where d and d’ are reported by UE per SCS for CSI spatial/frequency compression. Detailed values of d and d’ can be further discussed in UE feature.

Proposal 10: In CSI spatial/frequency compression, to calculate the inference report for compressed CSI,

- For PU occupancy, support:

  - Dedicated AI/ML PU (OAPU) and/or legacy CPU (OCPU) are occupied,

  - OAPU= 0 or N is reported by UE

  - OCPU=0 or M is reported by UE

  - Note: Detailed values of N and M can be further discussed in UE feature.

  - Note: Combination of OAPU= 0 and OCPU=0 is not allowed

  - Note: if any of the unoccupied PU cannot satisfy the corresponding required PU by the CSI report, the CSI report will follow the legacy behaviour of exceeding the CPU limit, neither of the PUs are occupied

- For occupancy duration of CPU and APU,

  - the CPU occupancy time for CSI report with reportQuantity set to 'cri-RI-PMI-CQI' can be reused

  - the same occupancy duration is used if both CPU and APU are reported non-zero value

Proposal 11: In CSI spatial/frequency compression, for a CSI report for inference, extend legacy Z2/Z2’ to Z2+d / Z2’+d’, where d and d’ are reported by UE per SCS for CSI spatial/frequency compression.

- Detailed values of d and d’ can be further discussed in UE feature.

  - Priority/omission rules for CSI reports

In CSI spatial/frequency compression, for model inference, the introduction of new reporting quantity (i.e., compressed CSI) necessitates the definition of additional new priority rule for inference report (i.e., AI/ML-based report) and non-AI/ML-based report. Furthermore, for NW-side performance monitoring, UE is required to report both target CSI and compressed CSI. It is noteworthy that the compressed CSI reported for performance monitoring differs from that reported during model inference. For UE-side performance monitoring (if the proxy CSI reconstruction part is proven to be feasible), UE needs to report performance metric (e.g., SGCS, NMSE), thereby requiring the introduction of additional new CSI priority rule for inference report and monitoring report.

Proposal 12: In CSI spatial/frequency compression, support to define additional new priority rule for the following collisions:

- AI/ML-based CSI report and non-AI/ML-based CSI report.

- Inference CSI report and monitoring CSI report.

When the uplink resources are insufficient to transmit an entire CSI report, it becomes necessary to omit portions of the CSI through efficient means. For the non-AI/ML-based CSI report, subband based priority/omission rule is used. However, for CSI spatial/frequency compression based CSI report, subband based priority/omission rule may not be applicable because the precoding matrices corresponding to all subbands of a given layer are jointly compressed into a latent space. To address this issue, two CSI omission methods as follows were proposed in Release 18:

- Layer-based priority rule.

- CSI payload reduction for layer(s), e.g., different quantization method/granularity, different size of latent space, or CSI puncturing/truncating.

From our perspective, layer-based priority rule implies the complete omission of the compressed CSI for an entire layer. While this approach is straightforward, its omission granularity is excessively coarse, making it an overly simplistic and somewhat reckless solution. CSI payload reduction for layer(s) appears to be a novel technique tailor-made for AI/ML-based CSI report, offering the potential for bit-level omission. It effectively circumvents the issues that layer based priority rule might entail. Moreover, if acceptable performance can be achieved after CSI puncturing/truncating, this avenue holds significant promise. Therefore, in CSI spatial/frequency compression, for CSI omission, CSI payload reduction for layer(s) should be supported.

Proposal 13: In CSI spatial/frequency compression, for CSI omission, support CSI payload reduction for layer(s), e.g., CSI puncturing/truncating.

- Conclusion

In this contribution, we provide our views on the potential specification impacts related to inference aspects for CSI spatial/frequency compression without temporal aspects. Specifically, we have the following proposals:

Proposal 1: In CSI spatial/frequency compression without temporal aspects, support precoding matrix (or eigenvector) in spatial-frequency domain as target CSI type.

Proposal 2: In CSI spatial/frequency compression, introduce new reportQuantity for compressed PMI in CSI-ReportConfig.

Proposal 3: In CSI spatial/frequency compression, introduce new codebookConfig in CSI-ReportConfig to provide quantization (e.g., SQ, VQ) related configuration.

Proposal 4: For CQI determination in CSI report, if a proxy CSI reconstruction part at UE side is proven to be feasible, support Option 2a; if not, support Option 1a/1b.

Proposal 5:For CQI determination, Options 1c and Option 2b can be deprioritized due to the respective disadvantages in computational efficiency and signaling overhead.

Proposal 6: In CSI spatial/frequency compression, determination of CSI payload (i.e., compressed CSI) for a CSI report should be based on the following:

- Configuration of spatial/frequency domain for the CSI report, e.g., number of antenna ports, number of subbands.

- Configuration of quantization for the CSI report.

- Dynamic compression ratio and quantization determination based on channel quality in the CSI report, e.g., RI, CQI.

Proposal 7: In CSI spatial/frequency compression, for quantization schemes, support both SQ and VQ.

Proposal 8: In CSI spatial/frequency compression, support quantization per layer for compressed bits.

Proposal 9: In CSI spatial/frequency compression, support two-part as CSI report format. The CSI Part 1 includes CRI, RI, CQI (for first codeword) and information determining the payload size of the compressed PMI, while CSI Part 2 includes compressed PMI.

- Whether/which information determining the payload size of the compressed PMI is included in the CSI Part 1 depends on the outcome of discussion about ‘payload determination’.

Proposal 10: In CSI spatial/frequency compression, to calculate the inference report for compressed CSI,

- For PU occupancy, support:

  - Dedicated AI/ML PU (OAPU) and/or legacy CPU (OCPU) are occupied,

  - OAPU= 0 or N is reported by UE

  - OCPU=0 or M is reported by UE

  - Note: Detailed values of N and M can be further discussed in UE feature.

  - Note: Combination of OAPU= 0 and OCPU=0 is not allowed

  - Note: if any of the unoccupied PU cannot satisfy the corresponding required PU by the CSI report, the CSI report will follow the legacy behaviour of exceeding the CPU limit, neither of the PUs are occupied

- For occupancy duration of CPU and APU,

  - the CPU occupancy time for CSI report with reportQuantity set to 'cri-RI-PMI-CQI' can be reused

  - the same occupancy duration is used if both CPU and APU are reported non-zero value

Proposal 11: In CSI spatial/frequency compression, for a CSI report for inference, extend legacy Z2/Z2’ to Z2+d / Z2’+d’, where d and d’ are reported by UE per SCS for CSI spatial/frequency compression.

- Detailed values of d and d’ can be further discussed in UE feature.

Proposal 12: In CSI spatial/frequency compression, support to define additional new priority rule for the following collisions:

- AI/ML-based CSI report and non-AI/ML-based CSI report.

- Inference CSI report and monitoring CSI report.

Proposal 13: In CSI spatial/frequency compression, for CSI omission, support CSI payload reduction for layer(s), e.g., CSI puncturing/truncating.

- References

- RP-251870, New WI: Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface enhancements, Qualcomm