3GPP TSG RAN WG1 #122						        R1- 2505949

Bengaluru, India, Aug 25th – 29th, 2025

Agenda Item:	10.6.1

Source: 	LG Electronics

Title: 	Discussion on GNSS resilient NR-NTN operation

Document for:	Discussion and decision

- Introduction

In RAN#108, Rel-20 NTN SI objectives for GNSS resilient NR-NTN operation were agreed as follows [1]:

The study will be carried assuming that - The GNSS information in UE with GNSS capability may be 1/ temporarily unavailable or 2/ available but with GNSS position accuracy degradation or 3/ available but with increased GNSS measurement period for power saving purpose - This study will initially focus on 1/ (where GNSS information is temporarily unavailable) - Note: This study does not include any enhancement on positioning for the above scenario Study GNSS resilient NR-NTN operation [RAN1, RAN4] - Assess impact on initial access and connected mode procedures for NR-NTN, including potential implications if any on RAN4 RRM specifications - Aim to minimize physical layer procedures impact - Avoid physical layer channel/signal changes - Backward compatible issues with legacy NTN capable UEs will be assessed and should be prevented - Applicable to NGSO and GSO constellations, NTN operating in below and in above 10 GHz frequency bands, transparent and regenerative satellites - Check in RAN#112 (June, 2026) to decide whether to proceed with normative work, extend the study item, or postpone normative work to Release 21

This contribution discusses the potential impacts on initial access and connected mode procedures when GNSS resilient NR-NTN operation is introduced.

- Discussion

  - GNSS-resilient operation in initial access

To support GNSS-resilient operation for initial access, network should provide reference information for time offset (TO) and/or frequency offset (FO) pre-compensation. This is because UE-specific TA for Msg1 (i.e., PRACH) transmission is based on GNSS-based UE location, which cannot be applied when GNSS information is unavailable, and therefore alternative information is required when GNSS-resilient operation is supported. For example, the network may provide reference location which the satellite beam is directed and/or applicable value for TO/FO pre-compensation to the UE. Then, the UE can perform TO/FO pre-compensation using the reference location/value transmitting Msg1 (i.e., PRACH) (denoted by “reference TO/FO pre-compensation” hereafter). However, in scenarios with large beam footprint size, changes to the physical channel (i.e., PRACH) design may be required even when using the reference TO/FO pre-compensation. For example, in a GEO scenario, the residual TO can be up to 3.27 msec since the maximum differential delay within a beam footprint with 250 km diameter would be 1.63 msec even after the reference TO/FO pre-compensation.

Observation #1: To support GNSS-resilient operation during initial access, the network should provide reference location/value for TO/FO pre-compensation before Msg1 (PRACH) transmission.

Observation #2: To support GNSS-resilient operation during initial access, depending on the satellite beam and/or cell size, change to Msg1 (PRACH) design may be required.

Moreover, GNSS-resilient operation for the initial access process can lead to further specification changes when considering FO correction. For example, assume that the UE transmit Msg1 using the reference TO/FO pre-compensation. In this case, residual TO/FO values may exist due to the difference between the UE location and the reference location. Since the residual TO/FO for Msg3 should be kept small enough to be multiplexed with normal data transmission, the network should provide information to correct the detected residual TO/FO upon receiving Msg1. However, the conventional Msg2 (i.e., RAR) can only provide TO related information (e.g., TA) to the UE and does not support fields that can provide FO-related information. Alternatively, a downlink synchronization signal capable of estimating residual FO information can be linked to the Msg3 transmission resource, separate from the residual TO correction. However, this approach imposes constraints on the timing of synchronization signals and Msg3 transmission, and can have significant impacts on UE operation.

Observation #3: To support GNSS-resilient operation during initial access, the network should be able to provide residual FO correction information/resource to the UE before transmitting Msg3.

Overall, supporting GNSS-resilient operation during initial access may require many physical layer channel and/or physical layer procedure changes compared to legacy specification. For example, this may require that the network provides TO/FO pre-compensation information such as reference location/value, and even if the TO/FO pre-compensation information is provided, reception performance may still deteriorate without a design change of Msg1 due to residual TO/FO. Moreover, it may require additional compensation processes for residual TO and/or FO should be supported during the RACH process. Therefore, it may be reasonable to deprioritize GNSS-resilient operation during the initial access process in Rel-20 NR NTN. For 6GR, GNSS-free operation, including the initial access process, can be discussed in 6GR SI, which discusses TN/NTN combination from the beginning.

Proposal #1: In Rel-20 NR-NTN, GNSS-resilient support during initial access is deprioritized.

  - GNSS-resilient operation in connected mode

To support GNSS-resilient operation for connected mode, the existing GNSS enhancements for Rel-17/18 IoT NTN should be considered [2]. This is because the scenario for the GNSS enhancements is same with the one of scenarios in NR-NTN SI (e.g., scenario #1) and hence the solution may also valid for the NR-NTN with little specification works. For example, UL synchronization can be maintained by network TA command and time alignment timer (TAT) when the UE is temporarily unable to utilize GNSS information in connected mode. In addition, the UE can report GNSS assistance information including GNSS validity duration. Therefore, GNSS enhancement techniques in IoT NTNs should be considered as a starting point for supporting GNSS-resilient operation in connected mode in NR-NTN.

Proposal #2: In Rel-20 NR-NTN, RAN1 studies GNSS-resilient operation in connected mode considering the existing GNSS enhancements for IoT-NTN as a starting point:

- Close loop control for TO

- GNSS assistance information report (e.g., GNSS validity duration)

Meanwhile, in IoT-NTN, the GEO scenario was mainly assumed, but in NR-NTN, not only the GEO but also the LEO scenario should be considered and improvement techniques targeting the corresponding scenario should be discussed. For example, when considering high-speed scenarios such as LEO, the process of the network tracking TO changes by providing TA commands every time can be inefficient. In such cases, the network can additionally provide reference location/value information for TO/FO correction, and the UE can perform coarse TO/FO correction based on this information (i.e., reference TO/FO pre-compensation). Then, additional close loop control can be performed for fine TO/FO correction on top of the reference TO/FO pre-compensation. For FO pre-compensation, the network may provide information about the pre-compensation jointly or separately from TO. For example, the network can provide downlink reference signal which can be used for FO pre-compensation of uplink transmission.

Proposal #3: In Rel-20 NR-NTN, RAN1 studies GNSS-resilient operation in connected mode considering the following enhancements for TO/FO pre-compensation:

- Reference location based TO/FO pre-compensation

- Close loop control for TO/FO on top of the reference TO/FO pre-compensation

  - Other aspects

RAN1 needs to study how UE can detect cases where GNSS positioning accuracy is low. This is because the Rel-20 NR-NTN SID considers the scenario where the UE can use GNSS information but the GNSS position accuracy is low (e.g., scenario #2). Here, the determination of low GNSS positioning accuracy can be based on errors in TO/FO pre-compensation rather than positional deviation. For example, if the change in TO/FO due to GNSS positioning accuracy errors exceeds a threshold set by the network, the UE may determine that GNSS-based TO/FO pre-compensation is unavailable. Upon making this determination, the UE may request support for GNSS-resilient operation from the network.

Proposal #4: In Rel-20 NR-NTN, RAN1 studies whether/how to determine GNSS positioning accuracy degradation.

When a UE changes from a GNSS-based TO/FO pre-compensation procedure to a GNSS-resilient TO/FO pre-compensation procedure, RAN1 study is needed on how to handle TO/FO changes due to the change between the two techniques. For example, the accumulated TO/FO by network commands under GNSS-based operation may not be suitable when UE applies close loop control for TO/FO under GNSS resilient operation. Therefore, the UE may need to handle TO/FO values when the TO/FO pre-compensation scheme changes. For example, the UE can control the TO/FO value so that the total TA value is continuous.

Proposal #5: In Rel-20 NR-NTN, RAN1 studies potential impacts from switching between GNSS-based operation and GNSS-resilient operation.

- Conclusions

In this contribution, we discussed GNSS-resilient NR-NTN operation and the following observation(s) and proposal(s) are made:

Observation #1: To support GNSS-resilient operation during initial access, the network should provide reference location/value for TO/FO pre-compensation before Msg1 (PRACH) transmission.

Observation #2: To support GNSS-resilient operation during initial access, depending on the satellite beam and/or cell size, change to Msg1 (PRACH) design may be required.

Observation #3: To support GNSS-resilient operation during initial access, the network should be able to provide residual FO correction information/resource to the UE before transmitting Msg3.

Proposal #1: In Rel-20 NR-NTN, GNSS-resilient support during initial access is deprioritized.

Proposal #2: In Rel-20 NR-NTN, RAN1 studies GNSS-resilient operation in connected mode considering the existing GNSS enhancements for IoT-NTN as a starting point:

- Close loop control for TO

- GNSS assistance information report (e.g., GNSS validity duration)

Proposal #3: In Rel-20 NR-NTN, RAN1 studies GNSS-resilient operation in connected mode considering the following enhancements for TO/FO pre-compensation:

- Reference location based TO/FO pre-compensation

- Close loop control for TO/FO on top of the reference TO/FO pre-compensation

Proposal #4: In Rel-20 NR-NTN, RAN1 studies whether/how to determine GNSS positioning accuracy degradation.

Proposal #5: In Rel-20 NR-NTN, RAN1 studies potential impacts from switching between GNSS-based operation and GNSS-resilient operation.

Reference

[1] RP-251863, “Study on GNSS resilient NR-NTN operation,” Thales (Moderator), 3GPP TSG RAN Meeting #108, Prague, Czech Republic, June 9-13, 2025.

[2] RP-220939, “Revised WID on IoT NTN enhancements,” MediaTek Inc., 3GPP TSG RAN Meeting #95e, Electronic Meeting, March 17-23, 2022.