3GPP TSG RAN WG1 #122												   R1-2506002

Bengaluru, India, Aug 25th – 29th, 2025

Agenda Item:	11.1

Source:	HONOR

Title:	Discussion on overview of 6GR air interface

Document for:	Discussion and Decision

Introduction

In RAN#108, a New SID “Study on 6G Radio” was approved with the following objective [1]:

4 Objective 4.1 Objective of SI or Core part WI or Testing part WI The study aims to develop one non-backward compatible radio access technology (hereafter “6GR”) to meet a broad range of use cases, and requirements as defined during the RAN requirements study [TR38.914]. The study should strive at dimensioning an appropriate set of functionalities, minimizing the adoption of multiple options for the same functionality, focusing on practical user experience. The study should identify principles to ensure extensibility and deliver superior performance. NOTE: Section 4.1.1 shows interim milestones to reach certain agreements, including early consensus items. From a technology perspective, the study will address frequency ranges up to 52.6GHz, including at least the full range of FR1 (up to 7.125GHz), the range between FR1 and FR2-1 (including around ~7GHz), and FR2-1 (24.25 GHz – 52.6GHz). NOTE: TR 38.921, TR 38.922 will be taken into account. NOTE: Frequency ranges beyond 52.6 GHz are not in scope of the work. The detailed objectives of the study are: - Single technology framework based on a stand-alone architecture (Note1) to support the agreed existing and new services, and to satisfy the usage scenarios, requirements, deployment scenarios and design principles with acceptable performance/complexity trade-off, as determined by the RAN requirements in [RP-250810] and [TR38.914], including: [RAN1], [RAN2], [RAN3], [RAN4] - Ensuring appropriate set of functionalities, minimize the adoption of multiple options for the same functionality, avoid excessive configurations, excessive UE capabilities and UE capabilities reporting. - Energy efficiency and energy saving: both for network and device. - Enhanced spectral efficiency. - Enhanced overall coverage, focus on cell-edge performance and UL coverage. - Wider channel bandwidth (at least 200MHz) support for 6G deployments at least above 2 GHz, around 7 GHz. - Re-use of existing 5G mid-band (~3.5GHz) site grid for 6G deployments in at least around 7 GHz and targeting comparable coverage to 5G mid-band. - Target scalable and forward compatible design for diverse device types. - Improved spectrum utilization and operations taking into account diverse spectrum allocations. - Aim at using common 6G Radio design, which meets mobile broadband service requirements as high priority, to also meet vertical needs. - Aim at a harmonized 6G Radio design for TN and NTN, including their integration. - System simplification, including reducing configuration complexity, enabling more efficient Cell/UE management, etc. Note1: the term stand-alone architecture does not imply any particular Core network architecture, which is up to SA2 discussion. - Physical Layer structure for 6GR, - Waveforms (OFDM-based) and modulations. 5G NR Waveforms and modulation should be considered for 6GR and is also the benchmark for other potential proposals. [RAN1, RAN4] - Frame structure, including compatibility with 5G NR to allow for efficient 5G-6G Multi-RAT Spectrum Sharing (MRSS). [RAN1] - Channel coding, using LDPC and Polar Code as baseline, considering applicable extensions to satisfy 6G requirements and characteristics with acceptable performance/complexity trade-off [RAN1] - Channel Bandwidth (at least minimum and maximum), Numerology, avoiding multiple numerologies for the same band / sub-range (e.g., enabling synergies among frequency bands in the ~7GHz range) [RAN1, RAN4] - Physical layer control, data scheduling and HARQ operation [RAN1, RAN2] - MIMO operation [RAN1, RAN4] - Duplexing [RAN1, RAN4] - Initial access [RAN1, RAN2, RAN4] - Studies on synchronization signal and raster, broadcast signals/channel and physical random access channel [RAN1, RAN4] - Studies on initial access procedure, random access procedures, system information and paging [RAN2, RAN1, RAN4] - 6GR spectrum utilization and aggregation. [RAN1, RAN2, RAN4] - Other physical layer signals, channels and procedures [RAN1, RAN2, RAN4] - Evaluate performance of at least energy efficiency, spectrum efficiency, and coverage compared to 5G NR, and deliver the initial result at the end of study [RAN1]. - RAN4 can be involved, if necessary, based on the LS from RAN1 - Radio interface protocol architecture and procedures for 6GR [RAN2, RAN1, RAN4, RAN3] - User Plane architecture and protocol design [RAN2] - Control Plane architecture (including RRC states) and protocol design [RAN2, RAN3] - Access stratum security aspects, in alignment with requirements from SA3 [RAN2] - Radio signalling framework for UE capabilities, aiming at improvements and simplification compared to 5G NR [RAN2, RAN1, RAN4] - Data transfer design to support various types of data (e.g. AI/ML, Sensing, etc) [RAN2, RAN3] - Mobility for 6GR (for all RRC states), including related RRM [RAN2, RAN1, RAN4, RAN3] - 6GR core and performance requirements - General RF aspects [RAN4] - Intra-3GPP co-existence studies - Study RF related system parameters and requirements - BS RF requirement aspects including band [RAN4] - BS RF requirement and testing framework aiming at improvements and/or simplification compared to 5G NR, including MSR and AAS operation - Study how to improve 6G BS core, conformance specifications, including structure and drafting principles - UE RF requirement aspects including band and band combination [RAN4] - UE RF requirement framework aiming at improvements and/or simplification compared to 5G NR - Study how to improve 6G UE RF specification(s), including structure, drafting principles, and database for band combination - Study UE RF capabilities considering different device types and implementations - RRM aspects for 6GR [RAN4, RAN1, RAN2] - RRM requirement and procedure aspects aiming at improvements and/or simplification compared to 5G NR - Study how to improve 6G requirement specification, including structure and drafting principles - Demodulation and performance aspects [RAN4] - Demodulation and performance requirement framework and key assumptions, aiming at improvements and/or simplification compared to 5G NR for UE and BS - Study how to improve 6G demodulation and performance specifications, including structure and drafting principles for UE and BS - Aspects related to the testability [RAN4] - Testability methodology framework and key assumptions, aiming to ensure that requirements can be properly tested considering the applicability and feasibility of conductive and/or OTA testing with reasonable complexity - Other aspects - Handling irregular channel bandwidths including the definition [RAN4, RAN1] - Definition of ‘frequency range(s)’[RAN4] - Radio Access Network architecture, interface protocols and procedures considering support of various services and functionalities (e.g. AI/ML, sensing, etc). [RAN3, RAN2] - Overall RAN architecture aspects [RAN3, RAN2] - RAN-CN functional split, interface, protocol stack and procedures [RAN3] - RAN internal functional split, interfaces, protocol stacks and procedures, [RAN3, RAN2] - Migration from 5G NR to 6GR as well as interworking and mobility between 5G NR and 6GR: - 5G-6G Multi-RAT Spectrum Sharing for migration [RAN1, RAN2, RAN4, RAN3] - Study if any additional migration mechanism is necessary. [RAN] [RAN2, RAN1, RAN3, RAN4] NOTE: the start of this study objective (b) should be triggered by RAN plenary in time to guarantee proper completion of the WG study. - Mobility between 5G NR and 6GR [RAN2, RAN3, RAN4] Note: Inclusion of LTE/6G interworking/coexistence aspects may be further discussed based on the requirement from RAN plenary - AI/ML for 6GR and Radio Access Network, leveraging 5G AI/ML framework, as appropriate [See TR38.843] [RAN1, RAN2, RAN3, RAN4] - Identify Use Case(s) of interest (either existing or new) with compelling trade-off between e.g., performance, complexity, etc… Coordinated discussion needs to be ensured with related design areas, where needed (e.g., MIMO, Mobility, etc…) NOTE: lead WG depends on the use case. - AI/ML framework: Extensible AI/ML enablers based on the identified Use Case(s), including - LCM procedures [RAN2, RAN1, RAN3, RAN4] - Data collection and data management, in coordination with SA WGs [RAN2, RAN3, RAN1] Note: NW for AI is assumed to be covered by new services 6GR and RAN design shall ensure that the 6G System can also operate without AI/ML - Sensing – Studies to be based on use cases and associated requirements, as defined in [TR38.914] - PHY functions and procedures for sensing technology (e.g., waveform. reference signals, measurement feedback, etc…) [RAN1, RAN4] - Evaluate sensing performance and if necessary, extend channel modelling, for the selected use cases [RAN1] - Aspects of integration with communication services [RAN1] - higher layer procedures and protocol aspects [RAN2] - RAN4 aspects of sensing including RF, coexistence, and testability in coordination with other WGs [RAN4] Note: RAN1 identify detailed requirements, if triggered by TSG RAN NOTE: Coordination with TSG-SA and SA WGs will be done, as necessary.

Based on the SID, we will discuss some aspects and design proposals on 6GR air interface in this contribution.

Discussion on designing a single RAT to serve diverse devices

6G is expected to support a large variety of services ranging from LPWA to the extremely high data-rate and low-latency mobile broadband required by e.g. XR. Designing a single RAT to serve diverse devices in 6G can avoid market fragmentation and form scale advantages of the industry. Thus designing a single RAT for serving diverse devices is an outstanding issue in 6GR. This can be done with a future-proof design of initial access with narrow bandwidth essential channel(s)/signal(s), not necessarily supporting all possible devices (optimally) on Day 1. To reduce the complexity of 6GR specifications, only necessary device categories requested by commercial scenarios are considered on 6G Day 1.

Proposal 1: 6G Day 1 supports unified RAT for different device types ranging from high-performance FWA, smart-phones, wearables, IoT, etc.

Proposal 2: Only necessary device categories requested by commercial scenarios are considered on 6G Day 1.

Discussion on operation of bandwidth

On the high end, wider CBW (and 8K or larger FFT size) is a key aspect to improve performance and user experience. To enable higher data rates and capacity that would be essential for support of immersive communications and real-time services, wider channel bandwidth i.e. 200MHz should be supported in 6GR. Considering the implementation complexity of NW and UE, channel bandwidth shouldn’t be too large.

Proposal 3: Wider channel bandwidth i.e. 200MHz should be supported in 6GR.

The operations of bandwidth in NR include variable carrier bandwidth, carrier aggregation, and BWP activation and deactivation. Carrier aggregation is an effective method to obtain wider bandwidth requested by high data rate service. SCell activation and deactivation in CA operation need long time, burst broadband services cannot be well supported with CA operation. To support high data rate service, wider carrier bandwidth i.e. 200MHz is another reasonable means.

NR supports the transmission bandwidth up to several 100MHz on a single carrier. This is useful for rapid delivery of large payloads but is not needed for smaller payload sizes or for monitoring the downlink control channels when not scheduled. Hence, NR supports BWP mechanism such that the device can use a narrow bandwidth for monitoring control channels and only open the full bandwidth when a large amount of data is scheduled, thereby reducing the device power consumption.

Proposal 4: The variable carrier bandwidth, CA, BWP used in NR should be the starting point of operations of bandwidth in 6GR.

Discussion on coverage of 6GR

Coverage is one of the key factors that an operator considers when commercializing cellular communication networks due to its direct impact on service quality as well as CAPEX and OPEX. Coverage enhancements were introduced in NR later release from Rel-17. Taking into account a large number of legacy terminals, NR coverage enhancement features has not been deployed in commercial network. Therefore coverage is an important factor on 6G Day 1 spec design.

Proposal 5: Enhanced coverage should be supported on 6G Day 1.

Discussion on initial access and common channel

Initial access including related signals and channels (e.g., synchronization signals, raster configurations, PBCH, PRACH) and procedures (e.g., random access, system information acquisition, paging). Common channels include SSB, CSS PDCCH, SIB PDSCH, PDSCH carrying paging message etc.

The time it takes for cell search and connection establishment greatly affects user experience. Therefore, fast and reliable connection establishment should be supported in 6GR. Considering the universality of the network and the forward compatibility for some new services, unified initial access framework should be used for diverse device types. In order to support NES and NTN beam hopping, enhancements including on demand SSB, on demand SIB1, larger default SSB periodicity were introduced in later releases in NR. These fragmented designs hinder real commercial deployment. Different initial access and common channels can be used for different cell types.

Proposal 6: Initial access and common channel on 6G Day 1 should consider the following aspects:

- Supporting fast and reliable connection establishment

- Should be unified for different device types

- Should be adaptive for different cell types including NES cell, normal cell, NTN cell

Discussion on MRSS

The deployment of 6G networks will be a gradual process, so MRSS from 5G to 6G should be considered in 6GR design. Spectrum sharing unit in time-frequency domain should be designed carefully. In order to support co-existence and spectrum sharing, alignment in time and frequency domain resources with adaptive granularity is needed. On the other hand, signaling should be considered to support spectrum sharing between 5G and 6G.

Proposal 7: In order to support MRSS from 5G to 6G, alignment of the resource grid of 5G and 6G should be considered in 6GR designing.

Proposal 8: A signaling mechanism to support spectrum sharing between 5G and 6G should be considered on 6G Day 1.

Discussion on spectrum efficiency

As service demands increase and spectrum resources become increasingly scarce, improving the spectrum efficiency becomes particularly important in 6GR. To improve spectrum efficiency of 6GR,  technologies including ultra-massive MIMO, higher order modulation, and ultra-dense network can be considered. With the application of massive MIMO, more antenna ports need to be supported, and RS overhead has become an issue that cannot be ignored.

Proposal 9: Higher spectrum efficiency compared with 5G should be supported.

Proposal 10: RS overhead must be reasonably controlled in 6GR

Discussion on duplex types

6G does not have backward compatibility constraints, enabling an opportunity for SBFD and TDD to be unified as a single duplex technology. The integrated frame structure and slot configuration could serve as a starting point for the design. Furthermore, in adherence to the 6G simplification principle, redundant configurations or diversified UE capabilities for identical functionalities should be minimized. For instance:

- A unified PRACH occasion configuration applicable to the SBFD/TDD-integrated frame structure

- All 6G UEs support transmission/reception across SBFD and non-SBFD symbols to satisfy 6G performance requirements.

Proposal 11: Design integrated frame structure and slot configuration signaling for FDD, TDD and SBFD.

UE-side SBFD requires intra-carrier full-duplex capability for UEs. Due to size constraints, the antenna isolation of UE is limited, resulting in more severe self-interference compared with gNB counterparts, which puts requirements on advanced filter designs and interference cancellation algorithms for intra-carrier full-duplex operation. Considering that intra-carrier full duplexing is more challenging than intra-band carrier aggregation with UE-side full duplexing across component carriers, UE-side SBFD is currently difficult to implement. Therefore, we prefer that 6G Day 1 only supports gNB-side SBFD but could maintain forward compatibility for UE-side SBFD considering technology evolution, e.g. reserving configuration bits for UE-side SBFD activation.

Proposal 12: 6G Day 1 should only support gNB-side SBFD while maintaining forward compatibility for potential UE-side SBFD.

Discussion on harmonization of TN and NTN

Non-3GPP NTN technologies have been deployed widely in commercial network. 5G NR introduced the NTN feature in Release 17, but its commercial deployment within 3GPP networks has not achieved significant success. Unified RAT should be supported for both TN and NTN in 6GR. Economies of scale achieved by such unified design with TN can be an advantage against non-3GPP NTN.

TN can provide high capacity, while NTN can provide wider coverage. In order to give full play to the advantages of TN and NTN, the cooperation between TN and NTN is crucial. Handover between TN cell and NTN cell should be enhanced in 6GR. The DC between TN cell and NTN cell should be studied in 6GR.

Proposal 13: Unified RAT should be supported for both TN and NTN in 6GR.

Proposal 14: An enhanced handover mechanism between TN cell and NTN cell should be supported in 6GR from Day 1.

Proposal 15: The DC between TN cell and NTN cell should be studied in 6GR.

Conclusions

In this contribution, we provide our views on NR-NTN downlink coverage enhancements with the following proposals:

Proposal 1: 6G Day 1 supports unified RAT for different device types ranging from high-performance FWA, smart-phones, wearables, IoT, etc.

Proposal 2: Only necessary device categories requested by commercial scenarios are considered on 6G Day 1.

Proposal 3: Wider channel bandwidth i.e. 200MHz should be supported in 6GR.

Proposal 4: The variable carrier bandwidth, CA, BWP used in NR should be the starting point of operations of bandwidth in 6GR.

Proposal 5: Enhanced coverage should be supported on 6G Day 1.

Proposal 6: Initial access and common channel on 6G Day 1 should consider the following aspects:

- Supporting fast and reliable connection establishment

- Should be unified for different device types

- Should be adaptive for different cell types including NES cell, normal cell, NTN cell

Proposal 7: In order to support MRSS from 5G to 6G, alignment of the resource grid of 5G and 6G should be considered in 6GR designing.

Proposal 8: A signaling mechanism to support spectrum sharing between 5G and 6G should be considered on 6G Day 1.

Proposal 9: Higher spectrum efficiency compared with 5G should be supported.

Proposal 10: RS overhead must be reasonably controlled in 6GR

Proposal 11: Design integrated frame structure and slot configuration signaling for FDD, TDD and SBFD.

Proposal 12: 6G Day 1 should only support gNB-side SBFD while maintaining forward compatibility for potential UE-side SBFD.

Proposal 13: Unified RAT should be supported for both TN and NTN in 6GR.

Proposal 14: An enhanced handover mechanism between TN cell and NTN cell should be supported in 6GR from Day 1.

Proposal 15: The DC between TN cell and NTN cell should be studied in 6GR.

- References

- RP-251881	New SID: Study on 6G Radio