3GPP TSG RAN WG1 #122  	  R1-2506084

Bengaluru, India, Aug 25th – 29th, 2025

Source: 	CMCC

Title:	Maintenance of NR-NTN uplink capacity/throughput enhancement

Agenda item:	8.11.3

Document for:	Discussion & Decision

Introduction

Rel-19 WI on Non-Terrestrial Networks (NTN) for NR Phase 3 was updated in RP-243300[1] with the following objectives.

- Uplink Capacity/Cell Throughput Enhancement for FR1-NTN [RAN1, RAN2, RAN4] - Specify Orthogonal Cover Codes (OCC) for DFT-s-OFDM PUSCH at least for multiplexing 2 or 4 UEs when PUSCH repetitions are used - Specify necessary signalling - Update RF requirements accordingly, if needed - Notes for this objective: - The enhancement is not targeting improvements/impacts of MU-MIMO capability - The enhancement is not targeted to PUSCH DMRS - No enhancement for initial access - Enhancements to PRACH are not in scope. - This feature may be applicable for UEs operating in terrestrial networks based on a common design

In this contribution, we discuss the remaining issues of the enhancements of PUSCH multiplexing.

Signaling aspects of PUSCH with OCC

OCC indication/ configuration

In the last meeting, some progress has been made for the configuration and indication of the OCC multiplexing.

Agreement For the OCC length applied for OCC DG PUSCH and CG PUSCH Type 2, support: - Option 3: Candidates values are configured by RRC higher-layer parameter as part of the TDRA table with a new column for configuring the OCC length, and the applied value is indicated by scheduling DCI for DG PUSCH or by activation DCI for CG PUSCH Type 2. - No additional rows for the TDRA table

The next step is to discuss the higher layer parameters for OCC. In the last meeting, RRC parameters were not discussed, due to the unfinished discussion of the OCC indication/ configuration design. As we already agreed, candidate values are configured by RRC higher-layer parameter as part of the TDRA table with a new column for configuring the OCC length, the following table of RRC parameters should be adopted.

Proposal 1:

To support the OCC configuration, the following table of RRC parameters should be adopted.

| RAN2 Parent IE | Parameter name in the spec | New or existing? | Parameter name in the text | Description | Value range | Default value aspect | Per (UE, cell, TRP, …) | UE-specific or Cell-specific |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PUSCH-TimeDomainResourceAllocationList | OCClength-r19 | New | OCClength-r19 | Length of OCC for inter-slot OCC with CG-PUSCH Type 1/2, DG-PUSCH. OCC2 is applied only when the value of AggregationFactor, repK, numberOfRepetitions is equal to or larger than 2. OCC4 is applied only when the value of AggregationFactor, repK, numberOfRepetitions is equal to or larger than 4. | {2, 4} | N/A | Per UE | UE-specific |

Other aspects

Agreement RAN1 assumes that the UE is required to maintain phase continuity and power consistency for the duration of one OCC group with PUSCH. - FFS: under which conditions the above applies, e.g. under the same conditions that phase continuity applies for DMRS bundling

It was agreed that within one OCC group the phase continuity and power consistency should be maintained by UE. The details that which conditions should be applied is still FFS. In the Rel-17 coverage enhancement, the events which will break the power consistency and phase continuity was discussed and captured in TS38.214 clause 6.1.7, which is quoted in the Annex. Considering the current OCC multiplexed PUSCH transmission is based on the PUSCH repetition and consecutive slot-based transmission would be used for the OCC multiplexed PUSCH, then the conditions defined for DMRS bundling can be reused.

Proposal 2:

To maintaining the phase continuity and power consistency for the duration of one OCC group with PUSCH, the conditions of DMRS bundling can be reused. Thus, power control shall only be applied in between OCC groups, no enhancement for power control is supported.

The candidate number of PUSCH repetitions is configured in PUSCH-Allocation-r16 in RRC.

PUSCH-Allocation-r16 ::= SEQUENCE { mappingType-r16 ENUMERATED {typeA, typeB} OPTIONAL, -- Cond NotFormat01-02-Or-TypeA startSymbolAndLength-r16 INTEGER (0..127) OPTIONAL, -- Cond NotFormat01-02-Or-TypeA startSymbol-r16 INTEGER (0..13) OPTIONAL, -- Cond RepTypeB length-r16 INTEGER (1..14) OPTIONAL, -- Cond RepTypeB numberOfRepetitions-r16 ENUMERATED {n1, n2, n3, n4, n7, n8, n12, n16} OPTIONAL, -- Cond Format01-02 ..., [[ numberOfRepetitionsExt-r17 ENUMERATED {n1, n2, n3, n4, n7, n8, n12, n16, n20, n24, n28, n32, spare4, spare3, spare2,spare1} OPTIONAL, -- Cond Format01-02-For-TypeA numberOfSlotsTBoMS-r17 ENUMERATED {n1, n2, n4, n8, spare4, spare3, spare2, spare1} OPTIONAL, -- Need R extendedK2-r17 INTEGER (0..128) OPTIONAL -- Cond MultiPUSCH ]] }

It should be noted that some of the repetition factors are not integer multiples of the OCC length. As RAN1 already agreed slot-level OCC as the OCC scheme for PUSCH, repetition factor ‘n1’ cannot apply OCC and ‘n2’ cannot apply OCC4. When the UE is indicated with repetition factor ‘n1’, the OCC multiplexing is considered as deactivated for this transmission. And the UE will not perform OCC multiplexing of 4 when the repetition factor is indicated as n2 or n3.

Proposal 3:

OCC multiplexing can only be performed when the repetition number is equal or larger than the indicated repetition number. Otherwise, the OCC multiplexing is considered as deactivated.

For repetition factor ‘n3’ and ‘n7’, after mapping to several OCC groups, remainder PUSCHs cannot apply a full OCC sequence. Three options can be considered to solve this problem. The 1st option is to define that the n3 and n7 will not be applied to OCC multiplexing, which is considered as an error case. The 2nd option is that when n3 and n7 is indicated, the OCC multiplexing is considered as deactivated. The 3rd option is to apply the OCC sequence directly and sequentially.

Proposal 4:

For PUSCH repetition factor which is not the multiple of the OCC length, e.g. ‘n3’ and ‘n7’, consider the following options:

- Option 1: The repetition factor which is not the multiple of the OCC length is not allowed when OCC multiplexing enabled.

- Option 2: When n3 and n7 is indicated, the OCC multiplexing is considered as deactivated.

- Option 3: When n3 and n7 is indicated, the OCC sequence is applied sequentially.

Conclusions

In this contribution, we discuss the remaining issues of the enhancements of PUSCH multiplexing. The observations and proposals are listed below.

Proposal 1:

To support the OCC configuration, the following table of RRC parameters should be adopted.

| RAN2 Parent IE | Parameter name in the spec | New or existing? | Parameter name in the text | Description | Value range | Default value aspect | Per (UE, cell, TRP, …) | UE-specific or Cell-specific |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PUSCH-TimeDomainResourceAllocationList | OCClength-r19 | New | OCClength-r19 | Length of OCC for inter-slot OCC with CG-PUSCH Type 1/2, DG-PUSCH. OCC2 is applied only when the value of AggregationFactor, repK, numberOfRepetitions is equal to or larger than 2. OCC4 is applied only when the value of AggregationFactor, repK, numberOfRepetitions is equal to or larger than 4. | {2, 4} | N/A | Per UE | UE-specific |

Proposal 2:

To maintaining the phase continuity and power consistency for the duration of one OCC group with PUSCH, the conditions of DMRS bundling can be reused. Thus, power control shall only be applied in between OCC groups, no enhancement for power control is supported.

Proposal 3:

OCC multiplexing can only be performed when the repetition number is equal or larger than the indicated repetition number. Otherwise, the OCC multiplexing is considered as deactivated.

Proposal 4:

For PUSCH repetition factor which is not the multiple of the OCC length, e.g. ‘n3’ and ‘n7’, consider the following options:

- Option 1: The repetition factor which is not the multiple of the OCC length is not allowed when OCC multiplexing enabled.

- Option 2: When n3 and n7 is indicated, the OCC multiplexing is considered as deactivated.

- Option 3: When n3 and n7 is indicated, the OCC sequence is applied sequentially.

References

- RP-243300, Revised WID: Non-Terrestrial Networks (NTN) for NR Phase 3, RAN#106, Madrid, Spain, December 9-12, 2024

- Chairman’s Notes RAN1#122, St Julian’s, Malta, May 19th – 23rd, 2025

Annex

Descriptions on the conditions of maintaining the power consistency and phase continuity in TS 38.214 section 6.1.7

Events which cause power consistency and phase continuity not to be maintained across PUSCH transmissions of PUSCH repetition type A scheduled by DCI format 0_1, 0_2 or 0_3, or PUSCH repetition Type A with a configured grant, or PUSCH repetition type B or TB processing over multiple slots, or PUCCH transmissions of PUCCH repetition, within the nominal TDW, are: - A downlink slot or downlink reception or downlink monitoring based on tdd-UL-DL-ConfigurationCommon and tdd-UL-DL-ConfigurationDedicated for unpaired spectrum. - For the UE indicating the capability dmrs-BundlingNonBackToBackTX or dmrs-BundlingNonBackToBackTX-PerBC in [13, TS 38.306], the gap between any two consecutive PUSCH transmissions, or the gap between any two consecutive PUCCH transmissions, exceeds 13 symbols for normal cyclic prefix or exceeds 11 symbols for extended cyclic prefix. - For the UE not indicating either of the capabilities dmrs-BundlingNonBackToBackTX or dmrs-BundlingNonBackToBackTX-PerBC in [13, TS 38.306], a non-zero symbol gap is scheduled between any two consecutive PUSCH transmissions or between any two consecutive PUCCH transmissions. - The gap between any two consecutive PUSCH transmissions, or the gap between any two consecutive PUCCH transmissions, does not exceed 13 symbols but other uplink transmissions are scheduled between the two consecutive PUSCH transmissions or the two consecutive PUCCH transmissions. - For PUSCH transmissions of PUSCH repetition type A, or PUSCH repetition type B or TB processing over multiple slots, a dropping or cancellation of a PUSCH transmission according to clause 9, clause 11.1 and clause 11.2A of [6, TS 38.213] or due to cell DRX operation. - For PUCCH transmissions of PUCCH repetition, a dropping or cancellation of a PUCCH transmission according to clause 9, clause 9.2.6 and clause 11.1 of [6, TS 38.213] or due to cell DRX operation. - For any two consecutive PUSCH transmissions of PUSCH repetition type A, or PUSCH repetition type B, and when neither multipanelSchemeSDM nor multipanelSchemeSFN is configured and two SRS resource sets are configured in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with higher layer parameter usage in SRS-ResourceSet set to 'codebook' or 'nonCodebook', a different SRS resource set association is used for the two PUSCH transmissions of PUSCH repetition type A, or PUSCH repetition type B, according to Clause 6.1.2.1. - For any two consecutive PUCCH transmissions of PUCCH repetition, and when a PUCCH resource used for repetitions of a PUCCH transmission by a UE includes first and second spatial relations or first and second sets of power control parameters, as described in [10, TS 38.321] and in clause 7.2.1 of [6, TS 38.213], different spatial relations or different power control parameters are used for the two PUCCH transmissions of PUCCH repetition, according to Clause 9.2.6 of [6, TS 38.213]. - Uplink timing adjustment in response to a timing advance command according to clause 4.2 of [6, TS 38.213]. - Frequency hopping. - For reduced capability half-duplex UEs, - a dropping or cancellation of a PUSCH or PUCCH transmission according to clause 17.2 of [6, TS 38.213] or - an overlapping of the gap between two consecutive PUSCH or two consecutive PUCCH transmissions and any symbol of downlink reception or downlink monitoring The UE shall maintain power consistency and phase continuity within an actual TDW, across PUSCH transmissions of PUSCH repetition Type A scheduled by DCI format 0_1, 0_2 or 0_3, or PUSCH repetition Type A with a configured grant, or PUSCH repetition type B or TB processing over multiple slots, or across PUCCH transmissions of PUCCH repetition, in case the actual TDW is created in response to frequency hopping, or in response to the use of a different SRS resource set association for the two PUSCH transmissions of PUSCH repetition type A, or PUSCH repetition type B, or in response to the use of different spatial relations or different power control parameters for the two PUCCH transmissions of PUCCH repetition, or in response to any event not triggered by DCI or MAC-CE. The UE maintains power consistency and phase continuity within an actual TDW, across PUSCH transmissions of PUSCH repetition Type A scheduled by DCI format 0_1, 0_2 or 0_3, or PUSCH repetition Type A with a configured grant, or PUSCH repetition type B or TB processing over multiple slots, or across PUCCH transmissions of PUCCH repetition, in case the actual TDW is created in response to an event triggered by DCI other than frequency hopping or the use of a different SRS resource set association for the two PUSCH transmissions of PUSCH repetition type A, or PUSCH repetition type B, or the use of different spatial relations or different power control parameters for the two PUCCH transmissions of PUCCH repetition, or in response to an event triggered by MAC-CE, subject to UE capability. of dmrs-BundlingRestart [13, TS 38.306] and when pusch-WindowRestart or pucch-WindowRestart is enabled.