3GPP TSG RAN WG1 Meeting #122	 								                         R1-2506327

Bengaluru, India, Aug 25th – 29th, 2025

Agenda Item:	11.1

Source:	Thales, Eutelsat Group, Airbus, ESA, EchoStar, Viasat, Novamint, Sateliot, Iridium, TNO, ST Engineering iDirect

Title:	Overview of 6GR air interface

Document for:	Discussion and Decision

# Introduction

The 6G Radio Study Item was approved at the RAN#108 meeting (RP-251881) [1]. The main objectives are:

- Single technology framework based on a stand-alone architecture (Note1) to support the agreed existing and new services, and to satisfy the usage scenarios, requirements, deployment scenarios and design principles with acceptable performance/complexity trade-off, as determined by the RAN requirements in [RP-250810] and [TR38.914], including: [RAN1], [RAN2], [RAN3], [RAN4] - Ensuring appropriate set of functionalities, minimize the adoption of multiple options for the same functionality, avoid excessive configurations, excessive UE capabilities and UE capabilities reporting. - Energy efficiency and energy saving: both for network and device. - Enhanced spectral efficiency. - Enhanced overall coverage, focus on cell-edge performance and UL coverage. - Wider channel bandwidth (at least 200MHz) support for 6G deployments at least above 2 GHz, around 7 GHz. - Re-use of existing 5G mid-band (~3.5GHz) site grid for 6G deployments in at least around 7 GHz and targeting comparable coverage to 5G mid-band. - Target scalable and forward compatible design for diverse device types. - Improved spectrum utilization and operations taking into account diverse spectrum allocations. - Aim at using common 6G Radio design, which meets mobile broadband service requirements as high priority, to also meet vertical needs. - Aim at a harmonized 6G Radio design for TN and NTN, including their integration. - System simplification, including reducing configuration complexity, enabling more efficient Cell/UE management, etc. Note1: the term stand-alone architecture does not imply any particular Core network architecture, which is up to SA2 discussion.

While in the context of 5G, the support of NTN was added once the basic design principles of the New Radio (NR) access were already defined, it is strongly recommended to define and specify the basic enablers to support NTN as part of the 6G radio access definition right from the initial 6G specification in release 21. This approach will allow for:

- Achieving ubiquitous connectivity and availability of services in remote and sparsely populated areas.

- Improving network resiliency, especially in situations where disasters disrupt terrestrial networks or damage undersea communication cables.

By integrating the fundamental NTN enablers in the initial stages of 6G development, users will experience several benefits, including seamless connectivity regardless of geographic location. This will empower individuals, communities, and industries in the most isolated regions, ensuring that they are not left behind in the digital revolution. Moreover, enhanced network resiliency will provide robust and reliable communication channels during emergencies, significantly reducing downtime and maintaining critical infrastructure and services. This harmonized design approach for both TN and NTN components will pave the way for a resilient, flexible, and inclusive communication network for future generations.

The intent is therefore to achieve a 6G Radio design harmonized for both TN and NTN and ready for deployment scenarios integrating NTN and TN components since Rel-21.

In this contribution, we share our views on 6G NTN-TN harmonization.

# Design implications of native NTN support in 6GR

A unified radio interface that enables seamless integration between terrestrial and non-terrestrial in 6G systems is emerging as a significant technological trend within the IMT-2030 framework. This approach is also regarded as a fundamental design principle in the new 3GPP RAN1-led 6G Radio study [1].

By natively incorporating non-terrestrial networks (NTN), 6G can deliver unprecedented levels of performance, resilience, and global coverage, fulfilling the evolving demands of the future mobile ecosystem. Achieving comprehensive integration requires a unified and adaptable radio interface that seamlessly supports both terrestrial and satellite connections. This harmonization should address:

- Compatibility with diverse satellite orbits, such as Very LEO, LEO, MEO, and GEO.

- Robustness against challenges like Doppler shifts, significant and variable round-trip times (RTT), delay spreads, and low signal-to-noise ratios (SNRs).

- Flexibility in adapting waveforms, modulation schemes, and multiple access technologies to suit differing radio link conditions.

# Potential building blocks for a harmonized 6G Radio design for TN and NTN

The 6G NTN-TN harmonization is driven by key technological advancements across radio interface design, waveform innovation, and network architecture. The following table presents key building blocks for a harmonized 6G Radio design for TN and NTN.

Table 1 Key features for native support of NTN in 6GR

| Capability | Justification | Requirements |
| --- | --- | --- |
| NTN resiliency to GNSS service | Given that the GNSS service may be unavailable or degraded, NTN (GSO and NGSO, including VLEO) should be designed to operate independently of GNSS. | The 6G RAT shall be able to accommodate the extended and variable Doppler and latency associated to NTN deployment scenarios. |
| Satellite PAPR requirements for NTN downlink transmission | PAPR (peak-to-average power ratio) is a very important metric when it comes to operate as efficiently as possible a transmission equipment. In Very High Throughput Satellite (VHTS) payload, it is common to have several carriers amplified simultaneously by the same amplifier. PAPR increases drastically compared to mono-carrier scenario | PAPR challenges/requirements to be overcome in satellite payload designs in particular when OFDM variants are considered |
| Flexible frame structure | Unlike the TN channel, the NTN propagation channel is mainly characterized with low multi path conditions and low delay spread typically measuring in the few tens of nanoseconds | The 6G RAT shall be designed to support a frame structure optimized to the NTN channel. |
| Extended coverage | Provide service in light indoor/in vehicle environment | The 6G RAT shall be able to support a higher Coupling Loss at the expense of the service rate (i.e. same as NB-IoT’s MCL) |
| Flexible duplex mode support at User Equipment level | Most satellite service allocated frequency bands are paired while some are unpaired bands. Half Duplexing at the UE can reduce antenna size. | The 6G RAT shall be designed to operate in FDD, TDD or HD-FDD via NTN. |
| Support of HD-FDD at Network side | Although the duplexing mode is FDD at UE level, HD FDD operation at network side would be beneficial to cope with satellite payload design constraints. | The 6G RAT shall be designed to optionally support HD-FDD Satellite Payload |
| Mitigate specific NTN Propagation impairments | In specific usage conditions (on board helicopter, car in a forest), the UE may experiences regular deep fades of tens of ms duration | The 6G RAT shall be designed to implement time diversity over several time frames |
| Operation at low error rate | Due to the long propagation delay, HARQ causes latency which degrades the QoS | The 6G RAT shall be designed to achieve a 10-6 BLER without HARQ |
| Massive messaging capability | Support a high number of terminals willing to transmit quasi simultaneously low bandwidth services (i.e. messaging) within a beam | The 6G RAT shall be designed to provide an efficient grant free access protocol |
| Extension of satellite foot print | Limited RF power on board may prevent to illuminate all beams generated by the satellite at a given time (ratio can be as low as 1%) | The 6G RAT shall be designed to support an equivalent of the “SSB periodicity” which can be extended up to 320 ms |
| Positioning, Navigation and Timing | Integrating GNSS into IoT devices operating over satellite access may significantly impacts battery consumption, especially in battery-powered UEs requiring frequent positioning fixes/updates. Wide range of sectors, e.g. transportation, critical infrastructure and regulatory services (e.g. emergency services) are fully dependent on GNSS to provide PNT services, impacting their resilience. | The 6G RAT shall provide means for the UE to determine its location independently of non-3GPP positioning technologies (e.g. GNSS) |
| Enhanced network verified UE location service | The current (Rel-18 defined) network verified UE location scheme may not be able to determine reliably the UE location with an accuracy of few kms with large beam size | The 6G RAT shall provide means for the network to determine the location of a UE without GNSS service |
| Seamless TN/NTN mobility in connected mode | The TN coverage may be patchy and NTN can be used to bridge two separate TN coverage areas | The 6G RAT shall be designed to support mobility in connected mode between NTN/TN without any packet loss while addressing the difference in RTD and QoS |
| Multi connectivity/carrier aggregation | Provide high speed and/or highly reliable broadband service | The 6G RAT shall support multi connectivity/carrier aggregation between two satellite access with different radio link availability: e.g. one operating in high frequency band (> 10 GHz) and another operating in low frequency band (< 7 .125 GHz) |
| 6G NTN coexistence with IoT-NTN or NR-NTN in same beam | Need to support a smooth migration path to 6G for space segment designed for IoT-NTN or NR-NTN | It shall be possible to deploy/operate NTN capable 6G RAT and NTN capable 4G/5G RAT in adjacent channels within the same beam |

Other potential features for 6GR NTN are listed in Table 2.

Table 2 Other potential features for 6GR NTN

| Capability | Justification | Requirements |
| --- | --- | --- |
| RAT energy consumption determination for one-to-many service | When there are several RAT (e.g. TN and NTN), the energy consumption of a one-to-many service may be considered to select a RAT for the delivery of the service | The 6G RAT shall provide means to determine the energy consumption of a one-to-many service |
| Distributed MIMO across several satellites | To be able to increase the throughput for a UE | Support MIMO across multiple TRP on board different satellites |

Proposal 1: RAN1 to study in priority the following NTN capabilities for the 6GR as part of the Rel-20

- NTN resiliency to GNSS service

- PAPR requirements for NTN downlink transmission

- Flexible frame structure

- Extended coverage

- Flexible duplex mode support at UE level

- Support of HD-FDD at Network side

- Mitigate specific NTN Propagation impairments

- Operation at low error rate

- Massive messaging capability

- Extension of satellite footprint

- Positioning, Navigation and Timing

- Enhanced network verified UE location service

- Seamless TN/NTN mobility in connected mode

- Multi connectivity/carrier aggregation

- 6G NTN coexistence with IoT-NTN or NR-NTN in same beam

# Network architecture for a native NTN support in 6G RAT

Satellite network architectures are evolving toward flexible and intelligent designs supporting enhanced mobility management, broadcast/multicast services, mobile edge computing (MEC) and RAN slicing. AI-enabled end-to-end slice management facilitates autonomous slicing with QoS assurance. Optimized mobility management enables seamless handover across service and feeder links, minimizing service interruption. Inter-satellite collaboration, utilizing optical inter-satellite links (ISLs), supports joint processing of user and control plane data between GSO and non-GSO satellites, enhancing network resiliency, efficiency, and resource utilization. Broadcast/multicast capabilities provide efficient group communication for public safety and emergency response. MEC extends computing to the edge, including on-board satellites, to reduce latency for critical applications.

Regenerative payloads enable flexible on-board signal processing, inter-satellite communication, and integrated sensing. Native NTN deployment requires an integrated architecture supporting functional splits (e.g., DU/CU) between space and ground segments.

## Orbits

Below are some considerations for practical NTN scenarios for 6G. The following orbits should be considered:

- VLEO based scenario at minimum altitude of 300 km: Satellites operating at this altitude benefit from reduced latency due to their proximity to the surface of Earth, which can enhance communication and observation capabilities. At this altitude, the Round Trip Delay becomes closer to the one of terrestrial extended cells. This reduced latency would be particularly beneficial in supporting Time Division Duplexing (TDD) and Half-Duplex Frequency Division Duplexing (HD-FDD) modes, enabling more efficient and reliable real-time data transmission. Further, this closer proximity to Earth can offer more accurate ground metrics, and potentially lower cost for certain types of satellite operations. However, deployment at a VLEO orbit would require a very high number of satellites to generate overlapping foot print and hence service continuity in the targeted coverage. This is now made possible with the ever decreased launch cost. Typically, a maximum satellite beam footprint diameter of [250] km or lower should be supported.

- Medium and High LEO (M-LEO and H-LEO, respectively) based scenarios (respectively 600 and 1200 km altitude): While these scenarios are already supported in the context of 5G, the maximum beam footprint can be reduced in the context of 6G since the targeted service rate is likely higher than NB-IoT-NTN capability and hence require satellites able to achieve high antenna gain and hence generate smaller beams. Typically, a maximum satellite beam footprint diameter of [250] km or lower should be supported.

- MEO based scenarios (10000 km altitude): In line with the above, typically, a maximum satellite beam footprint diameter of 250 km or lower should be supported.

- GSO based scenarios: GSO differs from GEO by a freedom to orbit around the 0° latitude. This allows to extend the satellite lifetime by optimizing the necessary fuel for station keeping.

The table below provides the list of reference practical scenarios (single orbit) to be supported by the 6G radio access with the related parameters.

Table 3: NTN reference orbit scenarios for 6G

| Scenarios | Very LEO based NTN | Medium LEO based NTN | High LEO based NTN | MEO based NTN | GSO based NTN |
| --- | --- | --- | --- | --- | --- |
| Orbit type | circular around the earth | circular around the earth | circular around the earth | circular around the earth | station keeping around a nominally fixed position in terms of elevation/azimuth with respect to a given earth point (NOTE 6) |
| Altitude | 300 km (NOTE 1) | 600 km | 1200 km | 8000 km | 35,786 km |
| Earth-fixed beams | Yes (Satellite generated steerable beams, NOTE 2) |  |  |  |  |
| Max beam footprint size (edge to edge) regardless of the elevation angle (NOTE 3) | [<250] km | [<250] km | [<250] km | [<250] km | [<250] km |
| Min Elevation angle for both sat-gateway and User Equipment | 10° for service link and 10° for feeder link |  |  |  |  |
| Max distance between satellite and User Equipment at min elevation angle | 1160 km | 1 932 km | 3 131 km | TBD km | 40 581 km |
| Max Round Trip Delay (propagation delay only NOTE 4) | 15.48 ms | 25.77 ms | 41.77 ms | 180 ms | 541.46 ms |
| Max Doppler shift (earth fixed user equipment) (NOTE 5) | 24.23ppm | 22.68ppm | 20.04ppm | TBD ppm | 0.93ppm |
| Max Doppler shift variation (earth fixed user equipment) (NOTE 4) | TBD ppm/s | 0.27 ppm/s | 0.13 ppm/s | TBD ppm/s | 0.000 045 ppm/s |
| NOTE 1: VLEO is characterized by altitudes ranging from approximately 180 to 600 kilometers above Earth. However, the lower the altitude, the higher the number of satellites is required to ensure service continuity and the higher the atmospheric drag which would need constant propulsion and re-boosting and hence reduced lifetime. NOTE 2: Each satellite has the capability to steer beams towards fixed points on earth using beamforming techniques. This is applicable for a period of time corresponding to the visibility time of the satellite. NOTE 3: This beam size refers to the Nadir pointing of the satellite. NOTE 4: Assuming transparent payload with service and feeder links and return NOTE 5: Max Doppler shift and Max Doppler shift variation in the absence of any device pre-compensation of satellite Doppler shift on the service link. NOTE 6: In case of GSO, the inclination may be non-zero. This means the satellite’s ground track may trace a figure-eight pattern (analemma) as seen from Earth. |  |  |  |  |  |

Proposal 2: The 6G radio interface/access shall be defined to support all the practical NTN deployment scenarios described mainly in terms of orbits, and related service link characteristics in the table above.

## Constellation

Typical sizing of constellation according to the orbit:

- GSO constellation may be based on one to three satellites depending on the targeted coverage.

- NGSO constellations providing real time services are sized to ensure at least one satellite in visibility of all UEs (above the min elevation angle) in the targeted coverage

- NGSO constellations providing non real time services such as messaging shall be made of at least one satellite. A given UE can connect to one of the serving satellite(s) from time to time depending on the satellite(s) revisit time.

Additionally, the 6G radio access should also support multi orbit space segment combining at least a GSO with an NGSO based scenario, where NGSO may cover VLEO, medium/high LEO and/or MEO.

## Payloads and feeder links

The following payload types along with 6G functional mapping shall be supported:

Table 4: 6G NTN payload types with respect to 6G system architecture

| Payload type | 3GPP function on board | Feeder link type (NOTE 1) | Other capabilities |
| --- | --- | --- | --- |
| Transparent | Equivalent to RF repeater | 6G radio interface | - |
| Semi transparent | Radio Unit (RU) | Fronthauling of RU-Base station interface | - |
| Regenerative embarking a full base station (BS) | BS | Backhauling of NG interface | - Support of Store and forward - And/or Support of Inter Satellite Links |
| Regenerative embarking part of a base station (Functional BS split) | Radio Unit and a Distributed unit | Base station internal interface (F1 like) | Inter Satellite transport links can be used |
| NOTE1: It refers to the radio link between satellite and a gateway. |  |  |  |

Proposal 3: The 6G radio interface/access shall be defined to support all the NTN payload types in the table above.

## Inter-Satellite Link support

Regenerative satellite payloads enable routable front or backhaul traffic which can exploit Intersatellite Links (ISLs) within the underlying transport network. ISLs can be deployed between two satellites of the same constellation (intra-/inter- orbital plane) or between two satellites of different constellations/orbits (e.g., between LEO and GSO satellites). ISL will allow to provide service in areas where there are no NTN-gateways and provide coverage of remote areas like the oceans. Inter-satellite links may operate in RF frequency or optical bands. The route taken between the GW and the satellite may thus change, adding or removing inter-satellite links to reach the GW.

Note that there are different types of inter satellite links:

- Between satellite of the same orbit which can further be distinguished between

  - Intra orbital plane ISL: between (adjacent) satellites of the same orbital plane

  - Inter orbital plane ISL: between satellites of (adjacent) orbital planes

- Between satellites of different orbits (e.g., GSO and NGSO, or LEO and MEO)

## Duplexing mode

6G should support both duplexing methods — FDD (Frequency Division Duplexing), and TDD (Time Division Duplexing) as well as Half Duplex Frequency Division Duplexing (HD-FDD) mode.

The target Payload types are:

- Full-duplex FDD Satellite Payload:

  - Full duplex networks should support a mix of full and half duplex UEs

- Half-duplex FDD Satellite Payload: For both base station and terminal, a duplexer is not needed.

The targeted UE types are:

•	Smartphone and IoT devices

•	Fixed and Mobile VSAT

•	Full and Half Duplex FDD Mode

•	GNSS receiver capability is not required at UE side for physical layer operation.

The UE will have to support the frequency band and duplex mode of the satellite network.

Table 5: Duplex mode supported by 6GR NTN

| NTN capable UE type | Paired frequency band | Unpaired frequency band |
| --- | --- | --- |
| IoT and smartphones | FDD or HD-FDD | TDD |
| Vehicle and building mounted | FDD or HD-FDD | TDD |

Proposal 4: The 6G radio interface/access shall be defined to support all the duplex modes at UE and network level in the table above.

## Service links

In all cases, the service link (which refers to the radio link between satellite and User Equipment) will provide the 3GPP defined NTN capable 6G radio interface.

It operates in satellite service allocated frequency bands which can be paired (Downlink and Uplink in separate bands) or unpaired (Downlink and Uplink in the same band).

While paired bands can apply to all orbits, unpaired band is restricted to the Low Earth Orbits (VLEO, MLEO and HLEO).

# 6GR UE

6G access network shall be able to serve a wide range of devices (User Equipment - UE). We shall distinguish the following ones:

- Smartphone and IoT devices with omnidirectional antenna which should be addressed with satellite networks operating in satellite service allocated frequency bands up to ~7 GHz.

- Vehicle or building mounted devices with directional antenna operating in all satellite service allocated frequency bands. Vehicle encompass cars, buses, railway trains, vessels and aeronautic platforms.

The 6G access network shall be defined to be able to serve devices with the following performances:

Table 6: NTN capable 6G UE types

| UE types | Min antenna gain | Min Transmit Power | Max Noise figure | UE Ground speed | Coverage |
| --- | --- | --- | --- | --- | --- |
| IoT and smartphones | -5 dB | 20 dBm | 7 dB | 300 km/h (on board high speed trains) | Outdoor, Light indoor (NOTE1) |
| Vehicle, vessel, aircraft, and building mounted | Equivalent to the one of 20 cm aperture antenna (FR2-NTN), 0.3 dBic (FR1-NTN) [3], [4] | Equivalent to 1 Watt (FR2-NTN) | [4] dB (FR2-NTN), 7 dB (FR1-NTN) | 1500 km/h (aircraft) | Outdoor only |
| NOTE1: Light indoor requires Maximum Coupling Loss comparable to 3GPP defined NB-IoT. It may lead to service rate degradation. |  |  |  |  |  |

Proposal 5: The 6G radio interface/access shall be defined to support all the NTN capable UE types in the table above.

# NTN – TN Interworking

Interworking of NTN and TN is needed for ubiquitous coverage, service continuity and/or resiliency purposes. This requires for 6G to support mobility and simultaneous multi connectivity between NTN and TN components.

## NTN-TN Mobility

The purpose is to ensure a global service continuity in both idle and connected modes. NTN is expected to provide coverage in overlapping areas with TN but also in areas beyond TN coverage.

While mobility from TN to NTN is mostly triggered by the unavailability of the TN service, the mobility from NTN to TN may be determined by other criteria.

In both cases, the UE shall be able to camp on a TN cell while being able to monitor NTN cells and vice versa.

The related requirements on the UE differ on the type of targeted devices:

Table 7: NTN capable 6G UE possible architecture characteristics

| NTN capable UE types | Modem | Antenna |
| --- | --- | --- |
| IoT and smartphones | Same for both NTN and TN | Same for both NTN and TN |
| Vehicle mounted | Same or distinct respectively both NTN and TN | Two for respectively NTN and TN (FR2, FR1 broadband); Same for both NTN and TN for narrowband IoT services in FR1. |
| Building mounted | Same or distinct respectively both NTN and TN | Two for respectively NTN and TN |

Proposal 6: The 6G radio interface/access shall be defined to support TN/NTN mobility considering the different UE architecture described in the table above

## NTN-TN Dual connectivity

Dual Connectivity or Multi-connectivity between NTN and TN can be used for Coverage and Quality of Service enhancement. Also, it can ensure seamless mobility, exploiting Heterogeneous Network (HetNet) environment, connection to a large NTN macro cell for mobility anchor and connection to a local TN small cell for data transfer in optimal radio conditions. Thereby, multi-connectivity can provide a very fluent (no added data latency during handovers) and robust mobility (connection failures minimized). NTN-TN Multi-connectivity also provides increased throughput especially in case of VLEO and LEO; using multiple radio resources can obviously improve data rates experienced by users.

NTN-TN Dual connectivity may contribute to provide seamless mobility by eliminating handover interruption delays and errors, and optimizes capacity, coverage and mobility for devices connected in a heterogeneous NTN-TN network

Given that the NTN may feature a different bandwidth compared to the one offered by TN, only part of the traffic (most critical one) may be duplicated via the NTN connectivity.

Hence, the traffic is partially duplicated between the two NTN and TN connectivity.

Proposal 7: The 6G radio interface/access shall be defined to support TN/NTN multi connectivity with simultaneous traffic flow in both legs.

# Positioning, Navigation and Timing

The provision of robust, high-accuracy positioning is driven by regulatory requirements, demand for ultra-precise location services, need for resilient alternatives to GNSS, and the expansion of service areas. NTN is a key enabler of ubiquity, resilience, and wide area coverage representing a significant opportunity to enhance not only global connectivity but also positioning services especially in remote, underserved, and maritime areas where traditional terrestrial networks are not feasible or cost-effective. Complementing TN with NTN is crucial not only for connectivity but also for positioning services to maximize synergies between the two.

Proposal 8: The 6G radio interface/access shall be defined to provide high-accuracy and resilient positioning without GNSS service.

# Conclusion

In this contribution, we made the following proposals:

Proposal 1: RAN1 to study in priority the following NTN capabilities for the 6GR as part of the Rel-20

- NTN resiliency to GNSS service

- PAPR requirements for NTN downlink transmission

- Flexible frame structure

- Extended coverage

- Flexible duplex mode support at UE level

- Support of HD-FDD at Network side

- Mitigate specific NTN Propagation impairments

- Operation at low error rate

- Massive messaging capability

- Extension of satellite footprint

- Positioning, Navigation and Timing

- Enhanced network verified UE location service

- Seamless TN/NTN mobility in connected mode

- Multi connectivity/carrier aggregation

- 6G NTN coexistence with IoT-NTN or NR-NTN in same beam

Proposal 2: The 6G radio interface/access shall be defined to support all the practical NTN deployment scenarios described in terms of orbits, and related service link characteristics in the table within clause 4.1 of R1-2506327)

Proposal 3: The 6G radio interface/access shall be defined to support all the NTN payload types in the table within clause 4.3 of R1-2506327)

Proposal 4: The 6G radio interface/access shall be defined to support all the duplex mode at UE and network level in the table within clause 4.5 of R1-2506327)

Proposal 5: The 6G radio interface/access shall be defined to support all the NTN capable UE types in the table within clause 5 of R1-2506327)

Proposal 6: The 6G radio interface/access shall be defined to support TN/NTN mobility considering the different UE architecture described in the table within clause 6.1 of R1-2506327)

Proposal 7: The 6G radio interface/access shall be defined to support TN/NTN multi connectivity with simultaneous traffic flow in both legs.

Proposal 8: The 6G radio interface/access shall be defined to provide high-accuracy and resilient positioning without GNSS service.

# References

- RP-251881, New SID: Study on 6G Radio, RAN#108