3GPP TSG RAN WG1 #122			R1- 2506368

Bengaluru, India, Aug 25th – 29th, 2025

Agenda Item:	11.1

Source: 	NICT

Title:	Views on 6GR air interface design criteria

Document for:	Discussion

Introduction

The study aims to develop one non-backward compatible radio access technology (hereafter “6GR”) to meet a broad range of use cases, and requirements as defined during the RAN requirements study [TR38.914].

The study should strive at dimensioning an appropriate set of functionalities, minimizing the adoption of multiple options for the same functionality, focusing on practical user experience. The study should identify principles to ensure extensibility and deliver superior performance.

NOTE: Section 4.1.1 shows interim milestones to reach certain agreements, including early consensus items.

From a technology perspective, the study will address frequency ranges up to 52.6GHz, including at least the full range of FR1 (up to 7.125GHz), the range between FR1 and FR2-1 (including around ~7GHz), and FR2-1 (24.25 GHz – 52.6GHz).

NOTE: TR 38.921, TR 38.922 will be taken into account.

NOTE: Frequency ranges beyond 52.6 GHz are not in scope of the work.

The detailed objectives of the study are:

- Single technology framework based on a stand-alone architecture (Note1) to support the agreed existing and new services, and to satisfy the usage scenarios, requirements, deployment scenarios and design principles with acceptable performance/complexity trade-off, as determined by the RAN requirements in [RP-250810] and [TR38.914], including: [RAN1], [RAN2], [RAN3], [RAN4]

  - Ensuring appropriate set of functionalities, minimize the adoption of multiple options for the same functionality, avoid excessive configurations, excessive UE capabilities and UE capabilities reporting.

  - Energy efficiency and energy saving: both for network and device.

  - Enhanced spectral efficiency.

  - Enhanced overall coverage, focus on cell-edge performance and UL coverage.

  - Wider channel bandwidth (at least 200MHz) support for 6G deployments at least above 2 GHz, around 7 GHz.

  - Re-use of existing 5G mid-band (~3.5GHz) site grid for 6G deployments in at least around 7 GHz and targeting comparable coverage to 5G mid-band.

  - Target scalable and forward compatible design for diverse device types.

  - Improved spectrum utilization and operations taking into account diverse spectrum allocations.

  - Aim at using common 6G Radio design, which meets mobile broadband service requirements as high priority, to also meet vertical needs.

  - Aim at a harmonized 6G Radio design for TN and NTN, including their integration.

  - System simplification, including reducing configuration complexity, enabling more efficient Cell/UE management, etc.

Note1: the term stand-alone architecture does not imply any particular Core network architecture, which is up to SA2 discussion.

In RAN#108, the study item of 6G radio (6GR) has been approved [1]. The objectives of the study item include:

The first part of the detailed objectives gives criteria to be considered when designing 6GR air interface, some of which are:

- Energy efficiency

- Spectral efficiency

In this contribution, we give our views and thoughts on those 6GR air interface design criteria.

Views and thoughts on 6GR air interface design criteria

Energy efficiency

Energy efficiency has been continuously demanded performance criteria to reduce network operators’ OPEX and/or for longer battery life of UEs. Moreover, by the fact that telecommunication industry is already consuming nonnegligible part of the energy produced worldwide, it can be obviously said that increasing energy consumption proportional to increasing data traffic is not allowed. It should be taken as an issue of all telecommunication industry rather than being driven by part of participants’ business interests, where performance metrics relating to data transfer (e.g., peak data rate) or implementation and operating cost are sometimes prioritized.

In 5G NR, while DFT-s-OFDM waveform was adopted in UL, which enables power amplifiers to operate effectively by its low peak to average power ratio (PAPR) characteristics, it was not adopted as DL waveform since saving battery life was not a concern for gNBs. For 6GR, we think that low PAPR characteristics should be pursued in both UL and DL.

As projects directly aimed at energy saving, the study and specification works of network energy saving (NES) and low-power wake-up signal and receiver (LPWUS) were carried out during Release 18 and 19. As the results, on demand SSB/SIB1, low power wake-up signal to UE were specified. On demand SSB was not defined for primary cell because UEs of previous releases have to expect that SSBs are kept transmitted with certain periodicity. Low power wake-up signal in 5G NR was for UEs especially with low duty cycle operation (e.g., smart meters, environment sensors). As 6GR, since backward compatibility will not be the issue, on demand SSB may be defined on primary cells if frame structure is defined so that UEs do not expect SSBs are kept transmitted. At the same time, wake-up signal from UEs may be defined. By such design, drastic energy saving can be expected for cells or beam directions with less or no traffic, which would more frequently happen in FR2 or FR3 frequencies.

Observation 1: Improving energy efficiency is not only for reducing mobile operators’ running cost or giving advantageous feature to UEs, but it is the subject of whole wireless network industry for not to increase power consumption proportional to increasing data traffic.

Proposal 1: More weight should be put on energy efficiency among other criteria such as operating cost, implementation simplicity, or performance metrics relating to data transfer.

Spectral efficiency

Spectral efficiency can be improved by introducing waveforms that has low out of band emission (OOBE) performance, enabling to use higher order modulation and coding or to combine with other modes of multiplexing such as special domain multiplex(SDMA), full duplex, etc.

Introducing waveforms of low OOBE performance enables to reduce guard bands between the adjacent bands and allows more resource blocks to be allocated. When defining waveforms of 5G NR, priority was put on system flexibility to accommodate various numerologies (e.g., subcarrier spacing, frame/slot structure, CP length, etc.) suitable for various use case scenarios (e.g., eMBB, URLLC, IoT) as well as implementation complexity. Therefore, though some waveform candidates were proposed to suppress the OOBE [2][3], none was adopted due to the concern on flexibility or increased signal processing.

To enable using higher order modulation and coding, the link budget has to be improved. It can be done by increasing transmit power and/or antenna gain. Interference coordination will also be required. We think that those can be handled in the context of increasing data rate, where increasing spectral efficiency can be aimed at as secondary benefit. Similarly, other modes of multiplexing to combine are also to be handled in the increasing data rate context.

Observation 2: Waveforms that suppress OOBE were not adopted in 5G NR since system flexibility and implementation complexity were prioritized.

Proposal 2: RAN1 to consider introducing waveform(s) that supress OOBE as long as there is no significant increase of implementation complexity.

Conclusion

Observation 1: Improving energy efficiency is not only for reducing mobile operators’ running cost or giving advantageous feature to UEs, but it is the subject of whole wireless network industry for not to increase power consumption proportional to increasing data traffic.

Observation 2: Waveforms that suppress OOBE were not adopted in 5G NR since system flexibility and implementation complexity were prioritized.

Proposal 1: More weight should be put on energy efficiency among other criteria such as operating cost, implementation simplicity, or performance metrics relating to data transfer.

Proposal 2: RAN1 to consider introducing waveform(s) that supress OOBE as long as there is no significant increase of implementation complexity.

References

[1] RP-251881, New SID: Study on 6G Radio

[2] R1-163110, The outline of the signal processing of the OP is given in Appendix. NTT DoCoMo

[3] R1-162199, Waveform Candidates, Qualcomm