3GPP TSG RAN WG1 #122			R1-2506385

Bengaluru , India, August 25th – 29th, 2025

Agenda item:	10.6.1

Title:	Discussion on NR-NTN GNSS resilience

Source:	ViaSat Satellite Holdings Ltd, Inmarsat, Terrestar, Ligado, Space42, Thuraya

Document for:	Discussion and Decision

# 1. Introduction

In RAN #108, a new SI on GNSS resilient NR-NTN operation [1] was approved for potential Rel-20 target release. The following objectives of Core part WI were specified:

The study will be carried assuming that

- The GNSS information in UE with GNSS capability may be 1/ temporarily unavailable or 2/ available but with GNSS position accuracy degradation or 3/ available but with increased GNSS measurement period for power saving purpose

  - This study will initially focus on 1/ (where GNSS information is temporarily unavailable)

  - Note: This study does not include any enhancement on positioning for the above scenario

Study GNSS resilient NR-NTN operation [RAN1, RAN4]

- Assess impact on initial access and connected mode procedures for NR-NTN, including potential implications if any on RAN4 RRM specifications

  - Aim to minimize physical layer procedures impact

  - Avoid physical layer channel/signal changes

  - Backward compatible issues with legacy NTN capable UEs will be assessed and should be prevented

  - Applicable to NGSO and GSO constellations, NTN operating in below and in above 10 GHz frequency bands, transparent and regenerative satellites

- Check in RAN#112 (June, 2026) to decide whether to proceed with normative work, extend the study item, or postpone normative work to Release 21

In this contribution, we expand on our prior proposals on this topic [2]:

- RAN#108 to approve a 6month study phase to be followed by a possible normative phase that

  - Studies the feasibility of a GNSS free NR-NTN operation without requiring a new silicon eco-system

  - Studies frequency and timing adjustment during both initial access and in connected mode

  - Consider the co-existence of legacy and new UEs in the same satellite beam

  - Study the performance of signaling without GNSS measurements

  - Study GNSS free NR-NTN for both NGSO and GSO constellations

# 2. Discussion

In this contribution, we focus on the simulation parameters for frequency and timing adjustment during both initial access and in connected mode. As an example, we consider typical operational modes for NGSO constellations in the 1.5-2.2 GHz carrier frequency range.

During initial access, when there is no frequency offset information available to the UE, the UE must perform cell search by detecting PSS over a wide range of frequencies around the carrier frequency. After acquiring the synchronization signals, the UE can reduce the search window for subsequent stages.

For timing synchronization, in the absence of local time information at the UE, the UE can initially perform a coarse delay estimation based on the reference location. However, during subsequent random-access procedure, the pre-compensated timing advance parameter may be large due to lack of precise GNSS location information.

Observation 1: The worst-case scenario of interest is when GNSS information is not available for timing and frequency adjustment during initial access.

Observation 2: Due to lack of accurate GNSS position information, the UE must perform cell search by detecting PSS over a wide range of frequencies around the carrier frequency.

Observation 3: Due to lack of accurate GNSS position information, the coarse delay estimation at the UE may result in a large pre-compensated timing advance parameter during the random-access procedure.

For the geometry and parameters discussed below, the following definitions are used.

| Parameter | Definition |
| --- | --- |
| Orbit altitude/height | Distance of satellite from the surface of Earth |
| Elevation angle | The elevation angle (θ) is the angle between the local horizon and the satellite, as seen from a ground station or user equipment (UE). |
| Angle from Nadir | In the context of satellite communication, nadir refers to the point on the Earth's surface directly below the satellite. As the satellite moves away from nadir, the elevation angle decreases, and the off-nadir angle increases. |
| Beam diameter | The width or size of the satellite's antenna beam as it projects onto the Earth's surface or within the network's coverage area. More specifically, for NTN, the beam diameter and related concepts are often defined in terms of the 3dB beam width of the satellite antenna pattern. |
| Differential timing and frequency error | Differential timing and frequency errors refer to the difference in these errors between different locations within the cellular network, particularly between the cell center and the cell edge. In the context of this document, the differential errors are quoted as the worst-case timing and frequency difference between the reference signal at the center of the cell and the signal as seen by the UEs located at the cell-edges (near edge and far edge). |

See Figure 1 for a sketch of the geometry of the NGSO use case.

Figure 1 Geometry of the NGSO use case

Proposal 1: The study should consider the timing and frequency offset parameters depicted in the table below. These parameters are for a stationary user. For mobile users, the UE velocity needs to be taken into account.

Table 1 Differential Frequency and Timing Offset for NGSO constellations

| Min User Elevation Angle (deg) | Max Angle from Nadir (deg) | Orbit Height (km) | Carrier Frequency (MHz) | Beam Diameter (km) | Differential Timing Error Magnitude (usec) | Differential Doppler Error (ppm) |
| --- | --- | --- | --- | --- | --- | --- |
| 30.0 | 60.0 | 500 | 2200 | 75 | 214.9 | 0.369 |
| 10.0 | 80.0 | 1200 | 1500 | 773 | 2516.5 | 0.684 |

3. Conclusions

Observation 1: The worst-case scenario of interest is when GNSS information is not available for timing and frequency adjustment during initial access.

Observation 2: Due to lack of accurate GNSS position information, the UE must perform cell search by detecting PSS over a wide range of frequencies around the carrier frequency.

Observation 3: Due to lack of accurate GNSS position information, the coarse delay estimation at the UE may result in a large pre-compensated timing advance parameter during the random-access procedure.

Proposal 1: The study should consider the timing and frequency offset parameters depicted in the table below. These parameters are for a stationary user. For mobile users, the UE velocity needs to be taken into account.

Table 1 Differential Frequency and Timing Offset for NGSO constellations

| Min User Elevation Angle (deg) | Max Angle from Nadir (deg) | Orbit Height (km) | Carrier Frequency (MHz) | Beam Diameter (km) | Differential Timing Error Magnitude (usec) | Differential Doppler Error (ppm) |
| --- | --- | --- | --- | --- | --- | --- |
| 30.0 | 60.0 | 500 | 2200 | 75 | 214.9 | 0.369 |
| 10.0 | 80.0 | 1200 | 1500 | 773 | 2516.5 | 0.684 |

4. References

- RP-251863; Study on GNSS resilient NR-NTN operation

- RP-251707; Operator views on NR-NTN priorities for 5GAdv for Rel-20