3GPP TSG RAN WG1 #122							   R1-2506402

Bengaluru, India, Aug 25th – 29th, 2025

Source:	Moderator (MediaTek)

Title:	FL summary 4 for on-demand SIB1 in idle/inactive mode

Agenda item:	8.5.2

Document for:	Discussion and Decision

Introduction

This document is used to assist the discussions for “8.5.2 On-demand SIB1 for idle/inactive mode UEs” of the Rel-19 working item on “Enhancements of network energy savings for NR” based on proposals from companies.

This contribution provides discussion points in Section 2, resulted RAN1 conclusion/agreement in Section 3, and reference (companies tdoc list) in Section 4.

In RAN1 email discussion, Chairman(Sorour) has guidance that:

- “It is the proponent responsibility to provide the adequate information/motivation (cover page info) for the proposed TP to be discussed.”

Hence, if any of the TPs discussed below are agreed in online session, current plan is to rely on the proponent company to submit the draft/final CR including the cover page.

Discussion points

- Issue 1: Parameter name alignment of search space zero

Background

In [1, Nokia], it is mentioned that in latest TS 38.213 R19 text [12], the parameter SearchSpaceZero is not aligned with legacy parameter searchSpaceZero. The following proposal is drawn.

Proposal-1: To align with the legacy parameter, it is proposed to revise the “SearchSpaceZero” to “searchSpaceZero” in Section-23 in TS 38.213 and adopt the corresponding TP.

- FL Proposal 1-1

Adopt the following TP to revise the “SearchSpaceZero” to “searchSpaceZero” in Section 23 of latest R19 TS 38.213.

23 UE procedure to request SIB1 reception *** Unchanged parts are omitted *** In response to the PRACH transmission, the UE monitors PDCCH on the second cell to detect a DCI format 1_0 with CRC scrambled by a corresponding RA-RNTI during a window controlled by ra_ResponseWindow, as described in Clause 8.2. The UE monitors PDCCH according to a Type1-PDCCH CSS set provided by ra-SearchSpace, if provided; else provided by SsearchSpaceZero as described in Clause 10.1, in a CORESET provided by controlResourceSetZero. If the higher layers at the UE identify a RAPID associated with a corresponding PRACH transmission from the UE in a PDSCH reception scheduled by the DCI format 1_0 with CRC scrambled by the RA-RNTI, the higher layers indicate to the physical layer to monitor PDCCH on the second cell to detect a DCI format 1_0 with CRC scrambled by the SI-RNTI according to a Type0-PDCCH CSS set provided by SsearchSpaceZero. If the UE is provided sib1-pdcchRestrictionToPRACHAssociatedSSBs, the UE monitors PDCCH only in monitoring occasions associated with the SS/PBCH block; otherwise, the UE assumes that PDCCH is transmitted in at least one PDCCH monitoring occasion corresponding to each transmitted SSB. The UE monitors PDCCH to detect the DCI format 1_0 with CRC scrambled by the SI-RNTI after a number of slots provided by od-sib1-windowStartOffset from the start of the first slot of the window controlled by ra_ResponseWindow, and for a number of slots provided by od-sib1-WindowDuration. *** Unchanged parts are omitted ***

| Company | Support or not | Comment |
| --- | --- | --- |
| MTK | Yes | Fine to correct this typo. |
| Google | Yes |  |
| Xiaomi | Yes |  |
| Fujitsu | Yes |  |
| ZTE Corporation, Sanechips | Yes |  |
| Samsung | Yes but, | Editorial. As a general comment, we think this type of typos can be raised up in the CR review phase after RAN1 meeting and easily corrected by the editor. And then, in RAN1 meeting, a group can focus on more essential discussion. |
| Tejas | Support |  |
| CATT | OK |  |
| Moderator |  | We can have a quick try during offline. If any concern raised, we leave it to the editor. |
| LG | Support |  |
| Sharp | Support |  |
| Moderator |  | During the Thursday morning offline session, companies think this can be handled relying on Samsung delegate to inform the 38.213 editor to apply the correction. Discussion closed. |

- Issue 2: Clarification of PRACH transmission for SIB1 request

Background

In [2, Google], it is mentioned that some clarification of PRACH transmission for SIB1 request is needed in 38.211 and the corresponding 38.211 TP is proposed.

Proposal 1: Endorse the following TP for 38.211 to clarify the PRACH transmission for SIB1 request:

--------------------------------- start of TP for 38.211-------------------------------------

5.3.2	OFDM baseband signal generation for PRACH

<omitted text>

-	Δf is the subcarrier spacing of the initial uplink bandwidth part during initial access. If the PRACH transmission is for a candidate cell Δf is provided by ltm-PRACH-SubcarrierSpacing in EarlyUL-SyncConfig. If the PRACH transmission is for SIB1 request, Δf is provided by SIB1-RequestConfig. Otherwise,  is the subcarrier spacing of the active uplink bandwidth part;

-	μ0 is the largest μ value among the subcarrier spacing configurations by the higher-layer parameter scs-SpecificCarrierList;

-	NBWP,istart is the lowest numbered resource block of the initial uplink bandwidth part and is derived by the higher-layer parameter initialUplinkBWP or initialUplinkBWP-RedCap during initial access and from the higher-layer parameters bwp-GenericParameters in EarlyUL-SyncConfig if the PRACH transmission is for a candidate cell and from the higher-layer parameters SIB1-RequestConfig if the PRACH transmission is for SIB1 request. Otherwise, NBWP,istart is the lowest numbered resource block of the active uplink bandwidth part and is derived by the higher-layer parameter BWP-Uplink;

<omitted text>

6.3.3.1	Sequence generation

<omitted text>

There are 64 preambles defined in each time-frequency PRACH occasion, enumerated in increasing order of first increasing cyclic shift  of a logical root sequence, and then in increasing order of the logical root sequence index, starting with the index obtained from the higher-layer parameter prach-RootSequenceIndex or rootSequenceIndex-BFR or by msgA-PRACH-RootSequenceIndex if configured and a type-2 random-access procedure is initiated as described in clause 8.1 of [5, TS 38.213] or by prach-RootSequenceIndex in EarlyUL-SyncConfig if the PRACH transmission is for a candidate cell or by prach-RootSequenceIndex in SIB1-RequestConfig if the PRACH transmission is for SIB1 request. Additional preamble sequences, in case 64 preambles cannot be generated from a single root Zadoff-Chu sequence, are obtained from the root sequences with the consecutive logical indexes until all the 64 sequences are found. The logical root sequence order is cyclic; the logical index 0 is consecutive to LRA-2. The sequence number  is obtained from the logical root sequence index according to Tables 6.3.3.1-3 to 6.3.3.1-4B.

--------------------------------- end of TP for 38.211-------------------------------------

To moderator’s understanding, this topic was discussed in the RAN1 #121 FL summary [13] (Issue 13), and companies’ comments during RAN1 #121 are copied below for ease of reference:

| Company | Support or not | Comment |
| --- | --- | --- |
| Google | Support | This is to implement RAN1’s agreement |
| OPPO |  | We understand the intention. But according to editor’s comment, all the parameters provided by SIB1-RequestConfig should be captured in Clause 23. So we may make editor aware of this issue and leave editor find a way to capture this in Clause 23. |
| Apple | Support |  |
| LG |  | Agree with OPPO |
| vivo |  | Agree with OPPO |

Moderator hence has the following proposal according to the majority view above:

- FL Proposal 2-1

RAN1 to draw the following conclusion:

- It is up to the 38.213 editor to capture the difference of PRACH used for UL-WUS (compared to the legacy PRACH specified in 38.211) in Clause 23.

| Company | Support or not | Comment |
| --- | --- | --- |
| Google |  | We think this should be up to 38.211 editor instead of 38.213 editor. The problem is that current description in 38.211 is not correct instead of 38.213. One way is to capture the TP in 38.211 as we proposed, the other way is to add a reference to 38.213 Clause 23 in 38.211. |
| Fujitsu | Support | We are also fine with the proposal of adding a reference to Clause 23 of TS 38.213 in 38.211. |
| vivo |  | We are fine with both ways: capture it in TS 38.211 directly or capture it in TS 38.213 and add a reference to 38.213. |
| Samsung |  | We think this is more about other specs other than 213. Editors may also need a general sentence like in 213 to clarify the use of RRC parameters in SIB1-RequestConfig (this was done in 38.213). |
| Apple |  | Agree with Google. |
| CATT |  | OK |
| Moderator |  | We can briefly discuss in offline whether to leave it to editor or adopt a CR in 38.211 and add reference of 38.211 in 38.213 Clause 23. |
| LG |  | We are also fine with both ways as mentioned by vivo. |

- FL Proposal 2-2

RAN1 to agree on one of the following two options:

- Option 1: It is up to the 38.213 editor to capture the difference of PRACH used for UL-WUS (compared to the legacy PRACH specified in 38.211) in Clause 23.

  - 1 company supports

- Option 2: Adopt the following 38.211 CR to clarify the PRACH transmission for SIB1 request:

  - 4 companies support

- Option 3: Some high level text in 38.211 to describe this similar to 213

--------------------------------- start of TP for 38.211-------------------------------------

5.3.2	OFDM baseband signal generation for PRACH

<omitted text>

-	Δf is the subcarrier spacing of the initial uplink bandwidth part during initial access. If the PRACH transmission is for a candidate cell Δf is provided by ltm-PRACH-SubcarrierSpacing in EarlyUL-SyncConfig. If the PRACH transmission is for SIB1 request, Δf is provided by SIB1-RequestConfig as described in clause 23 of [5, TS 38.213]. Otherwise,  is the subcarrier spacing of the active uplink bandwidth part;

-	μ0 is the largest μ value among the subcarrier spacing configurations by the higher-layer parameter scs-SpecificCarrierList;

-	NBWP,istart is the lowest numbered resource block of the initial uplink bandwidth part and is derived by the higher-layer parameter initialUplinkBWP or initialUplinkBWP-RedCap during initial access and from the higher-layer parameters bwp-GenericParameters in EarlyUL-SyncConfig if the PRACH transmission is for a candidate cell and from the higher-layer parameters SIB1-RequestConfig if the PRACH transmission is for SIB1 request as described in clause 23 of [5, TS 38.213]. Otherwise, NBWP,istart is the lowest numbered resource block of the active uplink bandwidth part and is derived by the higher-layer parameter BWP-Uplink;

<omitted text>

6.3.3.1	Sequence generation

<omitted text>

There are 64 preambles defined in each time-frequency PRACH occasion, enumerated in increasing order of first increasing cyclic shift  of a logical root sequence, and then in increasing order of the logical root sequence index, starting with the index obtained from the higher-layer parameter prach-RootSequenceIndex or rootSequenceIndex-BFR or by msgA-PRACH-RootSequenceIndex if configured and a type-2 random-access procedure is initiated as described in clause 8.1 of [5, TS 38.213] or by prach-RootSequenceIndex in EarlyUL-SyncConfig if the PRACH transmission is for a candidate cell or by prach-RootSequenceIndex in SIB1-RequestConfig if the PRACH transmission is for SIB1 request as described in clause 23 of [5, TS 38.213]. Additional preamble sequences, in case 64 preambles cannot be generated from a single root Zadoff-Chu sequence, are obtained from the root sequences with the consecutive logical indexes until all the 64 sequences are found. The logical root sequence order is cyclic; the logical index 0 is consecutive to LRA-2. The sequence number  is obtained from the logical root sequence index according to Tables 6.3.3.1-3 to 6.3.3.1-4B.

--------------------------------- end of TP for 38.211-------------------------------------

- FL Proposal 2-3

RAN1 to agree on one of the following options:

- Option 2: Adopt the following 38.211 TP to clarify the PRACH transmission for SIB1 request (Supported by 4 companies)

- Option 3: RAN1 to discuss some other high level text in 38.211 to describe this clarification similar to 213 (Samsung)

--------------------------------- start of TP for 38.211-------------------------------------

5.3.2	OFDM baseband signal generation for PRACH

<omitted text>

-	Δf is the subcarrier spacing of the initial uplink bandwidth part during initial access. If the PRACH transmission is for a candidate cell Δf is provided by ltm-PRACH-SubcarrierSpacing in EarlyUL-SyncConfig. If the PRACH transmission is for SIB1 request, Δf is provided by SIB1-RequestConfig as described in clause 23 of [5, TS 38.213]. Otherwise,  is the subcarrier spacing of the active uplink bandwidth part;

-	μ0 is the largest μ value among the subcarrier spacing configurations by the higher-layer parameter scs-SpecificCarrierList;

-	NBWP,istart is the lowest numbered resource block of the initial uplink bandwidth part and is derived by the higher-layer parameter initialUplinkBWP or initialUplinkBWP-RedCap during initial access and from the higher-layer parameters bwp-GenericParameters in EarlyUL-SyncConfig if the PRACH transmission is for a candidate cell and from the higher-layer parameters SIB1-RequestConfig if the PRACH transmission is for SIB1 request as described in clause 23 of [5, TS 38.213]. Otherwise, NBWP,istart is the lowest numbered resource block of the active uplink bandwidth part and is derived by the higher-layer parameter BWP-Uplink;

<omitted text>

6.3.3.1	Sequence generation

<omitted text>

There are 64 preambles defined in each time-frequency PRACH occasion, enumerated in increasing order of first increasing cyclic shift  of a logical root sequence, and then in increasing order of the logical root sequence index, starting with the index obtained from the higher-layer parameter prach-RootSequenceIndex or rootSequenceIndex-BFR or by msgA-PRACH-RootSequenceIndex if configured and a type-2 random-access procedure is initiated as described in clause 8.1 of [5, TS 38.213] or by prach-RootSequenceIndex in EarlyUL-SyncConfig if the PRACH transmission is for a candidate cell or by prach-RootSequenceIndex in SIB1-RequestConfig if the PRACH transmission is for SIB1 request as described in clause 23 of [5, TS 38.213]. Additional preamble sequences, in case 64 preambles cannot be generated from a single root Zadoff-Chu sequence, are obtained from the root sequences with the consecutive logical indexes until all the 64 sequences are found. The logical root sequence order is cyclic; the logical index 0 is consecutive to LRA-2. The sequence number  is obtained from the logical root sequence index according to Tables 6.3.3.1-3 to 6.3.3.1-4B.

--------------------------------- end of TP for 38.211-------------------------------------

| Company | Which option do you prefer | Comment |
| --- | --- | --- |
| Ericsson |  | We should go for Option 2, the original text-proposal by Google was good. We can remove that part “as described by clause 23 of 213”, that is not needed. It actually not clear what it refers to, because clause 23 doesn’t say so much about PRACH, that is mainly in Clause 8 of 213. |
| vivo |  | We prefer Option 2. For Option 3, it is better to make it clear what high level text is and then determine whether it is OK. |
| Samsung2 |  | If this is the only text change needed for 211, we can be ok with Option 2. One comment on the detailed TP – the exact RRC parameter for the SCS of PRACH shall be provided, instead of using “SIB1-RequestConfig”. Also the location of “as described in clause 23 of [5, TS 38.213]” should be changed. - Δf is the subcarrier spacing of the initial uplink bandwidth part during initial access. If the PRACH transmission is for a candidate cell Δf is provided by ltm-PRACH-SubcarrierSpacing in EarlyUL-SyncConfig. If the PRACH transmission is for SIB1 request as described in clause 23 of [5, TS 38.213], Δf is provided by msg1-SubcarrierSpacing in SIB1-RequestConfig. Otherwise, is the subcarrier spacing of the active uplink bandwidth part; - μ0 is the largest μ value among the subcarrier spacing configurations by the higher-layer parameter scs-SpecificCarrierList; - NBWP,istart is the lowest numbered resource block of the initial uplink bandwidth part and is derived by the higher-layer parameter initialUplinkBWP or initialUplinkBWP-RedCap during initial access and from the higher-layer parameters bwp-GenericParameters in EarlyUL-SyncConfig if the PRACH transmission is for a candidate cell, and from the higher-layer parameter locationAndBandwidth in SIB1-RequestConfig, if the PRACH transmission is for SIB1 request as described in clause 23 of [5, TS 38.213]. Otherwise, NBWP,istart is the lowest numbered resource block of the active uplink bandwidth part and is derived by the higher-layer parameter BWP-Uplink; |
| Moderator |  | Samsung’ version seems like a good try. Let’s try it and remove the 213 reference as suggested by Ericsson in FL Proposal 2-4. |

- FL Proposal 2-4

Adopt the following 38.211 TP to clarify the PRACH transmission for SIB1 request

--------------------------------- start of TP for 38.211-------------------------------------

5.3.2	OFDM baseband signal generation for PRACH

<omitted text>

-	Δf is the subcarrier spacing of the initial uplink bandwidth part during initial access. If the PRACH transmission is for a candidate cell Δf is provided by ltm-PRACH-SubcarrierSpacing in EarlyUL-SyncConfig. If the PRACH transmission is for SIB1 request, Δf is provided by msg1-SubcarrierSpacing in SIB1-RequestConfig. Otherwise,  is the subcarrier spacing of the active uplink bandwidth part;

-	μ0 is the largest μ value among the subcarrier spacing configurations by the higher-layer parameter scs-SpecificCarrierList;

-	NBWP,istart is the lowest numbered resource block of the initial uplink bandwidth part and is derived by the higher-layer parameter initialUplinkBWP or initialUplinkBWP-RedCap during initial access and from the higher-layer parameters bwp-GenericParameters in EarlyUL-SyncConfig if the PRACH transmission is for a candidate cell, and from the higher-layer parameter locationAndBandwidth in SIB1-RequestConfig, if the PRACH transmission is for SIB1 request. Otherwise, NBWP,istart is the lowest numbered resource block of the active uplink bandwidth part and is derived by the higher-layer parameter BWP-Uplink;

<omitted text>

6.3.3.1	Sequence generation

<omitted text>

There are 64 preambles defined in each time-frequency PRACH occasion, enumerated in increasing order of first increasing cyclic shift  of a logical root sequence, and then in increasing order of the logical root sequence index, starting with the index obtained from the higher-layer parameter prach-RootSequenceIndex or rootSequenceIndex-BFR or by msgA-PRACH-RootSequenceIndex if configured and a type-2 random-access procedure is initiated as described in clause 8.1 of [5, TS 38.213] or by prach-RootSequenceIndex in EarlyUL-SyncConfig if the PRACH transmission is for a candidate cell or by prach-RootSequenceIndex in SIB1-RequestConfig if the PRACH transmission is for SIB1 request. Additional preamble sequences, in case 64 preambles cannot be generated from a single root Zadoff-Chu sequence, are obtained from the root sequences with the consecutive logical indexes until all the 64 sequences are found. The logical root sequence order is cyclic; the logical index 0 is consecutive to LRA-2. The sequence number  is obtained from the logical root sequence index according to Tables 6.3.3.1-3 to 6.3.3.1-4B.

--------------------------------- end of TP for 38.211-------------------------------------

| Company | Support or not | Comment |
| --- | --- | --- |
| LG | Support |  |
| Moderator |  | Agreed in online session. Discussion closed. |

- Issue 3: Source cell of searchSpaceZero and controlResourceSetZero

Background

In [3, CATT], it is mentioned that the source of the two parameters searchSpaceZero and controlResourceSetZero is from NES cell, currently the specification ([12]) is not clear and may be interpreted as from a first cell.

Proposal 1: Adopt TP #1 below for correcting the definition of the sources of the two parameters SearchSpaceZero and controlResourceSetZero.

-------------------------------------------- Start of text proposal to TS 38.213 v19.0.0 ---------------------------------------

23	UE procedure to request SIB1 reception

Unless otherwise mentioned, the higher layer parameters in this clause and in referenced clauses are provided by SIB1-RequestConfig on a first cell.

<<< UNCHANGED PART OMITTED >>>

In response to the PRACH transmission, the UE monitors PDCCH on the second cell to detect a DCI format 1_0 with CRC scrambled by a corresponding RA-RNTI during a window controlled by ra_ResponseWindow, as described in Clause 8.2. The UE monitors PDCCH according to a Type1-PDCCH CSS set provided by ra-SearchSpace, if provided; else provided by SearchSpaceZero from PBCH on a second cell as described in Clause 10.1, in a CORESET provided by controlResourceSetZero from PBCH on a second cell.

<<< UNCHANGED PART OMITTED >>>

-------------------------------------------- End of text proposal to TS 38.213 v19.0.0 ---------------------------------------

To moderator’s understanding, the searchSpaceZero and controlResourceSetZero for the PDCCH monitoring of OD-SIB1 on the NES cell (the second cell) both come from the UL-WUS configuration on the non-NES cell (the first cell). As the first sentence of 38.213 Clause 23 in [12] states that

- “Unless otherwise mentioned, the higher layer parameters in this clause and in referenced clauses are provided by SIB1-RequestConfig on a first cell.”

The proposed TP seems not needed. Open to hear views from more companies.

- FL Proposal 3-1

RAN1 to discuss whether to adopt the following TP from [3, CATT]

-------------------------------------------- Start of text proposal to TS 38.213 v19.0.0 ---------------------------------------

23	UE procedure to request SIB1 reception

Unless otherwise mentioned, the higher layer parameters in this clause and in referenced clauses are provided by SIB1-RequestConfig on a first cell.

<<< UNCHANGED PART OMITTED >>>

In response to the PRACH transmission, the UE monitors PDCCH on the second cell to detect a DCI format 1_0 with CRC scrambled by a corresponding RA-RNTI during a window controlled by ra_ResponseWindow, as described in Clause 8.2. The UE monitors PDCCH according to a Type1-PDCCH CSS set provided by ra-SearchSpace, if provided; else provided by SearchSpaceZero from PBCH on a second cell as described in Clause 10.1, in a CORESET provided by controlResourceSetZero from PBCH on a second cell.

<<< UNCHANGED PART OMITTED >>>

-------------------------------------------- End of text proposal to TS 38.213 v19.0.0 ---------------------------------------

| Company | Support or not | Comment |
| --- | --- | --- |
| MTK | No | To our understanding, the searchSpaceZero and controlResourceSetZero for the PDCCH monitoring of OD-SIB1 on the NES cell (the second cell) both come from the UL-WUS configuration on the non-NES cell (the first cell). Hence, current spec seems correct. |
| Google | No | Agree with FL |
| Xiaomi | No | Agree with FL. |
| Fujitsu | No | Agree with FL |
| vivo | No | Agree with FL |
| ZTE Corporation, Sanechips | No | Agree with FL |
| Samsung | No | Agree with FL’s assessment |
| Tejas | No |  |
| Apple | No | Agree with FL, our understanding is that the searchSpaceZero and controlResourceSetZero come from by SIB1-RequestConfig on a first cell. |
| Qualcomm | No | We agree with FL’s assessment |
| CATT |  | RAN1#119 Agreement CORESET0 of NES cell is used for RAR CORESET of that NES cell. 在RAN1#120 bis中，有： RAN1#120bis Agreement The parameter ra-SearchSpace in the UL-WUS configuration is of type SearchSpace. It can be seen that the RAR’s CORESET can be configured in two ways: one is through the ra-SearchSpace parameter provided in the WUS configuration, and the other is through CORESET#0 of the NES cell. The CR states: “The UE monitors PDCCH according to a Type1-PDCCH CSS set provided by ra-SearchSpace, if provided; else provided by SearchSpaceZero from PBCH on a second cell as described in Clause 10.1.” The first part of this sentence corresponds to the ra-SearchSpace parameter in the WUS configuration. Since this parameter is not mandatory, if it is not configured, CORESET#0 should be used instead. |
| Moderator |  | @CATT: For the CR text you mentioned: - “The UE monitors PDCCH according to a Type1-PDCCH CSS set provided by ra-SearchSpace, if provided; else provided by SearchSpaceZero from PBCH on a second cell as described in Clause 10.1.” To my understanding, the red part seems not correct as here both the ra-SearchSpace and SearchSpaceZero are provided from the UL-WUS configuration. Although ra-SearchSpace is optional, SearchSpaceZero should be mandatory. |
| LG | No | Agree with FL |
| Sharp | No | Agree with FL |

- Issue 4: Correction of k_SSB used for CORESET 0 determination

Background

In [4, Huawei/HiSilicon], it is mentioned that for on-demand SIB1, it is not clear which k_SSB between the one provided by UL WUS configuration from cell A and the k_SSB from NES Cell SSB is used to determine CORESET0 offset according to current specification ([12]).

- Adopt TP#1 to clarify the kSSB in table 13-7/8/10/10A as in Clause 13 is provided by UL WUS configuration from cell A rather than the kSSB of NES Cell SSB.

----------------------------------------------Start of TP#1 for section 23 of TS 38.213------------------------------------------------

23  UE procedure to request SIB1 reception

Unless otherwise mentioned, the higher layer parameters in this clause and in referenced clauses are provided by SIB1-RequestConfig on a first cell.

A UE can be provided, by NES_CellId, a physical cell identity of a second cell and an ARFCN by ARFCN-ValueNR for SS/PBCH block receptions on the second cell. When

-	the UE receives an SS/PBCH block on the second cell, and

-	kSSB>23 for FR1 or kSSB>11 for FR2 is indicated by the SS/PBCH block on the second cell, and

-	conditions for PRACH transmission associated with the SS/PBCH block on the second cell are satisfied [12, TS 38.331],

the UE is not required to monitor PDCCH on the second cell to detect the DCI format 1_0 with CRC scrambled by the SI-RNTI and can transmit a PRACH associated with the SS/PBCH block on the second cell based on:

-	a timing adjustment indicated by n-TimingAdvanceOffset, if provided, as described in Clause 4.2

-	a power determined as described in Clause 7.4

-	a procedure determined as in Clause 8.1 based on Type-1 random access procedure.

where kSSB for determining the common resource block [4, TS 38.211] is provided by k-ssb provided by SIB1-RequestConfig on a first cell is used for determining the common resource block [4, TS 38.211] and the offset RBs in Tables 13-7, 13-8, 13-10 and 13-10A.

---------------------------------------------End of TP#1 for section 23 of TS 38.213------------------------------------------------

To moderator’s understanding, the k_SSB provided by UL WUS configuration from cell A should be used to determine CORESET0 offset instead of the one from NES Cell SSB. Hence, the proposed TP#1 seems to be needed.

- FL Proposal 4-1

RAN1 to adopt TP#1 below to clarify the k_SSB provided by UL WUS configuration from cell A should be used to determine CORESET0 offset instead of the one from NES Cell SSB.

----------------------------------------------Start of TP#1 for section 23 of TS 38.213------------------------------------------------

23  UE procedure to request SIB1 reception

Unless otherwise mentioned, the higher layer parameters in this clause and in referenced clauses are provided by SIB1-RequestConfig on a first cell.

A UE can be provided, by NES_CellId, a physical cell identity of a second cell and an ARFCN by ARFCN-ValueNR for SS/PBCH block receptions on the second cell. When

-	the UE receives an SS/PBCH block on the second cell, and

-	kSSB>23 for FR1 or kSSB>11 for FR2 is indicated by the SS/PBCH block on the second cell, and

-	conditions for PRACH transmission associated with the SS/PBCH block on the second cell are satisfied [12, TS 38.331],

the UE is not required to monitor PDCCH on the second cell to detect the DCI format 1_0 with CRC scrambled by the SI-RNTI and can transmit a PRACH associated with the SS/PBCH block on the second cell based on:

-	a timing adjustment indicated by n-TimingAdvanceOffset, if provided, as described in Clause 4.2

-	a power determined as described in Clause 7.4

-	a procedure determined as in Clause 8.1 based on Type-1 random access procedure.

where kSSB for determining the common resource block [4, TS 38.211] is provided by k-ssb provided by SIB1-RequestConfig on a first cell is used for determining the common resource block [4, TS 38.211] and the offset RBs in Tables 13-7, 13-8, 13-10 and 13-10A.

---------------------------------------------End of TP#1 for section 23 of TS 38.213------------------------------------------------

| Company | Support or not | Comment |
| --- | --- | --- |
| MTK | Yes | The TP seems to match our understanding. |
| Google | Support |  |
| Xiaomi | Support. |  |
| Fujitsu | Support |  |
| vivo | Support | Just a minor comment: “where” is still needed for the TP |
| ZTE Corporation, Sanechips | Yes |  |
| Samsung |  | Tend to agree with the intention and the exact wording can be discussed |
| Tejas | Support |  |
| Apple |  | Understanding is the same as TP. However, similar to previous comments for issue 2, as mentioned at the beginning of the clause, “Unless otherwise mentioned, the higher layer parameters in this clause and in referenced clauses are provided by SIB1-RequestConfig on a first cell.’’, then it is already clear k_ssb should be provided by SIB1-RequestConfig on a first cell. |
| Qualcomm |  | The deleted text is clear to us. In addition, we support that the value of k-ssb is according to the listed table which can be clarified in 331 or 213. If it is clarified in TS 38.213, we think the following should be sufficient: “where kSSB for determining the common resource block [4, TS 38.211] is provided by k-ssb and the value of k-ssb is according to Clause 13.” |
| CATT | support |  |
| Moderator |  | With the point mentioned by Apple, I tend to think the TP may not be needed. We can discuss in offline session and companies’ further comments are welcomed. |
| LG |  | We share the view with Qualcomm. |
| Sharp |  | Same view with Apple |

- FL Proposal 4-2

RAN1 to adopt the one of the following two TPs to clarify the k-ssb provided by UL WUS configuration from cell A should be used to determine CORESET0 offset instead of the one from NES Cell SSB.

- (TP1 from HW, TP2 from QC)

----------------------------------------------Start of TP#1 for section 23 of TS 38.213------------------------------------------------

23  UE procedure to request SIB1 reception

Unless otherwise mentioned, the higher layer parameters in this clause and in referenced clauses are provided by SIB1-RequestConfig on a first cell.

<Unchanged text omitted>

where kSSB for determining the common resource block [4, TS 38.211] is provided by k-ssb provided by SIB1-RequestConfig on a first cell is used for determining the common resource block [4, TS 38.211] and the offset RBs in Tables 13-7, 13-8, 13-10 and 13-10A.

---------------------------------------------End of TP#1 for section 23 of TS 38.213------------------------------------------------

----------------------------------------------Start of TP#2 for section 23 of TS 38.213------------------------------------------------

23  UE procedure to request SIB1 reception

Unless otherwise mentioned, the higher layer parameters in this clause and in referenced clauses are provided by SIB1-RequestConfig on a first cell.

<Unchanged text omitted>

where kSSB for determining the common resource block [4, TS 38.211] is provided by k-ssb and the value of k-ssb is according to Clause 13.

---------------------------------------------End of TP#2 for section 23 of TS 38.213------------------------------------------------

| Company | Which TP do you prefer | Comment |
| --- | --- | --- |
| Sharp |  | After additional checking, we are OK to add “and the offset RBs in Tables 13-7, 13-8, 13-10 and 13-10A”, but we don’t need to add “provided by SIB1-RequestConfig on a first cell”, since it is already clear as mentioned by several companies. <Unchanged text omitted> where kSSB for determining the common resource block [4, TS 38.211] and the offset RBs in Tables 13-7, 13-8, 13-10 and 13-10A is provided by k-ssb. |
| Ericsson |  | We have the same view as Apple. |
| Qualcomm |  | kssb is to determine RE offset (in TS 38.211) and RB offset (in Clause 13 of tS 38.213) for determining the common resource block. Hence, we think it is sufficient to add Clause 13 as reference as follows: ---------------- Unless otherwise mentioned, the higher layer parameters in this clause and in referenced clauses are provided by SIB1-RequestConfig on a first cell. <Unchanged text omitted> where kSSB for determining the common resource block [4, TS 38.211] and Clause 13 is provided by k-ssb. -------------- |
| Sharp2 |  | >Qualcomm The RB offset in Clause 13 is not intended for determining the CRB, but rather for specifying the RB offset between the SSB and CORESET0. On the other hand, we agree with Qualcomm’s approach to simplify the reference by pointing directly to Clause 13. ---------------- <Unchanged text omitted> where kSSB for determining the common resource block [4, TS 38.211] and referenced in Clause 13 is provided by k-ssb. -------------- |
| Samsung2 |  | Generally referring to Clause 13 may not work correctly, since there will be use of k_SSB still based on the indication from PBCH of the second cell. |
| Moderator |  | Looking at companies’ reply, maybe the version from Sharp is a good compromise. Let’s try it in FL Proposal 4-3. |
| LG |  | We are fine with the proposal suggested by Sharp. |

- FL Proposal 4-3

RAN1 to adopt the following TP1 to clarify the k-ssb provided by UL WUS configuration from cell A should be used to determine CORESET0 offset instead of the one from NES Cell SSB.

----------------------------------------------Start of TP#1 for section 23 of TS 38.213------------------------------------------------

23  UE procedure to request SIB1 reception

Unless otherwise mentioned, the higher layer parameters in this clause and in referenced clauses are provided by SIB1-RequestConfig on a first cell.

<Unchanged text omitted>

where kSSB for determining the common resource block [4, TS 38.211] and the offset (RBs) in Tables 13-7, 13-8, 13-10 and 13-10A is provided by k-ssb.

---------------------------------------------End of TP#1 for section 23 of TS 38.213------------------------------------------------

| Company | Support or not | Comment |
| --- | --- | --- |
| LG | Support |  |
| Moderator |  | According to the Thursday morning offline discussion, a revised version is provided in FL Proposal 4-4. |

- FL Proposal 4-4

RAN1 to adopt the following TP1 to clarify the k-ssb provided by UL WUS configuration from cell A should be used to determine CORESET0 offset referenced in Clause 13.

----------------------------------------------Start of TP#1 for section 23 of TS 38.213------------------------------------------------

23  UE procedure to request SIB1 reception

Unless otherwise mentioned, the higher layer parameters in this clause and in referenced clauses are provided by SIB1-RequestConfig on a first cell.

<Unchanged text omitted>

where kSSB for determining the common resource block [4, TS 38.211] and offset (RBs) referenced in Clause 13 is provided by k-ssb.

---------------------------------------------End of TP#1 for section 23 of TS 38.213------------------------------------------------

| Company | Support or not | Comment |
| --- | --- | --- |
| Moderator |  | A revised version agreed in online session. Discussion closed. |

- Issue 5: Reference point of DMRS for PDCCH scheduling OD-SIB1 and PDSCH carrying OD-SIB1

Background

In [5, vivo], it is mentioned that

- “For OD-SIB1 reception, the reference point for DMRS of PDCCH scheduling OD-SIB1 and PDSCH carrying OD-SIB1 are different based on current 38.211 spec, which is different from legacy UE behaviours.”

Proposal 1: Adopt the TP1 in appendix A to support the following:

- ensure that the UE receives OD-SIB1 in the same way as legacy SIB1 reception, i.e., the reference point of DMRS for Type 0 PDCCH and DMRS for SIB1 is the same.

- align the reference point of DMRS for PDCCH when SIB1 is periodically transmitted or when SIB1 is on-demand triggered.

<TP 1> 3GPP TS 38.211 V18.6.0 (2025-03) ##### 7.4.1.3.2 Mapping to physical resources The UE shall assume the sequence rlm is mapped to resource elements k,lp,μ according to where the following conditions are fulfilled - they are within the resource element groups constituting the PDCCH the UE attempts to decode if the higher-layer parameter precoderGranularity equals sameAsREG-bundle, - all resource-element groups within the set of contiguous resource blocks in the CORESET where the UE attempts to decode the PDCCH if the higher-layer parameter precoderGranularity equals allContiguousRBs. The reference point for k is - subcarrier 0 of the lowest-numbered resource block in the CORESET if the CORESET is configured by the PBCH or by the controlResourceSetZero field in the PDCCH-ConfigCommon IE or by the controlResourceSetZero field in the SIB1-RequestConfig. - subcarrier 0 in common resource block 0 otherwise The quantity l is the OFDM symbol number within the slot. The antenna port p=2000. A UE not attempting to detect a PDCCH in a CORESET shall not make any assumptions on the presence or absence of DM-RS in the CORESET.

To moderator’s understanding, on DMRS for PDCCH scheduling OD-SIB1 and PDSCH carrying OD-SIB1, the reference point should be based on the controlResourceSetZero provided by UL WUS configuration from cell A instead of the one configured by the PBCH or by the controlResourceSetZero field in the PDCCH-ConfigCommon IE. Hence, the proposed TP#1 seems to be correct. However, like Issue 2, maybe this can be up to 38.213 editor to capture it in Clause 23.

- FL Proposal 5-1

RAN1 to draw the following conclusion:

- It is up to the 38.213 editor to capture in Clause 23 that the reference point of DMRS for PDCCH scheduling OD-SIB1 and PDSCH carrying OD-SIB1 should be based on the controlResourceSetZero provided by UL WUS configuration from cell A instead of the one configured by the PBCH or by the controlResourceSetZero field in the PDCCH-ConfigCommon IE.

| Company | Support or not | Comment |
| --- | --- | --- |
| MTK | Yes | The proposed TP seems to match our understanding. However, like Issue 2, maybe this can be up to 38.213 editor to capture it in Clause 23. We are open to capture it in 38.211 if majority of companies prefer this way. |
| Google | Support | We do not think we should capture everything in 38.213. Relevant part should be captured in correct spec, instead of everything in one spec. |
| Xiaomi | Yes |  |
| Fujitsu | Support | Same view as Issue 2. Adding a reference to Clause 23 of TS 38.213 in 38.211 is also fine with us. |
| vivo |  | Actually we prefer to keep this in 38.211 spec since DM-RS generation is a pure 211 issue. Besides, the proposal is more accurate to have the following modification: the reference point of DMRS for PDCCH scheduling OD-SIB1 and PDSCH carrying OD-SIB1 should be based on the subcarrier 0 of the lowest-numbered resource block in the controlResourceSetZero provided by UL WUS configuration from cell A instead of subcarrier 0 in common resource block 0 the one configured by the PBCH or by the controlResourceSetZero field in the PDCCH-ConfigCommon IE. |
| ZTE Corporation, Sanechips | Support |  |
| Samsung | Yes | Fine with the FL proposal. |
| Apple |  | We support the vivo proposal |
| CATT | Support |  |
| Moderator |  | We can briefly discuss in offline whether to leave it to editor or adopt a CR in 38.211 and add reference of 38.211 in 38.213 Clause 23. |
| LG | Support |  |

- FL Proposal 5-2

RAN1 to agree on one of the following two options:

- Option 1: It is up to the 38.213 editor to capture in Clause 23 that the reference point of DMRS for PDCCH scheduling OD-SIB1 and PDSCH carrying OD-SIB1 should be based on the controlResourceSetZero provided by UL WUS configuration from cell A instead of the one configured by the PBCH or by the controlResourceSetZero field in the PDCCH-ConfigCommon IE.

- Option 2: Adopt the following 38.211 TP1 to clarify the reference point of DMRS for PDCCH scheduling OD-SIB1 and PDSCH carrying OD-SIB1:

<TP 1> 3GPP TS 38.211 V18.6.0 (2025-03) ##### 7.4.1.3.2 Mapping to physical resources The UE shall assume the sequence rlm is mapped to resource elements k,lp,μ according to where the following conditions are fulfilled - they are within the resource element groups constituting the PDCCH the UE attempts to decode if the higher-layer parameter precoderGranularity equals sameAsREG-bundle, - all resource-element groups within the set of contiguous resource blocks in the CORESET where the UE attempts to decode the PDCCH if the higher-layer parameter precoderGranularity equals allContiguousRBs. The reference point for k is - subcarrier 0 of the lowest-numbered resource block in the CORESET if the CORESET is configured by the PBCH or by the controlResourceSetZero field in the PDCCH-ConfigCommon IE or by the controlResourceSetZero field in the SIB1-RequestConfig as described in clause 23 of [5, TS 38.213]. - subcarrier 0 in common resource block 0 otherwise The quantity l is the OFDM symbol number within the slot. The antenna port p=2000. A UE not attempting to detect a PDCCH in a CORESET shall not make any assumptions on the presence or absence of DM-RS in the CORESET.

- FL Proposal 5-3

RAN1 to agree on one of the following options:

- Option 2: Adopt the following 38.211 TP1 to clarify the reference point of DMRS for PDCCH scheduling OD-SIB1 and PDSCH carrying OD-SIB1. (Supported by 4 companies)

- Option 3: RAN1 to discuss some other high level text in 38.211 to describe this clarification similar to 213 (Samsung)

<TP 1> 3GPP TS 38.211 V18.6.0 (2025-03) ##### 7.4.1.3.2 Mapping to physical resources The UE shall assume the sequence rlm is mapped to resource elements k,lp,μ according to where the following conditions are fulfilled - they are within the resource element groups constituting the PDCCH the UE attempts to decode if the higher-layer parameter precoderGranularity equals sameAsREG-bundle, - all resource-element groups within the set of contiguous resource blocks in the CORESET where the UE attempts to decode the PDCCH if the higher-layer parameter precoderGranularity equals allContiguousRBs. The reference point for k is - subcarrier 0 of the lowest-numbered resource block in the CORESET if the CORESET is configured by the PBCH or by the controlResourceSetZero field in the PDCCH-ConfigCommon IE or by the controlResourceSetZero field in the SIB1-RequestConfig as described in clause 23 of [5, TS 38.213]. - subcarrier 0 in common resource block 0 otherwise The quantity l is the OFDM symbol number within the slot. The antenna port p=2000. A UE not attempting to detect a PDCCH in a CORESET shall not make any assumptions on the presence or absence of DM-RS in the CORESET.

| Company | Which option do you prefer | Comment |
| --- | --- | --- |
| Ericsson |  | Option 2. We actually prefer the original TP of Vivo. The reference to 213: “as described in clause 23 of [5, TS 38.213]” is not clear – what specifically is described there? It can be removed. |
| vivo |  | We prefer Option 2 our original TP. Agree with Ericsson, it seems no reason to refer to 213. For option 3, we can’t understand how to reflect this with a high level text. |
| Samsung |  | We are ok with Option 2, and reference to TS 38.213 may not be needed. |
| Moderator |  | Let’s try Option 2 with reference to 213 removed as suggested by companies in FL Proposal 5-4. |

- FL Proposal 5-4

Adopt the following 38.211 TP1 to clarify the reference point of DMRS for PDCCH scheduling OD-SIB1 and PDSCH carrying OD-SIB1 should be based on the controlResourceSetZero provided by UL WUS configuration from cell A instead of the one configured by the PBCH or by the controlResourceSetZero field in the PDCCH-ConfigCommon IE.

<TP 1> 3GPP TS 38.211 V18.6.0 (2025-03) ##### 7.4.1.3.2 Mapping to physical resources The UE shall assume the sequence rlm is mapped to resource elements k,lp,μ according to where the following conditions are fulfilled - they are within the resource element groups constituting the PDCCH the UE attempts to decode if the higher-layer parameter precoderGranularity equals sameAsREG-bundle, - all resource-element groups within the set of contiguous resource blocks in the CORESET where the UE attempts to decode the PDCCH if the higher-layer parameter precoderGranularity equals allContiguousRBs. The reference point for k is - subcarrier 0 of the lowest-numbered resource block in the CORESET if the CORESET is configured by the PBCH or by the controlResourceSetZero field in the PDCCH-ConfigCommon IE or by the controlResourceSetZero field in the SIB1-RequestConfig. - subcarrier 0 in common resource block 0 otherwise The quantity l is the OFDM symbol number within the slot. The antenna port p=2000. A UE not attempting to detect a PDCCH in a CORESET shall not make any assumptions on the presence or absence of DM-RS in the CORESET.

| Company | Support or not | Comment |
| --- | --- | --- |
| LG | Support |  |
| Moderator |  | Agreed in online session. Discussion closed. |

- Issue 6: RedCap UEs in UL-WUS configuration

Background

In [6, Samsung], the following analysis based on 38.331 text is provided:

| searchSpaceOtherSystemInformation (from 38.331) ID of the Search space for other system information, i.e., SIB2 and beyond (see TS 38.213 [13], clause 10.1). If the field is absent, the UE does not receive other system information in this BWP. This field is absent for the RedCap-specific initial DL BWP, if it does not include CD-SSB and the entire CORESET#0. In that case, a RedCap UE in RRC_IDLE or RRC_INACTIVE shall monitor PDCCH to receive other system information using searchSpaceOtherSystemInformation in the initial DL BWP that includes CD-SSB and the entire CORESET#0. |  |  |
| --- | --- | --- |
| searchSpaceSIB1 (from 38.331) ID of the search space for SIB1 message. In the initial DL BWP of the UE′s PCell, the network sets this field to 0. If the field is absent, the UE does not receive SIB1 in this BWP. (see TS 38.213 [13], clause 10). This field is absent for the RedCap-specific initial DL BWP, if it does not include CD-SSB and the entire CORESET#0. In that case, a RedCap UE in RRC_IDLE or RRC_INACTIVE shall monitor PDCCH to receive SIB1 using searchSpaceSIB1 in the initial DL BWP that includes CD-SSB and the entire CORESET#0. |  |  |
|  | Cell A supports RedCap UEs | Cell A bars RedCap UEs |
| Whether or how to acquire WUS configuration by RedCap UEs in Cell A | If RedCap specific initial DL BWP does not include CD-SSB and the entire CORESET#0, RedCap UEs will monitor both Type0-PDCCH or Type0A-PDCCH in common initial DL BWP which implies WUS configuration can be acquired by RedCap UEs. | Since WUS configuration is provided in SIBx (x>1) based on current RAN2 discussion, RedCap UEs will not have a chance to read WUS configuration because SIB1 indicates barring. |

Proposal 1: Add a parameter “RedCapAllowed” in WUS configuration.

To moderator’s understanding, whether to specify this “RedcCapAllowed” parameter in the UL-WUS configuration has been discussed for multiple meeting. Moderator tends to think we can have a final try with the new analysis provided from [6, Samsung] above.

- FL Proposal 6-1

Add a parameter “RedCapAllowed” into the UL-WUS configuration.

| Company | Support or not | Comment |
| --- | --- | --- |
| Google | No | We failed to see ambiguity without spec change. |
| Vivo | Yes | Without this, Redcap UEs camping in Cell A will send unnecessary OD-SIB1 request to NES cell resulting in both energy waste to both network and UE |
| Samsung | Support | As Google said, it is not related to an ambiguity. However, it is related to NES cell energy consumption and RedCap UE power consumption. Given RedCap feature is currently being deployed in real networks, it is clear this feature provides significant gain to both NES cell and RedCap UE as unnecessary UE/NW operations are avoided from the beginning. |
| Apple |  | Can be determined by RAN2. |
| Qualcomm |  | It has been being discussed in RAN2 |
| CATT |  | Not treat in ran1 |
| Moderator |  | If this has been discussed in RAN2, moderator tend to think we can leave it up to RAN2. Further comments are welcomed. |
| LG |  | It can be determined by RAN2. |
| Ericsson |  | - This is not critical for the feature - It has been discussed before and no agreement On a technical side, I acknowledge that adding the “RedCapAllowed” flag to the configuration may reduce the signalling for certain scenarios. But these are corner-case configurations. Moreover, it is preferred to minimize the size of UL WUS configuration and to reuse legacy procedures for access as much as possible, meaning that “RedCapAllowed” is provided in SIB1. |
| Moderator |  | During the Thursday offline session, Qualcomm delegate mentioned this has been discussed by RAN2 for multiple meetings and RAN1 can leave this to RAN2. |

- Issue 7: Indication of association between transmitted OD-SIB1 and SSB(s)

Background

In [7, ZTE/Sanechips] and [8, OPPO], it is mentioned that the following RAN1 #120b agreement is not totally reflected in the latest 38.213 R19 text [12] as highlighted in blue (ZTE), and the corresponding TP revision is provided.

Agreement in RAN1#120b The UE assumes that, in the OD-SIB1 window, PDCCH for an OD-SIB1 message is transmitted in PDCCH monitoring occasions corresponding only to the SSB associated with the transmitted PRACH for UL-WUS if this is indicated via UL WUS configuration by a 1-bit indication - If the 1-bit indication is included in UL WUS config and indicated as TRUE, the UE assumes that, in the OD-SIB1 window, PDCCH for an OD-SIB1 message is transmitted in PDCCH monitoring occasions corresponding only to the SSB associated with the PRACH for UL-WUS. - If the 1-bit indication is not included in UL WUS config or the 1-bit indication is FALSE, the UE assumes the same as legacy, i.e., in the OD-SIB1 window, PDCCH for an OD-SIB1 message is transmitted in at least one PDCCH monitoring occasion corresponding to each transmitted SSB.

TP from [7, ZTE/Sanechips]

23 UE procedure to request SIB1 reception -----------------------omitted text----------------------- If the higher layers at the UE identify a RAPID associated with a corresponding PRACH transmission from the UE in a PDSCH reception scheduled by the DCI format 1_0 with CRC scrambled by the RA-RNTI, the higher layers indicate to the physical layer to monitor PDCCH on the second cell to detect a DCI format 1_0 with CRC scrambled by the SI-RNTI according to a Type0-PDCCH CSS set provided by SearchSpaceZero. If the UE is provided sib1-pdcchRestrictionToPRACHAssociatedSSBs with a TRUE value, the UE monitors PDCCH only in monitoring occasions associated with the SS/PBCH block; otherwise, the UE assumes that PDCCH is transmitted in at least one PDCCH monitoring occasion corresponding to each transmitted SSB. The UE monitors PDCCH to detect the DCI format 1_0 with CRC scrambled by the SI-RNTI after a number of slots provided by od-sib1-windowStartOffset from the start of the first slot of the window controlled by ra_ResponseWindow, and for a number of slots provided by od-sib1-WindowDuration.

TP from [8, OPPO]

--------------- TP#1 of TS 38.213 in Clause 23 -----------------------

If the UE identifies a RAPID associated with a corresponding PRACH transmission from the UE in a PDSCH reception scheduled by the DCI format 1_0 with CRC scrambled by the RA-RNTI, the UE can be indicated by higher layers to monitor PDCCH on the second cell to detect a DCI format 1_0 with CRC scrambled by the SI-RNTI according to a Type0-PDCCH CSS set provided by SearchSpaceZero. If the UE is provided XYZ, the UE monitors PDCCH only in monitoring occasions associated with the SS/PBCH block. The UE starts monitoring PDCCH to detect the DCI format 1_0 with CRC scrambled by the SI-RNTI after a number of slots provided by od-sib1-windowStartOffset from the starting slot of the window controlled by ra_ResponseWindow, and for a number of slots provided by od-sib1-WindowDuration.

--------------- end of TP#1-------------------------------------------------------

To moderator’s understanding, it seems the proposed TP from [7, ZTE/Sanechips] is more aligned with the RAN1 #120b agreement compared to latest spec text [12]. As for the TP from [8, OPPO], it seems not formulated based on the latest spec text [12]. Moderator hence suggests to try the TP from [7, ZTE/Sanechips].

- FL Proposal 7-1

Adopt the following TP for clause 23 of TS 38.213 to reflect the RAN1 #120b agreement that SSB association with OD-SIB1 transmission is enabled if the 1-bit indication is included in UL WUS config and indicated as TRUE.

23 UE procedure to request SIB1 reception -----------------------omitted text----------------------- If the higher layers at the UE identify a RAPID associated with a corresponding PRACH transmission from the UE in a PDSCH reception scheduled by the DCI format 1_0 with CRC scrambled by the RA-RNTI, the higher layers indicate to the physical layer to monitor PDCCH on the second cell to detect a DCI format 1_0 with CRC scrambled by the SI-RNTI according to a Type0-PDCCH CSS set provided by SearchSpaceZero. If the UE is provided sib1-pdcchRestrictionToPRACHAssociatedSSBs with a TRUE value, the UE monitors PDCCH only in monitoring occasions associated with the SS/PBCH block; otherwise, the UE assumes that PDCCH is transmitted in at least one PDCCH monitoring occasion corresponding to each transmitted SSB. The UE monitors PDCCH to detect the DCI format 1_0 with CRC scrambled by the SI-RNTI after a number of slots provided by od-sib1-windowStartOffset from the start of the first slot of the window controlled by ra_ResponseWindow, and for a number of slots provided by od-sib1-WindowDuration.

| Company | Support or not | Comment |
| --- | --- | --- |
| MTK | Yes | The TP seems to align with previous RAN1 #120b agreement. |
| Google | No | We can revisit it after the ASN.1 is stable. Usually in ASN.1, only one candidate value ‘true’ is provided for such kind of parameter. Then for such kind of parameters, the description should be whether it is provided or not in spec. |
| Xiaomi | Yes |  |
| Fujitsu | Support |  |
| vivo | Yes |  |
| ZTE Corporation, Sanechips | Support | From the agreed RRC parameters it can be seen that the sib1-pdcchRestrictionToPRACHAssociatedSSBs can be set to TRUE or FALSE. So, align the agreement is needed. |
| Samsung | No | Similar view with Google. It will depend on how RAN2 captures the value range. In RAN2, when a parameter has “TRUE”/”FALSE” value (or “enable/disable” … value), the “FALSE” is default when the parameter is not provided and the “TRUE” is default when it is provided. |
| Tejas | Support |  |
| Apple | Yes |  |
| CATT | Yes | The TP seems to align with previous RAN1 #120b agreement. |
| Moderator |  | Moderator tends to think as the TRUE and FALSE settings are from RAN1 #120b agreement, maybe we do not need RAN2 to capture the value range and RAN1 can do the work: Agreement in RAN1#120b … by a 1-bit indication - If the 1-bit indication is included in UL WUS config and indicated as TRUE, the UE assumes that, in the OD-SIB1 window, PDCCH for an OD-SIB1 message is transmitted in PDCCH monitoring occasions corresponding only to the SSB associated with the PRACH for UL-WUS. - If the 1-bit indication is not included in UL WUS config or the 1-bit indication is FALSE, the UE assumes the same as legacy, i.e., in the OD-SIB1 window, PDCCH for an OD-SIB1 message is transmitted in at least one PDCCH monitoring occasion corresponding to each transmitted SSB. |
| LG | Support |  |
| Sharp | Support | The TP aligns with current RAN2 parameter description in running CR, and no issue even if the FALSE value is removed from the final parameter. |
| Moderator |  | During the online discussion, some companies think the RAN1 agreement on description of TRUE and FALSE of the 1-bit indication should be specified in RAN2 spec (Ex. 38.331), and normally RAN1 spec does not use the wording of “provided with a TRUE value”. This issue can be revisited if it is checked that the RAN1 agreement is not captured in all related RAN1/RAN2 spec. |
| Ericsson |  | Without the TP the paragraph is incorrect: Assume sib1-pdcchRestrictionToPRACHAssociatedSSBs is provided with value “false”. Then, according to the paragraph, since the parameter is provided “the UE monitors PDCCH only in monitoring occasions associated with the SS/PBCH block.” Obviously this is incorrect. Either we accept the TP, or we rewrite the paragraph in order not to duplicate specification text in 311 and 213. The following text-proposal can be used: 23 UE procedure to request SIB1 reception -----------------------omitted text----------------------- If the higher layers at the UE identify a RAPID associated with a corresponding PRACH transmission from the UE in a PDSCH reception scheduled by the DCI format 1_0 with CRC scrambled by the RA-RNTI, the higher layers indicate to the physical layer to monitor PDCCH on the second cell to detect a DCI format 1_0 with CRC scrambled by the SI-RNTI according to a Type0-PDCCH CSS set provided by SearchSpaceZero. If the UE is provided sib1-pdcchRestrictionToPRACHAssociatedSSBs, Based on higher layer configuration, the UE monitors PDCCH only in monitoring occasions associated with the SS/PBCH block; otherwise or, the UE assumes that PDCCH is transmitted in at least one PDCCH monitoring occasion corresponding to each transmitted SSB. The UE monitors PDCCH to detect the DCI format 1_0 with CRC scrambled by the SI-RNTI after a number of slots provided by od-sib1-windowStartOffset from the start of the first slot of the window controlled by ra_ResponseWindow, and for a number of slots provided by od-sib1-WindowDuration. |
| Moderator |  | We can revisit this issue in next meeting. |

- Issue 8: NES cell SI modification issue for idle UE

Background

In [8, OPPO], it is mentioned that when an NES cell updates SI by triggering short message to idle UEs, the Kssb is not expected to be changed in order not to confuse any legacy UE. The Kssb value is still maintained as larger than 24 for FR1 and 11 for FR2. If following the current specification, as the UE cannot determine the CORESET0 and search space 0, the UE will see this NES cell to be barred, which is not desired.

Proposal 2: For an idle UE of an NES cell, if the UE receives a short message indicating SI modification, and if after reading the MIB, the Kssb value is still larger than 24 for FR1 and 11 for FR2, the UE assumes that the CORESET0 and search space locations are same as before receiving the short message.

To moderator’s understanding, the Kssb value setting after SI modification is up to NW implementation to ensure correct UE behavior for Kssb value larger than 24 for FR1 or 11 for FR2. RAN1 can further discuss whether there is an issue for the brough up scenario.

- FL Proposal 8-1

RAN1 to discuss whether to draw the following conclusion:

- For an idle UE of an NES cell, if the UE receives a short message indicating SI modification, and if after reading the MIB, the Kssb value is still larger than 24 for FR1 and 11 for FR2, the UE assumes that the CORESET0 and search space locations are same as before receiving the short message.

| Company | Support the conclusion or not | Comment |
| --- | --- | --- |
| MTK |  | Not sure whether there is an issue for the mentioned scenario. Open to discuss/hear other companies’ views. |
| Google | No | This seems to be a RAN2’s issue. |
| Xiaomi | No | Legacy UE cannot camp on the NES cell as kSSB>23 for FR1 or kSSB>11 for FR2. |
| Fujitsu |  | We share the same view as FL that the setting of Kssb value after SI modification can be ensured by NW implementation. |
| vivo |  | We support this proposal. It seems RAN2 is also discussing this as open issue in this meeting. |
| ZTE Corporation, Sanechips | No | Agree that it can be up to NW implementation. |
| Samsung |  | Tend to agree with the FL’s assessment and unclear yet it is an issue. |
| Tejas | Support | Can be solved by network implementation |
| Apple |  | Tend to agree with Google |
| CATT | No | To our understanding, it is up to network implementation to decide the value of Kssb. Furthermore, the UE behavior after SI update is RAN2’s issue. |
| Moderator |  | Moderator suggests to leave this up to RAN2. More comments are welcomed. |
| LG | No | We also think it can be handled by gNB. |
| Moderator |  | During the Thursday morning offline session, OPPO mentioned that this issue has been solved by RAN2 in R2-2506218. Discussion is closed. |

- Issue 9: 38.213 section title text modification

Background

In [9, Ericsson], it is mentioned that the current title can be improved to capture the OD-SIB1 feature more accurately. UE can request the SIB1 transmission by the gNB, but it cannot request SIB1 reception.

Proposal 1 RAN1 to adopt the TP below for clause 23 of TS 38.213.

--------------- TP of TS 38.213 in Clause 23 -----------------------

<Unrelated part omitted>

23	UE procedure to request SIB1 reception for on demand SIB1 acquisition

<Unrelated part omitted>

--------------- end of TP#1-------------------------------------------------------

To moderator’s understanding, the proposed new title does seem better.

- FL Proposal 9-1

RAN1 to adopt the TP below for clause 23 of TS 38.213 to capture the OD-SIB1 feature more accurately.

--------------- Start of TP for TS 38.213 in Clause 23 -----------------------

<Unrelated part omitted>

23	UE procedure to request SIB1 reception for on demand SIB1 acquisition

<Unrelated part omitted>

--------------- End of TP-------------------------------------------------------

| Company | Support or not | Comment |
| --- | --- | --- |
| MTK | Yes |  |
| Google | No | Spec is not broken without the change. |
| Xiaomi | Yes |  |
| vivo | Yes |  |
| Samsung | No | Someone can think it is a procedure when a UE wants to receive SIB1. In this perspective, we don’t see it has a critical issue. Nothing essential to require a correction. |
| Tejas | Support |  |
| CATT | OK |  |
| Moderator |  | We can quicky try this during the offline session. If quick consensus is not achievable, I would suggest to drop it for now. |
| LG | Support |  |
| Sharp | OK |  |
| Moderator |  | Not agreeable during online session as some companies think not that essential. Discussion closed. |

- Issue 10: Value range for od-sib1-windowStartOffset

Background

In [9, Ericsson], [10, Apple], and [11, MediaTek], it is mentioned that the value range for od-sib1-windowStartOffset is still not determined (only agreed to be 0 or positive value, and unit should be in slot.).

Ericsson

- RAN2 has had an e-mail discussion about the value range of od-sib1-windowStartOffset, which concluded that this is up to RAN1 to decide.

- The value range {0, 20, 40, 80} ms appears sufficient and it occupies only 2 bits of information.

Apple

- The value range of od-sib1-windowStartOffset is the same as that of ra-ResponseWindow ({sl1, sl2, sl4, sl8, sl10, sl20, sl40, sl80}).

MediaTek

- For the value range of od-sib1-windowStartOffset (unit in slot), adopt INTEGER (0..15).

For ease of discussion, a figure illustrating the time offset to OD-SIB1 window is provided below:

Figure. Reference time point and time offset to OD-SIB1 window (revised from [R1-2408856, Qualcomm])

To moderator’s understanding, all proposed value range can work. As mentioned by Ericsson, RAN1 can make decision in RAN1 #122. Moderator suggests to follow majority views with the following proposal.

- FL Proposal 10-1

For the value range of od-sib1-windowStartOffset (only agreed to be 0 or positive value with unit in slot), RAN1 selects one of the following options:

- Option 1: {0, 20, 40, 80} ms (need to revert previous agreement that unit is in slot)

- Option 2: Same as ra-ResponseWindow ({sl1, sl2, sl4, sl8, sl10, sl20, sl40, sl80})

- Option 3: INTEGER (0..15) with unit in slot

| Company | Which option do you prefer | Comment |
| --- | --- | --- |
| MTK | Option 3 | We are open to all options, while Option 3 may have the smallest delay for UE to acquire the OD-SIB1. |
| Google | Option 3 |  |
| Xiaomi | Option 3 |  |
| Fujitsu | Option 2 |  |
| ZTE Corporation, Sanechips | Option 3 |  |
| Samsung |  | In our view, Option 1 is too broad while Option 3 has too small granularity. So, fine with Option 2. |
| Apple | Option 2 | According to the previous agreement, the OD-SIB1 monitoring window size is at least 20ms. Agreement On the value range for od-sib1-duration, adopt the following: - {20, 40, 80, 160, 320} ms We do not see the benifit to provide such fine granularity of the offset as proposed by Option 3. On the other hand, Option 2 can also provide small delay for UE by configuring 1,2,4...slots, while giving more options for a larger time offset if NW wants to send OD-SIB1 in a later time and saves UE power. |
| Qualcomm |  | Some combination of Option 1 and Option 2 should be supported {sl0, sl1, sl2, sl4, sl8, sl10, sl20, sl40, sl80} Option 3 is less preferred: unnecessary granularity (1 slot) and lacking support of large value(s). |
| CATT | Option 2 | But we can accept other options. The issue needs to be solved anyway. |
| Moderator |  | During the offline session, we can have a quick selection among the following: - Option 2: Same as ra-ResponseWindow ({sl1, sl2, sl4, sl8, sl10, sl20, sl40, sl80}) - Option 3: INTEGER (0..15) with unit in slot - QC: Option 3 is too fine granularity, ok with option 2 - Option 4: {sl0, sl1, sl2, sl4, sl8, sl10, sl20, sl40, sl80} - Ericsson: Add sl0 and remove sl80 |
| LG | Option 4 | We support Option 4. |

- FL Proposal 10-2

For the value range of od-sib1-windowStartOffset (only agreed to be 0 or positive value with unit in slot), RAN1 selects one of the following options:

- Option 2: Same as ra-ResponseWindow ({sl1, sl2, sl4, sl8, sl10, sl20, sl40, sl80}) (Apple)

- Option 4: {sl0, sl1, sl2, sl4, sl8, sl10, sl20, sl40, sl80} (Qualcomm)

- Option 5: {sl0, sl1, sl2, sl4, sl8, sl10, sl20, sl40} (Ericsson)

- Option 6: Option 2 with sl0 if od-sib1-windowStartOffset is not configured (Nokia)

| Company | Which option do you prefer | Comment |
| --- | --- | --- |
| Moderator |  | Option 4 is agreed during online session. Discussion closed. |

- Issue 11: Other topics

Discussion place holder for issues not included above.

- FL Proposal 11-1

Companies are invited to bring up other proposals which (you think) should be treated in this meeting. Moderator would try to devise further discussion based on received comments.

- Proposal receiving support/attention from multiple companies would be visited first.

| Company | Comment |
| --- | --- |

Resulted RAN1 conclusion/agreement

The following is achieved in this meeting:

Agreement

The following is supported for the value range of od-sib1-windowStartOffset

- {sl0, sl1, sl2, sl4, sl8, sl10, sl20, sl40, sl80}

Agreement:

Adopt the following TP for Clause 5.3.2 and 6.3.3.1 in TS38.211 to clarify the PRACH transmission for SIB1 request.

--------------------------------- start of TP for 38.211-------------------------------------

5.3.2	OFDM baseband signal generation for PRACH

<omitted text>

-	 is the subcarrier spacing of the initial uplink bandwidth part during initial access. If the PRACH transmission is for a candidate cell  is provided by ltm-PRACH-SubcarrierSpacing in EarlyUL-SyncConfig. If the PRACH transmission is for SIB1 request,  is provided by msg1-SubcarrierSpacing in SIB1-RequestConfig. Otherwise,  is the subcarrier spacing of the active uplink bandwidth part;

-	 is the largest  value among the subcarrier spacing configurations by the higher-layer parameter scs-SpecificCarrierList;

-	 is the lowest numbered resource block of the initial uplink bandwidth part and is derived by the higher-layer parameter initialUplinkBWP or initialUplinkBWP-RedCap during initial access and from the higher-layer parameters bwp-GenericParameters in EarlyUL-SyncConfig if the PRACH transmission is for a candidate cell, and from the higher-layer parameter locationAndBandwidth in SIB1-RequestConfig, if the PRACH transmission is for SIB1 request. Otherwise,  is the lowest numbered resource block of the active uplink bandwidth part and is derived by the higher-layer parameter BWP-Uplink;

<omitted text>

6.3.3.1	Sequence generation

<omitted text>

There are 64 preambles defined in each time-frequency PRACH occasion, enumerated in increasing order of first increasing cyclic shift  of a logical root sequence, and then in increasing order of the logical root sequence index, starting with the index obtained from the higher-layer parameter prach-RootSequenceIndex or rootSequenceIndex-BFR or by msgA-PRACH-RootSequenceIndex if configured and a type-2 random-access procedure is initiated as described in clause 8.1 of [5, TS 38.213] or by prach-RootSequenceIndex in EarlyUL-SyncConfig if the PRACH transmission is for a candidate cell or by prach-RootSequenceIndex in SIB1-RequestConfig if the PRACH transmission is for SIB1 request. Additional preamble sequences, in case 64 preambles cannot be generated from a single root Zadoff-Chu sequence, are obtained from the root sequences with the consecutive logical indexes until all the 64 sequences are found. The logical root sequence order is cyclic; the logical index 0 is consecutive to . The sequence number  is obtained from the logical root sequence index according to Tables 6.3.3.1-3 to 6.3.3.1-4B.

--------------------------------- end of TP for 38.211-------------------------------------

Agreement:

Adopt the following 38.211 TP1 to clarify the reference point of DMRS for PDCCH scheduling OD-SIB1 and PDSCH carrying OD-SIB1 should be based on the controlResourceSetZero provided by UL WUS configuration from cell A instead of the one configured by the PBCH or by the controlResourceSetZero field in the PDCCH-ConfigCommon IE.

- Reason for change:

  - UE behavious of OD-SIB1 reception are different from legacy SIB1 reception.

  - The reference point for DMRS for PDCCH is different when SIB1 is periodically transmitted or when SIB1 is on-demand triggered.

- Summary of change:

  - The reference point for DMRS for Type 0 PDCCH scheduling OD-SIB1 is subcarrier 0 of the lowest-numbered resource block in the CORESET0.

- Consequences if not approved:

  - The reference point for PDCCH scheduling OD-SIB1 and PDSCH carrying OD-SIB1 is different, which is different from legacy UE behaviours.

  - The reference point for PDCCH will change frequently with the change of NES state, which may bring more UE complexity.

<TP 1> 3GPP TS 38.211 V18.6.0 (2025-03) ##### 7.4.1.3.2 Mapping to physical resources The UE shall assume the sequence is mapped to resource elements according to where the following conditions are fulfilled - they are within the resource element groups constituting the PDCCH the UE attempts to decode if the higher-layer parameter precoderGranularity equals sameAsREG-bundle, - all resource-element groups within the set of contiguous resource blocks in the CORESET where the UE attempts to decode the PDCCH if the higher-layer parameter precoderGranularity equals allContiguousRBs. The reference point for is - subcarrier 0 of the lowest-numbered resource block in the CORESET if the CORESET is configured by the PBCH or by the controlResourceSetZero field in the PDCCH-ConfigCommon IE or by the controlResourceSetZero field in the SIB1-RequestConfig. - subcarrier 0 in common resource block 0 otherwise The quantity is the OFDM symbol number within the slot. The antenna port . A UE not attempting to detect a PDCCH in a CORESET shall not make any assumptions on the presence or absence of DM-RS in the CORESET.

Agreement:

Adopt the following TP1 to clarify the k-ssb provided by UL WUS configuration from cell A should be used to determine CORESET0 offset referenced in Clause 13.

---------------------------------------Start of TP#1 for section 23 of TS 38.213--------------------------------------

23  UE procedure to request SIB1 reception

Unless otherwise mentioned, the higher layer parameters in this clause and in referenced clauses are provided by SIB1-RequestConfig on a first cell.

<Unchanged text omitted>

where  for determining the common resource block [4, TS 38.211] and frequency offset for the CORESET referenced in Clause 13 is provided by k-ssb.

---------------------------------------End of TP#1 for section 23 of TS 38.213--------------------------------------

- 4 References (from RAN1 #122 if not specified)

- R1-2505137, On-demand SIB1 for Idle/Inactive mode UEs, Nokia, Nokia Shanghai Bell

- R1-2505258, On-demand SIB1 for Idle/Inactive Mode UE, Google

- R1-2505328, Maintenance on on-demand SIB1, CATT

- R1-2505356, Maintenance on on-demand SIB1 for eNES, Huawei, HiSilicon

- R1-2505379, Maintenance on on-demand SIB1 for idle/inactive Ues, vivo

- R1-2505543, Remaining issues on on-demand SIB1 for idle/inactive mode UEs, Samsung

- R1-2505596, Discussion on on-demand SIB1 for NES, ZTE Corporation, Sanechips

- R1-2505705, Remaining issue for on demand SIB1 for idle/inactive mode UE, OPPO

- R1-2505826, Maintenance on on-demand SIB1 for idle and inactive mode UEs, Ericsson

- R1-2505878, Remaining issues on on-demand SIB1 for IDLE/INACTIVE mode UEs, Apple

- R1-2506034, On-demand SIB1 for idle or inactive mode UEs, MediaTek Inc.

- R1-2504969, “Introduction of enhancements of network energy savings for NR”, Samsung, RAN1 #121

- R1-2504683, “FL summary 5 for on-demand SIB1 in idle/inactive mode”, Moderator (MediaTek), RAN1 #121