3GPP TSG RAN WG1 #122			R1-2506418

Bengaluru, India, Aug 25th – 29th, 2025

Agenda item:	9.10

Source: 	Moderator (NTT DOCOMO, INC.)

Title: 	Summary of discussion on Rel-19 NR UE features for IoT-NTN TDD mode

Document for:	Discussion and Decision

- Introduction

This document summarizes contributions submitted to AI 9.10. regarding Rel-19 NR UE features for IoT-NTN TDD mode.

[122-R19-UE_features] Email discussion on Rel-19 UE features – Ralf (AT&T), Naoya (DOCOMO), Ralf (AT&T) - To be used for sharing updates on online/offline schedule, details on what is to be discussed in online/offline sessions, tdoc number of the moderator summary for online session, etc

- Companies’ inputs

The following inputs have been provided in companies’ contributions in this meeting:

| Company | Proposal/observation |
| --- | --- |
| - CATT | In RAN1# #121 meeting [3], the following agreements have been made with respect to IoT NTN TDD mode. Thus, some components should be supplied for FG2-1. Agreement Confirm the working assumption of RAN1#120b with the following modifications DL bitmap (downlinkBitmap / downlinkBitmapNonAnchor) does not apply to a NB-IoT NTN TDD cell. - NOTE1: All D NB-IoT subframes (except those carrying NPSS/NSSS/SIB1-NB/NPBCH) are NB-IoT DL subframes. - NOTE2: “D NB-IoT subframes” are subframes contained within the set of D=8 usable consecutive downlink subframes in the TDD structure. - Up to RAN2 to decide whether DL bitmap (downlinkBitmap / downlinkBitmapNonAnchor) is constrained to not being present in a NB-IoT NTN TDD cell, or it can be configured with values all set to ‘1’. Agreement Confirm the following working assumption with modifications: For precompensation, from RAN1 perspective: - The UE may adjust its time/frequency pre-compensation before the beginning of each set of consecutive 8 uplink subframes. No pre-compensation gap is needed before the beginning of each set of consecutive 8 uplink subframes. - The UE may adjust its time/frequency pre-compensation at the beginning of an NPUSCH/NPRACH transmission (same behavior as Rel-18) - Segmented precompensation is not supported. - It is not supported to perform precompensation within the set of 8 consecutive uplink subframes other than at the beginning of an NPUSCH/NPRACH transmission - FFS: whether spec impact is in RAN1, RAN4 or both. NOTE: RAN1 may revisit this agreement if RAN4 reply LS shows concerns, including concerns on meeting the requirements without segmented precompensation. Agreement Confirm the following WA with the following modification For NPDCCH monitoring, new periodicities are introduced for NPDCCH monitoring by scaling the periodicity by a scaling factor F = 11.25 supporting values of G={11.25*4, 11.25*8} instead of G={4,8} Agreement For the issue of handling DL gaps in NB-IoT NTN TDD mode: - DL gaps apply based on current specifications, with the following modification: - dl-GapPeriodicity with a value of “sf1024” is supported instead of the value of “sf64”. As mentioned above, RAN1#121 meeting achieved some more detailed updates, such as new periodicities introduced for NPDCCH monitoring, modifications for DL bitmap and segmented pre-compensation, and etc. So there are some necessary changes for FG2-1 as listed in table1. Proposal 1: Some necessary changes should be applied to UE feature for IoT-NTN TDD as listed in table1 to follow RAN1 #121 progress. In initial WID [4]，the pattern is designed for TDD in band249. Besides, iridium system is operating normally within this band. So, in our perspective, there is no reason to use FG 2-1 for FDD operation. Finally, in “Need of FDD/TDD differentiation” column, it should be “TDD only”, and the bracket can be removed. Proposal 2: In “Need of FDD/TDD differentiation” column, it should be “TDD only”, and the bracket can be removed. Table 1 UE feature for IoT NTN TDD mode Index; Feature group; Components; Prerequisite feature groups; Need for the eNB to know if the feature is supported; Need for the UE to know if the feature is supported (only for V2X WI, where the PC5-RRC capability signalling is delivered between the UEs); Consequence if the feature is not supported by the UE; Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC); Need of FDD/TDD differentiation; Capability interpretation for mixture of FDD/TDD; Note; Mandatory/Optional. 2-1; Support of IoT-NTN TDD mode; 1. Support a TDD pattern with periodicity of N=9 radio frames, which consists of D=8 contiguous DL subframes and U=8 contiguous UL subframes. The guard period between the end of the DL subframes and the beginning of the UL subframes is fixed to be 50 ms at the ULSRP 2. The DL subframes of pattern are fixed to subframes [3 4 5 6 7 8 9 0] across two consecutive radio frames. 3. The non-U NB-IoT subframes are not considered by the UE as “NB-IoT UL subframes” 4. The non-D NB-IoT subframes are not considered by the UE as “NB-IoT DL subframes” 5. Support NPSS/NSSS/NPBCH/SIB1-NB transmissions dropped in non-D NB-IoT subframes. 6. Postponement of NPRACH transmissions in non-U NB-IoT subframes until the next U NB-IoT subframe(s), where the unit of postponement is one PRU. 7. Support of NPRACH periodicities of 90ms and 180ms, instead of 40ms and 80ms. 8、DL bitmap (downlinkBitmap / downlinkBitmapNonAnchor) does not apply to a NB-IoT NTN TDD cell. 9、UE may adjust time/frequency pre-compensation at the beginning of each set of consecutive 8 uplink subframes and/or an NPUSCH/NPRACH transmission, not support segmented pre-compensation and to perform pre-compensation within each set of consecutive 8 uplink subframes. No pre-compensation gap is needed before the beginning of each set of consecutive 8 uplink subframes. 10、For NPDCCH monitoring, support of G={11.25*4,11.25*8} instead of G={4,8}. 11、For DL gap, support of “s1024” instead of “s64”.; Rel-17 2-1b; Yes; N/A; IoT-NTN TDD mode is not supported; N/A; TDD only; N/A; Applicable only for 1616-1626.5 MHz MSS allocated band. Note: UE supporting band 249 must support this FG. Note: Rel-17 2-1b as prerequisite FG doesn’t imply the UE supporting this FG must support IoT NTN FDD band.; Optional without capability signalling. |
| - Huawei, HiSilicon | There are two highlight parts as the remaining issues in the UE feature list: the need of FDD/TDD differentiation and capability interpretation for mixture of FDD/TDD. For the need of FDD/TDD differentiation, because this feature is for IoT-NTN TDD mode, it’s nature to support TDD only. Therefore, RAN1 should remove the bracket of “TDD only”. Similarly, for the capability interpretation for mixture of FDD/TDD, RAN1 should remove the bracket of “N/A”. Proposal 1: Support to remove the brackets of “TDD only” and “N/A”. In RAN1#121 meeting [2], RAN1 achieved the following agreement: Agreement Confirm the following working assumption with modifications: For precompensation, from RAN1 perspective: - The UE may adjust its time/frequency pre-compensation before the beginning of each set of consecutive 8 uplink subframes. No pre-compensation gap is needed before the beginning of each set of consecutive 8 uplink subframes. - The UE may adjust its time/frequency pre-compensation at the beginning of an NPUSCH/NPRACH transmission (same behavior as Rel-18) - Segmented precompensation is not supported. - It is not supported to perform precompensation within the set of 8 consecutive uplink subframes other than at the beginning of an NPUSCH/NPRACH transmission - FFS: whether spec impact is in RAN1, RAN4 or both. NOTE: RAN1 may revisit this agreement if RAN4 reply LS shows concerns, including concerns on meeting the requirements without segmented precompensation. In Rel-17, the segmented UL transmission (Rel-17 FG2-1c) is mandatory supported for NGSO. Thus, RAN2 did not assign separate IODT bit for it and merged it into ntn-Connectivity-EPC-r17 corresponding to Rel-17 FG2-1b without TDD/FDD differentiation. Considering Rel-17 FG2-1b is prerequisite of Rel-19 FG2-1, it forced a TDD only UE to support segmented UL transmission which is not used according to RAN1 specifications. Therefore, we think it should be clarified that Rel-17 2-1b as prerequisite FG doesn’t imply the UE supporting this FG should support segmented UL transmission on IoT NTN TDD band. Proposal 2: Add a note: Rel-17 2-1b as prerequisite FG doesn’t imply the UE supporting this FG should support segmented UL transmission on IoT NTN TDD band. 2-1; Support of IoT-NTN TDD mode; 1. Support a TDD pattern with periodicity of N=9 radio frames, which consists of D=8 contiguous DL subframes and U=8 contiguous UL subframes. The guard period between the end of the DL subframes and the beginning of the UL subframes is fixed to be 50 ms at the ULSRP 2. The DL subframes of pattern are fixed to subframes [3 4 5 6 7 8 9 0] across two consecutive radio frames. 3. The non-U NB-IoT subframes are not considered by the UE as “NB-IoT UL subframes” 4. The non-D NB-IoT subframes are not considered by the UE as “NB-IoT DL subframes” 5. Support NPSS/NSSS/NPBCH/SIB1-NB transmissions dropped in non-D NB-IoT subframes 6. Postponement of NPRACH transmissions in non-U NB-IoT subframes until the next U NB-IoT subframe(s), where the unit of postponement is one PRU. 7. Support of NPRACH periodicities of 90ms and 180ms, instead of 40ms and 80ms.; Rel-17 2-1b; Yes; N/A; IoT-NTN TDD mode is not supported; N/A; [TDD only]; [N/A]; Applicable only for 1616-1626.5 MHz MSS allocated band Note: UE supporting band 249 must support this FG Note: Rel-17 2-1b as prerequisite FG doesn’t imply the UE supporting this FG must support IoT NTN FDD band Note: Rel-17 2-1b as prerequisite FG doesn’t imply the UE supporting this FG support segmented UL transmission on IoT NTN TDD band.; Optional without capability signalling. |
| - vivo | According to [1], there are still two open issues for the FG of the IoT NTN TDD mode: Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC); Need of FDD/TDD differentiation; Capability interpretation for mixture of FDD/TDD; Note; Mandatory/Optional. n/a; [TDD only]; [N/A]; Applicable only for 1616-1626.5 MHz MSS allocated band Note: UE supporting band 249 must support this FG Note: Rel-17 2-1b as prerequisite FG doesn’t imply the UE supporting this FG must support IoT NTN FDD band; Optional without capability signalling. Since the type of FG for IoT TDD is defined as N/A, there is no need to further distinguish between TDD and FDD, given that the corresponding UE behaviour and TDD frame structure are already specified in the specification. Therefore, the label [TDD only] can be removed or replaced with ‘N/A’. Similarly, capability interpretation for mixture of FDD/TDD is not required. Regarding the components, since new periodicity/scaling factor for NPDCCH monitoring is supported, it is suggested adding a new component: Support of new periodicities for NPDCCH monitoring by scaling the periodicity by a scaling factor F = 11.25 supporting values of G={11.25*4, 11.25*8} instead of G={4,8}. Proposal 1. Agree on the FG with the following clarifications - [TDD only] is changed to ‘N/A’ or removed - Remove the bracket around [N/A] for Capability interpretation for mixture of FDD/TDD - Add a component for new periodicities for NPDCCH monitoring by scaling the periodicity by a scaling factor F = 11.25 supporting values of G={11.25*4, 11.25*8} instead of G={4,8} 2-1; Support of IoT-NTN TDD mode; 1. Support a TDD pattern with periodicity of N=9 radio frames, which consists of D=8 contiguous DL subframes and U=8 contiguous UL subframes. The guard period between the end of the DL subframes and the beginning of the UL subframes is fixed to be 50 ms at the ULSRP 2. The DL subframes of pattern are fixed to subframes [3 4 5 6 7 8 9 0] across two consecutive radio frames. 3. The non-U NB-IoT subframes are not considered by the UE as “NB-IoT UL subframes” 4. The non-D NB-IoT subframes are not considered by the UE as “NB-IoT DL subframes” 5. Support NPSS/NSSS/NPBCH/SIB1-NB transmissions dropped in non-D NB-IoT subframes 6. Postponement of NPRACH transmissions in non-U NB-IoT subframes until the next U NB-IoT subframe(s), where the unit of postponement is one PRU. 7. Support of NPRACH periodicities of 90ms and 180ms, instead of 40ms and 80ms 8. Support of new periodicities for NPDCCH monitoring by scaling the periodicity by a scaling factor F = 11.25 supporting values of G={11.25*4, 11.25*8} instead of G={4,8}; Rel-17 2-1b; Yes; N/A; IoT-NTN TDD mode is not supported; n/a; [TDD only] N/A; [N/A]; Applicable only for 1616-1626.5 MHz MSS allocated band Note: UE supporting band 249 must support this FG Note: Rel-17 2-1b as prerequisite FG doesn’t imply the UE supporting this FG must support IoT NTN FDD band; Optional without capability signalling. |
| - Xiaomi | Regarding to the UE features for IoT-NTN TDD mode, the following two issues should be further discussed in this meeting: - Need of FDD/TDD differentiation - Capability interpretation for mixture of FDD/TDD Since the IoT NTN TDD mode would operate only over the Iridium band, it’s natural to assume the FG only works over this specific band where no IoT NTN UE operates in FDD. In this sense, "TDD only" shall be filled in the column of "Need of FDD/TDD differentiation" and "NA" should be filled to "Capability interpretation for mixture of FDD/TDD". Proposal 1: The FG 2-1 only operates in TDD only mode and the relevant columns shall be filled accordingly. Proposal 2: Take the “Capability interpretation for mixture of FDD/TDD” as “NA”. |
| - ZTE Corporation, Sanechips | As lists in Table 1, following changes are suggested for UE feature for IoT-NTN TDD: - The FG is only applicable for TDD band. Hence, the column of FDD/TDD differentiation can be “TDD only”, and no need to consider the mixture of TDD and FDD. Proposal 1: Adopt the feature groups in Table 1 for Rel-19 IoT-NTN TDD mode. 2-1; Support of IoT-NTN TDD mode; 1. Support a TDD pattern with periodicity of N=9 radio frames, which consists of D=8 contiguous DL subframes and U=8 contiguous UL subframes. The guard period between the end of the DL subframes and the beginning of the UL subframes is fixed to be 50 ms at the ULSRP 2. The DL subframes of pattern are fixed to subframes [3 4 5 6 7 8 9 0] across two consecutive radio frames. 3. The non-U NB-IoT subframes are not considered by the UE as “NB-IoT UL subframes” 4. The non-D NB-IoT subframes are not considered by the UE as “NB-IoT DL subframes” 5. Support NPSS/NSSS/NPBCH/SIB1-NB transmissions dropped in non-D NB-IoT subframes 6. Postponement of NPRACH transmissions in non-U NB-IoT subframes until the next U NB-IoT subframe(s), where the unit of postponement is one PRU. 7. Support of NPRACH periodicities of 90ms and 180ms, instead of 40ms and 80ms; Rel-17 2-1b; Yes; N/A; IoT-NTN TDD mode is not supported; n/a; [TDD only]; [N/A]; Applicable only for 1616-1626.5 MHz MSS allocated band Note: UE supporting band 249 must support this FG Note: Rel-17 2-1b as prerequisite FG doesn’t imply the UE supporting this FG must support IoT NTN FDD band; Optional without capability signalling. |
| - Samsung | Need for FDD/TDD differentiation: Prefer to consider TDD-only, according to the WID. There is no motivation to consider this feature in FDD. Capability interpretation for mixture of FDD/TDD: N/A Proposal 1: Consider the above updates for FG 2-1. |
| - OPPO | In RAN1#121 meeting, the UE features for IoT-NTN TDD mode have been extensively discussed, and the FGs are defined as attached in section 5. In the last meeting, RAN1 further agreed to make adjustments on periodicity for NPDCCH monitoring and DL gaps to adapt the TDD structure, and support performing pre-compensation before the beginning of each set of consecutive 8 uplink subframes. The related agreements are recopied below and should be captured as the components of the FG for IoT-NTN TDD mode. Agreement Confirm the following working assumption with modifications: For precompensation, from RAN1 perspective: - The UE may adjust its time/frequency pre-compensation before the beginning of each set of consecutive 8 uplink subframes. No pre-compensation gap is needed before the beginning of each set of consecutive 8 uplink subframes. - The UE may adjust its time/frequency pre-compensation at the beginning of an NPUSCH/NPRACH transmission (same behavior as Rel-18) - Segmented precompensation is not supported. - It is not supported to perform precompensation within the set of 8 consecutive uplink subframes other than at the beginning of an NPUSCH/NPRACH transmission - FFS: whether spec impact is in RAN1, RAN4 or both. NOTE: RAN1 may revisit this agreement if RAN4 reply LS shows concerns, including concerns on meeting the requirements without segmented precompensation Agreement Confirm the following WA with the following modification For NPDCCH monitoring, new periodicities are introduced for NPDCCH monitoring by scaling the periodicity by a scaling factor F = 11.25 supporting values of G={11.25*4, 11.25*8} instead of G={4,8} Agreement For the issue of handling DL gaps in NB-IoT NTN TDD mode: - DL gaps apply based on current specifications, with the following modification: - dl-GapPeriodicity with a value of “sf1024” is supported instead of the value of “sf64”. Proposal 1: Capture “A UE may adjust its time/frequency pre-compensation before the beginning of each set of consecutive 8 uplink subframes” as the component of the FG for IoT-NTN TDD mode. Proposal 2: Capture “Support of values of G={11.25*4, 11.25*8} instead of G={4,8} for NPDCCCH monitoring periodicity” as the component of the FG for IoT-NTN TDD mode. Proposal 3: Capture “Support of dl-GapPeriodicity with a value of “sf1024” instead of the value of “sf64” ” as the component of the FG for IoT-NTN TDD mode. |
| - Apple | Proposal 1: Remove the brackets in the “Need of FDD/TDD differentiation” column and the “Capability interpretation for mixture of FDD/TDD” column. |
| - Qualcomm Incorporated | This contribution includes the following proposed changes to UE features for IOT NTN TDD mode: - For FDD/TDD differentiation, we propose to add “TDD only” since this feature was only designed for TDD bands. - In line with the previous change, we propose to set the FDD/TDD diff to “N/A” Proposal: Endorse the following changes to IOT NTN TDD mode UE features 2-1; Support of IoT-NTN TDD mode; 1. Support a TDD pattern with periodicity of N=9 radio frames, which consists of D=8 contiguous DL subframes and U=8 contiguous UL subframes. The guard period between the end of the DL subframes and the beginning of the UL subframes is fixed to be 50 ms at the ULSRP 2. The DL subframes of pattern are fixed to subframes [3 4 5 6 7 8 9 0] across two consecutive radio frames. 3. The non-U NB-IoT subframes are not considered by the UE as “NB-IoT UL subframes” 4. The non-D NB-IoT subframes are not considered by the UE as “NB-IoT DL subframes” 5. Support NPSS/NSSS/NPBCH/SIB1-NB transmissions dropped in non-D NB-IoT subframes 6. Postponement of NPRACH transmissions in non-U NB-IoT subframes until the next U NB-IoT subframe(s), where the unit of postponement is one PRU. 7. Support of NPRACH periodicities of 90ms and 180ms, instead of 40ms and 80ms; Rel-17 2-1b; Yes; N/A; IoT-NTN TDD mode is not supported; n/a; TDD only; N/A; Applicable only for 1616-1626.5 MHz MSS allocated band Note: UE supporting band 249 must support this FG Note: Rel-17 2-1b as prerequisite FG doesn’t imply the UE supporting this FG must support IoT NTN FDD band; Optional without capability signalling. |
| - NTT DOCOMO, INC. | Regarding the FG 2-1 “support of IoT-NTN TDD mode”, we believe that new periodicities for NPDCCH monitoring was introduced as agreed in RAN1#120bis meeting, which can be captured as a new component. Besides, for the yellow highlighted part, the “Need of FDD/TDD differentiation” column should be confirmed as “TDD only” as the FG is applicable only for TDD band, and mixture of TDD and FDD is not needed. In summary, we have the following proposal. Proposal 1: Update FG 2-1 as follows. Index; Feature group; Components; Prerequisite feature groups; Need for the eNB to know if the feature is supported; Need for the UE to know if the feature is supported (only for V2X WI, where the PC5-RRC capability signalling is delivered between the UEs); Consequence if the feature is not supported by the UE; Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC); Need of FDD/TDD differentiation; Capability interpretation for mixture of FDD/TDD; Note; Mandatory/Optional. 2-1; Support of IoT-NTN TDD mode; 1. Support a TDD pattern with periodicity of N=9 radio frames, which consists of D=8 contiguous DL subframes and U=8 contiguous UL subframes. The guard period between the end of the DL subframes and the beginning of the UL subframes is fixed to be 50 ms at the ULSRP 2. The DL subframes of pattern are fixed to subframes [3 4 5 6 7 8 9 0] across two consecutive radio frames. 3. The non-U NB-IoT subframes are not considered by the UE as “NB-IoT UL subframes” 4. The non-D NB-IoT subframes are not considered by the UE as “NB-IoT DL subframes” 5. Support NPSS/NSSS/NPBCH/SIB1-NB transmissions dropped in non-D NB-IoT subframes 6. Postponement of NPRACH transmissions in non-U NB-IoT subframes until the next U NB-IoT subframe(s), where the unit of postponement is one PRU. 7. Support of NPRACH periodicities of 90ms and 180ms, instead of 40ms and 80ms 8. Support new periodicities for NPDCCH monitoring, by scaling the periodicity by a scaling factor F = 11.25; Rel-17 2-1b; Yes; N/A; IoT-NTN TDD mode is not supported; N/A; TDD only; N/A; Applicable only for 1616-1626.5 MHz MSS allocated band Note: UE supporting band 249 must support this FG Note: Rel-17 2-1b as prerequisite FG doesn’t imply the UE supporting this FG must support IoT NTN FDD band; Optional without capability signalling. |

- Discussion

Summary of companies’ views

FG 2-1: - Components: - Add “DL bitmap (downlinkBitmap / downlinkBitmapNonAnchor) does not apply to a NB-IoT NTN TDD cell.”: CATT - Add “UE may adjust time/frequency pre-compensation at the beginning of each set of consecutive 8 uplink subframes and/or an NPUSCH/NPRACH transmission, not support segmented pre-compensation and to perform pre-compensation within each set of consecutive 8 uplink subframes. No pre-compensation gap is needed before the beginning of each set of consecutive 8 uplink subframes.”: CATT, OPPO (before “and/or” part) - Add “For NPDCCH monitoring, support of G={11.25*4,11.25*8} instead of G={4,8}”: CATT, vivo, OPPO, DCM, - Add “For DL gap, support of “s1024” instead of “s64”: CATT, OPPO - FDD/TDD differentiation - TDD only: CATT, HW/Hisi, xiaomi, ZTE, Samsung, Apple, Qualcomm, DCM, - N/A: vivo, - Interpretation of mixture - N/A: CATT, HW/Hisi, vivo, Xiaomi, ZTE, Samsung, Apple, Qualcomm, DCM, - Note: - Add “Note: Rel-17 2-1b as prerequisite FG doesn’t imply the UE supporting this FG support segmented UL transmission on IoT NTN TDD band.”: HW/Hisi,

Proposal 1:

- Confirm “TDD only” in “Need of FDD/TDD differentiation” column in FG 2-1.

- Confirm “N/A” in “Capability interpretation for mixture of FDD/TDD” column in FG 2-1.

| Company | Comment |
| --- | --- |
| Vivo1 | Given that the TDD frame structure are already specified in the specification, we don’t think TDD only is really needed in the FG description, but if the majority wants to keep it, we can live with that. For 2nd bullet: support |
| Apple | Support Proposal 1. |
| Nordic | - Confirm “TDD only” in “Need of FDD/TDD differentiation” column in FG 2-1. -> OK - Confirm “N/A” in “Capability interpretation for mixture of FDD/TDD” column in FG 2-1. -OK |
| Huawei, HiSilicon | Support |
| Xiaomi | Support. |
| ZTE | Support |
| Nokia, NSB | Support |
| Iridium | Support proposal |
| OPPO | Support |
| NTT DOCOMO | Support |

Question 2:

Companies are encouraged to provide their view on the need of the following updates for FG 2-1 component:

- Q2-1: Add “DL bitmap (downlinkBitmap / downlinkBitmapNonAnchor) does not apply to a NB-IoT NTN TDD cell.”

- Q2-2: Add “UE may adjust time/frequency pre-compensation at the beginning of each set of consecutive 8 uplink subframes and/or an NPUSCH/NPRACH transmission, not support segmented pre-compensation and to  perform pre-compensation within each set of consecutive 8 uplink subframes. No pre-compensation gap is needed before the beginning of each set of consecutive 8 uplink subframes.”

- Q2-3: Add “For NPDCCH monitoring, support of G={11.25*4,11.25*8} instead of G={4,8}”

- Q2-4: Add “For DL gap, support of “s1024” instead of “s64”

| Company | Comment |
| --- | --- |
| Moderator | I prefer not to start “capturing all behaviours in 306” - although we agreed quite some components capturing behaviour details already . As long as it is captured in stage-3 spec, no strong need to capture in 306 in my view. We are not creating general stage-2 spec such as 300. Having said that, I would like to see how the group think about the proposals above. |
| Vivo1 | Q2-1: not needed, from RAN1 spec, it is clear that DL bitmap (downlinkBitmap / downlinkBitmapNonAnchor) does not apply when determining IoT TDD subframe. Q2-2: this can be captured in RAN4 specification. Q2-3: considering FG captures the new periodicity for PRACH, we are also fine to capture new values for NPDCCH monitoring Q2-4: considering FG captures the new periodicity for PRACH, we are also fine to capture new values for DL gap |
| Apple | Agree with moderator, we don’t need to capture all the agreements in the FG. It does not impact the implementation without introducing new components. |
| Nordic | Q2-1 no need, enough that this is captured in specifications Q2-2: no need, enough that this is captured in specifications Q2-3: no need, enough that this is captured in specifications Q2-4: no need, enough that this is captured in specifications |
| Huawei, HiSilicon | Agree with moderator |
| Xiaomi | No need to capture the proposals from Q2-1 to Q2-4 as components of FG 2-1, which have already reflected in specifications. |
| ZTE | Agree with moderator that no need of too much details. |
| Nokia, NSB | We think no need to add as UE should support these if UE access into the IoT NTN TDD cell. |
| Iridium | Agree with moderator, components are detailed in other specifications |
| OPPO | The same principle should be followed for all the components. If we do not capture NPDCCH periodicity and DL gap adjustments into the FG, the component 7 (PRACH periodicity adjustment) should also be removed. |
| NTT DOCOMO | Agree with moderator. Similar view with OPPO that if Q2-3/4 is not needed, component 7 is also not needed. |

Question 2:

Companies are encouraged to provide their view on the need of the following updates for FG 2-1 note:

- Add “Note: Rel-17 2-1b as prerequisite FG doesn’t imply the UE supporting this FG support segmented UL transmission on IoT NTN TDD band.”

| Company | Comment |
| --- | --- |
| Vivo1 | fine |
| Apple | Rel-17 2-1b Basic IoT over NTN support for NB-IoT and 2-1c Segmented UL transmission for NB-IoT are two separated features. The note seems not needed. |
| Nordic | No: in our understanding 2-1c deals with segmentation, not 2-1b |
| Huawei, HiSilicon | Support. As for the comment from Apple and Nordic, RAN2 decided to merge them instead of two separate FG as in RAN1 TR, because it is mandatory supported in NGSO. In order not to make it confused, we hope to make RAN2 note the potential impact to the Rel-17 2-1b. |
| Xiaomi | Not support. Share the same view as Apple and Nordic. \ |
| ZTE | Not support. 2-1c is for segment pre-compensation. |
| Nokia, NSB | Not support as it is still segment pre-compensation. |
| Nordic | We do not agree with Huawei comment. We see in 36.306 the following standalone capability 4.3.38.6 ntn-SegmentedPrecompensationGaps-r17 This field indicates the supported gap length between segments for PUSCH and PUCCH required by a UE supporting ce-ModeA-r13 or for NPUSCH required by a UE supporting ue-category-NB, for TA pre-compensation. This feature is only applicable if the UE supports either ue-category-NB or ce-ModeA-r13 and also supports ntn-Connectivity-EPCr17. If a UE does not include this field but includes ntn-Connectivity-EPC-r17, in case of overlapped transmission between successive uplink segments, UE shall follow the procedure specified in TS 36.213 [22]. This field is not applicable for UEs indicating support of ue-Category-NB and ntn-ScenarioSupport-r17 with value GSO. |
| Iridium | We do not support. |
| OPPO | Not support, and segmented pre-compensation is still necessary in NTN TDD. In our understanding, U=8 subframes can be regarded as a segment for time/frequency pre-compensation. Based on RAN1’s discussion, the time/frequency pre-compensation behavior may be captured into RAN4 specification. |
| NTT DOCOMO | Not support as segment pre-compensation is not included by 2-1b. |

- Conclusions

TBD

References

- R1-2505341	Discussion on UE features for IoT-NTN TDD mode	CATT

- R1-2505347	UE features for IoT-NTN TDD mode	Huawei, HiSilicon

- R1-2505402	UE features for IoT-NTN TDD mode	vivo

- R1-2505449	Discussion on UE features for IoT-NTN TDD mode	Xiaomi

- R1-2505507	Discussion on the UE feature for IoT-NTN TDD	ZTE Corporation, Sanechips

- R1-2505569	UE features for IoT NTN TDD mode	Samsung

- R1-2505710	Discussion on UE features for IoT-NTN TDD mode	OPPO

- R1-2505899	Views on UE features for Rel-19 IoT-NTN TDD mode	Apple

- R1-2506203	UE features for IOT-NTN TDD mode	Qualcomm Incorporated

- R1-2506291	Discussion on UE features for IoT-NTN TDD mode	NTT DOCOMO, INC.

Appendix: IoT-NTN TDD mode UE features after RAN1#120bis

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the eNB to know if the feature is supported | Need for the UE to know if the feature is supported (only for V2X WI, where the PC5-RRC capability signalling is delivered between the UEs) | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Capability interpretation for mixture of FDD/TDD | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2. IoT_NTN_TDD | 2-1 | Support of IoT-NTN TDD mode | 1. Support a TDD pattern with periodicity of N=9 radio frames, which consists of D=8 contiguous DL subframes and U=8 contiguous UL subframes. The guard period between the end of the DL subframes and the beginning of the UL subframes is fixed to be 50 ms at the ULSRP 2. The DL subframes of pattern are fixed to subframes [3 4 5 6 7 8 9 0] across two consecutive radio frames. 3. The non-U NB-IoT subframes are not considered by the UE as “NB-IoT UL subframes” 4. The non-D NB-IoT subframes are not considered by the UE as “NB-IoT DL subframes” 5. Support NPSS/NSSS/NPBCH/SIB1-NB transmissions dropped in non-D NB-IoT subframes 6. Postponement of NPRACH transmissions in non-U NB-IoT subframes until the next U NB-IoT subframe(s), where the unit of postponement is one PRU. 7. Support of NPRACH periodicities of 90ms and 180ms, instead of 40ms and 80ms | Rel-17 2-1b | Yes | N/A | IoT-NTN TDD mode is not supported | n/a | [TDD only] | [N/A] | Applicable only for 1616-1626.5 MHz MSS allocated band Note: UE supporting band 249 must support this FG Note: Rel-17 2-1b as prerequisite FG doesn’t imply the UE supporting this FG must support IoT NTN FDD band | Optional without capability signalling |