3GPP TSG RAN WG1 #122bis		R1-2507145

Prague, Czech Republic, October 13th – 17th, 2025

Source:	OPPO

Title:	Discussion on LS on OCC for IoT-NTN TDD mode

Agenda Item:	5

Document for:	Discussion and Decision

# Introduction

In RAN2#131 meeting, RAN2 assumed that OCC defined for R19 IoT NTN is applicable to IoT NTN TDD system, and an LS was sent to RAN1 for confirmation [1].

1. Overall Description: During RAN2#131 meeting, RAN2 discussed OCC defined for IoT NTN Rel-19 and assumes it can be supported for IoT NTN TDD mode without any spec impact: RAN2 assumes that OCC defined for IoT NTN Rel-19 can be supported for IoT NTN TDD mode but ask RAN1 for confirmation. As OCC for IoT NTN Rel-19 is defined by RAN1, RAN2 would like to check with RAN1 whether there are any concerns regarding this RAN2 assumption. 2. Actions: To RAN1: ACTION: RAN2 respectfully asks RAN1 to provide feedback if there is any concern on RAN2 assumption. 3. Date of Next TSG-RAN WG2 Meetings: TSG-RAN2 Meeting #131bis 13 - 17 October, 2025 Prague, CZ TSG-RAN2 Meeting #132 17 - 21 November, 2025 Dallas, USA

In this contribution, we provide our view and analysis on the applicability of OCC defined in R19 IoT NTN for IoT-NTN TDD system, and the draft reply LS is provided in our companion contribution [2].

# Discussion

In R19 IoT-NTN OCC, slot-level OCC2 and symbol-level OCC2 are supported for single-tone NPUSCH format1 with 15kHz SCS and 3.75kHz SCS respectively. Besides that, CDM DMRS with OCC spreading and TDM DMRS pattern are introduced for 15kHz SCS OCC and 3.75kHz SCS OCC respectively. Furthermore, in RAN1#121 meeting, RAN1 agreed to postpone the NPUSCH to the specific slot to align the TDM DMRS structure for OCC multiplexed UEs, as follows:

Agreement For the TDM DMRS positions that are activated within the TDM DMRS pattern: For an NPUSCH format 1 with 3.75kHz SCS allocated to start in slot m, the NPUSCH is postponed to start in the next slot, whose index satisfies (SFN * 5 + ns) mod 4 = 0. The UE transmits TDM DMRS in the nth slot after the start of its NPUSCH transmission according to: Criterion; DMRS activity. OCC sequence index 0 [1 1]; OCC sequence index 1 [1 -1]. n mod 4 = 0; ON; OFF. n mod 4 = 1; ON; OFF. n mod 4 = 2; OFF; ON. n mod 4 = 3; OFF; ON.

In R19 IoT NTN TDD, a frame structure constituting of D=8 consecutive DL subframes and U=8 consecutive UL subframes separated by fixed GPs at the ULSRP is defined and repeated every N=9 radio frames, where the D=8 DL subframes are downlink subframes 3, 4, 5, 6, 7, 8, 9, and 0 across two consecutive radio frames, as illustrated in Fig 1.

Fig 1. Frame Structure type 1 for IoT NTN TDD.

The TDM DMRS structure for 3.75kHz SCS OCC is not compatible with the frame structure defined in IoT NTN TDD, because the starting subframe of the U=8 UL subframes does not satisfy (SFN * 5 + ns) mod 4 = 0. If the NPUSCH with 3.75kHz SCS OCC is postponed to the slot satisfying (SFN * 5 + ns) mod 4 = 0, the TDM DMRS group consisting of 4 slots (i.e., 8ms) will be separated by two sets of the U=8 UL subframes. In this case, the TDM DMRS structure for 3.75kHz SCS will be broken.

Observation 1: The TDM DMRS structure for 3.75kHz SCS OCC is not compatible with the frame structure defined in IoT NTN TDD, because the starting subframe of the set of the U=8 UL subframes does not satisfy (SFN * 5 + ns) mod 4 = 0.

In addition, RAN2 assumes that OCC can be supported for IoT NTN TDD without any spec impact, which is inconsistent with our understanding. According to the current specification, it is explicitly captured that OCC only applies to NTN FDD. If OCC is applied to IoT NTN TDD, the specification impact is expected, whether for 15kHz SCS or 3.75kHz SCS.

TS36.211 10.1.3.6 Mapping to physical resources For a UE communicating over NTN in FDD operation, if the higher layer parameter npusch-OCC-Enabled is configured, OCC is indicated as enabled in DCI Format N0 as described in [3], and MrepNPUSCH≥2, - For Δf=15 kHz, after mapping to a slot, the slot shall be repeated NidenticalNPUSCH-1 additional times and pairs of slots shall be multiplied by q(m) before continuing the mapping of z(∙) to the following slot, the process repeats for all the slots in the NPUSCH format 1 transmission before OCC is applied, where for NscRU=1, MOCC=2, NidenticalNPUSCH=MOCC, and q(m) equals +1+1 or +1-1 as indicated by the DCI Format N0 as described in TS36.212 [3]. - For Δf=3.75 kHz, after mapping to a data symbol, the data symbol shall be repeated NidenticalNPUSCH-1 additional times and pairs of data symbols shall be multiplied by q(m) while skipping the DMRS symbol and guard period within the slot, before continuing the mapping of z(∙) to the following data symbol, the process repeats for all the data symbols in the NPUSCH Format 1 transmission before OCC is applied, where for NscRU=1, MOCC=2, NidenticalNPUSCH=MOCC, and q(m) equals +1+1 or +1-1 as indicated by the DCI Format N0 as described in TS36.212 [3].

Observation 2: It is explicitly captured that OCC only applies to NTN FDD in RAN1 specification. If OCC is applied to IoT NTN TDD, there will be RAN1 specification impact.

# Conclusion

In this contribution, we discussed the RAN2 LS regarding the applicability of OCC for IoT NTN TDD. The following proposals are made:

Observation 1: The TDM DMRS structure for 3.75kHz SCS OCC is not compatible with the frame structure defined in IoT NTN TDD, because the starting subframe of the set of the U=8 UL subframes does not satisfy (SFN * 5 + ns) mod 4 = 0.

Observation 2: It is explicitly captured that OCC only applies to NTN FDD in RAN1 specification. If OCC is applied to IoT NTN TDD, there will be RAN1 specification impact.

# Reference

[1]       R2-2506300, LS on OCC for IoT-NTN TDD mode, RAN2.

[2]       R1-2507146, Draft reply LS on OCC for IoT-NTN TDD mode, OPPO.